Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2017

| # Sub. Code : | # 14AE3008 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name: | # AEROSPACE STRUCTURAL ANALYSIS | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. |  | A beam having the cross-section shown in Figure is subjected to a bending moment of 1500 N m in a vertical plane. Calculate the maximum direct stress due to bending stating the point at which it acts. | CO2 | 20 |
| (OR) |  |  |  |  |
| 2. |  | Determine the direct stress distribution in the thin-walled Z-section shown in Figure produced by a positive bending moment Mx. | CO2 | 20 |
| 3. |  | Determine the shear flow distribution in the beam section shown in Fig.when it is subjected to a shear load in its vertical plane of symmetry. The thickness of the walls of the section is 2 mm throughout. | CO2 | 20 |
| (OR) |  |  |  |  |
| 4. | a. | How do you differentiate symmetrical bending and Unsymmetrical Bending? | CO2 | 05 |
|  | b. | A Cross section of a rectangular tube with constant thickness as shown in fig. Derive the following formula for the distance from the center line of the wall. The shear center value ise = b(2h+3b)/2(h+3b) | CO2 | 15 |
| 5. |  | Calculate the shear flow for the given section and angle of Twist. | CO2 | 20 |
| (OR) |  |  |  |  |
| 6. |  | The thin-walled single cell beam shown in Fig. has been idealized into a Combination of direct stress carrying booms and shear stress only carrying walls. If the section supports a vertical shear load of 10 KN acting in a vertical plane through booms 3 and 6, calculate the distribution of shear flow around the section.Boom areas: B1 = B8 = 200mm2 B2 = B7 = 250mm2 B3 = B6 = 400mm2 , B4=B5=100mm2 | CO2 | 20 |
| 7. |  | Figure shown as a single cell rectangular beam carrying the load of 100N.Find the internal resisting shear and draw the shear flow Pattern for the given section ABCD | CO3 | 20 |
| (OR) |  |  |  |  |
| 8. |  | Auniform flat plate of thickness t has a width b in the y direction and length Iin the x direction see in Fig. The edges parallel to the x axis are clamped and those parallel to the y axis are simply supported. Auniform compressive stress (T is applied in the x direction along the edges parallel to the y axis. Usingan energy method, find an approximate expression for the magnitude of the stress which causes the plate to buckle, assuming that the deflected shape of the plate is given by. | CO3 | 20 |
|  |  | Compulsory: |  |  |
| 9. | a. | What are the Advantage and Disadvantages of Finite Element Method? | CO3 | 10 |
|  | b. | Explain in detail the process of Discretization. |  | 10 |

ALL THE BEST