| # Course Code | # 14ME2004/17ME2006/18ME2020 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # MANUFACTURING PROCESSES | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | The upper molding flask used in sand casting is called__________. |  | CO1 | U | 1 |
| 2. | In sand casting, incomplete filling of mould cavity is called ____________. |  | CO1 | R | 1 |
| 3. | In _________ extrusion, the problem of friction is prevalent. |  | CO2 | U | 1 |
| 4. | Material which is forced out of the dies during forging, is called __________. |  | CO2 | R | 1 |
| 5. | __________ is defined as the straining of the sheet metal around a straight edge. |  | CO3 | U | 1 |
| 6. | In bending, the elastic recovery is called __________. |  | CO3 | R | 1 |
| 7. | __________is a welding technique used to connect parts which are nearly parallel and don't overlap in resistance. |  | CO4 | U | 1 |
| 8. | __________ is an arc welding process that uses a non-consumable tungsten electrode to produce the weld. |  | CO4 | R | 1 |
| 9. | Mention few advantages of powder metallurgy over manufacturing process. |  | CO6 | U | 1 |
| 10. | __________ is the process of manufacturing metal powders and making products from these metal powders. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Comment the purpose of gate in sand casting. |  | CO1 | An | 3 |
| 12. | Differentiate forward and backward extrusion. |  | CO2 | U | 3 |
| 13. | State Deep drawing. |  | CO3 | R | 3 |
| 14. | List out the various defects in Welding operations with neat sketch. |  | CO4 | U | 3 |
| 15. | Define Quenching. |  | CO5 | R | 3 |
| 16. | Mention the processes used for producing metal powders for powder metallurgy applications. |  | CO6 | A | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.No 17 to 23, Q.No 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Briefly explain the various defects in casting with relevant sketches. | CO1 | U | 12 |
| 18. |  | Describe the rolling principle and discuss the various types of rolling mills in detail with sketches. | CO2 | A | 12 |
| 19. |  | Describe the sheet metal bending process, its types and applications. | CO3 | An | 12 |
| 20. |  | Explain the TIG welding process with neat sketch. Also discuss its applications. | CO4 | An | 12 |
| 21. |  | With neat sketch, describe the working of electric discharge machining. | CO5 | U | 12 |
| 22. |  | Using a detailed diagram, explain how submerged arc welding is performed. | CO4 | R | 12 |
| 23. |  | Using a clear diagram, explain the working of electron beam machining. | CO5 | U | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | With mandatory flow chart explain the various steps involved in the process of powder metallurgy. | CO6 | A | 12 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Select the appropriate casting process to produce complex parts. |
| CO2 | Develop products with superior mechanical properties using bulk forming processes. |
| CO3 | Perform various sheet metal operations on metal sheets. |
| CO4 | Create permanent joints in working assemblies using welding techniques. |
| CO5 | Suggest appropriate unconventional manufacturing process for the machining of difficult-to machine materials. |
| CO6 | Enhance the properties of components using powder metallurgy. |

| Assessment Pattern as per Bloom’s Level |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 1 | 13 | - | 3 | - | - | 17 |
| CO2 | 1 | 4 | 12 | - | - | - | 17 |
| CO3 | 4 | 1 | - | 12 | - | - | 17 |
| CO4 | 13 | 4 | - | 12 | - | - | 29 |
| CO5 | 3 | 24 | - | - | - | - | 27 |
| CO6 | 1 | 1 | 15 | - | - | - | 17 |
|  |  |  |  |  |  |  | 124 |

| # Course Code | # 14ME2019/16ME2011/17ME2024/18ME2010 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # HEAT AND MASS TRANSFER | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Conduction resistance due to a plane wall of thickness L of material with thermal conductivity K and area A is ____________. |  | CO1 | U | 1 |
| 2. | Critical radius of insulation for a cylinder is _________. |  | CO1 | R | 1 |
| 3. | The velocity profile obtained for fully developed laminar flow in a tube is ____________. |  | CO2 | R | 1 |
| 4. | In a flow over a flat plate the Reynolds Number is 25600 and Prandtl number is 0.834. Find the average Nusselt number. |  | CO2 | A | 1 |
| 5. | Define emissive power. |  | CO3 | R | 1 |
| 6. | The value of Stefan Boltzmann constant is __________. |  | CO3 | R | 1 |
| 7. | When is the LMTD method most suitable for heat exchanger design calculations? |  | CO4 | U | 1 |
| 8. | For a parallel flow heat exchanger the capacity ratio is 1.5 and the NTU value is 2.0. The effectiveness is ____________. |  | CO4 | A | 1 |
| 9. | Mention a few industrial applications where boiling and condensation are important. |  | CO5 | U | 1 |
| 10. | Write the significance of Schmidt number. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | State Fourier’s law of heat conduction with neat sketches. |  | CO1 | R | 3 |
| 12. | Consider the square, which is divided equal increment in both the x and y directions. The left face is maintained at 1000C and the top face at 5000C, while two other faces are exposed to an environment at 1000C. Calculate the temperatures T1, T2, T3 and T4 at the four nodes. |  | CO2 | A | 3 |
| 13. | State Wien’s displacement law in heat radiation. |  | CO3 | R | 3 |
| 14. | # Define fouling. Describe its effects on heat exchanger performance. |  | CO4 | R | 3 |
| 15. | Draw the heat flux curve for various regions of flow boiling. |  | CO5 | R | 3 |
| 16. | Define mass concentration, molar concentration and mass fraction. |  | CO6 | R | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.No 17 to 23, Q.No 24 is Compulsory) |  |  |  |  |  |
| 17. |  | A wall is constructed of several layers. The first layer consists of masonry brick of 20 cm thick with thermal conductivity of 0.66 W/m K, the second layer consists of 3 cm thick mortar of thermal conductivity 0.6 W/m K, the third layer consists of 8 cm thick lime stone of thermal conductivity 0.58 W/m K and the outer layer consists of 1.2 cm thick plaster of thermal conductivity 0.6 W/m K. The heat transfer co-efficient of the interior and exterior of the wall is 5.6 W/m2K and 11 W/m2K respectively. Interior room temperature is 22oC and outside air temperature is - 5oC. Calculate i) Overall thermal resistance ii) Rate of heat transfer iii) Temperature at the junction between mortar and lime stone. | CO1 | A | 12 |
| 18. |  | A steam pipe 10 cm inner diameter 11 cm outer diameter is covered with an insulating material with thermal conductivity of 1W/mK. The steam temperature and the ambient temperatures are 200oC and 20oC respectively. If the convection heat transfer co-efficient between the insulating surface and air is 8W/m2K. Find the critical radius of insulation and the heat lost per meter of pipe for the value of rc. And also find the outer surface temperature. | CO1 | A | 12 |
| 19. |  | An aluminium sphere weighing 7.5 kg and initially at a temperature of 2900C is suddenly immersed in a fluid at 150C. The convective heat transfer coefficient is 60 W/m2K. Estimate the time required to cool the aluminium to 1000C, Using the lumped capacity method of analysis. [density = 2700 kg / m3, C= 900 J/kg/K, k = 205 W/mK]. | CO2 | A | 12 |
| 20. |  | State Buckingham’s π theorem. Explain the various parameters used in forced convection. Using dimensional analysis show that Nu=φ(Re,Pr). | CO2 | A | 12 |
| 21. |  | Calculate the net radiant heat exchange per m2 area for two large parallel plates at temperature of 247oC and 27oC respectively. (hot plate) = 0.9 and (cold plate) = 0.6. If the polished aluminium shield is placed between them, find the percentage reduction in the heat transfer, (shield) = 0.4. | CO3 | An | 12 |
| 22. |  | 0.5 kg of ethylene glycol flows through a thin walled copper tube of 1.25 cm diameter 0.375 kg/s of water flows in opposite direction through the annular space formed by this tube and the tube diameter is 2 cm. The ethylene glycol which enters at 1000C is required to leave at 60 0C, while water enters at 10 0C. Calculate the length of the tube required if the water flows in the same direction as ethylene glycol? S. No; Fluid property; Ethylene glycol at 80oC; Water at 27oC. 1; Density (kg/m3); 1078; 995. 2; Absolute viscosity(kg/ms); 3200x 10-6; 853 x10-6. 3; Specific heat (J/kgK); 2050; 4180. 4; Thermal conductivity(W/mK); 0.261; 0.614. | CO4 | An | 12 |
| 23. |  | A vertical tube of 60 mm outside diameter and 1.2 m long is exposed to steam at atmospheric pressure. The outer surface of the tube is maintained at a temperature of 50oC by circulating cold water through the tube. Calculate the i) rate of heat transfer to the coolant and ii) rate of condensation of steam. | CO5 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | An open pan, 20 cm diameter and 8 cm deep contains water at 250C and is exposed to dry atmospheric air. Estimate the diffusion coefficient of water in air, if the rate of diffusion of water is 8.54 x 10-4 kg/h. | CO6 | A | 12 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Formulate and analyze a heat transfer problem involving any of the three modes of heat transfer. |
| CO2 | Obtain exact solutions for the temperature variation using analytical methods where possible or employ approximate methods or empirical correlations to evaluate the rate of heat transfer. |
| CO3 | Evaluate radiation heat transfer between black, gray surfaces and the surroundings. |
| CO4 | Design devices such as heat exchangers and also estimate the insulation needed to reduce heat losses where necessary. |
| CO5 | Apply boiling and condensation correlations to two phase flow processes. |
| CO6 | Apply mass transfer correlations to process–based problems. |

| Assessment Pattern as per Bloom’s Level |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 4 | 1 | 24 | - | - | - | 29 |
| CO2 | 1 | - | 28 | - | - | - | 29 |
| CO3 | 5 | - | - | 12 | - | - | 17 |
| CO4 | 3 | 1 | 1 | 12 | - | - | 17 |
| CO5 | 3 | 1 | 12 | - | - | - | 16 |
| CO6 | 3 | 1 | 12 | - | - | - | 16 |
|  |  |  |  |  |  |  | 124 |

| # Course Code | # 14ME2028/17ME2027/18ME2017 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # DESIGN OF TRANSMISSION SYSTEMS | # Max. Marks | # 100 |

| Q. No. | Questions | CO | BL | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | When the length of the journal is equal to the diameter of the journal, then the bearing is said to be a __________. | CO1 | R | 1 |
| 2. | Teflon is used for bearings because of _________. | CO1 | U | 1 |
| 3. | To have a smooth operation, the minimum number of teeth on the smaller sprocket, for moderate speeds should be __________. | CO2 | U | 1 |
| 4. | The wire ropes make contact at _________. | CO2 | R | 1 |
| 5. | The size of the gear is usually specified by _________. | CO3 | R | 1 |
| 6. | The helix angle for single helical gears ranges from _________. | CO3 | U | 1 |
| 7. | When large gear reductions are needed _________ gears are used. | CO4 | R | 1 |
| 8. | The normal lead, in a worm having multiple start threads, is given by ___________. | CO4 | U | 1 |
| 9. | If the structure formula of a gearbox design is z = 2(1) 2(2), then the approximate number of gears required will be __________. | CO5 | R | 1 |
| 10. | Geneva mechanism is mainly used for__________. | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| 11. | List the various applications of bearings. | CO1 | U | 3 |
| 12. | Write the difference between flat and V-belt drive. | CO2 | R | 3 |
| 13. | Write short notes on types of gear tooth failures. | CO3 | An | 3 |
| 14. | Define module and Addendum circle. | CO4 | U | 3 |
| 15. | Write short notes on speed reducer. | CO5 | U | 3 |
| 16. | Discuss power screws, list of types of power screws used in industries. | CO6 | R | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.No 17 to 23, Q.No 24 is Compulsory) |  |  |  |  |
| 17. | Design a Journal Bearing for a centrifugal pump, if the diameter of the Journal is 120 mm, the load on Journal is 30 kN and the speed of the Journal is 1000 rpm. | CO1 | A | 12 |
| 18. | Design a fabric belt to transmit 12kW at 450rpm from an engine to a line shaft at 1200rpm. The diameter of the engine pulley is 600mm and the distance of the shaft from the engine is 2m. | CO2 | An | 12 |
| 19. | Design a spur gear to transmit 22.5 kW at 900 rpm; speed reduction is 2.5; Material for pinion is C15 steel. Take a pressure angle of 20° and the working life of the gears as 10,000 hrs. | CO3 | An | 12 |
| 20. | Design a right angle bevel gear drive with the speed of the pinion shaft = 300 rpm and that of the gear shaft = 150 rpm; the pinion is to have 20 teeth of involute profile with a module of 20 mm and a pressure angle of 20° and is to be of suitable material. Gear is of forged steel having allowable stress of 1500 kg/cm2, kW at gear shaft = 56; Assume service factor = 2. | CO3 | A | 12 |
| 21. | Design a worm gear drive to transmit 12 kW at 1200 rpm. The speed reduction desired is 30:1. The worm is made of hardened steel and the wheel of phosphor bronze. Check the heating capacity of gears and determine the efficiency. | CO4 | An | 12 |
| 22. | Select a wire rope for a vertical mine hoist to lift a load of 55 kN from a depth of 350 meters. A rope speed of 600 m/min is to be attained in 20 seconds. | CO2 | A | 12 |
| 23. | A radial cam rotating at 150 rpm is driving a 15 mm diameter translating roller follower to produce the following motion, the rise of 25 mm with SHM in 150 degrees of cam rotation, dwell for 60°, fall 25 mm with SHM in 120° of cam rotation and dwell for remaining 30°. Construct the cam contour with a pressure angle limited to approximately 20°. | CO6 | An | 12 |
| COMPULSORY QUESTION |  |  |  |  |
| 24. | Design a layout of a 12 speed gearbox for a milling machine having an output of speeds ranging from 180 to 2000 rpm. Power is applied to the gearbox from a 6 kW induction motor at 1440 rpm. Calculate the number of teeth on each gear wheel. Sketch the arrangement of the gearbox. | CO5 | An | 12 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Identify the working principles of mechanical components employed in mechanical transmission systems. |
| CO2 | Apply suitable theories and basic engineering principles and procedures to design the transmission elements. |
| CO3 | Select appropriate engineering design data from standard data books for mechanical transmission components. |
| CO4 | Design, transmission systems based on the requirements. |
| CO5 | Design and Draw speed reducer, multispeed gear box. |
| CO6 | Evaluate the torque, power and other functional requirements of power transmission Elements. |

| Assessment Pattern as per Bloom’s Level |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 1 | 4 | 12 | - | - | - | 17 |
| CO2 | 4 | 1 | 12 | 12 | - | - | 29 |
| CO3 | 1 | 1 | 12 | 15 | - | - | 29 |
| CO4 | 1 | 4 | - | 12 | - | - | 17 |
| CO5 | 1 | 3 | - | 12 | - | - | 16 |
| CO6 | 1 | 3 | - | 12 | - | - | 16 |
|  |  |  |  |  |  |  | 124 |

| # Course Code | # 14ME2029 / 17ME2020 / 18ME2016 | # Duration | # 3 hrs |
| --- | --- | --- | --- |
| # Course Name | # DESIGN OF MACHINE ELEMENTS | # Max. Marks | # 100 |

| Q. No. | Questions (Use of Approved Data Book & Data Sheets are permitted) |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) (Answer all the questions) |  |  |  |  |  |
| 1. | State about Young’s Modulus. |  | CO1 | R | 1 |
| 2. | Define the term Poisson’s ratio. |  | CO1 | R | 1 |
| 3. | Discuss on hydrodynamic lubrication. |  | CO2 | U | 1 |
| 4. | Discuss the use of Soderberg equation. |  | CO2 | U | 1 |
| 5. | Define critical speed of shafts. |  | CO3 | R | 1 |
| 6. | State any two types of keys. |  | CO3 | R | 1 |
| 7. | Generalize about Unwin’s formula. |  | CO4 | U | 1 |
| 8. | Tell any two types of welded joints. |  | CO4 | R | 1 |
| 9. | Describe the use of crankshaft. |  | CO5 | U | 1 |
| 10. | Cite the purpose of a flywheel. |  | CO5 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) (Answer all the questions) |  |  |  |  |  |
| 11. | Describe the terms (i) Elastic limit, and (ii) Factor of safety. |  | CO1 | R | 3 |
| 12. | Discuss the terms (i) Eccentricity of Journal bearing, and (ii) Minimum film thickness. |  | CO2 | U | 3 |
| 13. | Illustrate the effect of the keyway cut into a shaft. |  | CO3 | U | 3 |
| 14. | Define the terms (i) Spring index, and (ii) Spring ratio. |  | CO4 | R | 3 |
| 15. | Summarize the use of ribs in pistons. |  | CO5 | U | 3 |
| 16. | Mention any two theories of failure. |  | CO1 | R | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | A steel bracket is subjected to a load of 4.5 kN, as shown in figure. Determine the required thickness of the section at A-A in order to limit the tensile stress to 70 MPa. All dimensions are in mm | CO1 | An | 12 |
| 18. |  | A 6207 radial bearing is to operate in the following work cycle. - Radial load of 4500 N at 150 rpm for 30 % of the time - Radial load of 6750 N at 600 rpm for 10 % of the time - Radial load of 2250 N at 300 rpm for 60 % of the time The inner-ring rotates and loads are steady. Evaluate the expected average life of the bearing. | CO2 | An | 12 |
| 19. |  | Design a rigid flange coupling to transmit 20 kW at 1000 rpm. Assume allowable shear stress for shaft and key as 40 N/mm2; allowable crushing stress for key and bolt as 80 N/mm2 and allowable shear stress for coupling as 15 N/mm2. | CO3 | An | 12 |
| 20. |  | Design a knuckle joint for transmitting an axial load of 60 kN, for the following stresses, - In tension = 60 MPa - In compression = 75 MPa - In shear = 40 MPa. | CO4 | An | 12 |
| 21. |  | Design a cast iron piston for a single acting four stroke engine for the following data: Cylinder bore = 100 mm ; Stroke = 125 mm ; Maximum gas pressure = 5 N/mm2 ; Indicated mean effective pressure = 0.75 N/mm2 ; Mechanical efficiency = 80% ; Fuel consumption = 0.15 kg per brake power per hour ; Higher calorific value of fuel = 42 × 103 kJ/kg ; Speed = 2000 rpm. Any other data required for the design may be assumed. | CO5 | An | 12 |
| 22. |  | Design a close coiled helical compression spring for a service load ranging from 2250 N to 2750 N. The axial deflection of the spring for the load range is 6 mm. Assume a spring index of 5. The permissible shear stress intensity is 420 MPa and modulus of rigidity, G = 84 kN/mm2. Neglect the effect of stress concentration. | CO4 | A | 12 |
| 23. |  | A solid circular shaft is subjected to a bending moment of 3000 N-m and a torque of 10 000 N-m. The shaft is made of 45C8 steel having ultimate tensile stress of 700 MPa and a ultimate shear stress of 500 MPa. Assuming a factor of safety as 6, calculate the diameter of the shaft. | CO3 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | The turning moment diagram for a petrol engine is drawn to the following scales: Turning moment, 1 mm = 5 N-m; Crank angle, 1 mm = 1º. The turning moment diagram repeats itself at every half revolution of the engine and the areas above and below the mean turning moment line, taken in order are 295, 685, 40, 340, 960, 270 mm2. Determine the mass of 300 mm diameter flywheel rim when the coefficient of fluctuation of speed is 0.3% and the engine runs at 1800 rpm. | CO5 | A | 12 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the standard design procedure for Design of machine elements. |
| CO2 | Analyze stresses acting on components and determine the size based on theories of failure. |
| CO3 | Design machine components for a given load condition using design data hand books. |
| CO4 | Decide specifications as per standards given in design data and select standard components to improve interchangeability. |
| CO5 | Design and develop nonstandard machine components. |
| CO6 | Prepare a detail design layout and drawing of machine components. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | R | U | A | An | E | C | Total |
| CO1 | 8 | - | - | 12 | - | - | 20 |
| CO2 | - | 5 | - | 12 | - | - | 17 |
| CO3 | 2 | 3 | 12 | 12 | - | - | 29 |
| CO4 | 4 | 1 | 12 | 12 | - | - | 29 |
| CO5 | 2 | 3 | 12 | 12 | - | - | 29 |
| CO6 | - | - | - | - | - | - | - |
|  |  |  |  |  |  |  | 124 |

| # Course Code | # 17ME3033 | # Duration | # 3 hrs |
| --- | --- | --- | --- |
| # Course Name | # DESIGN AND ANALYSIS OF HEAT EXCHANGERS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (4 X 20 = 80 MARKS) (Answer all the Questions) |  |  |  |  |  |
| 1. | a. | Explain the factors to be considered while selecting heat exchangers. | CO1 | U | 10 |
|  | b. | Differentiate between regenerators and recuperators and state their scope of application. | CO1 | An | 10 |
|  |  | (OR) |  |  |  |
| 2. | a. | Discuss the classification of heat exchangers based on construction features. | CO1 | U | 10 |
|  | b. | Illustrate about differential thermal expansion in heat exchangers. | CO1 | A | 10 |
| 3. | a. | Deduce under what condition, the effectiveness NTU method is preferred over LMTD method as a method of analysis of Heat exchange. | CO2 | An | 10 |
|  | b. | Examine the causes of development of stress in a heat exchanger and how thermal stress can be minimized. | CO2 | A | 10 |
|  |  | (OR) |  |  |  |
| 4. | a. | Discuss how fouling is dealt while designing heat exchangers. | CO2 | U | 10 |
|  | b. | Illustrate the basic issues while designing a heat exchanger. | CO2 | A | 10 |
| 5. | a. | Sketch the schematic of a two shells heat exchanger. | CO3 | U | 10 |
|  | b. | In a co-current heat exchanger, an oil stream cooled from 450 K to 410 K by water. The inlet and outlet temperatures are 300 K and 350 K respectively. The exchanger consists of a number of tubes of 1 m length each. It is now desired to cool the oil to 390 K (instead of 410 K) while maintaining the flow rate of oil, flow rate of water, inlet temperatures of oil and water and the number of tubes at the same value as before. Calculate the length of each tube required for this purpose. Assume that the physical properties remain unchanged. | CO3 | An | 10 |
|  |  | (OR) |  |  |  |
| 6. | a. | Analyze and justify if there is extra advantage by providing Baffles in Shell and tube heat exchanger. | CO3 | An | 10 |
|  | b. | Construct the design procedure for a shell and tube heat exchanger. | CO3 | A | 10 |
| 7. | a. | Explain about a compact heat exchanger and write the importance of Area density in it. | CO4 | U | 10 |
|  | b. | Chart the reasons why fouling fluids are not used in compact heat exchangers. | CO4 | A | 10 |
|  |  | (OR) |  |  |  |
| 8. | a. | Illustrate why compact heat exchangers are more suitable for gaseous fluid. | CO4 | A | 10 |
|  | b. | Evaluate the design considerations for Compact Heat Exchangers. | CO4 | An | 10 |
| PART – B (1 X 20 = 20 MARKS) COMPULSORY QUESTION |  |  |  |  |  |
| 9. | a. | In a food processing plant, a brine solution is heated from -10°C to – 5.0°C in a double pipe parallel flow heat exchanger by water entering at 35°C and leaving at 20.5°C at the rate of 9 kg/min. Determine the heat exchanger area for an overall heat transfer coefficient of 860 W/m2 K. For water cp = 4.186 103 J/kgK. | CO5 | An | 10 |
|  | b. | Differentiate simple effect evaporators and multiple effect evaporators in terms of design, efficiency, operational costs, and steam economy. | CO6 | An | 10 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Identify the constructional aspects of various types of heat exchangers. |
| CO2 | Predict the effectiveness of heat exchangers NTU method. |
| CO3 | Calculate the design parameters of shell-and-tube heat exchanger. |
| CO4 | Analyze compact heat exchanger. |
| CO5 | Evaluate the performance of condensers. |
| CO6 | Formulate concepts of single and multi-effect evaporators. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | R | U | A | An | E | C | Total |
| CO1 | - | 20 | 10 | 10 | - | - | 40 |
| CO2 | - | 10 | 20 | 10 | - | - | 40 |
| CO3 | - | 10 | 10 | 20 | - | - | 40 |
| CO4 | - | 10 | 20 | 10 | - | - | 40 |
| CO5 | - | - | - | 10 | - | - | 10 |
| CO6 | - | - | - | 10 | - | - | 10 |
|  |  |  |  |  |  |  | 180 |

| # Course Code | # 17ME4001 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # APPLIED THERMAL ENGINEERING AND # EXPERIMENTAL METHODS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (4 X 20 = 80 MARKS) (Answer all the Questions) |  |  |  |  |  |
| 1. | a. | Steam enters a heat exchanger at 100oC and 0.01 MPa and leaves as a saturated liquid at 0.01 MPa. The steam mass flow rate is 10 kg/s. Water enters to cool the steam at 20oC and 0.01 MPa. Determine the minimum mass flow rate of cooling water so that the water does not entirely vaporize. | CO1 | A | 10 |
|  | b. | A fixed mass of oxygen undergoes a thermodynamic cycle comprising the following three processes consecutively. Process 1-2, constant pressure cooling from P1 = 0.5 MPa, V1=0.01m3 Process 2-3, Isothermal heating to P3 = 0.1 MPa, T3 = 25oC, V3=0.01m3 Process 3-1, constant volume heating back to state 1 Using the ideal gas equations with Cp =0.9kJ/kg K to determine the entropy change in each process. | CO1 | E | 10 |
|  |  | (OR) |  |  |  |
| 2. | a. | In a refrigeration cycle, the Qin =120 kg/s and Qout=180 kg/s. The corresponding ideal cycle would have Qout = 160 kg/sec with the same Qin. Determine the second law efficiency of the cycle. | CO1 | An | 10 |
|  | b. | Steam at 5 MPa and 600 oC enters a turbine at a steady rate of 10 kg/s and leaves at 0.2 MPa and 140oC. The steam losses 500 kW of heat to the environment, which is at 25oC and 1atm. Determine - The exergy of the steam at the inlet - The turbine power output - The maximum possible turbine power output - The irreversibility. | CO1 | A | 10 |
| 3. |  | Derive the expression for conservation of momentum and energy equation for fluid flow. | CO2 | A | 20 |
|  |  | (OR) |  |  |  |
| 4. | a. | Elucidate the various regimes of boundary layer in a flat plate with a neat sketch. | CO3 | An | 16 |
|  | b. | Describe the significance of the laminar sublayer. | CO3 | A | 4 |
| 5. |  | Explain how nanoparticles and nanofluids are characterized for heat transfer applications in detail. | CO4 | U | 20 |
|  |  | (OR) |  |  |  |
| 6. |  | Outline the various techniques used to measure the key thermophysical properties of nanofluids. | CO5 | A | 20 |
| 7. | a. | A certain obstruction-type flowmeter shown in the figure below is used to measure the flow of air at low velocities. The relation describing the flow rate is m=CA2gcP1RT1(P1-P2)1/2 where C = empirical-discharge coefficient, A = flow area, p1 and p2 = upstream and downstream pressures, respectively. T1 = upstream temperature, R = gas constant for air. Calculate the percent of uncertainty in the mass flow rate for the following conditions: C = 0.92 ± 0.005 (from calibration data) p1 = 25 psia ± 0.5 psia T1 = 70◦F ± 2◦F T1 = 530◦R ∆p = p1 − p2 = 1.4 psia ± 0.005 psia (measured directly) A = 1.0 in2 ± 0.001 in2 | CO5 | E | 20 |
|  |  | (OR) |  |  |  |
| 8. | a. | Summarize the thermal conductivity measurement techniques of fluids. | CO4 | U | 4 |
|  | b. | Explain in detail the transient hot wire method of measuring thermal conductivity. | CO4 | An | 16 |
| PART – B (1 X 20 = 20 MARKS) COMPULSORY QUESTION |  |  |  |  |  |
| 9. | a. | Explain any two types of optimization techniques with examples. | CO6 | A | 10 |
|  | b. | Deduce the problem using newton Raphson method f(x) = x2 + 54/x. | CO6 | A | 10 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Design various experimental systems based on the conservation laws of physics. |
| CO2 | Relate the knowledge in analyzing the heat transfer systems to the fundamental heat transfer laws. |
| CO3 | Identify high performance heat transfer fluids for thermal systems. |
| CO4 | Apply the knowledge of measurement techniques in the modern energy systems. |
| CO5 | Conduct the experiments and record data for further analysis. |
| CO6 | Demonstrate knowledge of various modelling techniques for optimization of experimental results. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | R | U | A | An | E | C | Total |
| CO1 |  |  | 20 | 10 | 10 |  | 40 |
| CO2 |  |  | 20 |  |  |  | 20 |
| CO3 |  |  | 4 | 16 |  |  | 20 |
| CO4 |  | 24 |  | 16 |  |  | 40 |
| CO5 |  |  | 20 |  | 20 |  | 40 |
| CO6 |  |  |  | 20 |  |  | 20 |
|  |  |  |  |  |  |  | 180 |

| # Course Code | # 18ME2012 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # STRENGTH OF MATERIALS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) (Answer all the questions) |  |  |  |  |  |
| 1. | State: Hooke’s law. |  | CO1 | R | 1 |
| 2. | Differentiate between stress and strain. |  | CO1 | U | 1 |
| 3. | List the different types of beams. |  | CO2 | U | 1 |
| 4. | Draw the shear force diagram for a cantilever beam. |  | CO2 | A | 1 |
| 5. | Define: neutral axis. |  | CO3 | U | 1 |
| 6. | Expand: UDL. |  | CO3 | R | 1 |
| 7. | Recognize the moment of inertia of a rectangular section about Y-Y axis. |  | CO4 | U | 1 |
| 8. | Discuss the term “Centroid”. |  | CO4 | U | 1 |
| 9. | Highlight the importance of torsion tests of shafts. |  | CO5 | A | 1 |
| 10. | Write the expression for deflection of helical springs. |  | CO5 | A | 1 |
| PART – B (6 X 3 = 18 MARKS) (Answer all the questions) |  |  |  |  |  |
| 11. | Draw the stress strain curve for an elastic material. |  | CO1 | A | 3 |
| 12. | List the types of transverse loading on beams. |  | CO2 | U | 3 |
| 13. | List all the assumptions made in the theory of simple bending. |  | CO3 | U | 3 |
| 14. | Describe: Perpendicular axis theorem. |  | CO4 | R | 3 |
| 15. | Write the expression of deflection of a shaft fixed at both ends. |  | CO5 | A | 3 |
| 16. | Define: Hoop stress in a cylinder. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Mention the expressions of the elastic constants. Also, derive the relationship between the elastic constants. | CO1 | A | 12 |
| 18. |  | A steel bar having cross-sectional area of 800 mm2, is subjected to axial forces as shown in figure given below. Find the total elongation of the bar. Take E = 2.0 x105 N/mm2. | CO1 | E | 12 |
| 19. |  | A cantilever beam of length 2 m carries an UDL of 2 kN/m length over the whole length and a point load of 3 kN at the free end. Draw the shear force and bending moment diagrams. | CO2 | An | 12 |
| 20. |  | A rectangular beam 200 mm deep and 300 mm wide is simply supported over a span of 8 meters. What UDL per meter the beam may carry, if the bending stress should not exceed 100 N/mm2. Take I = 6 x 106 mm4. | CO3 | An | 12 |
| 21. |  | Derive an expression of moment of inertia of a rectangular section, about an axis passing through the C.G. of the section and perpendicular to the base. | CO4 | A | 12 |
| 22. |  | A solid cylindrical shaft is to transmit 300 kW power at 100 rpm, if the shear stress is not to exceed 80 N/mm2, find its diameter. | CO5 | E | 12 |
| 23. |  | A beam 3 m long, simply supported at its ends, is carrying a point load W at its centre. If the slope at the ends of the beam should not exceed 1o, find the deflection at the centre of the beam. | CO2 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | A long cylinder is rotating at a speed of 3600 rpm. The diameter of the cylinder is 500 mm and density of its material is 8000 kg/m3. If Poisson’s ratio = 0.3, then, calculate the maximum stress in the cylinder. | CO6 | An | 12 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Recognize various types loads applied on machine components. |
| CO2 | Understand the nature of internal stresses that will develop within the components. |
| CO3 | Analyze the stresses acting simple geometry of structures. |
| CO4 | Evaluate strains and deformation due to the elastic stresses developed. |
| CO5 | Compute slopes and deflection in beams. |
| CO6 | Determine the torsional stresses of shaft and hoop stresses in cylinders. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | R | U | A | An | E | C | Total |
| CO1 | 1 | 1 | 15 | - | 12 | - | 29 |
| CO2 | - | 4 | 13 | 12 | - | - | 29 |
| CO3 | 1 | 4 | - | 12 | - | - | 17 |
| CO4 | 3 | 2 | 12 | - | - | - | 17 |
| CO5 | - | - | 5 | - | 12 | - | 17 |
| CO6 | - | 3 | - | 12 | - | - | 15 |
|  |  |  |  |  |  |  | 124 |

| # Course Code | # 18ME2014 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # SOLID MECHANICS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | ______ load is one which is considered to act at a point. |  | CO1 | U | 1 |
| 2. | Modulus of elasticity is defined as the ratio of____________. |  | CO1 | R | 1 |
| 3. | The ratio of lateral strain to linear strain is known as__________. |  | CO2 | R | 1 |
| 4. | A cantilever is a beam whose one support is _________and other is a _________ end. |  | CO3 | R | 1 |
| 5. | If diameter of a shaft is doubled the power transmitted capacity will be __________. |  | CO4 | U | 1 |
| 6. | In a solid shaft, maximum shear stress occurs at radius = radius of the shaft. True/False. |  | CO4 | U | 1 |
| 7. | Bending moment at supports in case of simply supported beams is always__________. |  | CO4 | A | 1 |
| 8. | At the neutral axis, bending stress is _________. |  | CO4 | U | 1 |
| 9. | _________ is ability of a system's structural capacity to be viable beyond its expected or actual loads. |  | CO6 | R | 1 |
| 10. | The ability of the material to deform without breaking is called ______. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | A square steel rod 20mm x 20mm in section is to carry an axial load (compressive) of 100 kN. Calculate the shortening in a length of 50mm. E=2.14x108 kN/m2 |  | CO2 | E | 3 |
| 12. | List the types of beams with suitable examples. |  | CO3 | R | 3 |
| 13. | State Hooke’s law. |  | CO1 | R | 3 |
| 14. | A beam 3 m long, simply supported at its ends, is carrying a point load W at the centre. If the slope at the ends of the beam should not exceed 1, find the deflection at the centre of the beam. |  | CO3 | E | 3 |
| 15. | Define the term neutral axis. |  | CO4 | R | 3 |
| 16. | Define the terms Principal plane and Principal stresses. |  | CO5 | R | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.No 17 to 23, Q.No 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Find the change in the dimensions and the change in volume of the bar shown in figure. Px =600 kN, Py =750 kN and Pz =300 kN. Take E = 200 GPa and Poisson’s ratio =0.3 | CO2 | E | 12 |
| 18. |  | Draw the S.F and B.M diagram for a cantilever beam as shown in figure. | CO3 | E | 12 |
| 19. |  | A beam is simply supported and carries a uniformly distributed load of 40 kN/m run over the whole span. The section of the beam is rectangular having depth as 500 mm. If the maximum stress in the material is 120 N/mm2 and the moment of inertia is 7 × 108 mm4, find the span of the beam. | CO3 | E | 12 |
| 20. |  | Two shafts of the same material and of same length are subjected to the same torque, if the first shaft is of a solid circular section and the second shaft is of hollow circular section, whose internal diameter is 2/3 of the outside diameter and the maximum shear stress developed in each shaft is the same, compare the weights of the shafts | CO4 | E | 12 |
| 21. |  | A steel girder of uniform section 14m long is simply supported at its ends A and B. It carries concentrated load of 90kN and 60kN at two points 3 m and 9.5 m from the end A. Calculate a. The deflection of the girder at the points under the two loads b. The maximum deflection Take I = 0.0064 m4and E = 210 GN/m2. | CO4 | E | 12 |
| 22. |  | A hollow cast iron cylinder 4 m long, 300 mm outer diameter, and thickness of metal 50 mm is subjected to a central load on the top when standing straight. The stress produced is 75000 kN/m2. Assume young’s modulus for cast iron as 1.5 × 108 kN/m2 and find (i) magnitude of the load, and (ii) longitudinal strain produced, (iii) total decrease in length. | CO1 | E | 12 |
| 23. |  | A solid round bar 60 mm in diameter and 2.5 m long is used as a strut. One end of the strut is fixed, while the other end is hinged. Find the safe compressive load for this strut, using Euler’s formula. Assume E = 200 GN/m2 and factor of safety is 3. | CO6 | E | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | At a point within a body subjected to two mutually perpendicular directions, the stresses are 80 N/mm2 tensile and 40 N/mm2 tensile. Each of the above stresses is accompanied by a shear stress of 60 N/mm2. Determine the normal stress, shear stress and resultant stress on an oblique plane inclined at an angle of 45º with the axis of minor stress. Confirm the results using analytical and graphical methods. | CO5 | E | 12 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand stress and strain relations in simple solids. |
| CO2 | Estimate stress and strain values in simple solids subjected thermal loads. |
| CO3 | Analyze the different types of loading and the consequent deflection. |
| CO4 | Determine maximum stress and angular deflection of solid and hollow shafts. |
| CO5 | Evaluate stress and strain using Mohr’s circle. |
| CO6 | Apply concepts of failure theories to determine safe design. |

| Assessment Pattern as per Bloom’s Level |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 4 | 1 |  |  | 12 |  | 17 |
| CO2 | 1 |  |  |  | 15 |  | 16 |
| CO3 | 4 |  |  |  | 27 |  | 31 |
| CO4 | 3 | 3 | 1 |  | 24 |  | 31 |
| CO5 | 3 |  |  |  | 12 |  | 15 |
| CO6 | 2 |  |  |  | 12 |  | 14 |
|  |  |  |  |  |  |  | 124 |

| # Course Code | # 18ME2015 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # KINEMATICS AND THEORY OF MACHINES | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | A ball and a socket joint forms a ___________. |  | CO1 | U | 1 |
| 2. | The component of the acceleration, parallel to the velocity of the particle, at the given instant is called_________. |  | CO1 | R | 1 |
| 3. | In a radial cam, the follower moves in a direction ________ to the cam axis. |  | CO2 | R | 1 |
| 4. | The size of a gear is usually specified by ________. |  | CO2 | R | 1 |
| 5. | A pair of action and reaction forces acting on a body are known as __________. |  | CO3 | U | 1 |
| 6. | In a turning moment diagram, the variations of energy above and below the mean resisting torque line is called _____________. |  | CO3 | R | 1 |
| 7. | The balancing of rotating and reciprocating parts of an engine is necessary when it runs at ___________ speed. |  | CO4 | U | 1 |
| 8. | For dynamic balancing of a shaft, both dynamic force and net couple should be ________. |  | CO4 | R | 1 |
| 9. | The natural frequency of transverse vibration for a shaft carrying a number of point loads and uniformly distributed load is obtained using ____________. |  | CO5 | U | 1 |
| 10. | When the pitching of a ship is upward, the effect of gyroscopic couple acting on it will be to move the ship _________ star-board. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Sketch and explain any one inversions of a single slider crank chain. |  | CO1 | An | 3 |
| 12. | Classify types of followers and follower motions. |  | CO2 | R | 3 |
| 13. | Write the conditions for a body to be in equilibrium under the action of two forces, three forces and two forces and a torque. |  | CO3 | U | 3 |
| 14. | Differentiate static and dynamic balancing with neat diagrams. |  | CO4 | U | 3 |
| 15. | Define ‘whirling speed’ or ‘critical speed’ of a shaft and also give the expression for the natural frequency of free transverse vibrations for a simply supported beam carrying a number of point loads by dunkerley’s method. |  | CO5 | R | 3 |
| 16. | Describe the gyroscopic effect on sea going vessels. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.No 17 to 23, Q.No 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Explain the following joints with neat sketches: i) Revolute joint, ii) Prismatic joint, iii) Spherical joint, iv) Cylindrical joint | CO1 | An | 8 |
|  | b. | For the kinematic linkages shown in the figure, calculate the degree of freedom. | CO1 | A | 4 |
| 18. |  | In a four bar chain ABCD, AD is fixed and is 15 cm long. The crank AB is 4 cm long and rotates at 120 r.p.m. clockwise, while the link CD = 8 cm oscillates about D. BC and AD are of equal length. Find the linear and angular velocities of links CD and BC when angle BAD = 60°. | CO1 | E | 12 |
| 19. |  | Draw the profile of a cam operating a knife-edged follower and with the following data: Minimum radius of cam= 25 mm, lift = 30mm. The cam lifts the follower for 120° with simple harmonic motion (SHM) followed by a dwell period of 30°. Then the follower lowers down during 150° of the cam rotation with uniform acceleration and deceleration followed by dwell period. If the cam rotates at a uniform speed of 180 rpm in the clockwise direction, calculate the maximum velocity and acceleration during the ascent and descent of the follower. | CO2 | E | 12 |
| 20. | a. | State and explain dynamic D’Alembert’s principle. | CO2 | R | 4 |
|  | b. | A pinion having 40 teeth drives a gear having 120 teeth. The profile of the gears is involute with 20° pressure angle, 12 mm module and 10 mm addendum. Find the length of path of contact, arc of contact and the contact ratio. | CO3 | E | 8 |
| 21. |  | The turning moment diagram for a petrol engine is drawn to the following scales : Turning moment, 1 mm = 5 N-m ; crank angle, 1 mm = 1°. The turning moment diagram repeats itself at every half revolution of the engine and the areas above and below the mean turning moment line taken in order are 295, 685, 40, 340, 960, 270 mm2. The rotating parts are equivalent to a mass of 36 kg at a radius of gyration of 150 mm. Determine the coefficient of fluctuation of speed when the engine runs at 1800 r.p.m. | CO3 | E | 12 |
| 22. |  | Four masses m1, m2, m3 and m4 are 200 kg, 300 kg, 240 kg and 260 kg respectively. The corresponding radii of rotation are 0.2 m, 0.15 m, 0.25 m and 0.3 m respectively and the angles between successive masses are 45°, 75° and 135°. Find the position and magnitude of the balance mass required, if its radius of rotation is 0.2 m. | CO4 | E | 12 |
| 23. | a. | Discuss briefly with neat sketches the longitudinal, transverse and torsional free vibrations. | CO5 | U | 4 |
|  | b. | A cantilever shaft 50 mm diameter and 300 mm long has a disc of mass 100 kg at its free end. The Young's modulus for the shaft material is 200 GN/m2. Determine the frequency of longitudinal and transverse vibrations of the shaft. | CO5 | E | 8 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Discuss the application of gyroscopic principles to aircrafts. | CO6 | U | 4 |
|  | b. | A Porter governor has equal arms each 250 mm long and pivoted on the axis of rotation. Each ball has a mass of 5 kg and the mass of the central load on the sleeve is 25 kg. The radius of rotation of the ball is 150 mm when the governor begins to lift and 200 mm when the governor is at maximum speed. Find the minimum and maximum speeds and range of speed of the governor. | CO6 | E | 8 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CO1 | Determine mobility, position, velocity and acceleration of links in mechanism. |  |  |  |  |  |  |  |
| CO2 | Design cam profiles to meet the motion requirements in mechanisms. |  |  |  |  |  |  |  |
| CO3 | Determination of forces on parts of slider-crank mechanism and design of flywheel. |  |  |  |  |  |  |  |
| CO4 | Predict balancing mass requirement in rotary and reciprocating unbalanced systems. |  |  |  |  |  |  |  |
| CO5 | Determine frequency of translational and longitudinal vibration. |  |  |  |  |  |  |  |
| CO6 | Apply the use of governors to control speed and gyroscopes to navigate. |  |  |  |  |  |  |  |
| Assessment Pattern as per Bloom’s Level |  |  |  |  |  |  |  |  |  |
| CO / P |  | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |  |
| CO1 |  | 1 | 1 | 4 | 11 | 12 |  | 29 |  |
| CO2 |  | 9 |  |  |  | 12 |  | 21 |  |
| CO3 |  | 1 | 4 |  |  | 20 |  | 25 |  |
| CO4 |  | 1 | 4 |  |  | 12 |  | 17 |  |
| CO5 |  | 3 | 5 |  |  | 8 |  | 16 |  |
| CO6 |  |  | 8 |  |  | 8 |  | 16 |  |
|  |  |  |  |  |  |  |  | 124 |  |

SUPPLEMENTARY EXAMINATION – JUNE 2023

| # Course Code | # 18ME2015 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # KINEMATICS AND THEORY OF MACHINES | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Differentiate lower pair and higher pair. | CO1 / U | 1 |
| 2. | Define: Mechanism. | CO1 / R | 1 |
| 3. | State any one practical application of cams. | CO2 / R | 1 |
| 4. | Name the type of cam follower extensively used in air-craft engines. | CO2 / R | 1 |
| 5. | Define coefficient of fluctuation of speed. | CO3 / R | 1 |
| 6. | State the function of a flywheel? | CO3 / R | 1 |
| 7. | State the conditions for a body to be in equilibrium under the action of two forces. | CO4 / R | 1 |
| 8. | List the effects of partial balancing of locomotives? | CO4 / R | 1 |
| 9. | Classify the different types of vibrations? | CO5 / A | 1 |
| 10. | Define ‘hunting of governor’. | CO6 / R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Differentiate between a Machine and Structure. | CO1 / U | 3 |
| 12. | Classify types of followers. | CO2 / A | 3 |
| 13. | Differentiate between flywheel and governor. | CO3 / U | 3 |
| 14. | Differentiate: static and dynamic balancing. | CO4 / U | 3 |
| 15. | Difference between harmonic motion and periodic motion. | CO5 / U | 3 |
| 16. | Differentiate centrifugal governor and inertia governor. | CO6 / U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | Sketch and explain various inversions of four bar mechanism with neat diagrams. | CO1 / U | 12 |
| 18. | a. | The crank and connecting rod of a theoretical steam engine are 0.5 m and 2 m long respectively. The crank makes 180 rpm. in the clockwise direction. When it has turned 45° from the inner dead centre position, estimate:1. velocity of piston, 2. angular velocity of connecting rod. | CO1 / E | 12 |

| 19. | a. | A cam is to give the following motion to a knife-edged follower : 1. Outstroke during 90° of cam rotation; 2. Dwell for the next 30° of cam rotation ; 3. Return stroke during next 60° of cam rotation, and 4. Dwell for the remaining 180° of cam rotation. The stroke of the follower is 40 mm and the minimum radius of the cam is 50 mm. The follower moves with uniform velocity during both the outstroke and return strokes. Draw the profile of the cam when the axis of the follower passes through the axis of the cam shaft. | CO2/A | 12 |
| --- | --- | --- | --- | --- |
| 20. | a. | An epicyclic gear consists of three gears A, B and C as shown in Figure. The gear A has 72 internal teeth and gear C has 32 external teeth. The gear B meshes with both A and C and is carried on an arm EF which rotates about the centre of A at 18 r.p.m.. If the gear A is fixed, estimate the speed of gears B and C. | CO2/E | 12 |
| 21. | a. | The turning moment diagram for a multi-cylinder engine has been drawn to a scale 1 mm = 600 N-m vertically and 1 mm = 3° horizontally. The intercepted areas between the output torque curve and the mean resistance line, taken in order from one end, are as follows: + 52, – 124, + 92, – 140,+ 85, – 72 and + 107 mm2, when the engine is running at a speed of 600 rpm. If the total fluctuation of speed is not to exceed ± 1.5% of the mean, evaluate the necessary mass of the flywheel of radius 0.5 m. | CO3/E | 12 |
| 22. | a. | A single cylinder reciprocating engine has speed 240 r.p.m., stroke 300mm, mass of reciprocating parts 50 kg, mass of revolving parts at 150 mm radius 37 kg. If twothirdof the reciprocating parts and all the revolving parts are to be balanced, estimate : 1. The balancemass required at a radius of 400 mm, and 2. The residual unbalanced force when the crank hasrotated 60° from top dead centre. | CO4/E | 12 |
| 23. | a. | A shaft 50 mm diameter and 3 metres long is simply supported at the endsand carries three loads of 1000 N, 1500 N and 750 N at 1 m, 2 m and 2.5 m from the left support.The Young's modulus for shaft material is 200 GN/m2. Estimate the frequency of transverse vibration. | CO5/E | 12 |
|  |  | Compulsory: |  |  |
| 24. | a. | An aeroplane makes a complete half circle of 50 metres radius, towards left, when flying at 200 km per hour. The rotary engine and the propeller of the plane has a mass of 400 kg with a radius of gyration of 300 mm. The engine runs at 2400 r.p.m. clockwise, when viewed from the rear. Find the gyroscopic couple on the aircraft and state its effect on it. Illustrate the effect, if the aeroplane turns to its right instead of to the left? | CO6/An | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Determine mobility, position, velocity and acceleration of links in mechanism. |
| CO2 | Design cam profiles to meet the motion requirements in mechanisms. |
| CO3 | Determination of forces on parts of slider-crank mechanism and design of flywheel |
| CO4 | Predict balancing mass requirement in rotary and reciprocating unbalanced systems |
| CO5 | Determine frequency of translational and longitudinal vibration |
| CO6 | Apply the use of governors to control speed and gyroscopes to navigate. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 1 | 16 |  |  | 12 | - | 29 |
| CO2 | 2 |  | 15 | - | 12 |  | 29 |
| CO3 | 2 | 3 | - | - | 12 | - | 17 |
| CO4 | 2 | 3 | - | - | 12 |  | 17 |
| CO5 | - | 3 | 1 | - | 12 | - | 16 |
| CO6 | 1 | 3 | - | 12 | - | - | 16 |
|  |  |  |  |  |  |  | 124 |

| # Course Code | # 18ME2022 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # MANUFACTURING TECHNOLOGY | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | The process of removing unwanted material from a work piece in the form of chips is called as __________. |  | CO1 | R | 1 |
| 2. | The type of cutting tools widely used on lathes machine is __________. |  | CO1 | U | 1 |
| 3. | A notch or groove in the face of a tool parallel to the cutting edge, to avoid the formation of continuous chips is called __________. |  | CO2 | R | 1 |
| 4. | The chip space between the back of one tooth and the face of the next tooth in a plain milling cutter is named as __________. |  | CO2 | U | 1 |
| 5. | __________ is the standard point angle of a drill bit. |  | CO3 | R | 1 |
| 6. | The taper generally provided on drill spindles is known as __________. |  | CO3 | U | 1 |
| 7. | The cutting tool material composed of nonferrous cast material with cobalt, chromium and tungsten is called __________. |  | CO4 | R | 1 |
| 8. | __________ is widely used in tool steels to maintain its hardness even at red heat. |  | CO4 | U | 1 |
| 9. | The __________ wear is formed due to chemical reaction between the tool surface and atmospheric oxygen at high temperature, also forms a layer of oxides on the surface. |  | CO5 | R | 1 |
| 10. | __________ is used as binding material in cemented carbide tools. |  | CO5 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Define Back rake angle. |  | CO1 | R | 3 |
| 12. | List the reasons for tool wear. |  | CO2 | U | 3 |
| 13. | Difference between shaper and planer with sketch. |  | CO3 | R | 3 |
| 14. | Outline simple indexing. |  | CO4 | U | 3 |
| 15. | Express Wheel Balancing. |  | CO5 | R | 3 |
| 16. | Clarify on Wheel Dressing and Truing. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.No 17 to 23, Q.No 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Draw the comparison table and explain the different types of chips during metal cutting with neat sketch. | CO1 | A | 12 |
| 18. |  | Draw and explain the nomenclature of single point cutting tool with neat sketch. | CO2 | R | 12 |
| 19. |  | With mandatory sketch explain the nomenclature of milling cutter. | CO3 | U | 12 |
| 20. |  | Explain a quick return mechanism used in shaper with neat sketch. | CO4 | An | 12 |
| 21. |  | Explain with neat sketch, the working principle of radial drilling machine. | CO5 | R | 12 |
| 22. |  | Describe the Secondary finishing operations with neat sketch. | CO4 | U | 12 |
| 23. |  | Explicate the Gear Hobbing operation with neat sketch. | CO5 | U | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Define Rapid Prototyping. Explain the different types of Rapid Prototyping. | CO6 | A | 12 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Select the machining processes suitable for machining a component. |
| CO2 | Generate the process sequences for machining in machine tools to reduce the lead time. |
| CO3 | Analyze and choose the optimized machining parameters. |
| CO4 | Select cutting tools for the identified machining sequences. |
| CO5 | Appraise the abrasive machining process based on the surface finish requirements. |
| CO6 | Implement the non-conventional machining processes for machining hard materials. |

| Assessment Pattern as per Bloom’s Level |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 4 | 1 | 12 |  |  |  | 17 |
| CO2 | 13 | 4 |  |  |  |  | 17 |
| CO3 | 4 | 13 |  |  |  |  | 17 |
| CO4 | 1 | 16 | 12 |  |  |  | 29 |
| CO5 | 16 | 13 |  |  |  |  | 29 |
| CO6 |  | 3 | 12 |  |  |  | 15 |
|  |  |  |  |  |  |  | 124 |

| # Course Code | # 18ME2025 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # MATERIALS ENGINEERING | # Max. Marks : | # 100 |

| Q. No. | Questions | BL/CO | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Atomic number equals __________. | CO1 / R | 1 |
| 2. | Appraise polymorphism. | CO1/ R | 1 |
| 3. | Density of a crystal remains unchanged as a result of __________. | CO2 / R | 1 |
| 4. | Define resilience. | CO2 / U | 1 |
| 5. | The plastic deformation of metal takes place when the stress induced in the metal, due to the applied forces, reached the __________. | CO3 / R | 1 |
| 6. | List a few types of fatigue load. | CO3 / R | 1 |
| 7. | Define yield strength. | CO4 / R | 1 |
| 8. | Why are metals tested? | CO 4 / U | 1 |
| 9. | Cast iron has carbon content of __________. | CO 5 / R | 1 |
| 10. | List the types of annealing. | CO 5/ R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Describe with neat sketches the arrangement of atoms in the BCC, FCC lattices. | CO1 / U | 3 |
| 12. | Define burgers vector. | CO2 / R | 3 |
| 13. | Differentiate amorphous and crystalline structures. | CO1 / U | 3 |
| 14. | Mention the various cyclic stresses. | CO3 / R | 3 |
| 15. | Differentiate destructive and non-destructive testing. | CO2/ U | 3 |
| 16. | What is the purpose of normalizing? | CO4/ U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | a. | Classify the materials with a neat flowchart diagram. | CO1 / R | 6 |  |
|  | b. | List the mechanical properties of materials. | CO1 / R | 6 |  |
| 18. |  | Interpret point and surface defects in crystals. | CO2 / U | 12 |  |
| 19. | a. | Outline hardness. Elaborate on any two hardness testing procedures. | CO2 / R | 6 |  |
|  | b. | With neat sketch explain tensile testing and the specimen specifications for the same. | CO2/ U | 6 |  |
| 20. | a. | Compare and contrast ductile and brittle fractures. | CO3/ A | 6 |  |
|  | b. | Explain the various methods to improve the fatigue strength of the material. | CO 3 / U | 6 |  |
| 21. | a. | Explain the S –N curve with neat sketches. | CO 3 / U | 6 |  |
|  | b. | With a neat sketch explain the fatigue testing procedure. | CO 3 / U | 6 |  |
| 22. | a. | Discuss the various stages in creep with neat sketches. | CO4 / U | 6 |  |
|  | b. | Summarize viscous and transient creep. | CO4/ U | 6 |  |
| 23. |  | Explain Fe-C equilibrium diagram with neat sketches. | CO5 / An | 12 |  |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Explain the following in detail - Nitriding. - Induction hardening. - Hardenability. | CO5/ E |  | 12 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Identify crystal structures of common engineering materials. |
| CO2 | Understand the principle of various microscopes. |
| CO3 | Identify the various behaviors of materials and defects. |
| CO4 | Analyze failures and predict service behavior of materials for various applications. |
| CO5 | Interpret and determine the right compositions of metals. |
| CO6 | Select the heat treatment process based on the metals. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 17 | 3 |  |  |  |  | 20 |
| CO2 | 13 | 22 |  |  |  |  | 35 |
| CO3 | 5 | 18 | 6 |  |  |  | 29 |
| CO4 | 2 | 16 |  |  |  |  | 18 |
| CO5 | 2 | - |  | 12 | 12 |  | 26 |
| CO6 | - | - |  |  |  |  | - |
|  |  |  |  |  |  |  | 100 |

| # Course Code | # 18ME2028 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # HYDRAULICS AND PNEUMATICS | # Max. Marks | # 100 |

| Q. No. | Questions |  |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |  |
| 1. | Give some examples of Newtonian fluids. |  |  | CO1 | U | 1 |
| 2. | Define fluid power. |  |  | CO1 | R | 1 |
| 3. | Write the main elements of hydraulic system. |  |  | CO2 | R | 1 |
| 4. | Draw the graphical symbol of double acting cylinder. |  |  | CO2 | U | 1 |
| 5. | List the components of hydraulic power pack unit. |  |  | CO3 | R | 1 |
| 6. | Mention the causes of overheating in hydraulic systems. |  |  | CO3 | U | 1 |
| 7. | Give applications of pneumatic systems. |  |  | CO4 | U | 1 |
| 8. | Write the equation for general gas law. |  |  | CO4 | R | 1 |
| 9. | Write the advantages of feedback control system. |  |  | CO5 | U | 1 |
| 10. | State the function of servo valve. |  |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |  |
| 11. | Describe the comparison between pneumatic and hydraulic systems. |  |  | CO1 | An | 3 |
| 12. | Sketch the graphical symbol of pressure relief valve and 3/2 directional control valve. |  |  | CO2 | U | 3 |
| 13. | Identify the kind of failures occur in hydraulic systems. |  |  | CO3 | An | 3 |
| 14. | Draw and explain the three stages of air treatment in pneumatic system. |  |  | CO4 | U | 3 |
| 15. | Develop displacement step diagram of pneumatic circuit for drilling operation. |  |  | CO5 | A | 3 |
| 16. | Draw the symbols for pneumatic and electrical switching contact normally open and close. |  |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.No 17 to 23, Q.No 24 is Compulsory) |  |  |  |  |  |  |
| 17. |  | a. | Name the different types of hydraulic fluid properties and describe them. | CO1 | A | 8 |
|  |  | b. | The diameters of plunger and Ram of a hydraulic press (shown in Figure) are 50 mm and 125 mm respectively. Find the weight lifted by the hydraulic press when the force applied at the plunger is 750 N. | CO1 | E | 4 |
| 18. |  | a. | Mention the types of the vane pumps, with neat sketch explain construction and working principle of any type of vane pump. | CO2 | An | 8 |
|  |  | b. | Sketch and explain about the diaphragm type gas loaded accumulator. | CO2 | A | 4 |
| 19. |  | a. | Illustrate key specifications of hydraulic cylinders. | CO3 | A | 6 |
|  |  | b. | Design and describe a hydraulic circuit for shaping machine. | CO3 | E | 6 |
| 20. |  | a. | List the types of pneumatic linear and rotary actuator. | CO4 | U | 4 |
|  |  | b. | Explain the working of Filter, Regulator and Lubricator for a pneumatic system with neat sketches. | CO4 | A | 8 |
| 21. |  |  | Design a system in which cylinder A is used to clamp the workpiece; cylinder B is used for punching and cylinder C removes the workpiece from the station. | CO5 | E | 12 |
| 22. |  | a. | A hydraulic pump delivers 12 L of fluid per minute against a pressure of 200 bar. (a) Calculate the hydraulic power. (b) If the overall pump efficiency is 60%, what size of electric motor would be needed to drive the pump? | CO2 | An | 8 |
|  |  | b. | For a hydraulic motor, define volumetric efficiency, mechanical efficiency and overall efficiency. | CO2 | U | 4 |
| 23. |  | a. | Design a fail-safe circuit for hydraulic system to protect from overload. | CO3 | An | 6 |
|  |  | b. | Design and explain the regenerative circuit for hydraulic system. | CO3 | A | 6 |
| COMPULSORY QUESTION |  |  |  |  |  |  |
| 24. |  |  | Design an Electro-Pneumatic circuit to implement following the sequence: A+ B+ B- A-. | CO6 | An | 12 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the salient features and constructional details of both hydraulic and Pneumatic systems. |
| CO2 | Understand the various types of actuation modes and control system design procedures for design of circuits and to control them. |
| CO3 | Understand the concepts of servo and proportional valves. |
| CO4 | Analyze various application circuits. |
| CO5 | Apply the above outcomes to design pneumatic and hydraulic circuits. |
| CO6 | Build a PLC programme for a particular application. |

| Assessment Pattern as per Bloom’s Level |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 1 | 1 | 8 | 3 | 4 |  | 17 |
| CO2 | 1 | 8 | 4 | 16 |  |  | 29 |
| CO3 | 1 | 1 | 12 | 9 | 6 |  | 29 |
| CO4 | 1 | 8 | 8 |  |  |  | 17 |
| CO5 |  | 1 | 3 |  | 12 |  | 16 |
| CO6 | 1 | 3 |  | 12 |  |  | 16 |
|  |  |  |  |  |  |  | 124 |

SUPPLEMENTARY EXAMINATION - JUNE 2023

| # Course Code | # 18ME2034 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # OPERATIONS RESEARCH | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO |  | BL | Marks |
| --- | --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) (Answer all the questions) |  |  |  |  |  |  |
| 1. | All simplex problems should be solved in the manner of----------------- |  |  | CO 1 | U | 1 |
| 2. | Graphical method could be adopted to solve a LPP with -------- decision variables only |  |  | CO 1 | U | 1 |
| 3. | Name the three methods to arrive initial solution in transportation problem |  |  | CO 2 | U | 1 |
| 4. | All assignment problems should be solved in the manner of----------- |  |  | CO 2 | U | 1 |
| 5. | Draw network for the following activities and find the critical path Activities; 1-2; 1-3; 2-4; 3-4. Duration (Weeks); 4; 6; 5; 2. | CO 3 | A | 1 |
| 6. | Identify the use of a dummy activity in a network |  |  | CO 3 | U | 1 |
| 7. | Distinguish ‘inventory ordering cost’ and ‘inventory holding cost’ |  |  | CO 4 | U | 1 |
| 8. | Interpret the concept of Economic Ordering Quantity (EOQ) |  |  | CO 5 | U | 1 |
| 9. | Differentiate ‘inter arrival time’ and ‘arrival rate’ |  |  | CO 6 | U | 1 |
| 10. | FCFS stands for------------------------- |  |  | CO 6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) (Answer all the questions) |  |  |  |  |  |  |
| 11. | List the advantages and limitations of LPP |  | CO 1 |  | A | 3 |
| 12. | Normal duration and normal cost crash cost and crash duration of the activities 1-2 and 1-3 are given below Interpret the cost slope (Delta cost) and Delta time (Time available for crashing) of these activities Activities; Normal Cost (Rupees); Normal Duration (Weeks); Crash Cost (Rupees); Crash Duration (Weeks). 1-2; 7,000; 6; 14,500; 3. 1-3; 4,000; 8; 8,500; 5. | CO 2 |  | A | 3 |
| 13. | Interpret the method to resolve degeneracy in transportation problem |  | CO 3 |  | A | 3 |
| 14. | Summarize the applications of assignment problem |  | CO 4 |  | A | 3 |
| 15. | In a mechanical workshop, it takes 10 days to get the stock of coolant oil after placing an order. The daily requirement of coolant oil in the workshop is 50 litres. Based on the past experience it is determined that the safety is 5 days stock. Infer the re-order point |  | CO 5 |  | A | 3 |
| 16. | List the properties of Transient and Steady state of Queuing system |  | CO 6 |  | A | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |  |
| 17. | a. | Solve the following LPP by using Graphical Method Maximize Z = 3X1+2X2 Subjected to X1-X2 ≥ 1 X1+X2 ≥ 3 With non-negative restrictions X1, X2 ≥ 0 | CO 1 |  | A |  |
| 18. | a. | There are three factories located at places P,Q and R. .These factories supply products to whole sale agents located at places S, T and W. The weekly capacities of factories P,Q and R are 76,82 and 72 units respectively. Weekly requirements of agents S, T and W are 72,102 and 41 units respectively. the unit transportation cost in rupees from P to S, T and W are5,8 and 8 respectively, from Q to S,T and W are16,25 and 15 respectively and R to S,T and W are 9,16 and 25 respectively. Use least cost method to arrive initial solution. | CO 1 |  | An |  |
| 19. | a. | There are 5 Jobs and 5 machines; the associated cost (In Rupees) of allocating a job to the machines is given in the table .Find the optimal allocation and optimal allocation (assignment cost) Machines 11; 17; 8; 16; 20. 9; 7; 12; 6; 15. 13; 16; 15; 12; 16. 21; 24; 17; 28; 26. 14; 10; 12; 11; 15. Jobs | CO 2 |  | An |  |
| 20. | a. | The various time estimates of activities involved in a project are given below Activities; 1-2; 1-3; 2-4; 2-3; 3-4; 3-5; 4-6; 5-6. Optimistic Time (Days); 2; 4; 2; 2; 0; 3; 6; 1. Normal Time (Days); 6; 8; 3; 4; 0; 6; 10; 3. Pessimistic Time (Days); 10; 12; 4; 6; 0; 9; 14; 5. Draw the network and determine the expected completion time of the project. Find the variance and SD of project. Also, determine the probability of completing the project within 23 days and 25 days. What is the probability that the project will be completed at least 5 days earlier than the expected time? | CO 3 |  | An |  |
| 21. | a. | A machine shop has one shearing, one punching and one de-burring machine. Time in minutes for shearing, punching and de-burring operations is given for each job. Determine the optimal order (sequence) in which the jobs are to be processed to minimize the total time. Find the total time to process all the jobs and idle time of each machine Job; Shearing Time (Minutes); Punching Time (Minutes); De-burring Time (Minutes). 1; 40; 50; 80. 2; 20; 60; 100. 3; 80; 20; 60. 4; 50; 30; 70. 5; 60; 40; 110. | CO 4 |  | E |  |
| 22. | a. | Solve the following Linear Programming problem by simplex method Maximize Z=X 1 –X2+3X3 Subjected to 2X1+X2+X3≤ 10 2X1- X3≤ 2 - 2X1+2X2-3X3 ≤ 0 X1, X2 and X3 ≥ 0 | CO 2 |  | E |  |
| 23. | a. | Draw a graph and conduct ABC analysis for the following 10 items in an inventory Items; 1; 2; 3; 4; 5. Annual Usage(units); 200; 3000; 25; 1100; 60. Unit Cost(Rs); 11; 14; 9; 6; 5. Items; 6; 7; 8; 9; 10. Annual Usage(units); 250; 140; 850; 550; 80. Unit Cost(Rs); 90; 6; 6; 15; 9. | CO 6 |  | An |  |
| COMPULSORY QUESTION |  |  |  |  |  |  |
| 24. | a. | Arrivals at a public telephone booth with an average of 8 minutes between one arrival and the next. The length of the telephone calls is assumed with a mean value of 2 minutes, find Utilization factor Average queue length Average waiting time of customer | CO 5 |  | E |  |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Correlate this subject knowledge with the engineering problems. |
| CO2 | Construct flexible appropriate mathematical model to represent physical problem. |
| CO3 | Schedule their engineering projects by using network analysis. |
| CO4 | Analyze the transportation problem and optimize the resources and output. |
| CO5 | Apply knowledge in solving their engineering queuing problems. |
| CO6 | Develop their skills in decision making analysis by allocation of resources. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | R | U | A | An | E | C | Total |
| CO1 | - | 2 | 15 | 12 | - | - | 29 |
| CO2 | - | 2 | 3 | 12 | 12 | - | 29 |
| CO3 | - | 1 | 4 | 12 | - | - | 17 |
| CO4 | - | 1 | 3 | - | 12 | - | 16 |
| CO5 | - | 1 | 3 | - | 12 | - | 16 |
| CO6 | - | 2 | 3 | 12 |  | - | 17 |
|  |  |  |  |  |  |  | 124 |

| # Course Code | # 18ME2034 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # OPERATIONS RESEARCH | # Max. Marks | # 100 |

Use of standard normal distribution table is permitted

| Q. No. | Questions |  |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |  |
| 1. | One of the main advantage of Operations Research is the use of models that simulate the exact real-world problems. True or False |  |  | CO1 | R | 1 |
| 2. | Define surplus variable. |  |  | CO1 | R | 1 |
| 3. | In the northwest corner method, the first allocation is made at ______________ corner of the matrix. |  |  | CO2 | R | 1 |
| 4. | In the MODI method of optimization, the penalty in each cell is calculated using the equation Pij = Ui + Vj+ Cij. True or False |  |  | CO2 | R | 1 |
| 5. | The minimum cost estimated by the following allocation is Rs. ____ . |  |  | CO3 | U | 1 |
| 6. | PERT stands for ______________________ . |  |  | CO3 | R | 1 |
| 7. | Write the relation between optimistic, pessimistic, most probable and average time of completion of an activity. |  |  | CO3 | U | 1 |
| 8. | Identify the type of error in the network diagram given below. |  |  | CO4 | U | 1 |
| 9. | Two Bin system is also known as ______________ system. |  |  | CO5 | R | 1 |
| 10. | According to Pareto’s principle, most businesses have a _________ number of “A” items, and a ______________ number of “C” items. |  |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |  |
| 11. | List the characteristics of a primal and its dual. |  |  | CO1 | R | 3 |
| 12. | Detail how the key column and key row is chosen in a simplex problem. |  |  | CO2 | U | 3 |
| 13. | The critical path duration in the project shown below is ____________ weeks. |  |  | CO3 | U | 3 |
| 14. | Compare and contrast transportation and assignment problems. |  |  | CO4 | U | 3 |
| 15. | Compare and contrast the characteristics of a transient and steady-state queuing system. |  |  | CO5 | U | 3 |
| 16. | In a medium scale industry, a tool and cutter grinder operator finds that the time spent on each tool has an exponential distribution with mean 25 minutes. If he grinds tools in the order in which they come in and arrival of tools for grinding is approximately Poisson with an average rate of 11 per 8 hours day, what is the operators expected idle time each day ? |  |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.No 17 to 23, Q.No 24 is Compulsory) |  |  |  |  |  |  |
| 17. |  |  | Solve the following LPP using Simplex method. Maximize Z = 7X1 + 6X2 Subject to X1 + X2 ≤ 4 2X1 + X2 ≤ 6 Also, X1, X2 ≥ 0 | CO1 | Apply | 12 |
| 18. |  |  | Compare the initial basic feasible solutions obtained using North-West corner method, Least cost method and Vogel’s approximation method in the following transportation problem. A; B; C; D; E; Supply. 1; 5; 1; 8; 7; 5; 15. 2; 3; 9; 6; 7; 8; 25. 3; 4; 2; 7; 6; 5; 42. 4; 7; 11; 10; 4; 9; 35. Demand; 30; 20; 15; 10; 20. | CO1 | Apply | 12 |
| 19. |  |  | Solve the following assignment problem to maximize the sales. Territories. I; II; III; IV. A; 45; 38; 30; 22. B; 35; 29; 20; 14. C; 35; 29; 20; 14. D; 27; 20; 15; 10. | CO4 | Apply | 12 |
| 20. |  |  | The jobs involved in a small project are listed in Table. The jobs are identified by their node numbers and the duration is shown in days. Job; 1-2; 1-3; 1-4; 2-6; 3-7; 4-5; 5-6; 5-7; 5-8; 6-8; 8-9; 7-9. Duration (Days); 4; 6; 8; 8; 12; 10; 0; 0; 4; 9; 5; 8. - Draw a network diagram and compute the early start and late start for each activity. - Determine the total slack and free slack of each activity. | CO3 | Apply | 12 |
| 21. |  |  | The jobs of a project with the respective time estimates are given in the table. Jobs; 1-2; 1-6; 2-3; 2-4; 3-5; 4-5; 6-7; 5-8; 7-8. to; 3; 2; 6; 2; 5; 3; 3; 1; 4. tm; 6; 5; 12; 5; 11; 6; 9; 4; 19. tp; 15; 14; 30; 8; 17; 15; 27; 7; 28. - Draw the network and calculate the following : Variance of each job Length of the project Variance of each job b) What is the probability that the jobs on the critical path will be completed by the due date of 42 days ? | CO3 | Apply | 12 |
| 22. |  |  | The demand for a computer monitor cable is 1050 cables per month and shortages are allowed. If the cost per cable is Rs. 125, cost of making one purchase is Rs. 700, the holding cost of one cable is Rs. 3 per year and the cost of one shortage is Rs. 50 per year, determine the following : - Optimum purchase quantity - Optimum number of shortages - Optimum total yearly cost - Number of orders per year - Time between order - Maximum inventory | CO5 | Apply | 12 |
| 23. |  |  | Customers arriving at an industrial consultants office is according to Poisson’s distribution at the rate of 28 per hour. The waiting room can accommodate not more than 14 customers. Consultation time per customer is exponential with a mean rate of 20 per hour. Find the effective arrival rate at the consultants office. What is the expected waiting time of a customer in the consultant’s office ? | CO6 | Apply | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |  |
| 24. |  |  | A machine shop has one shearing, one punching and one deburring machine. Time in minutes for shearing, punching and deburring operations are given for each job. Determine the order in which the jobs are to be processed in order to minimize the total time required to process all the jobs. Find the total time required to process all the jobs and the idle time of each machine. Jobs; Machine. Shearing; Punching; Deburring. 1; 40; 50; 80. 2; 20; 60; 100. 3; 80; 20; 60. 4; 50; 30; 70. 5; 60; 40; 110. | CO2 | Apply | 12 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Correlate this subject knowledge with the engineering problems. |
| CO2 | Construct flexible approximate mathematical models to represent physical Problems. |
| CO3 | Schedule their engineering projects by using network analysis. |
| CO4 | Analyze the transportation problem and optimize resources and output. |
| CO5 | Apply knowledge in solving their engineering queuing problems. |
| CO6 | Develop their skills in decision making analysis by allocating resources. |

| Assessment Pattern as per Bloom’s Level |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 5 | - | 24 | - | - | - | 29 |
| CO2 | 2 | 3 | 12 | - | - | - | 17 |
| CO3 | 1 | 5 | 24 | - | - | - | 30 |
| CO4 | - | 4 | 12 | - | - | - | 16 |
| CO5 | 1 | 3 | 12 | - | - | - | 16 |
| CO6 | 1 | 3 | 12 | - | - | - | 16 |
|  |  |  |  |  |  |  | 124 |

| # Course Code | # 18ME2039 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # FLUID MECHANICS AND FLUID MACHINES | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | The fluid in which shear stress is proportional to rate of shear strain is called _________. |  | CO1 | U | 1 |
| 2. | Write the different types of forces acting on a fluid particle. |  | CO1 | R | 1 |
| 3. | Define ‘center of pressure’. |  | CO2 | U | 1 |
| 4. | Write the condition to choose manometric fluid in the manometer. |  | CO2 | U | 1 |
| 5. | The stream lines and equipotential lines are _________ to each other. |  | CO3 | R | 1 |
| 6. | Define Reynold’s number. |  | CO3 | R | 1 |
| 7. | Write the different types of forces acting on a fluid particle. |  | CO4 | R | 1 |
| 8. | Write the formula for measuring the velocity of flow using pitot tube. |  | CO4 | U | 1 |
| 9. | Write Buckingham theorem. |  | CO5 | U | 1 |
| 10. | Define ‘no-slip condition’. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | What happens when you add detergents to water during washing? |  | CO1 | U | 3 |
| 12. | Define ‘Buoyancy’. |  | CO2 | U | 3 |
| 13. | Differentiate Steady and uniform flow. |  | CO3 | R | 3 |
| 14. | Write the principle of ‘orifice meter’ in measuring the flow rate. |  | CO4 | U | 3 |
| 15. | Define i) Froude number ii) Mach number iii) Weber number |  | CO5 | R | 3 |
| 16. | Explain the different regions of boundary layer in a pipe flow. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.No 17 to 23, Q.No 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Two large plane surface are 2.4 cm apart. The space between the surfaces is filled with oil. What force is required to drag a very thin plate of surface area 0.5 sq.meter between the two large plane surfaces at a speed of 0.6 m/s. If, - The thin plate is in the middle of the two plane surfaces - The thin plate is at a distance of 0.8 cm from one of the plane surfaces? Take the dynamic viscosity of oil=8.10×10-1Ns/m2 | CO1 | An | 12 |
| 18. |  | A differential manometer is connected at the two points A and B of two pipes as shown in figure 2. The pipe A contains a liquid of specific gravity=1.5 while pipe B contains a liquid of specific gravity =0.9. The pressures at A and B are 9.81N/cm2and 17.65 N/cm2 respectively. Find the difference in mercury level in the differential manometer. | CO2 | An | 12 |
| 19. |  | Derive the three-dimensional mass conservation equation for a compressible fluid. | CO3 | A | 12 |
| 20. |  | Derive the Euler’s equation of motion and deduce the expression to Benoulli’s equation. | CO4 | A | 12 |
| 21. |  | Derive the equation of motion for vortex flow. | CO5 | A | 12 |
| 22. |  | A pipe 450 mm diameter branches into two pipes of diameter are 300 mm and 200 mm respectively. If the average velocity in 450 mm diameter pipe is 3 m/s. Find , - Discharge through 450 mm diameter pipe. ii. Velocity in 200 mm diameter pipe if the average velocity in 300 mm pipe is 2.5 m/s. | CO3 | An | 12 |
| 23. |  | A 30 cm x 15 cm venturimeter is provided in a vertical pipe line carrying oil of specific gravity 0.9, the flow being upwards. The difference in elevation of the throat section and entrance section of the venturimeter is 30 cm. The differential U tube mercury manometer shows a gauge deflection of 25 cm. Calculate, - the discharge of oil. - the pressure difference between the entrance section and the throat section. Take Cd=0.98 and specific gravity of mercury as 13.6. | CO4 | An | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Explain different regions of boundary layer over a flat plate using a neat sketch. | CO6 | A | 12 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Apply principles of fluid statics, kinematics and dynamics. |
| CO2 | Describe the terminology in fluid mechanics. |
| CO3 | Contrast the different types of fluid flow. |
| CO4 | Apply the continuity, momentum and energy principles. |
| CO5 | Apply dimensional analysis. |
| CO6 | Examine the characteristics of a boundary layer. |

| Assessment Pattern as per Bloom’s Level |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 1 | 4 | - | 12 | - | - | 17 |
| CO2 | - | 5 |  | 12 | - | - | 17 |
| CO3 | 5 | - | 12 | 12 | - | - | 29 |
| CO4 | 1 | 4 | 12 | 12 | - | - | 29 |
| CO5 | 3 | 1 | 12 | - | - | - | 16 |
| CO6 | 1 | 3 | 12 | - | - | - | 16 |
|  |  |  |  |  |  |  | 124 |

| # Course Code | # 18ME2040 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # COMPUTATIONAL FLUID DYNAMICS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | What types of energy is considered as source term in energy equation? |  | CO1 | U | 1 |
| 2. | Write the principle of conservation of mass. |  | CO1 | R | 1 |
| 3. | Define divergence and write the significance. |  | CO2 | U | 1 |
| 4. | What is the significance of Taylor series? |  | CO2 | U | 1 |
| 5. | Write the general discretized form of equation at node in 1 D diffusion problem. |  | CO3 | R | 1 |
| 6. | What is the finite volume method of approximation of source term? |  | CO3 | R | 1 |
| 7. | Define diffusion conductance. |  | CO4 | R | 1 |
| 8. | Differentiate between convection and diffusion process. |  | CO4 | U | 1 |
| 9. | The convection of scalar variable depends on magnitude and direction of ----------------. |  | CO5 | U | 1 |
| 10. | What are characteristics of free turbulent flows? |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Write the integral form of general transport equation. |  | CO1 | An | 3 |
| 12. | Write the importance of properties of discretization schemes. |  | CO2 | U | 3 |
| 13. | Write the differential form of 3D steady diffusion. |  | CO3 | R | 3 |
| 14. | Write the general equation which represents unsteady heat conduction. |  | CO4 | U | 3 |
| 15. | Write the expression for pressure gradient for u-cell control volume in staggered grid. |  | CO5 | An | 3 |
| 16. | Explain the different regions of boundary layer on a flat plate. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.No 17 to 23, Q.No 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Derive the momentum equation for 3D flows. | CO1 | An | 12 |
| 18. |  | Explain the advantages and applications of different types of grids used in CFD. Also Write the importance of grid independence study. | CO2 | A | 12 |
| 19. |  | Derive the discretized form of equation for 2D steady diffusion problem. | CO3 | An | 12 |
| 20. |  | A property is transported by means of convection and diffusion through a one-dimensional domain. Consider the case to be steady one-dimensional convection diffusion, with boundary conditions as 0 = 1at x = 0 and L = 0 at x = L. Using five equally spaced cells and the central difference scheme for convection and diffusion, calculate the distribution of as a function of x. Apply the following data: u=0.1 m/s, L= 1 m, = 0.1 kg/m/s, = 1 kg/m3 | CO4 | An | 12 |
| 21. |  | Describe the method of solving discretization equations using SIMPLE algorithm. | CO5 | An | 12 |
| 22. |  | Derive the Energy equation from the fundamentals of thermodynamics? | CO1 | A | 12 |
| 23. |  | Derive the discretized equation for convection diffusion using central difference scheme. | CO4 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | With neat sketch explain the transition processes in i) boundary layer flow over a flat plate ii) jet flow | CO6 | U | 12 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Formulate the required governing equations for flow and heat transfer problems. |
| CO2 | Discretize the governing equations of flow and heat transfer problems. |
| CO3 | Solve the diffusion equations. |
| CO4 | Solve the diffusion-convection equations. |
| CO5 | Use appropriate algorithms to solve the discretized equations. |
| CO6 | Apply turbulence models to accurately predict the variables based on the flow characteristics. |

| Assessment Pattern as per Bloom’s Level |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 1 | 1 | 12 | 15 |  |  | 29 |
| CO2 |  | 5 | 12 |  |  |  | 17 |
| CO3 | 5 |  |  | 12 |  |  | 17 |
| CO4 | 1 | 4 | 12 | 12 |  |  | 29 |
| CO5 | 1 |  |  | 15 |  |  | 16 |
| CO6 | 1 | 15 |  |  |  |  | 16 |
|  |  |  |  |  |  |  | 124 |

| # Course Code | # 18ME2044 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # REFRIGERATION AND AIR-CONDITIONING | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) (Answer all the questions) |  |  |  |  |  |
| 1. | Write the expression of first law of thermodynamics for a change of state. |  | CO1 | R | 1 |
| 2. | Why the C.O.P of the refrigerator is improved by decreasing the temperature of hot body? |  | CO1 | U | 1 |
| 3. | Define ‘critical temperature of refrigerant’. |  | CO2 | R | 1 |
| 4. | Write any two factors affecting the condenser capacity. |  | CO2 | U | 1 |
| 5. | Define the term ‘Cryogenic’. |  | CO3 | R | 1 |
| 6. | Write the triple point temperature of carbon dioxide. |  | CO3 | R | 1 |
| 7. | Define ‘Dew point temperature’. |  | CO4 | R | 1 |
| 8. | Write any two factors governing the optimum effective temperature. |  | CO4 | U | 1 |
| 9. | Define ‘Sol Air Temperature’. |  | CO5 | R | 1 |
| 10. | Write the different methods used for food freezing. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) (Answer all the questions) |  |  |  |  |  |
| 11. | Distinguish between ‘heat engine’, ‘Refrigerator’, and ‘heat pump’. |  | CO1 | U | 3 |
| 12. | Write the classification of compressors according to the location of prime mover. |  | CO2 | R | 3 |
| 13. | Write the limitations of vapour compression refrigeration systems for production of low temperature. |  | CO3 | U | 3 |
| 14. | Define the terms i) Room sensible heat factor, ii) Grand sensible heat factor. |  | CO4 | R | 3 |
| 15. | Write the two main components of cooling load imposed on air conditioning plant. |  | CO5 | U | 3 |
| 16. | Draw the schematic diagram representing the milk chilling process. |  | CO6 | R | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Explain Air Refrigerator working on a Bell- Coleman Cycle and derive the expression for C.O.P. | CO1 | A | 12 |
| 18. |  | Derive the expression for C.O.P of an ideal vapour absorption system and write the advantages of vapour absorption systems over vapour compression refrigeration system. | CO2 | A | 12 |
| 19. |  | Write the processes to obtain liquid hydrogen and liquid helium with a neat sketch. | CO3 | A | 12 |
| 20. |  | For a sample of air having 22 C DBT, relative humidity 30% at barometric pressure of 760 mm of Hg. Calculate i) Vapour pressure, ii) Humidity ratio, iii) Vapour density and iv) Enthalpy. Verify the results by psychrometric chart. | CO4 | An | 12 |
| 21. |  | A conference room for seating 100 persons is to be maintained at 22 C DBT and 60% RH. The outdoor conditions are 40 C DBT and 27C WBT. The various loads in the auditorium are as follows. Sensible and Latent heat loads per person, 80W and 50 W respectively; Lights and fans 15000 W; sensible heat gain through walls and ceiling 15000W; the air infiltration is 20 m3/min. and fresh air supply is 100 m3/min. Two third of recirculated room air and one third of fresh air are mixed before entering the cooling coils. The bypass factor of the coil is 0.1. Determine the apparatus dew point, the grand total heat load and effective room sensible heat factor. | CO5 | An | 12 |
| 22. |  | A simple air-cooled system is used for an aero plane having a load of 10 tones. The atmospheric pressure and temperature are 0.9 bar and 10 C respectively. The pressure increases to 1.013 bar due to ramming. The temperature of air is reduced by 50 C in the heat exchanger. The pressure in the cabin is 1.01 bar and the temperature of air leaving the cabin is 25 C. Determine i) Power required to take the load of cooling in the cabin, and ii) C.O.P of the system. Assume that all the compressions and expansions are isentropic. The pressure of the compressed air is 3.5 bar. | CO1 | An | 12 |
| 23. |  | A single stage reciprocating compressor is required to compress 1.5 m3/min. of vapour refrigerant from 1 bar to 8 bar. Find the power required to drive the compressor, if the compression of the refrigerant is i) isothermal, ii) polytropic with polytropic index as 1.12 and, iii) isentropic with isentropic index as 1.31. | CO2 | An | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Explain the refrigeration systems in i) Ice plant ii) Train. | CO6 | A | 12 |

CO-COURSE OUTCOME					BL – BLOOMS’ LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand various refrigeration systems. |
| CO2 | Demonstrate the working of refrigeration equipment. |
| CO3 | Understand the various psychrometric processes. |
| CO4 | Estimate the space cooling load. |
| CO5 | Design the air-conditioning equipments. |
| CO6 | Select suitable refrigeration and air-conditioning systems for various applications. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | R | U | A | An | E | C | Total |
| CO1 | 1 | 4 | 12 | 12 | - | - | 29 |
| CO2 | 4 | 1 | 12 | 12 | - | - | 29 |
| CO3 | 2 | 3 | 12 | - | - | - | 17 |
| CO4 | 4 | 1 | - | 12 | - | - | 17 |
| CO5 | 1 | 3 | - | 12 | - | - | 16 |
| CO6 | 4 | - | 12 | - | - | - | 16 |
|  |  |  |  |  |  |  | 124 |

SUPPLEMENTARY EXAMINATION - JUNE 2023

| # Course Code | # 18ME2044 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # REFRIGERATION AND AIR-CONDITIONING | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) (Answer all the questions) |  |  |  |  |  |
| 1. | Write the expression of first law of thermodynamics for a cycle. |  | CO1 | R | 1 |
| 2. | Draw the basic sketch of vapour compression refrigeration cycle. |  | CO1 | U | 1 |
| 3. | Write any two advantages of rotary compressors. |  | CO2 | R | 1 |
| 4. | Write any two factors considered in the selection of refrigerant. |  | CO2 | U | 1 |
| 5. | Define the term ‘Cryogenic’. |  | CO3 | R | 1 |
| 6. | Write the triple point temperature of water. |  | CO3 | R | 1 |
| 7. | Define ‘Wet bulb temperature’. |  | CO4 | R | 1 |
| 8. | Define the term’ bypass factor’. |  | CO4 | R | 1 |
| 9. | Define ‘Infiltration’. |  | CO5 | R | 1 |
| 10. | Write the advantages of central air conditioning systems.. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) (Answer all the questions) |  |  |  |  |  |
| 11. | Write the advantages of Bell- Coleman cycle. |  | CO1 | U | 3 |
| 12. | Write the classification of condensers according to the direction of flow. |  | CO2 | R | 3 |
| 13. | Draw the schematic diagram representing the liquefaction of helium |  | CO3 | U | 3 |
| 14. | Explain ‘Heating and Humidification’. |  | CO4 | R | 3 |
| 15. | Write the factors considered to calculate the cooling load. |  | CO5 | U | 3 |
| 16. | Draw the schematic diagram representing the ice plant. |  | CO6 | R | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Explain Aircraft Refrigeration cycle and derive the expression for C.O.P. | CO1 | A | 12 |
| 18. | a. | Explain ammonia water vapour absorption system with a neat sketch. | CO2 | A | 12 |
| 19. | a. | Write the advantages of cascade refrigeration systems and explain the process for liquefaction of air. | CO3 | A | 12 |
| 20. | a. | Air having 22 C DBT, relative humidity 30%. Using the Psychrometric chart calculate i) Wet bulb temperature, ii) Humidity ratio, iii) specific volume iv) Enthalpy and v) Dew point temperature. | CO4 | An | 12 |
| 21. | a. | Explain Summer and Winter Air conditioning systems with a neat sketch. | CO5 | A | 12 |
| 22. | a. | Atmospheric air at pressure 1 bar and temperature -5 C is drawn in the cylinder of the compressor of a Bell- Coleman refrigerating machine. It is compressed isentropically to a pressure of 5 bar. In the cooler the compressed air is cooled to 15 C, pressure remaining the same. It is then expanded to a pressure of 1 bar in an expansion cylinder, from where it is passed to the cold chamber. Find the work done per kg of air and the COP of the plant. Consider Pv1.2= C for expansion and Pv1.4= C for compression and specific heat of air at constant pressure is 1 kJ/kg K. | CO1 | An | 12 |
| 23. | a. | Derive the expression for volumetric efficiency of reciprocating compressors and discuss the effect of pressure drop on volumetric efficiency. | CO2 | An | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Explain the refrigeration systems in i) Milk chilling plants ii) car. | CO6 | A | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand various refrigeration systems |
| CO2 | Demonstrate the working of refrigeration equipment |
| CO3 | Understand the various psychrometric processes |
| CO4 | Estimate the space cooling load |
| CO5 | Design the air-conditioning equipment |
| CO6 | Select suitable refrigeration and air-conditioning systems for various applications |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | R | U | A | An | E | C | Total |
| CO1 | 1 | 4 | 12 | 12 | - | - | 29 |
| CO2 | 4 | 1 | 12 | 12 | - | - | 29 |
| CO3 | 2 | 3 | 12 | - | - | - | 17 |
| CO4 | 5 | - | - | 12 | - | - | 17 |
| CO5 | 1 | 3 | 12 | - | - | - | 16 |
| CO6 | 3 | 1 | 12 | - | - | - | 16 |
|  |  |  |  |  |  |  | 124 |

| # Course Code | # 18ME2055 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # COMPUTER AIDED DESIGN | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) (Answer all the questions) |  |  |  |  |  |
| 1. | CGI stands for__________. |  | CO1 | R | 1 |
| 2. | IGES is a widely used file format for __________. |  | CO1 | R | 1 |
| 3. | On raster system, lines are plotted with __________. |  | CO2 | R | 1 |
| 4. | Expansion of line DDA algorithm is __________. |  | CO3 | R | 1 |
| 5. | List two reasons for implementing CAD systems in the industry. |  | CO3 | R | 1 |
| 6. | _________ provide designers with greater flexibility and control of a curve shape by changing the position of the control points. |  | CO3 | R | 1 |
| 7. | DVST stands for __________. |  | CO4 | R | 1 |
| 8. | Raster display maximum pixel resolution is __________. |  | CO4 | R | 1 |
| 9. | Natural coordinate system lies between________ to____________. |  | CO5 | R | 1 |
| 10. | A tetrahedron element has __________nodes. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) (Answer all the questions) |  |  |  |  |  |
| 11. | Draw the architecture of a CAD system. |  | CO1 | U | 3 |
| 12. | Write short notes on Translation, Rotation, and Scaling. |  | CO2 | U | 3 |
| 13. | Write a brief note on the parametric representation of a Bezier curve. |  | CO3 | U | 3 |
| 14. | Discuss the advantages of the random scanning technique over the raster scanning technique. |  | CO4 | U | 3 |
| 15. | Describe the significance of discretization in FEA. |  | CO5 | U | 3 |
| 16. | Discuss briefly the Johnson method of optimization. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Explain the needs of the CAD system and write merits and demerits. | CO1 | R | 4 |
|  | b. | Discuss various CAD standards with suitable examples. | CO1 | U | 8 |
| 18. | a. | Discuss in detail the Hidden line removal algorithm. | CO2 | R | 6 |
|  | b. | Elaborate on the hidden surface removal algorithm with a neat sketch. | CO2 | U | 6 |
| 19. | a. | Distinguish between Wireframe, Solid and Surface Modeling with their advantages, limitations and applications. | CO3 | A | 8 |
|  | b. | Write briefly about the CSG and B-Rep. | CO3 | A | 4 |
| 20. |  | With neat sketches explain the various graphics display units in detail. | CO4 | A | 12 |
| 21. | a. | Explain the steps involved in solving FEM. | CO5 | A | 4 |
|  | b. | Calculate the nodal displacements at points 1 and 2 with an axial load of P=200kN applied at node 1. | CO5 | An | 8 |
| 22. |  | Explain in detail the Bezier curve with suitable sketches and examples. | CO3 | A | 12 |
| 23. | a. | Derive the shape function for the global coordinate system. | CO6 | An | 8 |
|  | b. | Explain the significance of applying boundary conditions in FEA. | CO6 | A | 4 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Distinguish between FDM and SLA processes. List their limitations and advantages. | CO6 | An | 8 |
|  | b. | Discuss the process of reverse engineering with a schematic diagram. | CO6 | A | 4 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Demonstrate the basic structure and components of CAD. |
| CO2 | Outline the process of representing graphical entities in a CAD environment. |
| CO3 | Construct the geometric model using different techniques to represent a product. |
| CO4 | Illustrate various techniques and devices involved in CAD hardware. |
| CO5 | Analyze the models for design solutions using FEM. |
| CO6 | Discuss the various computer-aided tools implemented in various industrial applications. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | R | U | A | An | E | C | Total |
| CO1 | 6 | 9 |  |  |  |  | 15 |
| CO2 | 8 | 7 |  |  |  |  | 15 |
| CO3 | 8 | 1 | 24 |  |  |  | 33 |
| CO4 | 2 | 7 | 12 |  |  |  | 21 |
| CO5 | 1 | 1 | 4 | 8 |  |  | 14 |
| CO6 | 1 | 1 | 8 | 16 |  |  | 26 |
|  |  |  |  |  |  |  | 124 |

SUPPLEMENTARY EXAMINATION - JUNE 2023

| # Course Code: | # 18ME2055 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Course Title: | # COMPUTER AIDED DESIGN | # Max. Marks : | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
|  |  | PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Abbreviate CAM/CAE____________. |  | CO1 | R | 1 |
| 2. | List a few graphics standards used in the exchange of CAD data. |  | CO1 | R | 1 |
| 3. | State the use of Raster scan graphics. |  | CO2 | R | 1 |
| 4. | List the two line drawing algorithms that are widely used in graphics. |  | CO3 | R | 1 |
| 5. | State the use of synthetic curves in surface modelling. |  | CO3 | R | 1 |
| 6. | Changing the position of the control points can change the shape of the curve with greater___________. |  | CO3 | R | 1 |
| 7. | List the important I/O devices for graphics design. |  | CO4 | R | 1 |
| 8. | State the importance of the prototype. |  | CO4 | R | 1 |
| 9. | List the softwares used for finite element analysis |  | CO5 | R | 1 |
| 10. | How the STL file is generated? |  | CO6 | R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Articulate the need for implementing the CAD system. | CO1 | U | 3 |
| 12. | Write short notes on the concatenation. | CO2 | U | 3 |
| 13. | Distinguish between active and passive graphics displays. | CO3 | U | 3 |
| 14. | How clipping line algorithm is useful in computer graphics? | CO4 | U | 3 |
| 15. | Sketch neatly the product life cycle. | CO5 | U | 3 |
| 16. | List out a few polymer and metal materials used in rapid prototyping. | CO6 | U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | a. | Summarize the advantages of implementing CAD tools. Write their merits and demerits. | CO1 | A | 6 |
|  | b. | Draw the detailed architecture of the CAD system and provide examples of the same in design and analysis. | CO1 | A | 6 |
| 18. | a. | Discuss in detail the z-buffer algorithm for removing hidden lines. | CO2 | A | 6 |
|  | b. | Elaborate the painter’s algorithm with relevant sketches. | CO2 | A | 6 |
| 19. | a. | Illustrate the application of solid modelling. How it is different from wireframe modelling? | CO3 | A | 6 |
|  | b. | Explain the significance of constructive solid geometry using neat sketches. | CO3 | A | 6 |
| 20. | a. | Explain in detail the B-spline curve with suitable sketches and examples. | CO3 | An | 12 |
| 21. | a. | With neat sketches explain the following graphics displays in detail. (a) CRT (b) DVST. | CO4 | An | 12 |
| 22. | a. | Explain the procedural steps involved in solving FEM problems in Ansys. | CO5 | An | 12 |
| 23. | a. | Derive the shape function for the global coordinate system. | CO5 | E | 12 |
| Compulsory: |  |  |  |  |  |
| 24. | a. | Distinguish between SLA and SLS types of 3d printing technologies with neat sketches. | CO6 | E | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Demonstrate the basic structure and components of CAD. |
| CO2 | Outline the process of representing graphical entities in a CAD environment. |
| CO3 | Construct the geometric model using different techniques to represent a product. |
| CO4 | Illustrate various techniques and devices involved in CAD hardware. |
| CO5 | Analyze the models for design solutions using FEM. |
| CO6 | Discuss the various computer-aided tools implemented in various industrial applications. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 2 | 3 | 12 |  |  |  | 17 |
| CO2 | 2 | 3 | 12 |  |  |  | 17 |
| CO3 | 2 | 3 | 12 | 12 |  |  | 29 |
| CO4 | 2 | 3 |  | 12 |  |  | 17 |
| CO5 | 1 | 3 |  | 12 | 12 |  | 28 |
| CO6 | 1 | 3 |  |  | 12 |  | 16 |
|  |  |  |  |  |  |  | 124 |

| # Course Code | # 18ME2058 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # MECHATRONICS SYSTEM | # Max. Marks | # 100 |

| Q. No. | Questions | CO | BL | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) (Answer all the questions) |  |  |  |  |
| 1. | Define automation. | CO1 | R | 1 |
| 2. | Discuss LVDT principle. | CO2 | U | 1 |
| 3. | Expand SMA and ABS. | CO1 | R | 1 |
| 4. | Define the term Mechatronics. | CO3 | R | 1 |
| 5. | Draw the simple sketch of compound type DC motor. | CO2 | U | 1 |
| 6. | Define protective relay. | CO4 | R | 1 |
| 7. | List different types of electromechanical devices. | CO4 | R | 1 |
| 8. | List various applications of PLC. | CO6 | R | 1 |
| 9. | Draw the the pin diagram of any microprocessor and name it. | CO5 | U | 1 |
| 10. | State the principle of piezoelectric effect. | CO5 | R | 1 |

| PART – B (6 X 3 = 18 MARKS) (Answer all the questions) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Explain the various elements that might be present in a control system involving a thermostatically controlled electric heater. | CO1 | A | 3 |
| 12. | Appraise the working principle of hall effect sensor. | CO2 | E | 3 |
| 13. | Illustrate the components of a basic pneumatic system. | CO3 | A | 3 |
| 14. | Discuss about Shape Memory Alloy. | CO4 | U | 3 |
| 15. | Illustrate the working of a magneto strictive transducers | CO5 | A | 3 |
| 16. | Analyse the data handling instructions in PLC programming | CO6 | An | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | a. | The mechatronics is an interdisciplinary field in which the disciplines those act together- justify. | CO1 | E | 6 |
|  | b. | Illustrate the levels of integration in Mechatronics systems. | CO1 | A | 6 |
| 18. | a. | Deduce the various elements that might be present in a control system involving a thermostatically controlled electric heater. | CO2 | An | 8 |
|  | b. | Analyze one application of mechatronics systems like washing machine. | CO2 | A | 4 |
| 19. | a. | Describe in detail the principle, application and the uses of potentiometer. | CO3 | U | 8 |
|  | b. | Discuss the working of optical encoder with neat diagram. | CO3 | U | 4 |
| 20. | a. | Assess stepper motor basics, types, uses and working principles with neat diagram. | CO6 | E | 6 |
|  | b. | Appraise the principle of operation for variable reluctance stepper motor. | CO6 | E | 6 |
| 21. | a. | Describe the concept of Piezoelectric Effect with neat drawing and discuss the working principle. | CO5 | U | 6 |
|  | b. | Discuss the working of a magneto restrictive transducer. | CO5 | U | 6 |
| 22. | a. | Examine the working of Electro-hydraulic power steering system in automobile with neat diagram. | CO4 | A | 6 |
|  | b. | Interpret some of the I/O devices used for PLC. | CO4 | A | 6 |
| 23. | a. | Estimate the principle of X-ray Lithography with neat diagram. | CO5 | E | 6 |
|  | b. | Summarize the SMA sensor with an example. | CO5 | E | 6 |
|  |  | COMPULSORY QUESTION |  |  |  |
| 24. | a. | Explain the architecture of PLC and describe the function of various components. | CO6 | A | 6 |
|  | b. | Define PLC and discuss briefly about the LD programming methods. | CO6 | R | 6 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
|  | After completing the course, the student will be able to |
| CO1 | Summarize and recall the overview of mechatronics applications. |
| CO2 | Demonstrate knowledge of electrical circuits and logic design. |
| CO3 | Develop and formulate engineering solutions and techniques to solve design problems. |
| CO4 | Design mechatronic components and systems. |
| CO5 | Classify and select various micro-sensors and microprocessors for a specific problem. |
| CO6 | Develop PLC programs for a given task. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 2 | - | 9 | - | 6 | - | 17 |
| CO2 | 0 | 2 | 4 | 8 | 3 | - | 17 |
| CO3 | 1 | 12 | 3 | - | - | - | 16 |
| CO4 | 2 | 3 | 12 | - | - | - | 17 |
| CO5 | 1 | 13 | 3 | - | 12 | - | 29 |
| CO6 | 7 | - | 6 | 3 | 12 | - | 28 |
| Total | 12 | 31 | 37 | 11 | 33 | - | 124 |

SUPPLEMENTARY EXAMINATION – JUNE 2023

| # Course Code | # 18ME2058 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # MECHATRONICS SYSTEM | # Max. Marks: | # 100 |

| Q. No. | Questions | CO | BL | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) (Answer all the questions) |  |  |  |  |
| 1. | State programmable automation. | CO1 | R | 1 |
| 2. | Differentiate between sensor and actuator. | CO2 | U | 1 |
| 3. | Abbreviate HMI. | CO2 | R | 1 |
| 4. | State the term Mechatronics. | CO3 | R | 1 |
| 5. | Draw the simple sketch of DC shunt motor. | CO3 | U | 1 |
| 6. | Define relay. | CO4 | R | 1 |
| 7. | Name two electromechanical elements. | CO4 | R | 1 |
| 8. | List advantages and disadvantages of PLC. | CO6 | R | 1 |
| 9. | Draw the the pin diagram of microprocessor. | CO5 | U | 1 |
| 10. | Define e.m.f.. | CO5 | R | 1 |

| PART – B (6 X 3 = 18 MARKS) (Answer all the questions) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | In bourdon pressure gauge, identify the sensor, signal conditioner and display elements. | CO1 | U | 3 |
| 12. | Explain the working of relay circuit. | CO2 | A | 3 |
| 13. | Compare between electric, hydraulic and pneumatic systems. | CO3 | An | 3 |
| 14. | Discuss about piezoelectric material. | CO4 | U | 3 |
| 15. | Examine the working of a inductive proximity sensor. | CO5 | A | 3 |
| 16. | Analyze the basic logics in PLC programming. | CO6 | An | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | a. | Justify the statement -- Mechatronics is an interdisciplinary field. | CO1 | E | 6 |
|  | b. | List various levels of integration in a mechatronics system. | CO1 | R | 6 |
| 18. | a. | Identify the various elements that might be present in a control system involving a thermostatically controlled electric heater circuit. | CO2 | U | 8 |
|  | b. | List the applications of mechatronics systems. | CO2 | R | 4 |
| 19. | a. | Describe the principle and applications of rotary potentiometer. | CO3 | U | 8 |
|  | b. | Describe the working of an optical encoder with a neat diagram. | CO3 | U | 4 |
| 20. | a. | Illustrate PM type stepper motor working principle with neat diagram. | CO4 | A | 6 |
|  | b. | Discuss the principle of operation for VR stepper motor. | CO4 | U | 6 |
| 21. | a. | Describe the concept of Piezoelectric Effect with neat drawing. | CO5 | U | 6 |
|  | b. | Summarize the working of a magneto restrictive transducer. | CO5 | U | 6 |
| 22. | a. | Discuss the working of PLC based Electro-hydraulic power steering system in automobile with neat diagram. | CO6 | U | 6 |
|  | b. | Describe some of the I/O devices used for PLC. | CO6 | U | 6 |
| 23. | a. | Describe the basic principle of X-ray Lithography with neat diagram. | CO3 | U | 6 |
|  | b. | Discuss any two types of micro-joining processes. | CO3 | U | 6 |
|  |  | Compulsory |  |  |  |
| 24. | a. | Construct the architecture of PLC with a neat block diagram. | CO6 | A | 6 |
|  | b. | Describe the LD programming basic symbols of Siemens Step7. | CO6 | U | 6 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
|  | After completing the course, the student will be able to |
| CO1 | Summarize and recall the overview of mechatronics applications. |
| CO2 | Demonstrate knowledge of electrical circuits and logic design. |
| CO3 | Develop and formulate engineering solutions and techniques to solve design problems. |
| CO4 | Design mechatronic components and systems. |
| CO5 | Classify and select various micro-sensors and microprocessors for a specific problem. |
| CO6 | Develop PLC programs for a given task. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 7 | 3 | - | - | 6 | - | 16 |
| CO2 | 5 | 9 | 3 | - | - | - | 17 |
| CO3 | 1 | 1 | 24 | 3 | - | - | 29 |
| CO4 | 2 | 9 | 6 | - | - | - | 17 |
| CO5 | 1 | 13 | 3 | - | - | - | 17 |
| CO6 | 1 | 18 | 6 | 3 | - | - | 28 |
| Total | 17 | 53 | 42 | 6 | 6 | 0 | 124 |

| # Course Code | # 18ME2060 | # Duration | # 3 hrs |
| --- | --- | --- | --- |
| # Course Name | # INDUSTRIAL SAFETY ENGINEERING | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) (Answer all the questions) |  |  |  |  |  |
| 1. | Quote the mission statement of OSHA. |  | CO2 | R | 1 |
| 2. | Differentiate industrial accident and industrial hazard. |  | CO3 | U | 1 |
| 3. | Distinguish between unsafe act and unsafe condition. |  | CO3 | U | 1 |
| 4. | Identity the functions of machine guards. |  | CO5 | U | 1 |
| 5. | State an example for non- ionizing radiation. |  | CO5 | R | 1 |
| 6. | Examine the need for providing ventilation in a plant. |  | CO6 | A | 1 |
| 7. | Define ‘Toxicology’. |  | CO5 | R | 1 |
| 8. | ‘STEL’ stands as an acronym for ___________. |  | CO2 | R | 1 |
| 9. | Give examples for products which are manufactured using fixed position layout. |  | CO1 | U | 1 |
| 10. | Identify any two psychological hazards in industries. |  | CO4 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) (Answer all the questions) |  |  |  |  |  |
| 11. | Distinguish ‘product layout’ and ‘process layout’. |  | CO1 | U | 3 |
| 12. | Indicate the benefits of ensuring safety in workplace. |  | CO1 | U | 3 |
| 13. | Identify the non-auditory effects of noise on human health. |  | CO3 | U | 3 |
| 14. | Indicate the factors affecting the effects of toxic action on human body. |  | CO4 | U | 3 |
| 15. | Classify the various industrial safety hand gloves with respect to their applications. |  | CO6 | U | 3 |
| 16. | Indicate the benefits of good illumination. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Illustrate the relationship of industrial safety with productivity. | CO1 | A | 6 |
|  | b. | Explain the role of employee’s participation in the safety measures. | CO2 | A | 6 |
| 18. | a. | Illustrate the three major zones / applications to be safeguarded in manufacturing industries. | CO3 | A | 6 |
|  | b. | Explain the different types of machine guards with their uniqueness in ensuring industrial safety. | CO3 | A | 6 |
| 19. | a. | Illustrate the types of radiation and their effects on occupational health. Describe the protective measures for controlling radiation. | CO5 | A | 6 |
|  | b. | Connect the effect of temperature and humidity with occupational health. How the above factors can be effectively controlled for improving employee efficiency? | CO5 | An | 6 |
| 20. | a. | Discuss the guidelines to be followed when working with toxic substances. | CO2 | U | 6 |
|  | b. | Explain the importance of industrial hygiene survey. | CO6 | A | 6 |
| 21. | a. | Explain the basic practices to be followed in the workplace to ensure safety. | CO1 | A | 6 |
|  | b. | Analyze the different types of costs involved in occupational injury / disease. | CO1 | An | 6 |
| 22. | a. | Explain the different types of hearing and respiratory protecting devices used in industries. | CO4 | A | 6 |
|  | b. | Illustrate the symptoms of bad layout. Analyze the role of good layout in minimizing the possibility of industrial accidents. | CO1 | A | 6 |
| 23. | a. | Describe the approaches to prevent electrical accidents. | CO5 | U | 6 |
|  | b. | Illustrate the effects of noise on industrial accidents and its preventive measures. | CO5 | A | 6 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Explain the levels and characteristics of good illumination in industries. | CO6 | A | 6 |
|  | b. | Sanitation and housekeeping at the workplace reduces illnesses and injuries. Justify. | CO6 | E | 6 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understanding the importance of safety in process industries |
| CO2 | Understanding the ethical issues that may arise from industrial processes |
| CO3 | Communicate the difference between Hazard and Risk |
| CO4 | Be able to express Safety in terms of Risk and to recognize unacceptable/inappropriate levels of Risk. |
| CO5 | Be able to Assess & identify the potential hazards in process industries |
| CO6 | Appreciation and applying safety procedures in a process industries. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | R | U | A | An | E | C | Total |
| CO1 | - | 7 | 18 | 6 | - | - | 31 |
| CO2 | 2 | 6 | 6 | - | - | - | 14 |
| CO3 | - | 5 | 12 | - | - | - | 17 |
| CO4 | - | 4 | 6 | - | - | - | 10 |
| CO5 | 2 | 7 | 12 | 6 | - | - | 27 |
| CO6 | - | 6 | 13 | - | 6 | - | 25 |
|  |  |  |  |  |  |  | 124 |

SUPPLEMENTARY EXAMINATION – JUNE 2023

| # Course Code | # 18ME2060 | # Duration | # 3 hrs |
| --- | --- | --- | --- |
| # Course Name | # INDUSTRIAL SAFETY ENGINEERING | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) (Answer all the questions) |  |  |  |  |  |
| 1. | Give example for a machinery which may lead to workplace amputations. |  | CO3 | U | 1 |
| 2. | Identify a product which is manufactured using product layout. |  | CO1 | U | 1 |
| 3. | Define ‘Nip-point’. |  | CO3 | R | 1 |
| 4. | Define Zero Mechanical state. |  | CO5 | R | 1 |
| 5. | Give examples for ionizing radiation. |  | CO6 | U | 1 |
| 6. | Identify the adverse effects of noise on human beings. |  | CO4 | U | 1 |
| 7. | Define ‘Threshold limit value’. |  | CO2 | R | 1 |
| 8. | Identify the health issues faced by workers due to poor indoor air quality. |  | CO3 | U | 1 |
| 9. | Differentiate natural and artificial lighting. |  | CO6 | U | 1 |
| 10. | Identify any two benefits of effective workplace housekeeping. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) (Answer all the questions) |  |  |  |  |  |
| 11. | Distinguish ‘Engineering controls’ and ‘Administrative control’ in ensuring safety in work places. |  | CO1 | U | 3 |
| 12. | Identify the important functions of machine guards. |  | CO5 | U | 3 |
| 13. | Identify few two safety measures while handling pressurized equipment. |  | CO5 | U | 3 |
| 14. | Compare local effects and systemic effects of toxic substances. |  | CO4 | U | 3 |
| 15. | Distinguish between product layout and fixed-position layout. |  | CO1 | U | 3 |
| 16. | Indicate any three ill effects of bad illumination on human body and mind. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Plant safety in an organization improves plant productivity. Justify. | CO1 | E | 6 |
|  | b. | Analyze the various causes of industrial accidents. Explain in detail the methods to prevent them from occurring. | CO2 | An | 6 |
| 18. | a. | Illustrate the procedure to do Lockout / Tag-out in a machine / equipment in a sequential manner. | CO4 | A | 6 |
|  | b. | Explain different types of safety control devices used in industries with their uniqueness in ensuring industrial safety. | CO5 | A | 6 |
| 19. | a. | Identify few protective measures in industries that can followed to control the heat exposure on human body. | CO5 | U | 6 |
|  | b. | Illustrate the biological hazards in workplace. Also discuss their effects and preventive measures. . | CO3 | A | 6 |
| 20. | a. | Explain about the toxic chemicals used in industries and their harmful effects on human system. | CO4 | A | 6 |
|  | b. | Write about the different modes of entry of toxic substances into the human body. | CO4 | A | 6 |
| 21. | a. | Classify and explain the different types of unsafe conditions in industry. | CO1 | An | 6 |
|  | b. | Personal Protective Equipment minimize exposure to a variety of hazards. Justify with suitable examples. | CO6 | E | 6 |
| 22. | a. | Identify the information described in MSDS and explain its significance in hazard control. | CO6 | U | 6 |
|  | b. | Supervisors and managers are playing an important role in ensuring safety in workplace. Justify. | CO3 | E | 6 |
| 23. | a. | Analyze the role of good layout in minimizing industrial hazards and accidents. | CO5 | An | 6 |
|  | b. | Explain the factors influencing the effects of toxic materials on human body. | CO5 | A | 6 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Explain the principles that are to be followed in designing good illumination. Identify its benefits. | CO6 | A | 6 |
|  | b. | Good Sanitation and housekeeping at the workplace reduces illnesses and injuries. Justify. | CO6 | E | 6 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understanding the importance of safety in process industries |
| CO2 | Understanding the ethical issues that may arise from industrial processes |
| CO3 | Communicate the difference between Hazard and Risk |
| CO4 | Be able to express Safety in terms of Risk and to recognize unacceptable/inappropriate levels of Risk. |
| CO5 | Be able to Assess & identify the potential hazards in process industries |
| CO6 | Appreciation and applying safety procedures in a process industries. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | R | U | A | An | E | C | Total |
| CO1 | - | 7 | - | 6 | 6 | - | 19 |
| CO2 | 1 | - | - | 6 | - | - | 7 |
| CO3 | 1 | 2 | 6 | - | 6 | - | 15 |
| CO4 | - | 4 | 18 | - | - | - | 22 |
| CO5 | 1 | 12 | 12 | 6 | - | - | 31 |
| CO6 | - | 12 | 6 | - | 12 | - | 30 |
|  |  |  |  |  |  |  | 124 |

| # Course Code | # 18ME2063 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # RAPID MANUFACTURING TECHNOLOGIES | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) (Answer all the questions) |  |  |  |  |  |
| 1. | Expand the term: STL. |  | CO1 | U | 1 |
| 2. | Mention two applications of rapid prototyping processes. |  | CO1 | R | 1 |
| 3. | List any two software used for slicing of CAD models. |  | CO2 | R | 1 |
| 4. | State any one example of reverse engineering. |  | CO2 | R | 1 |
| 5. | Define: SLA process. |  | CO3 | U | 1 |
| 6. | State true or false: FDM is a solid based RP system. |  | CO3 | R | 1 |
| 7. | Explain the basic principle of SGC process. |  | CO4 | U | 1 |
| 8. | Recall the process variables of LOM system. |  | CO4 | R | 1 |
| 9. | Differentiate: SLA and 3DP. |  | CO5 | U | 1 |
| 10. | Expand the term: LENS. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) (Answer all the questions) |  |  |  |  |  |
| 11. | Explain the applications of Rapid Manufacturing. |  | CO1 | An | 3 |
| 12. | Recall the applications of digitization techniques in CAD. |  | CO2 | U | 3 |
| 13. | Evaluate the common issues in SLS. |  | CO3 | An | 3 |
| 14. | Illustrate the construction of a 3DP system with a neat sketch. |  | CO4 | U | 3 |
| 15. | Estimate the strengths and weaknesses of SLA system. |  | CO5 | An | 3 |
| 16. | Give examples for direct tooling methods. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Describe the salient features of RP systems. | CO1 | C | 12 |
| 18. |  | Highlight the aspects of part orientation and support generation in RP systems. | CO2 | An | 12 |
| 19. | a. | Mention the mechanism behind photo polymerization of SLA resins. | CO3 | A | 6 |
|  | b. | Construct the strengths, weaknesses and applications of SLS. | CO3 | A | 6 |
| 20. |  | Explain the working principle and materials used in LOM. | CO4 | C | 12 |
| 21. |  | Express the advantages, disadvantages and applications of powder based 3DP systems. | CO5 | E | 12 |
| 22. |  | Recall the products developed using FDM systems with suitable case study. | CO4 | A | 12 |
| 23. |  | Discover the process involved in generation of tool path for CAD modelling. | CO2 | An | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Explain the construction and working of Controlled Metal Build-up (CMB) with neat sketch. | CO6 | An | 12 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Differentiate between conventional and rapid manufacturing approach. |
| CO2 | Demonstrate the knowledge of Rapid Manufacturing technologies. |
| CO3 | Understand the need and place for RP in an integrated manufacturing environment. |
| CO4 | Get exposed to commercial Rapid Prototyping systems. |
| CO5 | Possess knowledge on Rapid Prototyping software. |
| CO6 | Model and manufacture RP components. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | R | U | A | An | E | C | Total |
| CO1 | 1 | 1 | - | 3 | - | 12 | 17 |
| CO2 | 2 | 3 | - | 24 | - | - | 29 |
| CO3 | 1 | 1 | 12 | 3 | - | - | 17 |
| CO4 | 1 | 4 | 12 | - | - | 12 | 29 |
| CO5 | - | 1 | - | 3 | 12 | - | 16 |
| CO6 | - | 4 | - | 12 | - | - | 16 |
|  |  |  |  |  |  |  | 124 |

| # Course Code | # 18ME2067 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # AUTOMOBILE ENGINEERING | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Identify the differences between S.I. engine and C.I. engine. |  | CO1 | U | 1 |
| 2. | Define hybrid vehicles. |  | CO1 | R | 1 |
| 3. | State the function of a transfer box in an automobile. |  | CO2 | R | 1 |
| 4. | Recall the function of a torque converter. |  | CO2 | R | 1 |
| 5. | State the condition for perfect steering. |  | CO3 | U | 1 |
| 6. | Identify the difference between ‘rocking lever dampers’ and ‘telescopic dampers’. |  | CO3 | R | 1 |
| 7. | Identify any two methods to control rapid tyre wear. |  | CO4 | U | 1 |
| 8. | State the main function of a secondary brake. |  | CO4 | R | 1 |
| 9. | Identify the role of thyristor in capacitive discharge ignition system. |  | CO5 | U | 1 |
| 10. | Identify the difference in refueling between hydrogen cars and other electric cars. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Distinguish between chassis and a frame. |  | CO1 | An | 3 |
| 12. | Identify the differences between manual transmission and automatic transmission. |  | CO2 | U | 3 |
| 13. | Distinguish between ‘bouncing’ and ‘rolling’ of an automobile. |  | CO3 | An | 3 |
| 14. | Identify the merits of disc brakes. |  | CO4 | U | 3 |
| 15. | Categorize the role of oxidation catalyst and reduction catalyst in a catalytic converter. |  | CO5 | An | 3 |
| 16. | Indicate how water can be used as a fuel in automobiles. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.No 17 to 23, Q.No 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Inline engine arrangement not popular for high power automobiles. Justify. | CO1 | E | 6 |
|  | b. | Outline the different types of loads acting on the frame of an automobile. | CO1 | An | 6 |
| 18. | a. | Illustrate the functions and working of a differential in a transmission system with a neat sketch. | CO2 | A | 6 |
|  | b. | Explain the working principle, requirements and functions of a clutch. | CO2 | U | 6 |
| 19. | a. | Compare dependent and independent suspension systems in terms their constructional characteristics. | CO3 | An | 6 |
|  | b. | Discuss the major types of springs used in suspension system. | CO3 | U | 6 |
| 20. | a. | Compare the constructional features and merits of Tubeless tyres and Tubed Tyres. | CO4 | E | 6 |
|  | b. | Describe the important functions and required properties of tyres. | CO4 | U | 6 |
| 21. | a. | Illustrate the working principle and advantages of electronic fuel injection system. | CO5 | A | 6 |
|  | b. | Illustrate how turbocharger improves horse power, fuel economy and efficiency of an engine. | CO5 | A | 6 |
| 22. | a. | Anti-lock braking system has become a vital safety feature even in common vehicles. Justify. | CO4 | E | 6 |
|  | b. | Identify the various causes of tyre wear. | CO4 | U | 6 |
| 23. | a. | Compare Hotchkiss and Torque tube drive. List their advantages and limitations. | CO2 | An | 6 |
|  | b. | Outline how automated manual transmission system differs from intelligent manual transmission system. | CO2 | An | 6 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Even though Hydrogen fuel cell vehicles produce no pollution, it is not flourishing worldwide. Justify. | CO6 | E | 6 |
|  | b. | Discuss the top five biggest problems and challenges with electric vehicles. | CO6 | U | 6 |

CO – COURSE OUTCOME			BL – BLOOMS’ LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand different types of internal combustion engines. |
| CO2 | Demonstrate the functions of clutch and gear box systems. |
| CO3 | Describe the types of steering and suspension systems. |
| CO4 | Summarize the construction and operating principles of brakes and tyres. |
| CO5 | Express the functions and components of fuel injection and ignition systems. |
| CO6 | Analyze the performance & emissions of alternate fuels. |

| Assessment Pattern as per Bloom’s Level |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 1 | 1 | - | 9 | 6 | - | 17 |
| CO2 | 2 | 9 | 6 | 12 | - | - | 29 |
| CO3 | 1 | 7 | - | 9 | - | - | 17 |
| CO4 | 1 | 16 |  | - | 12 | - | 29 |
| CO5 | 1 | - | 12 | 3 | - |  | 16 |
| CO6 | - | 10 |  |  | 6 |  | 16 |
|  |  |  |  |  |  |  | 124 |

SUPPLEMENTARY EXAMINATION – JUNE 2023

| # Course Code | # 18ME2067 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # AUTOMOBILE ENGINEERING | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) (Answer all the questions) |  |  |  |  |  |
| 1. | Name any two types of chassis used in transport vehicles |  | CO1 | R | 1 |
| 2. | Distinguish the function of inlet and exhaust manifolds used in automobile engine. |  | CO1 | E | 1 |
| 3. | The inertia of the rotating parts of the clutch should be __________. |  | CO2 | A | 1 |
| 4. | Define perfect steering. |  | CO2 | U | 1 |
| 5. | List any two benefits of pneumatic braking system. |  | CO3 | An | 1 |
| 6. | State the any two significance of tyre specification. |  | CO4 | A | 1 |
| 7. | State the importance of propellor shaft in automobles. |  | CO4 | C | 1 |
| 8. | Identify the function of Multi Point Fuel Injection (MPFI) in automobile engine. |  | CO5 | A | 1 |
| 9. | List the importance for the term ‘octane rating’ of petrol used in Indian automobiles. |  | CO5 | U | 1 |
| 10. | State the function of a double arm type contact breaker. |  | CO6 | A | 1 |
| PART – B (6 X 3 = 18 MARKS) (Answer all the questions) |  |  |  |  |  |
| 11. | Enumerate the importance and function of chassis used in passenger vehicles. |  | CO1 | U | 3 |
| 12. | List out the need and importance of suspension system used in transport Vehicles. |  | CO2 | A | 3 |
| 13. | Indicates the benefits of four wheel drive in automobile vehicles. |  | CO3 | An | 3 |
| 14. | List any three benefits of wheel alignment of passenger vehicles. |  | CO4 | A | 3 |
| 15. | Differentiate between electronic injection system in SI and CI engine. |  | CO5 | C | 3 |
| 16. | List the benefits and function of electronically controlled automatic transmission system. |  | CO6 | E | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | In detail describe the working of a single plate clutch with the help of a simple diagram. | CO1 | A | 12 |
| 18. |  | Describe in detail various types of fuels used in automotive engines. Give specific examples in each case. | CO2 | C | 12 |
| 19. |  | Explain the principle of steering and with neat sketch discuss the detail working principle of Centre point steering mechanism. | CO3 | An | 12 |
| 20. |  | Discuss the need and working principle of coil and leaf spring suspension system used in transport vehicles with neat sketch. | CO4 | A | 12 |
| 21. | a. | Describe the need and importance of antilock braking system (ABS) used in passenger vehicles | CO4 | E | 6 |
|  | b. | List out the causes of tyre wear and remedial actions to be carried out in transport vehicles to reduce tyre wear. | CO4 | A | 6 |
| 22. |  | Explain the working of a single plate clutch with the help of a simple diagram. | CO3 | U | 12 |
| 23. |  | Describe the function of an automobile transmission system using a neat sketch. | CO5 | An | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |  |  |  |  |
| 24. | a. | Illustrate the principles of common rail direct injection system and its application. | CO6 | A | 12 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand different types of internal combustion engines. |
| CO2 | Demonstrate the functions of clutch and gear box systems. |
| CO3 | Describe the types of steering and suspension systems. |
| CO4 | Summarize the construction and operating principles of brakes and tyres. |
| CO5 | Express the functions and components of fuel injection and ignition systems. |
| CO6 | Analyze the performance and emissions of alternate fuels. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | R | U | A | An | E | C | Total |
| CO1 | 1 | 3 | 12 |  | 1 | - | 17 |
| CO2 |  | 1 | 4 |  |  | 12 | 17 |
| CO3 | - |  | 1 | 16 | - | - | 17 |
| CO4 | - |  | 20 | - | 8 | 1 | 29 |
| CO5 |  | 1 | - | 12 | - | 3 | 16 |
| CO6 |  | 12 | 13 | - | 3 | - | 28 |
|  |  |  |  |  |  |  | 124 |

SUPPLEMENTARY EXAMINATION – JUNE 2023

| # Course Code: | # 18ME2073 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Course Name : | # MODERN MANUFACTURING TECHNIQUES | # Max. Marks: | # 100 |

| Q. No. |  | Questions | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Identify the manufacturing process where goods are produced in huge quantities using standardized designs, machinery, and assembly line techniques. |  | CO1 | U | 1 |
| 2. | Name the person who introduced the concept of assembly line. |  | CO1 | R | 1 |
| 3. | Identify the function of piezoelectric transducers in USM process. |  | CO2 | U | 1 |
| 4. | State the mechanism of metal removal in ECM process. |  | CO2 | R | 1 |
| 5. | Name any two high explosives used in explosive forming. |  | CO3 | R | 1 |
| 6. | Identify any two deformation processes that are classified under bulk forming. |  | CO3 | U | 1 |
| 7. | Name the metal casting process which is used for making billets. |  | CO4 | R | 1 |
| 8. | Identify the function of core in metal casting. |  | CO4 | U | 1 |
| 9. | Identify the purpose of shielding the electrode in SMAW process. |  | CO5 | U | 1 |
| 10. | State the applications of Ultrasonic welding. |  | CO5 | R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Differentiate thermoplastics with thermosetting plastics. | CO1 | U | 3 |
| 12. | Identify the functions of dielectric fluid in EDM process. | CO2 | U | 3 |
| 13. | Distinguish between plastic and elastic deformation. | CO3 | An | 3 |
| 14. | Distinguish between runner and riser in foundry process. | CO4 | U | 3 |
| 15. | Compare TIG welding and MIG welding. | CO5 | An | 3 |
| 16. | Distinguish between closed and open loop NC systems. | CO6 | An | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | a. | Classify different types of manufacturing processes with their applications. | CO1 | An | 6 |
|  | b. | Categorize different composite types based on reinforcement forms. | CO1 | An | 6 |
| 18. | a. | Discuss the effect of process parameters on the machining through AJM process. | CO2 | U | 6 |
|  | b. | Describe the working of Electrochemical machining process. | CO2 | U | 6 |
| 19. | a. | Describe the working of Electro Hydraulic forming. | CO3 | U | 6 |
|  | b. | Identify the advantages and limitations of hot working. | CO3 | U | 6 |
| 20. | a. | Illustrate the components of gating system and their functions. | CO4 | A | 6 |
|  | b. | Describe squeeze casting with suitable sketches. | CO4 | U | 6 |
| 21. | a. | Illustrate the working of Ultrasonic welding equipment with suitable sketches. | CO5 | A | 6 |
|  | b. | Various forces during metal transfer affect the quality of welding. Justify. | CO5 | E | 6 |
| 22. | a. | Illustrate the working principle and equipment setup of Electric Discharge Machining. | CO2 | A | 12 |
| 23. | a. | Describe Explosive Forming with a neat sketch. List its advantages, limitations and applications. | CO3 | U | 12 |
| COMPULSORY |  |  |  |  |  |
| 24. | a. | Explain the basic components and functions of NC System. | CO6 | U | 6 |
|  | b. | Illustrate the prevalent applications of industrial robots. | CO6 | A | 6 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand different manufacturing processes and the economic considerations |
| CO2 | Understand the theory of metal cutting and the sciences of advanced machining processes |
| CO3 | Learn the theories of advanced metal forming. |
| CO4 | Know about the process of metal casting in detail |
| CO5 | Understand the physics of arc welding and theory of advanced welding techniques |
| CO6 | Demonstrate an understanding of competitive manufacturing environment |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 1 | 4 |  | 12 | - | - | 17 |
| CO2 | 1 | 16 | 12 |  |  |  | 29 |
| CO3 | 1 | 25 |  | 3 | - | - | 29 |
| CO4 | 1 | 10 | 6 |  |  |  | 17 |
| CO5 | 1 | 1 | 6 | 3 | 6 | - | 17 |
| CO6 |  | 6 | 6 | 3 | - | - | 15 |
|  |  |  |  |  |  |  | 124 |

| # Course Code | # 18ME2073 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # MODERN MANUFACTURING TECHNIQUES | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Identify the process which combines both of injection molding and compression molding process. |  | CO1 | U | 1 |
| 2. | Name the type of plastics which can be repeatedly processed by melting and subsequent solidification without undergoing any chemical change. |  | CO1 | R | 1 |
| 3. | State the mechanism of metal removal in USM process. |  | CO2 | R | 1 |
| 4. | Recall the difference between amplifying and non-amplifying type of tool holders in USM. |  | CO2 | R | 1 |
| 5. | Identify the reason for the use of drivers in Electro Magnetic Forming. |  | CO3 | U | 1 |
| 6. | Name any two important explosives used in explosive forming. |  | CO3 | R | 1 |
| 7. | Identify the casting process which is used for making billets. |  | CO4 | U | 1 |
| 8. | Name the material used for making pattern in evaporative pattern casting. |  | CO4 | R | 1 |
| 9. | Recall how arc stability can be improved. |  | CO5 | U | 1 |
| 10. | Identify the role of ‘Design for assembly’ in modern manufacturing. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Differentiate matrix and reinforcement in composites. |  | CO1 | An | 3 |
| 12. | Identify how a cutting tool differs from machine tool. |  | CO2 | U | 3 |
| 13. | Differentiate between elastic and plastic deformation. |  | CO3 | An | 3 |
| 14. | Identify how a runner differs riser in metal casting. |  | CO4 | U | 3 |
| 15. | Distinguish between DC arc welding and AC arc welding. |  | CO5 | An | 3 |
| 16. | Identify the difference between closed loop and open loop NC systems. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.No 17 to 23, Q.No 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Describe the process of injection moulding for thermoplastic polymers. | CO1 | U | 6 |
|  | b. | Categorize and explain different types of composites based on the type of material used for matrix. | CO1 | An | 6 |
| 18. | a. | Describe the working principle and equipment used in Ultrasonic machining process. | CO2 | U | 6 |
|  | b. | Discuss the working principle and applications of Electric Discharge Machining process. | CO2 | U | 6 |
| 19. | a. | Compare hot working and cold working processes. | CO3 | An | 6 |
|  | b. | Many difficult to form materials can be deformed under high strain rates Explosive forming. Justify the statement. Also discuss its working principle. | CO3 | E | 6 |
| 20. | a. | Identify the various functions of gating systems. | CO4 | U | 6 |
|  | b. | Describe squeeze casting with suitable sketches. | CO4 | U | 6 |
| 21. | a. | Explain electron beam welding with help of neat sketch. | CO5 | U | 6 |
|  | b. | Analyze and suggest different measures that can improve arc stability. | CO5 | An | 6 |
| 22. |  | Illustrate the working principle, equipment, applications, advantages and disadvantages of Abrasive Jet Machining process. | CO2 | A | 12 |
| 23. |  | Illustrate the working principle, equipment, applications, advantages and disadvantages of Electro Magnetic forming. | CO3 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Describe the basic components and functions of NC System. | CO6 | U | 6 |
|  | b. | Discuss the applications of Industrial Robots in manufacturing industries. | CO6 | U | 6 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand different manufacturing processes and the economic considerations. |
| CO2 | Understand the theory of metal cutting and the sciences of advanced machining processes. |
| CO3 | Learn the theories of advanced metal forming. |
| CO4 | Know about the process of metal casting in detail. |
| CO5 | Understand the physics of arc welding and theory of advanced welding techniques. |
| CO6 | Demonstrate an understanding of competitive manufacturing environment. |

| Assessment Pattern as per Bloom’s Level |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 1 | 7 | - | 9 | - | - | 17 |
| CO2 | 2 | 15 | 12 | - | - | - | 29 |
| CO3 | 1 | 1 | 12 | 9 | 6 | - | 29 |
| CO4 | 1 | 16 | - | - | - | - | 17 |
| CO5 | 7 | - | - | 9 | - | - | 16 |
| CO6 | - | 16 | - | - | - | - | 16 |
|  |  |  |  |  |  |  | 124 |

| # Course Code | # 18ME2074 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # RENEWABLE ENERGY TECHNOLOGIES | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) (Answer all the questions) |  |  |  |  |  |
| 1. | List a few renewable energy resources. |  | CO1 | R | 1 |
| 2. | Distinguish the conventional and non-conventional energy systems. |  | CO1 | A | 1 |
| 3. | Recall the application of the Pyrometer. |  | CO2 | R | 1 |
| 4. | Define solar constant. |  | CO2 | R | 1 |
| 5. | Illustrate the chemical reactions of the gasification process |  | CO3 | U | 1 |
| 6. | Summarise a few beneficiation processes. |  | CO3 | U | 1 |
| 7. | Define power coefficient. |  | CO4 | R | 1 |
| 8. | Give the equation to calculate the axial force on the turbine. |  | CO4 | U | 1 |
| 9. | What is a fuel cell? |  | CO5 | R | 1 |
| 10. | Expand the term OTEC. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) (Answer all the questions) |  |  |  |  |  |
| 11. | Distinguish short waves and long waves. |  | CO1 | An | 3 |
| 12. | Elucidate the principle of solar chimneys. |  | CO2 | E | 3 |
| 13. | What is the principle of solar cells? |  | CO3 | R | 3 |
| 14. | What are the factors that determine the output power from the wind? |  | CO4 | R | 3 |
| 15. | Give examples of wave energy devices. |  | CO5 | U | 3 |
| 16. | Write short notes on mini hydro systems. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Explain the prospects of renewable energy resources in India. | CO1 | U | 8 |
|  | b. | Discuss the history of the energy consumption pattern of the world. | CO1 | C | 4 |
| 18. | a. | Describe the solar cell's principle and explain the construction and operation with necessary sketches. | CO2 | E | 6 |
|  | b. | Describe the construction and working of an indirect solar water heating system with an antifreeze solution. | CO2 | U | 6 |
| 19. | a. | The following data is given for a family biogas digester suitable for the output of 8 cows. The Calorific value of methane is 28MJ/m3; Burner efficiency: 70%; Retention period: 20 days; Temperature of fermentation: 30∘C; Dry matter (cow dung) collected per cow per day: 2Kg; Density of dry matter in fluid(slurry) in the digester: 50kg/m3; Biogas yield: 0.2m3 per kg of dry input; Methane proportion in the biogas: 0.7; Calculate the volume of the biogas digester and the power available from the digester | CO3 | E | 6 |
|  | b. | Discuss the construction and working of a floating dome-type biogas plant with the help of a neat sketch. | CO3 | C | 6 |
| 20. | a. | Explain in detail the site selection considerations for wind energy conversion systems. | CO4 | E | 12 |
| 21. | a. | Explain any one of the wave energy conversion devices with a neat sketch. | CO5 | E | 4 |
|  | b. | Describe the working of the Closed Cycle OTEC system with the help of a neat sketch | CO5 | U | 8 |
| 22. | a. | Explain the various energy extraction technologies used with hydrothermal resources. | CO6 | E | 8 |
|  | b. | List the advantages and disadvantages of the Fuel cell. | CO6 | An | 4 |
| 23. | a. | Explain the working of the Pyranometer with a neat circuit diagram | CO2 | E | 8 |
|  | b. | List the advantages of Solar energy | CO2 | An | 4 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Explain the operation of Proton-exchange Membrane Fuel Cell (PMFC) with a neat diagram. | CO6 | A | 12 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Identify the various renewable energy sources. |
| CO2 | Summarize the application of solar energy systems. |
| CO3 | Develop technology to convert waste biomass into useful energy. |
| CO4 | Evaluate the performance of wind energy systems. |
| CO5 | Estimate power production from tidal and ocean energy systems |
| CO6 | Understand the energy conversion from geothermal and hydrogen sources |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | R | U | A | An | E | C | Total |
| CO1 | 1 | 8 | 1 | 3 | - | 4 | 17 |
| CO2 | 2 | 6 | - | 4 | 17 | - | 29 |
| CO3 | 3 | 2 | - | - | 6 | 6 | 17 |
| CO4 | 4 | 1 | - | - | 12 | - | 17 |
| CO5 | 1 | 11 | - | - | 4 | - | 16 |
| CO6 | - | 4 | 12 | 4 | 8 | - | 28 |
|  |  |  |  |  |  |  | 124 |

| # Course Code | # 18ME3010 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # ADVANCED MANUFACTURING PROCESSES | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (4 X 20 = 80 MARKS) (Answer all the Questions) |  |  |  |  |  |
| 1. | a. | Summarize nine different metal casting processes and tabulate their advantages and limitations. | CO1 | A | 10 |
|  | b. | Sketch the outline of production steps in a typical sand-casting operation and explain in detail. | CO1 | A | 10 |
|  |  | (OR) |  |  |  |
| 2. | a. | Discuss elaborately the process, advantages and limitations of investment casting with relevant sketches. | CO1 | A | 10 |
|  | b. | Illustrate the sequence of operations involved in the ceramic casting with neat schematics. | CO1 | A | 10 |
| 3. | a. | Compare the following extrusion processes with relevant sketches: (a) Indirect (b) Hydrostatic (c) Lateral. | CO2 | A | 10 |
|  | b. | Derive the expression for the drawing force, F, under ideal and frictionless conditions that are similar to that of extrusion and explain the process variables in wire drawing. Use necessary sketches. | CO2 | A | 10 |
|  |  | (OR) |  |  |  |
| 4. | a. | Summarize the general characteristics of major processes of sheet forming and the equipment used to make sheet-metal products in a table. | CO3 | An | 10 |
|  | b. | Illustrate schematically the process of shearing with a punch and die, indicating some of the process variables, and characteristic features of a punched hole. | CO3 | An | 10 |
| 5. |  | Derive equations and explain the significance of the following: (a) Radius of Bend Allowance (b) Minimum Bend Radius (c) Spring Back in Bending (d) Bending Force. | CO3 | An | 20 |
|  |  | (OR) |  |  |  |
| 6. |  | Categorize the advanced machining processes and explain the general characteristics process parameters and typical material-removal rate/ cutting speed. | CO4 | An | 20 |
| 7. | a. | Draw the schematics and illustrate the operation and working principle of the electrical-discharge machining process with some examples. | CO4 | An | 10 |
|  | b. | Sketch neatly and explain the working construction and working principle of the electrical-discharge wire cutting machine. List a few examples. | CO4 | An | 10 |
|  |  | (OR) |  |  |  |
| 8. | a. | Classify and compare various lithography techniques involved in (a) Masking to achieve pattern transfer and (b) Scribing a pattern without masking with necessary sketches. | CO5 | E | 10 |
|  | b. | Discuss the following steps involved in photolithography for prebaking the wafer and removing the solvent using illustrative sketches. (a) Wafer stepper technique for pattern transfer, and (b) Step-and-scan technique. | CO5 | E | 10 |
| PART – B (1 X 20 = 20 MARKS) COMPULSORY QUESTION |  |  |  |  |  |
| 9. |  | Classify all the types of joining processes and discuss the following in detail with neat sketches. (a) Shielded Metal Arc Welding (b) Friction Stir Welding (c) Adhesive Bonding (d) Brazing. | CO6 | E | 20 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Evaluate and select suitable manufacturing processes for particular applications. |
| CO2 | Recognize the need for unconventional manufacturing processes for various applications. |
| CO3 | Apply the latest manufacturing process for microfabrication. |
| CO4 | Develop new products by making use of new materials and processes. |
| CO5 | Establish newer manufacturing methods to replace conventional fabrication methods. |
| CO6 | Adapt the powder metallurgy technique to fabricate components for diverse applications. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | R | U | A | An | E | C | Total |
| CO1 |  |  | 40 |  |  |  | 40 |
| CO2 |  |  | 20 |  |  |  | 20 |
| CO3 |  |  |  | 40 |  |  | 40 |
| CO4 |  |  |  | 40 |  |  | 40 |
| CO5 |  |  |  |  | 20 |  | 20 |
| CO6 |  |  |  |  | 20 |  | 20 |
|  |  |  |  |  |  |  | 180 |

| # Course Code | # 18ME3031 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # TWO PHASE FLOW AND HEAT TRANSFER | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (4 X 20 = 80 MARKS) (Answer all the Questions) |  |  |  |  |  |
| 1. |  | With a neat sketch elucidate various flow patterns in vertical heated tube. | CO1 | A | 20 |
|  |  | (OR) |  |  |  |
| 2. |  | A horizontal pipe is given, in which a two phase flow is occurring, and it is given that the volume fraction or hold up of air is 45%. It is given that the mixture flow rate is 10 kg/sec and the diameter of the pipe is specified as 0.05m. Assume the gas as air and liquid as water, calculate the pressure drop in the pipeline. Given the overall length of the pipeline as 1000m. | CO1 | An | 20 |
| 3. |  | Deliberate the water circulation methods in boilers with neat sketch. | CO2 | U | 20 |
|  |  | (OR) |  |  |  |
| 4. |  | Describe major factors contribute to the hydrodynamics stability of a vapor-liquid system in a boiler. | CO2 | A | 20 |
| 5. |  | Explain the determination of minimum fluidization velocity of fluidized beds in detail. | CO3 | A | 20 |
|  |  | (OR) |  |  |  |
| 6. |  | Explain the following: - Expanded bed - Elutriation - Semi fluidization - Dilute phase | CO3 | A | 20 |
| 7. |  | Distinguish between dropwise condensation and film wise condensation. Explain various mechanisms involved in dropwise condensation process of pure gases. | CO4 | U | 20 |
|  |  | (OR) |  |  |  |
| 8. |  | Saturated steam at 1.43 bar and 383 K condenses on a vertical tube, 1.9 cm outer diameter and 20 cm long. The tube surface is maintained at a temperature of 382 K. Calculate the average heat transfer coefficient and local heat transfer coefficient at the bottom edge of the tube. Also calculate the thickness of condensate at the bottom edge. | CO5 | E | 20 |
| PART – B (1 X 20 = 20 MARKS) COMPULSORY QUESTION |  |  |  |  |  |
| 9. |  | Explain the bubble formation process, bubble growth gas hold up, and gas mixing liquid holdup in detail. | CO6 | A | 20 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand vertical, horizontal and inclined two phase flow. |
| CO2 | Determine effective pressure head in boiler tubes. |
| CO3 | Choose various types of fluidized beds. |
| CO4 | Evaluate heat transfer during condensation. |
| CO5 | Analyze heat transfer with change of phase in boiling. |
| CO6 | Explain various gas- liquid fluidizations. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | R | U | A | An | E | C | Total |
| CO1 | - | - | 20 | 20 | - | - | 40 |
| CO2 | - | 20 | 20 | - | - | - | 40 |
| CO3 | - | - | 40 | - | - | - | 40 |
| CO4 | - | 20 | - | - | - | - | 20 |
| CO5 | - | - | - | - | 20 | - | 20 |
| CO6 | - | - | 20 | - | - | - | 20 |
|  |  |  |  |  |  |  | 180 |

| # Course Code | # 18ME3062 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # COMPOSITE MATERIALS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (4 X 20 = 80 MARKS) (Answer all the Questions) |  |  |  |  |  |
| 1. | a. | Assess the general characteristics and applications of composite materials. | CO 1 | A | 10 |
|  | b. | Articulate the functions and types of smart materials. | CO 1 | A | 10 |
|  |  | (OR) |  |  |  |
| 2. | a. | Distinguish composites with respect to the different matrix materials. | CO 2 | An | 10 |
|  | b. | Explain the salient features and applications of fibers. | CO 2 | A | 10 |
| 3. | a. | Examine interlaminar stresses in composites. | CO 5 | A | 10 |
|  | b. | Appraise the fatigue and impact properties. | CO 5 | An | 10 |
|  |  | (OR) |  |  |  |
| 4. | a. | Illustrate the Fracture behaviour of composites. | CO 5 | A | 10 |
|  | b. | Evaluate the ‘Damage Tolerance’ in composites. | CO 5 | E | 10 |
| 5. | a. | Compare the ‘Hand lay-up processes’ and ‘Compression moulding’ process in synthesizing polymer composites. | CO 3 | An | 10 |
|  | b. | Justify the use of ‘Spray up processes’ in making composites. | CO 3 | E | 10 |
|  |  | (OR) |  |  |  |
| 6. | a. | Predict the suitability of Resin transfer moulding process in the preparation of composites. | CO 3 | E | 10 |
|  | b. | Summarize the salient features of injection moulding process. | CO 3 | E | 10 |
| 7. | a. | Survey the various Characteristics of MMC and their Various types. | CO 4 | E | 10 |
|  | b. | Distinguish ‘stir casting’ and ‘squeeze casting’ processes. | CO 4 | E | 10 |
|  |  | (OR) |  |  |  |
| 8. | a. | Evaluate the Stress analysis of laminated Composite beams and plates. | CO 6 | E | 10 |
|  | b. | Rank the reliability of composites in comparison with other engineering materials. | CO 6 | E | 10 |
| PART – B (1 X 20 = 20 MARKS) COMPULSORY QUESTION |  |  |  |  |  |
| 9. | a. | Propose laminate design consideration with respect to a specific component. | CO 5 | C | 10 |
|  | b. | Generalize failure mode Predictions in composites. | CO 5 | C | 10 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Predict elastic properties of composites. |
| CO2 | Predict mechanical properties of fiber reinforced composite materials. |
| CO3 | Design a composite laminate for a given load condition. |
| CO4 | Describe fundamental fabrication processes for polymer matrix composites. |
| CO5 | Analyze the stresses using laminated plate theories. |
| CO6 | Compare and contrast different processes of manufacture of polymer composites. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | R | U | A | An | E | C | Total |
| CO1 | - | - | 20 | - | - | - | 20 |
| CO2 | - | - | 10 | 10 | - | - | 20 |
| CO3 | - | - | - | 10 | 30 | - | 40 |
| CO4 | - | - | 20 | 10 | 10 | - | 40 |
| CO5 | - | - | 10 | 10 | 20 | 20 | 60 |
| CO6 | - | - | - | - | 20 | - | 20 |
|  |  |  |  |  |  |  | 180 |

| # Course Code | # 19ME1003 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # ENGINEERING MECHANICS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) (Answer all the questions) |  |  |  |  |  |
| 1. | Discuss parallelogram law of forces. |  | CO1 | U | 1 |
| 2. | State the principle of transmissibility of forces. |  | CO1 | R | 1 |
| 3. | Discuss about Varignon’s theorem. |  | CO2 | U | 1 |
| 4. | Define moment of a force. |  | CO2 | U | 1 |
| 5. | The moment of inertia (Ixx) of a triangle of width (b) and height (h) is given by _____________. |  | CO3 | R | 1 |
| 6. | If the area of a section is in mm2 and the distance of the centre of area from a lines is in mm, then units of the moment of inertia of the section about the line is expressed in _____________. |  | CO3 | R | 1 |
| 7. | Define the term ‘Time of flight’ of a projectile. |  | CO4 | U | 1 |
| 8. | State Newton’s second law of motion. |  | CO4 | U | 1 |
| 9. | Name the types of impacts when two bodies collide with each other. |  | CO5 | R | 1 |
| 10. | Illustrate about coefficient of friction. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) (Answer all the questions) |  |  |  |  |  |
| 11. | Differentiate between collinear and concurrent force system. |  | CO1 | U | 3 |
| 12. | State Lami’s Theorem. |  | CO2 | R | 3 |
| 13. | Discuss on the parallel axis theorem. |  | CO3 | U | 3 |
| 14. | A car starts from rest and moves with a constant acceleration of 1.2 m/s2. Determine its velocity, after it has travelled for 60 meters. |  | CO4 | A | 3 |
| 15. | Discuss on Impulse and momentum method. |  | CO5 | U | 3 |
| 16. | Describe about the laws of friction. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | The following forces act at a point : (i) 20 N inclined at 30° towards North of East. (ii) 25 N towards North. (iii) 30 N towards North West and (iv) 35 N inclined at 40° towards South of West. Find the magnitude and direction of the resultant force. | CO1 | A | 12 |
| 18. |  | Two spheres, each of weight 1000 N and of radius 25 cm rest in a horizontal channel of width 90 cm as shown in the figure. Find the reactions on the points of contact A, B and C. | CO2 | An | 12 |
| 19. |  | An I-section is made up of three rectangles as shown in Fig. Find the moment of inertia of the section about the horizontal axis passing through the centre of gravity of the section. | CO3 | An | 12 |
| 20. | a. | A car moves along a straight line whose equation of motion is given by s = 12t + 3t2 – 2t3, where (s) is in metres and (t) is in seconds. calculate (i) velocity and acceleration at start, and (ii) acceleration, when the velocity is zero. | CO4 | A | 6 |
|  | b. | A car of mass 2.5 tonnes moves on a level road under the action of 1 kN propelling force. Find the time taken by the car to increase its velocity from 36 km / h to 54 km / h. | CO4 | A | 6 |
| 21. | a. | A ball of mass 1 kg moving with a velocity of 2 m/s impinges directly on a ball of mass 2 kg at rest. The first ball, after impinging, comes to rest. Find the velocity of the second ball after the impact and the coefficient of restitution. | CO5 | A | 6 |
|  | b. | Find the acceleration of a solid body A of mass 10 kg, when it is being pulled by another body B of mass 5 kg along a smooth horizontal plane as shown in figure. Also find the tension in the string, assuming the string to be inextensible. Take g = 9.8 m/s2. | CO5 | An | 6 |
| 22. |  | Calculate the reactions at the supports of the overhanging beam shown in figure. | CO2 | An | 12 |
| 23. |  | Two blocks A and B of weight 80 N and 60 N are connected by a string, passing through a smooth pulley, as shown in the figure. Find the acceleration of the body and the tension in the string. Use work-energy method. | CO5 | An | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | A body of weight 500 N is lying on a rough plane inclined at an angle of 25° with the horizontal. It is supported by an effort (P) parallel to the plane as shown in figure. Determine the minimum and maximum values of P, for which the equilibrium can exist, if the angle of friction is 20°. | CO6 | An | 12 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Determine the resultant force and moment for a given system of forces. |
| CO2 | Understand basics of equilibrium of rigid bodies. |
| CO3 | Determine the centroid and second moment of area of simple solids. |
| CO4 | Apply fundamental concepts of kinematics and kinetics to the analysis of simple / practical problems. |
| CO5 | Understand basic kinematics concepts – displacement, velocity and acceleration. |
| CO6 | Determine friction and its effects as per the laws of friction. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | R | U | A | An | E | C | Total |
| CO1 | 1 | 4 | 12 | - | - | - | 17 |
| CO2 | 3 | 2 | - | 24 | - | - | 29 |
| CO3 | 2 | 3 | - | 12 | - | - | 17 |
| CO4 | - | 2 | 15 | - | - | - | 17 |
| CO5 | 1 | 3 | 6 | 18 | - | - | 28 |
| CO6 | - | 4 | - | 12 | - | - | 16 |
|  |  |  |  |  |  |  | 124 |

| # Course Code | # 19ME2020 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # DRONE TECHNOLOGY | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) (Answer all the questions) |  |  |  |  |  |
| 1. | List the advantages of unmanned aircraft systems. |  | CO1 | U | 1 |
| 2. | List the applications of UAV. |  | CO1 | R | 1 |
| 3. | What is meant by Moment of a couple? |  | CO1 | U | 1 |
| 4. | Define Force and Moment. |  | CO1 | R | 1 |
| 5. | Define Packaging density. |  | CO2 | U | 1 |
| 6. | What is the principle of operation of the rotary engine? |  | CO2 | R | 1 |
| 7. | List two basic types of payloads. |  | CO3 | U | 1 |
| 8. | List the sources of loss of communication during operations. |  | CO4 | R | 1 |
| 9. | List the subsystems incorporated by mission planning and control station. |  | CO4 | U | 1 |
| 10. | State the paramilitary roles of Unmanned Aircraft Systems. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) (Answer all the questions) |  |  |  |  |  |
| 11. | Explain environmentally critical roles by unmanned aircrafts. |  | CO1 | U | 3 |
| 12. | What is a free body diagram? Give example. |  | CO1 | U | 3 |
| 13. | Sketch a geometry of an airfoil section and show the directions of lift and drag. |  | CO2 | U | 3 |
| 14. | Briefly describe dispensable payloads. |  | CO3 | U | 3 |
| 15. | Explain the functions of an MPCS. |  | CO4 | U | 3 |
| 16. | Discuss civilian roles of Unmanned Aircraft Systems. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Discuss in detail the classes of UAV Systems. | CO1 | An | 12 |
| 18. |  | Briefly explain the following (i) Lift-induced Drag, (ii) Parasitic Drag and (iii) scale effects. | CO2 | A | 12 |
| 19. | a. | With a neat schematic, explain the working of a gas turbine engine. | CO2 | A | 8 |
|  | b. | Discuss the use of electric motors and batteries for UAVs as a source of propulsion. | CO2 | A | 4 |
| 20. |  | Explain in detail Acoustic signature and Visual signature in the design for stealth. | CO3 | A | 12 |
| 21. |  | Compare Electro-optic payloads and Radar imaging payloads. | CO3 | An | 12 |
| 22. |  | Explain Communication Media and Radio Communication. | CO4 | U | 12 |
| 23. |  | Discuss the roles of unmanned aircraft system in Naval, Army and Air Force. | CO6 | An | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Describe the three design stages involved in the development of unmanned aerial vehicles. | CO1 | An | 12 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the design parameters of UAV systems. |
| CO2 | Understand the aerodynamics and selection of power plants of UAV systems. |
| CO3 | Identify stealth and payload types of UAV systems. |
| CO4 | Analyze the principles of communication and control station systems used in UAV’s. |
| CO5 | Design launch and recovery systems of UAV’s. |
| CO6 | Apply the application of UAS for various applications. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | R | U | A | An | E | C | Total |
| CO1 | 2 | 8 |  | 24 |  |  | 34 |
| CO2 | 1 | 4 | 24 |  |  |  | 29 |
| CO3 |  | 4 | 12 | 12 |  |  | 28 |
| CO4 | 1 | 16 |  |  |  |  | 17 |
| CO5 |  |  |  |  |  |  | -- |
| CO6 | 1 | 3 |  | 12 |  |  | 16 |
|  |  |  |  |  |  |  | 124 |

| # Course Code | # 19ME2025 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # THERMODYNAMICS | # Max. Marks | # 100 |

USE OF STEAM TABLES AND PSYCHOMETRIC CHART IS PERMITTED

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) (Answer all the questions) |  |  |  |  |  |
| 1. | Write the difference between a closed system and an open system. |  | CO1 | R | 1 |
| 2. | Why does free expansion have zero work transfer? |  | CO1 | U | 1 |
| 3. | Define ‘steady flow process’. |  | CO2 | R | 1 |
| 4. | Write the entropy principle. |  | CO2 | U | 1 |
| 5. | Define vapor pressure. |  | CO3 | R | 1 |
| 6. | Write the critical pressure and temperature of water. |  | CO3 | R | 1 |
| 7. | Write Avagadro’s law. |  | CO4 | U | 1 |
| 8. | State Dalton’s law of partial pressures. |  | CO4 | U | 1 |
| 9. | Define dew point temperature. |  | CO5 | R | 1 |
| 10. | Write the expression for calculating the efficiency of Otto cycle. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) (Answer all the questions) |  |  |  |  |  |
| 11. | Distinguish between the terms ‘change of state’, ‘path’, and ‘process’. |  | CO1 | U | 3 |
| 12. | Define internal energy. How is energy stored in molecules and atoms? |  | CO2 | U | 3 |
| 13. | Draw the phase equilibrium diagram for a pure substance on p-T coordinates. |  | CO3 | A | 3 |
| 14. | Show that the enthalpy of an ideal gas is a function of temperature only. |  | CO4 | A | 3 |
| 15. | Explain the process of heating and humidification. |  | CO5 | U | 3 |
| 16. | State the four processes of a diesel cycle and represent them on T-s plot. |  | CO6 | A | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Derive the expression for pdV work in polytropic process. | CO1 | A | 8 |
|  | b. | Show that work is a path function and not a property. | CO1 | A | 4 |
| 18. |  | In a steady flow apparatus, 135 kJ of work is done by each kg of fluid. The specific volume of the fluid, pressure and velocity at the inlet are 0.37 m3/kg, 600 kPa, and 16 m/s. The inlet is 32 m above the floor, and the discharge pipe is at the floor level. The discharge conditions are 0.62 m3/kg, 100 kPa, and 270 m/s. The total heat loss between the inlet and discharge is 9 kJ/kg of fluid. In flowing through this apparatus, does the specific internal energy increase or decrease, and by how much? | CO2 | An | 12 |
| 19. |  | Explain the phase equilibrium diagram for a pure substance on a T-s plot with relevant constant property lines. | CO3 | A | 12 |
| 20. |  | A fluid at 200 kPa and 300C has a volume of 0.8 m3. In a frictionless process at constant volume the pressure changes to 100 kPa. Find the final temperature and heat transferred (a) if fluid is air, and (b) if the fluid is steam. | CO4 | An | 12 |
| 21. |  | Air at 20C, 40% RH is mixed adiabatically with air at 40C, 40% RH in the ratio of 1 kg of former with 2 kg of the latter (on dry basis). Find the final condition of air. | CO5 | An | 12 |
| 22. |  | Explain the Carnot heat engine cycle executed by (a) a stationary system, and (b) a steady flow system. | CO2 | A | 12 |
| 23. |  | Steam initially at 0.3 MPa, 250C is cooled at constant volume. (a) At what temperature will the steam become saturated vapor? (b) What is the quality at 80C? What is the heat transferred per kg of steam in cooling from 250C to 80C? | CO3 | An | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Explain air standard cycle of SI engine and derive the expression for thermal efficiency and net work output. | CO6 | A | 12 |

CO – COURSE OUTCOME			BL – BLOOMS’ LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the basic concepts in thermodynamics and energy balance to systems and control volumes, in situations involving heat and work interactions. |
| CO2 | Differentiate between high grade and low-grade energies. |
| CO3 | Evaluate changes in thermodynamic properties of pure substances. |
| CO4 | Apply gas laws to solve problems related to gas mixtures. |
| CO5 | Create psychrometric chart to perform moist air process calculations |
| CO6 | Recognize the significance of I law for reacting systems and heating value of fuels. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | R | U | A | An | E | C | Total |
| CO1 | 1 | 4 | 12 | - | - | - | 17 |
| CO2 | 1 | 4 | 12 | 12 | - | - | 29 |
| CO3 | 2 | - | 15 | 12 | - | - | 29 |
| CO4 | - | 2 | 3 | 12 | - | - | 17 |
| CO5 | 1 | 3 | - | 12 | - | - | 16 |
| CO6 | - | 1 | 15 | - | - | - | 16 |
|  |  |  |  |  |  |  | 124 |

| # Course Code | # 19ME2026 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # APPLIED THERMODYNAMICS | # Max. Marks | # 100 |

THE USE OF STEAM TABLES IS PERMITTED.

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) (Answer all the questions) |  |  |  |  |  |
| 1. | Define equivalent evaporation. |  | CO1 | R | 1 |
| 2. | List the advantages of high-pressure boilers. |  | CO1 | U | 1 |
| 3. | Sketch the Rankine cycle on which steam prime movers work. |  | CO2 | U | 1 |
| 4. | Express the formula for Rankine cycle efficiency. |  | CO2 | A | 1 |
| 5. | Interpret the importance of the divergent part in a nozzle. |  | CO3 | U | 1 |
| 6. | State velocity compounded steam turbine. |  | CO3 | R | 1 |
| 7. | Describe the clearance ratio in an Air Compressor. |  | CO4 | U | 1 |
| 8. | Explain the importance of an intercooler in a multi-stage Air compressor. |  | CO4 | U | 1 |
| 9. | Write the important applications of Refrigeration. |  | CO5 | U | 1 |
| 10. | Express the equation considering friction for the velocity of steam flowing through the nozzle. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) (Answer all the questions) |  |  |  |  |  |
| 11. | Analyze the heat losses which occur in a boiler plant. |  | CO1 | U | 3 |
| 12. | Distinguish between a Rankine cycle and a Carnot cycle. |  | CO2 | R | 3 |
| 13. | Establish the effect of friction on the steam flow through the nozzle and determine its efficiency. |  | CO3 | U | 3 |
| 14. | Construct the equation of work without clearance for a single-stage air compressor. |  | CO4 | A | 3 |
| 15. | Discuss the coefficient of performance of a refrigeration system. |  | CO5 | U | 3 |
| 16. | Evaluate the inlet blade angle from a composite superimposed diagram. |  | CO6 | A | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | A steam generator evaporates 18000 kg/h of steam at 12.5 bar and a quality of 0.97 from feed water at 105°C, when coal is fired at the rate of 2040 kg/h. If the higher calorific value of the coal is 27400 kJ/kg, find: (i) The heat rate of boiler in kJ/h; (ii) The equivalent evaporation; (iii) The thermal efficiency. | CO1 | A | 8 |
|  | b. | Explain the advantages of water tube boilers and fire tube boilers. | CO1 | U | 4 |
| 18. | a. | In a steam power cycle, the steam supply is at 15 bar and dry and saturated. The condenser pressure is 0.4 bar. Calculate the Carnot and Rankine efficiencies of the cycle. Neglect pump work. | CO2 | A | 8 |
|  | b. | Explain the modified Rankine cycle with the reheating condition of operation. | CO2 | U | 4 |
| 19. | a. | The velocity of steam from the nozzle is 1000m/s and nozzle angle is 20° and the mean blade velocity is 400m/s, Note inlet, and outlet blade angles are equal. The mass of the steam flowing through the turbine per hour is 1000kg. Calculate i) blade angles ii) the relative velocity of steam entering the blades and outlet. iii) tangential force on the blade iv) power developed v) blade efficiency. Balade velocity co. efficient is 0.8. | CO3 | A | 8 |
|  | b. | Estimate the diagram power from a superimposed diagram.. | CO3 | A | 4 |
| 20. | a. | Illustrate with a neat sketch the working principle of a reciprocating air compressor. | CO4 | U | 8 |
|  | b. | A single stage single acting compressor has to compress air isentropically from 1 bar and 303 K to 5 bar. Find the work of compression required for unit mass flow rate of air. | CO4 | A | 4 |
| 21. | a. | Explain the working principle of a vapour compression refrigeration system with a neat sketch. | CO5 | U | 12 |
| 22. | a. | Explain different types of refrigerants and their properties. | CO1 | U | 8 |
|  | b. | Determine the volumetric efficiency of an air compressor. | CO2 | U | 4 |
| 23. | a. | Examine the discharge through the nozzle and conditions for maximum value. | CO3 | U | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Explain with a neat sketch the pressure-compounded steam turbine working principle. | CO6 | U | 8 |
|  | b. | Represent the metastable expansion of steam in a nozzle with a neat sketch. | CO6 | U | 4 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Estimate the performance of a steam generator. |
| CO2 | Carry out analysis of vapour power cycles. |
| CO3 | Conduct analysis of steam nozzles and turbines. |
| CO4 | Evaluate the performance of reciprocating compressors. |
| CO5 | Apply principles of refrigeration and air conditioning for analysis and performance evaluation. |
| CO6 | Design turbine and nozzles and compressors. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | R | U | A | An | E | C | Total |
| CO1 | 1 | 16 | 8 |  |  |  | 25 |
| CO2 | 3 | 9 | 9 |  |  |  | 21 |
| CO3 | 1 | 16 | 12 |  |  |  | 29 |
| CO4 |  | 10 | 7 |  |  |  | 17 |
| CO5 |  | 16 |  |  |  |  | 16 |
| CO6 |  | 13 | 3 |  |  |  | 16 |
|  |  |  |  |  |  |  | 124 |

| # Course Code | # 19ME3001 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # ADDITIVE MANUFACTURING AND APPLICATIONS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (4 X 20 = 80 MARKS) (Answer all the Questions) |  |  |  |  |  |
| 1. | a. | Define Additive Manufacturing (AM) and discuss the current industrial scenario of this manufacturing technology in product development cycle. | CO1 | A | 10 |
|  | b. | Draw a flow chart depicting the product development cycle and the process flow. | CO1 | An | 10 |
|  |  | (OR) |  |  |  |
| 2. | a. | Compare and contrast the applications of virtual prototype and physical prototype. Articulate the merits and demerits of each technique. | CO1 | A | 10 |
|  | b. | Justify the application of AM in tooling industry. Describe the economic aspects of employing AM to tooling. | CO1 | E | 10 |
| 3. | a. | Classify the digitization techniques available for reverse engineering a product, represent it with a block diagram. | CO2 | R | 10 |
|  | b. | Explain the importance of data formats in CAD tools. How do they impact the CAD industry? | CO2 | E | 10 |
|  |  | (OR) |  |  |  |
| 4. | a. | Enumerate the advantage of adaptive slicing with respect to direct slicing. | CO2 | U | 10 |
|  | b. | Articulate the need for support structures in AM. How does the support structures affect the properties of built part? | CO2 | E | 10 |
| 5. | a. | Differentiate the types of light sources used in liquid-based AM processes. With a neat sketch distinguish the light sources used in Stereolithography Apparatus (SLA) process and Solid Ground Curing (SGC) process. | CO3 | An | 10 |
|  | b. | Estimate the applications of Fused Deposition Modelling (FDM) and Laminated Object Modelling (LOM). Justify the applications with suitable examples for both processes. | CO3 | C | 10 |
|  |  | (OR) |  |  |  |
| 6. | a. | Evaluate with a use case analysis, the merits and demerits of Z Corp’s 3D printer (3DP) over Selective Laser Sintering (SLS) technique. | CO4 | E | 10 |
|  | b. | Draw the construction of LENS process and list the process variable that influence the quality of parts made through LENS process. | CO4 | R | 10 |
| 7. | a. | Explain requirement for pre-processing and post-processing activities in AM. Mention the processes that are involved in pre-processing and post processing of a part manufactured using any one of the metal-based additive manufacturing techniques. | CO5 | A | 20 |
|  |  | (OR) |  |  |  |
| 8. | a. | With an industrial case study, articulate the necessity for a binder-based powder processing technology in AM. | CO5 | E | 10 |
|  | b. | Enumerate the critical factors that influence the performance and functions of Electron Beam Melting. Justify the answer with an example. | CO5 | A | 10 |
| PART – B (1 X 20 = 20 MARKS) COMPULSORY QUESTION |  |  |  |  |  |
| 9. | a. | Describe a commercial application and articulate the use of any one of the AM technology in Recreational Industry. | CO6 | A | 10 |
|  | b. | Where is AM applied in arts and architecture? List few applications. | CO6 | A | 10 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Recognize the need for additive manufacturing in the modern industry. |
| CO2 | Illustrate techniques for processing of CAD models for additive manufacturing. |
| CO3 | Summarize the importance of additive manufacturing in product development cycle. |
| CO4 | Articulate the construction, working principles and process variables of various additive manufacturing technologies. |
| CO5 | Correlate the selection of appropriate additive manufacturing techniques and product development. |
| CO6 | Design additive manufacturing process to solve real time industrial problems. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | R | U | A | An | E | C | Total |
| CO1 |  |  | 20 | 10 | 10 |  | 40 |
| CO2 | 10 | 10 |  |  | 20 |  | 40 |
| CO3 |  |  |  | 10 |  | 10 | 20 |
| CO4 | 10 |  |  |  | 10 |  | 20 |
| CO5 |  |  | 30 |  | 10 |  | 40 |
| CO6 |  |  | 20 |  |  |  | 20 |
|  |  |  |  |  |  |  | 180 |

| # Course Code | # 20ME1006 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # PROFESSIONAL ETHICS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) (Answer all the questions) |  |  |  |  |  |
| 1. | The general and abstract concepts of right and wrong behaviour from philosophy, theology, and professional societies are called ………………. |  | CO1 | U | 1 |
| 2. | Define Engineering Ethics. |  | CO1 | R | 1 |
| 3. | Differentiate ‘bribe’ and ‘gift’. |  | CO2 | U | 1 |
| 4. | Define ‘copyright’. |  | CO2 | U | 1 |
| 5. | List the senses of Responsibility. |  | CO3 | R | 1 |
| 6. | Name a tool that identifies the text overlapping in reports. |  | CO3 | R | 1 |
| 7. | It is essential to have a good ____________ to be successful in your career. |  | CO4 | U | 1 |
| 8. | Give the meaning of the bibliography. |  | CO4 | U | 1 |
| 9. | ## Illustrate the term risk assessment. |  | CO5 | U | 1 |
| 10. | ## A written statement of policies that guides the behaviour of all employees is called …….. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) (Answer all the questions) |  |  |  |  |  |
| 11. | Distinguish between ‘morality’ and ‘ethics’. |  | CO1 | U | 3 |
| 12. | Define conflict of interest. |  | CO2 | R | 3 |
| 13. | Explain respect for others. |  | CO3 | U | 3 |
| 14. | Summarise the primary responsibilities of an editor. |  | CO4 | A | 3 |
| 15. | State the benefit of risk analysis. |  | CO5 | R | 3 |
| 16. | Write short notes on employee misconduct form. |  | CO6 | A | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Explain in detail the various framework of ethics and the contrast among them. | CO1 | U | 12 |
| 18. |  | Discuss the common types of conflict of interest exists in the business world with necessary examples. | CO2 | U | 12 |
| 19. | a. | Discuss the role of engineering societies. | CO3 | R | 4 |
|  | b. | Explain in detail the effectiveness of society activities. | CO3 | U | 8 |
| 20. |  | Explain in detail the publication and communication process of engineering literature. | CO4 | R | 12 |
| 21. | a. | Define risk-benefit analysis, and discuss how to reduce the risk in general. | CO5 | A | 6 |
|  | b. | Discuss in detail the employee rights. | CO5 | A | 6 |
| 22. |  | List and elaborate on the unethical behaviours among individuals. | CO6 | R | 12 |
| 23. |  | What are intellectual property rights, and explain their types. | CO2 | U | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Discuss the unethical behaviours of professionals and politicians. | CO6 | U | 12 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the ethical framework in professional life. |
| CO2 | Know the psychology and philosophy of ethics. |
| CO3 | Recognize the ethics in scientific and engineering society. |
| CO4 | Diagnose the code of ethics and ethical standards. |
| CO5 | Understand the integrity in research. |
| CO6 | Realize the Enforcement of Code of Ethics. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | R | U | A | An | E | C | Total |
| CO1 | 1 | 16 | - | - | - | - | 17 |
| CO2 | 3 | 26 | - | - | - | - | 29 |
| CO3 | 6 | 11 | - | - | - | - | 17 |
| CO4 | 12 | 2 | 3 | - | - | - | 17 |
| CO5 | 3 | 1 | 12 | - | - | - | 16 |
| CO6 | 13 | 12 | 3 | - | - | - | 28 |
|  |  |  |  |  |  |  | 124 |

| # Course Code | # 20ME1007 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # 3D PRINTING TECHNOLOGY | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) (Answer all the questions) |  |  |  |  |  |
| 1. | Enumerate the advantages of 3D printing technology. |  | CO1 | R | 1 |
| 2. | Define Digitization. |  | CO2 | R | 1 |
| 3. | Draw a flow chart and classify the liquid-based 3D printing technologies. |  | CO3 | U | 1 |
| 4. | __________ is the file format used in 3D printers. |  | CO2 | U | 1 |
| 5. | Mention any one major difference between Laminated Object Modelling and Kira’s paper laminate technology. |  | CO4 | U | 1 |
| 6. | State why inert atmosphere required is for metal 3D printers. |  | CO5 | U | 1 |
| 7. | Name the 3D printing technologies that can manufacture multi-colored products. |  | CO5 | U | 1 |
| 8. | Differentiate between soft tooling, hard tooling and bridge tooling. |  | CO6 | U | 1 |
| 9. | Write the startegy required to encounter a crack generated on a turbine blade. |  | CO6 | E | 1 |
| 10. | Justify the application of Laser Engineered Net Shaping to manufacture gold jewelry. |  | CO5 | C | 1 |
| PART – B (6 X 3 = 18 MARKS) (Answer all the questions) |  |  |  |  |  |
| 11. | Differentiate fused deposition modeling and wire arc additive manufacturing. |  | CO4 | U | 3 |
| 12. | Make a neat sketch of Solid Ground Curing Process and mark the parts. |  | CO3 | A | 3 |
| 13. | With an example, illustrate any one application of Indirect Tooling. |  | CO6 | A | 3 |
| 14. | Justify the use of Laser Engineered Net Shaping Process over a Direct Metal Laser Sintering Process. |  | CO5 | E | 3 |
| 15. | How effectively 3D printing technology influence the medical equipment and medical surgery fields? |  | CO6 | E | 3 |
| 16. | Develop a possible solution to solve unemployment through 3D printing technology. |  | CO1 | C | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Discuss the impact of 3D printing technology in manufacturing. | CO1 | U | 6 |
|  | b. | With a flow diagram classify the 3D printing technologies that are available in the market. | CO1 | A | 6 |
| 18. | a. | Defend the application of reverse engineering in new product development. Mention the legal implications while reverse engineering a part/product. | CO2 | E | 6 |
|  | b. | Enumerate the digitization techniques available in the industries. Explain the working principle of the non-contact digitization technique that is used in the KITS laboratory. | CO2 | U | 6 |
| 19. | a. | Explain the application of Solid Ground Curing technology with respect to the materials used and parts produced. | CO3 | U | 6 |
|  | b. | In the event of a non-availability for pointed light source, which alternate technology can be used for 3D printing. Justify the statement with the working principle of the proposed technology. | CO3 | C | 6 |
| 20. | a. | Enumerate the merits and demerits of Laminated Object Modelling. | CO4 | A | 6 |
|  | b. | Explain how the process variables will influence the quality of the product in Fused Deposition Modelling (FDM). List the process variable to check while printing a part in FDM machine. | CO5 | U | 6 |
| 21. |  | Critically evaluate the use of Z-Corp’s 3D Printing technology over Selective Laser Sintering technology. Compare the merits and demerits. | CO5 | E | 12 |
| 22. |  | Draw a block diagram to list the application and specific examples of the 3D printing technology in the fields given below: - Mechanical - Automobile - Aerospace - Medical - Recreational Toys - Sports - Construction | CO4 | U | 12 |
| 23. |  | Distinguish between pre-processing and post-processing in additive manufacturing. Mention the process that are involved in pre-processing and post processing of a part manufactured using any one of the metal-based additive manufacturing techniques. | CO6 | U | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Explain with a neat sketch the construction, working principle, advantages, disadvantages, applications and process variables of Wire Arc Additive Manufacturing Technique. | CO3 | R | 12 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Conceptualize the product development cycle and identify the role of 3D Printing in industries. |
| CO2 | Illustrate appropriate 3D Printing techniques for developing products. |
| CO3 | Articulate the working principles of various 3D Printing Technologies. |
| CO4 | Identify suitable applications for every classification of 3D Printing Technology. |
| CO5 | Correlate the process variables with the quality of products built using 3D Printing Processes. |
| CO6 | Design materials for 3D Printing Process to solve real time industrial problems. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | R | U | A | An | E | C | Total |
| CO1 | 1 | 6 | 6 |  |  | 3 | 16 |
| CO2 | 1 | 7 |  |  | 6 |  | 14 |
| CO3 | 12 | 7 | 3 |  |  | 6 | 28 |
| CO4 |  | 16 | 6 |  |  |  | 22 |
| CO5 |  | 8 |  |  | 15 | 1 | 24 |
| CO6 |  | 13 | 3 |  | 4 |  | 20 |
|  |  |  |  |  |  |  | 124 |

| # Course Code | # 20ME2002 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # CNC PROGRAMMING | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | What is name of NC control system used for machining the profile shown in figure? |  | CO1 | U | 1 |
| 2. | The punched tape reader in NC is __________. |  | CO1 | R | 1 |
| 3. | Following error is proportional to the ___________. |  | CO2 | R | 1 |
| 4. | During transient state, larger __________ results in faster response. |  | CO2 | R | 1 |
| 5. | _______ interpolation method, in which interpolation is carried out using a computer program |  | CO3 | R | 1 |
| 6. | Which software interpolation method requires high memory storage? |  | CO3 | R | 1 |
| 7. | Define Position error. |  | CO5 | R | 1 |
| 8. | The adaptive control module plays the role of increasing _____ and decreasing machining time to increase productivity. |  | CO5 | R | 1 |
| 9. | What is dwell? |  | CO6 | R | 1 |
| 10. | M – codes are used to specify _________ functions in the program. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Brief the role of computers in computer assisted part programming. |  | CO1 | U | 3 |
| 12. | What are the advantages of servo motor compensation? |  | CO2 | U | 3 |
| 13. | Draw and brief the concept of interpolator, if a tool should move from point P1 to P2 at feed rate Vf in the XY plane |  | CO3 | U | 3 |
| 14. | List the advantages of PLC |  | CO4 | R | 3 |
| 15. | Why do CNC programmers offer canned cycles? |  | CO5 | U | 3 |
| 16. | How is the tool-length compensation specified in a Machining centre? |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No 17 to 23, Q. No 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | How does the structure of NC/CNC machine tools differ from conventional machine tools? | CO1 | U | 5 |
|  | b. | Explain utilization of CNC machines in batch production. | CO1 | U | 7 |
| 18. | a. | With a block diagram, explain the control systems used in CNC Machine tools. | CO2 | U | 8 |
|  | b. | Why the recirculating ball screw is used in the CNC Machine tools. | CO2 | U | 4 |
| 19. |  | Analyse how the circular interpolation is executed using Stairs Approximation Algorithm. | CO3 | An | 12 |
| 20. |  | Explain the functions of Programmable logic control and its architecture. | CO4 | U | 12 |
| 21. |  | Analyse the position response in CNC using PID controller. | CO5 | An | 12 |
| 22. |  | Explain the feed and spindle functions in CNC. | CO6 | U | 12 |
| 23. | a. | Construct the CNC program for the below shaft. | CO6 | A | 7 |
|  | b. | Develop the CNC program for the shown outer profile for the depth of 5 mm in the 10 mm thick block. Take the speed as 1500 rpm and feed as 100 mm/min. | CO6 | A | 5 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Explain the servo control for positioning. | CO5 | U | 12 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the control systems for CNC machine tool and select the components of CNC architecture. |
| CO2 | Articulate the principles of motors, Feedback devices and hydraulic system. |
| CO3 | Compare the interpolation methods in CNC control system. |
| CO4 | Propose the PLC programming Languages. |
| CO5 | Recommend PID controllers, servo controller, Numerical control Kernel types. |
| CO6 | Design and evaluate CNC programming techniques for various industrial applications. |

| Assessment Pattern as per Bloom’s Level |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 1 | 16 |  |  |  |  | 17 |
| CO2 | 2 | 15 |  |  |  |  | 17 |
| CO3 | 2 | 3 |  | 12 |  |  | 17 |
| CO4 | 3 | 12 |  |  |  |  | 15 |
| CO5 | 2 | 15 |  | 12 |  |  | 29 |
| CO6 | 2 | 15 | 12 |  |  |  | 29 |
|  |  |  |  |  |  |  | 124 |

| # Course Code | # 20ME2004 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # DESIGN OF MEDICAL DEVICES AND IMPLANTS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | ‘Design’ word is derived from __________________ Latin word, which means “to mark out, point out, describe, contrive.” |  | CO1 | R | 1 |
| 2. | _____________ need, is the need is not to produce something new but to produce something similar. |  | CO1 | R | 1 |
| 3. | Define Biocompatibility. |  | CO2 | U | 1 |
| 4. | Define dental implant. |  | CO2 | U | 1 |
| 5. | __________________ is a prosthetic device that is implanted into the body to replace the biological heart. |  | CO3 | R | 1 |
| 6. | Define brainstorming. |  | CO3 | R | 1 |
| 7. | _________________ strength is defined as the ratio of a compressive load applied by a UTM machine to the cross-sectional area of the material. |  | CO4 | U | 1 |
| 8. | _____________ is the ratio of decrease in length to original length under compression stress. |  | CO4 | U | 1 |
| 9. | The polymer compounds that can be found naturally in the environment are known as_____________polymers. |  | CO5 | U | 1 |
| 10. | ______________implants are artificial mechanical devices that are mounted to the skeletal system of the human body for various purposes, such as supporting bone, replacing bones or joints, and reattaching tendons or ligaments. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | List a minimum of ten characteristics that influences the design. |  | CO1 | R | 3 |
| 12. | Identify in-vitro and in-vivo methods from the list of test methods for the evaluation of adhesion property measurements. |  | CO2 | U | 3 |
| 13. | Discuss briefly, an Artificial Heart according to W.J. Kolff's group of the Cleveland Clinic. |  | CO3 | U | 3 |
| 14. | Illustrative the parts of the musculoskeletal system. |  | CO4 | U | 3 |
| 15. | List four major degradation mechanisms for polymers. |  | CO5 | R | 3 |
| 16. | #### Summarize the event symbols used in fault tree analysis. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.No 17 to 23, Q.No 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Criticize the life cycle of medical devices and distinguish the design process verses the design control. | CO1 | U | 6 |
|  | b. | Distinguish the bathtub and the sequential bathtub design life cycle according to Joseph Edward Shigley. | CO1 | U | 6 |
| 18. | a. | Elaborate the Classification of the implants with respect to various criteria. | CO2 | U | 8 |
|  | b. | Discuss the gold standard implant material, titanium with its historical developments, composition, merits, and demerits. | CO2 | U | 4 |
| 19. | a. | Inspect the features of soft tissues and discuss tissue engineering. | CO3 | U | 6 |
|  | b. | Survey the history of implants and classify the implants depending on various criteria | CO3 | U | 6 |
| 20. | a. | Develop the flow chart for the general processing history of a typical metallic implant device for a total hip replacement Implant. | CO4 | U | 6 |
|  | b. | Assess the biocompatibility and its need with the classification of biological tests to be conducted for a device used for implantation. | CO4 | E | 6 |
| 21. | a. | Describe the various functions of Cartilage. | CO4 | U | 6 |
|  | b. | Differentiate between Tendon and Ligament. | CO4 | E | 6 |
| 22. | a. | Classify biodegradable polymers used in medical devices. | CO5 | U | 6 |
|  | b. | Identify the differences between natural polymers from synthetic polymers with their merits, demerits and applications in the medical field. | CO5 | U | 6 |
| 23. | a. | Assess the process of Fault Tree Analysis (FTA) with an example. | CO6 | An | 8 |
|  | b. | List the merits and demerits of Fault Tree Analysis. | CO6 | U | 4 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Justify the importance of the following to be considered for the design of medical instruments or devices: - Biocompatibility tests. - Hemocompatibility. - Biodegradation. - Sterility tests. - Mechanical tests | CO6 | U | 12 |

CO – COURSE OUTCOME			BL – BLOOM’S LEVEL

| COURSE OUTCOMES |  |
| --- | --- |
| The student will be able to |  |
| CO1 | Understand the class of medical devices and design cycle. |
| CO2 | Apply the design process and different design models. |
| CO3 | Evaluate the design procedures. |
| CO4 | Have in-depth knowledge about blood interfacing implants. |
| CO5 | Evaluate the design quality and realization. |
| CO6 | Test the design and evaluate the risk management involved in the design of a new medical device. |

| Assessment Pattern as per Bloom’s Level |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 17 | - |  |  |  |  | 17 |
| CO2 | - | 17 |  |  |  |  | 17 |
| CO3 | 2 | 15 |  |  |  |  | 17 |
| CO4 | - | 17 |  |  | 12 | - | 29 |
| CO5 | 3 | 13 |  |  |  | - | 16 |
| CO6 | - | 20 |  | 8 |  | - | 28 |
|  |  |  |  |  |  |  | 124 |

| # Course Code | # 20ME2007 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # AUTOMATION OF PRODUCT LIFE CYCLE MANAGEMENT | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Sketch a circuit diagram showing the fundamentals of automation systems. |  | CO1 | R | 1 |
| 2. | ________is a type of computer system that is meant to gather and quickly analyze real-time data. |  | CO1 | R | 1 |
| 3. | Mention the various levels of industrial automation. |  | CO2 | R | 1 |
| 4. | List some of the part manufacturing attributes used in GT. |  | CO2 | R | 1 |
| 5. | One of the primary reasons for ________________is to reduce the worker fatigue or effort. |  | CO3 | R | 1 |
| 6. | Preventing non-conformance components or parts from shipping saves a huge amount of _________ and _______. |  | CO3 | R | 1 |
| 7. | A common strategy for solving any complex task is to decompose it into __________units that are easier to manage. |  | CO4 | R | 1 |
| 8. | Sketch neatly the steps in functional decomposition. |  | CO4 | R | 1 |
| 9. | List out the attributes considered in a successful design. |  | CO5 | R | 1 |
| 10. | Depict a tree diagram for an IC engine using the first level of functional decomposition. |  | CO5 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | List out a few significant applications of the Industrial Internet of Things. |  | CO1 | U | 3 |
| 12. | Sketch neatly the basic elements of the automation system and write their importance. |  | CO2 | U | 3 |
| 13. | State the three unique structures used in classification and coding schemes. |  | CO3 | U | 3 |
| 14. | List the important benefits of GT if implemented in the industry. |  | CO4 | U | 3 |
| 15. | List any four important reasons for implementing automation in the industries. |  | CO5 | U | 3 |
| 16. | Sketch a diagram depicting the three types of automation relative to production quantity and product variety |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.No 17 to 23, Q.No 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Tabulate the following: (1) Reasons for implementing manufacturing cells and (2) Important design considerations that should be taken into consideration during cell formation. | CO1 | A | 6 |
|  | b. | Explain the process adapted by Mitrofanov and Edwards on the concept of the composite part that is formed by merging the primitives of all the parts of a part family. | CO1 | A | 6 |
| 18. | a. | Enumerate the features and benefits of the ERP systems in detail with a neat sketch. | CO2 | An | 6 |
|  | b. | Sketch a neat flowchart that depicts the automation of self-driving cars using various levels of the automation system and explain its functions, merits, and demerits. | CO2 | An | 6 |
| 19. | a. | Tabulate the flow classes depicting the standard flow classes & member flow types and explain their significance in product design, | CO3 | An | 6 |
|  | b. | List and explain the seven important DFX methodologies used in concurrent engineering. | CO3 | An | 6 |
| 20. | a. | Sketch neatly the reversed triangle of the ‘4R-1D’ waste management concept and write its significance. | CO4 | An | 6 |
|  | b. | Enumerate the benefits of the steps involved in a paper recycling plant using a neat sketch. | CO4 | An | 6 |
| 21. | a. | Distinguish between sequential and concurrent engineering and suggest the best methodology to bring out a product in a shorter lead time. | CO5 | An | 6 |
|  | b. | Describe the steps and benefits involved in a glass recycling plant through a process cycle sketch. | CO5 | An | 6 |
| 22. | a. | Depict a functional decomposition tree model for a bike fender /mudguard. | CO3 | E | 6 |
|  | b. | Describe the steps involved in a plastic recycling plant using a neat sketch. | CO3 | E |  |
| 23. | a. | Explain in detail, the concept of FMEA and tabulate examples of the failures in mechanical engineering components. | CO4 | E | 6 |
|  | b. | How the FMEA methodology aims to accomplish the three important things. Describe the process and its uses. | CO4 | E | 6 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | List out five significant conceptual steps of the Product data specifications. Describe the implications involved in the following steps: (i) Social, political, and legal requirements (ii) Manufacturing Specifications. | CO6 | C | 6 |
|  | b. | Case Study: Explain the necessary steps involved in PDS for designing and manufacturing of a Compact Disc Jewel Case after the Problem Description and Need Identification is completed. | CO6 | C | 6 |

CO – COURSE OUTCOME				BL – BLOOMS’ LEVEL

|  | COURSE OUTCOMES |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CO1 | Explain various strategies and technologies adapted in industrial automation. |  |  |  |  |  |  |  |
| CO2 | Select appropriate evaluation methods used in the automation. |  |  |  |  |  |  |  |
| CO3 | Apply modern tools like AI, ANN and Fuzzy logic in the building of automation systems. |  |  |  |  |  |  |  |
| CO4 | Apply the concept of New Product Development and its structuring. |  |  |  |  |  |  |  |
| CO5 | Analyse the virtual product development. |  |  |  |  |  |  |  |
| CO6 | Develop new product development, product structure and supporting systems. |  |  |  |  |  |  |  |
| Assessment Pattern as per Bloom’s Level |  |  |  |  |  |  |  |  |
| CO / P |  | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 |  | 2 | 3 | 12 |  |  |  | 17 |
| CO2 |  | 2 | 3 |  | 12 |  |  | 17 |
| CO3 |  | 2 | 3 |  | 12 |  |  | 17 |
| CO4 |  | 2 | 3 |  |  | 12 |  | 17 |
| CO5 |  | 2 | 3 |  | 12 |  |  | 17 |
| CO6 |  |  | 3 |  |  |  | 12 | 15 |
|  |  |  |  |  |  |  |  | 100 |

| # Course Code | # 20ME2009 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # INTELLIGENT ROBOTIC SYSTEM | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | "Robot" comes from the word called ______________. |  | CO1 | U | 1 |
| 2. | Identify the part that is connected to the manipulator. |  | CO1 | A | 1 |
| 3. | The Robot manipulator is designed to perform a task in________ space. |  | CO2 | A | 1 |
| 4. | Define SHAKEY. |  | CO2 | R | 1 |
| 5. | The function of Navigation is to move the robot to a ______ location. |  | CO3 | An | 1 |
| 6. | Agents interact with other agents via some kind of agent communication language is known as ________. |  | CO3 | A | 1 |
| 7. | List the number of tasks involved in the classical Artificial Intelligence Approach to Robot Control. |  | CO4 | An | 1 |
| 8. | Identify the number of principles involved in the hybrid architecture for the interfacing strategy. |  | CO4 | A | 1 |
| 9. | The first computer orientation method to reduce the dimension of the unconstrained dynamic equation is________________. |  | CO5 | U | 1 |
| 10. | Define the sizing of the DC motor. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Discuss any four robot components. |  | CO1 | R | 3 |
| 12. | Illustrate the types of possible orientation angles for the Wrist motion. |  | CO2 | U | 3 |
| 13. | Recall the Land-Based Navigation. |  | CO3 | U | 3 |
| 14. | Explain the assumptions in a hybrid architecture. |  | CO4 | R | 3 |
| 15. | Classify the types of model learning approaches. |  | CO5 | U | 3 |
| 16. | List the robot kinematics with suitable examples. |  | CO6 | R | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.No 17 to 23, Q.No 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Illustrate with neat sketches and examples about Agent-Task Model. | CO1 | An | 12 |
| 18. | a. | Outline "STRIPS" with suitable sketches. | CO2 | U | 6 |
|  | b. | Explain the robot manipulator kinematics with suitable sketches. | CO2 | R | 6 |
| 19. |  | Explain the closed-loop feedback system for linear and non-linear control manipulators with neat sketches. | CO3 | A | 12 |
| 20. | a. | Illustrate the subsumption architecture with suitable examples. | CO4 | U | 6 |
|  | b. | Construct the detailed step-by-step procedural hybrid architecture for the Autonomous Robot Application. | CO4 | A | 6 |
| 21. |  | Analyze the model-based approaches and elaborate it with neat sketches. | CO5 | An | 12 |
| 22. |  | Construct the AVR Architecture in detail with suitable sketches. | CO1 | A | 12 |
| 23. |  | Outline in detail the Markov and Partial observable Markov Processes with suitable examples. | CO4 | An | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Explain the following 1. Actuators 2. Robot Locomotion 3. Power supply 4. Legged and Wheeled Locomotion | CO6 | An | 12 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Design, build and program simple autonomous robots. |
| CO2 | Implement standard signal processing and control algorithms. |
| CO3 | Describe and analyze robot processes using appropriate methods. |
| CO4 | Solve simple control problems by hand using appropriate methods. |
| CO5 | Write a detailed report on a robot project. |
| CO6 | Carry out and write up investigation using appropriate experimental methods. |

| Assessment Pattern as per Bloom’s Level |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 3 | 1 | 13 | 12 | - | - | 29 |
| CO2 | 7 | 9 | 1 | - | - | - | 17 |
| CO3 | - | 3 | 13 | 1 | - | - | 17 |
| CO4 | 3 | 6 | 7 | 13 | - | - | 29 |
| CO5 | - | 4 | - | 12 | - | - | 16 |
| CO6 | 1 | 3 | - | 12 | - | - | 16 |
|  |  |  |  |  |  |  | 124 |

| # Course Code | # 20ME2010 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # KINEMATICS AND DYNAMICS OF MACHINERY | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) (Answer all the questions) |  |  |  |  |  |
| 1. | Mention the different types of joints by type of contact. |  | CO1 | U | 1 |
| 2. | The lead screw of a lathe with nut forms a _________ pair. |  | CO1 | R | 1 |
| 3. | Write the expression for the radial component of acceleration. |  | CO2 | A | 1 |
| 4. | The direction of linear velocity of any point on a link with respect to another point on the same link is ---------- to the link joining the points. |  | CO2 | U | 1 |
| 5. | Write the factors for selecting the belt drive. |  | CO3 | U | 1 |
| 6. | Illuminate the expression for velocity ratio of belt drive if slip is considered. |  | CO3 | R | 1 |
| 7. | The radial distance of a tooth from the pitch circle to the bottom of the tooth is called -----------. |  | CO4 | U | 1 |
| 8. | List the different types of gear trains. |  | CO4 | U | 1 |
| 9. | The height of a Watt’s governor (in metres) in equal to ________. |  | CO5 | U | 1 |
| 10. | The engine of an aeroplane rotates in clockwise direction when seen from the tail end and the aeroplane takes a turn to the left. The effect of the gyroscopic couple on the aeroplane will be ____________. |  | CO5 | A | 1 |
| PART – B (6 X 3 = 18 MARKS) (Answer all the questions) |  |  |  |  |  |
| 11. | State Pantograph with suitable diagram. |  | CO1 | U | 3 |
| 12. | Write the conditions of the equilibrium of member with two forces and a torque, and show in the figure. |  | CO2 | A | 3 |
| 13. | Illustrate the expression for length of an open flat belt drive with schematic diagram. |  | CO3 | A | 3 |
| 14. | List out the advantages and disadvantages of gear drive. |  | CO4 | U | 3 |
| 15. | With neat diagram, explain the effect of the Gyroscopic Couple on a ship. |  | CO5 | A | 3 |
| 16. | Explain the necessity of balancing of rotating parts for high speed engines. |  | CO6 | A | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Determine the mobility (degrees of freedom) of the mechanism shown in Figure below using Kutzbach mobility criterion and classify them. | CO1 | A | 12 |
| 18. |  | The crank of a slider crank mechanism rotates clockwise at a constant speed of 300 r.p.m. The crank is 150 mm and the connecting rod is 600 mm long. Determine 1) Linear velocity and acceleration of the midpoint of the connecting rod, and 2) angular velocity and angular acceleration of the connecting rod, at a crank angle of 45° from inner dead centre position. | CO2 | An | 12 |
| 19. | a. | A shaft which rotates at a constant speed of 160 r.p.m. is connected by belting to a parallel shaft 720 mm apart, which has to run at 60, 80 and 100 r.p.m. The smallest pulley on the driving shaft is 40 mm in radius. Determine the remaining radii of the two stepped pulleys for a crossed belt. Neglect belt thickness and slip. | CO3 | E | 9 |
|  | b. | Write the expression for length of Chain drive. | CO3 | U | 3 |
| 20. |  | In an epicyclic gear train, an arm carries two gears A and B having 36 and 45 teeth respectively. If the arm rotates at 150 r.p.m. in the anticlockwise direction about the centre of the gear A which is fixed, determine the speed of gear B. If the gear A instead of being fixed, makes 300 r.p.m. in the clockwise direction, what will be the speed of gear B? | CO4 | An | 12 |
| 21. |  | A cam is to give the following motion to a knife-edged follower : 1) Outstroke during 60° of cam rotation; 2) Dwell for the next 30° of cam rotation ; 3) Return stroke during next 60° of cam rotation, and 4) Dwell for the remaining 210° of cam rotation. The stroke of the follower is 40 mm and the minimum radius of the cam is 50 mm. The follower moves with uniform velocity during both the outstroke and return strokes. Draw the profile of the cam when the axis of the follower passes through the axis of the cam shaft. | CO5 | E | 12 |
| 22. |  | A governor of the Proell type has each arm 250 mm long. The pivots of the upper and lower arms are 25 mm from the axis. The central load acting on the sleeve has a mass of 25 kg and the each rotating ball has a mass of 3.2 kg. When the governor sleeve is in mid-position, the extension link of the lower arm is vertical and the radius of the path of rotation of the masses is 175 mm. The vertical height of the governor is 200 mm. If the governor speed is 160 r.p.m. when in mid-position, find 1) length of the extension link; and 2) tension in the upper arm. | CO5 | E | 12 |
| 23. |  | A chain drive is used for reduction of speed from 250 r.p.m. to 130 r.p.m. The number of teeth on the driving sprocket is 20. Find the number of teeth on the driven sprocket. If the pitch circle diameter of the driven sprocket is 500 mm and centre to centre distance between the two sprockets is 700 mm, determine the pitch and length of the chain. | CO3 | An | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Define, in short, free vibrations, forced vibrations and damped vibrations. | CO6 | U | 6 |
|  | b. | A shaft 50 mm diameter and 3 metres long is simply supported at the ends and carries three loads of 1000 N, 1500 N and 750 N at 1 m, 2 m and 2.5 m from the left support. The Young's modulus for shaft material is 200 GN/m2. Find the frequency of transverse vibration. | CO6 | An | 6 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the basic concepts of Mechanisms, Machines and their relative motions, then apply it to appropriate environments. |
| CO2 | Carry out kinematic analysis (Displacement, Velocity and Acceleration) of simple mechanisms (Single slider Crank Mechanism and four bar Mechanisms) by graphical and analytical method. |
| CO3 | Construct & Design different CAM profiles for given conditions using graphical & Theoretical methods. |
| CO4 | Apply the concept of balancing and use it for reducing the unbalanced forces in rotating masses and reciprocating engines under operating conditions exposure to IS standards. |
| CO5 | Acquire knowledge on types of vibrations in different systems and damping methods to minimize vibrations. |
| CO6 | Understand, apply and analyze the control mechanisms in Governors and Gyroscopes. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | R | U | A | An | E | C | Total |
| CO1 | 1 | 4 | 12 |  |  |  | 17 |
| CO2 |  | 1 | 4 | 12 |  |  | 17 |
| CO3 | 1 | 4 | 3 | 12 | 9 |  | 29 |
| CO4 |  | 5 |  | 12 |  |  | 17 |
| CO5 |  | 1 | 4 |  | 24 |  | 29 |
| CO6 |  | 6 | 3 | 6 |  |  | 15 |
|  |  |  |  |  |  |  | 124 |

| # Course Code | # 20ME2011 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # FINITE ELEMENT METHODS IN ENGINEERING | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Define Discretization. |  | CO1 | Understand | 1 |
| 2. | List the stages in finite element analysis according to computer implementations. |  | CO6 | Analyze | 1 |
| 3. | Define aspect ratio. |  | CO1 | Remember | 1 |
| 4. | List the functions of node. |  | CO1 | Remember | 1 |
| 5. | Write the types of Boundary condition in FEA. |  | CO3 | Understand | 1 |
| 6. | Mention the application of FEA in Mechanical engineering. |  | CO5 | Apply | 1 |
| 7. | Write the purpose of Pascal’s Triangle. |  | CO2 | Understand | 1 |
| 8. | What do you mean by scalar field problem? |  | CO3 | Remember | 1 |
| 9. | Define Lagrangian element. |  | CO2 | Understand | 1 |
| 10. | Define spatial isotropy. |  | CO1 | Understand | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Differentiate between boundary value problem and initial value problem. |  | CO1 | Understand | 3 |
| 12. | Write the purpose of Grid Independence study in FEA. |  | CO4 | Analyze | 3 |
| 13. | Why linear triangular element is often called as constant strain triangular (CST) element? |  | CO3 | Understand | 3 |
| 14. | List the properties of stiffness matrix. |  | CO2 | Remember | 3 |
| 15. | Define higher order element. |  | CO1 | Understand | 3 |
| 16. | State Darcy’s Law. |  | CO5 | Remember | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.No 17 to 23, Q.No 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Discuss the factors to be considered in Finite Element Analysis during the process of discretization. | CO1 | Remember | 6 |
|  | b. | With example, explain Convergence criteria. | CO1 | Understand | 6 |
| 18. |  | A stepped bar is subjected to an axial load of 200kN at the place of change of cross section is shown in Figure 1. Find (a) nodal displacements (b) reaction forces (c) induced stress in each material. (1)-Aluminium bar (2)-steel bar A1=2400mm2 A2=600mm2 E1=7x104N/mm2 E2=2x105N/mm2 300 mm 400 mm (1)-Aluminium Bar (2)-Steel Bar 200kN Figure1 | CO5 | Evaluate | 12 |
| 19. |  | The following differential equation is available for a physical phenomenon d2y /dx2 + 50 = 0, 0 ≤ x ≤ 10; Trial function is y = a1 (10x – x2); Boundary conditions are y(0) = 0 and y(10)=0. Find the value of parameter a1 using the following methods: a. Point collocation b. Least square method c. Sub domain method d. Galerkin’s method | CO3 | Analyze | 12 |
| 20. |  | Determine three points on the 50oC contour line for the rectangular element shown in Figure2. The nodal values are T1=42oC, T2= 54oC,T3= 56oC and T4= 46oC. Nodal coordinates are node1 (5,3), node2 (8,3), node 3(8,5) and node4( 5,5). S x Y 2(8,3) 1(5,3) 3 (8,5) 4(5,5) t Figure2 | CO4 | Evaluate | 12 |
| 21. |  | A wall of an industrial oven consists of three different materials. The first layer is composed of 5 cm of insulating cement with a clay binder that has a thermal conductivity of 0.08 W/m K. The second layer is made from 15 cm of asbestos board with a thermal conductivity of 0.074 W/m K. The exterior consists of 10 cm common brick with a thermal conductivity of 0.72 W/m K. The inside (inlet) wall temperature of the oven is 200°C, and the outside air is 30°C with a convection coefficient of 40 W/m 2 K(convective heat loss occurs at the right side of brick material). Determine the temperature distribution along the composite wall. | CO5 | Apply | 12 |
| 22. | a. | Derive the shape functions for 1D simplex element. | CO2 | Create | 8 |
|  | b. | With example, explain the properties of Shape function. | CO1 | Remember | 4 |
| 23. |  | Derive the shape functions for Nine nodded rectangular element. | CO2 | Create | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | For the smooth pipe of variable cross section shown in Figure 3, determine the potential at the junctions, velocities in each section of the pipe and the volumetric flow rate. The potential at the left end is p1=10m and that at the right end is p4=2m. Permeability coefficient k=1m/s. 1 m 1 m 1 m A1=3m2 A2=2m2 A3=1m2 1 2 3 4 Figure 3. | CO6 | Apply | 12 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Acquire the fundamental theory of finite element analysis and develop characteristic equation. |
| CO2 | Derive element matrix equation by applying basic laws in mechanics and integration by parts |
| CO3 | Apply suitable boundary conditions to a global equation for field problems |
| CO4 | Analyse scalar and vector variable problems |
| CO5 | Understand the application and use FE method for solving heat transfer, fluid mechanics and structural problems |
| CO6 | Use professional level finite element software to solve engineering problems |

| Assessment Pattern as per Bloom’s Level |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 12 | 14 | - | - | - | - | 26 |
| CO2 | 3 | 2 | - | - | - | 20 | 25 |
| CO3 | 1 | 4 | - | 12 | - | - | 17 |
| CO4 | - | - | - | 3 | 12 | - | 15 |
| CO5 | 3 | - | 13 | - | 12 | - | 28 |
| CO6 | - | - | 12 | 1 | - | - | 13 |
|  |  |  |  |  |  |  | 124 |

| # Course Code | # 20ME2012 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # INTERNET OF THINGS FOR MECHANICAL SYSTEMS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) (Answer all the questions) |  |  |  |  |  |
| 1. | Mention a software used for visualization of data. |  | CO1 | R | 1 |
| 2. | Smart products can analyze big data and perform computations through software. (True/False). |  | CO1 | A | 1 |
| 3. | Name one area of research where High Performance Computing is used. |  | CO2 | R | 1 |
| 4. | The V of Big Data, which defines different formats of data from various sources, is termed as _____________. |  | CO2 | R | 1 |
| 5. | The ability of an organization to reduce waste in time, effort and materials as much as possible, while still producing a high-quality service or product is called _________________ |  | CO3 | R | 1 |
| 6. | Tip of the extruder where hot material comes out in a 3D printer is termed as print bed. (True/False) |  | CO3 | U | 1 |
| 7. | ________________ is designed to intelligently analyze data to automate key supply chain processes and supporting capabilities. |  | CO4 | R | 1 |
| 8. | Mention any one application of head mounted displays. |  | CO4 | A | 1 |
| 9. | List any 6 types of cybercrime. |  | CO5 | R | 1 |
| 10. | BIM model cannot contain information on design, construction, logistics, operation, maintenance, budgets and schedules. (True/False) |  | CO5 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) (Answer all the questions) |  |  |  |  |  |
| 11. | List two functions of the gateway in IoT. |  | CO1 | R | 3 |
| 12. | Compare structured and unstructured data. |  | CO2 | U | 3 |
| 13. | Give any 2 examples of industries who have adapted predictive maintenance and explain the predictive maintenance used by them. |  | CO3 | U | 3 |
| 14. | Sketch the enablers for synchronized planning. |  | CO4 | R | 3 |
| 15. | Contrast AI and Autonomy. |  | CO5 | U | 3 |
| 16. | Classify the types of electrical vehicle charging. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Summarize the working and classification of IoT with respect to networking, communication and connectivity protocols with suitable examples used in the market today. | CO1 | U | 12 |
| 18. |  | State the laws of robotics and examine the application of robots in any five domains with their benefits. | CO2 | U | 12 |
| 19. | a. | With the help of a neat diagram, explain the components of supply chain management. | CO3 | U | 6 |
|  | b. | Elaborate on the implantation of the intelligent supply chain management. | CO3 | U | 6 |
| 20. |  | Define Cyber physical systems and elaborate on five applications of cyber physical systems with IoT. | CO4 | U | 12 |
| 21. |  | Classify the resource constraints and explain any six-resource constraints with appropriate examples. | CO5 | U | 12 |
| 22. | a. | Define Big data. | CO2 | U | 3 |
|  | b. | Outline the steps to be followed to deploy a Big Data solution. | CO2 | U | 9 |
| 23. | a. | Differentiate Augmented Reality and Virtual Reality. | CO4 | U | 4 |
|  | b. | Mention in details two main applications of virtual reality in manufacturing. | CO4 | U | 8 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Justify the reason for the condition monitoring for industrial drives. | CO6 | U | 4 |
|  | b. | Explain any four technologies used for the condition monitoring for industrial drives. | CO6 | U | 8 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Interpret the Essentials of IoT for Modern Engineers. |
| CO2 | Examine the importance of Smart and Digital Factories. |
| CO3 | Make use of IoT in Manufacturing Process and Applications. |
| CO4 | Model IoT for Cyber-Physical Systems, Virtual Reality and Data Analytics. |
| CO5 | Interpret the IoT Challenges in Mechanical Systems. |
| CO6 | Apply IoT concepts in various applications. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | R | U | A | An | E | C | Total |
| CO1 | 4 | 12 | 1 |  |  | - | 17 |
| CO2 | 2 | 27 |  |  |  | - | 29 |
| CO3 | 1 | 16 |  |  |  | - | 17 |
| CO4 | 4 | 24 | 1 |  |  | - | 29 |
| CO5 | 1 | 16 |  |  |  | - | 17 |
| CO6 |  | 15 |  |  |  | - | 15 |
|  |  |  |  |  |  |  | 124 |

| # Course Code | # 20ME2014 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # INDUSTRIAL SAFETY AND QUALITY STANDARDS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) (Answer all the questions) |  |  |  |  |  |
| 1. | OEM stands for ____________. |  | CO1 | R | 1 |
| 2. | Mention any one protective tool used in welding process. |  | CO1 | U | 1 |
| 3. | The point at which equipment has been safely isolated from any chances of re-energization or release of residual energy is called as ____________. |  | CO2 | U | 1 |
| 4. | Give an example of protective tool for painting process. |  | CO3 | R | 1 |
| 5. | Define standard deviation. |  | CO5 | U | 1 |
| 6. | ____________ and ____________ are types of control charts. |  | CO4 | R | 1 |
| 7. | FMECA stands for ____________. |  | CO5 | R | 1 |
| 8. | Quality is the ratio between ____________ and ____________. |  | CO5 | U | 1 |
| 9. | Define reliability. |  | CO6 | R | 1 |
| 10. | DMAIC stands for _________ |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) (Answer all the questions) |  |  |  |  |  |
| 11. | Mention any five safety rules to be followed in a Production Industries. |  | CO1 | U | 3 |
| 12. | List different types of guarding techniques. |  | CO2 | R | 3 |
| 13. | What is meant by Risk Priority Number (RPN)? |  | CO3 | R | 3 |
| 14. | Define quality. What are the different dimensions of quality? |  | CO4 | U | 3 |
| 15. | Define MTBF and MTTF. |  | CO5 | R | 3 |
| 16. | Briefly write about bench marking. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | What are the responsibility and objective of safety management? | CO1 | U | 6 |
|  | b. | Explain in detail the functions of safety engineer. | CO1 | U | 6 |
| 18. | a. | Mention briefly about the safety measures to be followed in Metal Working Machinery. | CO1 | A | 4 |
|  | b. | With respect to manure manufacturing industries, explain the safety and protocols principles to be followed during Maintenance. | CO1 | An | 8 |
| 19. | a. | Mention the safety precautions to be taken into account in material handling quoting suitable examples with respect to chemical industries | CO2 | U | 8 |
|  | b. | What is the basic principle of machine guarding? | CO2 | U | 4 |
| 20. | a. | Imagine that you are working for NLC Power plant as an application engineer. What are the safety precautions will you make when you are deployed in Boiler division where the working temperature is above 800℃. | CO2 | An | 8 |
|  | b. | What are the different techniques available for risk measurements? | CO2 | U | 4 |
| 21. | a. | Mention seven traditional tools of quality. Explain any four tools in detail quoting relevant examples on your own. | CO3 | C | 8 |
|  | b. | What is process capability ratio? Explain the significance of it. | CO3 | U | 4 |
| 22. | a. | Explain in detail about series, parallel and mixed configurations and their relevance in assembling plant. Choose examples on your own | CO4 | C | 8 |
|  | b. | Briefly explain about Fault Tree Analysis. | CO4 | U | 4 |
| 23. |  | Consider yourself as a quality engineer and construct the house of quality for manufacturing of a toy for children. | CO5 | C | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Mention the quality standards of ISO 9001 : 2015. Evaluate and list the merits and demerits over ISO 14000 quality standards. | CO6 | E | 8 |
|  | b. | Mention and explain the significance of 5S principles. | CO6 | R | 4 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Apply safety principles, protocols and Personnel protective equipments (PPE) to engineering processes. |
| CO2 | Assess risk in manufacturing processes in term of Risk Priority Number (RPN), manage and mitigate them . |
| CO3 | Apply quality principles and control charts to maintain quality of the processes and products . |
| CO4 | Appraise quality costs in products and minimize failure and reworks . |
| CO5 | Experimenting failure analysis thereby improve the production process and develop fool proof |
| CO6 | Manufacturing processes . |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | R | U | A | An | E | C | Total |
| CO1 | 1 | 16 | 4 | 8 |  |  | 29 |
| CO2 | 4 | 17 |  | 8 |  |  | 29 |
| CO3 | 4 | 4 |  |  |  | 8 | 16 |
| CO4 | 1 | 7 |  |  |  | 8 | 16 |
| CO5 | 4 | 1 |  |  |  | 12 | 17 |
| CO6 | 6 | 3 |  |  | 8 |  | 17 |
|  |  |  |  |  |  |  | 124 |

| # Course Code | # 20ME2016 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # FLUID MECHANICS AND FLUID MACHINES | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) (Answer all the questions) |  |  |  |  |  |
| 1. | Define Dynamic viscosity. |  | CO1 | R | 1 |
| 2. | Relation between Gauge pressure and Absolute pressure. |  | CO1 | U | 1 |
| 3. | Define fluid kinematics. |  | CO2 | R | 1 |
| 4. | Incompressible flow means no change in_________. |  | CO2 | U | 1 |
| 5. | Reynolds number is the _______ ratio. |  | CO3 | R | 1 |
| 6. | The non dimensional number defined with respect to gravitational force is ___________. |  | CO3 | R | 1 |
| 7. | The sum of hydraulic gradient line and ___________ is called Energy gradient line. |  | CO4 | U | 1 |
| 8. | Define the Reynolds number for intermittent flow. |  | CO4 | R | 1 |
| 9. | Recall equation of force (Fx) excreted by a jet on a stationary curved surface. |  | CO5 | R | 1 |
| 10. | The efficiency of jet pump is around __________. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) (Answer all the questions) |  |  |  |  |  |
| 11. | Relate the terms center of gravity and center of buoyancy for stability of a floating body. |  | CO1 | R | 3 |
| 12. | Differentiate between free vortex and forced vortex. |  | CO2 | U | 3 |
| 13. | Define no slip condition in case of boundary layer theorem. |  | CO3 | R | 3 |
| 14. | Define coefficient of discharge and its expression. Write coefficient of discharge for an orifice meter. |  | CO4 | R | 3 |
| 15. | Define Cavitation and list its disadvantages. |  | CO5 | U | 3 |
| 16. | Write the characteristics of Kaplan turbine. |  | CO6 | R | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Distinguish different types of fluid with respect to shear stress and velocity gradient. | CO1 | U | 12 |
| 18. |  | Explain with a neat diagram. - Stream function - Velocity gradient | CO2 | R | 6+6 |
| 19. | a. | Explain in detail about Boundary Layer theorem. | CO3 | R | 9 |
|  | b. | Write the equation for displacement thickness (δ*), momentum thickness (ɵ) and energy thickness (δE). | CO3 | R | 3 |
| 20. |  | Explain the various regimes and characteristics of flow over a flat plate with neat sketch | CO3 | R | 12 |
| 21. | a | Derive expressions for total velocity and total acceleration of fluid particles. | CO4 | R | 8 |
|  | b | Explain flow condition with respect to convective and local acceleration. | CO4 | R | 4 |
| 22. | a | Explain the construction and working of reciprocating pump with neat diagram. | CO5 | R | 9 |
|  | b | Discuss about slip in reciprocating pump. | CO5 | R | 3 |
| 23. |  | Explain the working principle of Kaplan turbine with neat sketches | CO6 | R | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Explain the relation between metacentric height, metacentric radius and center of buoyancy with respect to stability of floating body with help of neat sketches | CO1 | U | 12 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Recognize the important fluid properties and determine forces acting on immersed bodies. |
| CO2 | Solve fluid flow problems using Conservation principles. |
| CO3 | Analyze the characteristics of boundary layer and relationship between different physical quantities of fluid flow. |
| CO4 | Determine rate of flow and calculate flow losses through pipes. |
| CO5 | Evaluate the performance of pumps. |
| CO6 | Evaluate the performance of turbines. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | R | U | A | An | E | C | Total |
| CO1 | 16 | 13 |  |  |  |  | 29 |
| CO2 | 13 | 4 |  |  |  |  | 17 |
| CO3 | 29 |  |  |  |  |  | 29 |
| CO4 | 16 | 10 |  |  |  |  | 17 |
| CO5 | 13 | 3 |  |  |  |  | 16 |
| CO6 | 16 |  |  |  |  |  | 16 |
|  |  |  |  |  |  |  | 124 |

| # Course Code | # 20ME2017 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # AUTOMOTIVE MATERIALS AND ELECTRONICS | # Max. Marks | # 100 |

| Q. No. | Questions |  |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) (Answer all the questions) |  |  |  |  |  |  |
| 1. | Define the term ductility. |  |  | CO1 | R | 1 |
| 2. | The ability of a material to deform under load and retain its new shape when the load is removed is called ______________. |  |  | CO1 | U | 1 |
| 3. | List some applications of grey cast iron material. |  |  | CO2 | R | 1 |
| 4. | Low carbon steel is also known as___________. |  |  | CO2 | U | 1 |
| 5. | The cylinder head is mounted on top of the ______________ block. |  |  | CO3 | U | 1 |
| 6. | Piston is made of _____________material. |  |  | CO3 | U | 1 |
| 7. | Identify the main requirements of fuel injection system. |  |  | CO4 | U | 1 |
| 8. | The______________ battery is used in IC engine. |  |  | CO4 | R | 1 |
| 9. | ________________converts an electrical signal to a physical output. |  |  | CO5 | U | 1 |
| 10. | Define headlight. |  |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) (Answer all the questions) |  |  |  |  |  |  |
| 11. | Classify plain carbon steel. |  |  | CO1 | U | 3 |
| 12. | Distinguish between material limits and material attributes. |  |  | CO2 | U | 3 |
| 13. | List some applications of straight bevel gear. |  |  | CO3 | R | 3 |
| 14. | Distinguish between multiplexing and de-multiplexing wring systems. |  |  | CO4 | U | 3 |
| 15. | Identify the measurement methods for CO, HC and NOx. |  |  | CO5 | U | 3 |
| 16. | Define engine management system. |  |  | CO6 | R | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |  |
| 17. | a. | Describe the different types, properties and application of plain carbon steel. | CO1 |  | U | 8 |
|  | b. | Mention any two Aluminium base alloys and their applications. | CO1 |  | R | 4 |
| 18. | a. | With neat sketches explain the material property of strength-density chart. | CO2 |  | U | 8 |
|  | b. | Define metals. Classify engineering metals. | CO2 |  | R | 4 |
| 19. |  | Discuss briefly the types of gears, manufacturing methods; materials that are used for gear manufacturing give their applications. | CO3 |  | U | 12 |
| 20. | a. | With neat sketches illustrate the working and constructional details CRDI and GDI fuel injection systems. | CO4 |  | U | 10 |
|  | b. | State the main requirements of ignition system. | CO4 |  | R | 2 |
| 21. |  | Write short notes on: (i) Vehicle condition monitoring systems (ii) Electric power steering. | CO5 |  | U | 12 |
| 22. | a. | Explain the composition properties and typical application of any four copper alloys. | CO1 |  | U | 6 |
|  | b. | Write the general mechanical properties of ceramics. | CO1 |  | U | 6 |
| 23. | a. | Illustrate the working method of any six automotive sensors. | CO4 |  | U | 8 |
|  | b. | Distinguish between starter motor and alternator. | CO4 |  | U | 4 |
| COMPULSORY QUESTION |  |  |  |  |  |  |
| 24. | a. | Describe briefly airbags systems. |  | CO6 | U | 6 |
|  | b. | Explain the role of electronics in modern automotive vehicles. |  | CO6 | U | 6 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Familiar with different materials used for automotive component manufacturing. |
| CO2 | Select proper material for Automobile applications. |
| CO3 | Choose a suitable material for selected part of the engine components. |
| CO4 | Know the working of electronic starting and ignition systems. |
| CO5 | Use the instrumentations and electronic controls. |
| CO6 | Understand the engine managements system, lighting and security systems. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | R | U | A | An | E | C | Total |
| CO1 | 5 | 24 | - | - | - | - | 29 |
| CO2 | 5 | 12 | - | - | - | - | 17 |
| CO3 | 3 | 14 | - | - | - | - | 17 |
| CO4 | 3 | 26 | - | - | - | - | 29 |
| CO5 | - | 16 | - | - | - | - | 16 |
| CO6 | 4 | 12 | - | - | - | - | 16 |
|  |  |  |  |  |  |  | 124 |

| # Course Code | # 20ME3003 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # INTERNET OF THINGS IN MANUFACTURING | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (4 X 20 = 80 MARKS) (Answer all the Questions) |  |  |  |  |  |
| 1. | a. | Describe various steps involved in IoT for completing a manufacturing Process. | CO1 | A | 20 |
|  |  | (OR) |  |  |  |
| 2. | a. | With a neat sketch describe the hardware & software component of 3D printer used in manufacturing process | CO2 | An | 20 |
| 3. | a. | Designate the 4 primary forms of values in terms of loT & manufacturing processes. | CO3 | A | 20 |
|  |  | (OR) |  |  |  |
| 4. | a. | List & brief different applications used in manufacturing industry using Augmented Reality. | CO4 | An | 20 |
| 5. | a. | Explain the Basics of block chain with a block diagram. | CO5 | A | 20 |
|  |  | (OR) |  |  |  |
| 6. | a. | Elucidate different techniques & processes involved in Additive manufacturing. | CO2 | A | 20 |
| 7. | a. | Explicate Internet of Things in terms of Industry 4.0 | CO4 | A | 20 |
|  |  | (OR) |  |  |  |
| 8. | a. | Discuss about different categories of cyber security & explain about security structure. | CO5 | An | 20 |
| PART – B (1 X 20 = 20 MARKS) COMPULSORY QUESTION |  |  |  |  |  |
| 9. | a. | With a neat diagram explain about Digital Twin technology and also describe an application. | CO6 | An | 20 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Interpret the Essentials of IoT for Modern Engineers |
| CO2 | Examine the importance of Smart and Digital Factories |
| CO3 | Make use of IoT in Manufacturing Process and Applications |
| CO4 | Model IoT for Cyber-Physical Systems, Virtual Reality and Data Analytics |
| CO5 | Interpret the IoT Challenges in Mechanical Systems |
| CO6 | Apply IoT concepts in various applications |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | R | U | A | An | E | C | Total |
| CO1 | - | - | 20 | - | - | - | 20 |
| CO2 | - | - | 20 | 20 | - | - | 40 |
| CO3 | - | - | 20 | - | - | - | 20 |
| CO4 | - | - | 20 | 20 | - | - | 40 |
| CO5 | - | - | 20 | 20 | - | - | 40 |
| CO6 | - | - | - | 20 | - | - | 20 |
|  |  |  |  |  |  |  | 180 |

| # Course Code | # 20ME3007 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # AUTOMATED INSPECTION SYSTEMS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (4 X 20 = 80 MARKS) (Answer all the Questions) |  |  |  |  |  |
| 1. | a. | Illustrate the principles of uncertainty in the context of measurements and explain its significance. | CO1 | A | 10 |
|  | b. | Discuss the sources of uncertainty that are typically considered in Type B evaluation and provide examples of each. | CO1 | A | 10 |
|  |  | (OR) |  |  |  |
| 2. | a. | Explain the concept of correlated components of uncertainty and why they arise in measurement processes. | CO1 | A | 10 |
|  | b. | Interpret the differences between (a) Uncertainty and Resolution, and (b) Uncertainty and Conformity. | CO1 | A | 10 |
| 3. | a. | Discuss the importance of high-precision measurements and their application in various fields, such as manufacturing, research, and quality control. | CO2 | A | 10 |
|  | b. | Explain the significance of interfacing in high-precision measurements, including the use of sensors, data acquisition systems, and communication protocols. | CO2 | A | 10 |
|  |  | (OR) |  |  |  |
| 4. | a. | Summarize the concepts of automated visual inspection in manufacturing and its significance in quality control processes. | CO2 | A | 10 |
|  | b. | Enumerate the principles and techniques used in automated visual inspection, including image processing, pattern recognition, and machine learning. | CO2 | A | 10 |
| 5. | a. | Explain how a laser interferometer can be used to accurately measure distances, displacements, and vibrations. | CO3 | An | 10 |
|  | b. | Summarize the applications of laser interferometers used in metrology, optics, and precision manufacturing. | CO3 | An | 10 |
|  |  | (OR) |  |  |  |
| 6. | a. | Explain the concept and working principles of non-contact CMMs that utilize electro-optical sensors for dimensional metrology. | CO4 | An | 10 |
|  | b. | Explain the working principles of non-contact sensors used for surface finish measurements, white light interferometry, confocal microscopy, and laser-based profilometers. | CO4 | An | 10 |
| 7. | a. | Discuss the techniques applied in the Machine Vision: (a) Measurement of length, diameters, and Surface roughness, (b) 3D and dynamic feature extraction. | CO5 | An | 20 |
|  |  | (OR) |  |  |  |
| 8. | a. | Explain the machine vision techniques applied in the image description, including feature extraction, edge detection, and shape analysis. | CO5 | E | 10 |
|  | b. | Discuss a few case studies and applications in food inspection and textile quality control industries where color analysis is crucial. | CO5 | E | 10 |
| PART – B (1 X 20 = 20 MARKS) COMPULSORY QUESTION |  |  |  |  |  |
| 9. | a. | Summarize a few sensor technologies for automated inspection for online quality control in automobile production industries. | CO6 | E | 10 |
|  | b. | Discuss the integration of robotic testing and inspection through robot vision systems in the car body manufacturing industries. | CO6 | E | 10 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the various methods of high-precision measurements and Ultrasonic techniques. |
| CO2 | Operate sophisticated measurement and inspection facilities. |
| CO3 | Assess the methods of laser interferometry, Atomic Force techniques to measure surface topography and interpret the results. |
| CO4 | Apply suitable programming commands to measure the critical features of a component using CMM. |
| CO5 | Select suitable Machine Vision systems for image acquisition, processing, and interpreting the results for online quality control. |
| CO6 | Choose the appropriate automated visual inspection method. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | R | U | A | An | E | C | Total |
| CO1 |  |  | 40 |  |  |  | 40 |
| CO2 |  |  | 40 |  |  |  | 40 |
| CO3 |  |  |  | 20 |  |  | 20 |
| CO4 |  |  |  | 20 |  |  | 20 |
| CO5 |  |  |  | 20 | 20 |  | 40 |
| CO6 |  |  |  |  | 20 |  | 20 |
| Total |  |  |  |  |  |  | 180 |

| # Course Code | # 20ME3009 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # NEW-AGE MATERIALS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (4 X 20 = 80 MARKS) (Answer all the Questions) |  |  |  |  |  |
| 1. |  | Compare the key properties of advanced and new-age materials and how do these properties contribute to their applications in modern engineering? | CO1 | U | 20 |
|  |  | (OR) |  |  |  |
| 2. | a. | Explain the classification of smart materials and provide examples of common smart materials along with their associated stimulus-response characteristics. | CO1 | U | 10 |
|  | b. | Discuss the unique properties and provide examples of common smart materials. How are these properties utilized in various applications, and what benefits do they offer in terms of performance and functionality? | CO1 | U | 10 |
| 3. | a. | Describe the principles of the direct and indirect piezoelectric effect. How are piezoelectric materials utilized in sensors, actuators, biomorphs and energy-harvesting applications? | CO2 | R | 10 |
|  | b. | Enumerate with examples the specific devices or systems where piezoelectric materials are employed, highlighting their functional roles and advantages. | CO2 | R | 10 |
|  |  | (OR) |  |  |  |
| 4. |  | Describe the effects that occur in piezoelectric materials, and what are the mechanisms involved. | CO2 | R | 20 |
| 5. |  | Explain the underlying principles and mechanisms behind the shape memory effects. | CO3 | U | 20 |
|  |  | (OR) |  |  |  |
| 6. |  | Explain the concepts of shape memory effect and super elastic effect in shape memory alloys. | CO3 | U | 20 |
| 7. | a. | Describe the concept and design strategies of chromogenic materials. | CO4 | R | 10 |
|  | b. | Enumerate the principles behind thermochromism, photochromism, electrochromism, halochromism and solvatochromism | CO4 | R | 10 |
|  |  | (OR) |  |  |  |
| 8. |  | Explain the principles and applications of CNT, graphene, metallic glasses and high entropy alloys. | CO4 | R | 20 |
| PART – B (1 X 20 = 20 MARKS) COMPULSORY QUESTION |  |  |  |  |  |
| 9. | a. | Discuss the synthesis, properties, and applications of thermally responsive polymers. How are they used in drug delivery systems and biocompatible materials? | CO5 | U | 10 |
|  | b. | Compare the utilization of smart corrosion protection coatings and self-healing materials in space engineering. | CO6 | U | 10 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the physical principles underlying the behaviour of advanced and new-age materials. |
| CO2 | The basic principles and mechanisms of the stimuli-response for the most important smart materials. |
| CO3 | Propose improvement on the design, analysis and manufacturing of advanced and new-age materials. |
| CO4 | Command on shape memory materials fabrication and shape memory effects. |
| CO5 | Smart polymers and new-age materials usage in space applications. |
| CO6 | Identify the application issued involved in integrating advanced and new-age materials to engineering smart structures and products. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | R | U | A | An | E | C | Total |
| CO1 | - | 40 | - | - | - | - | 40 |
| CO2 | 40 | - | - | - | - | - | 40 |
| CO3 | - | 40 | - | - | - | - | 40 |
| CO4 | 40 | - | - | - | - | - | 40 |
| CO5 | - | 10 | - | - | - | - | 10 |
| CO6 | - | 10 | - | - | - | - | 10 |
|  |  |  |  |  |  |  | 180 |

| # Course Code | # 20ME3010 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # PROCESS SAFETY MANAGEMENT IN INDUSTRY | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (4 X 20 = 80 MARKS) (Answer all the Questions) |  |  |  |  |  |
| 1. |  | Mention the roles of safety officer and safety committee. Explain about different types of safety committee. | CO1 | An | 20 |
|  |  | (OR) |  |  |  |
| 2. |  | Differentiate accidents and incidents. Record and analyse incidents and accidents with examples. | CO2 | An | 20 |
| 3. |  | Elucidate various types of safety inspections with its purpose. | CO3 | A | 20 |
|  |  | (OR) |  |  |  |
| 4. |  | Discuss the major accident investigation techniques and analysis. | CO4 | A | 20 |
| 5. |  | Using the following date calculate the severity rate of accident in an industrial plant where only one accident occurred during the year, the type of injury being cutting of the tip of a figure. - Number of workers=2000. - Number of days lost in a year due to accidents=100 - Average number of hours worked by worker per year=2000. | CO5 | E | 20 |
|  |  | (OR) |  |  |  |
| 6. |  | Explanation with a relevant example on how to compare the safety performance of any two industries. | CO5 | An | 20 |
| 7. |  | With an example, illustrate the various sections of a safety policy. | CO1 | A | 20 |
|  |  | (OR) |  |  |  |
| 8. |  | Explicate with examples, the different types of safety audits. | CO3 | A | 20 |
| PART – B (1 X 20 = 20 MARKS) COMPULSORY QUESTION |  |  |  |  |  |
| 9. |  | Discuss the role of government agencies and private consulting agencies in safety training. | CO6 | An | 20 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the importance of safety. |
| CO2 | Practice ergonomic principles to reduce accidents. |
| CO3 | Apply latest safety techniques |
| CO4 | Investigate and report the causes and remedies of industrial accidents. |
| CO5 | Monitor the safety performance |
| CO6 | Ensure safety through safety education and training. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | R | U | A | An | E | C | Total |
| CO1 | - | - | 20 | 20 | - | - | 40 |
| CO2 | - | - | - | 20 | - | - | 20 |
| CO3 | - | - | 40 | - | - | - | 40 |
| CO4 | - | - | 20 | - | - | - | 20 |
| CO5 | - | - | - | 20 | 20 | - | 40 |
| CO6 | - | - | - | 20 | - | - | 20 |
|  |  |  |  |  |  |  | 180 |

| # Course Code | # 20ME3011 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # MATERIALS AND CHARACTERIZATION TECHNIQUES | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (4 X 20 = 80 MARKS) (Answer all the Questions) |  |  |  |  |  |
| 1. | a. | List the various types of Melting furnaces used in foundry shops. Explain anyone in detail. | CO1 | R | 14 |
|  | b. | Discuss the problems encountered during the melting of steel and cast iron. | CO1 | U | 6 |
|  |  | (OR) |  |  |  |
| 2. | a. | Explain the detailed working process of Cupola furnaces. | CO1 | U | 10 |
|  | b. | Explain the various traditional methods of casting. | CO1 | U | 10 |
| 3. |  | Write the detailed procedure of centrifugal casting; also discuss the various casting defects. | CO2 | A | 20 |
|  |  | (OR) |  |  |  |
| 4. | a. | Explain in detail the stir casting method. | CO2 | U | 10 |
|  | b. | Discuss investment casting. | CO2 | U | 10 |
| 5. |  | Discuss the working procedure of additive manufacturing with a suitable example. | CO3 | U | 20 |
|  |  | (OR) |  |  |  |
| 6. | a. | Explain in detail two-photon polymerization 3d printing. | CO3 | U | 14 |
|  | b. | Describe biodegradable polymers; discuss natural biodegradable polymers. | CO3 | R | 6 |
| 7. | a. | Write the detailed working procedure of light microscopy. | CO4 | A | 14 |
|  | b. | Write the difference between bright-field and dark-field contrast. | CO4 | A | 6 |
|  |  | (OR) |  |  |  |
| 8. |  | Discuss the working procedure of the X-ray diffraction technique; also discuss the procedure to identify crystal structure and crystallite size. | CO5 | U | 20 |
| PART – B (1 X 20 = 20 MARKS) COMPULSORY QUESTION |  |  |  |  |  |
| 9. | a. | Describe Scanning electron microscopy; discuss the detailed working process. | CO6 | R | 10 |
|  | b. | Describe Transmission electron microscopy; explain the working principles. | CO6 | R | 10 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Familiarize the various melting practices. |
| CO2 | Understand the different special casting processes. |
| CO3 | Know the implication of biomaterials and RP techniques. |
| CO4 | Understand the principles of light microscopy and quantity microscopy. |
| CO5 | Appreciate the operating principle of X-Ray Diffraction and Diffractometry. |
| CO6 | Comprehend the operating principle of SEM & TEM. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | R | U | A | An | E | C | Total |
| CO1 | 14 | 26 | - | - | - | - | 40 |
| CO2 | - | 20 | 20 | - | - | - | 40 |
| CO3 | 6 | 34 | - | - | - | - | 40 |
| CO4 | - | - | 20 | - | - | - | 20 |
| CO5 | - | 20 | - | - | - | - | 20 |
| CO6 | 20 | - | - | - | - | - | 20 |
|  |  |  |  |  |  |  | 180 |

| # Course Code | # 20ME3012 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # MATERIALS FRACTURE AND FAILURE ANALYSIS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (4 X 20 = 80 MARKS) (Answer all the Questions) |  |  |  |  |  |
| 1. | a. | Classify the different types of fracture and explain the characteristics of the materials that fall under each category. | CO1 | U | 10 |
|  | b. | Articulate Griffith’s Crack Propagation theory with suitable example. | CO1 | R | 10 |
|  |  | (OR) |  |  |  |
| 2. | a. | Compare and contrast the crack-tip models by Irwin and Dugdale. | CO1 | An | 10 |
|  | b. | Derive and explain the Irwin’s model. | CO1 | A | 10 |
| 3. | a. | Describe the basics of Linear Elastic Fracture Mechanics (LEFM) and the modes of crack extensions with suitable diagrams. | CO2 | A | 10 |
|  | b. | Derive the flaw shape parameters for surface and internal cracks. | CO2 | A | 10 |
|  |  | (OR) |  |  |  |
| 4. | a. | With suitable example, explain the principle of Elastic-plastic fracture mechanics (EPFM). | CO2 | A | 10 |
|  | b. | Distinguish between LEFM and EPFM. | CO2 | R | 10 |
| 5. | a. | How does the mechanical properties are correlated with the metallurgical properties of a material? Justify with a minimum of 2 case studies. | CO3 | An | 10 |
|  | b. | Estimate the role of grain size, secondary phase particles, grain boundary and segregation on the hardness and strength of a material. | CO3 | C | 10 |
|  |  | (OR) |  |  |  |
| 6. | a. | Enumerate the cause and effect of different types of failures with suitable examples. | CO4 | A | 10 |
|  | b. | List the different analytical techniques to determine the chemical composition of a material. | CO4 | R | 10 |
| 7. | a. | Justify the necessity to follow BIS or ASME standards for testing materials. Explain the implications of disregarding those standards. | CO4 | U | 10 |
|  | b. | With a block diagram explain the different types of corrosion in materials. Enumerate the common corrosive environments in which materials are being used in day-to-day applications. | CO5 | A | 10 |
|  |  | (OR) |  |  |  |
| 8. | a. | Define stress corrosion cracking. Summarize the characteristics of stress corrosion cracking. | CO5 | U | 10 |
|  | b. | Evaluate the stress corrosion cracking (SCC) using time-to-failure (TTF) technique. Derive the SCC for crack growth. | CO5 | E | 10 |
| PART – B (1 X 20 = 20 MARKS) COMPULSORY QUESTION |  |  |  |  |  |
| 9. | a. | Define Fatigue Failure. Enumerate the effects of environment of fatigue. Describe the fatigue failure mode with a neat sketch. | CO6 | R | 10 |
|  | b. | With an industrial case study explain the importance of fatigue failure analysis. | CO6 | C | 10 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the mechanics of failure under static loading. |
| CO2 | Apply the techniques of testing of LEFM and EPFM. |
| CO3 | Analyze the microstructural aspects of failure. |
| CO4 | Know the general approach to analysis of failure. |
| CO5 | Examine the environment assisted failures. |
| CO6 | Evaluate the failure analysis of fatigue fracture. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | R | U | A | An | E | C | Total |
| CO1 | 10 | 10 | 10 | 10 |  |  | 40 |
| CO2 | 10 |  | 30 |  |  |  | 40 |
| CO3 |  |  |  | 10 |  | 10 | 20 |
| CO4 | 10 | 10 | 10 |  |  |  | 30 |
| CO5 |  | 10 | 10 |  | 10 |  | 30 |
| CO6 | 10 |  |  |  |  | 10 | 20 |
|  |  |  |  |  |  |  | 180 |

| # Course Code | # 21ME2002 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # STRENGTH OF MATERIALS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Describe the term Young’s modulus. |  | CO1 | R | 1 |
| 2. | State about volumetric strain. |  | CO1 | R | 1 |
| 3. | Tell the different types of loads acting on a beam. |  | CO2 | R | 1 |
| 4. | Describe an overhanging beam. |  | CO2 | R | 1 |
| 5. | Interpret the term neutral axis. |  | CO3 | U | 1 |
| 6. | Write the bending equation. |  | CO3 | A | 1 |
| 7. | Write torsional equation. |  | CO4 | A | 1 |
| 8. | Define the term polar modulus. |  | CO4 | U | 1 |
| 9. | Discuss any one advantage of Macaulay’s method. |  | CO5 | U | 1 |
| 10. | Discuss about equivalent length of a column. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | For a material, Young’s modulus is given as 1.2 x 105 N/mm2 and Poisson’s ratio is 0.25. Calculate the Bulk modulus. |  | CO1 | A | 3 |
| 12. | Sketch the shear force and bending moment diagrams for a cantilever beam of length L carrying a point load of W at the free end. |  | CO2 | A | 3 |
| 13. | Discuss about section modulus for a hollow circular section. (Assume the parameters) |  | CO3 | U | 3 |
| 14. | List the assumptions made in the theory of torsion. |  | CO4 | R | 3 |
| 15. | A thin cylindrical shell of diameter 1.5 m is provided with hemispherical ends. It is subjected to an internal pressure of 2 N/mm2. Determine the thickness of the hemispherical part, if the permissible stress is 100 N/mm2. |  | CO5 | A | 3 |
| 16. | Name the assumptions made in Euler’s column theory. |  | CO6 | R | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.No 17 to 23, Q.No 24 is Compulsory) |  |  |  |  |  |
| 17. |  | A load of 2 MN is applied on a short concrete column 500 mm x 500 mm. The column is reinforced with four steel bars of 10 mm diameter, one in each corner. Predict the stresses in the concrete and steel bars. Take Young’s modulus for steel as 2.1 x 105 N/mm2 and for concrete as 1.4 x 105 N/mm2. | CO1 | A | 12 |
| 18. | a. | Estimate the value of Young’s modulus and Poisson’s ratio of a metallic bar of length 300 mm; breadth 40 mm; depth 40 mm when the bar is subjected to an axial compressive load of 400 kN. The decrease in length is given as 0.75 mm and the increase in breadth is 0.03 mm. | CO1 | An | 06 |
|  | b. | An axial pull of 35 kN is acting on a bar consisting of three lengths as shown in Fig (a). If the Young’s modulus is 2.1 x 105 N/mm2, calculate: (i) stresses in each section and (ii) total extension of the bar. Fig (a) | CO1 | An | 06 |
| 19. |  | A simply supported beam of length 10 m carries a uniformly distributed load and two point loads as shown on the Fig (b). Draw the shear force and bending moment diagram for the beam. Also calculate position and magnitude of the maximum bending moment. Fig (b) | CO2 | An | 12 |
| 20. |  | A cast iron beam is of I-section as shown in Fig (c). The beam is simply supported for a span of 5 m. If the tensile stress is not to exceed 20 N/mm2, analyze the safe uniform load which the beam can carry, and also the maximum compressive stress. Fig (c) | CO3 | An | 12 |
| 21. |  | A solid steel shaft transmits 100 kW at 150 rpm. Calculate the suitable diameter of the shaft if the maximum torque transmitted exceeds the mean by 20% in each revolution. The shear stress is not to exceed 60 MPa. Also predict the maximum angle of twist in a length of 4 m of the shaft. Take G = 80 GPa. | CO4 | A | 12 |
| 22. |  | A beam of length 6 m, simply supported at its ends carries two point loads as shown in the Fig (d). Find the deflection under each load using Macaulay’s method. Take E = 2 x 105 N/mm2 and I = 85 x 106 mm4. Fig (d) | CO5 | An | 12 |
| 23. | a. | Summarize and derive an expression for the Euler’s crippling load for a long column when both ends are hinged. | CO6 | An | 6 |
|  | b. | Calculate the safe compressive load on a hollow cast iron column (one end fixed and other end hinged) of 150 mm external diameter; 100 mm internal diameter and 10 m in length. Use Euler’s formula with a factor of safety of 5 and E = 95 x 103 N/mm2. | CO6 | A | 6 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | A 1.5 m long column has a circular cross-section of 50 mm diameter. One of the ends of the column is fixed and the other end is free. Taking factor of safety as 3, estimate the safe load using: - Rankine’s formula; take yield stress σc = 560 N/mm2 and α = 11600 for pinned ends. - Euler’s formula; Young’s modulus for CI = 1.2 x 105 N/mm2. | CO6 | A | 12 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Demonstrate fundamental knowledge about various types of loading and stresses induced. |
| CO2 | Draw the SFD and BMD for different types of loads and support conditions. |
| CO3 | Analyze the stresses induced in basic mechanical components. |
| CO4 | Estimate the strain energy in mechanical elements. |
| CO5 | Analyze the deflection in beams. |
| CO6 | Evaluate buckling and bending phenomenon in columns, struts and beams. |

| Assessment Pattern as per Bloom’s Level |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 2 | - | 15 | 12 | - | - | 29 |
| CO2 | 2 | - | 3 | 12 | - | - | 17 |
| CO3 | - | 4 | 1 | 12 | - | - | 17 |
| CO4 | 3 | 1 | 13 | - | - | - | 17 |
| CO5 | - | 1 | 3 | 12 | - | - | 16 |
| CO6 | 3 | 1 | - | 18 | 6 | - | 28 |
|  |  |  |  |  |  |  | 124 |

| # Course Code | # 21ME2003 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # DESIGN OF MACHINE ELEMENTS | # Max. Marks | # 100 |

(Use of PSG Data Book and approved data books / materials are permitted)

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) (Answer all the questions) |  |  |  |  |  |
| 1. | Classify the types of engineering design. |  | CO1 | U | 1 |
| 2. | Define design stress. |  | CO2 | R | 1 |
| 3. | Write the type of stresses induced in shaft. |  | CO3 | U | 1 |
| 4. | Distinguish between coupling and clutch. |  | CO3 | U | 1 |
| 5. | List few applications of knuckle joint |  | CO5 | R | 1 |
| 6. | Indicate the appropriate material used to manufacture springs |  | CO3 | U | 1 |
| 7. | Define eccentricity |  | CO2 | R | 1 |
| 8. | Write the purpose of Clutch |  | CO5 | U | 1 |
| 9. | State Rankine’s theory of failure |  | CO2 | R | 1 |
| 10. | Classify the types of brake |  | CO5 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) (Answer all the questions) |  |  |  |  |  |
| 11. | List the factors affecting the machine design process |  | CO1 | R | 3 |
| 12. | Write the methods used to reduce stress concentration. |  | CO2 | U | 3 |
| 13. | Differentiate between cotter joint and knuckle joint |  | CO3 | U | 3 |
| 14. | Write the advantages of V belt drive over flat belt drive |  | CO5 | U | 3 |
| 15. | Identify the factors to be considered for the design of springs. |  | CO1 | U | 3 |
| 16. | Evaluate the diameter of shaft for transmitting a power of 20kW at 1000rpm. Assume allowable shear stress of shaft material as 40MPa. |  | CO3 | A | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | A vertical pillar of 50mm diameter is subjected to a vertical load of 1kN acting eccentrically at a distance of 30mm from the axis. Calculate the maximum and minimum stresses induced in the pillar and locate it | CO1 | A | 12 |
| 18. |  | A bolt is subjected to a tensile load of 18 kN and a shear load of 12 kN. The material has a yield stress of 328.6MPa. Taking factor of safety as 2.5, determine the core diameter of the bolt according to different theories of failure. Take Poisson's ratio as 0.3 | CO2 | A | 12 |
| 19. |  | A circular bar of 500 mm length is supported freely at its two ends. It is acted upon by a central concentrated cyclic load having a minimum value of 20 kN and a maximum value 50 kN. Calculate the diameter of bar by taking a factor of safety of 1.5, size effect factor of 0.85, surface finish factor of 0.9. The material properties of bar are given by: ultimate strength of 650 MPa, yield strength of 500 MPa and endurance strength of 350 MPa. | CO4 | An | 12 |
| 20. |  | A mild steel shaft transmits 20 kW at 200 rpm. It carries a central load of 900 N and is simply supported between the bearings 2.5 meters apart. Evaluate the size of the shaft, if the allowable shear stress is 42 MPa and the maximum tensile or compressive stress is not to exceed 56 MPa. What size of the shaft will be required, if it is subjected to gradually applied loads? | CO3 | E | 12 |
| 21. |  | Design a helical compression spring for a maximum load of 1000 N with a deflection of 25 mm using the value of spring index as 5. The maximum permissible shear stress for spring wire is 420 MPa and modulus of rigidity is 84 kN/mm2. | CO6 | A | 12 |
| 22. |  | Design a sleeve coupling to transmit 30 kW at 960 rpm. The allowable shear stress for the shaft and key is 40 N/mm2 and crushing stress for key is 90 N/mm2. The design shear stress for the coupling is 14 N/mm2. | CO5 | A | 12 |
| 23. |  | Determine the dimensions of I section at mid span of the connecting rod for a petrol engine of bore 110mm, stroke 150mm, speed 1500rpm, length of connecting rod is 325mm and explosion pressure is 2.5N/mm2. Select suitable material. | CO5 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Design a knuckle joint for transmitting an axial load of 60kN. The allowable stresses are 60N/mm2 in tension, 75N/mm2 in crushing and 40N/mm2 in shear. | CO3 | A | 12 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the standard design procedure for design of machine elements. |
| CO2 | Analyze stresses acting on components and determine the size based on theories of failure. |
| CO3 | Design machine components for a given load condition using design data hand book. |
| CO4 | Decide specifications as per standards given in design data and select standard components to improve interchangeability. |
| CO5 | Design and develop non- standard machine components. |
| CO6 | Prepare a detail design layout, drawing and computer coding of machine elements. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | R | U | A | An | E | C | Total |
| CO1 | 3 | 4 | 12 | - | - | - | 19 |
| CO2 | 3 | 3 | 12 | - | - |  | 18 |
| CO3 | - | 6 | 15 |  | 12 | - | 33 |
| CO4 | - | - | - | 12 | - | - | 12 |
| CO5 | 1 | 5 | 24 | - | - | - | 30 |
| CO6 | - | - | 12 | - | - |  | 12 |
|  |  |  |  |  |  |  | 124 |

| # Course Code | # 21ME3010 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # QUALITY CONCEPTS IN DESIGN | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (4 X 20 = 80 MARKS) (Answer all the Questions) |  |  |  |  |  |
| 1. | a. | Explain about the four types of cost of quality. | CO1 | A | 10 |
|  | b. | Describe the basic concepts in Quality engineering and management. | CO1 | U | 10 |
|  |  | (OR) |  |  |  |
| 2. | a. | Justify ‘Quality Function Deployment’ as a tool to improve the quality of a product or service. | CO2 | E | 10 |
|  | b. | Explain the concept of Total Quality Management with suitable examples. | CO3 | A | 10 |
| 3. | a. | Describe about frequency distributions and histograms. | CO1 | U | 10 |
|  | b. | Analyze the sig sigma sustainability with respect to the modern manufacturing management. | CO2 | An | 10 |
|  |  | (OR) |  |  |  |
| 4. | a. | Explain about lean production. Highlight its advantages. | CO2 | A | 10 |
|  | b. | Describe the process of statistical hypothesis testing. | CO3 | U | 10 |
| 5. | a. | Explain the DMAIC process for process improvement with neat sketches. | CO2 | A | 10 |
|  | b. | Describe Process Capability with neat sketches. | CO3 | U | 10 |
|  |  | (OR) |  |  |  |
| 6. | a. | Discuss about the application of variable and attribute control charts. | CO3 | U | 10 |
|  | b. | Explain briefly Gage Reproducibility and Repeatability. | CO3 | A | 10 |
| 7. | a. | Explain the Failure Mode Effect Analysis (FMEA) with a suitable example. | CO4 | A | 10 |
|  | b. | Explain briefly Taguchi methods for robust design. | CO4 | A | 10 |
|  |  | (OR) |  |  |  |
| 8. | a. | Justify the fault tree analysis as an effective tool in quality engineering. | CO4 | E | 10 |
|  | b. | Compare Fractional, Full and Orthogonal Experiments with suitable examples. | CO4 | U | 10 |
| PART – B (1 X 20 = 20 MARKS) COMPULSORY QUESTION |  |  |  |  |  |
| 9. | a. | Discuss the methods to arrive reliability of series, parallel and mixed configuration systems with suitable examples. | CO5 | U | 10 |
|  | b. | Explain briefly the tools to improve the reliability of a system. | CO6 | A | 10 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Apply the basic tools of quality in product development. |
| CO2 | Analyze the basic tools of quality in improving or redesigning the production process. |
| CO3 | Adopt/adept TQM and SPC tools in product/process industries. |
| CO4 | Conduct experiments and to analyze the significance of proceeds parameters. |
| CO5 | Compute reliability of parallel, series and mixed configurations. |
| CO6 | Improve the reliability of the systems by redundancy. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | R | U | A | An | E | C | Total |
| CO1 | - | 20 | 10 | - | - | - | 30 |
| CO2 | - | - | 20 | 10 | 10 | - | 40 |
| CO3 | - | 30 | 20 | - | - | - | 50 |
| CO4 | - | 10 | 20 | - | 10 | - | 40 |
| CO5 | - | 10 | - | - | - | - | 10 |
| CO6 | - | - | 10 | - | - | - | 10 |
|  |  |  |  |  |  |  | 180 |

| # Course Code | # 21ME3017 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # DIGITAL MANUFACTURING | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |  |
| 1. | a. | Articulate the importance of digital twins in manufacturing. Also, Identify and explain where the digital twins can be used in smart factory. | CO1 | A | 10 |
|  | b. | Interpret the impact of digital twins in product life cycle. | CO1 | U | 6 |
| 2. |  | Determine how topology optimization unlocks the design opportunities in digital manufacturing. | CO1 | A | 16 |
| 3. |  | Explain the digital manufacturing with a neat sketch. | CO2 | U | 16 |
| 4. |  | Explain the architecture of Digital Manufacturing System. | CO2 | U | 16 |
| 5. |  | Analyse the position and attitude of a robot, processing tools, measuring head using configuration space theory. | CO3 | An | 16 |
| 6. | a. | Write the application of sensor in processing and tool condition monitoring. | CO4 | A | 12 |
|  | b. | Compare the data mining and information fusion. | CO4 | U | 4 |
| 7. |  | Describe the role of knowledge engineering in product life cycle. | CO5 | U | 16 |
| PART – B (1 X 20 = 20 MARKS) (Compulsory Question) |  |  |  |  |  |
| 8. |  | Analyze the importance of powder structure in Selective Laser Sintering. | CO6 | An | 20 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Learn the Design processes and methods in product development. |
| CO2 | Get a basic knowledge on the importance of digital manufacturing. |
| CO3 | Understand the digital transformation in manufacturing. |
| CO4 | Implement decision knowledge in manufacturing. |
| CO5 | Integrate the digital technologies in product life cycle. |
| CO6 | Know the additive manufacturing technologies used in digital manufacturing. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | R | U | A | An | E | C | Total |
| CO1 |  | 6 | 26 |  |  |  | 32 |
| CO2 |  | 32 |  |  |  |  | 32 |
| CO3 |  |  |  | 16 |  |  | 16 |
| CO4 |  | 4 | 12 |  |  |  | 16 |
| CO5 |  | 16 |  |  |  |  | 16 |
| CO6 |  |  |  | 20 |  |  | 20 |
|  |  |  |  |  |  |  | 132 |

| # Course Code | # 21ME3017 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # DIGITAL MANUFACTURING | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (4 X 20 = 80 MARKS) (Answer all the Questions) |  |  |  |  |  |
| 1. | a. | Articulate the importance of digital twins in manufacturing. Also, Identify and explain where the digital twins can be used in smart factory. | CO1 | A | 12 |
|  | b. | Interpret the impact of digital twins in product life cycle. | CO1 | U | 8 |
| (OR) |  |  |  |  |  |
| 2. |  | Explain verification and validation in relation to FEA. | CO1 | A | 20 |
| 3. |  | Explain the operation reference mode of Digital manufacturing system. | CO2 | U | 20 |
| (OR) |  |  |  |  |  |
| 4. |  | Explain the architecture of Digital Manufacturing System. | CO2 | U | 20 |
| 5. |  | Examine how the geometric modeling and reasoning are useful in intelligent manufacturing. | CO3 | An | 20 |
| (OR) |  |  |  |  |  |
| 6. | a. | Write the application of sensor in processing and tool condition monitoring. | CO4 | A | 14 |
|  | b. | Compare data mining and information fusion. | CO4 | U | 6 |
| 7. |  | Describe the role of knowledge engineering in product life cycle | CO5 | U | 20 |
| (OR) |  |  |  |  |  |
| 8. |  | Develop a plan to protect the property right of network manufacturing product. | CO5 | A | 20 |
| PART – B (1 X 20 = 20 MARKS) (Compulsory Question) |  |  |  |  |  |
| 9. |  | Choose the appropriate process of building an RP of computer mouse made of a polymer and explain how the product will be manufactured. | CO6 | An | 20 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Learn the Design processes and methods in product development. |
| CO2 | Get a basic knowledge on the importance of digital manufacturing. |
| CO3 | Understand the digital transformation in manufacturing. |
| CO4 | Implement decision knowledge in manufacturing. |
| CO5 | Integrate the digital technologies in product life cycle. |
| CO6 | Know the additive manufacturing technologies used in digital manufacturing. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | R | U | A | An | E | C | Total |
| CO1 |  | 8 | 32 |  |  |  | 40 |
| CO2 |  | 40 |  |  |  |  | 40 |
| CO3 |  |  |  | 20 |  |  | 20 |
| CO4 |  | 6 | 14 |  |  |  | 20 |
| CO5 |  | 20 | 20 |  |  |  | 40 |
| CO6 |  |  |  | 20 |  |  | 20 |
|  |  |  |  |  |  |  | 180 |