Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 14AE2030 / 18AE2045 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # BASICS OF AEROSPACE ENGINEERING | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | Who is referred as the father of aerial navigation? | CO1 | Remember | 1 |
| 2. | The first human to be photographed in a flight was __________. | CO1 | Remember | 1 |
| 3. | State the basic difference between atmospheric vehicles and space vehicles. | CO2 | Remember | 1 |
| 4. | The tail end of the aircraft that assists in enabling stability of the flight is known as ___________. | CO2 | Remember | 1 |
| 5. | The lift increases with increase in angle of attack until ________. | CO3 | Remember | 1 |
| 6. | The force caused by air resistance that slows down an airplane is known as ___________. | CO3 | Remember | 1 |
| 7. | Major aircraft component which holds the passengers along with cargo is called as __________. | CO4 | Remember | 1 |
| 8. | Any part of the Aircraft structure that fails but is still permissible to fly due to the high reserve factor is known as ___________ structure. | CO4 | Remember | 1 |
| 9. | Each material retains its separate chemical, physical, and mechanical properties in ____________. | CO5 | Remember | 1 |
| 10. | The un-burned air mixes with the jet downstream at the nozzle in a __________ Engine. | CO5 | Remember | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | State the features of Aerial steam carriage proposed by Samuel Henson. | CO1 | Remember | 3 |
| 12. | State how roll motion is actuated in an aircraft. | CO2 | Remember | 3 |
| 13. | Draw an aerofoil and identify the leading edge and training edge. | CO3 | Remember | 3 |
| 14. | Quote the advantages and disadvantages of using wood for building aircraft structures. | CO4 | Remember | 3 |
| 15. | Describe the use of thrust reversers in jet engines. | CO5 | Remember | 3 |
| 16. | Define orbital velocity and escape velocity. | CO6 | Remember | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 17. | a. | Discuss the contributions of sir Otto Lilienthal to the developments of aviation. |  | CO1 | Understand | 6 |
|  | b. | Describe the features and importance of the first successful flight by Wright brothers. |  | CO1 | Understand | 6 |
| 18. |  | Discuss the 5 major parts of an aircraft and its functions with a neat sketch. |  | CO2 | Understand | 12 |
| 19. |  | Describe the different phases of flight while the aircraft travels between two destinations. |  | CO3 | Remember | 12 |
| 20. |  | Explain the structural parts of an aircraft wing. |  | CO4 | Understand | 12 |
| 21. | a. | Explain the working of a liquid propellant rocket and its types with line sketch. |  | CO5 | Understand | 6 |
|  | b. | Explain the different orbits used for positioning the satellites and its applications. |  | CO5 | Understand | 6 |
| 22. | a. | Reproduce the stress-strain curve of a composite material and a metal and examine the differences. |  | CO4 | Remember | 6 |
|  | b. | Name the secondary flight controls and explain the working of any 2 flight controls. |  | CO4 | Remember | 6 |
| 23. |  | Explain the types of drag forces developed in aircrafts. |  | CO3 | Understand | 12 |
|  |  | Compulsory: |  |  |  |  |
| 24. |  | Explain the construction and working of a turbojet engine. |  | CO6 | Understand | 12 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18AE2001 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # INTRODUCTION TO AEROSPACE ENGINEERING | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | Name 3 centres of ISRO. | CO1 | R | 1 |
| 2. | Name the city in which Gas Turbine Research Establishment (GTRE) is located. | CO1 | R | 1 |
| 3. | State the four forces acting on an airplane. | CO2 | R | 1 |
| 4. | Explain the difference between centre of pressure and aerodynamic centre. | CO2 | R | 1 |
| 5. | State three wing positions in an airplane. | CO3 | R | 1 |
| 6. | State three wing shapes in an airplane. | CO3 | R | 1 |
| 7. | List the engines that power most the flying machines. | CO4 | R | 1 |
| 8. | An aircraft piston engine, also commonly referred to as a reciprocating engine, is an internal combustion engine that uses one or more reciprocating pistons to convert ______ into a rotational motion. | CO4 | R | 1 |
| 9. | If weight (newton) is used, then specific impulse has units of ____. | CO5 | R | 1 |
| 10. | Specific impulse of cryogenic propellants is _____ than liquid propellants. | CO5 | R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Explain the functions of horizontal and vertical stabilizers in an airplane. | CO1 | R | 3 |
| 12. | A sample of gas occupies 12 L under a pressure of 1.2 atm. Calculate its volume if the pressure increases to 3.6 atm. Temperature is assumed constant. | CO2 | A | 3 |
| 13. | State different types of landing gears in an airplane. | CO3 | R | 3 |
| 14. | Explain the working principle of jet propulsion. | CO4 | R | 3 |
| 15. | Explain briefly PSLV launch vehicle. | CO5 | R | 3 |
| 16. | Write three Kepler’s laws of motion. | CO6 | R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | a. | Describe the functions of slats, spoiler, aileron and rudder in an airplane. | CO1 | R | 4 |
|  | b. | State the values of density, pressure and temperature at sea level of the standard atmosphere. | CO1 | R | 2 |
|  | c. | State the five important layers of the upper atmosphere with increase in altitude. | CO1 | R | 4 |
|  | d. | Calculate the value of air temperature at 11 km altitude. | CO1 | A | 2 |

| 18. | a. | With the help of neat figure of an airfoil, explain the different terms used in it. | CO2 | R | 5 |
| --- | --- | --- | --- | --- | --- |
|  | b. | A 100 cm meter rule is pivoted at its middle point (at 50 cm point). If weight of 2 N is hanged from the 20 cm point, Calculate the amount of weight required to be applied at the 80 cm mark to keep it in a balanced position. | CO2 | A | 4 |
|  | c. | Explain NACA2421. | CO2 | R | 3 |
| 19. | a. | Explain the terms ‘dead load’ and ‘live load’ used in an aircraft. | CO3 | R | 3 |
|  | b. | State the five major units of the airframe of a fixed-wing aircraft. | CO3 | R | 3 |
|  | c. | Describe briefly the wing construction of a fixed wing aircraft. | CO3 | R | 4 |
|  | d. | List some of the materials used in an aircraft. | CO3 | R | 2 |
| 20. | a. | Explain briefly ramjet engines, turboprop engines and turbofan engines. | CO4 | R | 12 |
| 21. | a. | From the first principles, derive the rocket equation Vb = g0 Isp ln(Mi/Mf), Where Vb is the burnout velocity, g0 is the acceleration due to gravity at sea level, Isp is specific impulse and Mi/Mf is the mass ratio. | CO5 | E | 8 |
|  | b. | If the Isp of a rocket using liquid hydrogen and oxygen as fuel and oxidizer is 430 s, and Vb is 10000 metres/s, calculate its mass ratio (g0= 9.8 m/s2). | CO5 | A | 4 |
| 22. | a. | Calculate the values of pressure, density and temperature for the standard atmosphere at an altitude of 14km. | CO1 | A | 12 |
| 23. | a. | A two-stage rocket has the following design characteristics. First stage: propellant mass = 30000 kg, structural mass = 12000 kg. Second stage: propellant mass = 15000 kg, structural mass = 4500 kg. The pay load mass is 100 kg. The specific impulse for first stage is 300 s and for the second stage is 400 s. Calculate the final burnout velocity (g0=9.8 m/s2). | CO5 | A | 8 |
|  | b. | The burnout velocity Vb of a single stage rocket is 9800 m/s. Its mass ratio (Mi/Mf)is 9. Find the specific impulse Isp of the rocket (g0=9.8 m/s2). | CO5 | A | 4 |
|  |  | Compulsory: |  |  |  |
| 24. | a. | The asteroid belt is the region of the Solar System located roughly between the orbits of the planets ____ and _____. | CO6 | R | 1 |
|  | b. | The observed average separation of Pluto from Sun is 39.44 astronomical units. From Kepler's 3rd Law, find the orbital period of Pluto. | CO6 | A | 3 |
|  | c. | Find the orbital period of an Earth satellite if its semi-major axis (a) is 8000 km. Earth’s gravitational constant (µ) is 398600 km3/s2. | CO6 | A | 4 |
|  | d. | Using Keplerꞌs equation M = E – e sin E, calculate the mean anomaly M in degrees if the eccentric anomaly E and eccentricity e are 30 degrees and 0.15, respectively. | CO6 | A | 4 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the nature of aerospace technologies. |
| CO2 | Identify the different types of Aircraft components and their functions. |
| CO3 | Assess the forces and moments due to flow over the aircraft components. |
| CO4 | Apply the principles of aerodynamics to different parts of an aeroplane. |
| CO5 | Evaluate the performance of propulsion system. |
| CO6 | Apply the knowledge of gravitational law, Kepler’s law and Newton’s law to the space vehicle. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 15 | - | 14 | - | - | - | 29 |
| CO2 | 10 | - | 7 | - |  |  | 17 |
| CO3 | 17 | - | - | - | - | - | 17 |
| CO4 | 17 | - | - | - |  |  | 17 |
| CO5 | 5 | - | 16 | - | 8 | - | 29 |
| CO6 | 4 | - | 11 | - | - | - | 15 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Arrear Examination – April / May – 2022

| # Code : | # 18AE2011 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # PROPULSION-I | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Define the second law of thermodynamics. | CO1/R | 1 |
| 2. | Mention the function of reheat in the modified cycle. | CO1/U | 1 |
| 3. | State the reason for increase in compressor exit temperature in a practical cycle. | CO2/U | 1 |
| 4. | Mention the purpose of intercooler in modified Brayton cycle. | CO2/U | 1 |
| 5. | Define surging. | CO4/R | 1 |
| 6. | Draw the velocity triangle for axial flow compressor. | CO4/R | 1 |
| 7. | Mention the function of liners in combustion chamber. | CO5/U | 1 |
| 8. | Mention how the aviation fuel differs from other fuels. | CO5/U | 1 |
| 9. | Draw the velocity diagram of turbine blade. | CO4/R | 1 |
| 10. | State the various types of blade cooling. | CO4/R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | A gas turbine operates on a pressure ratio of 4. The inlet air temperature to the compressor is 280 K and the air entering the turbine is at a temperature of 777 C. Assume that the cycle operates under ideal condition. Determine compressor exit temperature in Kelvin Take k = 1.4, Cp = 1.005 kJ/kg K. | CO1/A | 3 |
| 12. | Mention the influence of pressure thrust on the overall thrust of an aircraft. | CO2/U | 3 |
| 13. | State the importance of pre-whirl in the inlet of the compressor. | CO4/A | 3 |
| 14. | With the help of neat sketches, explain the variation of pressure and velocity in an axial compressor. | CO4/U | 3 |
| 15. | State the mechanism of the catalyst influencing the reaction rate. | CO5/U | 3 |
| 16. | Mention the purpose of stator, rotor, shroud and case of an axial flow turbine. | CO4/A | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | A gas turbine operates on a pressure ratio of 4. The inlet air temperature to the compressor is 270 K and the air entering the turbine is at a temperature of 677 ̊C. If the volume rate of air entering the compressor is 280 m3/s. Calculate the net power output of the cycle in MW. Also compute its efficiency. Assume that the cycle operates under ideal condition. | CO1/An | 12 |

| 18. | a. | In a gas turbine plant air enters the compressor at 1.1 bar and 277 K. It is compressed to 5.5 bar with an isentropic efficiency of 82%. The maximum temperature at the inlet to the turbine is 1000 C. The isentropic efficiency of the turbine is 85%. The calorific value of the fuel used is 43.1 MJ/kg. Calculate the following 1.Compressor work in KJ/kg 2. Heat supplied in KJ//kg 3. Turbine work in KJ/kg 4. Net-work in KJ/kg 5.Thermal efficiency 6. Air/Fuel ratio 7. Specific fuel consumption in kg Take Cpa = 1.005 kJ/kg K, ϒa = 1.4, Cpg = 1.147 kJ/kg K,ϒg = 1.33 | CO2/An | 12 |
| --- | --- | --- | --- | --- |
| 19. | a. | Determine the impeller diameters and the width at the impeller exit and the power required to drive the compressor, from the following given data Speed N = 12,500 rev/min Mass flow rate = 15 kg /s Pressure ration = 4:1 Isentropic efficiency = 75 % Slip factor =0.9 Flow coefficient at the impeller exit = 0.3 Hub diameter of the eye = 15 cm Axial velocity of air at the entry to and exit from the impeller = 150 m/s Stagnation temperature at the inlet = 295 K Stagnation pressure at inlet = 1 bar Assume equal pressure ratio in the impeller and diffuser. | CO4/An | 12 |
| 20. | a. | Air at a temperature of 273 K enters a ten stage axial flow compressor at the rate of 7 kg/s. The pressure ratio is 6.5 and the isentropic efficiency is 92%, the compression process being adiabatic. The compressor has symmetrical blades. The axial velocity of 110 m/s is uniform across the stage and the mean blade speed of each stage is 190 m/s. Determine the direction of the air at the entry to the exit from the rotor and the stator blades and also the power given to the air. Assume Cp = 1.005 kJ/kg K and ϒ= 1.4. | CO6/An | 12 |
| 21. | a. | Discuss the behavior of turboprop engine for fuel flow rate as function of true air speed with the help of a neat sketch. | CO1/A | 8 |
|  | b. | Mention the advantage and disadvantage of a turboprop engine. | CO1/U | 4 |
| 22. | a. | List the requirement of a good atomizer. | CO3/R | 4 |
|  | b. | With the help of a neat sketch explain the breakup mechanism for a swirl injector. | CO3/R | 8 |
| 23. | a. | Briefly explain the working of afterburner with neat sketch. | CO3/U | 12 |
|  |  | Compulsory: |  |  |
| 24. | a. | Differentiate between impulse stage and reaction stage turbine. | CO4/A | 4 |
|  | b. | Explain the various methods used for turbine blade cooling. | CO4/A | 8 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18AE2014 / 14AE2021 / 17AE2016 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # GAS DYNAMICS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | For transonic flow region, Mach number lies between ________ and ________. | CO1 / R | 1 |
| 2. | Calculate the speed of sound wave in air at 20°C and 1 atm. | CO1 / U | 1 |
| 3. | Static pressure ratio across the normal shock is given by ________. | CO3 / R | 1 |
| 4. | Is it possible to accelerate a gas to a supersonic velocity in a converging nozzle? | CO2 / U | 1 |
| 5. | Mach angle = ___________. | CO1 / R | 1 |
| 6. | The linearized potential flow equation is not applicable to ________. | CO4 / U | 1 |
| 7. | Wing sweep beneficial in that it decreases drag-divergences Mach number. (True / False) | CO4 / U | 1 |
| 8. | A Pitot probe facing a supersonic flow measures the ________ behind a normal shock. | CO2 / R | 1 |
| 9. | High-speed wings usually design with ____airfoils. | CO5 / R | 1 |
| 10. | Blow down wind tunnel is a __________ flow wind tunnel. | CO6 / R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Determine the stagnation temperature and stagnation pressure of air that is flowing at 44 kPa, 245.9 K, and 470 m/s. | CO1 / U | 3 |
| 12. | Compare the isentropic and Hugoniot curves. | CO3 / U | 3 |
| 13. | List out the changes across an oblique shock. | CO3 / R | 3 |
| 14. | At a given point on the surface of an airfoil, the pressure co-efficient is -0.3 at very low speeds. If the free stream Mach number is 0.6, find the Cp at this point. | CO4 / U | 3 |
| 15. | Distinguish between the lower and upper critical Mach numbers. | CO4 / U | 3 |
| 16. | Explain the tip effect. | CO4 / U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. |  | Explain speed of sound and derive an expression of speed of sound with suitable assumptions. | CO1 / U | 12 |
| 18. |  | A convergent and divergent nozzle is designed to expand air from a reservoir in which the pressure is 800 kPa and temperature is 40oC to give Mach number at exit of 2.5. Throat area is 25 cm2. Find a. Mass flow rate b. Exit area c. When a normal shock appears at a section where the area is 40 cm2, determine the pressure and temperature at exit. | CO2 / U | 4 4 4 |
| 19. | a. | Explain the shock polar and its characteristics with a neat sketch. | CO3 / R | 8 |
|  | b. | Discuss about the detached shock. | CO3 / R | 4 |
| 20. |  | Explain about the performance of C-D nozzle for various pressure ratios. | CO3 / U | 12 |
| 21. |  | Derive and explain the Prandtl-Glauert transformation for subsonic flow. | CO4 / U | 12 |
| 22. |  | Explain the conditions of fanno flow and highlight the findings from the fanno curve. | CO5 / U | 12 |
| 23. |  | Write short notes on - Drag divergence Mach number - Shock induced separation | CO4 / U | 6 6 |
|  |  | Compulsory: |  |  |
| 24. |  | Illustrate the working principle of shadowgraph technique with a neat sketch. | CO6 / R | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the influence of compressibility to distinguish between the flow regime |
| CO2 | Apply compressibility corrections for flow in C-D passages and instrument like Pitot static tube |
| CO3 | Estimate the sudden changes in the flow field |
| CO4 | Analyse the compressible flow field over an airfoil and finite wings |
| CO5 | Estimate the influence of friction and heat transfer in the flow field |
| CO6 | Choose proper flow visualisation techniques for the given situation |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 2 | 16 | - | - | - | - | 18 |
| CO2 | 1 | 13 | - | - | - | - | 14 |
| CO3 | 16 | 15 | - | - | - | - | 31 |
| CO4 | - | 35 | - | - | - | - | 35 |
| CO5 | 1 | 12 | - | - | - | - | 13 |
| CO6 | 13 | - | - | - | - | - | 13 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18AE2018 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # PROPULSION-II | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Mention the function of diffuser in the inlet of jet engine. | CO1 / U | 1 |
| 2. | State the various losses in C-D nozzle. | CO1 / R | 1 |
| 3. | Write down the thrust equation for rocket engine. | CO2 / R | 1 |
| 4. | Define thrust coefficient. | CO2 / U | 1 |
| 5. | Define specific heat ratio. | CO3 / R | 1 |
| 6. | List any two criteria for choice of propellants. | CO3 / U | 1 |
| 7. | Define burn rate. | CO4 / R | 1 |
| 8. | List the various types of igniters used in rocket motors. | CO4 / R | 1 |
| 9. | Mention the various types of injectors used in liquid rocket engines. | CO5 / R | 1 |
| 10. | Mention any two disadvantages of gel propellants. | CO6 / U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Mention the various types of thrust reversal and explain any one in detail. | CO1 / A | 3 |
| 12. | Classify the various types of chemical rockets. | CO2 / R | 3 |
| 13. | Mention the advantages of multi-staging of rocket engines. | CO3 / U | 3 |
| 14. | Explain the influence of perforation on the thrust time curve in solid rocket motor. | CO4 / U | 3 |
| 15. | Differentiate between bipropellant and monopropellant rocket motors. | CO5 / U | 3 |
| 16. | State the application of scramjet engine. | CO6 / A | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. |  | Write short note on i. Bell nozzle ii. Spike nozzle iii. Expansion deflection nozzle | CO1 / R | 4 4 4 |
| 18. |  | Write short note on i. Multi-staging ii. Stage separation | CO 2 / R | 6 6 |

| 19. |  | The following data are given of an optimum rocket propulsion system Average molecular mass = 24 kg/kg-mol Chamber pressure = 2.5 MPa External pressure = 0.090 Mpa Chamber temperature = 3000 k Throat area =0.00050 m2 Specific heat ratio = 1.30 Determine a. Throat velocity b. Specific volume at throatc. Propellant flow and specific impulse d. thrust | CO 3 / A | 12 |
| --- | --- | --- | --- | --- |
| 20. |  | With neat diagram, explain the various components of a solid rocket motor. | CO4 / R | 12 |
| 21. |  | With a neat sketch explain stage combustion cycle - Fuel rich stage combustion cycle. - Oxidizer rich stage combustion cycle. | CO5/U | 6 6 |
| 22. | a. | Illustrate the working principle of a hybrid rocket. | CO5 / U | 8 |
|  | b. | # Mention the advantages of hybrid rocket over solid rocket motor. | CO4 / R | 4 |
| 23. | a. | Briefly explain the heat transfer phenomena in a combustion chamber of a liquid rocket engine. | CO5 / R | 8 |
|  | b. | Explain the various methods for cooling a liquid rocket engine | CO5 / R | 4 |
|  |  | Compulsory: |  |  |
| 24. | a. | Explain arcjet thrusters with a neat sketch. | CO6 / U | 8 |
|  | b. | Mention the advantages of Ion thrusters over arc jet thrusters. | CO6 / R | 4 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18AE2027 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # HEAT AND MASS TRANSFER | # Max. Marks : | # 100 |

Properties Required – Conductivity of air = 26.3 x 10-3 W/(mK), Density of air = 1.16 kg/m3, Dynamic Viscosity of air = 184.6 x 10-7N.s/m2, Pr = 0.7, Thermal Diffusivity = 22.5 x 10-6m2/s

The heat transfer data book may be allowed into the Exam Hall

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | List the kinds of boundary conditions for the conduction equation (in mathematical form) that are typical. | CO1 | R | 1 |
| 2. | List the quantities that remain constant in 1-D steady state heat transfer with no heat generation? How do these quantities behave in Cartesian and non-Cartesian coordinate systems? | CO1 | R | 1 |
| 3. | List the conditions under which the concept of thermal resistance useful. | CO1 | U | 1 |
| 4. | Define a gray body. | CO1 | R | 1 |
| 5. | Briefly explain the difference between boiling and evaporation. | CO2 | A | 1 |
| 6. | Deduce the predominant mode of heat transfer in the wire during film boiling, based on the physical situation prevailing. | CO2 | An | 1 |
| 7. | Define the condition required for thermally fully developed flow. | CO3 | R | 1 |
| 8. | Write the main observation that is the basis of the Prandtl’s boundary layer approximations. | CO4 | R | 1 |
| 9. | Why is an LMTD required to calculate heat transfer in a heat exchanger? | CO5 | An | 1 |
| 10. | At a given instant of time, the temperature distribution within an isotropic body with no heat generation is given by the function . Determine whether this is a steady state or a transient situation? | CO6 | An | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Compare and contrast the pros and cons of the explicit, implicit schemes and Crank Nicholson method in a tabular form. | CO1 | U | 3 |
| 12. | Steel balls 12 mm in diameter are annealed by heating to 1150K and then slowly cooling to 400K in an air environment for which Tamb=325K and h = 20 W/m2/K. The properties of steel are thermal conductivity of 40 W/m/K, density of 7800 kg/m3, Specific Heat Capacity of 600 J/kg/K. Estimate the time required for the cooling process. List any assumptions you make with appropriate justifications. | CO1 | U | 3 |
| 13. | Televisions have electronic devices inside which dissipate heat. There are no fans inside the television and the maximum allowed temperature of any device or anywhere inside the television is 70oC. What modes of heat transfer are to be considered to design a heat sink for a device inside the television? | CO1 | An | 3 |

| 14. | Calculate the heat transfer coefficient inside an un-insulated pipe through which hot air is flowing at the rate of 0.05 kg/s using an appropriate correlation. The diameter of the pipe is 0.15 m. | CO3 | E | 3 |
| --- | --- | --- | --- | --- |
| 15. | Explain Reynolds Analogy. | CO4 | U | 3 |
| 16. | A heat exchanger is a device that is used to exchange heat between a hot fluid and a cold fluid. A shell and tube heat exchanger is one in which one fluid is passed through the tubes whereas the other fluid is passed over the tubes. And the external surfaces of the tubes are finned to increase the heat transfer area. The fin and tube bundle is placed in a closed shell. The fluid passing over the tubes is confined with this shell. Heat is exchanged from the hot fluid to the cold fluid through the tube walls and fins. You are to design a finned tube heat exchanger to exchange heat between hot water and air, which is expected to work under steady state conditions. Would you allow the water through the tubes (tube side) and air over the tubes (shell side) or air through the tubes and water over the tubes? Explain very briefly. | CO5 | An | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | The exact solution for heat transfer from a plane steel wall of thickness 2L = 2 m with convective boundary conditions (h = 25 W/(m2K), T∞ =300C) at both boundaries at x = -L = -1m and x = +L =1m and an initial condition of Ti =1000C is given by T-T∞Ti-T∞=n=1∞Cne-ζn2.FoCos(ζnxL) Cn=4Sinζn2ζn+Sin(2ζn)and the eigen values are given by ζntanζn=Bi.If Bi = 0.5, calculate the temperature at the mid-plane, for a time corresponding to Fo = 0.5 | CO1 | A | 12 |
| 18. | Consider a small surface of area A1=10-4 m2, which emits diffusely with a total hemispherical emissive power of E1 = 5 x 104 W/m2 (a) Calculate the rate at which this emission is intercepted by an infinitesimal area A2 = 5 x 10-4 m2, which is oriented as shown? (b) Calculate the irradiation G2 onA2? | CO1 | E | 12 |
| 19. | Draw the pool-boiling curve and identify the salient points and explain the various regimes in detail. | CO2 | R | 12 |

| 20. | Air at 60°C flows over the upper surface of a 5-m-long flat plate whose temperature is 20°C with a velocity of 2 m/s. Determine the the rate of heat transfer per unit width of the entire plate. | CO3 | E | 12 |
| --- | --- | --- | --- | --- |
| 21. | # The continuity equation and the energy equation for incompressible laminar flow of a highly viscous fluid are given by the following equations. Flow is not necessarily low speed, but can be assumed to incompressible. Convert the equations to non-dimensional form using appropriate references. Infer the non-dimensional numbers affect the flow and heat transfer? Perform an order of magnitude analysis with brief arguments and obtain the reduced equation for the boundary layer. | CO4 | U | 12 |
| 22. | In an open-heart surgery under hypothermic conditions, the patient’s blood is cooled before the surgery and rewarmed afterwards. It is proposed that a concentric tube counter flow heat exchanger of length 0.5m be used for this purpose, with the thin walled inner tube having a diameter of 55mm. If water at 60oC and 0.1 kg/s is used to heat blood entering the exchanger at 18oC and0.05 kg/s, Calculate the temperature of blood leaving the exchanger?The overall heat transfer coefficient is 500 W/(m2K). Cp of water = 4185 J/(kg K). Cp of blood = 3500 J/(kg K | CO5 | A | 12 |
| 23. | Derive the nodal finite difference equation for conduction on a plane surface with convection boundary conditions under steady state conditions | CO6 | A | 12 |
|  | Compulsory: |  |  |  |
| 24. | A thin steel flat plate of thermal conductivity of 25 W/(mK), that is 0.2 m x 0.2 m on a side is oriented parallel to an atmospheric airstream having a velocity of u∞ = 40 m/s. The plate is coated on both sides with a thick layer of naphthalene. The boundary layer temperature profile on both sides of the plate is given by T-TwT∞-Tw=1-e-Pr.yu∞υwherey is the distance normal to the surface and Pr=0.7 and  is the kinematic viscosity of air. The Schmidt Number for air is Sc = 0.7 The air is at a temperature of T∞ = 120oC while the plate is maintained at a temperature of Tw = 20oC,calculate the surface heat flux, drag force and the mass transfer coefficient on the plate. Note that both sides of the plate are actively interacting with the flow. | CO6 | A | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the fundamental modes of heat transfer. |
| CO2 | Understand the phase change heat transfer |
| CO3 | Use the heat transfer correlation for different heat transfer applications |
| CO4 | Understand the concept of hydrodynamic and thermal boundary layers |
| CO5 | Analyze and design the different types of heat exchangers |
| CO6 | Apply heat transfer principles of different applications |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 3 | 7 | 12 | 3 | 12 |  | 37 |
| CO2 | 12 |  | 1 | 1 |  |  | 14 |
| CO3 | 1 |  |  |  | 15 |  | 16 |
| CO4 | 1 | 15 |  |  |  |  | 16 |
| CO5 |  |  | 12 | 4 |  |  | 12 |
| CO6 | 1 |  | 24 |  |  |  | 25 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18AE2030 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # WIND TUNNEL TECHNIQUES | # Max. Marks: | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | In a low-speed wind tunnel, what is used to straighten the flow and what is their advisable L/D ratio. | CO1/U | 1 |
| 2. | From Bernoulli’s equation, how can you determine velocity at the exit of the contraction cone? | CO1/U | 1 |
| 3. | From Bernoulli’s equation, how can we determine velocity in the test section? Name the instrument that aids in measuring velocity. | CO2/U | 1 |
| 4. | Define Reynolds number and Mach number. | CO2/U | 1 |
| 5. | Define different type of flows based on Mach number. | CO3/U | 1 |
| 6. | What is driver section? | CO3/U | 1 |
| 7. | Name two gauge pressure measuring devises with elastic sensing elements. | CO4/U | 1 |
| 8. | Name the loads and moments acting on a body immersed in a stream of fluid. | CO4/U | 1 |
| 9. | Calculate the height of air column corresponding to atmospheric pressure. | CO5/U | 1 |
| 10. | Name the part that is used to make the laser beam into laser sheet in a PIV system. | CO5/U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | What are the uses of diffuser? List the problems experienced in short diffusers. | CO1/U | 3 |
| 12. | Write a note on the aspects of calibrating a high-speed wind tunnel. | CO2/U | 3 |
| 13. | Write differences between shock tube and shock tunnel. | CO3/U | 3 |
| 14. | Brief on the procedure to measure moments using internal balances. | CO4/U | 3 |
| 15. | Write a short note on interferometry. | CO5/U | 3 |
| 16. | Write briefly the smoke and tuft flow visualization. | CO6/U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.No 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | Write about different parts of low-speed wind tunnel. | CO1/U | 2 |
|  | b. | What are the losses in a low-speed wind tunnel? How do we estimate these losses? | CO1/A | 10 |
| 18. | a. | Obtain the mass flow rate of air through the test section of high-speed tunnel. | CO2/A | 9 |
|  | b. | List few advantages of indraft tunnels over blowdown tunnels. | CO2/U | 3 |
| 19. | a. | In a war scenario, a soldier suffered severe concussion due to blast. As an engineer, how do you simulate it in a laboratory environment and propose better plausible solutions. Write about the experimental setup and describe how you will achieve the required pressure ratios. | CO3/An | 12 |

| 20. | a. | Explain the working principle of wire type balance. | CO4/U | 7 |
| --- | --- | --- | --- | --- |
|  | b. | Write a short note on limitations of internal external balances. | CO4/U | 5 |
| 21. | a. | Describe working principle of bourdon pressure tube. | CO5/U | 8 |
|  | b. | What is Seebeck effect? Explain how dissimilar metals produce voltage. | CO5/U | 4 |
| 22. | a. | Starting with isentropic relations, explain why intermittent blowdown tunnels cannot be used to obtain flows with Mach number M = 6. | CO3/A | 8 |
|  | b. | List the advantages of intermittent blowdown tunnel. | CO3/U | 4 |
| 23. | a. | With a neat sketch, explain the working principle of PIV. | CO5/U | 8 |
|  | b. | List the differences between flow visualization technique and PIV. | CO5/U | 4 |
|  |  | Compulsory: |  |  |
| 24. | a. | Explain the Schlieren flow visualization technique. | CO6/U | 9 |
|  | b. | Define with sketch - streamline, streak lines and path lines. | CO6/U | 3 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the various types of wind tunnels and test techniques. |
| CO2 | Choose proper high speed wind tunnel for required test. |
| CO3 | Choose correct model for wind tunnel testing. |
| CO4 | Estimate the forces and moments for given model. |
| CO5 | Arrive the pressure, velocity and temperature using measurement techniques. |
| CO6 | Choose the proper flow visualization techniques. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 |  | 7 | 10 |  | - | - | 17 |
| CO2 |  | 8 | 9 |  |  |  | 17 |
| CO3 | - | 9 | 8 | 12 | - | - | 29 |
| CO4 | - | 17 |  | - |  |  | 17 |
| CO5 |  | 29 |  | - | - | - | 29 |
| CO6 |  | 15 |  | - | - | - | 15 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18AE2036 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # INTRODUCTION TO NON DESTRUCTIVE TESTING | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Indirect measurements are seen in _________. | CO1 / R | 1 |
| 2. | A cleaning fluid in LPT acts as ________. | CO1 / R | 1 |
| 3. | # The color of fluorescent penetrant under the presence of a UV light is________. | CO3 / R | 1 |
| 4. | Name the testing that is the largest application of ultrasonic testing. | CO 5 / R | 1 |
| 5. | All objects around us emit __________. | CO1 / R | 1 |
| 6. | Name the radiography technique when both sides of the specimen are accessible. | CO3 / U | 1 |
| 7. | Time period of burst emissions is _________. | CO5 / U | 1 |
| 8. | Signals from thickness changes relates _______damage in ECT. | CO5 / U | 1 |
| 9. | In a thermograph, heat is identified by _______. | CO5 / U | 1 |
| 10. | The thermography uses _____. | CO5 / R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Distinguish between DT and NDT. | CO2 / R | 3 |
| 12. | Define test block. | CO2 / R | 3 |
| 13. | Explain contact current method. | CO3 / U | 3 |
| 14. | Limitations of ultrasonic testing. | CO4 / U | 3 |
| 15. | Explain AE sensitivity. | CO4 / U | 3 |
| 16. | Mention two techniques of thermography. | CO3 / U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. |  | Explain the optical aids used in LPT. | CO1 / U | 12 |
| 18. |  | Discuss the steps involved in MPT to test a component. | CO3 / U | 12 |
| 19. |  | Explain the components used in eddy current testing. | CO3 / R | 12 |
| 20. |  | Explain the factors affecting radiographic imaging. | CO3 / U | 12 |

| 21. |  | Explain about the ultrasonic testing and highlight its merits and demerits. | CO5 / U | 12 |
| --- | --- | --- | --- | --- |
| 22. |  | Discuss about acoustic emission technique with a neat block diagram. | CO4 / R | 12 |
| 23. |  | Explain about the selection of NDT method for various defects. | CO6 / U | 12 |
|  |  | Compulsory: |  |  |
| 24. |  | Explain the salient features of thermography technique and its damage detection system. | CO3 / U | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understanding various types of discontinuities |
| CO2 | Knowledge in non – destructive testing, its scope and purpose |
| CO3 | Understand the different NDT processes |
| CO4 | Evaluate the properties of material without causing damage |
| CO5 | Learn dynamic behavior of defect with NDT tools |
| CO6 | Choose the best NDT method for different application |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 3 | 12 | - | - | - | - | 15 |
| CO2 | 6 | - | - | - | - | - | 6 |
| CO3 | 13 | 43 | - | - | - | - | 56 |
| CO4 | 12 | 6 | - | - | - | - | 18 |
| CO5 | 2 | 15 | - | - | - | - | 17 |
| CO6 | - | 12 | - | - | - | - | 12 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18AE2039 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # CRYOGENIC PROPULSION | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | H2 with same proton spin is characterized as ________ hydrogen. | CO1 | R | 1 |
| 2. | Boiling point of LN2 and LO2 are ________ &_______respectively. | CO1 | R | 1 |
| 3. | Define coefficient of performance. | CO2 | R | 1 |
| 4. | In short explain the significance of inversion curve. | CO2 | U | 1 |
| 5. | Classify various vacuum pump based on their operating range. | CO3 | U | 1 |
| 6. | Mention the reasons why the cryogenic vessels are not completely filled. | CO4 | A | 1 |
| 7. | State the purpose of copper and aluminum flakes in evacuated powder insulation. | CO5 | U | 1 |
| 8. | Gas filled power insulation in best suited for temperature range of ________. | CO5 | R | 1 |
| 9. | List the various cryogenic propellants used in a cryogenic rocket engine. | CO6 | R | 1 |
| 10. | State the purpose of pre-burner in a cryogenic rocket engine. | CO6 | U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | State the effect of cryogenic temperature on impact strength of metals. | CO1 | U | 3 |
| 12. | Briefly explain superconductivity. | CO2 | R | 3 |
| 13. | In short explain Joule-Thompson effect. | CO3 | U | 3 |
| 14. | State the importance of vent in a large cryogenic storage vessel. | CO4 | A | 3 |
| 15. | State the advantage of foam type insulator over vacuum type. | CO5 | R | 3 |
| 16. | Differentiate between pressure feed system and turbo-pump feed system. | CO6 | U | 3 |

| PART – C (6 X 12 = 72 MARKS) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. |  | Discuss the peculiar properties of Helium with neat sketches of T-S and Phase diagram. | CO1 | U | 12 |
| 18. |  | Explain Linde Hampson system of Liquefaction with T-S diagram. Derive the expressions for Liquid yield and work requirement. | CO2 | R | 12 |

| 19. |  | With a neat schematic diagram explain the working process of magnetic refrigeration system and also state its advantages and disadvantages. | CO3 | U | 12 |
| --- | --- | --- | --- | --- | --- |
| 20. | a. | With a neat sketch explain the construction of a Dewar vessel for storing cryogens. | CO4 | A | 6 |
|  | b. | Explain in detail the design procedure for inner and outer vessel. | CO4 | A | 6 |
| 21. |  | Explain in detail about expanded-foam insulations and gas-filled powders and fibrous insulations used in cryogenic systems. | CO5 | U | 12 |
| 22. |  | List the various types of refrigeration system along with the operating temperature range and explain any one of them in detail. | CO5 | R | 12 |
| 23. |  | Write short note on: i. Mechanical vacuum pump. ii. Diffusion vacuum pump. | CO4 | R | 12 |
|  |  | Compulsory: |  |  |  |
| 24. | a. | Briefly explain the need for chilling in cryogenic rocket engine. | CO6 | An | 4 |
|  | b. | Explain the starting sequence of a cryogenic engine for the various valve position with respect to time. | CO6 | A | 8 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the thermal, physical and flow properties of cryogenic fluids. |
| CO2 | Understand the liquefaction systems to produce cryogenic fluids |
| CO3 | Know the various method of cryogenic refrigeration systems |
| CO4 | Explain the various cryogenic fluid storage and transfer lines |
| CO5 | Design of various insulations for cryogenic propellant tanks |
| CO6 | Know the various applications of cryogenics in propulsion systems |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 2 | 15 |  |  |  |  | 17 |
| CO2 | 16 | 1 |  |  |  |  | 17 |
| CO3 |  | 16 |  |  |  |  | 16 |
| CO4 | 12 |  | 16 |  |  |  | 28 |
| CO5 | 16 | 13 |  |  |  |  | 29 |
| CO6 | 1 | 4 | 8 | 4 |  |  | 17 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18AE2041 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ADVANCED SPACE DYNAMICS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | Define a central orbit. | CO1 | R | 1 |
| 2. | If f is the law of attraction towards the origin, the equation of motion is d2r/dt2 = _________. | CO1 | U | 1 |
| 3. | If the orbital period (T) of a satellite is 6000 s, its mean motion (n) is _________. | CO2 | R | 1 |
| 4. | Using Keplerꞌs equation M = E – e sin E, calculate the mean anomaly M in degrees if the eccentric anomaly E and eccentricity e are 45 degrees and 0.1, respectively. | CO2 | A | 1 |
| 5. | Find the mass parameter (μ) of Sun-Jupiter system. Masses of Sun and Jupiter are 1.989x1030 and 1.899 x1027 kg, respectively. | CO3 | A | 1 |
| 6. | Find the center of mass of Sun-Jupiter system. Masses of Sun and Jupiter are 1.989 x1030 and 1.899 x1027 kg, respectively. The distance between them is 778.6 x106 km. | CO3 | A | 1 |
| 7. | There are ________ pure imaginary roots at the collinear points L1,2,3 of the characteristic equation in the planar restricted three-body problem. | CO4 | R | 1 |
| 8. | At the Lagrangian points of the planar restricted three-body problem, the partial derivatives w.r.t. x and y are: Ωx=____, Ωy=____. | CO4 | U | 1 |
| 9. | At the collinear points Li (i =1, 2, 3) in the three-dimensional restricted three-body problem, the angular frequency sz ______si (i =1, 2, 3). | CO5 | R | 1 |
| 10. | If in two-body problem, the angular momentum h = 50000 km2/s, hz = 5000 km2/s, find the value of the orbital inclination i in degrees. | CO5 | A | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | For one-dimensional motion, by assuming x ≥ 0, using the equation of energy K2/x – (dx/dt)2/2 = hK, for the case of zero energy (hK = 0), prove that x = (9 K2/2)1/3 t2/3. K2 = k2(M + m), k2 is the universal gravitational constant, M and m are the masses of the two bodies. | CO1 | E | 3 |
| 12. | Explain Lambert’s theorem with the help of a figure. | CO2 | R | 3 |
| 13. | With the help of a diagram, prove that X = x cos t – y sin t, Y = x sin t + y cos t, where (X, Y) and (x, y) are coordinates of a point in fixed and rotating (synodic) coordinate systems, respectively, with O as the origin. t is the angle between OX and Ox. | CO3 | E | 3 |

| 14. | Construct the principal system of co-ordinates at the equilateral point L4 to show the angle α, which the major-axis of the elliptic orbit makes with the ξ-axis for mass ratio μ = 0.2 in the planar restricted three-body problem. | CO4 | A | 3 |
| --- | --- | --- | --- | --- |
| 15. | Write equations of motion for three-dimensional restricted three-body problem. | CO5 | R | 3 |
| 16. | Write the equations of motion for planar restricted three-body problem when more massive primary is an oblate spheroid with its equatorial plane coincident with the plane of motion. | CO6 | R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | a. | Prove that the angular momentum is a constant vector. Prove that the motion takes place in a plane which passes through the origin. | CO1 | E | 4 |
|  | b. | If two point masses m1and m2 are acted upon only by the mutual force of gravity between them, prove that the centre of mass of this system moves with constant velocity in a straight line. Prove that the motion of m2 around m1 is governed by a second-order non-linear differential equation d2r/dt2 = - µr/r3, µ is the gravitational constant. | CO1 | E | 8 |
| 18. | a. | Derive Lambert’s theorem analytically. | CO2 | E | 12 |
| 19. | a. | Derive equations of motion for planar restricted three body problem in synodic (rotating) coordinate system. Write the two equations to find the locations of the five equilibrium points. Derive Jacobi’s integral. | CO3 | E | 12 |
| 20. | a. | To study the motion near the equilibrium points in the planar restricted three-body problem, expand the force function Ω up to second-order terms around a Lagrangian point. Find the linearized variational equation of motion in two dimensions in the planar restricted three-body problem. Derive the fourth-degree characteristic equation λ4 + (4 - Ωxx- Ωyy)λ2 + {ΩxxΩyy- (Ωxy)2}= 0, at the Lagrangian points in the planar restricted three-body problem. | CO4 | E | 12 |
| 21. | a. | Derive Tisserand criterion 1/2a + [a (1 – e2)]1/2 cos I = constant, for identification of comets, where a, e and I are initial semi-major axis, eccentricity and orbital inclination of a comet with respect to Sun-Jupiter orbital plane. | CO5 | E | 8 |
|  | b. | If a = 4.81 AU, e = 0.763 and I = 7.47 degrees are the orbital elements of a comet before having close encounter with Jupiter, and aꞌ =10.8 AU, eꞌ = 0.731 are its orbital parameter after the encounter with Jupiter, find the value of orbital inclination Iꞌ in degrees of the comet after the encounter. The distance between Sun and Jupiter is 5.2 AU. | CO5 | A | 4 |

| 22. | a. | Prove that the second-order derivatives at the equilateral point L4are Ωxx = 3/4, Ωxy = 3.31/2 (μ - 1/2)/2, Ωyy = 9/4.Using these values of partial derivatives, prove that the characteristic equation is λ4 + λ2 + 27μ (1 - μ)/4 = 0. Find the value of the critical mass μ0 = 0. 03852 at the equilateral equilibrium points. | CO4 | E | 12 |
| --- | --- | --- | --- | --- | --- |
| 23. | a. | For one-dimensional motion, using the transformation from ordinary to fictitious time performed by d/dt = (1/x)(d/ds), the equation of motion d2x/dt2 + K2/x2 = 0 and equation of energy given in (11) are transformed as xd2x/ds2 + (dx/ds)2 + K2x = 0 and (dx/ds)2 = 2(K2x – hKx2). For hK = 0, prove that x = K2s2/3 and t = K2s3/6. Prove that the equation of motion and equation of energy for one-dimensional motion can be transformed as d2x/ds2 + 2hK x = K2. Prove that the solution of this equation is x = {K2/(4Ꞷ2)}(1 – cos 2Ꞷs), where Ꞷ2=hK/2. | CO1 | E | 12 |
|  |  | Compulsory: |  |  |  |
| 24. | a. | Write the equations of motion for planar photo gravitational restricted three-body problem. Obtain the two equations to find locations of the five Lagrangian points. Prove that the locations of the triangular Lagrangian points is given by r1=q1/3, r2 = 1. Derive the fifth-degree algebraic equations to find the location of the collinear Lagrangian point L1. | CO6 | E | 12 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18AE2042 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # AIR TRAFFIC CONTROL AND AERODROME DETAILS | # Max. Marks : | # 100 |

| Q. No. | Questions |  | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. |  | Mention the military name for UTC. | CO1 | R | 1 |
| 2. |  | Expand SIGMET. | CO1 | R | 1 |
| 3. |  | The Separation minima depends on _______ of an aircraft. | CO2 | U | 1 |
| 4. |  | When it becomes either impossible or inadvisable to proceed to or land at the intended aerodrome, _________ is needed. | CO2 | U | 1 |
| 5. |  | NOTAM (India) stands for __________. | CO3 | R | 1 |
| 6. |  | An aircraft in a state of ___________ shall be given priority over other aircraft. | CO3 | R | 1 |
| 7. |  | The vertical clearance between the structure and the aircraft making an instrument approach to land shall be up to__________. | CO4 | R | 1 |
| 8. |  | The aerodrome reference temperature shall be the monthly mean of the daily maximum temperatures for the ___________ of the year. | CO4 | R | 1 |
| 9. |  | Navigation lights or beacons that are always illuminated while the aircraft is ________. | CO5 | U | 1 |
| 10. |  | When an aircraft is approaching an aerodrome in such a manner that there is a danger of an accident occurring, _______emergency phase is released. | CO5 | U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Name the flights permitted in class D airspace. | CO1 | R | 3 |
| 12. | Explain about the closing a flight plan. | CO2 | U | 3 |
| 13. | Compare primary radar with secondary radar. | CO3 | U | 3 |
| 14. | Highlight the importance of aerodrome elevation. | CO4 | R | 3 |
| 15. | Name the components of segmented circle used for landing. | CO5 | R | 3 |
| 16. | Define airworthiness. | CO6 | R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | a. | Explain the altimeter setting procedures and the terms associated with it. | CO1 | U | 12 |
| 18. | a. | Discuss about RNAV and RNP. | CO2 | U | 12 |
| 19. | a. | Discuss the role of Flight Information Service. | CO3 | R | 12 |
| 20. | a. | Explain the characteristics of Instrument runway. | CO4 | U | 12 |

| 21. | a. | Write short notes on: - VASI (ii) PAPI | CO5 | U | 6 6 |
| --- | --- | --- | --- | --- | --- |
| 22. | a. | Explain about the landing devices with a neat sketch. | CO5 | U | 12 |
| 23. | a. | Compare and contrast the features of VFR and IFR flights. | CO1 | U | 6 |
|  | b. | Explain the runway markings. | CO5 | U | 6 |
|  |  | Compulsory: |  |  |  |
| 24. | a. | Discuss the aviation environmental protection. | CO6 | U | 6 |
|  | b. | Explain the milestones of air transportation. | CO6 | U | 6 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Recall the basic concepts of ATS and its services |
| CO2 | Distinguish the flight operations between different altitudes |
| CO3 | Exemplify the working routines of radar services |
| CO4 | Identify the Aerodrome layouts and the design |
| CO5 | Illustrate the runway restrictions, various approach systems and guidance |
| CO6 | Understand the need for Civil Aviation Requirements |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 5 | 18 | - | - | - | - | 23 |
| CO2 | - | 17 | - | - | - | - | 17 |
| CO3 | 14 | 3 | - | - | - | - | 17 |
| CO4 | 5 | 12 | - | - | - | - | 17 |
| CO5 | 3 | 32 | - | - | - | - | 35 |
| CO6 | 3 | 12 | - | - | - | - | 15 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18AE2043 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # AIRCRAFT SYSTEMS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | The micronic element in the hydraulic filters are designed to prevent the passage of solids greater than ___________ microns. | CO1 | Remember | 1 |
| 2. | Opening and closing of doors is aircraft is actuated by ________ system. | CO1 | Remember | 1 |
| 3. | The gear feature used in helicopters for landing is called as __________. | CO2 | Remember | 1 |
| 4. | Most aircraft are steerable from the flight deck through a ___________ steering system. | CO2 | Remember | 1 |
| 5. | A measure of the tendency of a liquid substance to vaporize under given conditions is known as __________. | CO3 | Remember | 1 |
| 6. | ___________ system generates a spark or heats an electrode to a high temperature to ignite a fuel- air mixture in spark ignition internal combustion engines. | CO3 | Remember | 1 |
| 7. | ___________ point is the temperature at which its wax content begins to solidify and separate into tiny crystals appearing cloudy or hazy. | CO4 | Remember | 1 |
| 8. | In magneto-ignition system, as the magnet moves from the full-register position (0°) into the neutral position (45°), the flux flow ____________. | CO4 | Remember | 1 |
| 9. | ____________ cooling is performed by scooping air from the aircraft boundary layer or close to it. | CO5 | Remember | 1 |
| 10. | The condition resulting due to breathing air at reduced atmospheric pressure resulting in deficiency of oxygen supply to the body and brain tissues is known as _____________. | CO5 | Remember | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Describe the operation of a single action actuating and double action actuating hydraulic cylinder with a sketch. | CO1 | Remember | 3 |
| 12. | Specify the advantages of tricycle landing gear arrangement. | CO2 | Remember | 3 |
| 13. | Identify three characteristics required by gasoline fuels for good engine performance. | CO3 | Remember | 3 |
| 14. | Briefly recall the working of an air starter. | CO4 | Remember | 3 |
| 15. | Enumerate the need for cabin conditioning in aircrafts. | CO5 | Remember | 3 |
| 16. | Label the factors influencing the effect of icing in aircraft structures. | CO6 | Remember | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | a. | Describe the working of a power driven pump based hydraulic system with a neat sketch. | CO1 | Understand | 6 |
|  | b. | Explain the working of a vane pump used in hydraulic systems of aircrafts. | CO1 | Understand | 6 |
| 18. | a. | Explain the three basic landing gear arrangements used in aircrafts. | CO2 | Understand | 6 |
|  | b. | Explain how independent systems are used in small aircrafts for braking. | CO2 | Understand | 6 |
| 19. |  | Describe the basic system components in an aircraft fuel system. | CO3 | Remember | 12 |
| 20. | a. | Explain the wet sump lubrication system in reciprocating engine with a schematic representation. | CO4 | Understand | 12 |
| 21. | a. | Explain the process of ram air cooling in aircrafts. | CO5 | Understand | 6 |
|  | b. | Write a brief note on cabin air distribution systems used in cabin cooling of aircrafts. | CO5 | Understand | 6 |
| 22. |  | Explain the high, medium and low pressure pneumatic systems used in actuation of aircraft systems. | CO1 | Understand | 12 |
| 23. |  | Explain the types and properties of turbine engine fuels. | CO3 | Understand | 12 |
|  |  | Compulsory: |  |  |  |
| 24. | a. | Explain the position and warning systems used in aircrafts. | CO6 | Understand | 6 |
|  | b. | Explain in detail the functioning of water and waste water system in aircrafts. | CO6 | Understand | 6 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the principles Aircraft Hydraulic systems |
| CO2 | Obtain knowledge on the Landing Gear systems |
| CO3 | Obtain knowledge on Fuel systems and Engine starting systems |
| CO4 | Diagnose aircraft engine starting systems performance |
| CO5 | Obtain knowledge on Cabin atmosphere control systems |
| CO6 | Understand the basics of auxiliary systems in aircraft |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 5 | 14 | - | - | - | - | 29 |
| CO2 | 5 | 15 | - | - | - | - | 17 |
| CO3 | 17 | 12 | - | - | - | - | 29 |
| CO4 | 5 | 12 | - | - | - | - | 17 |
| CO5 | 5 | 12 | - | - | - | - | 17 |
| CO6 | 3 | 12 | - | - | - | - | 15 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – June – 2022

| # Code : | # 18AE3014 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # COMPUTATIONAL HEAT TRANSFER | # Max. Marks : | # 100 |

ANSWER ALL QUESTIONS (5 X 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| 1. |  | Compute one – dimensional steady state heat conduction without heat generation for plane wall. | CO1 / R | 20 |
| (OR) |  |  |  |  |
| 2. |  | Explain 2D-heat conduction problem and evaluate them for different boundary conditions. | CO2 / E | 20 |
| 3. | a. | Explain about simple implicit method. | CO3 / A | 10 |
|  | b. | Compare implicit, explicit and Crank Nicholson method. | CO3 / A | 10 |
| (OR) |  |  |  |  |
| 4. |  | Explain stream function-vorticity approach for heat transfer. | CO4 / U | 20 |
| 5. |  | Write a short note on absorption method for a flat surface with supporting diagram. | CO5 / R | 20 |
| (OR) |  |  |  |  |
| 6. |  | Derive the nodal equation for a boundary node with convective boundary condition. | CO4 / A | 20 |
| 7. |  | Derive the nodal temperature expression for interior node by implicit method assuming 2-D steady state condition. | CO4 / A | 20 |
| (OR) |  |  |  |  |
| 8. |  | Explain Crank-Nicolson schemes and explicit scheme for transient heat transfer. | CO3 / A | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | With neat sketch, explain the heat flux measurements using Gardon Gauge and Thin film sensors also mention their application. | CO6 / A | 20 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Know the mathematical concepts of computational heat transfer. |
| CO2 | Know the different applications of heat transfer. |
| CO3 | Understand the different computational methods of heat transfer. |
| CO4 | Understand the heat transfer methods in FDM and FEM. |
| CO5 | Apply the computational heat transfer methods in practical applications. |
| CO6 | Analyse the real time problems of heat transfer in aerospace applications. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 20 |  |  |  |  |  | 20 |
| CO2 |  |  |  |  | 20 |  | 20 |
| CO3 |  |  | 40 |  |  |  | 40 |
| CO4 |  | 20 | 40 |  |  |  | 60 |
| CO5 | 20 |  |  |  |  |  | 20 |
| CO6 |  |  | 20 |  |  |  | 20 |
|  |  |  |  |  |  |  | 180 |

Reg. No. _____________

End Semester Examination – June – 2022

| # Code : | # 18AE3023 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # BOUNDARY LAYER THEORY | # Max. Marks : | # 100 |

ANSWER ALL QUESTIONS (5 X 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| 1. |  | From Navier Strokes equation using the order of magnitude analysisand derive the boundary layer equation for momentum and energy. | CO1 / R | 20 |
| (OR) |  |  |  |  |
| 2. |  | Based on boundary layer equation for momentum and energy explain Reynolds analogy for flat plate. | CO2 / An | 20 |
| 3. |  | Explain the Couette flow and Poiseuille flow in detail. | CO3 / A | 20 |
| (OR) |  |  |  |  |
| 4. |  | Derive and evaluate the expression for laminar boundary layer on a flat plate at zero incidences. | CO4 / E | 20 |
| 5. |  | Discuss the Mangler transformation for flat plate. | CO3 / U | 20 |
| (OR) |  |  |  |  |
| 6. |  | Explain simple solutions of energy equation for thermal boundary layer. | CO4 / U | 20 |
| 7. |  | With a neat sketch discuss the plate flow with uniform blowing and suction. | CO3 / A | 20 |
| (OR) |  |  |  |  |
| 8. | a. | Derive the momentum integral equation for a boundary layer on flat plate. | CO5 / A | 12 |
|  | b. | State the advantage of integral method for solution of Navier Strokes equation. | CO2 / R | 8 |
|  |  | Compulsory: |  |  |
| 9. |  | Explain different kinds of boundary layer effects in hypersonic flows | CO6 / U | 20 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Define the fundamentals Boundary layer theory. |
| CO2 | Solve the equations involved in boundary layer theory. |
| CO3 | Analyze the different kinds of Boundary Layer control. |
| CO4 | Differentiate between the turbulent and laminar boundary layers . |
| CO5 | Estimate the boundary layer thickness for flow over a different body. |
| CO6 | Attain the knowledge of boundary layer effects in hypersonic flows. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 20 |  |  |  |  |  | 20 |
| CO2 | 8 |  |  | 20 |  |  | 28 |
| CO3 |  | 20 | 40 |  |  |  | 60 |
| CO4 |  | 20 |  |  | 20 |  | 40 |
| CO5 |  |  | 12 |  |  |  | 12 |
| CO6 |  | 20 |  |  |  |  | 20 |
|  |  |  |  |  |  |  | 180 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18AE3027 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # UNMANNED AIRCRAFT SYSTEMS | # Max. Marks : | # 100 |

| Q. No. | Sub Div. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |  |
| 1. |  | Categorize and explain various UAS based on Air vehicles type. | CO1 | R | 16 |
| 2. |  | Explain various parameters that are to be considered while selecting a UAS. | CO2 | U | 16 |
| 3. |  | Explain in detail about the selection of power plant in UAS. Explain the types of power plant with example. Discuss their advantages and disadvantages. | CO3 | A | 16 |
| 4. | a. | Explain about the UAS communication system and its types. | CO4 | R | 8 |
|  | b. | What are the four usual types of Antennae, discuss their advantages and disadvantages in UAS application. | CO4 | U | 8 |
| 5. | a. | Write how the ground testing of a complete UAV is done with examples, - for HTOL aircraft - for VTOL aircraft | CO5 | U | 11 |
|  | b. | Explain the inflight testing procedure for a UAS. | CO5 | U | 5 |
| 6. |  | Discuss the various UAS airframe configurations (with diagram) and their advantages and disadvantages. | CO2 | R | 16 |
| 7. |  | Discuss in detail about the types of Hybrid airframe configurations with examples. | CO2 | A | 16 |
| PART – B (1 X 20 = 20 MARKS) (Compulsory Question) |  |  |  |  |  |
| 8. | a. | Explain in detail the launch and recovery of HTOL and VTOL systems. | CO6 | U | 10 |
|  | b | Discuss the Naval, Army and Civilian roles of UAS. | CO6 | A | 10 |

Reg. No. _____________

End Semester Examination – May / June – 2022

| # Code : | # 20AE2001 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name: | # INTRODUCTION TO AEROSPACE ENGINEERING | # Max. Marks: | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | Name the city in which National Aeronautics Laboratories (NAL) is located. | CO1 | R | 1 |
| 2. | Name the city in which Vikram Sarabhai Space Centre (VSSC) is located. | CO1 | R | 1 |
| 3. | State the four forces acting on an airplane. | CO2 | R | 1 |
| 4. | Define centre of pressure. | CO2 | R | 1 |
| 5. | State three wing positions in an airplane. | CO3 | R | 1 |
| 6. | State three wing shapes in an airplane. | CO3 | R | 1 |
| 7. | List the engines that power most the flying machines. | CO4 | R | 1 |
| 8. | An aircraft piston engine, also commonly referred to as a reciprocating engine, is an internal combustion engine that uses one or more reciprocating pistons to convert ________ into a rotational motion. | CO4 | R | 1 |
| 9. | If weight (newton) is used, then specific impulse has units of ____. | CO5 | R | 1 |
| 10. | Specific impulse of cryogenic propellants is ________ than liquid propellants. | CO5 | R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Explain the functions of horizontal and vertical stabilizers in an airplane. | CO1 | R | 3 |
| 12. | A sample of gas occupies 24 L under a pressure of 1.2 atm. Calculate its volume if the pressure increases to 4.8 atm. Temperature is assumed constant. | CO2 | A | 3 |
| 13. | State different types of landing gears in an airplane. | CO3 | R | 3 |
| 14. | Explain the working principle of jet propulsion. | CO4 | R | 3 |
| 15. | Explain briefly PSLV launch vehicle. | CO5 | R | 3 |
| 16. | Write three Kepler’s laws of motion. | CO6 | R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | a. | Describe the functions of slats, spoiler, aileron and rudder in an airplane. | CO1 | R | 4 |
|  | b. | State the values of density, pressure and temperature at sea level of the standard atmosphere. | CO1 | R | 2 |
|  | c. | State the five important layers of the upper atmosphere with increase in altitude. | CO1 | R | 4 |
|  | d. | Calculate the value of air temperature at 8 km altitude. | CO1 | A | 2 |

| 18. | a. | With the help of neat figure of an airfoil, explain the different terms used in it. | CO2 | R | 5 |
| --- | --- | --- | --- | --- | --- |
|  | b. | A 100 cm meter rule is pivoted at its middle point (at 50 cm point). If weight of 4 N is hanged from the 20 cm point, Calculate the amount of weight required to be applied at the 80 cm mark to keep it in a balanced position. | CO2 | A | 4 |
|  | c. | Explain NACA2421. | CO2 | R | 3 |
| 19. | a. | Explain the terms ‘dead load’ and ‘live load’ used in an aircraft. | CO3 | R | 3 |
|  | b. | State the five major units of the airframe of a fixed-wing aircraft. | CO3 | R | 3 |
|  | c. | Describe briefly the wing construction of a fixed wing aircraft. | CO3 | R | 4 |
|  | d. | List some of the materials used in an aircraft. | CO3 | R | 2 |
| 20. |  | Explain briefly ramjet engines, turboprop engines and turbofan engines. | CO4 | R | 12 |
| 21. | a. | From the first principles, derive the rocket equation Vb = g0 Isp ln(Mi/Mf), where Vb is the burnout velocity, g0 is the acceleration due to gravity at sea level, Isp is specific impulse and Mi/Mf is the mass ratio. | CO5 | E | 8 |
|  | b. | If the Isp of a rocket using liquid hydrogen and oxygen as fuel and oxidizer is 400 s, and Vb is 8000 metres/s, calculate its mass ratio (g0= 9.8 m/s2). | CO5 | A | 4 |
| 22. |  | Calculate the values of pressure, density and temperature for the standard atmosphere at an altitude of 14km. | CO1 | A | 12 |
| 23. | a. | A two-stage rocket has the following design characteristics. First stage: propellant mass = 30000 kg, structural mass = 10000 kg. Second stage: propellant mass = 15000 kg, structural mass = 6000 kg. The pay load mass is 50 kg. The specific impulse for first stage is 300 s and for the second stage is 400 s. Calculate the final burnout velocity(g0=9.8 m/s2). | CO5 | A | 8 |
|  | b. | The burnout velocity Vb of a single stage rocket is 8800 m/s. Its mass ratio (Mi/Mf)is 11. Find the specific impulse Isp of the rocket (g0=9.8 m/s2). | CO5 | A | 4 |
|  |  | Compulsory: |  |  |  |
| 24. | a. | The asteroid belt is the region of the Solar System located roughly between the orbits of the planets ______ and _____. | CO6 | R | 1 |
|  | b. | The observed average separation of Pluto from Sun is 39.44 astronomical units. From Kepler's 3rd Law, find the orbital period of Pluto. | CO6 | A | 3 |
|  | c. | Explain the approximate catalog composition of space bodies with the help of a pie chart. | CO6 | A | 4 |
|  | d. | List sources and sinks of space debris. | CO6 | A | 4 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the nature of aerospace technologies. |
| CO2 | Identify the different types of Aircraft components and their functions. |
| CO3 | Assess the forces and moments due to flow over the aircraft components. |
| CO4 | Apply the principles of aerodynamics to different parts of an aeroplane. |
| CO5 | Evaluate the performance of propulsion system. |
| CO6 | Apply the knowledge of gravitational law, Kepler’s law and Newton’s law to the space vehicle. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 15 | - | 14 | - | - | - | 29 |
| CO2 | 10 | - | 7 | - |  |  | 17 |
| CO3 | 17 | - | - | - | - | - | 17 |
| CO4 | 17 | - | - | - |  |  | 17 |
| CO5 | 5 | - | 16 | - | 8 | - | 29 |
| CO6 | 4 | - | 11 | - | - | - | 15 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 20AE2002 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # BASICS OF FLUID MECHANICS | # Max. Marks : | # 100 |

Properties Required –Density of air = 1.16 kg/m3, Dynamic Viscosity of air = 184.6 x 10-7N.s/m2

| Q. No. | Questions | Course Outcome |  | Pattern | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | List four differences between fluids and solids. |  | CO1 | R | 1 |
| 2. | Differentiate the relation between stress and deformation, for solids and liquids. |  | CO1 | R | 1 |
| 3. | Do rotating flows (physically rotating) and rotational flows (flows that are not irrotational) mean the same? If not, explain very briefly. |  | CO1 | U | 1 |
| 4. | A tank containing water has 2 orifices of the same size at depths of 40cm and 90cm below the free surface of water. Calculate the ratio of discharges through these orifices |  | CO1 | A | 1 |
| 5. | Explain briefly the cavitation that could happen in a venturi meter? No more than 4 lines are required for the explanation. |  | CO4 | U | 1 |
| 6. | In a steady 2D incompressible flow, if u = 2x, then derive the most general form of v. |  | CO2 | An | 1 |
| 7. | Briefly describe the main observation by Prandtl based on which the boundary layer equations are developed. No more than 4 lines are required for the explanation. |  | CO2 | U | 1 |
| 8. | Define the condition required for hydrodynamically fully developed flow. |  | CO2 | R | 1 |
| 9. | Differentiate the Darcy friction factor and Fanning friction coefficient. |  | CO5 | A | 1 |
| 10. | An experiment is to be conducted in a laboratory to simulate the movement of a sphere shaped torpedo under water moving at a velocity of 1.6m/s. In the laboratory, the experiment can be conducted in a wind tunnel and tested with air, which is 750 times less dense and 60 times less viscous than water. The laboratory has only a sphere of twice the diameter of the torpedo available for testing. What should be the velocity of air in the wind tunnel? |  | CO6 | A | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | A vertical tube of diameter 5 cm closed at its bottom and open at the top contains a liquid and is rotated about its vertical axis at 700 rpm. Calculate the difference in height between the lowest and highest point of the free surface. | CO3 | A | 3 |
| ‘12. | (a) What conservation law does Bernoulli’s equation represent? (b) What are the conditions under which Bernoulli’s equation can be used? (c) What additional condition is required to use Bernoulli’s equation to calculate the pressure difference along the radius in a flow that has a streamline pattern represented by concentric circles? | CO1 | U | 3 |

| 13 | A certain two-dimensional flow field is given by V=4xyi+2(x2-y2)j. Show and explain that a stream function should exist for this flow. Calculate the stream function and sketch the streamline through the origin and the shape of other streamlines. Show that a velocity potential should exist for this flow. Finding velocity potential will be a waste of time and will not get credit if you do not write the reason why it should exist. | CO2 | An | 3 |
| --- | --- | --- | --- | --- |
| 14. | For a two-dimensional steady flow the velocity potential is given byϕ=53x3-5xy2. Is this a valid velocity potential? Is this flow rotational? Determine the stream function? Show whether this flow is incompressible or compressible. | CO2 | An | 3 |
| 15. | Express the most general law of conservation of mass in differential form If the flow is steady, what equation will result? Can this flow be compressible? ii) If the flow is incompressible, what equation will result? Can this flow be unsteady? If yes, which quantities would vary with time? | CO5 | E | 3 |
| 16. | What are favorable and adverse pressure gradients and how do they affect the flow in a boundary layer. Based on this, explain very briefly why a golf ball is dimpled? | CO5 | U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | A Newtonian fluid having a specific gravity of 0.92 and a kinematic viscosity of 5 x 10-4m2/s flows past a flat plate. The velocity profile is given by uU=32yδ-12yδ3. Usingthe momentum integral method the local boundary layer thickness for this velocity profile is given by the equation δx=4.64Rex, where the local Reynolds number is Rex=ρUxμ and x is the distance along the plate. Express the skin friction coefficient as a function of Rex. Calculate the shear stress at the plate surface at a point where the boundary layer thickness,  is 1 mm when the free stream velocity U = 10 m/s.. |  | CO2 | A | 12 |
| 18. | Oil of specific gravity 0.83 flows in the pipe shown in Figure. If viscous effects are neglected, calculate the flow rate? 7.5 cm 10cm 10 cm |  | CO1 | A | 12 |

| 19. | Water flows steadily through the large tanks as shown in figure and the levels are maintained. Determine the water depth hain the first tank. List the assumptions and mark the assumptions on the diagram if applicable. |  | CO4 | A | 12 |
| --- | --- | --- | --- | --- | --- |
| 20. | Discuss in detail the stability of floating and immersed bodies and the requirements to be met, using diagrams and appropriate definitions. |  | CO3 | E | 12 |
| 21. | A tank of 0.05 m3volume contains air at 800 kPA (absolute) and 15oC. At time t = 0, air escapes from the tank through a valve with a flow area of 65 mm2. The air passing through the valve at t = 0has a velocity of 311 m/s and a density of 6.13 kg/m3. Properties of the rest of the tank may be assumed to be uniform. Determine the instantaneous rate of change of density in the tank at t = 0. |  | CO4 | An | 12 |
| 22. | Air at 60°C flows over the upper surface of a 5-m-long flat plate whose temperature is 20°C with a velocity of 2 m/s. The Blasius solution gives the local skin friction coefficient at any axial location x as Cf, x=0.664Rex. Determine the average shear stress on the plate. Calculate the velocity gradient at the plate surface at a distance of 1m from the leading edge. |  | CO5 | E | 12 |
| 23. | State the Buckingham’s Pi Theorem. Illustrate an application with an example. Also demonstrate the derivation of the non-dimensional groups for some physical phenomenon by using the standard application procedure of the theorem or from a governing equation. |  | CO6 | U | 12 |
|  | Compulsory: |  |  |  |  |
| 24. | In a 2-dimensional flow V=-xti+ytj and ρ=kxt, where t, are time (s), density (kg/m3) and k is a constant and is equal to 1 kg/s/m4. (a) What is the rate of change of density recorded by an observer at a point (1,1) at t = 1 (b) What is the rate of change of density of a fluid particle at (1,1) at t = 1 (c) Is this a valid flow field (d) Is the flow steady? (e) Calculate the divergence of velocity? (f) What does the divergence signify physically? (g) Is this flow compressible? (h) If the divergence of velocity is zero, can we make any conclusions on the steadiness or the incompressibility of the fluid? If yes or If no, why? (i) What additional condition is required to ensure that the flow is incompressible? (j) What more is required to ensure that the flow is steady? |  | CO2 | An | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Know the properties of different fluids and pressure measurements. |
| CO2 | Apply mathematical knowledge to predict the properties and characteristics of a fluid. |
| CO3 | Understand the nature of buoyancy of submerged and floating bodies. |
| CO4 | Attain the Knowledge of flow measurement systems. |
| CO5 | Estimate the friction factor of pipe flow and losses associated it. |
| CO6 | Get knowledge of the non-dimensional parameters used in airflow. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 2 | 4 | 13 |  |  |  | 19 |
| CO2 | 1 | 1 | 12 | 19 |  |  | 33 |
| CO3 |  | 12 | 3 |  |  |  | 15 |
| CO4 |  | 1 | 12 | 12 |  |  | 25 |
| CO5 |  | 3 | 1 |  | 15 |  | 19 |
| CO6 |  | 12 | 1 |  |  |  | 13 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – May / June – 2022

| # Code : | # 20AE2002 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # BASICS OF FLUID MECHANICS | # Max. Marks : | # 100 |

Properties Required – Density of air = 1.16 kg/m3, Dynamic Viscosity of air = 184.6 x 10-7N.s/m2

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | List four differences between fluids and solids. | CO1 | R | 1 |
| 2. | Differentiate the relation between stress and deformation, for solids and liquids. | CO1 | R | 1 |
| 3. | Draw the stream line patterns of a forced vortex and a free vortex and express the velocity profiles as equations for both flows and also mention whether each of the flows is rotational or irrotational. | CO1 | U | 1 |
| 4. | A tank containing water has 2 orifices of the same size at depths of 40cm and 90cm below the free surface of water. Calculate the ratio of discharges through these orifices | CO1 | A | 1 |
| 5. | Explain briefly the cavitation that could happen in a venturi meter? No more than 4 lines are required for the explanation. | CO4 | U | 1 |
| 6. | In a steady 2D incompressible flow, if u = 2x, then derive the most general form of v. | CO2 | An | 1 |
| 7. | Briefly describe the main observation by Prandtl based on which the boundary layer equations are developed. No more than 4 lines are required for the explanation. | CO2 | U | 1 |
| 8. | Describe what is meant by hydrodynamically fully developed flow. | CO2 | R | 1 |
| 9. | Differentiate the Darcy friction factor and Fanning friction coefficient. | CO5 | A | 1 |
| 10. | An experiment is to be conducted in a laboratory to simulate the movement of a sphere shaped torpedo under water moving at a velocity of 1.6m/s. In the laboratory, the experiment can be conducted in a wind tunnel and tested with air, which is 750 times less dense and 60 times less viscous than water. The laboratory has only a sphere of twice the diameter of the torpedo available for testing. What should be the velocity of air in the wind tunnel? | CO6 | A | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | A vertical tube of diameter 5 cm closed at its bottom and open at the top contains a liquid and is rotated about its vertical axis at 700 rpm. Calculate the difference in height between the lowest and highest point of the free surface. | CO3 | A | 3 |

| ‘12. | (a) What conservation law does Bernoulli’s equation represent? (b) What are the conditions under which Bernoulli’s equation can be used? (c) What additional condition is required to use Bernoulli’s equation to calculate the pressure difference along the radius in a flow that has a streamline pattern represented by concentric circles? | CO1 | U | 3 |
| --- | --- | --- | --- | --- |
| 13 | A certain two-dimensional flow field is given by V=4xyi+2(x2-y2)j. Show and explain that a stream function should exist for this flow. Calculate the stream function and sketch the streamline through the origin and the shape of other streamlines. Show that a velocity potential should exist for this flow. Finding velocity potential will be a waste of time and will not get credit if you do not write the reason why it should exist. | CO2 | An | 3 |
| 14. | For a two-dimensional steady flow the velocity potential is given by ϕ=53x3-5xy2. Is this a valid velocity potential? Is this flow rotational? Determine the stream function? Show whether this flow is incompressible or compressible. | CO2 | An | 3 |
| 15. | Express the most general law of conservation of mass in differential form. If the flow is steady, what equation will result? Will the resulting equation represent a flow that is compressible or incompressible? ii) If the flow is incompressible, what equation will result? Can this flow be unsteady? If yes, which quantities would vary with time? | CO5 | E | 3 |
| 16. | A Newtonian fluid having a specific gravity of 0.92 and a kinematic viscosity of 5 x 10-4m2/s flows past a flat plate. The velocity profile is given by uU=32yδ-12yδ3. Usingthe momentum integral method the local boundary layer thickness for this velocity profile is given by the equation δx=4.64Rex, where the local Reynolds number is Rex=ρUxμ and x is the distance along the plate. Express the skin friction coefficient as a function of Rex. Calculate the shear stress at the plate surface at a point where the boundary layer thickness, δ is 1 mm when the free stream velocity U = 10 m/s. | CO6 | U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | Oil of specific gravity 0.83 flows in the pipe shown in Figure. If viscous effects are neglected, calculate the flow rate? 7.5 cm 10cm 10 cm |  | CO1 | A | 12 |
| 18. | Water flows steadily through the large tanks as shown in figure and the levels are maintained. Determine the water depth ha in the first tank. List the assumptions and mark the assumptions on the diagram if applicable. |  | CO4 | A | 12 |
| 19. | Using a combination of potential flows, create a flow around a circular cylinder. Find an expression for the pressure coefficient, (ps -po)/(0.5ρU2), where ps is the surface pressure, po is the pressure far away from the cylinder, ρ the density of the fluid and U the velocity far away from the cylinder. Sketch a plot of the pressure coefficient with angle θ along the surface of the cylinder and show d’Alembert’s paradox on the sketch. |  | CO2 | A | 12 |
| 20. | Discuss in detail the stability of floating and immersed bodies and the requirements to be met, using diagrams and appropriate definitions. |  | CO3 | U | 12 |

| 21. | A jet of water having a velocity of 20 m/s hits a curved vane on a wheel, which is moving with a linear velocity of 10 m/s. The jet makes an angle of 20o with the direction of motion of the vane at inlet and leaves at an angle of 130o to the direction of motion of the vane at outlet. Calculate the vane angles at inlet and outlet, so that the entry and exit of water is smooth and without any shocks. Calculate the work done per second per unit weight of water striking the vane per second. | CO4 | An | 12 |
| --- | --- | --- | --- | --- |
| 22. | Calculate the pressure drop per unit length inside along pipe through which hot air is flowing at the rate of 0.001kg/s. The diameter of the pipe is 0.15 m. Write any assumptions that you are using and justify the assumptions, particularly about the axial distance at which you are doing these calculations. | CO5 | E | 12 |
| 23. | State the Buckingham’s Pi Theorem. Answer either (a) or (b) below (a) Water sloshes and bounces back and forth in a tank as it is being moved along the road. The frequency of sloshing, ω times per second, is a function of the acceleration of gravity, g, the average depth of the water, h, when the water is still, and the length of the tank, l. Develop a set of dimensionless parameters for this problem, using g and l as repeating variables. (b) Demonstrate the derivation of the non-dimensional groups for the law of conservation of momentum by using the standard application procedure of the theorem or from a governing equation. | CO6 | U | 12 |
|  | Compulsory: |  |  |  |
| 24. | In a 2-dimensional flow V=-xti+ytj and ρ=kxt, where t, ρ are time (s), density (kg/m3) and k is a constant and is equal to 1 kg/s/m4. (a) What is the rate of change of density recorded by an observer at a point (1,1) at t = 1 (b) What is the rate of change of density of a fluid particle at (1,1) at t = 1 (c) Is this a valid flow field? (d) Is the flow steady? (e) Calculate the divergence of velocity (f) Explain the physical significance of divergence (g) Is this flow compressible? (h) If the divergence of velocity is zero, can we make any conclusions on the steadiness or the incompressibility of the fluid? If yes or If no, why? (i) What additional condition is required to ensure that the flow is incompressible? (j) What more is required to ensure that the flow is steady? | CO2 | An | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Know the properties of different fluids and pressure measurements. |
| CO2 | Apply mathematical knowledge to predict the properties and characteristics of a fluid. |
| CO3 | Understand the nature of buoyancy of submerged and floating bodies. |
| CO4 | Attain the Knowledge of flow measurement systems. |
| CO5 | Estimate the friction factor of pipe flow and losses associated it. |
| CO6 | Get knowledge of the non-dimensional parameters used in airflow. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 2 | 4 | 13 |  |  |  | 19 |
| CO2 | 1 | 1 | 12 | 19 |  |  | 33 |
| CO3 |  | 12 | 3 |  |  |  | 15 |
| CO4 |  | 1 | 12 | 12 |  |  | 25 |
| CO5 |  | 3 | 1 |  | 15 |  | 19 |
| CO6 |  | 12 | 1 |  |  |  | 13 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 20AE2005 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # STRENGTH OF MATERIALS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | Define Poisson’s ratio. | CO1 | R | 1 |
| 2. | Give example for a case where there exist a stress without strain. | CO1 | U | 1 |
| 3. | List the two types of supports used in beams. | CO2 | U | 1 |
| 4. | Draw the shear stress distribution across the rectangular cross section beam subjected to bending. | CO3 | R | 1 |
| 5. | Define neutral axis in beam bending. | CO3 | U | 1 |
| 6. | Slope at the fixed end of a cantilever beam subjected to a point load at the free end is __________. | CO4 | R | 1 |
| 7. | When a closely-coiled helical spring is subjected to an axial load, it is said to be under __________. (torsion / bending) | CO5 | U | 1 |
| 8. | Write the torque equation and explain the terms involved in it. | CO5 | R | 1 |
| 9. | The radius of Mohr’s circle is equal to __________ (maximum shear stress / maximum normal stress) | CO6 | U | 1 |
| 10. | Write the expression to find the longitudinal stress induced in a thin cylinder of thickness‘t’ subjected to internal pressure ‘P’. | CO6 | R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | A circular rod 100mm in diameter elongates 0.05mm over a length of 1000mm under a load of 50 kN. Determine the Young’s modulus for the material of the rod. | CO1 | U | 3 |
| 12. | Explain about fixed support with an example. | CO2 | U | 3 |
| 13. | State the assumptions made in theory simple bending. | CO3 | R | 3 |
| 14. | Discuss the boundary conditions of a cantilever beam subjected to a uniformly distributed load over the entire length. | CO4 | U | 3 |
| 15. | Write the expression for the energy stored and stiffness of the spring in terms of the load ‘W’ applied and the deflection ‘δ’ of the spring. | CO5 | R | 3 |
| 16. | Define principal plane and principal stress. | CO6 | U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. |  | A circular rod of diameter 16 mm and 500 mm long is subjected to a tensile force 40 kN. The modulus of elasticity forsteel may be taken as 200 kN/mm2. Find stress, strain and elongation of the bar due to applied load. | CO1 | A | 12 |
| 18. |  | Draw the shear force and bending moment diagram for the simply supported beam as shown in Fig. | CO2 | A | 12 |
| 19. |  | An I-section beam has two flanges of each 100 mm wide and 20 mm thick and web 120 mm high and 20 mm thick. If the section is subjected to a bending moment of 10 kNm, find the values of maximum bending stress. | CO3 | A | 12 |
| 20. |  | Determine the maximum deflection of a cantilever beam subject to the uniformly distributed load q, as shown in Fig. | CO4 | A | 12 |
| 21. |  | A solid shaft is subjected to a torque of 15 kNm. Find the minimum diameter of the shaft, if the allowable shear stress is 6 kN/cm2 and the allowable twist is 1o for every 260 cm of length of the shaft. (G = 8 x 106 N/cm2) | CO5 | A | 12 |
| 22. |  | Draw the shear force and bending moment diagram for the simply supported beam as shown in Fig. | CO1 | A | 12 |
| 23. |  | A truck weighing 20 kN and moving at 2.5 km/hr has to be brought to rest by a buffer. Find how many springs of 15 coils will be required to store the energy of motion during compression of 150 mm. The diameter of the wire is 20 mm and mean radius of curvature for the coil is 100 mm. Assume modulus of rigidity G = 80 GN/cm2. | CO5 | A | 12 |

|  |  | Compulsory: |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 24. |  | At a point in a strained material, the state of stress is as shown in figure. Determine both theoretically and graphically (Mohr’s circle) (i) principal stresses, (ii) location of principal planes and (iii) maximum shear stress and its location. | CO6 | A | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Describe the characteristics of conventional metals. |
| CO2 | Understand the effect loads acting at different sections of the beam. |
| CO3 | Calculate the stresses developed in beams. |
| CO4 | Compare different methods of beam deflection. |
| CO5 | Analyze the stresses developed in the shaft and spring. |
| CO6 | Analyze the states of stress in a 2D oblique plane. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 1 | 4 | 24 | - | - | - | 29 |
| CO2 | - | 4 | 12 | - | - | - | 16 |
| CO3 | 4 | 1 | 12 | - | - | - | 17 |
| CO4 | 1 | 3 | 12 | - | - | - | 16 |
| CO5 | 4 | 1 | 24 | - | - | - | 29 |
| CO6 | 1 | 4 | 12 | - | - | - | 17 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 20AE2007 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ENGINEERING THERMODYNAMICS | # Max. Marks : | # 100 |

Property Tables should be allowed into the examination hall

| Q. No. | Questions |  | Course Outcome |  | Pattern | Marks |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | How does the specific volume of water change during phase transformation from ice to water? How does the specific volume of water as temperature increases or decreases from 4oC |  |  | CO1 | U | 1 |
| 2. | Phase transformation from liquid in a vessel to vapor happens when the vapor pressure is ___________________________ atmospheric pressure |  |  | CO1 | U | 1 |
| 3. | For a given temperature T and pressure P, if P < Psat(T) the state of the substance is ______________ For a given temperature T and pressure P if P > Psat(T) the state of the substance is __________________ |  |  | CO2 | U | 1 |
| 4. | Given Tds = dh – vdp, the slope of the h-s curve at constant p will give ___________ |  |  | CO2 | A | 1 |
| 5. | Given a substance at temperature T and specific volume v, and if v > vg (Tsat = T) then state of the substance is ______________ Given a substance at temperature T and specific volume v, and if v < vg (Tsat = T) then state of the substance is ______________ |  |  | CO2 | U | 1 |
| 6. | When does a real gas behave like an ideal gas? Calculate the specific volume of Nitrogen at 7.78 MPa and 378.6 K. Critical Temperature and Critical Pressure of Nitrogen are 126.2 K and 3.39 MPa. Characteristic Gas constant of Nitrogen = 0.297(kJ/kg K) and Molecular Weight of Nitrogen = 28 kg/kmol |  |  | CO4 | A | 1 |
| 7. | Differentiate between first law efficiency and second law efficiency? Explain with an example of an engine? |  |  | CO4 | U | 1 |
| 8. | If an ideal gas is allowed to undergo a throttling expansion process at constant enthalpy, the temperature _________________ If a real gas is allowed to undergo a throttling expansion process at constant enthalpy, list the possible outcomes |  |  | CO4 | U | 1 |
| 9. | The Clausius-Clapeyron equation is used for calculating ____________ |  |  | CO4 | R | 1 |
| 10. | The regenerative Rankine cycle is used to ____________ of the cycle. The reheat Rankine cycle is used to ________________ of the cycle. |  |  | CO6 | U | 1 |

|  |  | PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 11. | Imagine that you are using a pressure cooker with water in Coimbatore where the ambient pressure is 100 kPA. While on the stove, the pressure cooker has the lid screwed on tight. A small opening with A = 5 mm2 is covered with a pressure cock (called a “weight” commonly) that lifts off when pressure exceeds a pre-decided value and steam escapes. Meat is to be cooked at 120oC for it to be softened and also all the bacteria are killed so that it does not cause any infection to the consumer. You are taking this pressure cooker to Ooty, where the ambient pressure is 95 kPA. In order to cook meat at Ooty and still cook at 120oC, will you increase or decrease the mass of the pressure cock. Justify your answer. What should be the minimum mass of the pressure cock in Coimbatore and Ooty? Assume that the change in the acceleration due to gravity between Ooty and Coimbatore is negligible. |  | CO1 | An | 3 |
| ‘12. | How are the two specific heats of gases defined? Derive Mayer’s relation between specific heats and the Universal Gas Constant for an ideal gas? |  | CO1 | A | 3 |
| 13 | The following expression is used for a thermodynamic system for the differential of pressure, p (T is temperature, v is specific volume, a, b, and R are constants) dp=Rv-bdT-RTv-b2-2av3dv Show that p is a property |  | CO2 | An | 3 |
| 14. | In order to improve the maximum efficiency of an ideal heat engine that works between heat source at T1 and heat sink at T2 one has a choice of altering the source or sink temperatures. What would be your choice? Justify your answer mathematically? |  | CO5 | An | 3 |
| 15. | Calculate the available energy in the heat energy of 1000 kJ in reservoir at 800 K. The surrounding is at 300 K. Can one expect a heat engine to develop 500 kJ of work? Can one expect a heat engine to develop 700 kJ of work? Justify both your answers |  | CO5 | An | 3 |
| 16. | Calculate the latent heat of vaporization of steam at 100oC. Assume linear variation of saturation pressure with saturation temperature in the interval of 95oC and 105oC. |  | CO4 | E | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | An R-12 heat pump cycle shown in figure has a refrigerant flow rate of 0.05 kg/s with a 4 kW input into the compressor. The following data are given. Calculate the heat transfer in the compressor, the condenser and in the evaporator. Identify heat transfer from and to the system correctly. State; 1; 2; 3; 4; 5; 6. P, kPA; 1250; 1230; 1200; 320; 300; 290. T, oC; 120; 110; 45; 0; 5. h, kJ/kg; 260; 253; 79.7; 188; 191. | CO3 | A | 12 |
| 18. | Carbon-di-oxide enters a steady state steady flow heater at 300 kPa, 650 K and exits at 275 kPA, 1500 K. Changes in kinetic and potential energies are negligible. Calculate the required heat transfer per kilogram of Carbon di-oxide, flowing through the heater. Make necessary assumptions to simplify the calculations, but justify them. If there is a more accurate way of calculation, briefly mention the method. Need not calculate. |  | CO3 | E | 12 |
| 19. | A finite heat sink and heat source of identical heat capacity of 200 J/K are available at 20oC and a 200oC respectively. If you run a heat engine between these, - What is the maximum work output possible? - What would be the temperature of the source after the maximum work is extracted? - Will the engine continue to run or stop after some time? If yes, why? If not, why not? - If you allow free heat exchange of heat between the source and sink without any external work interaction what is the lowest temperature that the source can be brought to? At that time, what would the temperature of the sink be? |  | CO4 | A | 12 |
| 20. | A fluid undergoes a reversible adiabatic compression from 0.5 MPa, 0.2 m3 to 0.05 m3 according to the polytropic process pv1.3 =constant. Determine the change in enthalpy, internal energy and entropy, and the heat transfer and work transfer during the process. |  | CO5 | E | 12 |

| 21. | At an island in the Indian Ocean, at a distance of 300 metres from the shore, the depth of water is 500 metres. The surface temperature of water at this distance is 20oC. Because of this sharp bathymetry curve (depth vs distance curve) the difference in temperature of water between the surface and the ocean floor is 15oC, at such a short distance. A power plant that utilizes this temperature difference could be built at the shore itself with pipelines pumping the water through this short distance. Determine the maximum efficiency of this plant? If the heat transfer at the source and sink needs a 2oC temperature difference to transfer heat to and from the working fluid of the power plant respectively, by what percentage will the maximum efficiency change? Is it an increase or decrease? Demonstrate the principle of increase of entropy for heat transfer through a temperature difference. What kind of irreversibility is this heat transfer? Depict the change of efficiency of the power plant engine above for the same heat input in a T-s diagram. |  | CO4 | An | 12 |
| --- | --- | --- | --- | --- | --- |
| 22. | Prove that the enthalpy of an ideal gas has to be a function of temperature only |  | CO2 | A | 12 |
| 23. | - Draw the schematic of a vapor compression and a vapor absorption cycle refrigeration cycle/ - Draw the TS and PV diagram for the vapor compression cycle - When are vapor absorption systems preferred? - What cycle is used in an air refrigeration system used in aircrafts? Draw the TS and PV diagram. - What are cogeneration and combined cycle plants? |  | CO6 | R | 12 |
|  |  | Compulsory: |  |  |  |
| 24. | A simple steam power plant working on the Rankine cycle uses solar energy for heat input instead of a boiler. Water in the cycle enters the pump as saturated liquid at 40oC, and is pumped to 2 bar (abs) and fed to the boiler. Steam enters the turbine as saturated vapor. At the turbine exhaust, the conditions are 40oC and 10% moisture. The flow rate is 150 kg/hr. (a) Sketch the cycle on a T-s diagram (b) the pump work, turbine work and net work output (c) Calculate the heat input required (d) Calculate the cycle efficiency (e) Calculate the isentropic efficiency of the turbine |  | CO6 | E | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the basic concepts of thermodynamics, laws of thermodynamics and types of work and heat interactions |
| CO2 | Evaluate the properties of pure substances, ideal gases and real gases from property tables or state equations |
| CO3 | Apply the 1st law of thermodynamics for closed and open systems undergoing different thermodynamic processes and cycles |
| CO4 | Understand the concept of entropy and properties of pure substances and real gases |
| CO5 | Perform energy calculations of engineering systems and analyze the feasibility of the processes undergone by the systems |
| CO6 | Evaluate the efficiency of efficiency and co-efficient of performance of thermal systems and vapor power cycles |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 |  | 2 | 3 | 3 |  |  | 8 |
| CO2 |  | 2 | 13 | 3 |  |  | 18 |
| CO3 |  |  | 12 |  | 12 |  | 24 |
| CO4 | 1 | 2 | 13 | 12 | 3 |  | 31 |
| CO5 |  |  |  | 6 | 12 |  | 18 |
| CO6 |  | 1 |  |  | 24 |  | 25 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 20AE2011 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # AEROSPACE STRUCTURES - I | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | Rectangular shape of the frame offers great rigidity. (True/ False) | CO1 | R | 1 |
| 2. | Number of equilibrium equations required to solve a 2D truss is ___________. | CO1 | U | 1 |
| 3. | State whether the fixed-fixed beam is a determinate or indeterminate structure. | CO2 | U | 1 |
| 4. | Write the expression for the strain energy induced in a beam subjected to a moment M. | CO3 | R | 1 |
| 5. | A propped cantilever of uniform flexural rigidity is loaded as shown in Fig. The bending moment at fixed end is a ____________ (Hogging/ Sagging) moment. | CO3 | U | 1 |
| 6. | Force in the members of a redundant truss is not dependent on the cross sectional area. (True/ False) | CO4 | U | 1 |
| 7. | State the end condition of the column for which the critical load is maximum. | CO5 | U | 1 |
| 8. | An ideal column fails by buckling. (True/ False) | CO5 | R | 1 |
| 9. | Name the failure theories which give satisfactory results for brittle materials. | CO6 | R | 1 |
| 10. | Maximum principal stress theory is not good for brittle materials. (True/ False) | CO6 | U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | State the assumptions in truss analysis. | CO1 | R | 3 |
| 12. | Explain the degree of indeterminacy in a beam with an example. | CO2 | U | 3 |
| 13. | State Castigliano's second theorem and write its expression in mathematical form. | CO3 | R | 3 |
| 14. | Define slenderness ratio. What is its significance? | CO5 | U | 3 |
| 15. | Explain the three different states of equilibrium in buckling with analogy. | CO5 | U | 3 |
| 16. | When a brittle material is subjected to torque, failure occurs at 45o to the longitudinal direction. Justify the statement. | CO6 | U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. |  | Find the forces in all the members of the truss shown in Fig. by method of joints. | CO1 | A | 12 |
| 18. |  | A beam simply supported at ends A and C and is continuous over the support B. Assuming constant EI, draw the shear force and bending moment diagram and also determine the support reactions using Clapeyron's three moment theorem. | CO2 | A | 12 |
| 19. |  | Prove that the vertical deflection at point C of a cantilever beam as shown in Fig. is P6EI2L3-3a2L+a3 using Castigliano's theorem. | CO3 | A | 12 |

| 20. |  | Determine the member forces and vertical deflection of point A for the truss shown in Fig. Assume that all the members have the same cross section and same modulus of elasticity. | CO4 | A | 12 |
| --- | --- | --- | --- | --- | --- |
| 21. |  | The cross section of a hinged-hinged column of length 1000 mm is shown in Fig. Determine the critical buckling load of the column. | CO5 | A | 12 |
| 22. |  | Determine the forces in the members of the truss shown in Fig. | CO1 | A | 12 |

| 23. |  | Determine the maximum deflection and slope for the cantilever beam as shown in figure using dummy load method. | CO3 | A | 12 |
| --- | --- | --- | --- | --- | --- |
|  |  | Compulsory: |  |  |  |
| 24. |  | A bar with a circular cross-sectional area is made of SAE 1045 carbon steel having a yield stress of σY = 1000 MPa. If the bar is subjected to a torque of 3.75 kNm and a bending moment of 7 kNm, determine the required diameter of the bar according to the maximum-distortion-energy theory. Use a factor of safety of 2 with respect to yielding. | CO6 | A | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Determine the forces of each member in a truss. |
| CO2 | Analyze statically indeterminate beam under different support/ loading conditions. |
| CO3 | Find the deflection of an elastic structure based on strain energy of the structure. |
| CO4 | Analyze the indeterminate trusses using energy method. |
| CO5 | Compare the buckling of columns with different support conditions. |
| CO6 | Predict failure of the structures made of conventional metals. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 4 | 1 | 24 | - | - | - | 29 |
| CO2 | - | 4 | 12 | - | - | - | 16 |
| CO3 | 4 | 1 | 24 | - | - | - | 29 |
| CO4 | - | 1 | 12 | - | - | - | 13 |
| CO5 | 1 | 7 | 12 | - | - | - | 20 |
| CO6 | 1 | 4 | 12 | - | - | - | 1 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 20AE2012 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # PROPULSION-I | # Max. Marks: | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | Define the second law of thermodynamics. | CO1 | R | 1 |
| 2. | Mention any four assumptions in ideal cycle analysis. | CO1 | R | 1 |
| 3. | State the factors influencing the thrust with increase in altitude. | CO2 | U | 1 |
| 4. | Mention the purpose of intercooler in modified Brayton cycle. | CO2 | U | 1 |
| 5. | Define annulus loss in a axial compressor. | CO4 | R | 1 |
| 6. | Draw the velocity triangle for axial flow compressor. | CO4 | U | 1 |
| 7. | State the function of dome in combustion chamber. | CO5 | A | 1 |
| 8. | Mention how the aviation fuel differs from other fuels. | CO5 | A | 1 |
| 9. | Liner is a thin metal shells extending from the dome to the turbine nozzle for control of combustion and dilution air jets and cooling air film. (State the above statement is True or False). | CO3 | R | 1 |
| 10. | State the various types of blade cooling. | CO4 | R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | A gas turbine operates on a pressure ratio of 5. The inlet air temperature to the compressor is 290 K and the air entering the turbine is at a temperature of 677 C. Assume that the cycle operates under ideal condition. Determine compressor exit temperature in KelvinTake k = 1.4, Cp = 1.005kJ/kg K | CO1 | A | 3 |
| 12. | Match the following based on application of each engines i. Turbofan Hypersonic transport ii Turboprop Medium and long range transport iii. Ramjet Short and medium range transport | CO1 | A | 3 |
| 13. | State the importance of pre-whirl in the inlet of the compressor. | CO4 | A | 3 |
| 14. | In short, explain rotating stall in axial flow compressor. | CO4 | R | 3 |
| 15. | State the mechanism of the catalyst influencing the reaction rate. | CO5 | U | 3 |
| 16. | Differentiate between high pressure and low pressure turbine. | CO4 | U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. |  | A gas turbine operates on a pressure ratio of 4.5. The inlet air temperature to the compressor is 280 K and the air entering the turbine is at a temperature of 677 ̊C. If the volume rate of air entering the compressor is 280 m3/s. Calculate the net power output of the cycle in MW. Also compute its efficiency. Assume that the cycle operates under ideal condition | CO1 | An | 12 |
| 18. |  | In a gas turbine plant air enters the compressor at 1 bar and 7 C. It is compressed to 4 bar with an isentropic efficiency of 82%. The maximum temperature at the inlet to the turbine is 800 C. The isentropic efficiency of the turbine is 85%. The calorific value of the fuel used is 43.1 MJ/kg. Calculate the following 1.Compressor work in KJ/kg 2. Heat supplied in KJ//kg 3. Turbine work in KJ/kg 4. Net-work in KJ/kg 5.Thermal efficiency 6. Air/Fuel ratio 7. Specific fuel consumption in kg Take Cpa = 1.005 kJ/kg K, ϒa = 1.4, Cpg = 1.147 kJ/kg K,ϒg = 1.33 | CO2 | An | 12 |
| 19. |  | Determine the impeller diameters and the width at the impeller exit and the power required to drive the compressor, from the following given data Speed N = 12,500 rev/min Mass flow rate = 15 kg /s Pressure ration = 4:1 Isentropic efficiency = 75 % Slip factor =0.9 Flow coefficient at the impeller exit = 0.3 Hub diameter of the eye = 15 cm Axial velocity of air at the entry to and exit from the impeller = 150 m/s Stagnation temperature at the inlet = 295 K Stagnation pressure at inlet = 1 bar Assume equal pressure ratio in the impeller and diffuser.. | CO4 | An | 12 |
| 20. |  | Air at a temperature of 290 K enters a ten stage axial flow compressor at the rate of 3 kg/s. The pressure ratio is 6.5 and the isentropic efficiency is 90%, the compression process being adiabatic. The compressor has symmetrical blades. The axial velocity of 110 m/s is uniform across the stage and the mean blade speed of each stage is 180 m/s. Determine the direction of the air at the entry to the exit from the rotor and the stator blades and also the power given to the air. Assume Cp = 1.005 kJ/kg K and ϒ= 1.4 | CO6 | An | 12 |

| 21. | a. | With a neat sketch explain the various sections of turbojet engine. | CO3 | U | 8 |
| --- | --- | --- | --- | --- | --- |
|  | b. | Mention in detail how does the practical gas turbine cycle differ from ideal cycle. | CO1 | U | 4 |
| 22. | a. | List the requirement of a good atomizer. | CO3 | R | 4 |
|  | b. | With the help of a neat sketch explain the breakup mechanism for a swirl injector. | CO3 | R | 8 |
| 23. |  | Briefly explain the flow through the centrifugal compressor components with necessary sketch. i. Inlet casing ii. Inducer iii. Impeller iv. impeller channel | CO3 | A | 12 |
|  |  | Compulsory: |  |  |  |
| 24. | a. | Differentiate between impulse stage and reaction stageturbine. | CO4 | A | 4 |
|  | b. | Explain in detail velocity compounded impulse stage turbine and pressure compounded impulse stage turbine. | CO4 | A | 8 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the performance of air breathing engines |
| CO2 | Analyse the performance of different propulsion cycles. |
| CO3 | Understand the working of sub-systems of the propulsion system. |
| CO4 | Assess the performance of compressor and turbine |
| CO5 | Evaluate the performance of combustion chamber, cooling and afterburner |
| CO6 | Find the causes of under-performance and remedial measures |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 2 | 4 | 6 | 12 | - | - | 24 |
| CO2 | -- | 2 | -- | 12 |  |  | 16 |
| CO3 | 13 | 8 | 12 | -- | - | - | 33 |
| CO4 | 4 | 3 | 15 | 12 |  |  | 34 |
| CO5 | -- | 3 | 2 | - | - | - | 5 |
| CO6 | -- | -- | -- | 12 | - | - | 12 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 20AE2014 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # AIRPLANE PERFORMANCE | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | List the basics sources of aerodynamic forces of any body. | CO1 | R | 1 |
| 2. | Define Aerodynamic Centre. | CO1 | # R | 1 |
| 3. | Draw a typical flight path for passenger aircraft. | CO2 | R | 1 |
| 4. | Write the equation for propulsive efficiency as a function of velocity ratios. | CO2 | U | 1 |
| 5. | A Gulf Stream-II aircraft cruising at 10 KM above the sea level. List the major types of forces acting on this aircraft. | CO3 | A | 1 |
| 6. | Write the equations of motion for unaccelerated-level flight. | CO3 | U | 1 |
| 7. | Mention the difference between absolute and service ceiling. | CO4 | An | 1 |
| 8. | Write the variation of Rate of climb at maximum climb angle with altitude. | CO4 | U | 1 |
| 9. | Mention the criterion for obtaining the largest possible turn rate. | CO5 | U | 1 |
| 10. | Write the equation for banking angle as a function of load factor. | CO5 | R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Draw the airfoil nomenclature. Using this sketch define the chamber and chord. | CO1 | U | 3 |
| 12. | Provide a brief explanation on rotational and vortex noise in a propeller driven aircraft. | CO2 | R | 3 |
| 13. | A CJ-1 Cessna cruising at 28,000 ft above the sea level with gross weight 2950 lb, drag due to zero lift is 0.22, AR is 6 (e =0.95). For this aircraft, find the ratio of CL3/2CDmaxCLCDmax. | CO3 | An | 3 |
| 14. | An aircraft with turbojet engine is cruising at 30,000 ft above the sea level. During the cruise the flight experienced the coefficient drag zero lift and drag due to lift coefficient are 0.015 and 0.08, respectively. In the case of power plant failure, calculate the minimum glide path angle required for the aircraft to cover the maximum range for safe landing. | CO4 | An | 3 |
| 15. | Explain Limiting and ultimate load factor. | CO5 | R | 3 |
| 16. | Define the take-off distance with the help of neat sketch. | CO6 | U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | a. | Explain critical Mach and drag-divergence Mach number with neat sketch. How could you delay the critical Mach and drag-divergency Mach number for military aircraft? | CO1 | A | 6 |
|  | b. | Write the significance of High-lift devices. Explain the working phenomenon of the following high-lift devices with appropriate sketches. - Split Flap - Double Slotted Flap | CO1 | U | 6 |
| 18. | a. | A propeller-driven aircraft with 2 blade and has the following characteristics: Shaft Horse Power (SHP) is 500 hp, N = 2,800 rpm, Diameter of propeller is 7.2 ft, design Coefficient of propeller blade airfoil is 0.50, Activity Factor is 120. Predict the following performance characteristics of an aircraft. [For two blade propeller, CT/CP at AF 120 and CLi 0.50 is 2.32], [For four blade propeller, (CT/CP)B=4 is 2.43] - Coefficient of Power - Coefficient of thrust - Static Thrust | CO2 | An | 7 |
|  | b. | For turbojet engines, the thrust increases with increasing the Mach number above 1 (or supersonic speed regimes). Explain the possible reasons for this characteristic. | CO2 | A | 5 |
| 19. | a. | Derive the expression for velocity at Thrust required minimum. | CO3 | U | 8 |
|  | b. | An aircraft is flying at 29,500 ft above the sea-level with the following flight conditions: Gross weight is 73,000 lb, wing surface area is 950 ft2, Cd,0 = 0.015, k=0.08. Find the minimum thrust needed to overcome the drag induced by the aircraft and velocity of the aircraft at that minimum thrust condition. | CO3 | An | 4 |
| 20. | a. | Prove that the endurance is a function of CL3/2CDfor propeller driven aircraft, andLDfor Jet-propelled aircraft. | CO4 | U | 8 |
|  | b. | A Boeing-737 took-off from Delhi airport with gross weight 80,000 lb to its destination Coimbatore. For this range (Delhi-Coimbatore) the thrust specific fuel consumption is 0.7 lb of fuel consumed per pound of thrust. The total fuel consumed for this flight journey was 30,000 lb. During the flight, the coefficient of drag due to zero-lift and drag due to lift coefficient are 0.016 and 0.09, respectively. Calculate the maximum endurance this aircraft could attain during the flight. | CO4 | An | 4 |
| 21. |  | Derive the expressions for the following conditions for a level-flight. - Velocity at turn radius minimum. - Load factor at turn radius minimum. - Minimum turn radius. | CO5 | R | 12 |

| 22. | a. | Show that velocity at CL3/2CDmax= 0.76*Velocity at CLCDmax | CO3 | U | 6 |
| --- | --- | --- | --- | --- | --- |
|  | b. | Show that the velocity at CL1/2CDmaxequals to the 31/4time the velocity at thrust required minimum. | CO3 | U | 6 |
| 23. |  | Derive the equation for calculating the maximum rate of climb for a jet-propelled aircraft. | CO4 | R | 12 |
|  |  | Compulsory: |  |  |  |
| 24. | a. | Derive the mathematics expression for the following events during the landing of an aircraft with suitable sketches. - Approach Distance - Flare Distance | CO6 | R | 7 |
|  | b. | An Airbus-320 is cleared for landing at Coimbatore airport (air-density is 0.002235 slug/ft3), while landing the aircraft has maintain the maximum lift coefficient around 2.39, wing loading 76.84 lb/ft2, load factor is 1.2. The approach angle the flight has maintained during landing is 3o. Calculate the approach distance the Airbus-320 has to attain for the given flight conditions. | CO6 | An | 5 |

Standard Atmosphere data needed for solving the numerical questions:

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | ﻿Understand the preliminary design of aircraft based on the performance. |
| CO2 | ﻿Differentiate performance characteristics of jet engine from propeller engine. |
| CO3 | ﻿Estimate the performance characteristics in level Flight. |
| CO4 | ﻿Assess the climbing performance characteristics of aircraft. |
| CO5 | ﻿Estimate the turning performance characteristics of aircraft. |
| CO6 | ﻿Realize the ground effects on performance. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 2 | 9 | 6 |  | - | - | 17 |
| CO2 | 4 | 1 | 5 | 7 | - | - | 17 |
| CO3 | - | 21 | 1 | 7 | - | - | 29 |
| CO4 | 12 | 9 | - | 8 | - | - | 29 |
| CO5 | 16 | 1 | - | - | - | - | 17 |
| CO6 | 7 | 3 | - | 5 | - | - | 15 |
|  |  |  |  |  |  |  | 124 |

Reg. No.

End Semester Examination – April / May – 2022

| Code : | 20AE2016 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | INTRODUCTION TO AEROSPACE MATERIALS | Max. Marks : | 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | The most mechanical properties are affected by electrons. | CO1 | R | 1 |
| 2. | Name the material used in the construction of the ZMC-2 airship. | CO1 | R | 1 |
| 3. | Ductility represents that how easily a material gets deformed under stress. | CO2 | U | 1 |
| 4. | The Phase transitions occur along . | CO2 | U | 1 |
| 5. | The Increasing and decreasing temperature results . | CO3 | R | 1 |
| 6. | Mention the temperature range of ultra high-temperature materials. | CO3 | R | 1 |
| 7. | Define composite prepregs. | CO4 | R | 1 |
| 8. | The superior levels of strength and stiffness are given to the composite by . | CO4 | R | 1 |
| 9. | Annealing relieves in a part during cold forging or casting processes. | CO5 | U | 1 |
| 10. | Grain refining uses and to create grain nucleus sites to achieve this fine structure. | CO5 | U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Compare the primary bonding with the secondary bonding. | CO1 | R | 3 |
| 12. | Define Hardenability. | CO2 | U | 3 |
| 13. | Explain the identification of high temperature materials. | CO3 | U | 3 |
| 14. | Classify composites. | CO4 | R | 3 |
| 15. | Mention the principle of spot-welding. | CO5 | R | 3 |
| 16. | Highlight the features of Graphene. | CO6 | R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. |  | Discuss about the crystal structures of metals with the neat sketches. | CO1 | U | 12 |
| 18. | a. | Explain the standards and applications of tension test. | CO2 | U | 6 |
|  | b. | Write short notes on - Three-point bending test - Four-point bending test | CO2 | U | 3 3 |

| 19. |  | Describe the characteristics of high temperature materials. | CO3 | U | 12 |
| --- | --- | --- | --- | --- | --- |
| 20. |  | Compare and contrast the RTM with pultrusion technique. | CO4 | U | 12 |
| 21. | a. | Explain the characteristics of Austenitic Manganese Steel. | CO5 | U | 6 |
|  | b. | Discuss the manufacturing method of forged product. | CO5 | U | 6 |
| 22. |  | Explain in detail about the heat treatment of steel. | CO5 | U | 12 |
| 23. |  | Discuss about the various imperfections in crystals. | CO1 | U | 12 |
|  |  | Compulsory: |  |  |  |
| 24. |  | Explain about the selection of materials for storage of cryogenic propellants and pumps. | CO6 | U | 12 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 20AE2017 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # GAS DYANAMICS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | Mach above which the flow is assumed to be compressible - 0.1 (b) 0.2 (c) 0.3 (d) 0.25 | CO1 | R | 1 |
| 2. | Write the formula for angle of Mach cone (Mach wave) | CO1 | R | 1 |
| 3. | Diffuser for subsonic flow should be__________ (Converging/Diverging/Converging-Diverging) | CO2 | U | 1 |
| 4. | Condition of the nozzle where the expansion waves exist at the nozzle exit is called _______ (Ideal/Over-expanded/Under-expanded) | CO2 | U | 1 |
| 5. | Total pressure ____________ across the normal shock (constant/increases/decreases) | CO3 | U | 1 |
| 6. | Total pressure loss in a normal shock is due to _______ (thermal loss/frictional loss/ enthalpy loss) | CO3 | U | 1 |
| 7. | Due to friction, the Mach number inside a duct ________ (increases/decreases/constant) | CO4 | U | 1 |
| 8. | Write the formula for maximum heat transfer q* in terms of total temperatures with nomenclature. | CO4 | E | 1 |
| 9. | Changes across the oblique shock are contributed by _____ component of flow w.r.t shock. (normal/tangential) | CO5 | E | 1 |
| 10. | Write the formula for deflection (θ) through expansion waves in terms of Prandtl Meyer function. | CO5 | R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Calculate the speed of sound at 5 Km altitude where the temperature and pressure are 270K and 0.5 bar respectively. | CO1 | A | 3 |
| 12. | Calculate the exit velocity from a reservoir (P0 = 1 bar) expanding to an atmospheric pressure and temperature of 0.5 bar and 270K respectively. | CO2 | A | 3 |
| 13. | Explain the shock reflection on a wall with the diagram and indicate the changes with streamlines and local changes. | CO3 | E | 3 |
| 14. | Calculate the Maximum heat transfer possible for a flow (air) with Mach 3 and total temperature 500K. | CO4 | A | 3 |
| 15. | Explain detached shock and calculate the minimum angle of wedge to create a detached shock for Mach 3. | CO5 | U | 3 |
| 16. | Explain the working of pitot tube and how to calculate Mach number. | CO6 | E | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | a. | Derive the relation of speed of sound (a2 = γRT) from isentropic relation. | CO1 | E | 8 |
|  | b. | Calculate the Mach number of a flight flying 500 m/s at 5 Km altitude where the ambient temperature is 270K. | CO1 | C | 4 |
| 18. | a. | Explain the working of a supersonic wind tunnel and discuss various parts. | CO2 | U | 6 |
|  | b. | Calculate the exit area of ideal nozzle for supersonic exit Mach number of 2.5 if the throat area is 1 m2. | CO2 | A | 3 |
|  | c. | Calculate the exit pressure for the above nozzle if the reservoir pressure is 10 bar. | CO2 | E | 3 |
| 19. | a. | A normal shock wave is standing in the test section of a supersonic wind tunnel. Upstream of the wave, M1= 3, P1= 0.5 atm, and T1= 200 K. Find M2, P2, T2.and u2downstream of thewave. u2 – velocity behind the normal wave. | CO3 | A | 6 |
|  | b | A blunt-nosed missile is flying at Mach 2 at standard sea level. Calculate the temperature and pressure at the nose of the missile. | CO3 | E | 6 |
| 20. | a. | Air enters a constant-area duct at M1 = 0.2, P1 = 1 atm, and T1 = 273 K. Inside the duct, the heat added per unit mass is q = 1.0 x 106 kg. Calculate the flow properties M2, P2, T2, ρ2, T02, and P02 at the exit of the duct. | CO4 | A | 12 |
| 21. | a. | A supersonic flow (M=2, P1 = 1 atm, T1 = 300K) is turning away at a corner at an angle of 100. Draw the flow pattern, leading and trailing expansion waves with exact angles. | CO5 | U | 6 |
|  | b. | Calculate the downstream flow properties of pressure, temperature and density after the expansion waves. | CO5 | A | 6 |
| 22. | a. | Consider a horizontal supersonic flow at Mach 2.8 with a static pressure and temperature of 1 atm and 288K, respectively. This flow passes over a compression corner with a defection angle of 160. The oblique shock generated at the corner propagates into the flow and is incident on a horizontal wall as shown in the figure above. Calculate the angle ϕ, made by the reflected shock wave with respect to the wall, and the Mach number, pressure, and temperature behind the reflected shock. | CO5 | E | 12 |

| 23. | a. | Design an ideal supersonic nozzle of Mach 4 for inlet area as 1 m2 and Mach as 0.2. Also Calculate the total reservoir pressure for exit pressure 1 bar. | CO2 | C | 12 |
| --- | --- | --- | --- | --- | --- |
|  |  | Compulsory: |  |  |  |
| 24. | a. | Explain the components and constructions of supersonic wind tunnel with diagram. | CO6 | U | 6 |
|  | b. | Draw and explain Schlieren visualization technique setup. | CO6 | U | 6 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the influence of compressibility to distinguish between the flow regimes. |
| CO2 | Apply compressibility corrections for flow in converging-diverging passages and instruments like Pitot static tube. |
| CO3 | Estimate the sudden changes in the flow field due to normal shocks. |
| CO4 | Estimate the influence of friction and heat transfer in the flow field. |
| CO5 | Understand oblique shocks and its effect on supersonic flow fields. |
| CO6 | Choose proper flow visualization techniques for any given situation. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 2 | - | 3 | - | 8 | 4 | 17 |
| CO2 | - | 8 | 6 | - | 3 | 12 | 29 |
| CO3 | - | 2 | 6 | - | 9 | - | 17 |
| CO4 | - | 1 | 15 | - | 1 | - | 17 |
| CO5 | 1 | 9 | 6 | - | 13 | - | 29 |
| CO6 | - | 12 | - | - | 3 | - | 15 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 20AE2018 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # AIRCRAFT INSTRUMENTATION AND AVIONICS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | Define Sensitivity of an instrument. | CO1 | R | 1 |
| 2. | Define Accuracy. | CO1 | R | 1 |
| 3. | List the use of vacuum pump in Vacuum driven Aircraft Systems. | CO2 | R | 1 |
| 4. | List the use of Gyroscopes. | CO2 | R | 1 |
| 5. | Extend/Expand the abbreviation of FCP. | CO3 | U | 1 |
| 6. | Explain piezoelectric effect. | CO3 | U | 1 |
| 7. | Extend/Expand the abbreviation AHRS | CO4 | U | 1 |
| 8. | Extend/Expand the abbreviation LRUs | CO4 | U | 1 |
| 9. | List the other name for ARINC 664 | CO5 | R | 1 |
| 10. | Identify the maximum number of devices can be connected in ARINC 429 data bus. | CO5 | An | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | In a measuring device, explain how the degree of damping affects the response of an instrument. | CO1 | E | 3 |
| 12. | List three of the Gyroscope based flight Instruments. | CO2 | R | 3 |
| 13. | Explain how C type Bourdon tube is used to measure the pressure with a diagram | CO3 | U | 3 |
| 14. | Explain any three ECS – Environmental control system functionalities. | CO4 | E | 3 |
| 15. | Explain the differences between the three types of redundancy | CO5 | U | 3 |
| 16. | List the Problems with Voice Recognition - Direct Voice Input. | CO6 | R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | a. | Name Fleming’s Left-hand Rule. | CO1 | R | 4 |
|  | b. | Explain the three forces operating inside the deflection instruments. | CO1 | U | 4 |
|  | c. | Name the three types of Deflection Instruments and give an example. | CO1 | R | 4 |
| 18. | a. | State the Principle behind the working of Altimeter and its usage. | CO2 | R | 4 |
|  | b. | State the Principle behind the working of Air Speed Indicator and its usage. | CO2 | R | 4 |
|  | c. | State the Principle behind the working of Vertical Speed Indicator and its usage. | CO2 | R | 4 |

| 19. | a. | Explain the principle behind Thermocouple and explain how a quick response time can be obtained in thermocouple? | CO3 | E | 4 |
| --- | --- | --- | --- | --- | --- |
|  | b. | Explain about the function of RTD. Mention the maximum measurable temperature by RTD. | CO3 | E | 4 |
|  | c. | Explain the use of Air data computers. | CO3 | E | 4 |
| 20. | a. | Explain how Reliability characterize a system's ability. | CO4 | U | 4 |
|  | b. | Explain how Maintainability characterize a system's ability. | CO4 | U | 4 |
|  | c. | Explain how Availability characterize a system's ability. | CO4 | U | 4 |
| 21. | a. | Explain FBW system. | CO5 | U | 4 |
|  | b. | Examine LVDT- Linear Differential Transmitter usage in FBW system. | CO5 | An | 4 |
|  | c. | Name the FBW system components in a simple implementation. | CO5 | E | 4 |
| 22. | a. | Construct the topology of the AFDX switched network. | CO5 | A | 4 |
|  | b. | Discuss the advantages of AFDX switched network. | CO5 | C | 4 |
|  | c. | Discuss the disadvantages of AFDX switched network. | CO5 | C | 4 |
| 23. | a. | Construct/Draw the PFD screen and mention the display parameters. | CO5 | A | 4 |
|  | b. | Analyze any three of its indicating parameters. | CO5 | An | 4 |
|  | c. | Name the two parameter that are indicated by Turn coordinator (TC). | CO5 | R | 4 |
|  |  | Compulsory: |  |  |  |
| 24. | a. | Explain HMD with an example. | CO6 | E | 4 |
|  | b. | Explain HUD with an example. | CO6 | E | 4 |
|  | c. | Explain HDD with an example. | CO6 | E | 4 |

Reg.No.

# End Semester Examination – June –2022

| Code : | 21AE3001 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub.Name: | ADVANCED AERODYNAMICS | Max.Marks : | 100 |

| Q. No. | SubDiv. | Questions |  |  | Course Outcome /Pattern |  | Marks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PART– A (5 X16 = 80 MARKS) (Answeranyfivefrom thefollowing) |  |  |  |  |  |  |  |
| 1. | a. | Describe a mathematical condition for irrotational flow field and comment whether viscous fluids produce irrotational flow fields. |  |  | CO | 1/U | 4 |
|  | b. | Derive equation of continuity for unsteady two-dimensional flow field. |  |  | CO | 1/U | 4 |
|  | c. | For an irrotational flow a scalar potential (φ) exists. From the continuity equation [Q 1. (b)] show that ∇2φ=0. |  |  | CO | 1/U | 4 |
|  | d. | Starting from Newton’s second law obtain momentum equation for inviscid; incompressible two-dimensional flow. |  |  | CO | 1/U | 4 |
| 2. | a. | Define conditions for fully developed flow. |  |  | CO | 1/U | 4 |
|  | b. | List differences between Couette and Planar Poiseuille flow. |  |  | CO | 1/U | 4 |
|  | c. | Consider a scenario as shown in the following figure. List necessary assumptions and show that ux= Uyh |  |  | CO | 1/U | 4 |
|  | d. | Assume the fluid in the above question (2. (c)) is inviscid. Obtain an appropriate expression for velocity variation. |  |  | CO | 1/A | 4 |
| 3. |  | For boundary layer solution over a flat plate of length unity, show that Blasius equation and its corresponding boundary conditions are ff''+2f'''=0 f0= f'(0)=0 f∞= 1 |  |  | CO | 2/An | 16 |
| 4. | a. | Starting from the definition of compressibility prove that speed of sound a= γRT |  |  | CO 3/ A |  | 10 |
|  | b. | From the ration of kinetic energy to internal energy obtain Mach number relation. | , |  | CO 3/ A |  | 6 |
| 5. |  | Starting from the governing equation of normal shock obtain Prandtl’s relation a*2= u1u2 |  |  | CO | 4/U | 16 |
| 6. |  | Obtain the pressure difference relation across a normal shock. |  |  | CO5/A |  | 16 |
| 7. | a. | Formulate an appropriate representation to understand the changes in property in time and space. |  |  | CO1/U |  | 10 |
|  | b. | Show thar the Moment generated by the viscous stresses about the center of the fluid element is zero. |  |  | CO1/U |  | 6 |
| PART– B (1 X20 = 20MARKS) (CompulsoryQuestion) |  |  |  |  |  |  |  |
| 8. |  | Define Fanno flow and Rayleigh flow. Write the differences between them. Write their governing equations. |  |  | CO6/U |  | 20 |

|  | COURSEOUTCOMES |
| --- | --- |
| CO1 | Assesstheforcesand momentsdueto flow. |
| CO2 | Understandtheflowbehaviorovervarious bodyshapes. |
| CO3 | Apply compressibility corrections for flow in C-D passages and instruments like Pitotstatictube. |
| CO4 | Assessthenatureofcompressibleflowover airfoilsandfinitewings. |
| CO5 | Usethecomputationaltoolstoevaluatehypersonicflows. |
| CO6 | Understandthebasic principlesofexpansion waves. |

| AssessmentPatternas perBloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO /P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 |  | 44 | 4 |  |  |  | 48 |
| CO2 |  |  |  | 16 |  |  | 16 |
| CO3 |  | 16 |  |  |  |  | 16 |
| CO4 |  |  | 16 |  |  |  | 16 |
| CO5 |  |  | 16 |  |  |  | 16 |
| CO6 | 20 | 5 | - | - | - | - | 20 |
|  |  |  |  |  |  |  | 132 |

Reg.No. _____________

End Semester Examination – June – 2022

| # Code : | # 21AE3002 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ADVANCED STRUCTURAL ANALYSIS | # Max. Marks : | # 100 |

| Q. No. | Sub Div. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |
| 1. |  | For the given configuration, determine by both analytical and geometrical method, the normal and tangential stress on a plane at an angle 30 degree with the vertical. | CO1 / A | 16 |
| 2. |  | A massless cantilever beam PQ has a solid square cross section (10 𝑚𝑚 × 10 𝑚𝑚). This beam is subjected to a load 𝑊 through a rigid massless link at the point Q, as shown below (figure not to scale). Draw the shear force and bending moment diagram. | CO1 / A | 16 |
| 3. |  | For a graphite/epoxy unidirectional lamina, find the following 1. Compliance matrix 2. Minor Poisson’s ratio 3. Reduced stiffness matrix 4. Strains in the 1–2 coordinate system if the applied stresses are σ1 = 2 MPa, σ2 = –3 MPa, and τ12 = 4 MPa.Assume E1 = 181 GPa, E2 = 10 GPa, G12 = 7 GPa and ν12 =0.28. | CO2 / E | 16 |

| 4. |  | Determine the direct stress distribution in a thin walled Z-section produced by a positive bending moment Mx. Height of the section = h and the flange width = h/2. | CO3 / E | 16 |
| --- | --- | --- | --- | --- |
| 5. |  | Find the shear flow and twist per unit length of the structure shown in Fig. Assume G = 25x105 N/cm2 and radius R = 10 cm. | CO4 / A | 16 |
| 6. |  | The sheet stringer panel shown in Fig. is loaded in compression by means of rigid members. The sheet is assumed to be simply supported at the loaded ends and at the rivet lines and to be free at the sides. Each stringer has an area of 0.65 cm2. Assume E = 70 GPa for the sheet and stringers. Find the total compressive load P: (a) when the sheet first buckles (b) When the stringers stress is 70 MN/m2 (c) When the stringers stress is 210 MN/m2. | CO5 / An | 16 |

| 7. |  | Find the shear flow and twist per unit length of the two cell tube made of aluminium as shown Fig. and is subjected to a torque 90000 Ncm | CO4 / A | 16 |
| --- | --- | --- | --- | --- |
| PART – B (1 X 20 = 20 MARKS) (Compulsory Question) |  |  |  |  |
| 8. |  | A cantilever beam as shown in Fig. carries concentrated loads as shown. Calculate the distribution of stiffener loads and the shear flow distribution in the web panels assuming that the latter are effective only in shear. | CO6 / A | 20 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand stress and strain compatibility conditions. |
| CO2 | Derive Stress-strain relationship of a lamina. |
| CO3 | Differentiate the symmetrical and unsymmetrical bending. |
| CO4 | Determine the shear center in various open and closed section of aircraft structures. |
| CO5 | Analyze the buckling of plates to predict the critical stress. |
| CO6 | Design aircraft composite panel for aerospace applications. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | - | - | 32 | - | - | - | 32 |
| CO2 | - | - | - | - | 16 | - | 16 |
| CO3 | - | - | - | - | 16 | - | 16 |
| CO4 | - | - | 32 | - | - | - | 32 |
| CO5 | - | - | - | 16 | - | - | 16 |
| CO6 | - | - | 20 | - | - | - | 20 |
|  |  |  |  |  |  |  | 132 |

Reg. No. _____________

End Semester Examination – June – 2022

| # Code : | # 21AE3005 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ELEMENTS OF DATA ANALYTICS | # Max. Marks : | # 100 |

| Q. No. | Sub Div. | Questions | Course Outcome/ Pattern |  | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |  |
| 1. |  | A certain polymer is used for evacuation systemsfor aircraft. It is important that the polymer be resistantto the aging process. Twenty specimens of thepolymer were used in an experiment. Ten specimens were selected randomly and exposed to an accelerated batchaging process that involved exposure to high temperaturesfor 10 days. The other specimens were not subjected to any aging process.Measurements of tensile strength of the specimens were made, and the following data wererecorded on tensile strength in psi: No Aging: 227, 222, 218, 217, 225, 218, 216, 229, 228, 221 Aging: 219, 214, 215, 211, 209, 218, 203, 204, 201, 205 - Represent the data in an appropriate plot that best describes the effect of aging - From your plot, does it appear as if the aging process has had an effect on the tensile strength of this polymer? Explain. |  | CO1/ An | 16 |
| 2. | (a) | Explain with some simple sketches and data, the different graphical methods for describing and plotting data. |  | CO1 / U | 8 |
|  | (b) | With the aid of some examples, show that the choice of the graphical method of describing data, depends on the type of data |  | CO1 / A | 8 |
| 3. | (a) | List the different measures of central tendency of data and explain the information that each measure provides and the relative advantage and disadvantage of each measure. |  | CO2 / U | 8 |
|  | (b) | For the data given in Question 1 - Calculate the sample mean tensile strength and median of the two samples. - Discuss the similarity or lack of similarity between the mean and median of each group |  | CO2 / A | 8 |
| 4. | (a) | If a letter is chosen at random from the English alphabet, find (i) the probability that the letter is listed somewhere ahead of the letter j (ii) the probability that the letter is listed somewhere after the letter g |  | CO3/E | 8 |
|  | (b) | How many three-digit numbers can be formed from the digits 0, 1, 2, 3, 4, 5, and 6 if each digit can be used only once? (ii) How many of these are odd numbers? (iii) How many are greater than 330? |  | CO3/E | 8 |

| 5. |  | A California study concluded that following 7 health rules could extend a man’s life by 11 years on the average and a woman’s life by 7 years. These 7 rules are: no smoking, regular exercise, use alcohol in moderation, 7 to 8 hours of sleep, maintain proper weight, eat breakfast, and do not eat between meals. In how many ways can a person adopt 5 of these rules to follow (a) if the person presently violates all 7 rules? (b) if the person never drinks and always eats breakfast? | CO3 / E | 16 |
| --- | --- | --- | --- | --- |
| 6. |  | An automobile manufacturer is concerned about a possible recall of its best-selling car. If there were a recall, there is a probability of 0.25 of a defect in the brake system, 0.18 of a defect in the transmission, 0.17 of a defect in the fuel system, and 0.40 of a defect in some other area. (i) What is the probability that the defect is the brakes or the fueling system if the probability of defects in both systems simultaneously is 0.15? (ii) What is the probability that there are no defects in either the brakes or the fueling system? | CO4 / E | 16 |
| 7. |  | The probability distribution of x, the number of imperfections per 10 meters of a synthetic fabric in continuous rolls of uniform width, x; 0; 1; 2; 3; 4. f(x); 0.41; 0.37; 0.16; 0.05; 0.01. Find the average number of imperfections per 10 meters of this fabric | CO4 / E | 16 |
| PART – B (1 X 20 = 20 MARKS) (Compulsory Question) |  |  |  |  |
| 8. |  | Twenty adult males between the ages of 30 and 40 participated in a study to evaluate the effect of a specific health regimen involving diet and exercise on the blood cholesterol. Ten were randomly selected to be a control group, and ten others were assigned to take part in the regimen as the treatment group for a period of 6 months. The following data show the reduction in cholesterol experienced for the time period for the 20 subjects: Control group: 7, 3, −4, 14, 2, 5,22, −7, 9, 5 Treatment group: −6, 5,9, 4,4, 2, 37,5, 3, 3 - Represent the data in an appropriate plot that best describes the effect of the health regimen - From your plot, does it appear as if the health regimen has had an effect on the reduction in cholesterol? Explain - Compute the mean and median for both groups. - Explain why the difference in means and the difference in medians suggest different conclusions, about the effect of the health regimen. | CO4 / An | 20 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Find a meaningful pattern in data |
| CO2 | Graphically interpret data |
| CO3 | Implement the analytic algorithms |
| CO4 | Handle large scale analytics projects from various domains |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 |  | 8 | 8 | 16 |  |  | 32 |
| CO2 |  | 8 | 8 |  |  |  | 16 |
| CO3 |  |  |  |  | 32 |  | 32 |
| CO4 |  |  |  | 20 | 32 |  | 52 |
|  |  |  |  |  |  |  | 132 |

Reg. No. _____________

End Semester Examination – June – 2022

| # Code : | # 21AE3006 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ADVANCED PROPULSION TECHNOLOGY | # Max. Marks : | # 100 |

| Q. No. | Sub Div. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |
| 1. | a. | Briefly explain the compressor performance for the injection of water alcohol mixture. | CO1 / U | 8 |
|  | b. | A jet propelled plane consuming air at the rate of 18.2 kg/s is to fly at Mach number of 0.3 at the altitude of 4500 m (pa = 0.55 bar, Ta = 255 K). The diffuser which has a pressure coefficient of 0.9, decreases the flow to a negligible velocity. The compressor pressure ratio is 5 and max temperature in the combustion chamber is 1273 K. After expanding in the turbine, the gases continue to expand in the nozzle to a pressure of 0.69 bar. The isentropic efficiency of the compressor, turbine and nozzle are 0.81,0.85 and 0.915 respectively. The heating value of the fuel is 45870 kJ/kg. Assuming Cp = 1.005 kJ/kg K, Cpg = 1.147 kJ/kg K. ϒair = 1.4, ϒgas = 1.33, calculate the thrust power. | CO2 / A | 8 |
| 2. | a. | With necessary performance graph explain how modified Brayton cycle improves the efficiency of the cycle. | CO1 / An | 8 |
|  | b. | Explain the torque–speed characteristics at various equilibrium running point for the following scheme. a. Open Cycle gas turbine with separate power turbine. b. Only one turbine runs both compressors as well as the load | CO2 / R | 8 |
| 3. |  | Design a nozzle for an ideal rocket that has to operate at 10 km altitudeand give 5000 N thrust at a chamber pressure of 2.068 MPa and a chamber temperature of 2800 K. Assuming that k - 1.30 and R = 355.4 J/kg-K, determine the throat area, exit area, throat velocity, and exit temperature.(The pressure at 10 km altitude is 0.026151 MPa). | CO3 / C | 16 |
| 4. |  | Explain the various gel propellants formulation and state the difficulties of gel propellant in atomization and combustion process. | CO4 / E | 16 |
| 5. | a. | Draw the process flow diagram for manufacturing and assembling of solid rocket motor. | CO3 / A | 6 |
|  | b. | Explain the manufacturing and assembling process involved in solid rocket motor. | CO3 / A | 10 |

| 6. | a. | Mention the need for green propulsion and list their importance. | CO5 / U | 8 |
| --- | --- | --- | --- | --- |
|  | b. | Explain any one green propellant in detail and state their advantages and disadvantages. | CO5 / R | 8 |
| 7. | a. | Briefly explain laser beam propulsion with necessary sketch. | CO6 / A | 8 |
|  | b. | Illustrate the working principle of nuclear propulsion system. | CO6 / A | 8 |
| PART – B (1 X 20 = 20 MARKS) (Compulsory Question) |  |  |  |  |
| 8. | a. | Briefly explain the hypersonic flight corridor for scramjet. | CO4 / A | 10 |
|  | b. | With neat sketch explain the aerothermodynamics of scramjet. | CO4 / A | 10 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Illustrate the performance of various cycles of turbine engine. |
| CO2 | Estimate the performance of aircraft engines. |
| CO3 | Design the subsystems for chemical rockets. |
| CO4 | Analyse and compare the performance of chemical rockets. |
| CO5 | Design the subsystems for green propulsion systems |
| CO6 | Evaluate the performance of space thrusters. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 |  | 8 |  | 8 |  |  | 16 |
| CO2 | 8 |  | 8 |  |  |  | 16 |
| CO3 |  |  | 16 |  |  | 16 | 32 |
| CO4 |  |  | 20 |  | 16 |  | 36 |
| CO5 | 8 | 8 |  |  |  |  | 16 |
| CO6 |  |  | 16 |  |  |  | 16 |
|  |  |  |  |  |  |  | 132 |

Reg. No. _____________

End Semester Examination – June – 2022

| # Code : | # 21AE3007 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # MODELING AND SIMULATION OF AEROSPACE VEHICLES | # Max. Marks : | # 100 |

| Q. No. | Sub Div. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |
| 1. | a. | Describe about dynamic models. | CO5 / A | 8 |
|  | b. | Describe the various steps in a simulation study | CO6 / A | 8 |
| 2. | a. | Explain the simulation of an auto pilot. | CO5 / AP | 8 |
|  | b. | Discuss about a barber shop simulator. | CO5 / AP | 8 |
| 3. | a. | Describe the hardware and software of fault-tolerant system. | CO3 / U | 8 |
|  | b. | Discuss about the Federal Aviation Regulations guide certification requirements. | CO4 / E | 8 |
| 4. | a. | Describe the concept of Causal loop for representing a dynamic system with the help of an example. | CO5 / AP | 8 |
|  | b. | Describe about stocks and flows and its usefulness to represent dynamic systems. | CO1 / AP | 4 |
|  | c. | Describe the time delay representation of Dynamic systems. | CO1 / AP | 4 |
| 5. | a. | Linearize the equations of motion of a dynamic system. | CO5 / A | 8 |
|  | b. | List the various components of Aircraft system and explain about each component. | CO3 / A | 8 |
| 6. | a. | Explain about Simulation language. | CO2 / U | 8 |
|  | b. | Discuss the types of simulation model. | CO3 / U | 8 |
| 7. | a. | Describe the static models of a system. | CO1 / A | 8 |
|  | b. | Discuss about Evaluating system design and future architecture. | CO4 / A | 8 |
| PART – B (1 X 20 = 20 MARKS) (Compulsory Question) |  |  |  |  |
| 8. | a. | Explain the advantages and disadvantages of simulators. | CO2 / U | 10 |
|  | b. | Explain the Simulator certification procedures. | CO4 / U | 10 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Analyze the concepts of system models. |
| CO2 | Practice system simulation for cockpit systems. |
| CO3 | Model and design aircraft elements |
| CO4 | Comprehend the principles behind system assessment, validation and certification. |
| CO5 | Relate system dynamics and methematical models for flight simulation |
| CO6 | Relate to the usage of flight simulator for various aircrafts. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 |  |  | 8 | 8 |  |  | 16 |
| CO2 |  | 18 |  |  |  |  | 18 |
| CO3 |  | 16 |  | 8 |  |  | 24 |
| CO4 |  | 10 |  | 8 | 8 |  | 26 |
| CO5 |  |  | 24 | 16 |  |  | 40 |
| CO6 |  |  |  | 8 |  |  | 8 |
|  |  |  |  |  |  |  | 132 |

Reg. No. _____________

End Semester Examination – June – 2022

| # Code : | # 20AE3012 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # AVIATION 4.0 | # Max. Marks : | # 100 |

| Q. No. | Sub Div. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |
| 1. |  | Discuss the salient features of the following era of the aviation Industry. i) Aviation 1.0 ii) Aviation 2.0 iii) Aviation 3.0 iv) Aviation 4.0 | CO1 / U | 16 |
| 2. | a. | Explain the difference between digital twin technology and digital twin model. | CO2 / R | 6 |
|  | b. | Describe the basic feature product digital twin model. | CO2 / R | 10 |
| 3. | a. | Explain the applications of Digital Twin Technology in the following industries i) Aerospace ii) Health and Medicine iii) Agriculture iv) Construction v) Electric Power Generation. | CO3 / U | 10 |
|  | b. | Explain the Unit, System and System of Systems, and describe the evaluation of digital twin shop floor. | CO3 / R | 6 |
| 4. | a. | Describe the characteristics of Big Data | CO4 / R | 6 |
|  | b. | Elaborate the life cycle of Big Data in Manufacturing with neat diagram | CO4 / R | 10 |
| 5. | a. | Describe the importance of elements of Digital Fly-by Wire system in aircrafts | CO5 / R | 10 |
|  | b. | Discuss the triplex and quadruplex architecture for Digital Flyby Wire system | CO5 / U | 6 |
| 6. | a. | Explain the Importance of Predictive Aircraft Maintenance with suitable examples. | CO1 / A | 8 |
|  | b. | Discuss how the aircraft distress tracking system works under the Aviation 4.0 era. | CO1 / A | 8 |
| 7. | a. | Describe the applications of digital twin technology in i) Product design ii) Man-Machine Interaction iii) Process Planning. | CO2 / U | 6 |
|  | b. | Explain 5-Dimensional Digital Twin Technology with neat sketch. | CO2 / R | 10 |
| PART – B (1 X 20 = 20 MARKS) (Compulsory Question) |  |  |  |  |
| 8. | a. | Define VR and describe how it can be used in i) Aircraft maintenance ii) Preflight Inspection | CO6 / U | 10 |
|  | b. | Explain how AR different form VR and elaborate its applications in Aviation Industry. | CO6 / A | 10 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | ﻿Understand the concepts of Aviation 4.0 |
| CO2 | ﻿Articulate the usage of Digital Twin in Aviation |
| CO3 | ﻿Understand use of digital technologies in smart manufacturing. |
| CO4 | ﻿Articulate the usage of the CPS, IOT and Big data in Avionics. |
| CO5 | ﻿Illustrate the concepts of Digital Fly-By-Wire |
| CO6 | ﻿Examine applications and case studies of AR, VR & MR in Manufacturing |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 5- | 16 | 16 | - | - | - | 32 |
| CO2 | 26 | 6 | - | - | - | - | 32 |
| CO3 | 6 | 10 | - | - | - | - | 16 |
| CO4 | 16 | - | - | - |  |  | 16 |
| CO5 | 10 | 6 | - | - | - | - | 16 |
| CO6 | - | 10 | 10 | - | - | - | 20 |
|  |  |  |  |  |  |  | 132 |

Reg. No. _____________

End Semester Examination – June – 2022

| # Code : | # 21AE3014 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ARTIFICIAL INTELLIGENCE AND MACHINE LEARNING IN AEROSPACE APPLICATIONS | # Max. Marks : | # 100 |

| Q. No. | Sub Div. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |
| 1. | a. | Outline the steps to be followed to solve any problem using the machine learning algorithm. | CO1 / U | 10 |
|  | b. | What is turing test? List the features required for a machine to pass the turing test. | CO1 / R | 6 |
| 2. | a. | Explain the types of uninformed search techniques and point out the advantages and disadvantages of each technique. | CO2 / U | 10 |
|  | b. | Summarize alpha beta pruning. | CO2 / U | 6 |
| 3. | a. | Analyze the usefulness of decision support expert system in aerospace applications. | CO3 / An | 10 |
|  | b. | List the advantages and disadvantages of expert systems. | CO3 / R | 6 |
| 4. | a. | Explain support vector machine in detail with necessary illustrations. | CO4 / U | 10 |
|  | b. | Compare machine learning and deep learning techniques. | CO4 / U | 6 |
| 5. | a. | Summarize the decision tree classifier with necessary illustrations. | CO5 / U | 10 |
|  | b. | What are the steps to be done to find the class label of the test data using k-NN algorithm? | CO5 / R | 6 |
| 6. | a. | Consider five datapoints belong to positive class, five datapoints belong to negative class and an outlier (negative data point present in the positive class). Utilize logistic regression algorithm to find the best plane which separates the data points with high accuracy. Also derive the objective function of logistic regression which is not affected by the outliers. | CO4 / A | 10 |
|  | b. | Explain the bias variance trade off in machine learning techniques. | CO4 / U | 6 |

| 7. | a. | Apply Naïve Bayes theorem and predict the probability of players will play tennis if weather is sunny for the given dataset. Weather; Play. Sunny; No. Overcast; Yes. Rainy; Yes. Sunny; Yes. Sunny; Yes. Overcast; Yes. Rainy; No. Rainy; No. Sunny; Yes. Rainy; Yes. Sunny; No. Overcast; Yes. Overcast; Yes. Rainy; No. | CO5 / A | 10 |
| --- | --- | --- | --- | --- |
|  | b. | List the advantages and disadvantages of Naïve Bayes classifier. | CO5 / R | 6 |
| PART – B (1 X 20 = 20 MARKS) (Compulsory Question) |  |  |  |  |
| 8. | a. | Explain the various data collection techniques involved in machine learning tasks. | CO6 / U | 10 |
|  | b. | Calculate the accuracy, sensitivity, specificity, precision and F1 score of a machine learning model whose TP=464, TN=672, FP=161, FN=41. | CO6 / A | 10 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Comprehend the concept of artificial intelligent systems. |
| CO2 | Execute suitable strategy for solving real world problems. |
| CO3 | Design expert systems for specific applications. |
| CO4 | Select and evaluate linear algorithms. |
| CO5 | Compare and contrast nonlinear and ensemble algorithms. |
| CO6 | Implement machine learning techniques. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 6 | 10 | - | - | - | - | 16 |
| CO2 | - | 16 | - | - | - | - | 16 |
| CO3 | 6 | - | 10 | - | - | - | 16 |
| CO4 | - | 22 | 10 | - | - | - | 32 |
| CO5 | 12 | 10 | 10 | - | - | - | 32 |
| CO6 | - | 10 | 10 | - | - | - | 20 |
|  |  |  |  |  |  |  | 132 |