Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 14CE2002 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # MECHANICS OF SOLIDS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | State Hooke’s law. | CO1/ R | 1 |
| 2. | The relation between young’s modulus and bulk modulus is given by __________. | CO1/ R | 1 |
| 3. | Bending moment is maximum at a section where the shear force is ________after changing its sign. | CO2/ U | 1 |
| 4. | A load acting at a point is known as ___________load. | CO2/U | 1 |
| 5. | The stresses produced due to constant bending moment are known as __________. | CO1/ R | 1 |
| 6. | What are the types of springs? | CO3/R | 1 |
| 7. | The planes of maximum and minimum normal stresses are at an angle of _______to each other. | CO1/U | 1 |
| 8. | The stresses acting on principal planes are known as ________stresses. | CO1/U | 1 |
| 9. | Deflection of a cantilever beam of length (L) at the free end is given by __________when the point load (W) is at the free end. | CO3/R | 1 |
| 10. | What do you mean by crippling load of a column? | CO3/R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | A rectangular bar made of steel is 2.8m long and 15mm thick. The rod is subjected to an axial tensile load of 40 kN. The width of the rod varies from 75mm at one end to 30mm at the other. Find the extension of the rod if E =2 x 105 N/mm2. | CO1/A | 3 |
| 12. | What are the different types of beams? Explain with sketches. | CO1/R | 3 |
| 13. | The shearing stress is a solid shaft is not to exceed 40 N/mm2 when the torque transmitted is 20000 N-m. Determine the minimum diameter of the shaft. | CO3/A | 3 |
| 14. | Find the diameter of a circular bar which is subjected to an axial pull of 160 kN, if the maximum allowable shear stress on any section is 65 N/mm2. | CO2/A | 3 |
| 15. | Explain with neat sketches, how the failure of short and of a long column takes place. | CO3/U | 3 |
| 16. | Define the term Shear Force and Bending Moment. | CO2/R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | A tensile test was conducted on a mild steel bar. The following data was obtained from the test: - Diameter of the steel bar = 3 cm - Gauge length of the bar = 20cm - Load at elastic limit = 250 kN - Extension at a load of 150 kN = 0.21 mm - Maximum load = 380 kN - Totalextension = 60 mm - Diameter of rod at failure = 2.25 cm Determine: (1) The Young’s modulus (2) The stress at elastic limit (3) The percentage of elongation (4) The percentage decrease in area. | CO1/A | 12 |
| 18. | a. | The stresses at a point in a bar are 200N/mm2 (tensile) and 100N/mm2 (compressive). Determine the resultant stress, normal stress and tangential stress on a plane inclined at 60o to the minor axis. | CO1/A | 12 |
| 19. | a. | Draw the shear force and bending moment diagrams for the beam shown in Fig | CO2/A | 12 |
| 20. | a. | A beam is of T section shown in figure. The beam is simply supported over a span of 4m and carries a uniformly distributed load of 1.7kN/m run over the entire span. Determine the maximum tensile and maximum compressive stress. | CO2/A | 12 |
| 21. | a. | A hollow shaft of external diameter 120 mm transmits 300 kW power at 200 r.p.m. Determine the maximum internal diameter if the maximum stress in the shaft is not to exceed 60 N/mm2. | CO3/A | 12 |

| 22. | a. | A compound tube consists of a steel tube 140 mm internal diameter and 160 mm external diameter and an outer brass tube 160 mm internal diameter and 180 mm external diameter. The two tubes are of same length. The compound tube carries an axial load of 900 kN. Find the stresses carried by each tube. | CO3/A | 12 |
| --- | --- | --- | --- | --- |
| 23. | a. | A rectangular beam 200 mm deep and 300 mm wide is simply supported over a span of 8 m. What uniformly distributed load per meter the beam may carry, if the bending stresses is not to exceed 120 N/mm2. | CO3/A | 12 |
|  |  | Compulsory: |  |  |
| 24. | a. | Determine the diameter of a solid shaft which will transmit 300kN at 250rpm. The maximum shear stress should not exceed 30N/mm2& twist should not be more than 1o in a shaft length of 2m. Take C= 1x105 N/mm2. | CO2/A | 12 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 14CE3006 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # FINITE ELEMENT METHODS IN ENGINEERING | # Max. Marks : | # 100 |

| Q. No. | Sub Div. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A(5 X 16= 80 MARKS) (Answer any five from the following) |  |  |  |  |
| 1. | a. | A beam AB of span ‘L’ simply supported at ends and carrying a concentrated load W at the centre ‘C’ as shown in figure. Determine the deflection at mid span by using Rayleigh – Ritz method. | CO1 / A | 16 |
| 2. | a. | The following differential equation is available for a physical phenomenon. d2ydx2+ 500 x2=0, 0 ≤x ≤1 Trial function is y = a1 (x – x4) Boundary conditions are y (0) = 0 y(1) = 0 Find the value of parameter a1 by the following methods i) Point Collocation (ii) Subdomain Collocation (iii) Least Squares (iv) Galerkin method. | CO2 / An | 16 |
| 3. | a. | Solve the shape functions for a linear one dimensional bar element. | CO3 / U | 6 |
|  | b. | A thin steel plate of uniform thickness 25 mm is subjected to a point load of 420 N at mid depth as shown in figure. The plate is also subjected to self weight. If young’s modulus, E = 2 X 105 N/mm2 and density, ρ = 0.8 X 10-4 N/mm3, calculate the following. (i) Displacement at each nodal point. (ii) Stresses in each element. | CO 3 / E | 10 |

| 4. | a. | For the two bar truss shown in figure, determine the displacements of node 1 and stress in element 1-3. | CO3 / A | 16 |
| --- | --- | --- | --- | --- |
| 5. | a. | Consider a three bar truss as shown in figure. It is given that E = 2 X 105 N/mm2. Calculate the following. - Nodal displacements. - Stress in each member. Take A1 = 2000 mm2, A2 = 2500 mm2 and A3 = 2500 mm2 | CO3 / An | 16 |
| 6. | a. | A stepped bar is subjected to an axial load of 200kN at the place of change of cross section and material as in the figure (a) Nodal displacement (b) Induced stresses in each material. | CO3 / An | 16 |
| 7. | a. | Give short notes on the history of FEA | CO 1 / R | 3 |
|  | b. | Write in detail about the application of FEA in different fields. | CO 1 / R | 3 |
|  | c. | Describe in detail about the step by step procedure in solving the FEA problems. | CO 2 / U | 10 |
| PART – B (1 X 20 = 20 MARKS) (Compulsory Question) |  |  |  |  |
| 8. | a. | What do you mean by uniqueness of mapping? | CO1 / R | 2 |
|  | b. | Derive the shape function for Eight Noded Rectangular Element. | CO3 / An | 18 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18CE2005 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # DISASTER PREPAREDNESS AND PLANNING | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | What is Disaster? | CO1 | R | 1 |
| 2. | Write a note on environmental hazards. | CO1 | R | 1 |
| 3. | Define the term vulnerability. | CO2 | R | 1 |
| 4. | Recall any three man-made disasters. | CO3 | U | 1 |
| 5. | Show any two environmental impacts of disasters. | CO4 | U | 1 |
| 6. | Name any three agencies which support forecasting disasters. | CO4 | R | 1 |
| 7. | What is NDMA? | CO6 | A | 1 |
| 8. | List the types of floods. | CO5 | U | 1 |
| 9. | What can you do to help mitigate climate change? | CO5 | A | 1 |
| 10. | List the negative mitigation measures of disasters. | CO6 | R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | List the administrative decisions and operational activities involved in disaster management act 2005. | CO1 | R | 3 |
| 12. | Explain relief and recovery process. | CO2 | R | 3 |
| 13. | Define the term cyclone. | CO3 | U | 3 |
| 14. | Identify the key elements of disaster preparedness. | CO4 | U | 3 |
| 15. | What are climate change adaptation strategies? | CO5 | R | 3 |
| 16. | Explain the term “Climate change mitigation”. | CO4 | R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 17. |  | Explain with neat sketch about disaster management cycle with its phases. |  | CO1 |  | R | 12 |
| 18. | a. | Explain structural and non-structural measures of disaster. |  | CO2 |  | U | 6 |
|  | b. | Write a short note on vulnerability assessment. |  | CO2 |  | R | 6 |
| 19. |  | Summarize types of disasters. |  | CO3 |  | U | 12 |
| 20. |  | Explain the impact and response components of a disaster. |  | CO4 |  | A | 12 |
| 21. |  | Describe in detail on Climate change adaptation and mitigation. |  | CO4 |  | A | 12 |
| 22. |  | Write a short note on approaches to disaster risk reduction. |  | CO5 |  | U | 12 |
| 23. |  | Explain the vulnerability profile of India. |  | CO6 |  | A | 12 |
|  |  | Compulsory: |  |  |  |  |  |
| 24. |  | Discuss a case study on natural disaster. | CO6 |  | A |  | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Recall the types of disasters and its causes |
| CO2 | Understand disaster cycle and assess the risks |
| CO3 | Apply disaster concepts to disaster management |
| CO4 | Analyze relationship between development and disasters |
| CO5 | Decide the roles and responsibilities of organizations and institutions to society and its organizational structure |
| CO6 | Design the disaster management and mitigation plan |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 17 |  |  | - | - | - | 17 |
| CO2 | 10 | 6 |  | - | - | - | 16 |
| CO3 |  | 16 |  | - | - | - | 16 |
| CO4 | 4 | 4 | 24 | - | - | - | 32 |
| CO5 | 3 | 13 | 1 | - | - | - | 17 |
| CO6 | 1 |  | 25 | - | - | - | 26S |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18CE2006 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # CONSTRUCTION ENGINEERING AND MANAGEMENT | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | Define the superstructure in any building. | CO1 | R | 1 |
| 2. | Name any 2 types of dewatering. | CO1 | R | 1 |
| 3. | Define masonry. | CO2 | R | 1 |
| 4. | Weigh batching or hand mixing which is technically better and analyze your answer with one reason. | CO2 | An | 1 |
| 5. | There are many points to be kept in consideration to design an economical section of formwork. List any one of them. | CO3 | R | 1 |
| 6. | Give the hook length of TOR steel in the case of stirrup of a column member. | CO3 | R | 1 |
| 7. | Define plastering. | CO4 | R | 1 |
| 8. | Write about struck pointing. | CO4 | R | 1 |
| 9. | Name the type of projects where CPM is adopted. | CO5 | R | 1 |
| 10. | Indicate which part of the project milestone refers to? | CO5 | U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | List the steps that have to be taken before we start any construction project. | CO1 | R | 3 |
| 12. | Show with diagram how is English Bond is constructed. | CO2 | A | 3 |
| 13. | Discuss pit method of underpinning. | CO3 | U | 3 |
| 14. | List the important consideration for doors and windows. | CO4 | R | 3 |
| 15. | Indicate any 3 objectives of programming of project. | CO5 | U | 3 |
| 16. | Name the causes of accidents. | CO5 | R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | a. | Discuss the aspects that have to be considered during site layout planning. | CO1 | U | 6 |
|  | b. | Present the different types of foundations that are applicable for area with black cotton soil. | CO1 | A | 6 |
| 18. | a. | Explain the stages of production of concrete. | CO2 | U | 8 |
|  | b. | List the need for contraction joints in concrete. | CO2 | R | 4 |
| 19. | a. | Summarize the requirements of good formwork. | CO3 | U | 7 |
|  | b. | Write two functions of scaffolding. | CO3 | R | 3 |
|  | c. | Mention the other name of dead shoring? | CO3 | R | 2 |

| 20. | a. | Name and discuss the components of paint. | CO4 | R | 4 |
| --- | --- | --- | --- | --- | --- |
|  | b. | Present the functional requirements of doors. | CO4 | A | 8 |
| 21. | a. | Explain bar chart giving its advantages and disadvantages. | CO5 | U | 6 |
|  | b. | Discuss the factors to be considered in preliminary planning. | CO5 | U | 6 |
| 22. | a. | Explain: (i) Sump Pumping method of dewatering and (ii) Deep well construction method of dewatering. | CO1 | U | 8 |
|  | b. | Write about dead shoring. | CO1 | R | 4 |
| 23. | a. | Discuss the functional requirement of roofing. | CO4 | U | 8 |
|  | b. | Explain any 2 defects in plastering. | CO4 | U | 4 |
|  |  | Compulsory: |  |  |  |
| 24. | a. | Discuss: (i) Storage and Inventory Management, (ii) Functions of Inventory Control and (iii) Need of Inventory Control. | CO6 | U | 9 |
|  | b. | List the components of operating cost of construction equipment. | CO6 | R | 3 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Recall the basics of building components |
| CO2 | Understand the items / facets of construction engineering |
| CO3 | Apply the construction engineering techniques for various components |
| CO4 | Classify the various techniques for different project |
| CO5 | Explain the importance of project management |
| CO6 | Propose an Engineering and Management Plan |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 9 | 14 | 6 | - | - | - | 29 |
| CO2 | 5 | 8 | 3 | 1 | - | - | 17 |
| CO3 | 7 | 10 | - | - | - | - | 17 |
| CO4 | 9 | 12 | 8 | - | - | - | 29 |
| CO5 | 4 | 16 | - | - | - | - | 20 |
| CO6 | 3 | 9 | - | - | - | - | 12 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18CE2009 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # INSTRUMENTATION AND SENSOR TECHNOLOGIES FOR CIVIL ENGINEERING APPLICATIONS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | What is measurement? | CO1 | R | 1 |
| 2. | What is a measure in the measurement process? | CO1 | R | 1 |
| 3. | Recall the full form of LVDT. | CO2 | R | 1 |
| 4. | Specify the material used to fill the area around the sensor in the installation of piezometer. | CO3 | R | 1 |
| 5. | Name two noise reduction methods. | CO4 | R | 1 |
| 6. | Define a continuous signal. | CO4 | R | 1 |
| 7. | What is a sensor? | CO5 | R | 1 |
| 8. | What ismeant by negative thermal coefficient in a thermistor? | CO2 | R | 1 |
| 9. | Define an Internet of Things gateway. | CO6 | R | 1 |
| 10. | Which is the foundation layer of Internet of Things? | CO6 | R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Outline three applications of measurement systems. | CO1 | U | 3 |
| 12. | Name atleast three applications of load cells in civil engineering. | CO2 | R | 3 |
| 13. | Demonstrate the process of signal stacking. | CO4 | U | 3 |
| 14. | Compare and contrast sensors and transducers. | CO2 | U | 3 |
| 15. | List the characteristics of Internet of Things. | CO6 | R | 3 |
| 16. | Identify atleastsix applications of RFID. | CO5 | A | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | a. | Show the elements of a measurement system. Interpret their functions. | CO1 | U | 7 |
|  | b. | Interpret accuracy, precision, tolerance, range, and sensitivity of measurements. | CO3 | U | 5 |
| 18. | a. | Identify a sensor to measure pore water pressure. Plan its installation. | CO5 | A | 7 |
|  | b. | Demonstrate the working of a strain gauge. Outline its applications in civil engineering. | CO2 | U | 5 |

| 19. | a. | Interpret the fundamental statistical concepts mean, standard deviation, median, mode, and range. | CO4 | U | 10 |
| --- | --- | --- | --- | --- | --- |
|  | b. | Define signal to noise ratio. How is it calculated? | CO4 | R | 2 |
| 20. | a. | Illustrate the working of thermocouples. | CO2 | U | 4 |
|  | b. | Explain the three types of optical sensors used for counting applications. | CO5 | U | 6 |
|  | c. | What are the two types of temperature transducers? | CO2 | R | 2 |
| 21. | a. | Explain the simplified Internet of Things architecture. | CO6 | U | 7 |
|  | b. | Outline the evolution of Internet of Things. | CO6 | U | 5 |
| 22. | a. | List the general characteristics of cloud computing. | CO6 | R | 3 |
|  | b. | Demonstrate M2M using a practical example. | CO6 | U | 4 |
|  | c. | Tell atleast10 modern day applications of Internet of Things. | CO6 | R | 5 |
| 23. | a. | Explain the essential characteristics, service models, and deployment models of cloud computing. | CO6 | U | 12 |
|  |  | Compulsory: |  |  |  |
| 24. | a. | Illustrate the seven characteristics of big data. | CO6 | U | 7 |
|  | b. | Explain RFID and its working. | CO5 | U | 5 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the principles of operation and characteristics of instrumentation and integrated sensor systems |
| CO2 | Predict sensor technologies for specific applications along with limitations |
| CO3 | Specify the requirements in the calibration of sensors and instruments |
| CO4 | Analyze the errors during measurements |
| CO5 | Design sensor systems and set up measurement systems |
| CO6 | Implement smart systems using IoT |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 2 | 10 |  |  |  |  | 12 |
| CO2 | 7 | 12 |  |  |  |  | 19 |
| CO3 | 1 | 5 |  |  |  |  | 6 |
| CO4 | 4 | 13 |  |  |  |  | 17 |
| CO5 | 1 | 11 | 10 |  |  |  | 22 |
| CO6 | 13 | 35 |  |  |  |  | 48 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18CE2020 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # REINFORCED CONCRETE ELEMENTS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | What is the partial safety factor for steel? | CO1 / R | 1 |
| 2. | In a beam, the actual neutral axis is 152.3 mm and the critical neutral axis is 141. Classify the beam. | CO2 / U | 1 |
| 3. | The length and breadth of a slab are 4.3 m and 3.5 m respectively. Classify the slab. | CO4 / U | 1 |
| 4. | Construct the stress block for a beam under limit state method of design. | CO3 / Ap | 1 |
| 5. | What is the limiting value of l/d ratio for a cantilever beam? | CO4 / R | 1 |
| 6. | The height of a column is 6.2m and the size of the column is 300 mm. Classify the column. | CO2 / An | 1 |
| 7. | The effective length of a column is 3200 m and the size of the column is 300 x 300 mm. Estimate the value of minimum eccentricity. | CO2 / E | 1 |
| 8. | Identify the critical section for shear in case of a footing for a concrete wall. | CO4 / Ap | 1 |
| 9. | The load on an isolated footing is 6000 kN. Estimate the self-weight of the footing. | CO6 / E | 1 |
| 10. | Identify the condition for maximum negative bending moment in a beam at the support while using substitute frame method. | CO6 / Ap | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Explain about under reinforced beams. | CO2 / E | 3 |
| 12. | The size of a slab is 5m x 4m. Draw the plan of the slab and mark the middle and edge strips. | CO5 / E | 3 |
| 13. | The percentage of tension reinforcement is 1.8. If the grade of concrete is M30, discover the permissible shearstress under limit state method of design. | CO3 / An | 3 |
| 14. | Explain about braced columns. | CO4 / E | 3 |
| 15. | Discover the choice of combined footings. | CO6 / An | 3 |
| 16. | List any three methods available for the analysis of building frames. | CO6 / R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | List the assumptions made in the working stress method of design. The beam is reinforced with 4 number of 16mm diameter Fe 415 rods. The grade of concrete is M30. | CO1 / An | 3 |
|  | b. | Evaluate the moment of resistance of a rectangular beam of size 300mm x 450mm by limit state method of design. | CO2 / E | 9 |
| 18. |  | Design a simply supported slab of size 7m x 3m. Adopt M20 grade concrete and Fe 415 steel. The live load is 3000 N/m2. [Draw a neat sketch and indicate the design details] | CO5 / C | 12 |

| 19. |  | Design a rectangular beam to carry a UDL of 15 kN/m over a span of 8m. Apply the check for shear also. . [Draw a neat sketch and indicate the design details] | CO5 / C | 12 |
| --- | --- | --- | --- | --- |
| 20. | a. | What is minimum eccentricity? | CO4 / R | 3 |
|  | b. | Design a column with the following data. Pu – 1800 kN, Size = 300 mm x 600mm, Grade of concrete = M30, Grade of steel – Fe 500, Mu = 150 kN.m | CO4 / C | 9 |
| 21. |  | The load on a column of size 300mm x 300mm is 1200 kN. Design a footing to support the column on a soil having a safe bearing capacity of 150 kN/m2. | CO6 / C | 12 |
| 22. |  | Design a simply supported slab of size 5m x 4m. The live load is 3000 N/m2. [Draw a neat sketch and indicate the design details] | CO5 / C | 12 |
| 23. |  | A column of size 300mm x 450mm is reinforced on all four sides. It is reinforced with 8 number of 16mm φ Fe 415 rods. The column is short. The column has to carry a factored load of 1500 kN and factored moments of 120 kNm and 80 kNm along the longer and shorter planes respectively. The grade of concrete is M30. Decide on the safety of the column. | CO4 / E | 12 |
|  |  | Compulsory: |  |  |
| 24. |  | The plan of the building is given below. 4; 4; 5. 3. 3. A 3; B1; B2; B3; A. Section AA is given below: 4; 4; 5. 4. 3. 3; B1; B2; B3. If the live load is 2000 N/m2 and the unit weight of brick masonry is 18 kN/m2, determine the loads on beams B1 and B2. | CO6 / E | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Identify the design philosophies for RCC elements |
| CO2 | Demonstrate the behaviour of elements for load calculations |
| CO3 | Illustrate the LSM for estimating stress resultants |
| CO4 | Design the section and reinforcement for the structural elements |
| CO5 | Develop suitable detailing diagrams |
| CO6 | Prepare the design for buildings |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 1 |  |  | 3 |  |  | 4 |
| CO2 |  | 1 |  | 1 | 13 |  | 15 |
| CO3 |  |  | 1 | 3 |  |  | 4 |
| CO4 | 4 | 1 | 1 |  | 15 | 9 | 30 |
| CO5 |  |  |  |  | 3 | 36 | 39 |
| CO6 | 3 |  | 1 | 3 | 13 | 12 | 32 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18CE2023 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # Foundation Engineering | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Define Area ratio. | CO 1 /R | 1 |
| 2. | What are the various methods of site investigation? | CO 1 /R | 1 |
| 3. | What is ultimate bearing capacity? | CO 2 /R | 1 |
| 4. | Define settlement and its types. | CO 2 /R | 1 |
| 5. | What is spread footing? | CO 3 /R | 1 |
| 6. | Under what circumstances, a strap footing is adopted? | CO 3 / R | 1 |
| 7. | List out the type of pile based on material used. | CO 4 /R | 1 |
| 8. | What are the two types of dynamic formulae? | CO 4 /R | 1 |
| 9. | Define backfill. | CO 5 /R | 1 |
| 10. | Write the types of retaining wall. | CO 5 /R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Explain Representative and Non-Representative Samples. | CO1 / U | 3 |
| 12. | Compare general and local shear failure. | CO2 / U | 3 |
| 13. | Explain floating column. | CO3 / U | 3 |
| 14. | List the factors governing selection of pile. | CO 4 / R | 3 |
| 15. | Compare Rankine’s and Coulomb’s theory. | CO 5 / An | 3 |
| 16. | Explain in detail about piezometer. | CO 6/ U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | Explain the various methods of drilling bore holes for sub surface investigations. | CO1 / E | 12 |
| 18. | a. | Explain plate load test with sketch. | CO2 / E | 12 |
| 19. | a. | Design a square footing of square isolated column having size 500 x 500 mm carrying 1600 kN axial load. Safe bearing capacity of soil is 200 kN/m2. Use M25 and Fe 415 grades. Check for one way and two way shear. | CO3 / C | 12 |

| 20. | a. | A group of 9 piles with 3 piles in a row was driven into soft clay extending from ground level to a great depth. The diameter and length of piles were 30 cm and 10 cm respectively. The unconfined compression strength of clay is 70 kN/m2 . If the piles were spaced at 90cm centre to centre, compute the allowable load on the pile group on the basis of shear failure criteria for a factor of safety of 2.5, neglect bearing at the tip of piles, take m = 0.6 for shear mobilization around each pile. | CO4 / An | 12 |
| --- | --- | --- | --- | --- |
| 21. | a. | A gravity retaining wall retains 10 m of a backfill, unit weight of soil =18 kN/m3, angle of shearing resistance =30° with a horizontal surface. Assume the wall interface to be vertical, determine (i) the magnitude and point of application of the total active pressure (ii) if the water table is at a height of 5m, and how far do the magnitude and the point of the application of active pressure changed. Take submerged unit weight = 10kN/m3 . | CO5 / An | 12 |
| 22. | a. | A strip footing 1m wide at its base. Is located at a depth of 0.8m below the ground surface. The properties of the foundation soil are 𝛾 = 18 kN/m3, C = 30 kN/m2 and ∅ = 𝟑𝟓° ,Nc=52, Nq=35 and Ny=42. Determine the safe bearing capacity. Use F.O.S as 3. | CO 4 / E | 12 |
| 23. | a. | What are various functions of foundation? | CO3 / R | 4 |
|  | b. | Explain the different types of shallow foundation. | CO3 / U | 8 |
|  |  | Compulsory: |  |  |
| 24. | a. | Explain the different sensors used in geotechnical engineering with their applications. | CO6 / U | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Recall the various site investigation methods |
| CO2 | Calculate the bearing capacity of soils and foundation settlements. |
| CO3 | Analyze the various foundation system |
| CO4 | Design the foundation system for larger depths. |
| CO5 | Suggest retaining structures catering to earth pressure conditions. |
| CO6 | Adopt suitable subsurface instrumentation. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 2 | 3 | - | - | 12 | - | 17 |
| CO2 | 2 | 3 | - | - | 12 | - | 17 |
| CO3 | 6 | 11 | - | - | - | 12 | 29 |
| CO4 | 5 | - | - | 12 | 12 | - | 29 |
| CO5 | 2 | - | - | - | 15 | - | 17 |
| CO6 | - | 15 | - | - | - | - | 15 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 20CE 2007 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # TRANSPORTATION ENGINEERING | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
|  | PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | State the classification of road system followed from third twenty year road plan. | CO1 | R | 1 |
| 2. | Mention the factors that control the highway alignment. | CO4 | U | 1 |
| 3. | List the items included in inventory survey. | CO3 | R | 1 |
| 4. | Differentiate travel time and running time. | CO3 | U | 1 |
| 5. | Write the significance of CBR value on selection of subgrade soil. | CO6 | U | 1 |
| 6. | Define the term subgrade reaction. | CO6 | R | 1 |
| 7. | List various data to be collected for railway track alignment. | CO2 | R | 1 |
| 8. | List different gauge systems used in India. | CO2 | R | 1 |
| 9. | State the functions of rail. | CO5 | U | 1 |
| 10. | What is creep? | CO2 | U | 1 |

|  |  | PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 11. | Describe Roman roads. |  | CO1 | U | 3 |
| 12. | Write the objectives of spot speed study. |  | CO3 | U | 3 |
| 13. | Describe the concept of considering the wheel loads for the design of pavements. |  | CO6 | U | 3 |
| 14. | Explain the various cross sectional elements of a highway. |  | CO4 | U | 3 |
| 15. | Describe theory of coning. |  | CO5 | U | 3 |
| 16. | Explain the types of Marshalling yard. |  | CO2 | U | 3 |

|  |  | PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 17. | a. |  | Explain the various data to be collected for the alignment of the highway. | CO1 | U | 6 |
|  | b. |  | Discuss the recommendations by Jayakar Committee. | CO1 | U | 6 |
| 18. | a. |  | Explain two methods for conducting spot speed surveys. | CO3 | U | 6 |
|  | b. |  | Discuss the importance of traffic volume studies. | CO3 | U | 6 |
| 19. | a. |  | Explain the desirable properties of coarse aggregate to be used for road construction. | CO6 | U | 4 |
|  | b. |  | Design the flexible pavement for a NH having the following data: CBR of subgrade soil = 4%, CBR of soil aggregate mix = 20%, CBR of soil kankar aggregate mix = 10%, CBR of WBM coarse = 85%. The minimum thickness of bituminous macadam surfacing may be taken as 5cm. The last traffic count is 1450 commercial Vehicles/day. | CO4 | An | 8 |

| 19. | a. | Explain the desirable properties of coarse aggregate to be used for road construction. | CO6 | U | 4 |
| --- | --- | --- | --- | --- | --- |
|  | b. | Design the flexible pavement for a NH having the following data: CBR of subgrade soil = 4%, CBR of soil aggregate mix = 20%, CBR of soil kankar aggregate mix = 10%, CBR of WBM coarse = 85%. The minimum thickness of bituminous macadam surfacing may be taken as 5cm. The last traffic count is 1450 commercial Vehicles/day. | CO4 | An | 8 |
| 20. | a. | Explain PIEV theory. | CO4 | U | 4 |
|  | b. | The speeds of overtaking and overtaken vehicles are 60 kmph and 50 kmph respectively on a two way traffic road. If the acceleration of overtaking vehicle is 0.9m/sec2, i) Calculate safe overtaking sight distance ii) Obtain the minimum length of overtaking zone and iii) Draw a neat sketch of overtaking zone show in the position of sign posts. Assume all other data as per IRC. | CO4 | A | 8 |
| 21. | a. | Draw the sketch of a turnout and explain its working principle. | CO5 | U | 6 |
|  | b. | Explain the various components of Railway Track with help of a neat sketch. | CO5 | U | 6 |
| 22. | a. | A new National Highway is to be aligned along a plain terrain. If the maximum radius which can be provided on the site is 200m, design all the geometric features of this curve. Assume suitable data. | CO4 | An | 8 |
|  | b. | Discuss the application of origin-destination survey. | CO3 | U | 4 |
| 23. | a. | Discuss various types of gradients which effect the design of vertical profile of a highway. | CO4 | U | 6 |
|  | b. | Discuss the importance of parking survey and explain the terms parking demand, parking volume and parking duration. | CO3 | U | 6 |
|  |  | Compulsory: |  |  |  |
| 24. | a. | Explain the classification of signals used in railways. | CO2 | U | 6 |
|  | b. | Discuss the advantages and disadvantages of precast sleepers. | CO2 | U | 6 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18CE2028 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # QUANTITY SURVEYING AND ESTIMATION | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | Define Estimation. | CO1 | R | 1 |
| 2. | List any two principles of units of measurement. | CO1 | R | 1 |
| 3. | Define analysis of rates. | CO2 | R | 1 |
| 4. | Recall the term lift. | CO2 | R | 1 |
| 5. | List the methods of taking out estimates. | CO3 | R | 1 |
| 6. | Recall the formula for finding number of bars. | CO3 | R | 1 |
| 7. | What are the main components of culvert? | CO4 | R | 1 |
| 8. | Define abutment. | CO4 | R | 1 |
| 9. | Define Plinth area. | CO5 | R | 1 |
| 10. | What do you mean by Gross income? | CO5 | R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Explain the need for an estimation and costing. | CO1 | U | 3 |
| 12. | Estimate the quantities of brickwork and plastering required in a wall 4m long, 3m high and 30 cm thick. Calculate also the cost if the rate of brickwork is Rs.32.00 per cu.m and of plastering is Rs. 8.50 per sq.m. | CO2 | E | 3 |
| 13. | Illustrate the term pointing and plastering. | CO3 | U | 3 |
| 14. | Determine the Length of the rod. Assume side cover is 40mm. | CO4 | E | 3 |
| 15. | The estimated value of a building is Rs.5,00,000. The carpet area of the building is 70 sq.m. If the plinth area is 20% more than this, what is the plinth rate of the building? | CO5 | A | 3 |
| 16. | List the different classification of tenders. | CO6 | R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. |  | Explain the different types of estimates. | CO1 | U | 12 |
| 18. |  | Analyse the rates of the following. Cement plaster with 1 : 4 mix - 20 masons, 20 mazdoors and 4 bhistes can finish 200 sq.m area in one day. Assume the thickness of the plastering is 12mm. Basic rates of materials are as below: Cement – Rs. 450/- per 50 kg bag. Fine Aggregate (river sand) – Rs. 1,450/- per cum. Assume suitable data wherever required. | CO2 | An | 12 |
| 19. |  | Prepare the quantity estimation of the following items, using given plan and section D = 1 x 2.1 m W = 1 x 1.2 m - Earthwork excavation in foundation - Lime concrete in foundation - Brick work in substructure. - Damp proof course. | CO3 | A | 12 |
| 20. |  | Determine the quantity estimation of the following items for the construction of a new state highway for one kilometer length. - Land acquisition - Earthwork in embankment - Preparation of subgrade. - Preparation of soiling coat - Miscellaneous items. | CO4 | E | 12 |

| 21. | a. | Calculate the approximate cost of the residential building having a carpet area of 2000 sq.m. Plinth area rate is Rs. 1000/Sq.m | CO5 | An | 6 |
| --- | --- | --- | --- | --- | --- |
|  | b. | A residential building of floor area 2500 m2, circulation Area 125 m2 and the plinth area is 2750 m2. Determine the Carpet area of building. | CO5 | An | 6 |
| 22. |  | Prepare the quantity estimation of a wall given in Sketch A. Lime concrete in foundation - Brick work in substructure. - Brickwork in superstructure. - Earth work excavation in foundation. | CO4 | A | 12 |
| 23. |  | Explain the following: - Depreciation - Obsolescence - Sinking fund - Capitalized vale - Net Income - Book Value | CO4 | U | 12 |
|  |  | Compulsory: |  |  |  |
| 24. |  | Explain the information to be given in the tender notice and tender document. | CO6 | U | 12 |

SKETCH ‘A’

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | List the detailed specification for different types of structures |
| CO2 | Plan the rate analysis of civil engineering works |
| CO3 | Determine the rates of various items of civil works |
| CO4 | Justify estimated cost of civil construction projects |
| CO5 | Evaluate the actual value of any property |
| CO6 | Explain specifications and tendering process for contracts |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 2 | 15 | - | - | - | - | 17 |
| CO2 | 2 | - | - | 12 | 3 |  | 17 |
| CO3 | 2 | 3 | 12 | - | - | - | 17 |
| CO4 | 2 | 12 | 12 | - | 15 |  | 41 |
| CO5 | 2 | - | 3 | 12 | - | - | 17 |
| CO6 | 3 | 12 | - | - | - | - | 15 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18CE2036 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name: | # CONSTRUCTION EQUIPMENT AND AUTOMATION | # Max. Marks: | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Define depreciation analysis. | CO2 / R | 1 |
| 2. | Describe the conventional methods used for construction. | CO1 / U | 1 |
| 3. | List out the Fundamentals of earth work operations. | CO2 / R | 1 |
| 4. | Discuss the types of earthwork operations. | CO1 / U | 1 |
| 5. | Discuss the equipment used for dredging. | CO3 / U | 1 |
| 6. | Discuss the equipment for drilling and blasting. | CO3 / U | 1 |
| 7. | Define feeders. | CO4 / R | 1 |
| 8. | Define crushers. | CO4 / R | 1 |
| 9. | Discuss about the installation of ducts and strands. | CO5 / U | 1 |
| 10. | Define tendon couplers. | CO5 / R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Interpret the cost of equipment in controlled manner. | CO1/ A | 3 |
| 12. | Appraise the compacting and finishing equipment. | CO2 / E | 3 |
| 13. | Appraise the equipment used for dewatering and grouting. | CO3 / E | 3 |
| 14. | Write in detail about the asphalt plant. | CO4 / C | 3 |
| 15. | Interpret tendons and anchorages. | CO6 / A | 3 |
| 16. | Describe briefly about the conveyors. | CO5 / C | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | Construct elaborately conventional construction methods and mechanized methods. | CO1 / C | 4 |
|  | b. | Investigate the selection and planning of equipment. | CO1 / C | 4 |
|  | c. | Develop the procedure for replacement analysis. | CO1 / C | 4 |
| 18. | a. | Formulate the earth moving operations. | CO2 / C | 4 |
|  | b. | Develop in detail about the earth moving equipment. | CO2 / C | 4 |
|  | c. | Appraise the fundamentals of Fundamentals of earth work operations. | CO2 / E | 4 |

| 19. | a. | Construct elaborately about equipment used for dredging, trenching, drag line and clamshells. | CO3 / C | 4 |
| --- | --- | --- | --- | --- |
|  | b. | Construct elaborately about equipment used fortunneling. | CO3 / C | 4 |
|  | c. | Construct elaborately about equipment used for drilling and blasting. | CO3 / E | 4 |
| 20. | a. | Construct elaborately about batching and mixing equipment. | CO4 / E | 4 |
|  | b. | Construct elaborately about pumping equipment. | CO4 / C | 4 |
|  | c. | Construct elaborately about ready mix concrete equipment. | CO4 / E | 4 |
| 21. | a. | Construct elaborately about stressing jacks grouting system. | CO5 / C | 4 |
|  | b. | Construct elaborately about grout mixer and agitator. | CO5 / C | 4 |
|  | c. | Construct elaborately about installation of ducts and strands. | CO5 / E | 4 |
| 22. | a. | Elaborate conventional construction methods. | CO1 / C | 4 |
|  | b. | Elaborately mechanized construction methods. | CO1 / C | 4 |
|  | c. | Develop the procedure for depreciation analysis. | CO1 / C | 4 |
| 23. | a. | Construct elaborately about tractors, motor graders, scrapers, front end waders. | CO2 / C | 4 |
|  | b. | Construct elaborately about dozer, excavators, rippers, loaders, trucks and hauling equipment. | CO2 / C | 4 |
|  | c. | Construct elaborately about compacting equipment, finishing equipment. | CO2 / C | 4 |
|  |  | Compulsory: |  |  |
| 24. | a. | Design Forklifts and related equipment. | CO6 / C | 4 |
|  | b. | Design in detail about the cranes, hoists and other equipment for lifting. | CO6 / C | 4 |
|  | c. | Design the equipment for transportation of materials. | CO6 / C | 4 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Differentiate the conventional method and mechanized methods |
| CO2 | Identify various types of equipments for different construction projects |
| CO3 | Demonstrate the prestressing using jacks and grouting |
| CO4 | Recommend the equipments used for lifting of materials |
| CO5 | Evaluate the material handling equipment and determine equipment productivity |
| CO6 | Adapt robots in construction sites |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | - | 2 | 3 | - | - | 24 | 29 |
| CO2 | 2 | - | - | - | 7 | 20 | 29 |
| CO3 | - | 2 | - | - | 7 | 8 | 17 |
| CO4 | 2 | - | - | - | 8 | 7 | 17 |
| CO5 | 1 | 1 | - | - | 4 | 11 | 17 |
| CO6 | - | - | 3 | - | - | 12 | 15 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18CE2051 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # BASICS OF REMOTE SENSING AND GEOGRAPHICAL INFORMATION SYSTEM | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | Optical region of electromagnetic spectrum consists of _________. | CO1 | R | 1 |
| 2. | Which is the first Indian Remote sensing satellite? | CO2 | U | 1 |
| 3. | The purpose of geostationary satellite is____________. | CO1 | U | 1 |
| 4. | Wavelength and frequency are related by the formula _________. | CO1 | R | 1 |
| 5. | Deep clear water appears in _________ colour on a satellite image. | CO4 | U | 1 |
| 6. | Which feature reflects highest in NIR region? | CO5 | A | 1 |
| 7. | WGS stands for _________. | CO6 | U | 1 |
| 8. | Location of the area is __ type of data. | CO3 | R | 1 |
| 9. | Nearest Neighbor is a resampling method - True or False. | CO6 | R | 1 |
| 10. | In QGIS, Q stands for _________. | CO1 | R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Define Remote Sensing. | CO2 | R | 3 |
| 12. | What is meant by Spectral and Spatial Resolution? | CO1 | U | 3 |
| 13. | List out any seven visual interpretation keys. | CO2 | U | 3 |
| 14. | Recall Small Scale and Large Scale. | CO5 | U | 3 |
| 15. | Difference between Vector and Raster Data. | CO3 | U | 3 |
| 16. | List out the GIS and image processing software used in Civil Engineering applications. | CO3 | U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | a. | Explain about components of remote sensing with suitable diagram. | CO2 | A | 6 |
|  | b. | Demonstrate about atmospheric effects on remote sensing. | CO4 | U | 6 |
| 18. | a. | Illustrate about various types of resolution concepts. | CO5 | U | 6 |
|  | b. | Classify Sun synchronous and Geo synchronous remote sensing satellites. | CO1 | U | 6 |
| 19. | a. | Explain about standard data products. | CO3 | U | 6 |
|  | b. | Explain about supervised classification methods. | CO3 | R | 6 |
| 20. | a. | Explain about three stages involved in map making process. | CO3 | U | 6 |
|  | b. | Explain about Components of GIS. | CO4 | U | 6 |

| 21. | a. | Summarize the different types of database structure in GIS. | CO1 | An | 6 |
| --- | --- | --- | --- | --- | --- |
|  | b. | Classify Vector and Raster data model in GIS. | CO3 | U | 6 |
| 22. | a. | Summarize the different types of Map projection. | CO6 | U | 6 |
|  | b. | Classify vector and raster overlay operations. | CO3 | U | 6 |
| 23. | a. | Enumerate the Civil Engineering applications used in the RS and GIS. | CO1 | U | 6 |
|  | b. | Summarize in detail about QGIS software. | CO2 | U | 6 |
|  |  | Compulsory: |  |  |  |
| 24. | a. | Summarize about digital image processing | CO6 | U | 6 |
|  | b. | Demonstrate about the History of GIS | CO6 | U | 6 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18CE2053 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # AIR AND NOISE POLLUTION CONTROL | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | What is air pollution? | CO1/ R | 1 |
| 2. | What is the secondary air pollutant? | CO2 / R | 1 |
| 3. | Define smog. | CO2 / R | 1 |
| 4. | Explain the greenhouse effects. | CO1 / U | 1 |
| 5. | What is an inversion? | CO4 / R | 1 |
| 6. | Explain the lapse rate. | CO4 / U | 1 |
| 7. | Explain the Gaussian dispersion model. | CO4 / U | 1 |
| 8. | What are air pollution indices? | CO1 / R | 1 |
| 9. | Define adsorption. | CO5/ R | 1 |
| 10. | Explain Freeze out sampling. | CO5 / U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Explain the fugitive emissions. | CO3 / U | 3 |
| 12. | Describe the air pollution episodes. | CO2 / U | 3 |
| 13. | Define the wind profile. | CO3 / R | 3 |
| 14. | Explain the stack monitoring. | CO4 / U | 3 |
| 15. | Define fabric filters. | CO5 / R | 3 |
| 16. | How is sound different from noise? Define the types of noise. | CO6 / R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | Explain the emission inventories by using a neat and clean diagram. | CO3 / U | 6 |
|  | b. | Describe the transport of air pollution and its impact on the receptor. | CO4 / U | 6 |
| 18. | a. | What is an ozone hole? Explain the mechanisms of formation of the hole. | CO4 /R | 6 |
|  | b. | Define the particulate matter? Demonstrate the effects of particulate matter on human health, materials, and plant with examples. | CO2 /R | 6 |

| 19. | a. | Explain the atmospheric stability by using a neat and clean diagram. | CO3 / U | 6 |
| --- | --- | --- | --- | --- |
|  | b. | Given the following temperature and elevation data, identify the stability of the atmosphere. Elevation (m) Temperature (oC) 2.00 14.35 324.00 11.13 | CO3 / U | 6 |
| 20. | a. | Explain the air quality monitoring and its importance for humans and the surrounding environment? | CO6 / U | 6 |
|  | b. | Identifythe effective height of a stack, if the stack is 203 m tall with 1.07 m inside diameter. The wind velocity is 3.56 m/s. The air temperature is 13 °C. Barometric pressure is 1000 millibars. Stack gas velocity is 9.14 m/s having 149 °C temperature. | CO4 / U | 6 |
| 21. | a. | Describe the possible methods for the removal of gaseous pollutants by using a neat and clean diagram. | CO5 / R | 6 |
|  | b. | Explain the general applicability of particulate control systems. | CO6 / U | 6 |
| 22. | a. | Illustrate the effects of lapse rate on plume behaviors by using neat and clean diagrams. | CO3 /A | 6 |
|  | b. | Explain the mechanism of the electronic precipitator (ESP), gravity settling chamber, and high volume sampler by using a neat and clean diagram. | CO5 / U | 6 |
| 23. | a. | Demonstrate the Iso-kinetic condition and non–Iso kinetic condition during stack sampling by using a neat and clean diagram. | CO3 /U | 6 |
|  | b. | Explain the national ambient air quality standards. Demonstrate the national ambient air quality standards for any five pollutants with their units. | CO6 /U | 6 |
|  |  | Compulsory: |  |  |
| 24. | a. | Explain the Indian standard and guidelines for noise pollution. | CO6 / U | 6 |
|  | b. | What is a decibel? Define the decibel scale by using a neat and clean diagram. | CO1 / R | 6 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand causes of air pollution |
| CO2 | Analyze different types of air pollutants |
| CO3 | Impart knowledge on different atmospheric components |
| CO4 | Evaluate air pollutant behavior in the atmosphere |
| CO5 | Choose appropriate control technology for air pollution |
| CO6 | Develop air pollution reduction techniques |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 8 | 1 | - | - | - | - | 9 |
| CO2 | 8 | 3 | - | - | - | - | 11 |
| CO3 | 3 | 27 | 6 | - | - | - | 36 |
| CO4 | 7 | 17 | - | - | - | - | 24 |
| CO5 | 10 | 7 | - | - | - | - | 17 |
| CO6 | 3 | 24 | - | - | - | - | 27 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18CE2055 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # AIRPORT PLANNING AND DESIGN | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | List any four components of an airport. | CO3 / R | 1 |
| 2. | Explain the purpose of a hangar. | CO3 / U | 1 |
| 3. | Explain about calm period. | CO2 / U | 1 |
| 4. | Identify a method to increase the wind coverage. | CO2 / Ap | 1 |
| 5. | Infer the term ‘gate’. | CO4 / U | 1 |
| 6. | Discover the capacity of two non-intersecting runways with divergent gliding paths under VFR. | CO4 / An | 1 |
| 7. | List the different aircraft parking configurations. | CO6 / R | 1 |
| 8. | Estimate the public parking facility area required for an airport. | CO6 / C | 1 |
| 9. | What is the percentage increase in the pavement thickness required for coverages of 15000? | CO5 / R | 1 |
| 10. | Outline the factors to be considered in the determination of pavement thickness. | CO5 / U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Explain classification of airports based on runway length. | CO2 / U | 3 |
| 12. | Outline airport reference temperature. | CO2 / U | 3 |
| 13. | With a neat sketch, explain about holding apron. | CO4 / E | 3 |
| 14. | Discuss about blast fences. | CO4 / C | 3 |
| 15. | Estimate the radius of the taxiway which is to be designed for an exit speed of 80 kmph. The friction between the tire and the pavement is 0.13. | CO2 / C | 3 |
| 16. | Explain about the different categories of instrument landing system. | CO3 / U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. |  | Draw a typical plan of an airport and briefly explain the salient features of any four components. | CO1 / U | 12 |
| 18. | a. | The length of a runway under standard conditions is 2500 m. The airport is to be located at an elevation of 400 m above the mean sea level. The airport reference temperature is 32°C. The effective gradient is 0.2%. Estimate the actual runway length required. | CO4 / C | 10 |
|  | b. | Summarize the limiting values for the following required for A type airports. - Longtitudinal gradient - Transverse gradient - Width of runway - Width of taxiway | CO2 / U | 2 |
| 19. | a. | Explain the various factors that affect the capacity of runways. | CO4 / U | 10 |
|  | b. | Examine the need for Sub Surface drainage. | CO4 / E | 2 |
| 20. |  | Draw a neat sketch of a terminal building and explain the salient points. | CO6 / U | 12 |
| 21. |  | Design a runway pavement for an airport with the following data. Design gross weight of the airport 70000 kg CBR value of subgrade 5% CBR value of ungraded gravel 16% CBR value of graded gravel 35% Number of coverages is 15000 | CO5 / C | 12 |
| 22. | a. | Explain with a neat sketch, the various markings on runways. | CO4 / U | 7 |
|  | b. | Explain with a neat sketch, the various markings at the intersection of a runway and exit taxiway. | CO4 / U | 5 |
| 23. |  | With a neat sketch, illustrate how type II wind rose diagram helps in identifying the best orientation for a runway? | CO2 / U | 12 |
|  |  | Compulsory: |  |  |
| 24. |  | Explain in detail about the subsurface drainage system for an airport. | CO4 / E | 12 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18CE2059 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name: | # GREEN BUILDINGS | # Max. Marks: | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Define green building. | CO2 / R | 1 |
| 2. | Recognize the codes and guidelines used for green buildings. | CO1 / U | 1 |
| 3. | List the stages in site design. | CO2 / R | 1 |
| 4. | Discuss the principles in the selection of green materials. | CO1 / U | 1 |
| 5. | Identify the construction waste. | CO3 / U | 1 |
| 6. | Identify the demolition waste. | CO3 / U | 1 |
| 7. | List the essentials of commissioning. | CO4 / R | 1 |
| 8. | Define HVAC. | CO4 / R | 1 |
| 9. | Recognize the full form of LEED. | CO5 / U | 1 |
| 10. | List the stages of LEED certification process. | CO5 / R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Interpret the green building goals and principles. | CO1 / A | 3 |
| 12. | Appraise the green building products and practices. | CO2 / E | 3 |
| 13. | Support the methods that helps in reducing the carbon footprint of construction operation. | CO3 / E | 3 |
| 14. | Construct the cost and benefit of building commissioning. | CO4 / C | 3 |
| 15. | Sketch green globes building assessment protocol international building assessment systems. | CO6 / A | 3 |
| 16. | Construct the trends in building rating systems. | CO5 / C | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | Design Holistic design. | CO1 / C | 4 |
|  | b. | Design Integrated design. | CO1 / C | 4 |
|  | c. | Construct Approaches to green building. | CO1 / C | 4 |
| 18. | a. | Design the passive design strategy. | CO2 / C | 4 |
|  | b. | Formulate the resource efficiency and internal load reduction. | CO2 / C | 4 |
|  | c. | Assemble the management of building water and waste. | CO2 / E | 4 |

| 19. | a. | Construct Site protection planning. | CO3 / C | 4 |
| --- | --- | --- | --- | --- |
|  | b. | Construct health and safety planning. | CO3 / C | 4 |
|  | c. | Investigate construction and demolition waste. | CO3 / E | 4 |
| 20. | a. | Appraise the essentials of commissioning. | CO4 / E | 4 |
|  | b. | Design HVAC commissioning of non-mechanical system. | CO4 / C | 4 |
|  | c. | Appraise cost and benefit of building commissioning. | CO4 / E | 4 |
| 21. | a. | Design Green Building Assessment USGBC LEED building assessment standard. | CO5 / C | 4 |
|  | b. | Construct LEED certification process. | CO5 / C | 4 |
|  | c. | Appraise green globes building assessment protocol international building assessment systems. | CO5 / E | 4 |
| 22. | a. | Formulate the environmental challenges faced in the current world. | CO1 / C | 4 |
|  | b. | Formulate Green building goals and principles. | CO1 / C | 4 |
|  | c. | Formulate codes and guidelines in green buildings. | CO1 / C | 4 |
| 23. | a. | Design the selection of green materials. | CO2 / C | 4 |
|  | b. | Assemble the indoor environment quality. | CO2 / C | 4 |
|  | c. | Design the products and practices in green buildings. | CO2 / C | 4 |
|  |  | Compulsory: |  |  |
| 24. | a. | Design Platinum LEED rating system. | CO6 / C | 4 |
|  | b. | Design Gold LEED rating system. | CO6 / C | 4 |
|  | c. | Design Silver LEED rating system. | CO6 / C | 4 |

Reg.No. _____________

End Semester Examination – June – 2022

| # Code : | # 18CE3038 | # Duration: | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name: | # WATER RESOURCES PLANNING AND SYSTEMS ENGINEERING | # Max. Marks: | # 100 |

| Q. No. | Sub Div. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |
| 1. | a. | Explain art of modelling in detail. | CO1 / Ap | 4 |
|  | b. | Explain in detail: Dynamic programming and Bellman’s principle of optimality and its characteristics. | CO2 / U | 6 |
|  | c. | Elaborate the function of water resource planning and development in detail. | CO1 / Ap | 6 |
| 2. | a. | Elaborate the term sensitivity analysis with three steps and also list out the applications of linear programming in water resources. | CO2 / U | 6 |
|  | b. | Solve the following problem using the development of LP model maximization method. A company manufactures two types of products A1, & A2. Each product uses milling & drilling machine. The process time per unit of A1 on the milling is 10 hours & on the drilling machine is 8 hours. The processing time per unit of A2 on the milling is 15 hours & on the drilling machine is 10 hours. The maximum number of hours available per week on the milling & the drilling machine are 80 hours & 60 hours respectively. Also, the profit per unit of selling A1, & A2 are Rs. 25 &Rs. 35 respectively. Formulate a LP model to determine the production volume of each of the products such that the total profit is maximized | CO2 / Ap | 4 |
|  | c. | Draw chart of phases of water resource planning. | CO4 / R | 6 |
| 3. | a. | - Elaborate objective function in both canonical and standard form Minimize Z = 2 x1 + 3 x2 + 5x3 Sub to -3x1 + 4x2 ≤ 5 x1 + 3x2 + x3 ≥ 4 x1 + x3 ≤ 3 x1, x2 ≥ 0, x3 is unrestricted in sign. | CO2 / Ap | 5 |
|  | b. | Solve the following formation of LPP. A furniture dealer deals on only two items via tables and chairs. He has to invest Rs. 10000 and a space to store almost 60 pieces. A table cost is Rs. 500 and chair cost Rs. 200. He can sell all the items he buys. He is getting a profit of Rs. 50 per table and Rs. 15 per chair. Formulate this problem as on LPP, so as to maximize the profit. | CO2 / Ap | 4 |
|  | c. | Derive Time complexity and check the pathway in Bellman Ford Algorithm as given below. 1-2 = 4, 1-3 = 5, 3-2 = 7, 2-4 = 7 and 4-3 = -15 | CO3 / R | 7 |

| 4. | a. | Explain National water policy and also list out the acts in India. | CO5 / R | 7 |
| --- | --- | --- | --- | --- |
|  | b. | Derive Bellman equation with every step. | CO3 / U | 9 |
| 5. | a. | Define curse of dimensionality. | CO3 / U | 4 |
|  | b. | Briefly explain in detail about the canonical and standard general format. | CO3 / U | 8 |
|  | c. | Explain the implementation and planning strategies of water resources projects. | CO6 / U | 4 |
| 6. | a. | Explain the levels of water resource planning and development. | CO1 / R | 6 |
|  | b. | Solve the following problem using Bernoulli’s Equation with diagram. The water level in the two reservoir A and B are 104.5 m and 100 m respectively above the datum. A pipe joins each to a common point D where pressure is 98.1KN/m2. Gauge and height is 83.5 m above datum. Another pipe connects D to another tank C. What will be the height of water level in C. Assuming the same value of (f) for all pipes. Take f = 0.0075. The diameter of pipes AB, BD & CD are 300 mm, 450 mm and 600 mm respectively and their lengths are 240 m, 270 m, 300 m respectively | CO1 / R | 10 |
| 7. | a. | A market executive located in a city A decided to travel to city B. He/She knew the distances of alternative routes from the city A to B, and then He/She tabulated all the data as given below. The city of origin A is city 1 and the destination city, B, is city 10, other cities through which the salesperson will have to pass are numbered from 2 to 9. The salesperson’s problem is to find the shortest route that covers all the selected cities from A to B. The given data is in kilometers by using backward recursive dynamic programming. 8; 9. 5; 4; 8. 6; 3; 7. 7; 8; 4. 2; 3; 4. 1; 4; 6; 3. 5; 6; 7. 2; 7; 10; 5. 3; 3; 8; 4. 4; 6; 10; 5. | CO4 / Ap | 10 |

|  | b. | The amounts of water flowing from a certain catchment area at the proposed dam site are given in the following table. Determine the minimum capacity of the reservoir if water is to be used to feed the turbined of the hydropower plant at a uniform rate and no water is to be spilled over. MONTH; INFLOW (× 103 m3). JAN; 2.83. FEB; 4.25. MARCH; 5.66. APRIL; 18.50. MAY; 22.64. JUNE; 22.64. JULY; 19.81. AUG; 8.69. SEP; 7.10. OCT; 7.10. NOV; 5.66. DEC; 5.66. | CO4 / Ap | 6 |
| --- | --- | --- | --- | --- |
| PART – B (1 X 20 = 20 MARKS) (Compulsory Question) |  |  |  |  |
| 8. | a. | Brief detail about the 16 projects and explain the implementation and planning strategies of water resources projects. | CO6 / R | 10 |
|  | b. | Elaborate the mechanics of water resource plan and international funding agencies. | CO6 / R | 10 |

Reg. No. _____________

End Semester Examination – June – 2022

| # Code : | # 18CE3039 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # WATER AND WASTEWATER TREATMENT TECHNOLOGY | # Max. Marks : | # 100 |

| Q. No. |  | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |
| 1. |  | Discuss water demand with its factors and recommendations as per BIS. | CO2 / R | 16 |
| 2. |  | Write a detailed note on water quality parameters and their significands for domestic purposes. | CO1 / U | 16 |
| 3. |  | Discuss in detail pipe appurtenances used in the water supply system | CO3 / AP | 16 |
| 4. |  | Draw and explain the types of joints in the water supply system. | CO4 / AP | 16 |
| 5. |  | Explain with a neat sketch on patterns of the collection sewage system. | CO5 / U | 16 |
| 6. |  | Compose the working principle of oxidation ponds. | CO6 / AP | 16 |
| 7. |  | Compile the working principle of Ion exchange and reverse osmosis. | CO4 / AP | 16 |
| PART – B (1 X 20 = 20 MARKS) (Compulsory Question) |  |  |  |  |
| 8. |  | Elaborate on sludge Disposal methods. | CO6 / AP | 20 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Impart knowledge on characteristics and standards of water and wastewater |
| CO2 | Understand principle of water and wastewater treatment systems |
| CO3 | Design water and wastewater treatment systems |
| CO4 | Evaluate efficiency of treatment units |
| CO5 | Illustrate the design principles of industrial wastewater |
| CO6 | Apply knowledge on the disposal and treatment of sludge |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | - | 16 | - | - | - | - | 16 |
| CO2 | 16 | - | 16 | - | - | - | 32 |
| CO3 | - | - | 16 | - | - | - | 16 |
| CO4 | - | - | 16 | - | - | - | 16 |
| CO5 | - | 16 | - | - | - | - | 16 |
| CO6 | - | - | 36 | - | - | - | 36 |
|  |  |  |  |  |  |  | 132 |

Reg. No. _____________

End Semester Examination – June – 2022

| # Code : | # 18CE3050 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # NOISE POLLUTION AND CONTROL TECHNIQUES | # Max. Marks : | # 100 |

| Q. No. | Sub Div. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |
| 1. | a. | Explain decibel. Demonstrate the decibel scale by using a neat and clean diagram. | CO1 / U | 8 |
|  | b. | Explain the Indian standard and guidelines for noise pollution. | CO6 / U | 8 |
| 2. | a. | Define the control and prevention measures of noise pollution. | CO2 / R | 8 |
|  | b. | Define 1. Transportation sources 2. Construction sources | CO2 / R | 8 |
| 3. | a. | Define 1. Noise pollution remedies 2. L10, L50, and L90 | CO1 / R | 8 |
|  | b. | While recording weighted sound levels, four readings have been at a site at different times of the day. These reading areas are 20, 56, 66, and 42 dB. Analyze the average noise produced by the source. | CO2 / A | 8 |
| 4. | a. | Describe the sound-absorbing materials. Explain the types of sound-absorbing materials (Any four). | CO5 / U | 8 |
|  | b. | Define 1. Noise propagation 2. Sound Level Meter | CO1 / R | 8 |
| 5. | a. | Demonstrate how does a noise barrier work? Show some examples of noise barriers by using a neat and clean diagram. | CO4 / A | 8 |
|  | b. | Define 1. OSHA Noise Standards 2. Public education on noise pollution | CO6 / R | 8 |
| 6. | a. | Demonstrate the process of measurement of noise by using a neat and clean diagram. | CO1 / U | 8 |
|  | b. | Define 1. Psychoacoustic 2. Acoustic Parameters | CO3 / R | 8 |
| 7. | a. | Demonstrate the noise pollution level and its harmful effects on human health. | CO3 / U | 8 |
|  | b. | Explain Psychoacoustic by using a neat and clean diagram. Describe the physiological effects due to noise pollution. | CO3 / U | 8 |

| PART – B (1 X 20 = 20 MARKS) (Compulsory Question) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 8. | a. | Explain the Environmental Protection Act – 1986 | CO6 / U | 10 |
|  | b. | Define (Any two) 1. Legislation and administrative function 2. The Rajasthan noise control act 1963 3. The Aircraft act 1934 (Related to noise only) 4. Factories act 1948 (Related to noise only) | CO6 / R | 10 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Identify the nature and characteristics of noise pollution |
| CO2 | Analyze the noise pollution problems |
| CO3 | Detect the various effects of noise pollution |
| CO4 | Apply suitable preventive measures and identify the technologies and methods to control of noise |
| CO5 | Choose suitable noise adsorbent materials |
| CO6 | Gain knowledge about the various noise pollution regulations |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 16 | 16 | - | - | - | - | 32 |
| CO2 | 16 |  | 8 | - | - | - | 24 |
| CO3 | 8 | 16 | - | - | - | - | 24 |
| CO4 | - | - | 8 | - | - | - | 8 |
| CO5 | - | 8 | - | - | - | - | 8 |
| CO6 | 18 | 18 | - | - | - | - | 36 |
|  |  |  |  |  |  |  | 132 |

Reg. No. _____________

End Semester Examination –May / June – 2022

| # Code : | # 20CE1002 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ENGINEERING MECHANICS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Define Centroid. | CO5 / R | 1 |
| 2. | Define the term Moment. | CO5 / R | 1 |
| 3. | Define the term Force. | CO1 / R | 1 |
| 4. | What is a collinear system of forces? | CO1 / R | 1 |
| 5. | Define the term Beam. | CO2 / R | 1 |
| 6. | State the three conditions of static equilibrium. | CO1 / U | 1 |
| 7. | Define time of flight. | CO3 / R | 1 |
| 8. | State Newton’s second law of motion. | CO3 / R | 1 |
| 9. | Define co-efficient of friction. | CO2 / R | 1 |
| 10. | Define Energy. | CO1 / R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Define Varignon’s theorem. | CO3 / R | 3 |
| 12. | List the three important End supports of a beam. | CO4 / U | 3 |
| 13. | Discuss about Types of Equilibrium. | CO1 / R | 3 |
| 14. | Distinguish between centroid and centre of gravity | CO5 / U | 3 |
| 15. | Give the Work-Energy equation and express it terms. | CO2 / R | 3 |
| 16. | A bullet of mass 1kg is fired from a gun with a velocity of 500m/s. The length of the barrel of the gun is 2m. Calculate the force exerted by the gun. | CO 6 / U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. |  | The following forces act at a point: a. 200N inclined at 300 towards North of East b. 250N towards North c. 300 N towards North of west at 45o 350 N inclined at 40o towards south of west. Find the resultant of the force system. | CO6 / A | 12 |
| 18. |  | A uniform ladder 3m long weighs 250N. It is placed against a wall making an angle of 60o with the floor. The coefficient of friction between the wall and the ladder is 0.25 and that between floor and ladder is 0.35. The ladder in addition to its own weight has to support a man of 700 N at its top. Find the minimum force required to prevent slipping of the ladder at floor level. | CO6 / A | 12 |

| 19. |  | Locate the centroid of the plane lamina as shown in the figure. | CO5 / A | 12 |
| --- | --- | --- | --- | --- |
| 20. |  | Mr. Sudhan is driving his car at 100km/hr. When he observes that a traffic light 50m ahead turns red, the traffic light is timed to remain red for 15 seconds. If he wishes to pass the light without stopping just as it turns green, determine: a. the required uniform deceleration of the car b. the speed of the car as it passes the traffic light. | CO6 / A | 12 |
| 21. |  | Find the reactions at A and B of the beam shown. A B 1 m 2 m 1 m 10 KN 5 KN/m 10 KN/m 20 KN 300 2 m 3 m | CO5 / A | 12 |
| 22. |  | Analyze the pin jointed truss as shown below by method of joints. | CO4 / A | 12 |

| 23. |  | Two cars A and B start from rest at the same time from two stations spaced ‘L’ apart along the same straight road. Car ‘A’ travels first with an acceleration of 0.6 m/s2 reaching a maximum velocity of 30 m/s and travels uniformly with this velocity. Car ‘B’ travels first with an acceleration of 0.75 m/s2 reaching a maximum velocity of 24 m/s. Both the cars cross each other at ‘C’ which is exactly mid way between the two stations. Determine ‘L’ and the time taken by each car to cross ‘C’ | CO5 / A | 12 |
| --- | --- | --- | --- | --- |
|  |  | Compulsory: |  |  |
| 24. |  | Find the Moment of Inertia of a I section of flange 100mm x 30mm and web 20mm x 80mm about its centroidal axes. | CO5 / A | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Illustrate the concepts of mechanics |
| CO2 | Identify the principles of dynamics |
| CO3 | Examine the concepts of kinetics |
| CO4 | Analyse the stresses in the members |
| CO5 | Apply the equilibrium concepts in analysis of members |
| CO6 | Apply the basic principles to solve problems in mechanics |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 6 | 1 | - | - | - | - | 7 |
| CO2 | 5 | - | - | - | - | - | 5 |
| CO3 | 5 | - | - | - | - | - | 5 |
| CO4 | - | 3 | 12 | - | - | - | 15 |
| CO5 | 2 | 3 | 48 | - | - | - | 53 |
| CO6 | - | 3 | 36 | - | - | - | 39 |
|  |  |  |  |  |  |  | 124 |

Reg.No. _____________

End Semester Examination April / May – 2022

| # Code : | # 20CE1004 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # SUSTAINABLE BUILDING MATERIALS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- |
| PART – A (10X1 = 10 MARKS) |  |  |  |
| 1. | What are the test carried for checking the strength of the stone? | CO1 | 1 |
| 2. | List out the steps for the production of concrete. | CO1 | 1 |
| 3. | Name some of the sustainable building materials. | CO2 | 1 |
| 4. | How rubber is used in civil construction? | CO2 | 1 |
| 5. | Why curing is needed? | CO3 | 1 |
| 6. | The blocks are stablized with a chemical binder are called_____ | CO3 | 1 |
| 7. | Name the two methods of creating the nanstructures | CO4 | 1 |
| 8. | How shipping containers used in contruction practices? | CO5 | 1 |
| 9. | Give the types of rheological fluids. | CO6 | 1 |
| 10. | Define thermochromic materials. | CO6 | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Define the term Workability of concrete. | CO1 | 3 |
| 12. | Why the sustainable construction is needed for current scenario? | CO3 | 3 |
| 13. | Identify the composition of good soil. | CO2 | 3 |
| 14. | Select the suitable structure that uses ferrocement in construction. | CO4 | 3 |
| 15. | How the building demolition waste is used again in industries? | CO5 | 3 |
| 16. | Summarize the context of absorbing brick in reducing pollution. | CO6 | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is a Compulsory Question) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | Classify the substructure and superstructure of the building and explain with diagram. | CO1 | 6 |
|  | b. | List the types of test conducted for checking the workability of concrete and explain with diagram. | CO1 | 6 |
| 18. | a. | Delinate the types of cement and its properties. | CO1 | 6 |
|  | b. | Distinguish the renewable material and recycled material with examples. | CO2 | 6 |
| 19. | a. | Describe the paper insulation in construction with its merits and demerits | CO2 | 4 |
|  | b. | Examine the properties need to be considered in selecting the paints, finishes and adhesives has for the residential construction. | CO2 | 8 |

| 20. | a. | Categorize the types of sustatainable mud blocks available in construction with the advantages and disadvantages. | CO3 | 6 |
| --- | --- | --- | --- | --- |
|  | b. | Elaborate the Funicular shell and its components. | CO3 | 6 |
| 21. | a. | Examine the reason Geopolymer concrete is used as sustainable material instead of OPC in construction. And state the constituents of geopolymer concrete. | CO4 | 6 |
|  | b. | Delinate the 3D printing technology and its usage. | CO4 | 6 |
| 22. | a. | Explain the fly-ash properties and its practice in construction field. | CO5 | 6 |
|  | b. | Select the suitable way to use the demolition waste in various fields of construction. | CO5 | 6 |
| 23. | a. | Enumerate the consumption of salvaged materials in contruction industy. | CO5 | 6 |
|  | b. | Inspect the provision of concrete in controlling the bacteria and smart building construction. | CO6 | 6 |
|  |  | Compulsory |  |  |
| 24. | a. | Discuss in detail about the photochromotic materials. | CO6 | 6 |
|  | b. | Distinguish the types of fiber optics available for construction. | CO6 | 6 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Identify the sustainable building materials |
| CO2 | Distinguish between conventional and modern construction materials |
| CO3 | Define the concepts of embodied carbon or carbon footprint |
| CO4 | Identify the different sustainable construction techniques |
| CO5 | Analyze the usage of waste materials for construction |
| CO6 | Analyze the use of smart materials |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 2 |  | 3 | 24 |  |  | 29 |
| CO2 |  | 4 | 12 |  |  |  | 16 |
| CO3 | 3 | 3 |  | 12 |  |  | 18 |
| CO4 |  | 4 |  | 12 |  |  | 16 |
| CO5 |  | 4 |  | 12 |  |  | 16 |
| CO6 | 2 | 3 |  | 24 |  |  | 29 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – May / June – 2022

| # Code : | # 20CE1004 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # SUSTAINABLE BUILDING MATERIALS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | What are the tests carried for checking the strength of the stone? | CO1 | R | 1 |
| 2. | List out the steps for the production of concrete. | CO1 | U | 1 |
| 3. | Name some of the sustainable building materials. | CO2 | R | 1 |
| 4. | How rubber is used in civil construction? | CO2 | U | 1 |
| 5. | Why curing is needed? | CO3 | U | 1 |
| 6. | The blocks are stablized with a chemical binder are called_________. | CO3 | R | 1 |
| 7. | Name the two methods of creating the nanostructures. | CO4 | R | 1 |
| 8. | How shipping containers used in contruction practices? | CO5 | R | 1 |
| 9. | Give the types of rheological fluids. | CO6 | U | 1 |
| 10. | Define thermochromic materials. | CO6 | U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Define the term Workability of concrete. | CO1 | U | 3 |
| 12. | Why the sustainable construction is needed for current scenario? | CO2 | U | 3 |
| 13. | Identify the composition of good soil. | CO3 | U | 3 |
| 14. | Select the suitable structure that uses ferrocement in construction. | CO4 | U | 3 |
| 15. | How the building demolition waste is used again in industries? | CO5 | U | 3 |
| 16. | Summarize the context of absorbing brick in reducing pollution. | CO6 | U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | a. | Classify the substructure and superstructure of the building and explain with diagram. | CO1 | U | 6 |
|  | b. | List the types of test conducted for checking the workability of concrete and explain with diagram. | CO1 | A | 6 |
| 18. | a. | Delineate the types of cement and its properties. | CO1 | A | 6 |
|  | b. | Distinguish the renewable material and recycled material with examples. | CO2 | U | 6 |

| 19. | a. | Describe the paper insulation in construction with its merits and demerits | CO2 | A | 6 |
| --- | --- | --- | --- | --- | --- |
|  | b. | Examine the properties need to be considered in selecting the paints, finishes and adhesives has for the residential construction. | CO2 | U | 6 |
| 20. | a. | Categorize the types of sustainable mud blocks available in construction with the advantages and disadvantages. | CO3 | An | 6 |
|  | b. | Elaborate the Funicular shell and its components. | CO3 | A | 6 |
| 21. | a. | Examine the reason Geopolymer concrete is used as sustainable material instead of OPC in construction. And state the constituents of geopolymer concrete. | CO4 | A | 6 |
|  | b. | Delineate the 3D printing technology and its usage. | CO4 | U | 6 |
| 22. | a. | Explain the fly-ash properties and its practice in construction field. | CO5 | A | 6 |
|  | b. | Select the suitable way to use the demolition waste in various fields of construction. | CO5 | An | 6 |
| 23. | a. | Enumerate the consumption of salvaged materials in construction industry. | CO5 | A | 6 |
|  | b. | Inspect the provision of concrete in controlling the bacteria and smart building construction. | CO6 | U | 6 |
|  |  | Compulsory: |  |  |  |
| 24. | a. | Discuss in detail about Shape Memory Alloy with suitable example. | CO6 | A | 6 |
|  | b. | Distinguish the types of smart structures available for construction. | CO6 | U | 6 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Identify the sustainable building materials |
| CO2 | Distinguish between conventional and modern construction materials |
| CO3 | Define the concepts of embodied carbon or carbon footprint |
| CO4 | Identify the different sustainable construction techniques |
| CO5 | Analyze the usage of waste materials for construction |
| CO6 | Analyze the use of smart materials |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 1 | 10 | 12 | - | - | - | 23 |
| CO2 | 1 | 8 | 6 | - | - | - | 15 |
| CO3 | 1 | 4 | 6 | 6 | - | - | 17 |
| CO4 | 1 | 9 | 6 | - | - | - | 16 |
| CO5 | 1 | 3 | 12 | 6 | - | - | 22 |
| CO6 | - | 16 | 6 | - | - | - | 32 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 20CE2001 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # SURVEYING AND GEOMATICS ENGINEERING | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Recall the least count of levelling staff. | CO 1 /R | 1 |
| 2. | Define levelling. | CO 3 /R | 1 |
| 3. | List the different methods to find horizontal angle between two points. | CO 2 /R | 1 |
| 4. | Define latitude. | CO2 /R | 1 |
| 5. | List the methods of tacheometric surveying. | CO 3 /R | 1 |
| 6. | What is tacheometer? | CO 1 / R | 1 |
| 7. | Define curve. | CO 4 /R | 1 |
| 8. | Specify the components of simple curve. | CO 4 /R | 1 |
| 9. | Write the advantages of total station. | CO 5 /R | 1 |
| 10. | What is setting out? | CO4 /R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | List the different types of bench marks. | CO3 / R | 3 |
| 12. | List out the temporary adjustment of a theodolite and explain them with neat sketch. | CO1 / R | 3 |
| 13. | Define latitude and departure | CO2 / R | 3 |
| 14. | Explain the advantages and disadvantages of anallaticlens. | CO 6 / U | 3 |
| 15. | Classify the components of simple curve. | CO 4 / U | 3 |
| 16. | Explain in detail about drone surveying. | CO 5/ U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | The following consecutive readings were taken with a level and a 4 -meter leveling staff on continuously sloping ground at common interval of 30m: 0.855(onA), 1.545m, 2.335, 3.115, 3.825, 0.455, 1.380, 2.055, 2.855, 3.455, 0.585, 1.015, 1.850, 2.755, 3.845 (on B) The RL of A was 380.500m. Make entries in a level book and apply the usual Checks. Also calculate the gradient of AB. | CO3 / A | 12 |

| 18. |  | The following records are obtained in a traverse survey, where the length and bearing of the last line were not recorded: Compute the length and bearing of the line DA. LINE; LENGTH (m); BEARING. AB; 75.50; 30o 24'. BC; 180.50; 110 o 36’. CD; 60.25; 210 o30’. DA; ?; ?. | CO3 / A |  | 12 |
| --- | --- | --- | --- | --- | --- |
| 19. |  | The following observations were taken with a tachometer fitted with an anallatic lens , the staff being held vertically. The constant of the tachometer is 100 and 0. Int. Station; Height of instrument; Staff station; Vertical angle; Staff readings (m); Remark. P; 1.255; BM; -4°20’; 1.325, 1.825,2.325; RL of BM = 255.750m. P; 1.255; A; +6°30’; 0.850, 1.600, 2.350. P; 1.450; A; -7°24’; 1.715, 2.315,2.915. Calculate the RL of B and the distance between A and B. | CO2 / A |  | 12 |
| 20. |  | Two straights intersect at chainage 1610 m, the deflection angle being 15o. Calculate all the data necessary to set out a 5o simple right-handed curve by the Rankine’s method of angles. Peg interval may be taken as 25m. Draw the table of deflection angles. | CO1 / An |  | 12 |
| 21. |  | Explain principle and classification of Total station. | CO5 / U |  | 12 |
| 22. |  | Explain the following : - Simple curve - Compound curve - Reverse curve - Transition curve | CO 4 / U |  | 12 |
| 23. |  | Explain the different classification of levelling. | CO6 / U |  | 12 |
|  |  | Compulsory: |  |  |  |
| 24. | a. | Explain the benefits of drone surveying. | CO5 / U | 6 |  |
|  | b. | Explain the application of drone surveying. | CO6 / U | 6 |  |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Recall the principles of surveying equipment’s |
| CO2 | Select methods to measure angles and distances |
| CO3 | Schedule field surveying operations |
| CO4 | Examine the implementation of surveying procedures for setting out curves |
| CO5 | Appraise the usage of equipment’s and methods in triangulation survey |
| CO6 | Formulate the surveying methods and executions |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 5 | - | - | 12 | - | - | 17 |
| CO2 | 5 | - | 12 | - | - | - | 17 |
| CO3 | 5 | - | 24 | - | - | - | 29 |
| CO4 | 3 | 15 | - | - | - | - | 18 |
| CO5 | 1 | 21 | - | - | - | - | 22 |
| CO6 | - | 21 | - | - | - | - | 21 |
|  |  |  |  |  |  |  | 124 |

Reg.No. _____________

End Semester Examination April / May – 2022

| # Code : | # 20CE2003 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # FLUID MECHANICS AND MACHINERY | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- |
| PART – A (10X1 = 10 MARKS) |  |  |  |
| 1. | Compare dynamic viscosity and kinematic viscosity. | CO1 | 1 |
| 2. | Infer metacentric height and stability of floating bodies. | CO1 | 1 |
| 3. | What is buoyancy? | CO3 | 1 |
| 4. | Deduce streak line. | CO3 | 1 |
| 5. | Name the type of notch / weir which is best suited for measuring small (low) discharges. | CO2 | 1 |
| 6. | What is hydraulic gradient line? | CO4 | 1 |
| 7. | Paraphrase laminar flow. | CO3 | 1 |
| 8. | Mention the value of Reynold`s number up to which laminar boundary layer occurs. | CO5 | 1 |
| 9. | Clarify dimensional homogeneity. | CO6 | 1 |
| 10. | State Reynolds Theorem. | CO6 | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Find the kinematic viscosity of a liquid whose specific gravity is 0.95 and dynamic viscosity 1.2 x 10-3 N-s/m2. | CO1 | 3 |
| 12. | The diameters of a pipe at two sections 1-1 and 2-2 are 200mm and 300mm respectively. If the velocity of water flowing through the pipe at section 1-1 is 4 m/s. Find : i) Discharge through the pipe and ii) Velocity of water at section 2-2 | CO3 | 3 |
| 13. | Give the formula to find friction loss highlighting the friction coefficient. | CO2 | 3 |
| 14. | Mention the importance of Moody’s diagram. | CO4 | 3 |
| 15. | List any three methods of preventing the separation of boundary layer. | CO5 | 3 |
| 16. | Identify the similarities that must be ensured between the model and the prototype. | CO6 | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is a Compulsory Question) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | An oil film of thickness 1.2mm is used for lubrication between a square plate of size 0.9m x 0.9m and an inclined plane having an angle of inclination 20˚. The weight of the square plate is 400N and it slides down the plane with a uniform velocity of 0.25m/sec. Find the dynamic viscosity of the oil. | CO1 | 6 |
|  | b. | A hydraulic press has a ram of 20 cm diameter and a plunger of 3 cm diameter. It is used for lifting a weight of 30 kN. Find the force required at the plunger. | CO1 | 6 |

| 18 | a. | Determine the total pressure on a circular plate of diameter 1.5 m which is placed vertically in water in such a way that the centre of the plate is 3 m below the free surface of water. Also find the position of centre of pressure. | CO1 | 6 |
| --- | --- | --- | --- | --- |
|  | b. | State Bernoulli’s equation for the flow of an incompressible fluid. Derive an expression for Bernoulli’s theorem from first principle and state the assumptions made for such a derivation. | CO1 | 6 |
| 19. | a. | Find the discharge over a rectangular weir of length 80m. The head of water over the weir is 1.2m. The velocity of approach is 1.5m/s. Assume Cd = 0.6 | CO2 | 4 |
|  | b. | A horizontal venturimeter with inlet diameter 200mm and throat diameter 100mm. is used to measure the flow of water. The pressure at inlet is 0.18 N/mm2 and the vaccum pressure at the throat is 280mm of mercury. Find the rate of flow. Cd = 0.98. | CO2 | 8 |
| 20. |  | A piping system consists of three pipes arranged in series. The lengths are 1200m, 750, 600m and diameters are 750mm, 600mm, and 450mm respectively. i) Transform the system to an equivalent of 450mm diameter pipe. ii) Determine an equivalent diameter pipe for a 2550m long pipe. | CO4 | 12 |
| 21. | a. | A 2mm diameter metallic ball of specific gravity 11 is allowed to fall in a fluid of specific gravity 0.9 and viscosity 1.4 Ns/m2. Determine: Drag force exerted by the fluid on the ball. | CO5 | 6 |
|  | b. | A 1.8m wide 5m long plate moves through stationary air of density 1.22 kg/m3 and viscosity 1.8 x 10-5 Ns/m2 at a velocity of 1.75 m/s parallel to its length. Determine the drag force on one side of the plate. i) Assuming laminar flow condition ii) Assuming turbulent flow condition | CO5 | 6 |
| 22. | a. | Discuss the criteria for selection of repeating variables in Buckingham method. | CO6 | 4 |
|  | b. | The resisting force F of a plane during flight can be considered as dependent on length of aircraft l, velocity v, viscosity , density , bulk modulus of air K. Express the functional relationship between these variables and the resisting force using Buckingham theorem method. | CO6 | 8 |
| 23. | a. | Differentiate between pipes in series and pipes in parallel. | CO4 | 4 |
|  | b. | A horizontal pipe 0.1m in diameter is joined by a sudden enlargement to a 0.15m diameter pipe. Water is flowing through it at the rate of 2m3/min. Find the loss of head due to abrupt expansion and the pressure difference in the two pipes. If the change of section is gradual with out loss, what would be the change pressure. | CO4 | 8 |
|  |  | Compulsory: |  |  |
| 24. |  | The pressure drop in a pipe of diameter D and length depends on the density and viscosity of the fluid flowing, mean velocity V of flow and the average height of protuberance t, show that pressure drop . | CO6 | 12 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 20CE2004 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name: | # SOIL MECHANICS AND FOUNDATION ENGINEERING | # Max. Marks: | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | List the types of soil. | CO2 | R | 1 |
| 2. | Sketch the soil phase diagram. | CO2 | A | 1 |
| 3. | Discuss quick condition. | CO3 | U | 1 |
| 4. | Formulate the equation for permeability. | CO1 | C | 1 |
| 5. | Differentiate consolidation and compaction in basic parameter. | CO1 | An | 1 |
| 6. | Defend compaction over consolidation. | CO3 | E | 1 |
| 7. | Define soil exploration. | CO4 | R | 1 |
| 8. | Classify the methods of penetration test. | CO4 | U | 1 |
| 9. | Write the use of Newmark’s influence chart. | CO4 | A | 1 |
| 10. | Differentiate Mohr coulomb’s strength theory and Modified Mohr coulomb’s theory. | CO4 | An | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Appraise the techniques used for sampling. | CO4 | E | 3 |
| 12. | Formulate Terzaghi’s one dimensional consolidation theory. | CO3 | C | 3 |
| 13. | Explain the physical characterization of soil. | CO1 | U | 3 |
| 14. | Explain field tests based on drainage conditions. | CO3 | U | 3 |
| 15. | Interpret the factors affecting permeability. | CO3 | A | 3 |
| 16. | Compare deep and shallow foundation. | CO5 | An | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | a. | Justify the wet and dry sieve analysis used for soil classification. | CO2 | E | 4 |
|  | b. | Construct the IS classification as per the code. | CO2 | C | 4 |
|  | c. | Discuss about the soil nomenclature. | CO2 | U | 4 |
| 18. | a. | Define Darcy's law and its validity. | CO1 | R | 4 |
|  | b. | Demonstrate the factors affecting permeability. | CO3 | A | 4 |
|  | c. | Compare permeability and seepage pressure. | CO3 | An | 4 |
| 19. | a. | Justify the consolidation and compaction. | CO3 | E | 4 |
|  | b. | Assemble the mechanism of consolidation through spring analogy. | CO3 | C | 4 |
|  | c. | List and explain the methods of compaction. | CO3 | R | 4 |

| 20. | a. | Discuss Boussinesq theory. | CO3 |  | U | 4 |
| --- | --- | --- | --- | --- | --- | --- |
|  | b. | Sketch the Newmark’s influence chart. | CO3 |  | A | 4 |
|  | c. | Differentiate Mohr's strength theory and Mohr coulomb’s strength theory. | CO3 |  | An | 4 |
| 21. | a. | Appraise the semi-direct methods of exploration. | CO4 |  | E | 4 |
|  | b. | Formulate the depth and spacing of boring. | CO4 |  | C | 4 |
|  | c. | Define bore-log report and data interpretation. | CO4 |  | R | 4 |
| 22. | a. | Discuss in detail the sampling methods. | CO4 |  | U | 4 |
|  | b. | Demonstrate SPT. | CO4 |  | A | 4 |
|  | c. | Elaborate the static test used for penetration. | CO4 |  | An | 4 |
| 23. | a. | Appraise the Atterberg’s indices. | CO2 |  | E | 4 |
|  | b. | Deign the phase diagram and geological cycle. | CO2 |  | C | 4 |
|  | c. | List the soil types and explain its formation. | CO2 |  | R | 4 |
|  |  | Compulsory: |  |  |  |  |
| 24. | a. | Explain deep foundation. | CO5 | U |  | 4 |
|  | b. | Interpret shallow foundation. | CO5 | A |  | 4 |
|  | c. | Compare shallow and deep foundation. | CO5 | An |  | 4 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | List the basic equations of elasticity |
| CO2 | Distinguish between the applications of different types of finite elements. |
| CO3 | Develop the finite element discrimination for seepage, consolidation soil structure interaction problems. |
| CO4 | Identify the suitable foundation construction |
| CO5 | Design the foundation system for shallow depth |
| CO6 | Analyse the earth retaining structures for different soil medium |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 4 | 3 | - | 1 | - | 1 | 9 |
| CO2 | 5 | 4 | 1 | - | 8 | 8 | 26 |
| CO3 | 4 | 8 | 11 | 7 | 5 | 7 | 42 |
| CO4 | 5 | 5 | 5 | 5 | 7 | 4 | 31 |
| CO5 | - | 5 | 4 | 7 | - | - | 16 |
| CO6 | - | - | - | - | - | - | - |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 20CE2005 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # WATER SUPPLY AND SANITARY ENGINEERING | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | List the sources of water supply. | CO1 / R | 1 |
| 2. | Mention any three chemical characteristics of water with its permissible limit. | CO1 / R | 1 |
| 3. | List the objectives of a waste water treatment. | CO2 / R | 1 |
| 4. | What is sewerage system? | CO2 / U | 1 |
| 5. | Name any five types of pipes used for water conveyance. | CO1 / AP | 1 |
| 6. | Classify water supply system. | CO3 / U | 1 |
| 7. | Recall the purposes of manhole. | CO3 / U | 1 |
| 8. | Differentiate Conservancy system from water carriage system. | CO4 / U | 1 |
| 9. | List the types of traps. | CO4 / AP | 1 |
| 10. | Name the components of house plumbing. | CO5 / U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Define water quality index. | CO3 / R | 3 |
| 12. | Distinguish unit operation from unit process. | CO4 / U | 3 |
| 13. | List the advantages of any three pipe joints. | CO6 / AP | 3 |
| 14. | Mention the safety measures for sewer men. | CO5 / AP | 3 |
| 15. | Define the term plumbing. | CO2 / U | 3 |
| 16. | Recall the meaning of “recycle of water”. | CO1 / R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | State all the population forecasting methods and explain. | CO1 / AP | 12 |
| 18. | a. | Enumerate and explain the various sewer appurtenances with a neat sketches. | CO3 / AP | 12 |
| 19. | a. | Explain the various physio-chemical characteristics of sewage and state their environmental significance. | CO5 / R | 12 |
| 20. | a. | Discuss a detailed note on leakage control system in water supply. | CO6 / AP | 6 |
|  | b. | Write a note on water conservation. | CO3 / AP | 6 |
| 21. | a. | Write a short note on sewer joints. | CO1 / R | 6 |
|  | b. | Describe the merits and demerits of different sewer sections. | CO2 / R | 6 |
| 22. | a. | Discuss in detail about house plumbing. | CO4 / R | 12 |
| 23. | a. | Describe in detail on advanced recycle methods. | CO5 / R | 12 |
|  |  | Compulsory: |  |  |
| 24. | a. | Draw a neat sketch of filter units and explain its working principle. | CO3 / U | 6 |
|  | b. | Discuss the operation and maintenance of sewage treatment system. | CO6 / U | 6 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 20CE2006 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # SOLID WASTE MANAGEMENT | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | Define solid waste. | CO1 | R | 1 |
| 2. | Define integrated solid waste management. | CO2 | U | 1 |
| 3. | Explain the landfill impacts. | CO1 | U | 1 |
| 4. | What are the uses of biogas? | CO1 | R | 1 |
| 5. | Explain the grading of risk. | CO4 | U | 1 |
| 6. | Show the dose-response assessment. | CO5 | A | 1 |
| 7. | Explain vermin composting. | CO6 | U | 1 |
| 8. | Define the passive composting. | CO3 | R | 1 |
| 9. | Define the segregation of waste. | CO6 | R | 1 |
| 10. | What is hospital waste? | CO1 | R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Explain the need for transfer operations. | CO2 | R | 3 |
| 12. | Explain the classification of waste by using different sources. | CO1 | U | 3 |
| 13. | Demonstrate a simplified material-flow diagram indicating the path of generation of solid wastes. | CO2 | U | 3 |
| 14. | Explain the steps in the management of biomedical waste by using a neat and clean diagram. | CO5 | U | 3 |
| 15. | Explain the advantage and limitations of the biogas plant. | CO3 | U | 3 |
| 16. | Explain the composting by using a neat and clean diagram. | CO3 | U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | a. | Apply the types of transfer stations by using a neat and clean diagram. | CO2 | A | 6 |
|  | b. | Explain the following terms by using neat a clean diagram. 1. Hauled container system (HCS) 2. Stationary container system (SCS) | CO4 | U | 6 |
| 18. | a. | Describe the solid waste management rule 2016. | CO5 | U | 6 |
|  | b. | Explain the hazardous/industrial waste. | CO1 | U | 6 |

| 19. | a. | Explain the separation of solid waste at the source. | CO3 | U | 6 |
| --- | --- | --- | --- | --- | --- |
|  | b. | Define the composition of solid waste. | CO3 | R | 6 |
| 20. | a. | Describe the basic difference between dump and landfill. Explain essential components of landfills. | CO3 | U | 6 |
|  | b. | Describe the landfill types and classification. | CO4 | U | 6 |
| 21. | a. | Analyze the color coding for the segregation of biomedical waste. | CO1 | An | 6 |
|  | b. | Define the environmental risk assessment framework by using a neat and clean diagram. | CO3 | U | 6 |
| 22. | a. | Explain the recycling of solid waste by using a neat and clean diagram. | CO6 | U | 6 |
|  | b. | Explain the biomedical waste treatment and disposal. | CO3 | U | 6 |
| 23. | a. | Define the factor causing variation in quantity and composition of solid waste (any two). i) Geographic locations ii) Population diversity iii) Public attitude | CO1 | U | 6 |
|  | b. | Classify the key functions of the waste exchange. | CO2 | U | 6 |
|  |  | Compulsory: |  |  |  |
| 24. | a. | Explain the types of a biogas plant in detail by using a neat and clean diagram. | CO6 | U | 6 |
|  | b. | Explain the advantage and disadvantages of incineration. Highlight the important factor for incineration. | CO6 | U | 6 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Analyse the nature and characteristics of municipal solid wastes |
| CO2 | Sort out the functional elements for solid waste management |
| CO3 | Apply the techniques and methods used in transformation, conservation and recovery of materials from solid waste |
| CO4 | Identify and design waste containment systems |
| CO5 | Gain knowledge in regulatory requirements regarding municipal solid waste management |
| CO6 | Apply the basic scientific principles for solving practical waste management challenges |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 3 | 16 | - | 6 | - | - | 25 |
| CO2 | 3 | 10 | 6 | - | - | - | 19 |
| CO3 | 7 | 30 | - | - | - | - | 37 |
| CO4 | - | 13 | - | - | - | - | 13 |
| CO5 | - | 9 | 1 | - | - | - | 10 |
| CO6 | 1 | 19 | - | - | - | - | 20 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 20CE2007 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # TRANSPORTATION ENGINEERING | # Max. Marks : | # 100 |

| Q. No. | Questions |  | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- | --- |
|  |  | PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | State the classification of road system followed from third twenty year road plan. |  | CO1 | R | 1 |
| 2. | Mention the factors that control the highway alignment. |  | CO4 | U | 1 |
| 3. | List the items included in inventory survey. |  | CO3 | R | 1 |
| 4. | Differentiate travel time and running time. |  | CO3 | U | 1 |
| 5. | Write the significance of CBR value on selection of subgrade soil. |  | CO6 | U | 1 |
| 6. | Define the term subgrade reaction. |  | CO6 | R | 1 |
| 7. | List various data to be collected for railway track alignment. |  | CO2 | R | 1 |
| 8. | List different gauge systems used in India. |  | CO2 | R | 1 |
| 9. | State the functions of rail. |  | CO5 | U | 1 |
| 10. | What is creep? |  | CO2 | U | 1 |

|  |  | PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 11. | Describe Roman roads. |  | CO1 | U | 3 |
| 12. | Write the objectives of spot speed study. |  | CO3 | U | 3 |
| 13. | Describe the concept of considering the wheel loads for the design of pavements. |  | CO6 | U | 3 |
| 14. | Explain the various cross sectional elements of a highway. |  | CO4 | U | 3 |
| 15. | Describe theory of coning. |  | CO5 | U | 3 |
| 16. | Explain the types of Marshalling yard. |  | CO2 | U | 3 |

|  |  | PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 17. | a. |  | Explain the various data to be collected for the alignment of the highway. | CO1 | U | 6 |
|  | b. |  | Discuss the recommendations by Jayakar Committee. | CO1 | U | 6 |
| 18. | a. |  | Explain two methods for conducting spot speed surveys. | CO3 | U | 6 |
|  | b. |  | Discuss the importance of traffic volume studies. | CO3 | U | 6 |
|  |  |  |  |  | U |  |
| 19. | a. |  | Explain the desirable properties of coarse aggregate to be used for road construction. | CO6 | U | 4 |
|  | b. |  | Design the flexible pavement for a NH having the following data: CBR of subgrade soil = 4%, CBR of soil aggregate mix = 20%, CBR of soil kankar aggregate mix = 10%, CBR of WBM coarse = 85%. The minimum thickness of bituminous macadam surfacing may be taken as 5cm. The last traffic count is 1450 commercial Vehicles/day. | CO4 | An | 8 |
| 20. | a. |  | Explain PIEV theory. | CO4 | U | 4 |
|  | b. |  | The speeds of overtaking and overtaken vehicles are 60 kmph and 50 kmph respectively on a two way traffic road. If the acceleration of overtaking vehicle is 0.9m/sec2, i) Calculate safe overtaking sight distance ii) Obtain the minimum length of overtaking zone and iii) Draw a neat sketch of overtaking zone show in the position of sign posts. Assume all other data as per IRC. | CO4 | A | 8 |
| 21. | a. |  | Draw the sketch of a turnout and explain its working principle. | CO5 | U | 6 |
|  | b. |  | Explain the various components of Railway Track with help of a neat sketch. | CO5 | U | 6 |
| 22. | a. |  | A new National Highway is to be aligned along a plain terrain. If the maximum radius which can be provided on the site is 200m, design all the geometric features of this curve. Assume suitable data. | CO4 | An | 8 |
|  | b. |  | Discuss the application of origin-destination survey. | CO3 | U | 4 |
|  |  |  |  |  | U |  |
| 23. | a. |  | Discuss various types of gradients which effect the design of vertical profile of a highway. | CO4 | U | 6 |
|  | b. |  | Discuss the importance of parking survey and explain the terms parking demand, parking volume and parking duration. | CO3 | U | 6 |
|  | Compulsory: |  |  |  |  |  |
| 24. | a. |  | Explain the classification of signals used in railways. | CO2 | U | 6 |
|  | b. |  | Discuss the advantages and disadvantages of precast sleepers. | CO2 | U | 6 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the concepts of development of highways and railway engineering |
| CO2 | Explain components of highway and railway engineering |
| CO3 | Carryout engineering surveys involved in planning of highway and railway engineering |
| CO4 | Design the geometric elements of highway and railway engineering |
| CO5 | Recognize the functions of structural elements of highway and railway engineering |
| CO6 | Identify the materials used for highway and railway construction |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 1 | 15 | - | - | - | - | 16 |
| CO2 | 4 | - | - | - | - | - | 4 |
| CO3 | 1 | 24 | - | - | - | - | 25 |
| CO4 | - | 14 | 6 | 16 |  |  | 36 |
| CO5 | - | 16 | - | - | - | - | 16 |
| CO6 | 1 | 8 | - | - | - | - | 9 |
|  |  |  |  |  |  |  | 106 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 20CE2011 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ANALYSIS OF STRUCTURES | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | Define Strain Energy. | CO1 | R | 1 |
| 2. | What is the deflection of the free end of a cantilever of length L subjected to a concentrated load P at the end? | CO2 | R | 1 |
| 3. | Find FEM for the member BC (MFBC) of the portal frame shown in fig. | CO5 | R | 1 |
| 4. | Outline the reason for sinking of supports in the beam. | CO1 | U | 1 |
| 5. | A two-span continuous beam ABC, fixed at ‘A’ and ‘C’ have an equal span of 0.5 each EI is same for both spans. Find The distribution factors for member BA. | CO6 | R | 1 |
| 6. | Write the other name of Moment Distribution method. | CO1 | R | 1 |
| 7. | What is meant by absolute maximum bending moment in a beam? | CO5 | R | 1 |
| 8. | Define influence line. | CO1 | R | 1 |
| 9. | The figure shows a two-hinged parabolic arch of spam L subjected to uniformly distributed load of intensity q per unit length. What is the maximum bending moment in the arch? | CO2 | R | 1 |
| 10. | Recall the degree of indeterminacy of a two hinged arch. | CO2 | R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Compare resilience and proof resilience. | CO1 | U | 3 |
| 12. | A rigid frame is having totally 10 joints including support joints. Out of slope-deflection and momentdistribution methods, which method would you prefer for analysis? Why? | CO6 | U | 3 |
| 13. | Find the fixed end moments for the beam shown in fig. | CO4 | R | 3 |
| 14. | Name the type of rolling loads for which the absolute maximum bending moment occurs at the midspan of a beam. | CO1 | R | 3 |
| 15. | Explain in detail the components of cables and their functions. | CO1 | U | 3 |
| 16. | Develop the stiffness matrix for a given frame shown in fig. | CO5 | A | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. |  | Using strain energy method, determine the vertical displacement at the free end of the cantilever beam shown in fig. below. Take E = 200 kN / mm2; I = 20 x 106 mm4. | CO3 | E | 12 |
| 18. |  | ### Analyse the continuous beam ABC shown in figure by slope deflection method. And draw Bending moment diagram. Take EI is constant. | CO4 | An | 12 |

| 19. |  | Analyse the structure loaded as shown in fig. by moment distribution method and sketch the bending moment diagram. | CO4 | An | 12 |
| --- | --- | --- | --- | --- | --- |
| 20. |  | Four point loads 8kN, 15kN, 15kN and 10kN have centre to centre spacing of 2m between consecutive loads and they traverse a girder of 30m span from left to right with 10kN load leading. Determine the maximum shear force at 8m from the left support. | CO3 | E | 12 |
| 21. |  | A three hinged parabolic arch, hinged at the crown and springing, has a horizontal span of 15m with a central rise of 3m. It carries a uniformly distributed load of 32kN/m over the left hand half of the span. Determine the Reactions at the supports and bending moment at 5m from the left hand hinge. | CO3 | E | 12 |
| 22. |  | A suspension cable of 130m horizontal span is supported at the same level. It is subjected to u.d.l. of 25kN/m. If the maximum tension in the cable is limited to 5000kN, Find the minimum central dip needed. | CO3 | R | 12 |
| 23. |  | Analyse the vertical deflection at joint B of the pin-jointed frame shown in figure below. AE is constant for all members. | CO4 | An | 12 |

|  |  | Compulsory: |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 24. |  | Analyse the continuous beam using Stiffness matrix method. EI is constant throughout the beam. | CO6 | An | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Illustrate the concepts and principles |
| CO2 | Explain the behaviour of structural elements |
| CO3 | Analyze the structural members for various forces |
| CO4 | Analyze the response of the Structural elements |
| CO5 | Develop suitable response intricacies |
| CO6 | Adapt suitable analysis procedure |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 6 | 7 | - | - | - | - | 13 |
| CO2 | 3 | - | - | - | - | - | 3 |
| CO3 | 12 | - | - | - | 36 | - | 48 |
| CO4 | - | - | - | 36 | - | - | 36 |
| CO5 | 2 | 3 | 3 | - | - | - | 8 |
| CO6 | 1 | 3 | - | 12 | - | - | 16 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 20CE2014 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name: | # WATER RESOURCES SYSTEMS | # Max. Marks: | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | Define evapotranspiration. | CO1 | R | 1 |
| 2. | What is direct flow? | CO3 | U | 1 |
| 3. | Define unit hydrograph. | CO1 | R | 1 |
| 4. | What is meant by surcharge storage? | CO1 | U | 1 |
| 5. | What is flood routing? | CO3 | U | 1 |
| 6. | Define perched aquifer. | CO5 | A | 1 |
| 7. | Define Darcy’s law. | CO1 | R | 1 |
| 8. | Define irrigation systems. | CO1 | R | 1 |
| 9. | What is consumptive irrigation requirement [CIR]? | CO1 | R | 1 |
| 10. | What are Weirs? | CO4 | R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Explain different forms of precipitation. | CO2 | U | 3 |
| 12. | Explain the components of runoff with a neat diagram. | CO1 | An | 3 |
| 13. | What is reservoir planning and write the objectives? | CO2 | U | 3 |
| 14. | Explain the assumptions of Dupuit-Theims theory for radial flow for groundwater. | CO5 | An | 3 |
| 15. | If wheat requires about 9.5cm of water every 28days, and the base period for wheat is140days, find out the value of delta for wheat. | CO3 | U | 3 |
| 16. | Discuss site selection for head works for river reach. | CO3 | R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | a | Explain Recording Rain gauges with diagrams. | CO2 | U | 6 |
|  | b | Recall the Empirical methods for Evapotranspiration (any three). | CO1 | R | 6 |
| 18. | a | A flood hydrograph of a river draining a catchment of 200 km2 due to a 6 h isolated storm is in the form of a triangle with a base of 70 h and a peak ordinate of 30 m3/s occurring at 10 hours from the start. Assuming zero base flow, develop the 6-hour unit hydrograph for this catchment. | CO4 | A | 6 |
|  | b | Characterize the physiographic and climatic factors affecting the Hydrographs. | CO2 | U | 6 |

| 19. | a | Classify the types of reservoirs. |  | CO4 |  | U |  | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | b | Discuss reservoir capacity. |  | CO4 |  | U |  | 6 |
| 20. | a | Elaborate the properties of the aquifers with diagrams, if necessary. |  | CO1 |  | A |  | 6 |
|  | b | Explain the specific capacity of a well. |  | CO6 |  | R |  | 6 |
| 21. | a | What are the irrigation and water efficiencies? |  | CO5 |  | R |  | 6 |
|  | b | Explain any 3 irrigation methods with their merits and demerits. |  | CO5 |  | U |  | 6 |
| 22. | a | What is river training works and write the objectives and purposes? |  | CO2 |  | An |  | 6 |
|  | b | Discuss the purposes of cross regulators and factors of selecting a suitable type of cross drain works? |  | CO4 |  | R |  | 6 |
| 23. | a | Describe the hydrological cycle process and write the basic components of the hydrological cycle with a relevant diagram |  | CO1 |  | U |  | 6 |
|  | b | The ordinate of 2-h unit hydrograph of a basin is given: Compute a 4-h unit hydrograph ordinate and plot: (i) the S-curve (ii) the 4-h UG Time; (h); 0; 2; 4; 6; 8; 10; 12. 2-h UH Ordinates; (m3/s); 0; 25; 100; 120; 160; 180; 110. Time; (h); 14; 16; 18; 20; 22; 24; 26. 2-h UH Ordinates; (m3/s); 70; 30; 20; 6; 0; 0; 0. | CO2 |  | An |  | 6 |
|  |  | Compulsory: |  |  |  |  |  |  |
| 24. | a | What are culverts? and differentiate between bridges and culverts. | CO4 |  | An |  | 6 |  |
|  | b | Explain Canal Fall, Canal Escapes. | CO3 |  | R |  | 6 |  |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Recognize the concepts to manage water resources and apply for hydrological modeling |
| CO2 | Decide and plan basic water resources projects |
| CO3 | Analyze the flow in streams |
| CO4 | Appreciate the importance of reservoirs and hydraulic structures |
| CO5 | Identify the irrigation methods |
| CO6 | Plan structures for research recharging groundwater |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 3 | 16 | - | 6 | - | - | 25 |
| CO2 | 3 | 10 | 6 | - | - | - | 19 |
| CO3 | 7 | 30 | - | - | - | - | 37 |
| CO4 | - | 13 | - | - | - | - | 13 |
| CO5 | - | 9 | 1 | - | - | - | 10 |
| CO6 | 1 | 19 | - | - | - | - | 20 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – June – 2022

| # Code : | # 20CE3001 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ADVANCED STRUCTURAL ANALYSIS | # Max. Marks : | # 100 |

| Q. No. |  | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |
| 1. |  | Analyze the continuous beam shown in figure using force method. | CO1 / An | 16 |
| 2. |  | Analyze the portal frame ABCD shown in figure by flexibility matrix method and sketch the bending moment diagram. | CO2 / An | 16 |
| 3. |  | Analyze the portal frame ABCD shown in figure by Stiffness matrix method and sketch the bending moment diagram. | CO2 / An | 16 |

| 4. |  | Determine the generalized displacement of pin jointed truss using direct stiffness method and hence determine element forces. AE is constant. A = 200 mm2 ; E = 200 kN/mm2. | CO2 / An | 16 |
| --- | --- | --- | --- | --- |
| 5. |  | Analyze the continuous beam shown in the fig. by Direct stiffness method. Assume that the supports are unyielding. Also assume that EI is constant for all members. | CO3 / An | 16 |
| 6. |  | Explain in detail the Static Condensation and sub structuring technique. | CO4 / U | 16 |
| 7. |  | Analyze the continuous beam shown in figure using stiffness method. | CO3 / An | 16 |
| PART – B (1 X 20 = 20 MARKS) (Compulsory Question) |  |  |  |  |
| 8. |  | Explain the term boundary condition and elaborate its various types applied on a structural frame with diagram. | CO5 / An | 20 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Identify the degrees of freedom and formulate flexibility and stiffness matrix |
| CO2 | Analyze the truss using stiffness methods |
| CO3 | Analyze the beams elements using appropriate methods |
| CO4 | Analyze the elements using advanced methods |
| CO5 | Evaluate the structural stability of frames |
| CO6 | Analyze the elements using software tools |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | - | - | - | 16 | - | - | 16 |
| CO2 | - | - | - | 48 | - | - | 48 |
| CO3 | - | - | - | 32 | - | - | 32 |
| CO4 | - | 16 | - | - | - | - | 16 |
| CO5 | - | - | - | 16 | - | - | 16 |
| CO6 | - | - | - | 20 | - | - | 20 |
|  |  |  |  |  |  |  | 148 |

Reg. No. _____________

End Semester Examination – June – 2022

| # Code : | # 20CE3002 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # DESIGN OF CONCRETE STRUCTURAL SYSTEMS | # Max. Marks : | # 100 |

| Q. No. |  | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |
| 1. |  | Write the steps to be followed for the calculation of crack width as per IS 456 : 2000 | CO1 / U | 16 |
| 2. |  | A three story RC building is located in seismic zone III on hard soil. The framing system of the building is moment-resisting frame without brick masonry infill panels. Find the base shear and its distribution along the height of the building. Column sections: 400mmx400mm, Beam sections: 230mmx300mm, Slab: 125mm thick RCC slab on all floors. Size of building 25x25m (equal bays in both direction), Height of each floor - 3m, Live load - 3kN/m2. Assume the relevant data. | CO2 / An | 16 |
| 3. |  | List the components of flat slab in RC buildings. Discuss the design procedure of flat slab under flexure and shear with sketches. | CO3 / R | 16 |
| 4. |  | Write short notes on shrinkage and thermal crack with examples. | CO1 / U | 16 |
| 5. |  | Calculate the design wind pressure of RC building for the following data; consider both methods as per IS 875 PIII Height of building – 40m (each floor 5m) Size of building – 30mx30m Location of building – Chennai Assume the relevant data | CO2 / An | 16 |
| 6. |  | Design the deep beam for the following data Effective depth – 1.6m Span – 2.4m UDL – 30kN/m M30 grade concrete, Fe500 steel, width of beam – 300mm Effective cover – 50mm | CO5 / A | 16 |
| 7. |  | Design the RCC corbel for the following data Axial load – 800kN Distance of loading from the column face – 150mm Bearing length – 600mm Clear cover – 40mm Column size – 600mmx600mm M30 Grade, Fe 500 steel | CO3 / E | 16 |

| PART – B (1 X 20 = 20 MARKS) (Compulsory Question) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 8. |  | Explain the ductile design and detailing requirement of RC beam and Column as per IS 13920 standards. | CO4 / A | 20 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Identify the behavior of structural elements |
| CO2 | Calculate the loads acting on a RC structure according to code provisions |
| CO3 | Design the RC structures for its behavior |
| CO4 | Design and detail the structural drawings using IS codes |
| CO5 | Apply the structural systems based on the field requirement |
| CO6 | Apply suitable techniques for failure of structures |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | - | 32 | - | - | - | - | 32 |
| CO2 | - | - | - | 32 | - | - | 32 |
| CO3 | 16 | - | - | - | 16 | - | 32 |
| CO4 | - | - | 20 | - | - | - | 20 |
| CO5 | - | - | 16 | - | - | - | 16 |
| CO6 | - | - | - | - | - | - | - |
|  |  |  |  |  |  |  | 132 |

Reg.No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 20CE3004 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # FINITE ELEMENT METHODS IN CIVIL ENGINEERING | # Max. Marks : | # 100 |

| Q. No. | Sub Div. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |
| 1. |  | A simply supported beam carrying a concentrated load W at the center. Analyze the deflection at midspan by using Rayleigh-Ritz method and compare with exact solution | CO1 / A | 16 |
| 2. |  | Determine the stiffness matrix for the constant strain triangular (CST) element shown in fig. The coordinates are given in units of millimeters. Assume plane stress conditions. Take E = 210 GPa, Poisson’s ratio = 0.25 and t = 10 mm | CO4 / E | 16 |
| 3. |  | Develop element stiffness matrix for the rigid frame shown in fig. Also explain how do you proceed further to solve problem. Take E = 200 GN/m2, Io = 4 x 10-6 m4 and A = 4 x 10-3 m2 | CO3 / Ap | 16 |

| 4. | a. | Explain about hexahedron elements. | CO 3 / U | 3 |
| --- | --- | --- | --- | --- |
|  | b. | Develop the shape function for 8 noded hexahedron element. | CO2 / Ap | 13 |
| 5. |  | Explain stiffness matrix formulation for rectangular plate element with 12 Degrees of freedom. | CO5 / E | 16 |
| 6. | a. | Explain the following shell elements. - Flat elements - Curved elements - Solid elements - Degenerated elements | CO3 / U | 8 |
|  | b. | Explain the conforming and non – conforming rectangular plate bending analysis. | CO1 / U | 8 |
| 7. |  | Develop element stiffness matrix for the member of plane frame shown in fig. if it is oriented at angle 30º to the X-axis. Take E = 200GPa, I = 4 x 10-6 m4 and A = 4 x 10-3 m2 | CO5 / Ap | 16 |
| PART – B (1 X 20 = 20 MARKS) (Compulsory Question) |  |  |  |  |
| 8. | a. | Explain mesh generation technique and basic conditions for developing mesh. | CO6 / U | 10 |
|  | b. | Explain different types of meshes. | CO6 / U | 10 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Identify the principles for the development of finite element models |
| CO2 | Develop shape function, strain displacement relation, stiffness matrix and consistent load vector matrix |
| CO3 | Explain the finite element procedure for structural element |
| CO4 | Analyze one, two and three dimensional problems |
| CO5 | Choose appropriate finite element for analysis depending on the nature of problem |
| CO6 | Develop finite element models using suitable software |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 |  | 8 |  | 16 |  |  | 24 |
| CO2 |  |  | 13 |  |  |  | 13 |
| CO3 |  | 11 | 16 |  |  |  | 27 |
| CO4 |  |  |  |  | 16 |  | 16 |
| CO5 |  |  | 16 |  | 16 |  | 32 |
| CO6 |  | 20 |  |  |  |  | 20 |
|  |  |  |  |  |  |  | 132 |

Reg. No. _____________

End Semester Examination – June – 2022

| # Code : | # 20CE3005 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ADVANCED DESIGN OF STEEL STRUCTURES | # Max. Marks : | # 100 |

| Q. No. |  | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |
| 1. |  | Calculate the moment carrying capacity of section ISMB400@61.5kg/m | CO6 / An | 16 |
| 2. |  | Analyze the moment and shear capacity of welded plate girder beam section in a pre-engineered building. Span of the beam is 4.5m. Assume the beam is simply supported. Width of flange – 300mm Thickness of flange – 10mm Depth of web – 600mm Thickness of web –8mm | CO3 / E | 16 |
| 3. |  | A column section ISHB 400@82.2kg/m carries a factored compressive load of 600kN and Bending moment of 40kNm. Design the base plate and its connection. | CO5 / A | 16 |
| 4. |  | Compute the buckling resistance and moment carrying capacity of column subjected to an Axial load of 500kN and Bending moment of 60kNm. Assume the column subjected to uniaxial bending in Z direction. Effective length of the column is 4.5m | CO2 / An | 16 |
| 5. |  | Analyse the truss shown in figure for the following data: Determine the loads (dead, live and wind) on the pitched roof truss for the following data Height of the column - 6m Span of the truss - 15m Height of the truss - 2m Spacing of the truss -4m Spacing b/w purlin - 1.65m c/c Assume the roof covered with GI sheets and the building located in Chennai. | CO3 / A | 16 |

| 6. |  | Design a stiffened welded seat connection to join ISMB 250@37.3kg/m with a column section ISHB 400@82.2kg/m. The beam transmits an end reaction of 600kN. The steel is of grade Fe410 | CO4 / E | 16 |
| --- | --- | --- | --- | --- |
| 7. |  | A three story steel building is located in seismic zone II on hard soil. The framing system of the building is moment-resisting frame without brick masonry infill panels. Find the base shear and its distribution along the height of the building. Column sections: ISHB 450 @872N/m, Beam sections: ISMB 400 @616 N/m Slab: 125mm thick RCC slab on all floors. Size of building 25x25m, Height of each floor - 3m, Live load - 3kN/m2. Assume the relevant data. | CO4 / U | 16 |
| PART – B (1 X 20 = 20 MARKS) (Compulsory Question) |  |  |  |  |
| 8. |  | Two channels of 180mmx80mm section with bent lips are connected with webs to act as beam. The thickness of the plate is 2.5mm and the depth of the lip is 25mm. The beam has an effective span of 6m. Determine the load carrying capacity of beam. Take fy-235N/mm2. | CO3 / An | 20 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Identify appropriate structural steel section |
| CO2 | Explain the behavior of different steel structural elements |
| CO3 | Perform analysis of steel structures |
| CO4 | Design the components of steel structural elements and connections |
| CO5 | Appraise on the quality parameters for steel structures |
| CO6 | Investigate the reasons for failure of a steel structure |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | - | - | - | - | - | - | - |
| CO2 | - | - | - | 16 | - | - | 16 |
| CO3 | - | - | 16 | 20 | 16 | - | 52 |
| CO4 | - | 16 | - | - | 16 | - | 32 |
| CO5 | - | - | 16 | - | - | - | 16 |
| CO6 | - | - | - | 16 | - | - | 16 |
|  |  |  |  |  |  |  | 132 |

Reg. No. _____________

End Semester Examination – June – 2022

| # Code : | # 20CE3006 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # DESIGN OF SUBSTRUCTURES | # Max. Marks : | # 100 |

| Q. No. | Sub Div. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |
| 1. | a. | Discuss the various methods of shallow foundations. | CO5 / U | 8 |
|  | b. | What is punching Shear? Why is it important in the design of foundation? | CO2 / U | 4 |
|  | c. | Discuss on the importance of determining the bearing capacity for foundation design. | CO1 / An | 4 |
| 2. |  | Design a pile under a column transmitting an axial load of 1200 kN. The pile is to be driven to a hard stratum available at a depth of 7 m. Use M20 concrete and Fe 415 steel. | CO6 / Ap | 16 |
| 3. |  | Explain the design principles of well foundation. | CO5 / An | 16 |
| 4. |  | Design the vertical stem of a cantilever retaining wall of height 4m above ground level. The Soil pressure is 150 kN/m2 and the SBC of soil is 200 kN/m2. The angle of internal friction is 25o. Use M20 concrete and Fe415 steel. Assume suitable data wherever necessary. | CO6 / Ap | 16 |
| 5. | a. | Broadly classify tunnel Based on its purpose and type of strata. | CO5 / An | 8 |
|  | b. | Explain the various tunneling methods. | CO4 / U | 8 |
| 6. |  | Determine the dimensions and calculate the B.M and S.F for a combine footing for two columns with size 500 x 500 m, 4 meter apart and each carrying a load of 1200 kN. The width is restricted to 2.0 m. The SBC of soil is 200 kN/m2. Use M20 Concrete and FE415 steel. | CO3 / An | 16 |
| 7. |  | Estimate the moments and draw the B.M diagram for the design of mat foundation for columns spaced at 4 m c/c, with three columns in x and y direction. The pmax and pmin at the ends are 12x104 N/m2 and 10 x 104 N/m2. | CO3 / An | 16 |
| PART – B (1 X 20 = 20 MARKS) (Compulsory Question) |  |  |  |  |
| 8. |  | Explain briefly the various types of coffer dam with figures. | CO5 / An | 20 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Identify the methods of subsoil exploration |
| CO2 | Evaluate the shear strength parameters. |
| CO3 | Determine the load carrying capacity of different foundation types. |
| CO4 | Analyze the concepts of settlement analysis |
| CO5 | Select appropriate foundations type based on available soil conditions. |
| CO6 | Design suitable foundations based on the soil conditions |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 |  |  |  | 4 | - | - | 4 |
| CO2 |  | 4 |  |  |  |  | 4 |
| CO3 |  |  |  | 32 | - | - | 32 |
| CO4 |  | 8 |  |  |  |  | 8 |
| CO5 |  | 8 | 40 |  | - | - | 48 |
| CO6 |  |  | 32 |  | - | - | 32 |

Reg. No. _____________

End Semester Examination – June – 2022

| # Code : | # 20CE3015 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # BRIDGE ENGINEERING | # Max. Marks : | # 100 |

IRC Code Books, Bridge Rule and IS 1343-2012 are permitted

| Q. No. | Sub Div. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |
| 1. | a. | When would you choose RCC T beam bridge? | CO2 / R | 4 |
|  | b. | A reinforced concrete simply supported slab is to be designed for a road bridge having the data given below: Width of carriageway - 7.5m Kerbs - 600mm wide Clear span - 6m Type of loading - IRC Class AA tracked vehicle Materials – M20 grade concrete, Fe415 grade tor steel Design the slab. | CO3 / An | 12 |
| 2. | a. | List all the conditions for adopting Courbon’s method. | CO2 / R | 3 |
|  | b. | An RCC Tee beam and slab deck is to be designed for a major river crossing in a National Highway. The following data is available Clear width of roadway - 7.5m Wearing coat - 100mm No. of main girders - 3 Span (c/c of bearing) - 20m Spacing of cross girder - 5m c/c Loading - IRC Class AA tracked vehicle M20 grade of concrete and Fe 415 grade steel is used Design the deck slab using Pigeud’s charts. | CO3 / An | 13 |
| 3. | a. | Suggest why box girders can be preferred for bridges? | CO2 / U | 2 |
|  | b. | Design a suitable section for the longitudinal girder of a post tensioned prestressed concrete T Beam Bridge for a National Highway crossing to suit the following data: Effective span - 15m Equivalent Live load -20kN/m Adopt M45 grade of concrete with cube strength at transfer as 40 N/mm2. Loss ratio – 0.80 and the 7mm HTS wires initially stretched to 1200MPaneed to be used | CO4 / A | 14 |

| 4. | a. | Can you list the role of stringers in railway steel bridges? | CO2 / R | 4 |
| --- | --- | --- | --- | --- |
|  | b. | The effective span of a through type plate girder railway bridge is 32m. The stringers are spaced 2m between centerlines. 0.60 kN per meter stock rails and 0.45 kN per metre checkrails are provided. Sleepers are spaced at 0.40m from center to center and are of size 2.8 m x 250 mm 250 mm. The weight of timber may be assumed as7.5 kN/m3. The spacing between main girders is 4m. Determine the maximum bending moment and shear force required for the design of the plate girder, if the bridge is to carry standard main lane loading for broad gauge track. | CO3 / An | 12 |
| 5. | a. | Sketch the cross sectional details of a top chord and bottom chord of a steel trussed girder bridge of large span. | CO2 / R | 4 |
|  | b. | The effective span of a through type truss girder highway through two lane bridge is 48m. The reinforced concrete slab is 250mm thick inclusive of wearing coat. The foot paths are provided on either side of the carriage way. The spacing between centre to centre of the truss girder is 12m. Suggest a suitable truss girder for the bridge. Analyse the truss and determine the force in central Top chord member due to dead load and live load. The highway bridge is to carry IRC Class A standard loading. | CO3 / An | 12 |
| 6. | a. | Can you list the different forces acting on a Pier? | CO1 / U | 4 |
|  | b. | Explain how would you check the stability of an Abutment. | CO4 / A | 12 |
| 7. | a. | Can you highlight the difficulties faced in incremental push launching method? | CO1/ U | 4 |
|  | b. | Enumerate the design procedure of well foundations in bridges. | CO4 / A | 12 |
| PART – B (1 X 20 = 20 MARKS) (Compulsory Question) |  |  |  |  |
| 8. |  | Enumerate a case study on a failure or construction of a major bridge. | CO6 / E | 20 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Classify bridges according to loading and site conditions |
| CO2 | Explain the behaviour of different types of bridges |
| CO3 | Analyze different types of bridges |
| CO4 | Design the different components of bridges |
| CO5 | Appraise on the quality investigation of bridge structures |
| CO6 | Investigate the reasons for the failure of bridge structures |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 |  | 5 |  |  |  |  | 5 |
| CO2 | 10 | 5 |  |  |  |  | 15 |
| CO3 |  |  | 30 |  |  |  | 30 |
| CO4 |  |  | 30 |  |  |  | 30 |
| CO5 |  |  |  |  |  |  |  |
| CO6 |  |  |  |  | 20 |  | 20 |
|  |  |  |  |  |  |  | 100 |

Reg. No. _____________

End Semester Examination – June – 2022

| # Code : | # 20CE3016 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # CONDITIONAL ASSESSMENT OF EXISTING # STRUCTURES | # Max. Marks : | # 100 |

ANSWER ALL QUESTIONS (5 X 20 = 100 Marks)

| Q. No. |  | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| 1. |  | Differentiate between proactive and reactive strategies in risk assessment. | CO2 / R | 20 |
| (OR) |  |  |  |  |
| 2. |  | What are the factors to be considered while planning repair in the concrete? Explain in detail. | CO1 / U | 20 |
| 3. |  | How a quality assurance report helps an engineer to assess the structural conditions of the building? Discuss in detail. | CO6 / U | 20 |
| (OR) |  |  |  |  |
| 4. |  | What are the different nondestructive testing methods adopted for conditional assessments? Explain any two methods in detail | CO4 / A | 20 |
| 5. |  | How cracks on concrete can be detected and how it can be sized? Discuss in detail with examples. | CO1 / U | 20 |
| (OR) |  |  |  |  |
| 6. |  | What is the procedure adopted for assessing structural strength of a RCC column? Explain in detail. | CO2 / E | 20 |
| 7. |  | What is the procedure to be followed during the inspection of a heritage masonry structure? Write the procedure of assessment. | CO3 / E | 20 |
| (OR) |  |  |  |  |
| 8. |  | Explain structural health monitoring. | CO2 / R | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | What is the importance of condition assessment survey on existing building and new buildings? Discuss in detail. | CO6 / A | 20 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the procedure for identifying the structure exposed to aggressive environment. |
| CO2 | Apply the guidelines for structural condition assessment of existing buildings |
| CO3 | Identify techniques for evaluating concrete masonry and wood |
| CO4 | Select the destructive and non-destructive techniques to suite the projects |
| CO5 | Interpret and use destructive and nondestructive test results |
| CO6 | Evaluate and report the conditional assessment of existing structure |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | - | 40 | - | - | - | - | 40 |
| CO2 | 40 | - | - | - | 20 | - | 60 |
| CO3 | - | - | - | - | 20 | - | 20 |
| CO4 | - | - | 20 | - | - | - | 20 |
| CO5 | - | - | - | - | - | - | - |
| CO6 | - | 20 | 20 | - | - | - | 40 |
|  |  |  |  |  |  |  | 180 |

Reg. No. _____________

End Semester Examination – June – 2022

| # Code : | # 20CE3019 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # EXPERIMENTAL TECHNIQUES AND INSTRUMENTATION | # Max. Marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. |  | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| 1. |  | List the different type of measurement systems with a detailed description. | CO3 / R | 20 |
| (OR) |  |  |  |  |
| 2. |  | Discuss in detail about the functions of LVDT and its application in element testing. | CO4 / U | 20 |
| 3. |  | What is ground penetrating radar? Explain its principle and applications. | CO4 / R | 20 |
| (OR) |  |  |  |  |
| 4. |  | Write the key factors to be considered while conducting experiment with suitable examples. | CO3 / A | 20 |
| 5. |  | Write short notes on shake table test with neat sketches. | CO4 / R | 20 |
| (OR) |  |  |  |  |
| 6. |  | Discuss the causes and types of experimental errors. | CO1 / U | 20 |
| 7. |  | Explain different types of crack measuring devices and its working procedure. | CO5 / E | 20 |
| (OR) |  |  |  |  |
| 8. |  | List different measurement systems and its application in Structural health monitoring. | CO6 / R | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | Prepare a case study report for conducting the experiment to evaluate load deflection behavior of Steel beam. | CO1 / A | 20 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Implement the principles of measurements for static and dynamic response of structures |
| CO2 | Plan various experiments and the instruments |
| CO3 | Adopt the various measuring devices for various parameters |
| CO4 | Choose the appropriate data recorders and improve data interpretation |
| CO5 | Evaluate the distress in structures |
| CO6 | Analyze the structure by non-destructive testing methods and model analysis |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 20 | - | 20 | - | - | - | 40 |
| CO2 | - | - |  | - | - | - | - |
| CO3 | 20 | - | 20 | - | - | - | 40 |
| CO4 | 40 | 20 | - | - | - | - | 60 |
| CO5 | - | - | - | - | 20 | - | 20 |
| CO6 | 20 | - | - | - | - | - | 20 |
|  |  |  |  |  |  |  | 180 |

Reg. No. _____________

End Semester Examination – June – 2022

| # Code : | # 20CE3020 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # FIRE RESISTANT DESIGN OF STRUCTURES | # Max. Marks : | # 100 |

ANSWER ALL QUESTIONS (5 X 20 = 100 Marks)

| Q. No. |  | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| 1. |  | Discuss the effect of cover and size of structural elements with regard to fire resistance. | CO3 / U | 20 |
| (OR) |  |  |  |  |
| 2. |  | Briefly explain the procedure of fire tests conducted for structural elements as per ISO standards. | CO1 / R | 20 |
| 3. |  | Explain the behaviour of different construction materials subjected to extreme fire conditions. | CO3 / U | 20 |
| (OR) |  |  |  |  |
| 4. |  | List different types of fiber and composites used for repair and rehabilitation of fire affected RCC structures | CO5 / E | 20 |
| 5. |  | Discuss the effect of fire on mechanical properties of concrete and rebar | CO3 / A | 20 |
| (OR) |  |  |  |  |
| 6. |  | Calculate the moment of resistance of the beam subjected to fire for the duration of 90minutes using isotherm method. Span = 8m, Width = 300mm, Depth = 600mm, Density of Concrete = 24kN/m3, Characteristic compressive strength of Concrete = 25MPa, Yield strength of steel = 415MPa, Bar diameter = 16mm, Number of bars = 8 (2 rows), Bottom cover = 25mm | CO6 / An | 20 |
| 7. |  | Explain the details of post fire investigation to be followed in reinforced concrete structures. | CO5 / E | 20 |
| (OR) |  |  |  |  |
| 8. |  | Analyse the spalling behaviour of normal and high strength concrete | CO2 / U | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | How the RCC structural system can be protected by active and passive system to improve the fire resistance? | CO5 / A | 20 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Classify different types of fire |
| CO2 | Identify the modes of failures |
| CO3 | Explain the material characteristics exposed to fire |
| CO4 | Analyse the distribution of temperature in concrete |
| CO5 | Evaluate the strength of fire affected member |
| CO6 | Design the structural elements for fire loading |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 20 | - | - | - | - | - | 20 |
| CO2 | - | 20 | - | - | - | - | 20 |
| CO3 | - | 40 | 20 | - | - | - | 60 |
| CO4 | - | - | - | - | - | - | - |
| CO5 | - | - | 20 | - | 40 | - | 60 |
| CO6 | - | - | - | 20 | - | - | 20 |
|  |  |  |  |  |  |  | 180 |