Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | 09EC224 / 14EC2014 / EC228 / EC245 / 12EC216 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # DIGITAL SIGNAL PROCESSING | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | The input of a DT LTI system is x[n]={1,2,3,4,5} and impulse response h[n]={2,4,5}. Find the length of the output of the DT system. | CO1/R | 1 |
| 2. | Define zero padding. | CO1/R | 1 |
| 3. | Find the number of complex multiplications for direct evaluation of 8-point DFT. | CO1/ R | 1 |
| 4. | Show the basic signal flow graph of DIT-FFT algorithm. | CO1/R | 1 |
| 5. | List any two properties of Butterworth filter. | CO2/R | 1 |
| 6. | Show the Magnitude Response of Type 1 Chebychev filter. | CO2/R | 1 |
| 7. | Spell out the conditions to be satisfied for constant phase delay in linear phase FIR filters. | CO2/R | 1 |
| 8. | Define windowing. | CO2/R | 1 |
| 9. | Find the ones complement of -732 | CO3/R | 1 |
| 10. | Define round off noise. | CO3/R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Compare linear and circular convolutions. | CO1/U | 3 |
| 12. | Find the DFT of the sequence | CO1/R | 3 |
| 13. | Outline one to one mapping and many to one mapping in digital filter design. | CO2/U | 3 |
| 14. | Compare FIR and IIR filters. | CO2/U | 3 |
| 15. | List the effects of product quantization. | CO3/R | 3 |
| 16. | Outline the harvard architecture. | CO3/U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. |  | Find sectioned convolution using overlap add method for the given sequences, and . | CO1/ E | 12 |
| 18. |  | Determine the 8 point DFT of a sequence x[n]=2n using radix-2 DIT FFT algorithm. | CO1/E | 12 |
| 19. |  | Design digital IIR low pass butterworth filter to meet the following specifications: Pass band gain=0.89 Pass band edge frequency=30 Hz Stop band attenuation=0.20 Stop band edge frequency=75 Hz Use Bilinear transformation. Assume sampling frequency is 200 Hz | CO2/C | 12 |
| 20. |  | A LPF has the desired frequency response Find the filter coefficients for N=7 using frequency sampling method. | CO2/R | 12 |
| 21. |  | Explain in detail about finite word length effects in digital filter. | CO3/ E | 12 |
| 22. |  | Design a Chebyshev filter for the following specifications Use impulse invariant technique. | CO2/C | 12 |
| 23. |  | An 8-point sequence is given byx [n] =[1, 2, 3, 4, 4, 3, 2,1]. Outline the 8 point DIF FFT of x (n). Draw the flow graph and tabulate the intermediate stage results. | CO1/U | 12 |
|  |  | Compulsory: |  |  |
| 24. |  | Explain the basic Wiener filter theory and discuss its limitations. | CO3/E | 12 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 11EC203/14EC2003/EC227/EC285/12EC214 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # SIGNALS AND SYSTEMS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Determine the power and RMS value of the signal x(t) = ejat Cosωct | CO1 / E | 1 |
| 2. | Test whether the system represented by y(n)=nx2(n) is linear or not. | CO1/An | 1 |
| 3. | Distinguish between natural response and forced response. | CO1 / An | 1 |
| 4. | Define convolutional Integral. | CO1 / R | 1 |
| 5. | When does aliasing occur? | CO2 / R | 1 |
| 6. | Find the unilateral Laplace transform of signal x(t)=δ(t). | CO2 / R | 1 |
| 7. | Determine the Fourier transform of. | CO3 / E | 1 |
| 8. | Define magnitude response. | CO3 / R | 1 |
| 9. | List any two properties of the region of convergence of X(z). | CO3 / R | 1 |
| 10. | Recall the time shift property of z-transform. | CO3 / R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Identify the even and odd components of the following signal. x(t) = cos t + sin t + cos t sin t | CO1 /A | 3 |
| 12. | Find Convolution sum for the sequences x1(n) = {1,2-1,3) and x2(n) = {1,0,3} | CO1 / R | 3 |
| 13. | Evaluate the Fourier transform of the signal x(t) = e-atu(t) | CO2 / E | 3 |
| 14. | Examine the Nyquist rate of a 10KHz signal to be sampled? | CO2 /An | 3 |
| 15. | Determine the spectral coefficients for x(n) = Cos (πn/4) | CO3 / E | 3 |
| 16. | Find the Z-transform of x(n)=(1/2)n u(-n). | CO3 / R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. |  | Construct the following signals which are derived from x(n) (i) x(3n+4) (ii) [x(n)+x(n+1)] (iii) 2x(n-1) (iv) x(-n+2) (v) 0.5x(n-4) | CO1 / A | 12 |

| 18. | a. | Determine the homogenous solution for the system described by the difference equation y(n) – 1.5y(n-1) + 0.5y(n-2) = x(n) | CO1 / E | 6 |
| --- | --- | --- | --- | --- |
|  | b. | Explain in detail about the steps to perform convolution between two discrete time sequences. | CO1 / U | 6 |
| 19. |  | Using Classical method, solve dy(t)dtif the initial conditions are y(0) = 9/4; = 5; and x(t) = e-3tu(t) | CO1 / A | 12 |
| 20. |  | The input and output of a causal LTI system are related by the differential equation . Using Fourier transform, determine the response of the system if x(t) = te-2tu(t). | CO2 / E | 12 |
| 21. |  | Realize the given systems. i) H(s) = in direct form II. ii)H(s) = in direct form I | CO2 /A | 12 |
| 22. |  | Explain the discrete time processing of continuous time signals with necessary diagrams and equations. | CO2 / U | 12 |
| 23. |  | Using Fourier transform, determine the impulse response and step response of the system characterized by the input output equation yn-34yn-1+18yn-2= x(n) | CO3 / E | 12 |
|  |  | Compulsory: |  |  |
| 24. | a. | Apply partial fraction expansion technique and find the inverse Z transform of . | CO3 / A | 8 |
|  | b. | Determine the z transform of x(n) = anu(n) and find the region of convergence. | CO3 / E | 4 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 12EC236 / 14EC2038 / 19EC2041 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # CELLULAR MOBILE COMPUTING | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | The choice of cell shape is __________. | CO1 / R | 1 |
| 2. | Frequency reuse reduces __________. | CO2 / U | 1 |
| 3. | Cluster is a __________. | CO1 / R | 1 |
| 4. | Cell splitting is used in __________. | CO2 / A | 1 |
| 5. | The data rate of 1G is __________. | CO3 / R | 1 |
| 6. | The capacity can be improved using __________. | CO4 / R | 1 |
| 7. | Aperture area is denoted by __________. | CO5 / U | 1 |
| 8. | 3G uses __________ technology. | CO6 / U | 1 |
| 9. | Isotropic antenna radiates the energy in __________ direction. | CO2 / A | 1 |
| 10. | Useful information is propogated in __________ lobe. | CO3 / R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Construct the cell structure with cluster size N=3. | CO2 / A | 3 |
| 12. | What is acquisition signal in mobile computation? | CO1 / R | 3 |
| 13. | List the components of base transceiver station. | CO2 / R | 3 |
| 14. | What are the advantages of frequency reuse operation? | CO2 / R | 3 |
| 15. | Illustrate the frequency spectrum of wireless communication. | CO1 / U | 3 |
| 16. | Distinguish the difference between wide band and ultra-wide band spectrum. | CO3/ An | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | Explain in detail about frequency reuse technique and frequency planning in mobile computing. | CO3 /U | 8 |
|  | b. | Find the distance D, if the cluster size N=9 and Radius of cell R=6 | CO2 / R | 4 |
| 18. | a. | Outline the source of interference in cellular system. | CO3 / U | 5 |
|  | b. | Simplify the equation for worst case co-channel interference in mobile communication. | CO4 /An | 7 |
| 19. |  | Discuss in detail about the techniques to improve the system capacity in cellular network. | CO5 / U | 12 |
| 20. | a. | Elaborate the concept microcell zones with an appropriate example. | CO4 / C | 8 |
|  | b. | What is the main role of gateways used in cellular network? | CO2 / R | 4 |

| 21. |  | Why adjacent channel interferences occur in cellular system? What is near and far effect? | CO3 / R | 12 |
| --- | --- | --- | --- | --- |
| 22. | a. | What is meant by cross talk? List the types of cross talk? | CO4 / R | 6 |
|  | b. | How to overcome the cross talk effect in real time scenario? | CO2 / R | 6 |
| 23. | a. | Distinguish the difference between various cellular technologies (1G, 2G, 3G, 4G and 5G). | CO5 /An | 7 |
|  | b. | List the security issues in cellular communication. Also suggest some possible solutions for the same. | CO6 / R | 5 |
|  |  | Compulsory: |  |  |
| 24. |  | Elaborate about various access techniques used in mobile computing. | CO4 / C | 12 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 14EC2008 / EC211 / 12EC208 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # LINEAR INTEGRATED CIRCUITS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | What is the first component inside IC741 op amp? | CO1 / R | 1 |
| 2. | What is the gain of voltage follower circuit using op amp? | CO1 / R | 1 |
| 3. | Infer on the following: An ideal differentiator can be obtained from an inverting amplifier by replacing its input resistance by which component? | CO1 / R | 1 |
| 4. | Calculate the input voltage of a precision rectifier with cut in voltage 0.7v and assuming ideal op amp characteristics. | CO1 / U | 1 |
| 5. | Which component replaces the feedback resistor by a common base transistor in an inverting op amp? | CO1 / U | 1 |
| 6. | What is the frequency of oscillation in an opamp monostable multivibrator? | CO1 / R | 1 |
| 7. | What is the type of feedback used in oscillator? | CO1 / R | 1 |
| 8. | List the applications of multiplier. | CO1 / U | 1 |
| 9. | How multiplier is configured as divider? | CO1 / U | 1 |
| 10. | List few open loop applications of op-amps. | CO1 / R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Make use of the concept of summer and determine the amount of current from point A to point B in this circuit: | CO1 / A | 3 |
| 12. | The two input terminals of an op-amp are connected to voltage signals of strengths 745 mV and 740 mV respectively. The gain of the op-amp in differential mode is 5 × 105 and CMRR is 80 dB. Calculate its common mode gain. | CO2 / An | 3 |
| 13. | If vx=Vx sinɷt , vy=Vy sin ɷt, What will be the output of multiplier block? | CO1 / U | 3 |
| 14. | Draw the circuit diagram to generate a triangular wave. | CO1 / R | 3 |
| 15. | Draw sine to square wave generator diagram using basic comparator circuit, also draw its input and output waveform. | CO1 / U | 3 |
| 16. | In the schmitt trigger R2=100Ω, R1=50k Ω Vref=0V,vi=1Vpp,Vsat=±14V.Determine VUT and VLT | CO1 / A | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | Show with the help of circuit diagram an op-amp that can be used as, i) Inverting Amplifier ii) Non-Inverting Amplifier | CO1 / R | 6 |
|  | b. | Design an Op-amp differentiator that will differentiate an input signal with fmax= 500Hz. | CO1 / R | 6 |
| 18. |  | Give the functional description of a 555 timer and also explain how it works as an astable multivibrator. Derive the value of T | CO2 / U | 12 |
| 19. |  | With necessary circuit diagrams prove that the operational amplifier can be used as a Integrator. State the problems associated with the basic integrator and design a practical integrator to overcome the drawbacks. | CO1 / U | 12 |
| 20. | a. | Explain the functional modules of a Phase locked loops with the necessary diagrams. | CO1 / R | 10 |
|  | b. | Draw the circuit diagram for first order wide band pass filter. | CO1 / U | 2 |
| 21. | a. | Design a wide band reject filter having fh=400Hz and fl=2KHz having pass band gain as 2. | CO1 / U | 7 |
|  | b. | Illustrate the mechanism of IC 723 voltage regulator with a neat functional diagram. | CO2 / A | 5 |
| 22. | a. | Derive the expression for time constant for a monostable multivibrator with necessary explanation and timing diagrams. | CO1 / U | 8 |
|  | b. | Write a short note on regenerative comparator. | CO1 / R | 4 |
| 23. |  | With a neat circuit diagram explain the methods used in weighted resistor, R-2R digital to analog converter. | CO1 / U | 12 |
|  |  | Compulsory: |  |  |
| 24. |  | With neat diagrams explain the basic planar process involved in chip fabrication. | CO3 / U | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Define and infer on IC 741 and its applications. |
| CO2 | Outline and discover on IC 555 and its applications. |
| CO3 | Examine on IC fabrication and aquire knowledge on its various methods. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 35 | 51 | 6 |  | - | - | 92 |
| CO2 |  | 12 | 5 | 3 |  |  | 20 |
| CO3 |  | 12 |  |  | - | - | 12 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 16EC3001 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # BIOLOGICAL EFFECTS OF MICROWAVE | # Max. Marks : | # 100 |

| Q. No. | Sub Div. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |
| 1. | a. | What are the different types of radiations? Explain in detail about ionization radiations. | CO1 / R | 12 |
|  | b. | Outline the frequency ranges in EM spectrum. | CO 1/ U | 4 |
| 2. | a. | Describe the effects of microwave on biological tissues. | CO2 /R | 8 |
|  | b. | Discuss the working principle of reflex klystron with necessary diagram. | CO 1/ C | 8 |
| 3. | a. | Criticize the radiation effects on Eye and draw the internal structure. | CO 2/ E | 8 |
|  | b. | Inspect the health status of Neurophysiological stress when subject to microwave radiation. | CO 2/ A | 8 |
| 4. | a. | Interpret the microwave radiation effects on skin. | CO2 / U | 6 |
|  | b. | Elaborate the functions of cardiovascular regulation system and its effects on radiation with necessary diagrams. | CO2 / C | 10 |
| 5. |  | Appraise the principle of Magnetron with relevant structure. | CO1 / E | 16 |
| 6. | a. | Infer the principle of terrestrial radiation. | CO1 / U | 8 |
|  | b. | Interpret the properties of low frequency radiation. | CO 1 / U | 8 |
| 7. | a. | Analyze how radiated power is absorbed by the human body with the help of validation test using flat phantom. | CO2 / An | 10 |
|  | b. | Appraise the radiation effects on living matter. | CO1 / E | 6 |
| PART – B (1 X 20 = 20 MARKS) (Compulsory Question) |  |  |  |  |
| 8. | a. | Summarize the RF and microwave radiation exposure standards. | CO 3 / U | 10 |
|  | b. | Discuss about Control and prevention of health Hazards. | CO3 / C | 10 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 17EC2015 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # LINEAR INTEGRATED CIRCUITS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | What is the first stage of an op-amp? | CO1R | 1 |
| 2. | For an ideal op-amp, the input impedance is--------- | CO1 / R | 1 |
| 3. | What do you mean by precision diode? | CO1 / R | 1 |
| 4. | Name some of the non linear applications of op-amp. | CO1 / U | 1 |
| 5. | List out commonly used active filters. | CO2 / R | 1 |
| 6. | Define PLL. | CO6 / U | 1 |
| 7. | How many comparators required for 3 bit flash type ADC? | CO3 / R | 1 |
| 8. | Which filter performs exactly the opposite of Band pass filter? | CO2 / U | 1 |
| 9. | If R1 = R2, β = 0.5, Vsat >> VD, then time period of pulse width of monostable multivibrator is______ | CO5 / U | 1 |
| 10. | The lock in range in PLL is ------- the capture range | CO6 / R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Define input offset voltage. | CO1 / U | 3 |
| 12. | Specify the condition for good differentiation. | CO1/ U | 3 |
| 13. | Show the circuit and waveforms of precision diode as half wave rectifier. | CO2 / R | 3 |
| 14. | Compare Schmitt trigger and comparator. | CO5 / U | 3 |
| 15. | Staten Barkhausen criterion. | CO5 / R | 3 |
| 16. | What is the disadvantage of binary weighted type DAC? | CO3 / U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | Show with the help of circuit diagram an op-amp that can be used as, i) Inverting Amplifier ii) Non-Inverting Amplifier | CO1 / U | 12 |
| 18. | a. | Construct the Lossy integrator with necessary equations and also explain its frequency response with neat diagram. | CO1 / A | 9 |
|  | b. | List the alternate name for the following circuits i) Schmitt Trigger ii) Clipper iii)Clamper | CO1 / R | 3 |

| 19. | a. | Design a second order Butterworth high pass filter having lower cut off frequency 1kHz. | CO2/ An | 8 |
| --- | --- | --- | --- | --- |
|  | b. | List the applications of PLL. | CO2 / R | 4 |
| 20. | a. | Explain the operation of R-2R ladder digital to analog converter with neat equivalent circuit diagram. | CO3 / A | 6 |
|  | b. | Describe the working of successive approximation type analog to digital converter with neat block diagram. | CO3 / A | 6 |
| 21. | a. | Design a RC phase shift oscillator to oscillate at a frequency of 200Hz and explain its operation. | CO5 / U | 7 |
|  | b. | Discuss the working of triangular wave generator and write the expression for frequency of oscillations. | CO5 / R | 5 |
| 22. | a. | Write short notes on astable multivibrator using op- amp. | CO 5/ A | 6 |
|  | b. | Show the input and output waveforms of clamper circuit using op-amp for the given specification. Vii=5V, Vm=5 and Vref = -2V. | CO5/ An | 6 |
| 23. | a. | Explain why active load used in Differential amplifier with necessary expressions. | CO1 / R | 7 |
|  | b. | Write short notes on the following. a)Slew rate b)Thermal drift | CO1 / U | 5 |
|  |  | Compulsory: |  |  |
| 24. | a. | Outline the mechanism of 555 IC timer for generating accurate time delay with neat functional block diagram. | CO4 / A | 8 |
|  | b. | Discuss the limitations of linear voltage regulators. | CO6 /R | 4 |

Reg. No. _____________

End Semester Examination – June – 2022

| # Code : | # 17EC3077 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # COMPUTER BASED MEDICAL INSTRUMENTATION | # Max. Marks : | # 100 |

ANSWER ALL QUESTIONS (5 X 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| 1. |  | Elaborate the major components of a PC motherboard with relevant diagram. | CO1 / C | 20 |
| (OR) |  |  |  |  |
| 2. | a. | Explain in detail the role of peripheral control chips in mother board of a PC. | CO1 / E | 15 |
|  | b. | Assess the functions of system control chips. | CO1 / E | 05 |
| 3. |  | Construct the architecture of 80X86 microprocessor. | CO2 / A | 20 |
| (OR) |  |  |  |  |
| 4. | a. | Analyse the Programming model of 16-bit microprocessor. | CO2 / An | 10 |
|  | b. | Classify the different operating modes of microprocessors. | CO2 / An | 10 |
| 5. |  | Compile short notes on following - Flash memory. - Serial and parallel ports. | CO5 / C | 20 |
| (OR) |  |  |  |  |
| 6. | a. | Explain various types of Random Access Memory. | CO5 / E | 10 |
|  | b. | Categorize the ROMs based on the characteristics. | CO5 / An | 10 |
| 7. | a. | Summarize the steps and processes involved in computer -aided diagnosis and therapy based on biomedical signal analysis. | CO6 / U | 15 |
|  | b. | Examine the role of DSP in Medical Applications. | CO4 / An | 05 |
| (OR) |  |  |  |  |
| 8. |  | Demonstrate the PC based data acquisition system. | CO3 / U | 20 |
|  |  | Compulsory: |  |  |
| 9. | a. | Identify an SoC based approach for biomedical signal processing. | CO4 / A | 10 |
|  | b. | Illustrate the architecture of a healthcare SoC for wearable device. | CO3 / U | 10 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand hardware behind data acquisition. |
| CO2 | Analyze the scope of virtual reality in health care. |
| CO3 | Develop an insight knowledge about the biometrics. |
| CO4 | Apply the features of network security. |
| CO5 | Evaluate computer based medical equipment. |
| CO6 | Compare and contrast with the traditional techniques. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | - | - | - | - | 20 | 20 | 40 |
| CO2 | - | - | 20 | 20 | - | - | 40 |
| CO3 | - | 30 | - | - | - | - | 30 |
| CO4 | - | - | 10 | 05 | - | - | 15 |
| CO5 | - | - | - | 10 | 10 | 20 | 40 |
| CO6 | - | 15 | - | - | - | - | 15 |
|  |  |  |  |  |  |  | 180 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18EC2003 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | DIGITAL SYSTEM DESIGN | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | Which number system has base 16? | CO1 | R | 1 |
| 2. | What is the decimal equivalent of the following hexadecimal number (1E.43) 16? | CO1 | R | 1 |
| 3. | How many bits are needed to store one BCD digit? | CO1 | R | 1 |
| 4. | Convert (312)8 into decimal. | CO1 | U | 1 |
| 5. | Which logic gates are known as universal gates? | CO2 | R | 1 |
| 6. | 2's complement of 1011011 is __________. | CO2 | U | 1 |
| 7. | How many select lines 4X1 mux has? | CO3 | R | 1 |
| 8. | How many inputs 3:8 Decoder has? | CO3 | R | 1 |
| 9. | What is race around condition? | CO4 | R | 1 |
| 10. | What is TTL? | CO5 | R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Minimize the following Boolean expression using K-map − F(A,B,C)=A′BC+A′BC′+AB′C′+AB′C | CO2 | C | 3 |
| 12. | Design a 3:8 Decoder using logic gates. | CO3 | C | 3 |
| 13. | Design a single bit full adder with sum and carry. | CO3 | C | 3 |
| 14. | Construct a JK flip flop. | CO4 | C | 3 |
| 15. | Develop a master slave flip flop. | CO4 | C | 3 |
| 16. | What is FPGA? | CO6 | R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | a. | Give the 8’s compliment of (562)8 ________________. Give the equivalent Excess 3 code for 7 __________. Give the equivalent BCD code for 17 __________. | CO1 | U | 6 |
|  | b. | Convert (250.5)10 to base 4. | CO1 | U | 6 |
| 18. | a. | Simplify the following using K map F (a,b,c,d) = ∑ m (0,2,6,8,10) + d (4,14). | CO2 | An | 6 |
|  | b. | Simplify the given Boolean function using k-map F (a,b,c,d) = Σ (11,13,14,15) | CO2 | An | 6 |

| 19. | a. | Design a 1 to 4 DeMUX circuit. | CO2 | C | 4 |
| --- | --- | --- | --- | --- | --- |
|  | b. | Design a look ahead carry generator circuit with neat diagram. | CO3 | C | 8 |
| 20. | a. | Design a full adder circuit using two half adder circuits (with truth table). | CO3 | C | 6 |
|  | b. | Construct a 3:8 Decoder using two 2:4 Decoder and one NOT gate. | CO3 | C | 6 |
| 21. | a. | Design a BCD to Excess3 code converter circuit. | CO3 | C | 6 |
|  | b | Design a 16:1 MUX using 4:1 MUX. | CO3 | C | 6 |
| 22. | a. | Design a MOD 6 up counter using J-K flip flops. | CO4 | C | 6 |
|  | b. | With sequence table and diagram explain Johnson counter. | CO4 | U | 6 |
| 23. |  | Implement the following two Boolean functions with a PLA. F1= Σm (0,1,2,4) F2= Σm (0,5,6,7) | CO5 | E | 12 |
|  |  | Compulsory: |  |  |  |
| 24. |  | Write Verilog code for single bit full adder and 4X1 MUX. | CO6 | U | 12 |

Reg. No. _____________

End Semester Examination – May / June – 2022

| # Code : | # 18EC2003 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | DIGITAL SYSTEM DESIGN | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
|  | PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Which number system has base 8? | CO1 | R | 1 |
| 2. | What is the decimal equivalent of the following hexadecimal number (1D.53)16? | CO1 | R | 1 |
| 3. | How many bits are needed to store one BCD digit? | CO1 | R | 1 |
| 4. | Convert (216)8 into decimal. | CO1 | U | 1 |
| 5. | Which logic gates are known as universal gates? | CO2 | R | 1 |
| 6. | 2's complement of 1101101 is ______ | CO2 | U | 1 |
| 7. | How many select lines 8X1 mux has? | CO3 | R | 1 |
| 8. | How many inputs 2:4 Decoder has? | CO3 | R | 1 |
| 9. | What is race around condition? | CO4 | R | 1 |
| 10. | What is CMOS? | CO5 | R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Minimize the following Boolean expression using K-map − F(A,B,C)=A′BC+A′BC′+AB′C′+AB′C | CO2 | C | 3 |
| 12. | Design a 2:4 Decoder using logic gates | CO3 | C | 3 |
| 13. | Design a full adder using half adders. | CO3 | C | 3 |
| 14. | Construct a RS flip flop | CO4 | C | 3 |
| 15. | Develop a JK flip flop | CO4 | C | 3 |
| 16. | What is VHDL? | CO6 | R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | a. | Give the 8’s compliment of (486)8 ________________ Give the equivalent Excess 3 code for 6 -------------------- Give the equivalent BCD code for 18 -------------------- | CO1 | U | 6 |
|  | b. | Convert (145.5)10 to base 5. | CO1 | U | 6 |
| 18. | a. | Simplify the following using K map F (a,b,c,d) = ∑ m (0,2,6,8,10) + d (4,14). | CO2 | An | 6 |
|  | b. | Simplify the given Boolean function using k-map F (a,b,c,d) = Σ (11,13,14,15) | CO2 | An | 6 |

| 19. | a. | Design a 4to 1 MUX circuit. | CO2 | C | 4 |
| --- | --- | --- | --- | --- | --- |
|  | b. | Design a 4-bit parallel adder circuit with neat diagram. | CO3 | C | 8 |
| 20. | a. | Design a single bit full subtractor circuit (with truth table). | CO3 | C | 6 |
|  | b. | Construct a 4:16 Decoder using 2:4 Decoder. | CO3 | C | 6 |
| 21. | a. | Design a BCD to Excess3 code converter circuit. | CO3 | C | 6 |
|  | b | Design a 4:1 MUX using 2:1 MUX | CO3 | C | 6 |
| 22. | a. | Design a MOD 5 up counter using J-K flipflops. | CO4 | C | 6 |
|  | b. | With sequence table and diagram explain Ring counter. | CO4 | U | 6 |
| 23. |  | Implement the following two Boolean functions with a PLA. F1= Σm(0,1,2,4) F2= Σm(0,5,6,7) | CO5 | E | 12 |
|  |  | Compulsory: |  |  |  |
| 24. |  | Write Verilog code for Half Adder and 8X1 MUX. | CO6 | U | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | illustrate the basic postulates of Boolean algebra and the operation of logic gates |
| CO2 | choose an optimal method for simplification of Boolean expressions |
| CO3 | design and distinguish various combinational logic circuits. |
| CO4 | design and compare various sequential logic circuits |
| CO5 | illustrate different logic families; classify memory devices and identify methods for implementation of logic circuits |
| CO6 | design simple logic circuits using HDL codes. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 3 | 13 | - | - | - | - | 16 |
| CO2 | 1 | 1 | - | 12 | - | 7 | 21 |
| CO3 | 2 | - | - | - | - | 38 | 40 |
| CO4 | 1 | 6 | - | - | - | 12 | 19 |
| CO5 | 1 | - | - | - | 12 | - | 13 |
| CO6 | 3 | 12 | - | - | - | - | 15 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18EC2006 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ANALOG AND DIGITAL COMMUNICATION | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | Identify the AM type if modulation index ma> 1. | CO1 | A | 1 |
| 2. | Find the modulation index of FM signal with modulation frequency 10KHz and peak frequency deviation 30KHz. | CO1 | R | 1 |
| 3. | What is the power spectral density of white noise? | CO2 | R | 1 |
| 4. | Recall an example of random process in communication. | CO2 | R | 1 |
| 5. | What is Nyquist rate? | CO3 | R | 1 |
| 6. | Name the circuit used to generate flat top samples. | CO3 | R | 1 |
| 7. | Which linear filter provides maximum SNR. | CO4 | R | 1 |
| 8. | Define bit error rate. | CO4 | R | 1 |
| 9. | Show the ASK waveform for binary data stream 010011. | CO5 | R | 1 |
| 10. | Name the modulation scheme which avoids sudden phase change. | CO5 | R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Determine the approximate bandwidth of the FM signal if a 20 MHz carrier is frequency modulated by a sinusoidal signal such that peak frequency deviation is 50KHz and the frequency of the modulating sinusoid is 1 KHz. | CO1 | E | 3 |
| 12. | Explain thermal noise in electronic devices. | CO2 | U | 3 |
| 13. | Explain sampling theorem. | CO3 | U | 3 |
| 14. | Examine the reason for ISI and draw the interpretation of eye pattern. | CO4 | An | 3 |
| 15. | List the categories in digital communication tradeoffs. | CO5 | R | 3 |
| 16. | What is carrier recovery in digital modulation? | CO6 | R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | a. | Formulate the mathematical equation of Amplitude Modulation, draw the spectrum of AM and give the equation for bandwidth. | CO1 | C | 8 |
|  | b. | Interpret the power relation of AM. | CO1 | E | 4 |
| 18. |  | Formulate the output signal to noise ratio of SSB-SC system. | CO2 | C | 12 |

| 19. |  | Discuss about the noises in Delta modulation and describe the DM transmitter and receiver. | CO3 | C | 12 |
| --- | --- | --- | --- | --- | --- |
| 20. |  | Illustrate the following line coding schemes for the given data stream: Data stream: 01001110 - NRZ-L ii) NRZ-I iii) Manchester encoding - Differential Manchester encoding - Bipolar AMI encoding - Bipolar pseudo ternary encoding | CO4 | U | 12 |
| 21. |  | Explain minimum shift keying with waveforms for the following data stream 10110101. | CO5 | U | 12 |
| 22. |  | Formulate the expression for amplitude modulation using square law modulator. | CO1 | C | 12 |
| 23. |  | Explain the working of pulse code modulator with block diagram. | CO3 | U | 12 |
|  |  | Compulsory: |  |  |  |
| 24. |  | Elaborate equalization techniques. | CO6 | C | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Distinguish different analog modulation schemes for their efficiency and bandwidth |
| CO2 | Predict the behavior of a communication system in presence of noise |
| CO3 | Investigate pulsed modulation system and analyze their system performance |
| CO4 | Recognize various optimal detection schemes |
| CO5 | Analyze different digital modulation schemes and can compute the bit error performance |
| CO6 | Relate different digital demodulation techniques |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 1 | - | 1 | - | 7 | 20 | 29 |
| CO2 | 2 | 3 | - | - | - | 12 | 17 |
| CO3 | 2 | 15 | - | - | - | 12 | 29 |
| CO4 | 2 | 12 | - | 3 | - | - | 17 |
| CO5 | 5 | 12 | - | - | - | - | 17 |
| CO6 | 3 | - | - | - | - | 12 | 15 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18EC2008 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ANALOG CIRCUITS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | Recall the ripple factor of Half Wave Rectifier. | CO1 | R | 1 |
| 2. | Identify a circuit that maintains a constant DC voltage for variation in input line voltage or in the load. | CO1 | A | 1 |
| 3. | Which biasing method has good stability? | CO2 | R | 1 |
| 4. | Which configuration of BJT is used as emitter follower? | CO2 | R | 1 |
| 5. | Name the phase-shift between input and output voltages of CE amplifier. | CO3 | R | 1 |
| 6. | Make use of the value β =100, to find the stability factor of fixed bias circuit. | CO3 | A | 1 |
| 7. | How will you analyze the performance of high frequency transistor model? | CO4 | R | 1 |
| 8. | Which power amplifier removes the problem of cross over distortion? | CO4 | R | 1 |
| 9. | Determine the value of CMRR, if ADM = 3500 and ACM = 0.35. | CO5 | E | 1 |
| 10. | What type of feedback is used in oscillator? | CO5 | R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Show the output of the clipper circuit shown for a sinusoidal input of 10V peak to peak amplitude. The reference voltage, VR = -4V. Assume that the diode is an ideal diode. | CO1 | U | 3 |
| 12. | Compare any three characteristics of Common Emitter, Common Collector and Common Base configuration. | CO2 | AN | 3 |
| 13. | Determine the value of collector current IC in CE amplifier if IB=2mA and β =100. | CO3 | E | 3 |
| 14. | Outline the Structure of Cascode Amplifier. | CO4 | U | 3 |
| 15. | Determine the Input impedance with feedback for voltage series feedback amplifier having A = -100, Zi = 10kΩ, Zo = 20kΩ for feedback of β = -0.4 | CO5 | E | 3 |
| 16. | Determine the frequency of oscillator for RC phase shift oscillator if R=68KΩ and C=2.4nF. | CO6 | E | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 17. |  | Explain the working principle of full wave rectifier with a neat circuit diagram and waveforms. Also mention its efficiency and ripple factor. |  | CO1 |  | U | 12 |
| 18. | a. | Determine the value of IBQ, ICQ, and VCEQ for the given circuit. |  | CO2 |  | E | 8 |
|  | b. | Construct the thevenin’s equivalent circuit of voltage amplifier. |  | CO2 |  | A | 4 |
| 19. |  | Analyze the low frequency re model of CE amplifier and determine the input impedance, output impedance, voltage gain and current gain. |  | CO3 |  | AN | 12 |
| 20. |  | Discuss the principle and working of Class A amplifier with relevant circuit diagram and determine the efficiency. |  | CO4 |  | C | 12 |
| 21. |  | Build the voltage series feedback system and explain its effect on gain, input impedance and output impedance. |  | CO5 |  | A | 12 |
| 22. | a. | Construct the single stage CE amplifier with frequency response and infer the reason for reduction in gain at low frequency and high frequency. |  | CO3 |  | A | 8 |
|  | b. | Draw the circuit diagram of Voltage Divider Bias of FET. |  | CO2 |  | R | 4 |
| 23. |  | Examine the current series feedback system and explain its effect on gain, input impedance and output impedance. |  | CO5 |  | AN | 12 |
|  |  | Compulsory: |  |  |  |  |  |
| 24. | a. | Discuss the working principle of Hartley Oscillator with a neat circuit diagram. | CO6 |  | C |  | 8 |
|  | b. | Infer the importance of Tuned amplifier. | CO6 |  | U |  | 4 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Summarize the application of diodes. |
| CO2 | Classify the characteristics of BJT and JFET amplifiers. |
| CO3 | Design and construct various amplifier circuits. |
| CO4 | Describe the function of power amplifier. |
| CO5 | Construct the differential amplifier for a given specification. |
| CO6 | Identify sinusoidal and non-sinusoidal oscillators. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 1 | 15 | 1 | - | - | - | 17 |
| CO2 | 6 | - | 4 | 3 | 8 | - | 21 |
| CO3 | 1 | - | 9 | 12 | 3 | - | 25 |
| CO4 | 2 | 3 | - | - | - | 12 | 17 |
| CO5 | 1 | - | 12 | 12 | 4 | - | 29 |
| CO6 | - | 4 | - | - | 3 | 8 | 15 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18EC2010 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # MICROCONTROLLERS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Mention the size of stack pointer in 8051 microcontroller. | CO1 / R | 1 |
| 2. | What will be the final contents of the A register after executing all the instructions? MOV A, #0A5H RR A RR A | CO2 / R | 1 |
| 3. | What is the value of SP when the 8051 is reset? | CO1 / R | 1 |
| 4. | Specify the signal that enables reading from external ROM. | CO3 / R | 1 |
| 5. | Find the time taken by the 8051 microcontroller to execute an instruction ADD A, R1 if the crystal frequency is 12 MHz? | CO2 / R | 1 |
| 6. | Which register of PIC serves a function similar to that of accumulator? | CO4 / R | 1 |
| 7. | Justify the need of prescalar in PIC. | CO4 / E | 1 |
| 8. | How many external interrupts are available in PIC 18 series? | CO4 / R | 1 |
| 9. | What is meant by duty cycle? | CO4 / R | 1 |
| 10. | Identify the bit which indicates the status of ADC conversion. | CO6 / A | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Predict the content of Accumulator after the execution of the following instruction SETB C MOV A, #AA RRC | CO1 / C | 3 |
| 12. | Develop an ALP to copy the value 55H into RAM locations 70H and 71H using register indirect addressing mode of 8051 microcontroller. | CO2 / A | 3 |
| 13. | List any three addressing modes of PIC18 microcontroller with examples. | CO4 / R | 3 |
| 14. | What is meant by watchdog timer and Brownout reset in PIC? | CO4 / R | 3 |
| 15. | Show the block diagram of PIC18 Timers. | CO4 / R | 3 |
| 16. | List the data types of Embedded C programming. | CO5 / R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | Discuss the features and functions of 8051 microcontroller with a neat block diagram. | CO1 / C | 12 |
| 18. | a. | Explain the serial communication modes of 8051 microcontroller with necessary diagrams. | CO1 / U | 12 |

| 19. | a. | Show the architecture of PIC18 microcontroller and explain each block in detail. | CO4 / R | 12 |
| --- | --- | --- | --- | --- |
| 20. | a. | Explain the following module of PIC18 Microcontroller - SPI - I2C bus | CO4 / R | 12 |
| 21. | a. | Explain the operators, Expressions and control statements of Embedded C programming with examples. | CO5 / R | 12 |
| 22. | a. | Show the circuit diagram of 8051 having 8Kb EPROM and an 8Kb RAM ICs. The starting address of EPROM is 0000H and RAM is 4000H | CO3 / R | 8 |
|  | b. | Develop an ALP to add two 16 bit numbers. The numbers are 3CE7H and 3B8DH. Store the lower byte in 4500H and higher Byte in 4501H | CO2 / A | 4 |
| 23. | a. | Discuss in detail the I/O ports of 8051 microcontroller with necessary diagrams. | CO3 / C | 12 |
|  |  | Compulsory: |  |  |
| 24. | a. | Illustrate and explain the interfacing of stepper motor with microcontroller with necessary diagrams. | CO6 / U | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Describe the architecture of 8051 microcontroller. |
| CO2 | Discus 8051 assembly language programs for the given applications. |
| CO3 | Illustrate the memory and I/O interfacing concepts for any microcontroller design. |
| CO4 | Illustrate the architectures of PIC microcontroller. |
| CO5 | Develop Microcontrollers based systems using C. |
| CO6 | Select the Microcontroller with proper specifications for various applications |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 2 | 12 | - | - | - | 15 | 29 |
| CO2 | 2 | - | 7 | - | - | - | 9 |
| CO3 | 9 | - | - | - | - | 12 | 21 |
| CO4 | 36 | - | - | - | 1 | - | 37 |
| CO5 | 15 | - | - | - | - | - | 15 |
| CO6 | - | 12 | 1 | - | - | - | 13 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18EC2012 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # LINEAR INTEGRATED CIRCUITS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
|  | PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | What is the gain of non-inverting amplifier if Rf=Ri? | CO2 | R | 1 |
| 2. | Identify the open loop gain of Operational Amplifier. | CO1 | R | 1 |
| 3. | Infer the output of multiplier if Vx= 4v and Vy=2v. | CO3 | U | 1 |
| 4. | Predict the output of inverting comparator if Vin = 10v and Vref = 12v. | CO3 | C | 1 |
| 5. | List the conditions for sustained oscillation (Barkhausen Criteria). | CO3 | R | 1 |
| 6. | Write the frequency of oscillation of Wein bridge oscillator. | CO3 | R | 1 |
| 7. | Infer the filter used to reject a single frequency. | CO4 | U | 1 |
| 8. | Inspect the gain of first order LPF when input frequency is greater than cutoff frequency. | CO4 | An | 1 |
| 9. | Mention the frequency of oscillation of Astable multivibrator using 555. | CO5 | R | 1 |
| 10. | Name the alternate name for Voltage Controlled Oscillator (VCO). | CO6 | R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Find the output voltage of the following circuit. | CO2 | R | 3 |
| 12. | Draw the circuit diagram of three input inverting summer. | CO3 | U | 3 |
| 13. | Build the circuit for generating triangular waveform. | CO3 | A | 3 |
| 14. | Determine the quality factor, if fl = 2KHz and fh = 6KHz and name the type of band pass filter. | CO4 | E | 3 |
| 15. | Draw the schematic diagram of Schmitt trigger using 555 IC. | CO5 | U | 3 |
| 16. | Recall the circuit of weighted resistor digital to analog convertor. | CO6 | R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 17. | a. | Discuss the working of inverting and non-inverting amplifier and derive the voltage gain. | CO2 | C |  | 8 |  |
|  | b. | List the DC characteristics of Op-amp | CO1 | R |  | 4 |  |
| 18. | a. | Build a circuit for differential amplifier and derive the expression for output voltage. | CO2 | A |  | 8 |  |
|  | b. | Illustrate the circuit diagram for log amplifier. | CO2 | U |  | 4 |  |
| 19. |  | Explain the working principle of Astable Multivibrator using Op-amp and derive the frequency of oscillation. | CO3 | U |  | 12 |  |
| 20. | a. | Estimate the output of first order low pass filter with necessary diagrams and expressions. | CO4 | C |  | 10 |  |
|  | b. | Infer the schematic of half wave precision rectifier. | CO3 | U |  | 2 |  |
| 21. |  | Make use of the circuit diagram, explain working principle of Monostable multivibrator using 555 timer with necessary equations and waveforms. | CO5 | A |  | 12 |  |
| 22. | a. | Demonstrate a circuit that uses the mathematical differentiation using op-amp and outline the input and output wave forms. | CO3 | U |  | 8 |  |
|  | b. | Design a first order HPF with a cutoff frequency of 1KHz and gain of 10. (Assume C=0.1µF and R1=10KΩ) | CO4 | E |  | 4 |  |
| 23. | a. | Name the internal blocks of Operational amplifier. | CO1 | R |  | 3 |  |
|  | b. | Analyze the working principle of Successive approximation ADC with an example. Outline the necessary diagram and waveforms. | CO6 | An |  | 9 |  |
|  |  | Compulsory: |  |  |  |  |  |
| 24. |  | Explain in detail the fabrication of ICs using silicon planar technology with necessary diagrams. | CO6 |  | E |  | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the fundamentals of OP-AMP and its characteristics |
| CO2 | Use OP-AMP to design circuits such as Amplifiers, differentiator and Integrator |
| CO3 | Infer the significance of OP-AMP in Multivibrators and Oscillators. |
| CO4 | Design filters using OP-AMP. |
| CO5 | Explore the usefulness of IC555 timer and Phase Locked Loop |
| CO6 | Design ADC, DAC and understand the IC fabrication. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 8 | - | - | - | - | - | 8 |
| CO2 | 4 | 4 | 8 | - | - | 8 | 24 |
| CO3 | 2 | 26 | 3 | - | - | 1 | 32 |
| CO4 | - | 1 | - | 1 | 7 | 10 | 19 |
| CO5 | 1 | 3 | 12 | - | - | - | 16 |
| CO6 | 4 | - | - | 9 | 12 | - | 25 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18EC2015/17EC2010 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # DIGITAL SIGNAL PROCESSING | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | If the input to a DT LTI system has N1 samples and impulse response has N2 samples Find the length of the output of the system. | CO1/R | 1 |
| 2. | Define zero padding. | CO1/R | 1 |
| 3. | Find the number of complex multiplications for direct evaluation of 8-point DFT. | CO2/ R | 1 |
| 4. | Show the basic signal flow graph of DIT-FFT algorithm. | CO2/R | 1 |
| 5. | List the methods to transform an analog filter into a IIR filter. | CO3/R | 1 |
| 6. | Show the Magnitude Response of Type 1 Chebychev filter. | CO3/R | 1 |
| 7. | Spell out the conditions to be satisfied for constant phase delay in linear phase FIR filters. | CO4/R | 1 |
| 8. | Define windowing. | CO4/R | 1 |
| 9. | Find the ones complement of -78 | CO5/R | 1 |
| 10. | Define dead band in limit cycle oscillations. | CO5/R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Compare linear and circular convolutions. | CO1/U | 3 |
| 12. | Find the computation efficiency of 1024 point FFT over 1024 point DFT. | CO2/R | 3 |
| 13. | Outline one to one mapping and many to one mapping in digital filter design. | CO3/U | 3 |
| 14. | Compare FIR and IIR filters. | CO4/U | 3 |
| 15. | List the effects of input quantization. | CO5/R | 3 |
| 16. | Outline the harvard architecture. | CO6/U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | Determine the output of linear FIR filter whose impulse response is h(n)={1,2,3} and input signal is x(n)={2,4,1,2,2,3,3,4,5,6,7,8,9} using overlap add method. | CO 1/ E | 12 |
| 18. | a. | Determine the 8 point DFT of a sequence x[n]=2n using radix-2 DIT FFT algorithm. | CO 2 / E | 12 |

| 19. | a. | Design a Butterworth digital IIR filter using the impulse variance method for the following specifications: Assume T = 1 sec. | CO 3 /C | 12 |
| --- | --- | --- | --- | --- |
| 20. | a. | A LPF has the desired frequency response. Find the filter coefficients for N=7 using frequency sampling method. | CO 4/ R | 12 |
| 21. | a. | Explain in detail about finite word length effects in digital filter. | CO 5 / E | 12 |
| 22. | a. | Compare and contrast fixed point and floating point representation with necessary examples. | CO 5 / U | 12 |
| 23. | a. | An 8-point sequence is given by x [n] =[1, 2, 3, 4, 4, 3, 2,1]. Outline the 8 point DIF FFT of x (n). Draw the flow graph and tabulate the intermediate stage results. | CO 2 /U | 12 |
|  |  | Compulsory: |  |  |
| 24. | a. | Explain the basic Wiener filter theory and discuss its limitations. | CO 6 / E | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Define signals and system mathematically in discrete time domain. |
| CO2 | Formulate the Discrete-Fourier Transform (DFT) and the FFT algorithms |
| CO3 | Explain the various transformations for digital IIR filter design procedures. |
| CO4 | Design FIR digital filters for various applications |
| CO5 | Demonstrate the signal processing concepts and the practical issues with the help of finite word length effects. |
| CO6 | Compare and select the DSP processor and techniques, suitable for the analysis of real-life signals. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 2 | 3 | - | - | 12 | - | 17 |
| CO2 | 5 | 12 | - | - | 12 | - | 29 |
| CO3 | 2 | 3 | - | - | - | 12 | 17 |
| CO4 | 14 | 3 | - | - | - | - | 17 |
| CO5 | 5 | 12 | - | - | 12 | - | 29 |
| CO6 | - | 3 | - | - | 12 | - | 15 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18EC2017 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # COMPUTER NETWORK | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | Which topology requires the most expensive cabling to form a network with 25 nodes? | CO1 | R | 1 |
| 2. | Specify the name of the OSI layer in which the trailer is added. | CO1 | R | 1 |
| 3. | Name the switch that connects n inputs to m outputs in a grid. | CO2 | R | 1 |
| 4. | What is time division switch? | CO2 | R | 1 |
| 5. | List the examples of random access MAC protocols. | CO3 | R | 1 |
| 6. | List the various configurations of High level data link control. | CO3 | R | 1 |
| 7. | Find out the phases involved in virtual circuit communication. | CO4 | R | 1 |
| 8. | Relate the major differences between Distance vector and Link State Routing. | CO4 | R | 1 |
| 9. | Recall the mechanism Back Pressure in closed loop congestion control. | CO5 | R | 1 |
| 10. | Define Leaky bucket. | CO5 | R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | List and define the key elements of a protocol. | CO1 | R | 3 |
| 12. | List the phases in a switched virtual circuit. | CO2 | R | 3 |
| 13. | Illustrate the access method for Token Ring. | CO3 | U | 3 |
| 14. | Summarize the three HDLC station types. | CO4 | U | 3 |
| 15. | Relate the frame format for TCP & UDP Datagram with a neat sketch. | CO5 | R | 3 |
| 16. | What is the necessity of the Domain Name System? | CO6 | R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | a. | Outline ISO/OSI model and explain the responsibilities of user support layers. | CO1 | U | 12 |
| 18. | a. | Elaborate time division switching and space division switching with necessary diagrams. | CO2 | C | 12 |
| 19. | a. | Explain the protocol specifications characteristics of IEEE 802.3 Ethernet. | CO3 | U | 8 |
|  | b. | What are the classifications of Ethernet? | CO3 | R | 4 |

| 20. | a. | Explain the performance of shortest path routing with appropriate diagrams. | CO4 | U | 12 |
| --- | --- | --- | --- | --- | --- |
| 21 | a | Elaborate various techniques to improve the quality of service to avoid congestion. | CO5 | C | 6 |
|  | b | Identify congestion detection using multiplicative decrease with the appropriate diagram. | CO5 | A | 6 |
| 22. | a. | Elaborate on the architecture of Router in detail. | CO4 | C | 10 |
|  | b. | How many cross points are needed in a single stage switch with 40 inputs and 50 outputs? | CO2 | R | 2 |
| 23. | a. | Discuss the issues in resource allocation under the network model. | CO5 | C | 6 |
|  | b. | Explain the mesh, star and ring topologies with a neat diagram and list its advantages and disadvantages. | CO1 | U | 6 |
|  |  | Compulsory: |  |  |  |
| 24. | a. | Elaborate the various web documents and relate hypertext transfer protocol to access data in the world wide web. | CO5 | C | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Demonstrate the functions of the different layers of the OSI protocol. |
| CO2 | Identify the performance of different kinds of switching in the network. |
| CO3 | Design a network for a particular application using IEEE standards. |
| CO4 | Interpret the concepts of networking thoroughly. |
| CO5 | Develop TCP/IP protocol for suitable application. |
| CO6 | Configure application layer protocol. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 5 | 18 | - | - | - | - | 23 |
| CO2 | 7 | - | - | - | - | 12 | 19 |
| CO3 | 6 | 11 | - | - | - | - | 17 |
| CO4 | 2 | 15 | - | - | - | 10 | 27 |
| CO5 | 5 | - | 6 | - | - | 24 | 35 |
| CO6 | 3 | - | - | - | - | - | 3 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18EC2020 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ANTENNA THEORY AND WAVE PROPAGATION | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | Relate Beam solid angle and directivity. | CO1 | R | 1 |
| 2. | Find the efficiency of the antenna if its radiation resistance is 72 Ω and loss resistance is 8Ω. | CO1 | R | 1 |
| 3. | Identify the radiation resistance of an infinitesimal dipole whose overall length is l=λ/50. | CO1 | A | 1 |
| 4. | Recall the conditions of small and large loop antenna. | CO1 | R | 1 |
| 5. | Infer the method used to calculate the total field pattern in antenna arrays. | CO2 | U | 1 |
| 6. | Construct the amplitudes of Binomial array with 4 sources using Pascal’s triangle. | CO2 | A | 1 |
| 7. | Explain Babinet’s principle. | CO3 | U | 1 |
| 8. | Find the power gain in dB of a parabolic reflector of open mouth aperture 10λ. | CO4 | U | 1 |
| 9. | Name a parasitic array antenna. | CO3 | R | 1 |
| 10. | Outline the importance of folded dipole driven element in Yagi-Uda antenna. | CO6 | U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Solve the maximum effective aperture of a microwave antenna having half power beam widths of 300 and 350. | CO1 | A | 3 |
| 12. | List the advantages of array antenna. | CO2 | R | 3 |
| 13. | Apply the concept of pattern multiplication to find the total radiation pattern of 4 isotropic point sources spaced λ/2 apart. | CO4 | A | 3 |
| 14. | Outline the concept of parabolic reflector and sketch its radiation pattern. | CO4 | U | 3 |
| 15. | What are the different layers in log periodic dipole array and how are they differentiated? | CO3 | R | 3 |
| 16. | Summarize on the different types of radio wave propagation. | CO5 | U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 17. |  | Explain the following terms: - Directivity - Gain - HPBW - Radiation Pattern - Beam efficiency - Radiation resistance | CO1 |  | U | 12 |
| 18. |  | Examine the electric field characteristics of a small loop antenna and sketch its radiation pattern. | CO1 |  | An | 12 |
| 19. |  | Explain the End Fire Array and sketch its field pattern by determining the maxima and minima directions. | CO2 |  | U | 12 |
| 20. | a. | Classify horn antenna based on the direction of flaring and also explain its working. | CO3 |  | U | 6 |
|  | b. | Explain about Cassegra in reflector antennas. | CO3 |  | U | 6 |
| 21. | a. | Explain the geometry and operation of the Log Periodic Dipole Array, and also derive its apex angle and spacing factor. | CO4 |  | U | 12 |
| 22. | a. | Show that the resultant field in a direction making an angle with the line of an array consisting of N aerials equally spaced ‘d’ apart and carrying equal aerial currents and in phase is given by , where . Inspect on its significance in antenna design. | CO2 |  | U | 9 |
|  | b. | Examine non-uniform Array with an example. | CO2 |  | An | 3 |
| 23. |  | Examine the geometry and operation of the 3 element Yagi-uda Array antenna and derive the input impedance of active element and parasitic element. | CO3 |  | An | 12 |
|  |  | Compulsory: |  |  |  |  |
| 24. | a. | Explain in detail about smart antenna concepts, benefits and its types. | CO6 | E |  | 8 |
|  | b. | Summarize about ground wave propagation. | CO5 | U |  | 4 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Examine the antenna basic parameters |
| CO2 | Interpret the array factor for uniform and non-uniform arrays |
| CO3 | Relate the fundamental concepts to obtain field distributions of broad band antennas |
| CO4 | Infer the field characteristics of special type antennas |
| CO5 | Categorize the radio wave propagation regions |
| CO6 | Design and analyze various types of antennas using simulation tools |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 3 | 12 | 4 | 12 | - | - | 31 |
| CO2 | 3 | 22 | 1 | 3 |  |  | 29 |
| CO3 | 4 | 13 | - | 12 | - | - | 29 |
| CO4 | - | 16 | 3 | - |  |  | 19 |
| CO5 | - | 7 | - | - | - | - | 7 |
| CO6 | - | 1 | - | - | - | 8 | 9 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18EC2023 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ELECTROMAGNETIC WAVES AND WAVE GUIDE | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Which charge produces the electromagnetic waves? | CO1/R | 1 |
| 2. | Write the condition for which the vector A and B are parallel. | CO1/R | 1 |
| 3. | What is Gauss’s law? | CO1/Un | 1 |
| 4. | How will express the product of the magnitude of charges? | CO2/Un | 1 |
| 5. | What is an insulator? | CO3/R | 1 |
| 6. | Define Biot-Savart law. | CO3/R | 1 |
| 7. | What is the unit of flux density? | CO4/R | 1 |
| 8. | How will calculate poynting vector? | CO5/Ap | 1 |
| 9. | Infer the condition for the law of reflection. | CO5/Ap | 1 |
| 10. | Write an example for perfect dielectric. | CO6/Un | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | What is the significance of divergence? | CO1/R | 3 |
| 12. | What is the relation between Poisson’s and Laplace equation? | CO1/R | 3 |
| 13. | Write down the general, integral and point form for Faraday’s law. | CO1/Un | 3 |
| 14. | Distinguish between solenoid and toroid. | CO2/Un | 3 |
| 15. | Relate the Maxwell equation I and II. | CO3/R | 3 |
| 16. | Express the condition for free space. | CO3/R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | State and Prove divergence theorem. | CO1/R | 8 |
|  | b. | Give the Cartesian coordinates of the points whose cylindrical coordinates are ρ=1; ϕ=450; z=2. | CO1/R | 4 |
| 18. | a. | Define Gauss’s law. Explain its applications in the field of infinite sheet of charge. | CO2/Un | 12 |
| 19. | a. | Derive the capacitance due to composite parallel plate capacitor. | CO2/Un | 8 |
|  | b. | Express the point form of ohms law. | CO2/Un | 4 |
| 20. | a. | Derive the energy density equation in magnetic field using inductance. | CO3/R | 12 |

| 21. | a. | What are the applications of Maxwell’s equation? | CO4/R | 4 |
| --- | --- | --- | --- | --- |
|  | b. | Derive the Maxwell’s equation in both differential and integral form. (any two) | CO4/An | 8 |
| 22. | a. | What is poynting vector? | CO5/Ap | 3 |
|  | b. | Derive the general equation of poynting vector. | CO6/Un | 9 |
| 23. | a. | Explain the characteristics of TE and TM waves. | CO5/Ap | 8 |
|  | b. | Write short notes on waves between parallel plates. | CO6/Un | 4 |
|  |  | Compulsory: |  |  |
| 24. | a. | Derive the wave impedance and characteristic impedance of TEM waves. | CO6/Un | 12 |

Reg. No. _____________

End Semester Examination – May / June – 2022

| # Code : | # 18EC2028 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # MICROPROCESSOR AND MICROCONTROLLER | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | List the flags in 8085 Flag register. | CO1 / R | 1 |
| 2. | Highlight on the purpose of Program counter. | CO1 / R | 1 |
| 3. | Identify the addressing modes of the following instructions: MOV A, @RO MOVC A, @A+DPTR | CO2 / R | 1 |
| 4. | Interpret how you will form the 16 bit memory address in absolute addressing mode. | CO2 / U | 1 |
| 5. | Identify the use of stack pointer. | CO2 / R | 1 |
| 6. | Identify any one advantage of using high level language. | CO3 / R | 1 |
| 7. | Highlight on the functions of the following signals in peripheral interfacing units. a) MEMR b) IOR | CO3 / R | 1 |
| 8. | List out the different modes of operation of 8051 Timers/counters. | CO4 / R | 1 |
| 9. | Quote the communication type, where start and stop bits are needed. | CO4 / R | 1 |
| 10. | Recall the communication protocol which is two-wire bidirectional interface between master and slave devices. | CO5 / R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Define Assembler. | CO1 / R | 3 |
| 12. | Summarize on ‘Bit Direct Addressing’ mode with an example. | CO2 / U | 3 |
| 13. | List any two instructions with syntax and example for 8051 logical instruction. | CO3 / R | 3 |
| 14. | Comment on any three applications of sensors. | CO4 / U | 3 |
| 15. | List out the applications of Zigbee technology. | CO5 / R | 3 |
| 16. | Distinguish between common anode and common cathode 7 segment LED display. | CO6 / A | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | Summarize on the few applications of embedded systems. | CO1 / U | 4 |
|  | b. | Explain the architecture of Intel 8085 Microprocessor. | CO1 / An | 8 |
| 18. |  | Sketch the general syntax of 8051 instruction. Discuss about ‘Data Transfer’ and ‘Arithmetic’ instructions with suitable examples. | CO2 / A | 12 |

| 19. |  | Describe the addressing modes of 8051 with suitable examples. | CO2 / R | 12 |
| --- | --- | --- | --- | --- |
| 20. | a. | Discuss about the 8255 programmable peripheral interface and their operating modes. | CO3 / U | 8 |
|  | b. | Define memory wait state and brief the need for memory wait states. | CO3 / R | 4 |
| 21. | a. | Summarize the features and significance of Serial Peripheral Interface. | CO4 / U | 8 |
|  | b. | Define the following: - Baud rate - Framing - DTE - DCE | CO4 / R | 4 |
| 22. | a. | Illustrate with a neat architecture on Zigbee network and the roles of each devices. | CO6 / An | 6 |
|  | b. | Describe in detail on the following popular external interfaces. - RS232 - I2C | CO6 / R | 6 |
| 23. |  | Discuss in detail on the following peripheral. - ADC - DAC | CO6 / U | 12 |
|  |  | Compulsory: |  |  |
| 24. |  | Illustrate and design a circuit to interface a 7 segment LED display to 8051 microcontroller. | CO6 / A | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Discuss the microprocessor organization and its evolution. |
| CO2 | Describe the architecture of 8051 controllers. |
| CO3 | Express their knowledge in designing a system using 8051 |
| CO4 | Differentiate controller / processor architecture and features. |
| CO5 | Write processor / controller specific programs in Embedded C. |
| CO6 | Simulate the real time system using integrated development environment. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 5 |  |  | 8 | - | - | 13 |
| CO2 | 14 | 4 | 12 |  |  |  | 30 |
| CO3 | 9 | 8 |  |  | - | - | 17 |
| CO4 | 11 | 11 |  |  |  |  | 22 |
| CO5 | 4 |  |  |  | - | - | 4 |
| CO6 | 7 | 12 | 12 | 7 | - | - | 38 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18EC2028 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # MICROPROCESSOR AND MICROCONTROLLER | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | CISC stands for ____________. | CO1 / R | 1 |
| 2. | Register stores data Faster (True/ False) | CO3 / U | 1 |
| 3. | 8085 uses __________ of memory. | CO2 / R | 1 |
| 4. | Microcontrollers are based on ___________ architecture. | CO2 / R | 1 |
| 5. | If RS0 =1 and RS1= 0, ___________ bank of registers will be selected. | CO3 / A | 1 |
| 6. | The I/O port that does not have a dual-purpose role is: | CO3 / U | 1 |
| 7. | Out of the given options, which one uses line by line conversion of source code __________. | CO4 / R | 1 |
| 8. | ORL C, Bit is a ___________ instruction. | CO4 / U | 1 |
| 9. | What is the length of TCON register in 8051? | CO3 / R | 1 |
| 10. | HC-05 Blue tooth has __________ voltage level in RXD and TXD pins. | CO5 / R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Write an assembly level program for performing arithmetic addition of two 8 bit numbers using 8085. | CO3 / U | 3 |
| 12. | Compare microprocessor with microcontroller. | CO4 / U | 3 |
| 13. | What is the function of program counter? | CO2 / U | 3 |
| 14. | What are the two modes of operation of Bluetooth? | CO6 / U | 3 |
| 15. | Differentiate Synchronous and Asynchronous communication. | CO6 / U | 3 |
| 16. | Which motor is used for robotic arm application, state why? | CO6 / A | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q. No. 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. |  | Write an assembly level program in 8085 to find out the largest and then the smallest of given ‘n’ numbers. | CO3 / U | 12 |
| 18. |  | Discuss in detail the various interrupts available in 8051 microcontrollers. Also explain about how the interrupts are enabled and prioritized. | CO2 / R | 12 |

| 19. |  | List out the special function registers available in 8051 microcontroller and explain. |  | CO3 / U |  | 12 |
| --- | --- | --- | --- | --- | --- | --- |
| 20. |  | Explain about TMOD, TCON, THX and TLX registers of 8051 timers along with their format. |  | CO4 / U |  | 12 |
| 21. |  | With necessary diagrams, explain the interfacing of Analog to Digital converter with 8051. |  | CO4 / U |  | 12 |
| 22. |  | Explain in detail about the Serial communication of 8051. Configure the serial communication module receive a message at 4800 baud rate, while the microcontroller is working at 11.0592 MHz. |  | CO3/ A |  | 12 |
| 23. |  | A keyboard and a seven-segment display are used to develop a human machine interface. In this, the user needs to enter the input using a hex keypad and the selected character has to be displayed from the left side of the display. Draw the block diagram of such a system and explain. |  | CO6 / A |  | 12 |
|  |  | Compulsory: |  |  |  |  |
| 24. |  | What is a stepper motor controller? Write an assembly language program to interface a stepper motor to 8051 microcontroller. Explain the Phase and Half pulse scheme of control. | CO6 / A |  | 12 |  |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Discuss the microprocessor organization and its evolution. |
| CO2 | Describe the architecture of 8051 controllers. |
| CO3 | Express their knowledge in designing a system using 8051 |
| CO4 | Differentiate controller / processor architecture and features. |
| CO5 | Write processor / controller specific programs in Embedded C. |
| CO6 | Simulate the real time system using integrated development environment. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 1 |  |  |  |  |  | 1 |
| CO2 | 14 | 3 |  |  |  |  | 17 |
| CO3 | 2 | 28 | 13 |  |  |  | 43 |
| CO4 | 1 | 28 |  |  |  |  | 29 |
| CO5 | 1 |  |  |  |  |  | 1 |
| CO6 |  | 6 | 27 |  |  |  | 33 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18EC2030 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # DIGITAL ELECTRONICS | # Max. Marks : | # 100 |

| Q. No. | Questions |  | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. |  | Mention the universal gates. | CO1 | R | 1 |
| 2. |  | Give an example for minterm. | CO1 | U | 1 |
| 3. |  | Give the representation for don’t care outputs in the truth table. | CO2 | R | 1 |
| 4. |  | Mention the other name for Quine McCluskey method. | CO2 | R | 1 |
| 5. |  | Specify the limitation of half subtractor. | CO3 | U | 1 |
| 6. |  | Interpret the working of Demultiplexer. | CO3 | U | 1 |
| 7. |  | Represent race around condition. | CO4 | U | 1 |
| 8. |  | Represent the states and transition between states in a state diagram. | CO4 | U | 1 |
| 9. |  | Indicate the stable states of a modulus 6 counter. | CO4 | U | 1 |
| 10. |  | Interpret the working operation of counter. | CO4 | U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Represent the logic diagram and working of an AND gate. | CO1 | U | 3 |
| 12. | Convert the hexadecimal number (1C)16to binary. | CO2 | U | 3 |
| 13. | Design the logic circuit for Half Adder. | CO3 | U | 3 |
| 14. | Draw the state diagram for Moore and Mealy circuit. | CO4 | R | 3 |
| 15. | Distinguish between Synchronous and Asynchronous counters. | CO4 | U | 3 |
| 16. | Discuss on the types of memories. | CO5 | U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | a. | Apply DeMorgan’s theorem to simplify AB+CD+EF | CO1 | A | 4 |
|  | b. | Write the standard SOP form for YA,B,C=A+BC | CO1 | U | 3 |
|  | c. | Explain the working of NAND gate and NOR gates with logic diagram and truth table. | CO1 | U | 5 |
| 18. | a. | Minimize the expression Y=ABCD+ABCD+ABCD+ABCD+ABCD+ABCD using K-Map and draw the logic diagram. | CO2 | U | 6 |
|  | b. | Explain the different types of multi level gate implementation with necessary diagrams. | CO2 | U | 6 |

| 19. | a. | Design a circuit that has one control line C and three data lines D1, D2, D3. When the control line is high, the circuit is to detect when one of the data lines has a 1 on it. No more than one data line will ever have a 1 on it. When the control line is low, the circuit will output a 0, regardless of what is on the data lines. | CO3 | A | 8 |
| --- | --- | --- | --- | --- | --- |
|  | b. | Design a 2 to 4 decoder and draw its logic diagram. | CO3 | A | 4 |
| 20. | a. | Construct a D-flip flop and explain the input and output waveforms of a clocked D-flip flop. | CO4 | A | 10 |
|  | b. | Draw the excitation table of T-flip flop. | CO4 | A | 2 |
| 21. | a. | Explain the concept of shift registers and demonstrate the working of Serial in Serial out shift Register. | CO4 | A | 12 |
| 22. | a. | Design an even parity generator for detecting errors during transmission of binary information. | CO3 | U | 8 |
|  | b. | Draw the excitation table of JK Flip flop. | CO4 | U | 4 |
| 23. | a. | Design a decimal to BCD encoder. | CO3 | A | 10 |
|  | b. | Draw the logic symbol and truthtable for 1:2 Demultiplexer. | CO3 | U | 2 |
|  |  | Compulsory: |  |  |  |
| 24. | a. | Explain the block diagram of Programmable Logic Array using neat diagram. | CO6 | U | 8 |
|  | b. | Compare and contrast PROM, PLA and PAL. | CO5 | U | 4 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Compute The Number System Conversions |
| CO2 | Simplify The Boolean Expression Using Various Simplification Techniques |
| CO3 | Design Various Combinational Circuits |
| CO4 | Design Various Sequential Circuits |
| CO5 | Implement Combinational Circuits Using Pld |
| CO6 | State And Compare Different Digital Logic Families |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 1 | 12 |  |  |  |  | 17 |
| CO2 | 2 | 15 |  |  |  |  | 17 |
| CO3 |  | 15 | 22 |  |  |  | 37 |
| CO4 | 3 | 11 | 24 |  |  |  | 38 |
| CO5 |  | 7 |  |  |  |  | 7 |
| CO6 |  | 8 |  |  |  |  | 8 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – June – 2022

| # Code : | # 18EC3001 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ADVANCED DIGITAL SIGNAL PROCESSING | # Max. Marks : | # 100 |

| Q. No. | Sub Div. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |
| 1. | a. | Determine the 8 point DFT of the input x[n]={1,2,3,4,4,3,2,1} using DIT-FFT algorithm. | CO1/E | 12 |
|  | b. | Outline one to one mapping and many to one mapping in digital filter design. | CO1/U | 4 |
| 2. |  | Explain three level sub-band encoding and decoding processwith neat diagrams. | CO2/U | 16 |
| 3. | a. | Examine the Wiener Hoff equation and minimum mean square errorfor FIR Wiener filter. | CO3/An | 12 |
|  | b. | Recall the properties of auto correlation and wide sensestationary process. | CO3/R | 4 |
| 4. |  | Explain the LMS algorithm and discuss its limitations. | CO4/U | 16 |
| 5. | a. | Examine the spectrum estimation using periodogram. | CO5/An | 10 |
|  | b. | Compare the parametric and non-parametric methods of spectrum estimation. | CO5/U | 6 |
| 6. |  | Build the cascade and parallel realization of the system described by y(n)-(1/4)y(n-1)-(1/8)y(n-2)=x(n)+3x(n-1)+2x(n-2) | CO1/A | 16 |
| 7. | a. | Explain the two channel QMF bank with necessary conditions. | CO2/U | 12 |
|  | b. | Determine the efficiency of 512 point FFT computations in-accordance with the direct DFT computations. | CO1/E | 4 |
| PART – B (1 X 20 = 20 MARKS) (Compulsory Question) |  |  |  |  |
| 8. | a. | Examine the digital filtering techniques that are used to process the digital images. | CO6/An | 10 |
|  | b. | Explain speech synthesis process with necessary diagrams. | CO6/E | 10 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Compute the digital filter coefficients for given specification. |
| CO2 | Recognize the need for Multi-rate signal processing applicable to communication systems |
| CO3 | Distinguish various types of prediction methods |
| CO4 | Explore the usefulness of adaptive filters in communication systems |
| CO5 | Compute the power spectrum estimation using various methods |
| CO6 | Infer the relevance and significance of Signal Processing in various applications. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 4 | - | 16 | - | 16 | - | 36 |
| CO2 | - | 28 | - | - | - | - | 28 |
| CO3 | 4 | - | - | 12 | - | - | 16 |
| CO4 | - | 16 | - | - | - | - | 16 |
| CO5 | - | 6 | - | 10 | - | - | 16 |
| CO6 | - | - | - | 10 | 10 | - | 20 |
|  |  |  |  |  |  |  | 132 |

Reg. No. _____________

End Semester Examination – June – 2022

| # Code : | # 18EC3001_PHD | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ADVANCED DIGITAL SIGNAL PROCESSING | # Max. Marks : | # 100 |

ANSWER ALL QUESTIONS (5 X 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Change the analog filter with system function Ha (s) = s + 0.5 (s + 0.5)2 + 16 into a digital IIR filter using the bilinear transformation. The digital filter should have a resonant frequency of ωr= π/2. | CO1 / C | 10 |
|  | b. | Develop the expression for impulse invariance method in IIR filter design. | CO1 / A | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Explain the procedure of designing linear phase FIR filters using the frequency sampling method. | CO1 / U | 10 |
|  | b. | Develop the parallel realizations for the following system y(n)=-0.1y(n-1)+0.2y(n-2)+3x(n)+0.6x(n-2). | CO1 / A | 10 |
| 3. |  | Explain the process of down sampling the signal by an integer factor with necessary sketches and expression. | CO2 / U | 20 |
| (OR) |  |  |  |  |
| 4. |  | Develop the sampling rate conversion systems with - Polyphase filter structure - Cascade integrator comb (CIC) filter structure | CO2 / A | 20 |
| 5. |  | Determine the parameters and sketch the Lattice-Ladder filter structure for the system with system function H(z) = 1-0.8z-1+0.15z-21+0.1z-1-0.72z-2 | CO3 / E | 20 |
| (OR) |  |  |  |  |
| 6. | a. | Outline the role of wiener filters for filtering and prediction of signals. | CO3 / U | 10 |
|  | b. | Consider a signal x(n) = s(n) + w(n) where s(n) is an AR (1) Process that satisfies the difference equation. S(n) = 0.8 s(n-1) + v(n) where {v(n)} is a white noise sequence with variance σ2v = 0.49 and {w(n)} is a white noise sequence with variance σ2w = 1. The process {v(n)} and {w(n)} are uncorrelated (i) Design a wiener filter of length M = 2 to estimate {s(n)} (ii) Determine the MMSE for r = 2. | CO3 / C | 10 |
| 7. |  | Explain how to suppress narrow band interference in a wideband signal sequence. | CO4 / U | 20 |
| (OR) |  |  |  |  |
| 8. | a. | Explain the blackman and tukey method of smoothing the periodogram. | CO5 / U | 10 |
|  | b. | Solve the expressions for quality factor for various non-parametric methods. | CO5 / U | 10 |

|  |  | Compulsory: |  |  |
| --- | --- | --- | --- | --- |
| 9. |  | “Wavelet theory incorporates a number of approaches that been developed independently for numerous signal processing applications”. Justify the statement. | CO6 / E | 20 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Compute the digital filter coefficients for given specification. |
| CO2 | Recognize the need for multi-rate signal processing applicable to communication systems |
| CO3 | Distinguish various types of prediction methods |
| CO4 | Explore the usefulness of adaptive filters in communication systems |
| CO5 | Compute the power spectrum estimation using various methods |
| CO6 | Infer the relevance and significance of Signal Processing in various applications. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | - | 10 | 20 | - | - | 10 | 40 |
| CO2 | - | 20 | 20 | - | - | - | 40 |
| CO3 | - | 10 | - | - | 20 | 10 | 40 |
| CO4 | - | 20 | - | - | - | - | 20 |
| CO5 | - | 20 | - | - | - | - | 20 |
| CO6 | - | - | - | - | 20 | - | 20 |
|  |  |  |  |  |  |  | 180 |

Reg. No. _____________

End Semester Examination – June – 2022

| # Code : | # 18EC3004 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name: | # OPTICAL NETWORKS AND PHOTONIC SWITCHING | # Max. Marks: | # 100 |

| Q. No. | Sub Div. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |
| 1. | a. | Demonstrate the working principle of the device with | CO2 / U | 10 |
|  | b. | Outline the optical communication system with neat block diagram. | CO 1/ U | 6 |
| 2. |  | Discuss different types of optical amplifiers with neat diagrams. | CO2 / U | 16 |
| 3. |  | Demonstrate the working principle of first-generation optical network. | CO3 / U | 16 |
| 4. |  | Analyze the light gathering capacity of the fiber based on its refractive index. | CO1 / An | 16 |
| 5. |  | Discuss passive components in wavelength division multiplexing. | CO4 / C | 16 |
| 6. |  | Analyze different types of interleaving techniques in OTDM. | CO5 / An | 16 |
| 7. |  | Demonstrate the working principle of largest first and smallest last RWA algorithm for with example. | CO4 / U | 16 |
| PART – B (1 X 20 = 20 MARKS) (Compulsory Question) |  |  |  |  |
| 8. | a. | Find the deployment method using a PON device to reduce the number of transceivers in the below diagram to half stating its advantages over other deployment methods. | CO5 / R | 10 |
|  | b. | Analyze the features of MONET for performing experimentation in optical network. | CO6 / An | 10 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the concept of telecom network |
| CO2 | Understand the components of optical networks |
| CO3 | Associate optical networks and its applications |
| CO4 | Know the design of wavelength network and its test beds |
| CO5 | Acquire knowledge on Photonic Packet Switching and access network |
| CO6 | Familiar on the network design and management |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 |  | 6 |  | 16 | - | - | 22 |
| CO2 |  | 26 |  |  |  |  | 26 |
| CO3 | - | 16 |  |  | - | - | 16 |
| CO4 | - | 16 |  | - |  | 16 | 32 |
| CO5 | 10 |  | - | - | - | 16 | 26 |
| CO6 |  |  | - | 10 | - | - | 10 |
|  |  |  |  |  |  |  | 132 |

Reg. No. _____________

End Semester Examination – June – 2022

| # Code : | # 18EC3005 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ANTENNAS AND RADIATION SYSTEMS | # Max. Marks : | # 100 |

| Q. No. | Sub Div. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |
| 1. | a. | Criticize the issues of polarization and the types of polarization in detail. | CO1 / E | 12 |
|  | b. | Conclude about the wire configurations for radiation and non-radiation conditions. | CO1 / A | 04 |
| 2. | a. | Analyze on the current distribution, radiation pattern, directivity,HPBW, and input impedance, If the length of the following antenna is decreased - Dipole:from 1.5to, /2, and /10. - Monopole:from 0.75, to 0.5, 0.25, and /20. | CO2 / An | 12 |
|  | b. | Define the aperture efficiency of an antenna and state the relation with physical aperture for wire and aperture antennas. | CO2 / R | 04 |
| 3. | a. | Develop the array factor of N- isotropic element linear array with uniform amplitude and spacing along z-axis. And also draw the radiation pattern for a linear uniform array of N isotropic elements, if: - β = 0 - β = − kd | CO3 / A | 12 |
|  | b. | Illustrate the principle of pattern multiplication with an example. | CO3 / U | 04 |
| 4. | a. | Assess the horn antenna design equations from phase difference because the wave travelled different distances from the apex to the aperture of horn. And list out the steps followed in the EM simulation tool for the horn antenna design. | CO4 / E | 12 |
|  | b. | Elaborateabout the Babinet’s principle for slot antennas. | CO4 / C | 04 |
| 5. | a. | Organizetheconstruction and feeding methods of micro-strip antennas for mobile phone application. | CO5 / A | 12 |
|  | b. | List out the advantages and drawbacks of the microstripantenna. | CO5 / R | 04 |

| 6. | a. | Elaborate the fundamental principles of Folded Dipole and Loop antennas. | CO2 / C | 08 |
| --- | --- | --- | --- | --- |
|  | b. | Show the transmission-line Thevenin equivalent of antenna in transmitting mode and explain the radiation mechanism. | CO2 / U | 04 |
|  | c. | Evaluate the bandwidth ratio with 100MHz optimum operating frequency (fc) for 10MHz bandwidth . | CO2 / E | 04 |
| 7. | a. | Find vector potential “A” for the Volumetric, Surface and line distributed current densities. | CO1 / R | 04 |
|  | b. | Categorizethe horn antennas and also draw the diagram. | CO4 / An | 04 |
|  | c. | Estimate the directivity in decibels for a rectangular aperture with Lx&Ly= 20λ for a completely uniform aperture illumination. | CO4 / C | 04 |
|  | d. | Outline the Huygen’s principle. | CO4 / U | 04 |
| PART – B (1 X 20 = 20 MARKS) (Compulsory Question) |  |  |  |  |
| 8. | a. | Assess the working principle of parabolic reflector antenna and drive the design expression. | CO6 / E | 12 |
|  | b. | Compose the three cases with respect to the relation between the focal length f and mouth diameter d for a front fed Paraboloidal reflector. | CO6 / C | 08 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Compute the far field distance, radiation pattern and gain of an antenna for given current distribution. |
| CO2 | Estimate the input impedance, efficiency and ease of match for linear wire antennas. |
| CO3 | Explain the array factor for an array of antennas. |
| CO4 | Use aperture concept for efficient antenna design |
| CO5 | Design Micro strip antennas for various desired radiation pattern characteristics. |
| CO6 | Determine the desired parameters of reflector antennas for specific application. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 04 |  | 04 |  | 12 | - | 20 |
| CO2 | 04 | 04 |  | 12 | 04 | 08 | 32 |
| CO3 | - | 04 | 12 |  | - | - | 16 |
| CO4 | - | 04 |  | 04 | 12 | 08 | 28 |
| CO5 | 04 |  | 12 | - | - | - | 16 |
| CO6 |  |  | - | - | 12 | 08 | 20 |
|  |  |  |  |  |  |  | 132 |

Reg.No. _____________

End Semester Examination – June – 2022

| # Code : | # 18EC3012 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # DATA COMPRESSION TECHNIQUES | # Max. Marks : | # 100 |

| Q. No. | Sub Div. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |
| 1. |  | Illustrate the different methodologies of scalar quantization techniques with necessary diagrams and mathematical equations. | CO1 / U | 16 |
| 2. | a. | Outline the various necessities for compressing the multimedia data. Also, point out the various check points to be noted during the compression approaches. | CO1 / U | 8 |
|  | b. | Estimate the codes for the sequence A=[a, b, c, d, e, f, g] with the probability values [0.25, 0.15, 0.05, 0.1, 0.2, 0.2, 0.05] respectively using Huffman coding technique . | CO2 / E | 8 |
| 3. | a. | The tag value of an encoded dataset A= {1,2,3} is given by 0.765625. Using this tag value, estimate the sequence which consists of 4 numerals. Assume the interval as [0, 0.8] for ‘1’, [0.8, 0.82] for ‘2’ and [0.82, 1] for ‘3’. | CO2 / E | 8 |
|  | b. | Estimate the codes for the sequence ‘dabracacabradabracacabracabradabraca’ using digram coding technique. Assume a sample dictionary with any 5 entries and the corresponding code. | CO2 / E | 8 |
| 4. | a. | Determine the codes for the sequence ‘wabbawabbawabbawabbawooxwooxwoox’ using the LZ78 algorithm. | CO2 / E | 8 |
|  | b. | Explain the subband coding method for speech compression applications with neat block diagram. | CO3 / U | 8 |
| 5. |  | Explain the EZW coding methodology used for image compression with a numerical example. | CO4 / U | 16 |
| 6. |  | Illustrate the methodology of channel vocoder for compression of speech signals. | CO3 / U | 16 |
| 7. |  | Illustrate the MPEG methodology of compression of video data with neat flow diagram. | CO5 / U | 16 |
| PART – B (1 X 20 = 20 MARKS) (Compulsory Question) |  |  |  |  |
| 8. |  | Outline the role of compression methodology for satellite image analysis applications. Support your answer with neat diagrams and mathematical equations. | CO6 / U | 20 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Distinguish the pros and cons of lossy and lossless compression techniques. |
| CO2 | Discriminate the compression algorithms of text data |
| CO3 | Develop hybrid compression algorithms for audio data |
| CO4 | Formulate methodologies for image compression approaches |
| CO5 | Determine the performances of video compression algorithms |
| CO6 | Solve practical problems using the coding techniques |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | - | 24 | - | - | - | - | 24 |
| CO2 | - | - | - | - | 32 |  | 32 |
| CO3 | - | 24 | - | - | - | - | 24 |
| CO4 | - | 16 | - | - | - | - | 16 |
| CO5 | - | 16 | - | - | - | - | 16 |
| CO6 | - | 20 | - | - | - | - | 20 |
|  |  |  |  |  |  |  | 132 |

Reg. No. _____________

End Semester Examination – June – 2022

| # Code : | # 18EC3013 / 17EC3018 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ADVANCED DIGITAL IMAGE PROCESSING | # Max. Marks : | # 100 |

ANSWER ALL QUESTIONS (5 X 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Summarize the fundamental steps involved in Digital image processing. | CO1 / U | 10 |
|  | b. | Discuss on the various elements of visual perception. | CO1 / C | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Illustrate the applications of Image processing in the field of medicine with a case study. | CO1 / U | 10 |
|  | b. | Briefly explain the concepts of spatial and intensity resolution. | CO1 / U | 10 |
| 3. |  | Illustrate the spatial filtering process and mention the advantages of various spatial smoothening filter. | CO2 / U | 20 |
| (OR) |  |  |  |  |
| 4. |  | Name the forward and reverse 2D- Discrete Fourier Transform expression and list out its applications and properties. | CO2 / R | 20 |
| 5. |  | Discuss about Erosion, Dilation and active contour. | CO3 / C | 20 |
| (OR) |  |  |  |  |
| 6. |  | Interpret on the various morphological operators used for processing the digital images. | CO3 / U | 20 |
| 7. |  | Briefly explain the concept of image fusion and its types. | CO5 / U | 20 |
| (OR) |  |  |  |  |
| 8. |  | Explain the transformation function used in Image registration process. | CO4 / U | 20 |
|  |  | Compulsory: |  |  |
| 9. | a. | Discuss an abnormality detection technique with a reliable application in medical fields. | CO6 / E | 10 |
|  | b. | What is scene understanding in natural images? Explain with an application. | CO6 / U | 10 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Explain the basic concepts of image formation and representation |
| CO2 | Design techniques for enhancing the quality of the images |
| CO3 | Frame morphology-based methodologies for image segmentation. |
| CO4 | Assess the performances of various image registration approaches |
| CO5 | Differentiate the concepts of 2D and 3D image processing approaches |
| CO6 | Solve practical problems using image processing techniques |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | - | 30 | - | - | - | 10 | 40 |
| CO2 | 20 | 20 | - | - | - | - | 40 |
| CO3 | - | 20 | - | - | - | 20 | 40 |
| CO4 | - | 20 | - | - |  | - | 20 |
| CO5 | - | 20 | - | - | - | - | 20 |
| CO6 | - | 10 | - | - | 10 | - | 20 |
|  |  |  |  |  |  |  | 180 |

Reg. No. _____________

End Semester Examination – June – 2022

| # Code : | # 18EC3014 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # PATTERN RECOGNITION AND MACHINE LEARNING | # Max. Marks : | # 100 |

ANSWER ALL QUESTIONS (5 X 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Discuss about Pattern Recognition and state the various applications of pattern recognition. | CO1 / U | 10 |
|  | b. | Elucidate parametric learning with its types, benefits and limitations. | CO1 / U | 10 |
|  |  | (OR) |  |  |
| 2. | a. | For the given height of 182cm, b0 = 0.1 and b1 = 0.5, predict theperson’s weight using linear regression. | CO2 / A | 5 |
|  | b. | Explain the various categories of linear regression based on its learning model. | CO2 / U | 15 |
| 3. | a. | Illustrate the algorithm to train the input and hidden layers of Backpropagation neural network with suitable diagram. | CO3 /A | 12 |
|  | b. | Discover the effectiveness of the following layers in a deep neural network: (i) convolution (ii) pooling (iii) dropout. | CO3 / U | 8 |
|  |  | (OR) |  |  |
| 4. | a. | Describe linear discriminant functions for minimum error rate classification. | CO4 / U | 10 |
|  | b. | Illustrate the formulation of a real-world problem into a mathematical model using linear programming. | CO4 / A | 10 |
| 5. | a. | Describe the significance of variance and bias in machine learning. | CO5 / U | 10 |
|  | b. | Examine the re-sampling process adopted for classifier design in machine learning. | CO5 / An | 10 |
|  |  | (OR) |  |  |
| 6. | a. | Discuss the working principle of Ho-Kashyap procedure in solving the linear discriminant function. | CO4/ U | 10 |
|  | b. | Examine the Perceptron algorithm for pattern classification and discuss its decision boundary. | CO3 / An | 10 |
| 7. | a. | Write short notes about the following terminologies. (i) Joint probability (ii) Conditional probability and (iii) Bayesian rule. | CO1 / U | 6 |
|  | b. | Discuss and analyze the strategy adopted in Adaboost algorithm to improve the classification accuracy. | CO3 / An | 14 |
|  |  | (OR) |  |  |
| 8 | a. | Discuss the working principle of support vector machine in solving the linear discriminant function. | CO4 / U | 10 |
|  | b. | Explain Hierarchical clustering with detailed algorithm and necessary diagrams. | CO6 / U | 10 |

|  |  | Compulsory: |  |  |
| --- | --- | --- | --- | --- |
| 9. | a. | Differentiate soft and hard clustering. Explain the stepwise procedure of k-means clustering algorithm with neat diagram. | CO6 / An | 10 |
|  | b. | Discuss the various categories of machine learning algorithms to solve the real-world problems. | CO6 / U | 10 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Explain the basics of pattern recognition and machine learning |
| CO2 | Illustrate the linear models for classification |
| CO3 | Select the natural network for classification |
| CO4 | Summarize the concept of linear discriminant function |
| CO5 | Design algorithm independent machine learning |
| CO6 | Develop unsupervised learning techniques and clustering |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 |  | 26 |  |  |  |  | 26 |
| CO2 |  | 15 | 5 |  |  |  | 20 |
| CO3 |  | 8 | 12 | 24 |  |  | 44 |
| CO4 |  | 30 | 10 |  |  |  | 40 |
| CO5 |  | 10 |  | 10 |  |  | 20 |
| CO6 |  | 20 |  | 10 |  |  | 30 |
|  |  |  |  |  |  |  | 180 |

Reg. No. _____________

End Semester Examination – June – 2022

| # Code : | # 18EC3019 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # WIRELESS SENSOR NETWORKS | # Max. Marks : | # 100 |

| Q. No. | Sub Div. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |
| 1. | a. | Design a Multi-hop Hierarchical Clustering based architecture Wireless sensor network for fire detection. Explain the flow of query and response using attribute-based naming in this four-quadrant network. | CO1 / C | 8 |
|  | b. | Solve the problems such as: traffic congestion, traffic violations, road accidents, vehicle theft using suitable wireless sensor network systems. | CO1 / A | 8 |
| 2. | a. | Distinguish Mica2 and MicaZ with their complete hardware and software details. | CO2 / An | 8 |
|  | b. | What is embedded OS? Explain with an example. | CO4 / R | 8 |
| 3. | a. | Solve the synchronization problem using various protocols/methods. Time synchronization is important in a sensor network. Justify. | CO3 / A | 8 |
|  | b. | Categorize the different transport protocols used for sensor networks.Is there any difference in the transport protocols used in the forward and reverse path? | CO3 / An | 8 |
| 4. | a. | Explain various methods to solve the positioning problem.GPS is not suitable for many applications of wireless sensor network. Indicate the reasons. | CO6 / U | 8 |
|  | b. | Illustrate the activities involved in tracking ifa Zebra moves in a sensor field. Assume we have an embedded wireless sensor network to track the Zebra in the forest. | CO6 / U | 8 |
| 5. | a. | Categorize the different types of queries made to sensor network database. | CO6 / An | 8 |
|  | b. | Distinguish the model that require sensor nodes with high storage, while discussing the different sensor database models. | CO6 / An | 8 |
| 6. | a. | Construct a WSN based Urban traffic Management system. | CO1 / A | 8 |
|  | b. | Explain the salient features of Smart dust. | CO2 / U | 8 |
| 7. | a. | Consider the scenario of 4 nodes N1, N2, N3 and N4 in a virtual cluster. In particular cycle if N2 and N4 goes to sleep and N3 transmits data to N1. Illustrate the co-ordinate sleeping of S-MAC with a neat diagram. | CO3 / U | 8 |
|  | b. | Explain the various phases of LEACH protocol. | CO5 / U | 8 |

| PART – B (1 X 20 = 20 MARKS) (Compulsory Question) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 8. | a. | Show in what way idle power management is achieved in ARM-SA 1100 processor and in Bluetooth radio. | CO6 / U | 10 |
|  | b. | Examine if IEEE 802.11 is suitable for Wireless sensor networks. | CO6 / An | 10 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Design wireless sensor network system for different applications under consideration. |
| CO2 | Understand the hardware details of different types of sensors and select right type of sensor for various applications. |
| CO3 | Understand radio standards and communication protocols to be used for wireless sensor network-based systems and application. |
| CO4 | Use operating systems and programming languages for wireless sensor nodes, performance of wireless sensor networks systems and platforms. |
| CO5 | Handle special issues related to sensors like energy conservation and security challenges. |
| CO6 | Students will be able to understand the concepts of sensor networks, applications and different types of protocols in WSN. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | - | - | 16 | - | - | 8 | 24 |
| CO2 | - | 8 | 8 | - | - | - | 16 |
| CO3 | - | 8 | 8 | 8 | - | - | 24 |
| CO4 | - | - | - | 8 | - | - | 8 |
| CO5 | - | 8 | - | - | - | - | 8 |
| CO6 | - | 26 | - | 26 | - | - | 52 |
|  |  |  |  |  |  |  | 132 |

Reg. No. _____________

End Semester Examination – June – 2022

| # Code : | # 18EC3021 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # HIGH PERFORMANCE NETWORKS | # Max. Marks : | # 100 |

| Q. No. | Sub Div. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A(5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |
| 1. |  | Identifythe network design issues and explain the different approaches of Streaming stored audio/video. | CO1 / A | 16 |
| 2. | a. | Explain the components used in VoIP system architecture with a neat bock diagram. | CO2 / U | 12 |
|  | b. | Compare the difference between PSTN and VoIP. | CO2 / An | 04 |
| 3. | a. | Elaborate on Multiprotocol Label Switching. | CO3 / C | 10 |
|  | b. | Illustrate the different types of Virtual Private Networks protocols. | CO3 / U | 06 |
| 4. | a. | Assume a data-communication network with three transmission lines; packets arrive at three different nodes with arrival rates λ1=100 packets/sec, λ2=200 packets/sec, and λ3=20 packets/sec, respectively. Assume that an average of 50,000 same- size packets float in this system. Find the average delay per packet in the system. | CO4 / An | 04 |
|  | b. | Elaborate the need for Traffic Modelling and the Network performance evaluation. | CO4 / C | 12 |
| 5. |  | Elaborate on Key Management and key distribution in Network Security. | CO5 / C | 16 |
| 6. |  | Classify the types of VPN and discuss about the security issues. Also criticize the merits and demerits of Virtual Private Networks. | CO3 / An | 16 |
| 7. |  | Elaborate the different types of Message authentication. Also analyze the merits and demerits of authentication protocols. | CO5 / C | 16 |
| PART – B (1 X 20 = 20 MARKS) (Compulsory Question) |  |  |  |  |
| 8. | a. | Explain the principal components for network management in detail with appropriate diagram. | CO6 / E | 10 |
|  | b. | Discuss the working of SNMP Protocol. | CO6 / C | 10 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Identify various type of networks and the protocols used for various applications. |
| CO2 | Explain VoIP system function and its applications. |
| CO3 | Outline the functioning of VPN. |
| CO4 | Apply different traffic modeling techniques to evaluate the network performance. |
| CO5 | Recognize the required security techniques used in different layer of communication. |
| CO6 | Illustrate various infrastructure framework for network management. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | - | - | 16 | - | - | - | 16 |
| CO2 | - | 12 | - | 04 | - | - | 16 |
| CO3 | - | 06 | - | 16 | - | 10 | 32 |
| CO4 | - | - | - | 04 | - | 12 | 16 |
| CO5 | - | - | - | 16 | - | 16 | 32 |
| CO6 | - | - | - | - | 10 | 10 | 20 |
|  |  |  |  |  |  |  | 100 |

Reg. No. _____________

End Semester Examination – June – 2022

| # Code : | # 18EC3039 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # TESTING AND TESTABILITY | # Max. Marks : | # 100 |

| Q. No. | Sub Div. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |  |
| 1. | a. | Justify that Moore’s law motivates testing. | CO1 | E | 4 |
|  | b. | Examine the VLSI realization process with a neat flow chart. | CO1 | An | 12 |
| 2. | a. | Determine the fault coverage for all the possible test pattern in Fig 1. Fig 1 | CO2 | E | 6 |
|  | b. | Determine the collapsed ratio for all possible stuck-at faults in the circuit of Fig 2, apply fault dominance and reduce the number of faults. Fig 2 | CO2 | E | 10 |
| 3. | a. | Show the block diagram of the Combinational and Sequential circuit. | CO3 | U | 4 |
|  | b. | Determine test patterns for detecting the fault in the circuit using the Boolean difference method shown in Fig 3 with respect to variable ‘h’. Fig 3 | CO3 | E | 12 |

| 4. | a. | Define fault simulation and mention the input and output required for fault simulator. | CO4 | R | 4 |
| --- | --- | --- | --- | --- | --- |
|  | b. | Identify the faults detected by the input vector for the circuit given in Fig 4, using deductive fault simulation. Fig 4 | CO4 | A | 12 |
| 5. | a. | Define Observability and Controllability. | CO5 | R | 4 |
|  | b. | Design IEEE 1149.1 chip architecture and discuss the operation of its architecture. Also, explain how to test a board using the Boundary Scan chain technique. | CO5 | C | 12 |
| 6. | a. | Justify the statement that “sequential circuit testing is difficult as compared to combinational circuit testing”. | CO3 | E | 4 |
|  | b. | Determine the test sequence for the state table of the sequential circuit shown in Table 1 using the checking experiment. Table 1 PS; X=0; X=1. A; A/1; E/0. B; A/0; C/0. C; B/0; D/1. D; C/1; C/0. E; C/0; D/0. | CO3 | E | 12 |
| 7. | a. | Explain various Ad Hoc design rules for improving testability with necessary block diagrams. | CO5 | U | 8 |
|  | b. | Design a single latch LSSD and Justify that it overcomes the demerits of double latch LSSD. | CO5 | C | 8 |
| PART – B (1 X 20 = 20 MARKS) (Compulsory Question) |  |  |  |  |  |
| 8. | a. | Explain the signature analysis compression technique used in BIST. | CO6 | U | 10 |
|  | b. | Design a LOCST BIST architecture and explain its operation. | CO6 | C | 10 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Identify the testing concepts at various levels of abstraction |
| CO2 | Compute the knowledge of modeling of faults |
| CO3 | Utilize the test generation algorithms for generating test vectors |
| CO4 | Perform modal analysis for fault simulation techniques |
| CO5 | Design and develop various architectures for DFT |
| CO6 | Develop various BIST architecture |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 |  |  |  | 12 | 4 |  | 16 |
| CO2 |  |  |  |  | 16 |  | 16 |
| CO3 |  | 4 |  |  | 28 |  | 32 |
| CO4 | 4 |  | 12 |  |  |  | 16 |
| CO5 | 4 | 8 |  |  |  | 20 | 32 |
| CO6 |  | 10 |  |  |  | 10 | 20 |
|  |  |  |  |  |  |  | 132 |

Reg. No. _____________

End Semester Examination – June – 2022

| # Code : | # 18EC3042 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # CAD FOR VLSI CIRCUITS | # Max. Marks : | # 100 |

| Q. No. | Sub Div. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |
| 1. | a. | Estimate the design methodology flow for the design automation for the Y-chart. | CO1 / A | 10 |
|  | b. | Explain the fabrication process and its impact on the Physical design automation. | CO1 / U | 6 |
| 2. |  | Organize the types of ASIC in detail used in CAD VLSI circuits? Give an illustration of the various types of ASIC with neat Diagram. | CO1 / An | 16 |
| 3. |  | Assessing the Dijkstra’s Shortest path algorithm for the graph given below, a. Write the Dijkstra’s Pseudo Code for the graph. b. Write the Dijkstra’s Shortest path algorithm for the graph and give the output for shortest path from vertex A-G. c. Compute the time complexity and space complexity for the graph. | CO2 / E | 16 |
| 4. | a. | Assessing the DFS algorithm for the graph given below a. Write the DFS Pseudo Code for the graph b. Write the DFS python program for the graph and give the output c. Calculate the time complexity and space complexity | CO2 / C | 12 |
|  | b. | Interpreting the Design flow for the design automation for the physical design automation. | CO6 / U | 4 |
| 5. |  | Organize the types of Floor planning in detail used in CAD VLSI circuits? Give an illustration of the various types of Floorplans with neat diagram. | CO3 / An | 16 |
| 6. | a. | Assessing the TSP algorithm for the graph given below - Write the TSP Pseudo Code for the graph - Write the TSP algorithm for the graph and give the output Calculate the cost of the TSP with starting node as 1 | CO4 / A | 12 |
|  | b. | Interpreting the placement algorithm for the design automation for the physical design automation. | CO3 / U | 4 |
| 7. |  | Organize the Back Tacking Algorithm (BTA) in detail used in CAD VLSI circuits for the given graph. - Write the BTA Pseudo Code for the graph - Write the BTA algorithm for the graph and give the output - Calculate the cost of the BTA with starting node as A | CO5 / C | 16 |
| PART – B (1 X 20 = 20 MARKS) (Compulsory Question) |  |  |  |  |
| 8. |  | Explain in detail the classical routing model used for channel routing. Route the following netlist using Left Edge algorithm: U={I1,I2,I3…I7};I1={1,4},I2={2,6},I3={3,8},I4={5,10},I5={7,1 1},I6={9,12}.Draw the interval graph corresponding to the solution | CO6 / An | 20 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | To design the circuit using Floor planning, Placement and Routing concepts |
| CO2 | To design and implement various algorithms onto FPGA |
| CO3 | To verify Simulation and Synthesis process in the circuit design |
| CO4 | Explain the concept of portioning and floor planning |
| CO5 | Explain various techniques related to Placement, Routing, Circuit extraction and DRC |
| CO6 | Impact of various algorithm on FPGA |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | - | 6 | 10 | 16 | - | - | 32 |
| CO2 | - | - | - | - | 16 | 12 | 28 |
| CO3 | - | 4 | - | 16 | - | - | 20 |
| CO4 | - | - | 12 | - | - | - | 12 |
| CO5 | - | - | - | - | - | 16 | 16 |
| CO6 | - | 4 | - | 20 | - | - | 24 |
|  |  |  |  |  |  |  | 132 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18EC3047 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # HARDWARE DESIGN VERIFICATION TECHNIQUES | # Max. Marks : | # 100 |

| Q. No. | Sub Div. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |
| 1. | a. | # With the importance of functional verification discuss in detail about the various functional approaches. | CO1 / C | 8 |
|  | b. | Distinguish Testing and verification with neat explanation. | CO1 / A | 8 |
| 2. | a. | # With neat diagram and timing Waveform explain in detail about Functional coverage | CO1 / U | 10 |
|  | b. | # What is the need for layered testbench? | CO1 / R | 6 |
| 3. | a. | How the procedural statements and routines are applied in Datatypes? | CO2 / R | 10 |
|  | b. | Define the basic object oriented concepts. | CO2 / R | 6 |
| 4. | a. | # With an example explain in detail about threads and interprocess communication. | CO3 / U | 8 |
|  | b. | With neat diagram explain in detail about levels of randomization. | CO3 / U | 8 |
| 5. | a. | Discuss in detail about Transaction-Level Modeling. | CO3 / C | 8 |
|  | b. | Explain in detail about OVM Mechanics. | CO3 / U | 8 |
| 6. | a. | Distinguish the concepts of OVM and UVM. | CO4 / A | 8 |
|  | b. | Explain in detail about the Object-oriented Testbench Design. | CO4 / U | 8 |
| 7. | a. | Explain in detail about verifying the UVM Components and UVM Communication with Ports. | CO5 / U | 10 |
|  | b. | List in detail about UVM Agents and UVM Sequences. | CO5 / R | 6 |
| PART – B (1 X 20 = 20 MARKS) (Compulsory Question) |  |  |  |  |
| 8. | a. | Explain in detail about Voice Over IP (VoIP) Network SoC Verification. | CO6 / U | 8 |
|  | b. | Illustrate briefly CPU Verification and Cache Memory Subsystem Verification. | CO6 / U | 8 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Compute the various functional design verification |
| CO2 | Generalize the basics of system verilog |
| CO3 | utilize the advanced system Verilog in digital design |
| CO4 | Illustrate the open verification methodology |
| CO5 | Evaluate about Universal Verification Methodology |
| CO6 | perform verification in various digital design circuits |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 6 | 10 | 8 | - | - | 8 | 32 |
| CO2 | 16 | - | - | - | - | - | 16 |
| CO3 | - | 24 | - | - | - | 8 | 32 |
| CO4 | - | 8 | 8 | - | - | - | 16 |
| CO5 | 6 | 10 | - | - | - | - | 16 |
| CO6 | - | 16 | - | - | - | - | 16 |
|  |  |  |  |  |  |  | 128 |

Reg. No. _____________

End Semester Examination – June – 2022

| # Code : | # 18EC3050 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # SOLID STATE DEVICE MODELING AND SIMULATION-MOS MODEL | # Max. Marks : | # 100 |

ANSWER ALL QUESTIONS (5 X 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Design an MOS Capacitance, and explain itsoperations in detail with relevant diagrams | CO1 / C | 10 |
|  | b. | Explain in detail about Device Physics of MOS Capacitance with equivalent circuit Explain its Models. | CO1 / U | 10 |
| (OR) |  |  |  |  |
| 2. |  | Illustrate an MOSFET and explain in detail about itsmodels with required equations. | CO1 / C | 20 |
| 3. |  | Designadvanced MOSFET modeling and designa n-Channel MOSFET with condition of Threshold Voltage (Vt=0.8V). | CO2 / C | 20 |
| (OR) |  |  |  |  |
| 4. |  | Explain RF Semiconductor modeling and Design a MOSFET with suitable equivalent circuit applicable for high frequency RF amplifier. | CO2 / C | 20 |
| 5. |  | Describe in detail on semiconductor noise and recite the different noise sources that accumulates in MOSFET. | CO3 / U | 20 |
| (OR) |  |  |  |  |
| 6. | a. | Identify CMOS circuit noise and explain in detail about non-linearities in CMOS devices and modeling | CO3 / U | 10 |
|  | b. | Analyze an analog CMOS circuit distortion with a relevant equation. | CO3 / U | 10 |
| 7. |  | Explain in detail BSIM4 MOSFET Model and how to obtainenhanced model for effective DC and AC channel length and width | CO4 / U | 20 |
| (OR) |  |  |  |  |
| 8. | a. | Analyze MOSFET’s Threshold voltage model with required equations | CO4 / An | 10 |
|  | b. | Analyze Silicon MOSFET’s Channel Charge Model with required equations. | CO4 / An | 10 |
|  |  | Compulsory: |  |  |
| 9. |  | Demonstrate required simulation of MOSFET with models, Non-quasi-static model, Noise model, Temperature effects, MOS device mismatch for Analog/RF Applications. | CO5 / U | 20 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Compute new mathematical models for various devices |
| CO2 | Demonstrate the physics behind the semiconductor devices |
| CO3 | Illustrate various noise modeling and non-linearities in CMOS Devices |
| CO4 | Summarize various BSIM4 MOSFET Models, EKV MOSFET Models |
| CO5 | Develop various SPICE models for MOS devices |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | - | 10 | - | - | - | 10 | 20 |
| CO2 | - | - | - | - | - | 20 | 20 |
| CO3 | - | 20 | - | - | - | 20 | 20 |
| CO4 | - | - | - | 20 | - | - | 20 |
| CO5 | - | 20 | - | - | - | - | 20 |
|  |  |  |  |  |  |  | 100 |

Reg. No. _____________

End Semester Examination –June – 2022

| # Code : | # 18EC3051 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # NANOSCALE DEVICES AND CIRCUIT DESIGN | # Max. Marks : | # 100 |

ANSWER ALL QUESTIONS (5 X 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| 1. |  | Explain in detail about nanoscale CMOS semiconductor device leakage current mechanism and its optimization. | CO1 / R | 20 |
| (OR) |  |  |  |  |
| 2. |  | Illustrate device technologies for sub 100nm CMOS: Silicidation and Cu-low k interconnects. | CO1 / R | 20 |
| 3. |  | Explain FINFETs with required diagram and its working operations, advantages and applications. | CO2 / R | 20 |
| (OR) |  |  |  |  |
| 4. |  | Explain in detail about quantum confinement and how the tunneling occurred in MOSFTEs. | CO2 / R | 20 |
| 5. |  | Explain the working of Silicon nanowire MOSFET and hetero-structure nanowire MOSFETs with required diagram. | CO3 / R | 20 |
| (OR) |  |  |  |  |
| 6. | a. | Explain in detail about Quantum wells on semiconductor channel. | CO3 / R | 10 |
|  | b. | Explain in detail on Resonant tunneling devices with relevant diagram. | CO3 / R | 10 |
| 7. |  | Explain in detail about CMOS logic power and performance with a relevant diagram. | CO4 / U | 20 |
| (OR) |  |  |  |  |
| 8. |  | Explain in detail about low power CMOS design and explain its different optimization technique with required diagram. | CO4 / U | 20 |
|  |  | Compulsory: |  |  |
| 9. | a. | Explain Sequential Statistical circuit design | CO5 / R | 5 |
|  | b. | Design an Non-classical CMOS: CMOS circuit design using non-classical devices; | CO5 / C | 15 |
|  |  | 1. Nanowire |  |  |
|  |  | 2. FINFETs |  |  |
|  |  | 3. Carbon nanotubes |  |  |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Use the effects of leakage current and its control and reduction techniques in CMOS devices |
| CO2 | Explore the device technologies for sub 100nm CMOS |
| CO3 | Comprehend the device scaling of single and multigate MOSFETs. |
| CO4 | Familiarize the low power design and voltage scaling issues in Nano scale devices. |
| CO5 | Gain knowledge on various nanoscale devices. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 20 | - | - | - | - | - | 20 |
| CO2 | 20 | - | - | - | - | - | 20 |
| CO3 | 20 | - | - | - | - | - | 20 |
| CO4 | - | 20 | - | - | - | - | 20 |
| CO5 | 5 | - | - | - | - | 15 | 20 |
|  |  |  |  |  |  |  | 100 |

Reg.No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 19EC1001 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # FUNDAMENTALS OF ELECTRICAL AND ELECTRONICS ENGINEERING | # Max. Marks : | # 100 |

| Q. No. | Questions |  | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. |  | Outline the need of star rating in electrical appliances. | CO1/ U | 1 |
| 2. |  | List the advantages of LED. | CO1/ R | 1 |
| 3. |  | Outline the benefits of BLDC motor. | CO2/ U | 1 |
| 4. |  | List the types of DC motor. | CO2 / R | 1 |
| 5. |  | Define doping. | CO3 / R | 1 |
| 6. |  | Compare active and passive components. | CO3 / E | 1 |
| 7. |  | Name the gate which produces high output only when both the inputs are high. | CO4 / R | 1 |
| 8. |  | List the types of flip flops. | CO4 / R | 1 |
| 9. |  | Outline the applications of sensors. | CO5 / U | 1 |
| 10. |  | Interpret the role of sensors in day-to-day life. | CO5 / U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Outline the function of a generator. | CO1 / U | 3 |
| 12. | List few applications of motor. | CO2 / R | 3 |
| 13. | Examine the application of transistor as a switch. | CO3 /An | 3 |
| 14. | Compare combinational and sequential circuits. | CO4 / E | 3 |
| 15. | List the various types of sensors. | CO5 / R | 3 |
| 16. | Compare 3G and 4G technologies. | CO6 / E | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. |  | Examine the working of Hydro Power Plant with neat diagram. | CO1 /An | 12 |
| 18. |  | Explain the construction and working of electrical motor with suitable diagrams. | CO2 / E | 12 |
| 19. |  | Interpret the operation and characteristics of Zener diode. | CO3 / U | 12 |
| 20. |  | Interpret the operation of various logic gates with its symbol and truth table. | CO4 / U | 12 |

| 21. |  | Appraise on the applications of sensors in food processing. | CO5 / E | 12 |
| --- | --- | --- | --- | --- |
| 22. |  | Analyse the characteristics of PN junction diode and explain its operation. | CO3 /An | 12 |
| 23. |  | Examine the types of combinational and sequential circuits with suitable examples. | CO4 /An | 12 |
|  |  | Compulsory: |  |  |
| 24. |  | Explain briefly about the various blocks of satellite communication. | CO6 / E | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Recognize importance and judicious use of energy systems in everyday life |
| CO2 | Identify the types of electrical machines used for various applications |
| CO3 | Understand and apply the concept of electronics to design simple circuits |
| CO4 | Understand and relate various digital circuits |
| CO5 | Understand the various sensing and instrumentation applications |
| CO6 | Identify the various generations of wireless communications |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 1 | 4 | - | 12 | - | - | 17 |
| CO2 | 4 | 1 | - | - | 12 | - | 17 |
| CO3 | 1 | 12 | - | 15 | 1 | - | 29 |
| CO4 | 2 | 12 | - | 12 | 3 |  | 29 |
| CO5 | 3 | 2 | - | - | 12 | - | 17 |
| CO6 | - | - | - | - | 15 | - | 15 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – May / June – 2022

| # Code : | # 19EC1001 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # FUNDAMENTALS OF ELECTRICAL AND ELECTRONICS ENGINEERING | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | Find the resistance of the current flowing through a 40 W filament lamp powered up by 240 V operating at normal operating temperature. | CO1 | E | 1 |
| 2. | In the circuit shown below the value of I1 is ___________. | CO1 | E | 1 |
| 3. | An electromechanical device which transforms electrical energy to mechanical energy is called as ____________. | CO2 | R | 1 |
| 4. | Mention a type of AC motor. | CO2 | R | 1 |
| 5. | The electronic device which stores charges is known as __________. | CO3 | U | 1 |
| 6. | A common base configuration of the NPN transistor has _________ current gain. | CO3 | U | 1 |
| 7. | A circuit that computes the sum of two bits and produces the correct carry bit is known as ____________. | CO4 | R | 1 |
| 8. | What is a logic circuit which can remember a previous output called as? | CO4 | R | 1 |
| 9. | A person who performs ultrasound scans is known as ___________. | CO5 | R | 1 |
| 10. | The input of a biosensor is also known as __________. | CO5 | R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Explain temperature resistance effect with suitable formula. | CO1 | R | 3 |
| 12. | List the types of DC motors with its important characteristics. | CO2 | U | 3 |
| 13. | An NPN Transistor has a DC current gain, (Beta) value of 200. Calculate the base current Ib required to switch a resistive load of 4mA. | CO3 | E | 3 |
| 14. | State the commutative and associative properties of Boolean Algebra based on OR gate with proper truth tables. | CO4 | U | 3 |
| 15. | Define comparator with suitable truth table. | CO5 | R | 3 |
| 16. | Differentiate ground waves and sky waves. | CO6 | U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 17. | a. | Discuss elaborately about electricity billing in India. |  | CO1 |  | An |  | 8 |
|  | b. | Explain the steps for maximum demand control in Electrical Systems. |  | CO1 |  | An |  | 4 |
| 18. | a. | Discuss briefly about the construction and working of an electrical motor. |  | CO2 |  | An |  | 6 |
|  | b. | Discuss about electric cars and the factors involved in manufacturing them. |  | CO2 |  | An |  | 6 |
| 19. | a. | Discuss about the types of transformers with the principles behind them. |  | CO3 |  | An |  | 8 |
|  | b. | Explain the basic principle of PN junction diode with suitable illustration. |  | CO3 |  | A |  | 4 |
| 20. | a. | Substantiate the Boolean Expressions of a Half Adder and a Full Adder with proper truth tables and logical circuit diagrams. |  | CO4 |  | An |  | 6 |
|  | b. | Prove the various properties of Boolean algebra with suitable truth tables based of AND & OR gates. |  | CO4 |  | A |  | 6 |
| 21. | a. | Discuss about the automatic irrigation system with suitable diagrams. |  | CO5 |  | An |  | 6 |
|  | b. | Explain briefly about the principles involved in biosensors. |  | CO5 |  | A |  | 6 |
| 22. | a. | Explain in briefly about any two Bipolar Transistor configurations with suitable circuit diagrams. |  | CO3 |  | A |  | 8 |
|  | b. | Draw the logical circuit diagram and Truth Table for the Boolean expression AB + AC. |  | CO4 |  | A |  | 4 |
| 23. | a | Discuss briefly about ultrasound scanner and its types. |  | CO5 |  | A |  | 6 |
|  | b | Differentiate between PN junction diode and Zener diode. |  | CO3 |  | A |  | 6 |
|  |  | Compulsory: |  |  |  |  |  |  |
| 24. |  | Discuss in detail about the blocks of communication systems. | CO6 |  | An |  | 12 |  |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Recognize importance and judicious use of energy systems in everyday life |
| CO2 | Identify the types of electrical machines used for various applications |
| CO3 | Understand and apply the concept of electronics to design simple circuits |
| CO4 | Understand and relate various digital circuits. |
| CO5 | Understand the various sensing and instrumentation applications |
| CO6 | Identify the various generations of wireless communications |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 3 | - | - | 12 | 2 | - | 17 |
| CO2 | 2 | 3 | - | 12 | - | - | 17 |
| CO3 | - | 2 | 18 | 8 | 3 | - | 31 |
| CO4 | 2 | 3 | 10 | 6 | - | - | 21 |
| CO5 | 5 | - | 12 | 6 | - | - | 23 |
| CO6 | - | 3 | - | 12 | - | - | 15 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 19EC2001 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ELECTRONICS FOR INTELLIGENT MACHINES | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Infer how many years it took for Pokemon go to reach 100M users? | CO1 / U | 1 |
| 2. | Expand ANI, AGI, ASI. | CO1 / R | 1 |
| 3. | Compare Industry 1.0 and Industry 2.0. | CO2 / A | 1 |
| 4. | Infer the unfair advantage of KFC | CO6 / U | 1 |
| 5. | Name a system of collaborating computational elements controlling physic entities. | CO3 / A | 1 |
| 6. | Name a robot with its body shape built to resemble the human body. | CO3 / R | 1 |
| 7. | List the different types of general IOT layers. | CO4 / R | 1 |
| 8. | Expand DBMS. | CO4 / A | 1 |
| 9. | Show an example for Big data. | CO5 / R | 1 |
| 10. | Illustrate data transformation. | CO5 / U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Explain the different phases of earlier three industrial evolution with an example. | CO2 / U | 3 |
| 12. | List the USP of Adidas and McDonald. | CO2 / R | 3 |
| 13. | List any 2 I/O devices connected to a smart phone. | CO4 / An | 3 |
| 14. | Differentiate Industry 3.0 and Industry 4.0. | CO4 / An | 3 |
| 15. | Explain the six design principle in industry 4.0. | CO5 / U | 3 |
| 16. | Distinguish the earlier taxi system and Ola system. | CO1 / An | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17 |  | Construct the block diagram of industry 4.0 and discuss the functions of each block. | CO1 / U | 12 |
| 18. | a. | Construct the block diagram of an Intelligent machine and explain the functioning. | CO2 / A | 8 |
|  | b. | Summarize the different machining operations in a manufacturing industry. | CO6 / U | 4 |

| 19. | a. | Explain the 4 layer architecture of IoT and discuss the function of each layer. | CO3 / U | 6 |
| --- | --- | --- | --- | --- |
|  | b. | Analyze the function of an IoT based health monitoring system with respect to the IoT architecture. | CO6 / A | 6 |
| 20. | a. | Infer the different types and characteristics of big data with examples. | CO4 / A | 3 |
|  | b. | Explain the GSM interface with a neat sketch. | CO4 / U | 9 |
| 21. | a. | Analyze the advantages and disadvantages of industry 4.0 standards. | CO1 / An | 6 |
|  | b. | Model the working of OYO room booking system. | CO1 / A | 6 |
| 22. | a. | Explain the working of GPS sensor. | CO4 / U | 4 |
|  | b. | List the characteristics and benefits of Big data. | CO5 / R | 8 |
| 23. | a. | Classify the different forms of data in Big data analytics. | CO5 / An | 3 |
|  | b. | Explain the Data processing cycle in detail. | CO5 / U | 9 |
|  |  | Compulsory: |  |  |
| 24. |  | Analyze the different types of Cloud computing services. Explain the benefits of cloud computing. | CO5 / An | 12 |

Reg. No. _____________

End Semester Examination – May / June – 2022

| # Code : | # 19EC2001 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name: | # ELECTRONICS FOR INTELLIGENT MACHINES | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Interpret the purpose of AI in industry 4.0. | CO1 / U | 1 |
| 2. | Identify the first nurse humanoid. | CO1 / R | 1 |
| 3. | Compare Industry 1.0 and Industry 2.0 | CO2 / A | 1 |
| 4. | Infer the unfair advantage of KFC. | CO6 / U | 1 |
| 5. | Identify the IoT layer which is connected to the mobile phone in smart healthcare. | CO3 / A | 1 |
| 6. | Define m2m communication. | CO3 / R | 1 |
| 7. | Name an application of the IR sensor module. | CO4 / R | 1 |
| 8. | Who coined the term Internet of things? | CO4 / A | 1 |
| 9. | Show an example of Big data. | CO5 / R | 1 |
| 10. | Mention an example of Artificial Narrow Intelligence. | CO5 / U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Explain the purpose of AI in the smart healthcare system with an example. | CO2 / U | 3 |
| 12. | List the USP of Adidas and McDonald. | CO2 / R | 3 |
| 13. | List any 2 I/O devices connected to a smart phone. | CO4 / An | 3 |
| 14. | Differentiate Industry 3.0 and Industry 4.0. | CO4 / An | 3 |
| 15. | Demonstrate with an example how data is converted to information. | CO5 / U | 3 |
| 16. | Distinguish the earlier taxi system and Ola system. | CO1 / An | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. |  | Construct the block diagram of industry 4.0 and discuss the functions of each block. | CO1 / U | 12 |
| 18. | a. | Construct the block diagram of an Intelligent machine and explain the functioning. | CO2 / A | 8 |
|  | b. | Compute the five-stage design thinking model proposed by Hasso-Plattner Institute of design. | CO6 / U | 4 |

| 19. | a. | Explain the 4 layer architecture of IoT and discuss the function of each layer with an application. | CO3 / U | 6 |
| --- | --- | --- | --- | --- |
|  | b. | Sketch the 1- page Lean canvas model for the OLA cab booking. | CO6 / A | 6 |
| 20. | a. | A linear temperature sensor with a range of 0 degrees to 100 degrees gives an output of 0 mv to 10mv. A 5v ADC (8 bit) is used to get the digital value out. Evaluate the digital output for a temperature of 25 degrees. | CO4 / A | 3 |
|  | b. | Explain the GSM interface with a neat sketch. | CO4 / U | 9 |
| 21. | a. | Analyze the advantages and disadvantages of industry 4.0 standards. | CO1 / An | 6 |
|  | b. | Model the working of the OYO room booking system. | CO1 / A | 6 |
| 22. | a. | Explain the working of the GPS sensor. | CO4 / U | 4 |
|  | b. | Sketch the necessity and the advantages of a smart city with its functional blocks. | CO5 / R | 8 |
| 23. | a. | Classify the different forms of data and their benefits in Big data analytics. | CO5 / An | 3 |
|  | b. | Explain the Data processing cycle in detail. | CO5 / U | 9 |
|  |  | Compulsory: |  |  |
| 24. |  | Analyze the different types of Cloud computing services. | CO5 / An | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | To compare the different industry standards |
| CO2 | To articulate the structure of an Intelligent machine |
| CO3 | To illustrate the m2m interface needed in intelligent machining |
| CO4 | To be able to categorize the sensors for various intelligent machines |
| CO5 | To assess the data requirements for cloud storage |
| CO6 | To be able to grade various types of Intelligent machines |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 1 | 13 | 6 | 9 |  |  | 29 |
| CO2 | 3 | 3 | 9 |  |  |  | 15 |
| CO3 | 1 | 6 | 1 |  |  |  | 8 |
| CO4 | 1 | 13 | 4 | 6 |  |  | 24 |
| CO5 | 9 | 13 |  | 15 |  |  | 37 |
| CO6 |  | 5 | 6 |  |  |  | 11 |
| Total Marks |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 19EC2020 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name: | # ANALYSIS AND DESIGN OF DIGITAL IC | # Max. Marks: | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | Name the minimum voltage required to turn ON the transistor. | CO1 | R | 1 |
| 2. | Name the region of operation of MOS transistor in which Ids is independent of Vds. | CO1 | R | 1 |
| 3. | What is the gate voltage to be applied to NMOS to turn ON for MOSFET to behave as a switch? | CO2 | R | 1 |
| 4. | Name the type of device scaling in which the electric field should remain constant during scaling. | CO2 | R | 1 |
| 5. | What type of MOS transistors are present in the PUN of static CMOS design? | CO3 | R | 1 |
| 6. | Name the type of biasing of pn diode in MOS transistor for capacitance to appear between drain and bulk. | CO3 | R | 1 |
| 7. | What type of input transitions are allowed at the inputs when cascading dynamic gates? | CO4 | R | 1 |
| 8. | Name the type of logic that is always implemented in Domino logic. | CO4 | R | 1 |
| 9. | Name the level-sensitive storage device. | CO5 | R | 1 |
| 10. | Name the design technique which is used to increase the speed of operation of data paths in digital processors. | CO5 | R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11. | Classify three levels of MOSFET models based on their physics. |  | CO1 |  | U |  | 3 |  |
| 12. | Justify the statement “NMOS transistors pass strong zero but not strong one”. |  | CO2 |  | E |  | 3 |  |
| 13. | Explain dynamic power dissipation with a mathematical equation |  | CO3 |  | U |  | 3 |  |
| 14. | Distinguish between combinational and sequential logic circuits. |  | CO4 |  | An |  | 3 |  |
| 15. | Show the schematic of CMOS Schmitt trigger. |  | CO5 |  | U |  | 3 |  |
| 16. | Distinguish between mesochronous and plesiochronous signals. |  | CO6 |  | An |  | 3 |  |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |  |  |  |
| 17. | a. | Explain different regions of operation of the NMOS transistor with a neat diagram and equation. Obtain the characteristic curve for the same. |  | CO1 |  | U |  | 12 |

| 18. | a. | Estimate the CMOS gate transistor sizing for the function F= A+B, when (W/L)p=10 and (W/L)n=15. | CO3 | E | 6 |
| --- | --- | --- | --- | --- | --- |
|  | b. | Analyze the voltage transfer curve of the CMOS inverter and indicate the status of PMOS and NMOS transistors in different regions | CO3 | An | 6 |
| 19. | a. | Analyze the different issues that occur in dynamic CMOS logic design and provide suitable solutions for the same with a neat diagram. | CO4 | An | 12 |
| 20. | a. | Design a 2 input OR gate using a true single-phase clocked approach and explain its operation. | CO4 | C | 12 |
| 21. | a. | Design a pipelined datapath for the computation of log (la+bl) and explain its operation by comparing it with the conventional design. | CO5 | C | 12 |
| 22. | a. | Analyze the impact of channel length modulation second-order effect in the drain current output of the MOS transistor with necessary diagrams and equations. | CO2 | An | 12 |
| 23. | a. | Build and explain the operation of the positive edge-triggered register with a neat timing diagram. | CO5 | A | 12 |
|  |  | Compulsory: |  |  |  |
| 24. | a. | Explain the operation of PLL and its application with a neat block diagram. | CO6 | U | 12 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 19EC2024 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # MACHINE LEARNING TECHNIQUES | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | State the different types of machine learning algorithms. | CO1/R | 1 |
| 2. | When to use regression. | CO1/R | 1 |
| 3. | Discuss the advantages of SVM. | CO1/U | 1 |
| 4. | What is an RBF network? | CO2/U | 1 |
| 5. | Write the Hopfield neural network. | CO3/R | 1 |
| 6. | How does ensemble of classifiers improve the accuracy of a classification system? | CO3/R | 1 |
| 7. | Explain about natural language processing. | CO4/R | 1 |
| 8. | Infer the applications of machine learning for pattern recognition. | CO5/Ap | 1 |
| 9. | Construct a Bayesian network simulating a teacher entering and leaving a class. | CO5/Ap | 1 |
| 10. | Distinguish over fitting and under fitting. | CO6/U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | What are applications of machine learning? | CO1/R | 3 |
| 12. | Explain the evaluation process of learning algorithm. | CO1/R | 3 |
| 13. | Write a short note on logistic regression. | CO1/U | 3 |
| 14. | Brief about kernel functions in SVM. | CO2/U | 3 |
| 15. | Difference between perceptron and neuron in ANN. | CO3/R | 3 |
| 16. | Explain any one use case in biometric applications. | CO3/R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | Compare the specialties of supervised learning with unsupervised learning. | CO1 /U | 4 |
|  | b. | Explain the unsupervised way of machine learning with an example. | CO2 /An | 8 |
| 18. | a. | Summarize on the terminologies in hypothesis space of inductive learning and deduce on the bias errors. | CO1/ E | 12 |
| 19. | a. | Explain the concept of perceptron with neat diagram and dissect its training rule. | CO3 /An | 8 |
|  | b. | Appraise the condition in which perceptron rule fails and recommend a method to overcome it. | CO4 / E | 4 |

| 20. | a. | Discuss on the iterative method of predicting the hypothesis function for linear regression using LMS algorithm. | CO5/U | 12 |
| --- | --- | --- | --- | --- |
| 21. | a. | Illustrate the structural representation of decision tree with an example. | CO5 / U | 8 |
|  | b. | Describe on logistic regression used for classification problems. | CO6 /R | 4 |
| 22. | a. | Paraphrase on support vector machines and cites its advantages and disadvantages. | CO6/U | 9 |
|  | b. | Write short notes on SMO algorithms. | CO6/U | 3 |
| 23. | a. | Explain the Hopfield neural network with an algorithm. | CO6/U | 6 |
|  | b. | Illustrate the Kohonen neural network with an example. | CO6/U | 6 |
|  |  | Compulsory: |  |  |
| 24. | a. | Infer the error introduced due to smaller test set and dissect on k-fold cross validation. | CO2 /An | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the techniques, mathematical concepts of machine learning |
| CO2 | Select the appropriate machine learning algorithm to solve real time problems |
| CO3 | Compare the data and efficiently execute the algorithm to solve the problems |
| CO4 | Analyze and compare the results to different machine learning algorithms |
| CO5 | Comprehend the statistical techniques to analyze the results |
| CO6 | Acquire knowledge about the artificial neural networks |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 8 | 8 | - | - | 12 | - | 28 |
| CO2 | - | 4 | - | 20 | - | - | 24 |
| CO3 | 8 | - | - | 8 | - | - | 16 |
| CO4 | 1 | - | - | - | 4 | - | 5 |
| CO5 | - | 20 | 2 | - | - | - | 22 |
| CO6 | 4 | 25 | - | - | - | - | 29 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 19EC2028 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # FUNDAMENTALS OF WIRELESS # COMMUNICATION | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | Name the measurement equipment used for path-loss modeling. | CO2 | R | 1 |
| 2. | List any two features of Zigbee radio. | CO3 | R | 1 |
| 3. | What is the unit of channel capacity? | CO6 | R | 1 |
| 4. | Define inter-system handoff. | CO1 | R | 1 |
| 5. | List any two mechanism of propagation. | CO2 | R | 1 |
| 6. | List any two outdoor propagation model. | CO2 | R | 1 |
| 7. | Define outage probability. | CO1 | R | 1 |
| 8. | Define Doppler effect. | CO1 | R | 1 |
| 9. | Relate bandwidth and signal-to-noise ratio. | CO1 | U | 1 |
| 10. | Name the digital modulation technique used in Bluetooth radio. | CO3 | R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Interpret the effect on coverage and interference of mobile link by decreasing the transmit power level. | CO1 | U | 3 |
| 12. | Compare soft handoff and hard handoff. | CO1 | U | 3 |
| 13. | Explain shadow fading. | CO2 | U | 3 |
| 14. | Outline the characteristics of Impulse response model of wireless channel. | CO4 | U | 3 |
| 15. | List any three features of flat fading channel. | CO6 | R | 3 |
| 16. | What are the advantages of cyclic prefix code in OFDM modulation? | CO5 | R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | a. | Summarize the technical challenges faced by the wireless communication system. | CO1 | U | 6 |
|  | b. | With a neat block diagram, explain the cellular network architecture. | CO1 | U | 6 |
| 18. | a. | Explain the two types of channel assignments strategies and its importance in mobile communications. | CO1 | U | 12 |

| 19. |  | Make use of the mathematical expression for free space propagation model and explain the parameters of mobile multipath channel. | CO2 | A | 12 |
| --- | --- | --- | --- | --- | --- |
| 20. |  | Explain the time variant two ray path loss model of a wireless propagation channel. | CO2 | U | 12 |
| 21. |  | Outline the effects of multipath propagation in a wireless environment. | CO4 | U | 12 |
| 22. | a. | Capacity in the AWGN channel is given by C = B log2(1 + P/N0B), where P is the received signal power, B is the signal bandwidth, and N0/2 is the noise PSD. Show that the capacity in the limit of infinite bandwidth B → ∞ as a function of P is finite. | CO6 | R | 6 |
|  | b. | Consider a wireless channel where power falls off with distance following the formula Pr(d) = Pt(d0/d)3 for d0 = 10 m. Assume the channel has bandwidth B = 30 kHz and AWGN with noise PSD N0/2, where N0= 10-9 W/Hz. For a transmit power of 1 W, find the capacity of this channel for a transmit–receive distance of 100 m and 1 km. | CO6 | R | 6 |
| 23. | a. | With neat block diagram, explain the Shannon’s channel capacity with channel state information known at transmitter and receiver. | CO6 | E | 6 |
|  | b. | Discuss the Shannon’s channel capacity in an AWGN channel. | CO6 | C | 6 |
|  |  | Compulsory: |  |  |  |
| 24. |  | With a neat block diagram, explain the orthogonal frequency division multiplexing implementation with IFFT/FFT techniques? | CO5 | U | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Explain the concepts of wireless communication. |
| CO2 | Interpret the various propagation methods and channel models. |
| CO3 | Identify various transceivers and its multiple access schemes. |
| CO4 | Model multichannel response |
| CO5 | Develop design of wireless system |
| CO6 | Analyze wireless channel capacity |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 3 | 31 | - | - | - | - | 34 |
| CO2 | 3 | 15 | 12 | - | - | - | 30 |
| CO3 | 2 | - | - | - | - | - | 2 |
| CO4 | - | 15 | - | - | - | - | 15 |
| CO5 | - | 15 | - | - | - | - | 15 |
| CO6 | 13 | 3 | - | - | 6 | 6 | 28 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 19EC2029 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # DATA SCIENCE AND DATA ANALYTICS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | Relate machine learning and artificial intelligence. | CO1 | R | 1 |
| 2. | Find the phase of data science process that handles the inconsistencies in data like missing value, redundant data, incorrect data format. | CO1 | R | 1 |
| 3. | Select a popular API which has a collection of many songs. | CO2 | A | 1 |
| 4. | Identify a service provider API which is used by a popular game “Pokemon go” to find the location information. | CO2 | A | 1 |
| 5. | Recall any two-performance metrics related to regression problem. | CO3 | R | 1 |
| 6. | Compare supervised and unsupervised machine learning algorithms. | CO3 | U | 1 |
| 7. | Choose a python library package to handle multidimensional array. | CO6 | R | 1 |
| 8. | Name a python library package to handle data frame. | CO6 | R | 1 |
| 9. | List any two retinal variables. | CO5 | R | 1 |
| 10. | Define data visualization. | CO5 | R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Explain exploratory data analysis. | CO1 | U | 3 |
| 12. | List the four categories of NoSQL database. | CO2 | R | 3 |
| 13. | Estimate the sensitivity and precision of the machine learning model whose TP=464, TN=672, FP=161, FN=41. | CO3 | A | 3 |
| 14. | Summarize the Microsoft power BI. | CO5 | U | 3 |
| 15. | Classify the different types of SQL commands. | CO6 | U | 3 |
| 16. | List any three big data visualization tools. | CO5 | R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. |  | Interpret the pipeline of data science process with necessary examples. | CO1 | U | 12 |
| 18. |  | Discuss the problems associated with data warehouse and data lake to store big data and identify the practical solutions offered by cloud services to store and handle big data. | CO2 | A | 12 |

| 19. |  | Consider five datapoints belong to positive class, five datapoints belong to negative class and an outlier (negative data point present in the positive class). Utilize logistic regression algorithm to find the best plane which separates the data points with high accuracy. Also derive the objective function of logistic regression which is not affected by the outliers. | CO3 | A | 12 |
| --- | --- | --- | --- | --- | --- |
| 20. |  | Summarize the visual encoding variables with necessary illustrations. | CO5 | U | 12 |
| 21. | a. | List the advantages of Python programming. | CO6 | R | 6 |
|  | b. | Explain the Python datatypes with suitable examples. | CO6 | U | 6 |
| 22. |  | Outline the data science toolkit which are more popular in the field of business intelligence and mention the salient features of each toolkit. | CO1 | U | 12 |
| 23. |  | Apply Naïve Bayes theorem and predict the probability of players will play tennis if weather is sunny for the given dataset. | CO3 | A | 12 |
|  |  | Compulsory: |  |  |  |
| 24. |  | Identify the data collection strategies to be carried out by the data analyst and discuss the practical advantages and disadvantages of the data collection strategies. | CO4 | A | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the key concepts in data science, its applications and the toolkit used by data scientists |
| CO2 | Realize how data is collected, managed and stored for data science |
| CO3 | Apply various machine learning techniques in real-world applications |
| CO4 | Implement data collection and management |
| CO5 | Apply visualization tools for data visualization |
| CO6 | Possess the required knowledge and expertise to become a proficient data scientist |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 2 | 27 | - | - | - | - | 29 |
| CO2 | 3 | 0 | 14 | - | - | - | 17 |
| CO3 | 1 | 1 | 27 | - | - | - | 29 |
| CO4 | - | - | 12 | - | - | - | 12 |
| CO5 | 5 | 15 | - | - | - | - | 20 |
| CO6 | 8 | 9 | - | - | - | - | 17 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 19EC2031 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # IOT EDGE COMPUTING | # Max. Marks : | # 100 |

| Q. No. | Questions |  | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. |  | Define bandwidth. | CO1 / R | 1 |
| 2. |  | Name any two IoT gaming devices. | CO1 / R | 1 |
| 3. |  | Interpret MQTT protocol. | CO2 / U | 1 |
| 4. |  | Identify the purpose of IIoT. | CO1 / A | 1 |
| 5. |  | What is CDN? | CO2 / R | 1 |
| 6. |  | What is the process in database audit? | CO2 / R | 1 |
| 7. |  | Define infant mortality in the lifecycle of a product. | CO4 / R | 1 |
| 8. |  | What is a glitch attack? | CO4 / R | 1 |
| 9. |  | Name any two IoT processors. | CO5 / R | 1 |
| 10. |  | What is V2V communication? | CO6 / R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | What is an IoT gateway? | CO1 / U | 3 |
| 12. | Compare edge and Fog computing. | CO1 / An | 3 |
| 13. | Identify the Non 3GPP standards. | CO2 / A | 3 |
| 14. | List any two best practices for digital security control. | CO4 / U | 3 |
| 15. | Identify the OS on which Raspberry Pi and Arduino work on. | CO5 / A | 3 |
| 16. | List the blocks of smart health care system. | CO6 / An | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17 | a. | Explain the IoT Edge architecture with a neat diagram. | CO1 / U | 8 |
|  | b. | List any two field protocols and their advantages. | CO1 / U | 4 |
| 18. | a. | Classify the 3GPP standards. | CO2 / An | 6 |
|  | b. | Identify the sensors and actuators used in IoT. | CO1 / U | 6 |
| 19. | a. | Identify the purpose and advantages of CDN. | CO2 / A | 4 |
|  | b. | List the various cloud service models with examples. | CO2 / An | 8 |
| 20. | a. | Classify the different phases of computing paradigms. | CO2 / U | 7 |
|  | b. | List the different types of cloud and their purpose. | CO2 / An | 5 |

| 21. | a. | Explain the functioning of Raspberry Pi board with a neat diagram. | CO5 / U | 8 |
| --- | --- | --- | --- | --- |
|  | b. | List the features of Arduino board. | CO5 / An | 4 |
| 22. | a. | List the stages in the lifecycle of an edge device. | CO4 / U | 4 |
|  | b. | Develop the IAM framework for an industry. | CO4 / A | 8 |
| 23. | a. | Compare cyber security attacks and hardware attacks. | CO4 / An | 8 |
|  | b. | What is dynamic QoS? | CO4 / U | 4 |
|  |  | Compulsory: |  |  |
| 24. |  | Explain with a diagram the complete working of Home automation system. | CO6 / U | 12 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 19EC2036 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # NEURAL NETWORKS AND DEEP LEARNING | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | Compare artificial neural network and biological neural network with any two technical aspects. | CO1 | U | 1 |
| 2. | List the characteristics of artificial neural networks. | CO1 | R | 1 |
| 3. | Outline the need for activation functions in artificial neural networks. | CO3 | U | 1 |
| 4. | Summarize the advantages of supervised learning methodologies. | CO2 | U | 1 |
| 5. | List the drawbacks of back propagation neural networks. | CO2 | R | 1 |
| 6. | Name the training methodology used in perceptron. | CO2 | R | 1 |
| 7. | Summarize the specific features of Hopfield neural network. | CO3 | U | 1 |
| 8. | Name the different passes used in the training process of back propagation neural network. | CO2 | R | 1 |
| 9. | Compare conventional machine learning techniques and deep learning techniques. | CO5 | U | 1 |
| 10. | List few pretrained models used for pattern recognition applications. | CO4 | R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Illustrate the operation of a biological neuron with neat diagram. | CO1 | U | 3 |
| 12. | Distinguish between supervised and unsupervised learning methodologies. | CO3 | An | 3 |
| 13. | Determine a solution for the X-OR problem of perceptron. Include graphical illustrations wherever necessary. | CO2 | E | 3 |
| 14. | Summarize the local minima problem in back propagation neural network. | CO2 | U | 3 |
| 15. | Compare the AlexNet and LeNet pretrained models with necessary diagrams. | CO6 | U | 3 |
| 16. | Outline the necessity for maxpooling layer in convolution neural networks. | CO5 | U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. |  | Illustrate the process of electrical communication and chemical communication inside human neuron with neat diagrams. | CO1 | U | 12 |
| 18. | a. | Summarize the functions of artificial neuron with neat diagrams. Include mathematical equations wherever necessary. | CO1 | U | 6 |
|  | b. | Distinguish between single layer networks and multi-layer networks with neat diagrams. | CO3 | An | 6 |
| 19. |  | Explain the perceptron model with neat architecture and training algorithms. Include mathematical equations wherever necessary. | CO2 | U | 12 |
| 20. |  | Illustrate the architecture and training algorithm of back propagation neural network with neat diagrams. Support your answer with necessary mathematical equations. | CO3 | U | 12 |
| 21. |  | Outline the technical aspects of the following pretrained models. (a) AlexNet, (b) ResNet, and (c) VGGNet. | CO6 | U | 12 |
| 22. | a. | Summarize the linear separable problem of perceptron with neat graphical illustrations. | CO3 | U | 6 |
|  | b. | Determine the weight values of a 2-input OR gate using perceptron after 2 iterations with initial weight values of 0 and 1. The learning rate is 0.5 | CO4 | E | 6 |
| 23. |  | Explain the different types of activation functions with neat graphical illustrations. Include mathematical equations wherever necessary. | CO2 | U | 12 |
|  |  | Compulsory: |  |  |  |
| 24. |  | Illustrate the functions of convolutional neural network with different layers of the architecture. Include neat diagrams and mathematical equations wherever necessary. | CO5 | U | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Compare and comprehend the functioning of human brain and ANN. |
| CO2 | Gain an understanding about training methodologies of neural networks. |
| CO3 | Summarize the pros and cons of different single layer ANN. |
| CO4 | Apply artificial neural networks for solving engineering problems. |
| CO5 | Outline the basic concepts and applications of deep learning. |
| CO6 | Make use of different Deep networks for real time applications. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 1 | 22 | - | - | - | - | 23 |
| CO2 | 3 | 28 | - | - | 3 | - | 34 |
| CO3 | - | 20 | - | 9 | - | - | 29 |
| CO4 | 1 | - | - | - | 6 | - | 7 |
| CO5 | - | 16 | - | - | - | - | 16 |
| CO6 | - | 15 | - | - | - | - | 15 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 19EC2038 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # IOT BASED DATA ACQUISITION SYSTEMS AND PROTOCOLS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | Find the layer in IoT Reference model which deals with the things. | CO1 | R | 1 |
| 2. | Recall the layer in IoTWF Reference model which evaluates and reformats the data for processing at higher levels. | CO1 | R | 1 |
| 3. | Interpret the component which makes the physical change, after a sensor communicates with a controller. | CO1 | U | 1 |
| 4. | Name the layer represents edge computing. | CO2 | R | 1 |
| 5. | Mention the model which uses application layer gateways to translate non-IP to IP layers. | CO2 | R | 1 |
| 6. | Show the encryption techniques used in IoT access technologies. | CO3 | U | 1 |
| 7. | Recall the layer in which LoRaWAN is built on top of LoRa modulation. | CO3 | R | 1 |
| 8. | Name the function which represents a collection of virtual channels with timing and triggering options. | CO4 | R | 1 |
| 9. | Give an example for structured data. | CO5 | R | 1 |
| 10. | List out social cites involved in participatory sensing. | CO6 | R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Outline simplified IoT architecture and its sections. | CO1 | U | 3 |
| 12. | Summarize the limitations of smart objects in wireless sensor networks. | CO2 | U | 3 |
| 13. | Explain the security feature of IEEE 1901.2a access technology. | CO3 | U | 3 |
| 14. | Demonstrate the working of CoAP IoT application layer protocol. | CO4 | U | 3 |
| 15. | Compare the properties of GPIB and RS232. | CO5 | U | 3 |
| 16. | Name few applications of IoT. | CO6 | R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. |  | Explain the IoT World Forum (IoTWF) Standardized Architecture with a neat layered representation. | CO1 | U | 12 |
| 18. | a. | Relate how range is an important communication criterion used for connecting smart objects. | CO2 | U | 6 |
|  | b. | Summarize the categories of sensors used in IoT applications. | CO2 | U | 6 |
| 19. | a. | Evaluate the working of LoRaWAN for IoT applications. | CO3 | E | 6 |
|  | b. | Explain the characteristics of LTE-M as access technology. | CO3 | U | 6 |
| 20. | a. | Identify the key advantages of Internet Protocol (IP) in Internet and IoT applications. | CO3 | A | 6 |
|  | b. | List the model used for IP with respected to constrained nature of nodes. | CO3 | An | 6 |
| 21. | a. | Illustrate the various counter terminals and its applications. | CO4 | U | 8 |
|  | b. | List out the characteristics of RS232. | CO4 | An | 4 |
| 22. |  | Determine how data rate, bandwidth, throughput, latency, response time, overhead and payload are used as a performance metrices of IoT applications. | CO5 | E | 12 |
| 23. |  | Summarize the working and IP adaptation done for SCADA. | CO5 | U | 12 |
|  |  | Compulsory: |  |  |  |
| 24. | a. | Discuss how IoT helps in Asset management. | CO6 | C | 6 |
|  | b. | Explain the importance and features of cloud in IoT (Any 2 cloud) | CO6 | E | 6 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Acquire knowledge about the various network architectures of IoT |
| CO2 | Understand the basic concepts of sensors and actuators |
| CO3 | Gain knowledge in various network protocols |
| CO4 | Gain knowledge in data acquisition methods and instruments |
| CO5 | Articulate the various applications of IoTnetworked systems |
| CO6 | Apply the acquired knowledge to develop an IoT networked application |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 2 | 16 |  |  | - | - | 18 |
| CO2 | 2 | 15 |  |  |  |  | 17 |
| CO3 | 1 | 10 | 6 | 6 | 6 | - | 29 |
| CO4 | 1 | 11 |  | 4 |  |  | 16 |
| CO5 | 1 | 15 |  | - | 12 | - | 28 |
| CO6 | 4 |  |  | - | 6 | 6 | 16 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 19EC2043 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # TESTING OF VLSI CIRCUITS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | Name the process to verify the behavior, performance, and reliability of a device or system against designed specifications. | CO1 | R | 1 |
| 2. | Identify the term that tells whether a good system will work after some time. | CO1 | A | 1 |
| 3. | Identify the fault that causes the logic value at a point in a circuit to become opposite to the specified value. | CO2 | A | 1 |
| 4. | Find an input sequence for a machine, if the machines response to the sequence is always sufficient to determine its final state. | CO2 | R | 1 |
| 5. | Identify the parameter that describes the input/output electrical behavior. | CO2 | A | 1 |
| 6. | List an input sequence which when applied to the machine will produce a different output sequence for each choice of initial state. | CO2 | R | 1 |
| 7. | The circuit is first simulated in the true-value mode for all vectors and primary output values are saved in a file.Identify the simulation process. | CO3 | A | 1 |
| 8. | Select the new test generation algorithm for combinational circuits which is more efficient in generating error correction logic. | CO4 | R | 1 |
| 9. | Select the five-valued logic in PODEM algorithm. | CO4 | R | 1 |
| 10. | Find the vector that selects the collection of uncertainties in the testing process. | CO6 | R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | What is Bridging Fault? | CO1 | R | 3 |
| 12. | Show how PODEM is efficient than the D algorithm. | CO2 | R | 3 |
| 13. | Outline the functions of Fault Simulator. | CO3 | U | 3 |
| 14. | Define Controllability. | CO4 | R | 3 |
| 15. | Find the disadvantage in multiplexer based testing. | CO5 | R | 3 |
| 16. | Define BIST. | CO6 | R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | a. | For the following CMOS circuit, if there is a short between Z1 and Z2find the output of Z1 and Z2 with short and without short.. | CO1 | R | 5 |
|  | b. | Determine the test pattern for the given combinational circuit, to detect the presence of fault using D-algorithm. | CO2 | E | 7 |
| 18. | a. | Find the Boolean difference with respectto X3 for the logic circuit shown below. | CO2 | U | 7 |
|  | b. | Model all the faults in the following CMOS circuit for the output function Z= (A+B) (C+D) .EF | CO1 | A | 5 |

| 19. | a. | Determine the test vector for the following diagram Path Sensitization Method. | CO2 | E | 8 |
| --- | --- | --- | --- | --- | --- |
|  | b. | Explain in detail the Bridging Faults with neat diagrams. | CO1 | U | 4 |
| 20. | a. | For the given figure find test vector using path sensitization method. | CO2 | R | 8 |
|  | b. | Evaluate Parallel fault simulation and find the faults in the following figure. | CO3 | E | 4 |

| 21. |  | With Convergence and Divergence, illustrate the concepts of Concurrent fault simulation. | CO3 | U | 12 |
| --- | --- | --- | --- | --- | --- |
| 22. | a. | Explain in detail the Random Access Scan in Design for Testability. | CO5 | U | 6 |
|  | b. | Explain in detail the test points in DFT technique. | CO4 | U | 6 |
| 23. |  | Demonstrate the Boundary Scan Standards with necessary diagrams. | CO5 | U | 12 |
|  |  | Compulsory: |  |  |  |
| 24. | a. | Outline the BIST Concepts including the Hardcore and Levels of Test. | CO6 | U | 4 |
|  | b. | Summarize the Test-pattern Generation for BIST with necessary diagrams. | CO6 | U | 8 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Gain basic knowledge in VLSI testing. |
| CO2 | Particular testing method for a Device under Test (DUT). |
| CO3 | Categorize various fault simulation techniques. |
| CO4 | Illustrate the Design for Testability (DFT) concepts. |
| CO5 | Articulate Scan Based Design concepts. |
| CO6 | Demonstrate the various BIST architectures. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 9 | 4 | 6 | - | - | - | 19 |
| CO2 | 13 | 7 | 2 | - | 15 | - | 37 |
| CO3 | - | 15 | 1 | - | 4 | -- | 20 |
| CO4 | 5 | 6 | - | - | - | - | 11 |
| CO5 | 3 | 18 | - | - | - | - | 21 |
| CO6 | 4 | 12 | - | - | - | - | 16 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 19EC2046 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # SPEECH PROCESSING | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | Assume that a 500 Hz tone and a 2000 Hz tone of the same intensity are played simultaneously. Identify the tone that would be more audible. | CO1 | An | 1 |
| 2. | Identify two English words that contain nasal sounds. | CO1 | A | 1 |
| 3. | Recall and draw the three-level centre clipping function. | CO2 | R | 1 |
| 4. | List a disadvantage of using the short-time energy function. | CO2 | R | 1 |
| 5. | Determine the length of X(n,k) along n, if X(n,k) is the discrete short-time Fourier transform of a signal x(n) of length400 samples and if the window function used, w(n) has a length of 100 samples. | CO3 | E | 1 |
| 6. | What is residual error in linear prediction? | CO3 | R | 1 |
| 7. | Identify the type of data distribution depicted by the following probability density function. | CO4 | A | 1 |
| 8. | Illustrate a left-right hidden Markov model pictorially. | CO4 | U | 1 |
| 9. | Identify the module of the text-to-speech system that interprets and expands “St.” as “Street” or “Saint”. | CO5 | A | 1 |
| 10. | List two recent deep neural networks-based automatic speech recognition engines. | CO5 | R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Compare and contrast voiced and unvoiced sounds. | CO1 | U | 3 |
| 12. | Summarize the properties of the short-time autocorrelation function computed from voiced sounds. | CO2 | U | 3 |
| 13. | Outline the process by which a cepstrum is derived from a speech signal. | CO3 | U | 3 |
| 14. | Distinguish between supervised, unsupervised and reinforcement learning. | CO4 | An | 3 |

| 15. | Recall the advantages and disadvantages of hidden Markov model-based text-to-speech system. |  | CO5 |  | R |  | 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16. | Define natural language understanding and list two of its applications. |  | CO6 |  | R |  | 3 |  |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |  |  |  |
| 17. |  | Explain the source-system model of speech production. |  | CO1 |  | U |  | 12 |
| 18. |  | Illustrate speech vs silence discrimination with a suitable example and specify the problem creators that can affect the accuracy of this task. |  | CO2 |  | U |  | 12 |
| 19. | a. | Explain the estimation of pitch period using the simplified inverse filter tracking algorithm. |  | CO3 |  | U |  | 8 |
|  | b. | Outline the process by which formant frequencies are estimated using linear predictive analysis. |  | CO3 |  | U |  | 4 |
| 20. | a. | Why are conventional feedforward neural networks unsuitable for modelling sequential data? |  | CO4 |  | R |  | 2 |
|  | b. | Explain in detail about recurrent neural networks and their variants. |  | CO4 |  | U |  | 10 |
| 21. |  | Explain unit selection synthesis and outline its advantages and disadvantages. |  | CO5 |  | U |  | 12 |
| 22. |  | Illustrate the basic concepts of linear prediction with equations. |  | CO3 |  | U |  | 12 |
| 23. | a. | Outline the k means clustering algorithm. |  | CO4 |  | U |  | 5 |
|  | b. | Illustrate hidden Markov models with an example. List the elements of the model and the three basic problems. |  | CO4 |  | U |  | 7 |
|  |  | Compulsory: |  |  |  |  |  |  |
| 24. | a. | Explain the three different speaker recognition tasks. |  | CO6 |  | U |  | 6 |
|  | b. | Compare and contrast text-dependent and text-independent speaker recognition. |  | CO6 |  | U |  | 2 |
|  | c. | Recall the four categories of noise and name the method used to remove/reduce these noises. |  | CO6 |  | R |  | 4 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Model speech production system and describe the fundamentals of speech |
| CO2 | Extract features from speech in time-domain |
| CO3 | Extract features from speech in frequency-domain |
| CO4 | Model speech signals using statistical modelling methods |
| CO5 | Develop speech synthesis and recognition systems |
| CO6 | Develop speaker and language identification system |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | - | 15 | 1 | 1 | - | - | 17 |
| CO2 | 2 | 15 | - | - | - | - | 17 |
| CO3 | 1 | 27 | - | - | 1 | - | 29 |
| CO4 | 2 | 23 | 1 | 3 | - | - | 29 |
| CO5 | 4 | 12 | 1 | - | - | - | 17 |
| CO6 | 7 | 8 | - | - | - | - | 15 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 19EC2050 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # SENSORS FOR IOT APPLICATIONS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Recall the definition of IoT. | CO 1/ R | 1 |
| 2. | List any two characteristics of an IoT system. | CO 1/ R | 1 |
| 3. | Relate data sovereignty in the context of security. | CO 2/ U | 1 |
| 4. | Interpret the concept of Eavesdropping in IoT. | CO 2/ U | 1 |
| 5. | Outline your perception about transducer. | CO 3/ U | 1 |
| 6. | List any two sensors for measuring motion and vibration. | CO 3/ R | 1 |
| 7. | List any two characteristics of wireless sensor networks. | CO 4/ R | 1 |
| 8. | Interpret the concept of Ad Hoc wireless networks. | CO 5/ U | 1 |
| 9. | Relate the concept of Flooding in sensor networks. | CO 5/ U | 1 |
| 10. | List any two applications of an IoT system. | CO 6/ R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Compare an IoT system with M2M. | CO 1/ U | 3 |
| 12. | Explain the security mechanisms in the IP architecture. | CO 2/ E | 3 |
| 13. | Compare the functionalities of sensors and actuators. | CO 3/ U | 3 |
| 14. | List the components of a smart sensor. | CO 4/ R | 3 |
| 15. | Explain the conventional data-dissemination schemes in wireless sensor networks. | CO 5/ E | 3 |
| 16. | Interpret the concept of Smart Payments methodology. | CO 6/ U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | Classify the various communication patterns in IoT with necessary diagrams in detail. | CO 1/ U | 12 |
| 18. | a. | Elaborate on the various IoT attacks, types and countermeasures in detail. | CO 2/An | 12 |
| 19. | a. | Explain the architecture Smart Sensor and Actuator in detail with necessary diagrams. | CO 3/ E | 12 |
| 20. | a. | Outline your perception about temperature sensors and proximity sensors. Explain the various types of temperature sensors and proximity sensors with neat diagrams. | CO 4/ U | 12 |

| 21. | a. | Construct the architecture of wireless sensor node and explain all the components in detail. | CO 5/ A | 12 |
| --- | --- | --- | --- | --- |
| 22. | a. | Elaborate on the various types of Access Restriction and control methodologies adopted in IoT systems. | CO 2/An | 12 |
| 23. | a. | Explain in detail about the various technologies involved in IoT development with necessary diagrams. | CO1 / U | 12 |
|  |  | Compulsory: |  |  |
| 24. | a. | Construct the architecture of Home Automation system and explain in detail. | CO 6/ A | 6 |
|  | b. | Outline your perception about Health and Fitness monitoring, explain the IoT system in detail for the same with appropriate diagrams. | CO 6/ U | 6 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the characteristics of IOT systems. |
| CO2 | Describe the security issues in IoT. |
| CO3 | Relate the various basic ideas behind sensors with respect to IOT. |
| CO4 | Choose appropriate sensors for measuring various parameters. |
| CO5 | Demonstrate the wireless sensor network in the aspects of IOT. |
| CO6 | Appraise sensors used in IOT for real life situations. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 2 | 15 | - | - | - | - | 17 |
| CO2 | - | 2 | - | 12 | 3 | - | 17 |
| CO3 | 1 | 4 | - | - | 12 | - | 17 |
| CO4 | 4 | 12 | - | - | - | - | 16 |
| CO5 | - | 2 | 12 | - | 3 | - | 17 |
| CO6 | 1 | 9 | 6 | - | - | - | 16 |
|  |  |  |  |  |  |  | 100 |

Reg. No. _____________

End Semester Examination – May / June – 2022

| # Code : | # 20EC1001 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # PYTHON PROGRAMMING | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Which one is the extension of the Python file? | CO1 / R | 1 |
| 2. | Who developed the Python language? | CO1 / R | 1 |
| 3. | Which operator is used to the power of numbers? | CO2 / R | 1 |
| 4. | What will be the output of the following Python code? str="hello" str[:2] | CO3 / R | 1 |
| 5. | How is a function declared in Python programming? | CO4 / R | 1 |
| 6. | What is the output of the add() function call? def add(a, b): return a+5, b+5 result = add(3, 2) print(result) | CO4 / R | 1 |
| 7. | Define polymorphism. | CO4 / R | 1 |
| 8. | Classify the various types of inheritance. | CO5 / U | 1 |
| 9. | What is the output of the following function call def fun1(num): return num + 25 fun1(5) print(num) | CO5 / R | 1 |
| 10. | What will be the output of the following Python code? class A(): def disp(self): print("A disp()") class B(A): pass obj = B() obj.disp() | CO6 / R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Demonstrate any three features of python programming | CO1 / U | 3 |
| 12. | What is tuple? Show the difference between list and tuple? | CO2 / U | 3 |
| 13. | What are the rules for writing an identifier? | CO3 / R | 3 |
| 14. | What is chained conditional statement? Give the syntax in python? | CO4 / R | 3 |
| 15. | Write the python code given an integer, perform the following conditional actions: - If is odd, print Weird - If is even and in the inclusive range of to , print Not Weird - If is even and in the inclusive range of to , print Weird - If is even and greater than , print Not Weird | CO5 / R | 3 |

| 16. | Write the python codegiven in the Gregorian calendar, three conditions are used to identify leap years: - The year can be evenly divided by 4, is a leap year, unless: - The year can be evenly divided by 100, it is NOT a leap year, unless: - The year is also evenly divisible by 400. Then it is a leap year. | CO6 / R | 3 |
| --- | --- | --- | --- |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | Identify the benefits of using Python. | CO1 / A | 4 |
|  | b. | Develop a python program to check which is greater or both are equal given two numbers. | CO1 / A | 8 |
| 18. | a. | Develop a python program to print ‘n’ natural numbers using while loop statement. | CO4 / A | 6 |
|  | b. | List the loop statement with suitable example program. | CO2 / An | 6 |
| 19. | a. | Develop a python program to prepare a list FRUITS={“APPLE”,MANGO”, “BANANA”} - Print all the items in the list - Find the length of the list - Update the second item of the list as “PINE APPLE” - Remove the last item in the list and print the list | CO3 / A | 6 |
|  | b. | Compare if else and nested if with sample code segments. | CO2 / An | 6 |
| 20. | a. | Compare and contrast to print the different sequence datatype with an example. | CO4 / An | 4 |
|  | b. | Explain various types of operators used in the python programming in detail. | CO5 / E | 8 |
| 21. | a. | List out the main differences between lists and tuples. | CO4 / An | 5 |
|  | b. | Build the code below and complete the following: i) Add two more people to the dictionary people. ii) Modify the name of person 1 from John to Jonathan. iii) Delete person 2 from the dictionary. iv) Show all key values pairs from the dictionary using a for loop. Be sure to nicely format the output of the all of the key value pairs. people = {1: {'name': 'John', 'age': '27', 'sex': 'Male'}, 2: {'name': 'Marie', 'age': '22', 'sex': 'Female'}} | CO6 /A | 7 |
| 22. | a. | Construct the two arguments taken by the open() function. | CO5 / A | 6 |
|  | b. | Create a Circle class and initialize it with radius. Make two methods get Area and get Circumference inside this class. | CO6 / C | 6 |
| 23. | a. | Examine in detail about various types of the data visualization Python plot with the illustration of visualization in details. | CO5/ An | 6 |
|  | b. | Justify polymorphism in python and give the types of polymorphism with an example. | CO5 / E | 6 |
|  |  | Compulsory: |  |  |
| 24. |  | Examine in detail about various types of inheritance of the Python with the illustration of employee database in details. | CO6 /An | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| sCO1 | Understand the basics of programming using python. |
| CO2 | Write and execute python programs |
| CO3 | Understand the concepts of using math library |
| CO4 | Adopt different techniques using functions in the program. |
| CO5 | Formulate algorithms and write programs using modules, packages and strings |
| CO6 | Apply python for real time application using object-oriented approach. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 2 | 3 | 12 | - | - | - | 17 |
| CO2 | 1 | 3 | - | 12 | - | - | 16 |
| CO3 | 4 | - | 6 | - | - | - | 10 |
| CO4 | 6 | - | 6 | 9 | - | - | 21 |
| CO5 | 4 | 1 | 6 | 6 | 14 | - | 31 |
| CO6 | 4 | - | 7 | 12 | - | 6 | 29 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 20EC1003 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # PROGRAMMING FOR PROBLEM SOLVING WITH C | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Identify the expected Output. #include <stdio.h> int main() { printf("All the Best!"); return 0; } | CO2 / A | 1 |
| 2. | Infer the problem solving technique. Step1: Start Step2: Include standard I/O header files Step3: Read length and breadth from the user Step4: Calculate the area, length X breadth Step5: Store the result in the variable area. Step6: Display the result Step7: Stop | CO1 / U | 1 |
| 3. | Identify the expected Output. #include <stdio.h> int main() { if( 10 < 9 ) printf("Hurray..\n"); else if(4 > 2) printf("England"); return 0; } | CO2 / A | 1 |
| 4. | Identify the expected Output. #include <stdio.h> int main() { int a=0; a = 5<2 ? 4 : 3; printf("%d",a); return 0; } | CO2 / A | 1 |

| 5. | Infer the output if the input is South Africa. #include<stdio.h> int main() { char str[25]; scanf("%s", str); printf("%s",str); return 0; } | CO3 / U | 1 |
| --- | --- | --- | --- |
| 6. | Identify the expected Output. #include<stdio.h> int main() { int a[] = {1,2,3,4}; int b[4] = {5,6,7,8}; printf("%d,%d", a[0], b[0]); } | CO5 / A | 1 |
| 7. | Identify the expected Output. #include<stdio.h> void show(); int main() { show(); printf("ARGENTINA "); return 0; } void show() { printf("AFRICA "); } | CO4 / A | 1 |
| 8. | Identify the expected Output. #include<stdio.h> int main() { int i=10, *j; void *k; k=&i; j=k; printf("%d\n", *j); return 0; } | CO5 / A | 1 |
| 9. | Infer: The variables declared inside a block ie., between { } is called as ___________ variables. (local/global). | CO4 / U | 1 |

| 10. | Infer: _____________method in C is used to dynamically allocate a single large block of memory with the specified size. | CO5 / U | 1 |
| --- | --- | --- | --- |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Distinguish between Hardware and Software. | CO1 /An | 3 |
| 12. | Build a C program to find the average of 50 numbers. | CO2 /A | 3 |
| 13. | Develop a C program to create a login application. Initialize two strings user []= “admin” and passwd [] =“karunya”. Allow the user to enter the username and password. Then compare the entered user credentials with the stored, to print “Login Successful” or “Invalid username or Password”. | CO5 / A | 3 |
| 14. | Build a C program to show the usage of string library functions. • Find the length of the string • Copying a string • Compare two strings • Concatenate two strings | CO3 / A | 3 |
| 15. | Develop a C-program to find the Size of the given Variables/ data types such as int, float, double, char. | CO4 / A | 3 |
| 16. | Explain the process of defining a Structure with a simple program. | CO5 / U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. |  | Compare: Algorithm, Flowchart and Pseudo code. | CO1 / E | 12 |
| 18. |  | Build a C program to design a simple arithmetic calculator using switch case statement. | CO2 / A | 12 |
| 19. |  | Develop a C Program to add two 3*3 matrices. | CO5 / A | 12 |
| 20. |  | Build a C program to sort a given array in both ascending and descending order by adopting the concept of functions. | CO5 / A | 12 |
| 21. |  | Build a C program to swap two numbers using pass by value and pass by reference. | CO5 / A | 12 |
| 22. |  | Build a program using structure to create a payroll report of employees in an organization. | CO5 / A | 12 |
| 23. |  | Build a C Program to pass an array in a function. Search for an element and replace it with ‘0’ inside the function. | CO4 / A | 12 |
|  |  | Compulsory: |  |  |
| 24. |  | Develop a C program to accept records of 5 different states using array of structures. The structure should contain ‘char’ state and number of ‘int’ engineering colleges, ‘int’ medical colleges, ‘int’ management colleges, and ‘int’ universities. Calculate the total colleges and display the state, which is having highest number of colleges. | CO6 / A | 12 |

Reg. No. _____________

End Semester Examination – May / June – 2022

| # Code : | # 20EC1003 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # PROGRAMMING FOR PROBLEM SOLVING WITH C | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Appraise on the role of Compiler in C Program. | CO1 / E | 1 |
| 2. | Infer: The number of bytes occupied by the operand is given by _________ operator. | CO1 / U | 1 |
| 3. | Interpret: ___________ loop is useful when we want that the statements within the loop to be executed at least once. | CO2 / U | 1 |
| 4. | List all the predefined data types in C. | CO2 / R | 1 |
| 5. | Distinguisha variable and an array. | CO3 / An | 1 |
| 6. | Name the header file that are used in string handling functions. | CO3 / R | 1 |
| 7. | Define function prototype. | CO4 / R | 1 |
| 8. | Appraise on the predefined scanf() function. | CO4 / E | 1 |
| 9. | Infer: ‘&’ operator. | CO5 / U | 1 |
| 10. | Define ‘Pointer to Pointer’. | CO5 / R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Appraise on the structure of a C program. | CO2 / E | 3 |
| 12. | Distinguish break and continue statements in C. | CO2 / An | 3 |
| 13. | Infer the following integer array. int num[] = {5,6,7,8}; Give sample C code to display the elements of the num array. | CO3 / U | 3 |
| 14. | List any three advantages of string functions. | CO3 / R | 3 |
| 15. | Construct a C program to calculate the area of a circle using the concept of function. | CO4 / A | 3 |
| 16. | List the advantages of structures over an array. | CO5 / R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | Appraise on the block diagram of a computer and explain it’s various blocks. | CO1 / E | 9 |
|  | b. | Infer on the input and output functions in C. | CO2 / U | 3 |
| 18. | a. | Explain the following: i. Logical operators ii. Conditional Operators iii. Bitwise operations | CO2 / U | 6 |
|  | b. | Construct a C program to find greatest among three numbers. | CO2 / U | 6 |

| 19. | a. | Define an array. Differentiate one dimensional and multi-dimensional array. | CO3 / R | 6 |
| --- | --- | --- | --- | --- |
|  | b. | Explain the various string handling functions with example. | CO3 / U | 6 |
| 20. |  | Discuss the following with suitable program: i. Function without argument & return type. ii. Function with argument & no return value. | CO4 / C | 12 |
| 21. | a. | Build a program to do the arithmetic operations on two integer values as per the choice given by the user. | CO2 / A | 9 |
|  | b. | Build a C program to swap two numbers. | CO5 / A | 3 |
| 22. | a. | Explain about Break Statement and goto Statement with examples. | CO2 / E | 3 |
|  | b. | Construct a program to develop basic arithmetic calculator (+,-,*,/) using switch case statement. | CO2 / A | 9 |
| 23. | a. | Build a C program to process 5 employee information(Emp-no, Emp-name, Date-of-joining, Salary, Bonus, Department) and display the net-Salary. | CO6 / C | 9 |
|  | b. | Construct a C program to display the consecutive digits 0, 1, 2, …. 9, with one digit on each line using while loop. | CO2 / A | 3 |
|  |  | Compulsory: |  |  |
| 24. | a. | Summarize the usage of Structures in C Programming. | CO5 / U | 3 |
|  | b. | Define a structure ‘Student’ that consists of the following members: Reg-no, name, marks in 3 subjects. Calculate the total, average and display the Rank wise list. | CO6 / R | 9 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the basics of computer and software development process. |
| CO2 | Understand the basics of programming skills using C language |
| CO3 | Apply innovative ideas for the problem using arrays and strings. |
| CO4 | Adopt different techniques for using functions in the program. |
| CO5 | Formulate algorithms and programs using arrays, pointers and structures |
| CO6 | Create a new application software to solve real world problems. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | - | 1 | - | - | 10 | - | 11 |
| CO2 | 1 | 16 | 21 | 3 | 6 | - | 47 |
| CO3 | 10 | 9 | - | 1 | - | - | 20 |
| CO4 | 1 | - | 3 | - | 1 | 12 | 17 |
| CO5 | 4 | 4 | 3 | - | - | - | 11 |
| CO6 | 9 | - | - | - | - | 9 | 18 |
|  |  |  |  |  |  |  | 124 |

Reg.No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 20EC2002 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ELECTRONIC DEVICES | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | How extrinsic semiconductor is formed from intrinsic semiconductor? | 1 /R | 1 |
| 2. | What are the types of carrier transport mechanisms? | 1 /R | 1 |
| 3. | Why PN diode is called as a bipolar device? | 2 /R | 1 |
| 4. | Show the current components of an unsymmetrically doped p-n diode. | 2 / R | 1 |
| 5. | Which transistor configuration provides unity voltage gain? | 3 /R | 1 |
| 6. | How much phase shift is introduced by a common emitter amplifier? | 3 / R | 1 |
| 7. | What are the different stages of operation of JFET? | 4 / R | 1 |
| 8. | What does MOSFET stands for? | 4 /R | 1 |
| 9. | In the case of photodiode, what is the relationship between the incident light and the current generated? | 5 / R | 1 |
| 10. | What is the other name of Tunnel diode? | 5 / R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Define carrier generation and recombination in a semiconductor. | 1 / R | 3 |
| 12. | Distinguish diffusion and transition capacitance. | 2 /An | 3 |
| 13. | Compare the three configurations of Bipolar Junction Transistor. | 3 /An | 3 |
| 14. | List the applications of UJT. | 4 / R | 3 |
| 15. | Explain the structure of MESFET. | 5 / U | 3 |
| 16. | Outline the principle behind the operation of photodiode. | 6 / U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | Determine the thermal equillibrium electron and hole concentration in a compensated semiconductor. Consider silicon at T=300k in which Na = 1016cm-3 and Nd = 3 x 1015 cm-3. Assume ni = 1.5 x 1010 cm-3. | 1 / E | 8 |
|  | b. | Determinethe conductivity of the intrinsic germanium at 3000 K, When ni at 3000 K =2.5X1013 /cm, µn and µp in germanium are 3800 and 1800 cm2 /vs respectively. | 1 / E | 4 |
| 18. | a. | Explain the structure of PN diode and its working principle during forward and reverse bias conditions. | 2 / U | 12 |

| 19. | a. | Explain the common emitter configuration of BJT with its DC input and output characteristics. | 3 / U | 12 |
| --- | --- | --- | --- | --- |
| 21. | a. | Write short notes on a) Zener diode b) Tunnel diode1 | 5 / R | 12 |
| 22. | a. | Explain the principle of Hall Effect and derive the expression of Hall voltage and Hall Coefficient. Also mention the significance of the Hall Effect. | 1 / U | 12 |
| 23. | a. | Appraise the structure of enhancement and depletion MOSFET and its features with necessary circuit diagram and Voltage-current characteristics. | 4 / E | 12 |
|  |  | Compulsory: |  |  |
| 24. | a. | Summarize the construction and operation of Silicon Controlled Rectifier with its transistor equivalent circuit. | 6 / U | 12 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 20EC2003 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # SIGNALS AND SYSTEMS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | Find the even components of the continuous time signal x(t)=1+tcost+t2 sint+t3sint cost | CO 1 | R | 1 |
| 2. | Show the signal x(t)=0.25u(t+1) | CO 1 | R | 1 |
| 3. | Find the number of samples in the output y[n]. If input x[n]=[ 1,1,2,-1,-2,2,4,2,3] and impulse response h[n]=[1,2,2] | CO 2 | R | 1 |
| 4. | Define LTI System | CO 2 | R | 1 |
| 5. | Find the CTFT of δ (t) | CO 3 | R | 1 |
| 6. | List the first harmonic equation in trigonometric fourier series | CO 3 | R | 1 |
| 7. | A signal having a spectrum ranging from 0 to 9 KHZ is to be sampled and converted to discrete form. Find the minimum number of samples per sec that must be taken to ensure recovery. | CO 4 | R | 1 |
| 8. | The signal x(t)=7cos(10Πt) is sampled at a rate of 8 samples per sec. Identify the transmitted signal is perfectly reconstructed or not in the receiver. | CO 4 | A | 1 |
| 9. | Find the DTFT of x(n) =u(n-k) | CO 5 | R | 1 |
| 10. | Find the frequency response of the system with the impulse response of h(n)=u(n)-u(n-1) | CO 5 | R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Test the following signal is energy signal or not. x(t) = e-3t u(t) | CO 1 | An | 3 |
| 12. | Test the linearity property of the system. y(t) = 0.5x[t+4] | CO 2 | An | 3 |
| 13. | Outline the Dirichlet conditions for Fourier Series. | CO 3 | U | 3 |
| 14. | Find the Laplace transform and ROC for the given signal. x(t)=e-5tu(t+5) | CO 4 | R | 3 |
| 15. | Recall the time shifting property of DTFT. | CO 5 | R | 3 |
| 16. | Determine the Z transform and ROC of the following signal. x(n)={1,2,3,4,5} | CO 6 | E | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | a. | Let be a DT signal with Find in graphical representation. (i) (ii) (iii) (iv) | CO1 | R | 12 |
| 18. | a. | Test the properties of the system y(t) = 2x(t)+3x(t–1) a) Static or Dynamic b) Linear or Non-linear c) Time invariant or variant | CO2 | An | 6 |
|  | b. | Find the convolution of input signal x(n) and impulse response h(n) using graphical method. x(n)={2,3,4,5} h(n)={3,3} | CO2 | R | 6 |
| 19. |  | Determine the exponential Fourier series for the continuous time periodic signal with fundamental period T=4 sec. | CO3 | E | 12 |
| 20. |  | Explain impulse train sampling and the reconstruction of signals from its samples with necessary diagrams | CO4 | U | 12 |
| 21. |  | Consider a causal and stable LTI system whose input x(n) and output y(n) are related through the second order difference equation y(n)-(3/4)y(n-1)+(1/8)y(n-2)=2x(n) - Determine the frequency response, impulse response - Determine the output response of the system for the given signal x(n)=(1/4)nu(n) | CO5 | E | 12 |
| 22. |  | Outline and prove the Time scaling, Time shifting, Time Reversal, Convolution properties of CTFT | CO3 | U | 12 |
| 23. | a. | Find the Laplace Transform and ROC of the given signal x(t)=e -4t u(t)+ e -3t u(-t) | CO4 | R | 6 |
|  | b. | Outline the transfer function of the system using Laplace Transform when the output y(t)=e-t-2e-2t+e-3t and input x(t)=e-0.5t | CO4 | U | 6 |
|  |  | Compulsory: |  |  |  |
| 24. |  | Find x(n) by using long division method X(Z)=(1+2z-1 )/(1-2z-1 +z -2 ) (i) x(n) is causal (ii) x(n) is anti-causal | CO6 | R | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Analyze different types of signals for mathematical modelling |
| CO2 | Realize the system properties to build basic model |
| CO3 | Represent continuous time system using fourier series and fourier transform |
| CO4 | Investigate the sampling process and Laplace Transform |
| CO5 | Signify discrete time system using fourier series and fourier transform |
| CO6 | Familiarize the frequency analysis of discrete time system using Z transform |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 14 | - | - | 3 | - | - | 17 |
| CO2 | 8 | - | - | 9 | - | - | 17 |
| CO3 | 2 | 15 | - | - | 12 | - | 29 |
| CO4 | 10 | 18 | 1 | - | - | - | 29 |
| CO5 | 5 | - | - | - | 12 | - | 17 |
| CO6 | 12 | - | - | - | 3 | - | 15 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 20EC2004 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # COMPUTER ARCHITECTURE | # Max. Marks: | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | What does the program counter content indicate? | CO1 | R | 1 |
| 2. | The memory has 4096 words in it. How many bits of the address bus are required to select a word in the memory? | CO1 | A | 1 |
| 3. | In which part of the computer system does the actual execution of instructions take place during processing operations? | CO2 | U | 1 |
| 4. | Brief on the fetch phase of the instruction cycle. | CO2 | U | 1 |
| 5. | Name the systematic method of controlling microoperation sequences stored in the controlled memory of a digital computer. | CO3 | R | 1 |
| 6. | What is the significance of program interrupt? | CO3 | U | 1 |
| 7. | What is the size of the register queue of 8086 in the Bus Interface Unit which prefetches the instruction from memory? | CO3 | R | 1 |
| 8. | In which mode will 8086 operate if MN/MX is low? | CO4 | U | 1 |
| 9. | Which is the only port of 8255 which can be operated in BSR mode. | CO5 | R | 1 |
| 10. | Mention an application of programmable interval timer 8254. | CO5 | U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | What is bus? Give the significance of a common bus structure. | CO1 | U | 3 |
| 12. | Write short notes on the instruction cycle and its different phases. | CO2 | R | 3 |
| 13. | Differentiate hardwired and microprogrammed control units. | CO3 | An | 3 |
| 14. | List out the various addressing modes of 8086. | CO4 | R | 3 |
| 15. | Brief on USART and give its features. | CO5 | U | 3 |
| 16. | Comment on memory hierarchy in terms of memory size and access speed. | CO6 | E | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. |  | Construct a single-bit bus line for accessing data from 4 registers using a decoder and three-state buffer and explain. | CO1 | A | 12 |
| 18. |  | Illustrate the bus system for implementing the fetch phase of the instruction cycle for register transfer instruction and explain. | CO2 | An | 12 |

| 19. |  | Write an assembly language program for the 8086 microprocessor to find the average of 5 numbers stored in the memory unit. | CO4 | A | 12 |
| --- | --- | --- | --- | --- | --- |
| 20. |  | Discuss in detail the architecture of the Intel 8086 microprocessor. | CO2 | U | 12 |
| 21. |  | Demonstrate the interfacing of 8251 USART with an 8086 microprocessor with neat diagrams. | CO5 | A | 12 |
| 22. |  | Organize the blocks of a microprogrammed control unit and provide the decision-making capabilities in the control unit. | CO3 | A | 12 |
| 23. |  | Sketch the functional blocks and control word format of 8254 Programmable Interval Timer and summarize the modes of operation. | CO5 | U | 12 |
|  |  | Compulsory: |  |  |  |
| 24. |  | Design a memory system for the microprocessor such that it should contain 8 Kbyte of EPROM and 8 Kbyte of RAM. | CO6 | C | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Illustrate the basics of computer organization |
| CO2 | Compute Arithmetic and Logic Unit |
| CO3 | Categorize the performance of memory systems. |
| CO4 | Illustrate and Implement programs on 8086 microprocessor. |
| CO5 | Compute I/O and Memory circuits and 8086 ALP. |
| CO6 | Formulate Memory Interfacing circuits. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 1 | 3 | 13 | - | - | - | 17 |
| CO2 | 3 | 14 | - | 12 | - | - | 29 |
| CO3 | 2 | 1 | 12 | 3 | - | - | 18 |
| CO4 | 3 | 1 | 12 | - | - | - | 16 |
| CO5 | 1 | 15 | 12 | - | - | - | 28 |
| CO6 | - | - | - | - | 3 | 12 | 15 |
|  |  |  |  |  |  |  | 123 |

Reg. No. _____________

End Semester Examination – May / June – 2022

| # Code : | # 20EC2014 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # BASIC ELECTRONICS FOR # AEROSPACE ENGINEERS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Capacitor is used to store __________. | CO1 | 1 |
| 2. | When resistors are connected in series, the current flowing through each resistor is __________. | CO1 | 1 |
| 3. | __________ is the suitable motor for domestic fan. | CO2 | 1 |
| 4. | Material used to make field coils in D.C. generator is ………………… | CO2 | 1 |
| 5. | Zener diode is used as __________regulator. | CO3 | 1 |
| 6. | A semiconductor is formed by __________ bond. | CO3 | 1 |
| 7. | Speed of a microcontroller is relatively __________ compared to microprocessor. | CO4 | 1 |
| 8. | Register that is used to store the output generated by ALU is __________ | CO4 | 1 |
| 9. | Sensors convert from analog to __________ form. | CO5 | 1 |
| 10. | Abbreviate the term WWAN. | CO6 | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | List out the functions of a capacitor. | CO1 | 3 |
| 12. | Differentiate between AC and DC motor. | CO2 | 3 |
| 13. | State the principle on which a PN junction diode works. | CO3 | 3 |
| 14. | Mention few applications of logic gates. | CO4 | 3 |
| 15. | What are the devices used for pressure measurement in aircraft? | CO5 | 3 |
| 16. | How 4G differ from 3G communication network? | CO6 | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | With a neat schematic, briefly explain the types of transformers. | CO1 | 8 |
|  | b. | Write short notes on UAV. | CO1 | 4 |
| 18. | a. | Enumerate the working principle of a brushless DC motor. List out its merits and demerits. | CO2 | 8 |
|  | b. | How does an electric bike operate? | CO2 | 4 |

| 19. | a. | Explain the construction and salient features of a full wave rectifier with the help of a diagram. | CO3 | 8 |
| --- | --- | --- | --- | --- |
|  | b. | Plot the performance characteristics of a BJT. | CO3 | 4 |
| 20. | a. | Draw the block diagram of a microcontroller and discuss its working. | CO4 | 8 |
|  | b. | Mention the salient features of an Arduino microcontroller. | CO4 | 4 |
| 21. | a. | Describe the construction and working of an altimeter. | CO5 | 8 |
|  | b. | Write short notes on the applications of electronic sensors in aircraft. | CO5 | 4 |
| 22. | a. | With the aid of a schematic, explain the working principle of a generator. | CO2 | 8 |
|  | b. | Highlight the merits and demerits of a generator. | CO2 | 4 |
| 23. | a. | What is the principle of a Zener diode? Discuss its construction and operation. | CO3 | 8 |
|  | b. | How does a digital multi meter operate? | CO3 | 4 |
|  |  | Compulsory: |  |  |
| 24. | a. | Explain the components of a cellular mobile communication system? Mention its applications. | CO6 | 8 |
|  | b. | Draw the block diagram of a communication system. | CO6 | 4 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Recognize importance and judicious use of electronic components in everyday life. |
| CO2 | Identify the types of electrical machines used for various applications. |
| CO3 | Understand and apply the concept of electronics to design simple circuits. |
| CO4 | Understand and relate various digital circuits. |
| CO5 | Understand the various sensing and instrumentation applications. |
| CO6 | Identify the various generations of wireless communications. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 4 | 13 |  |  | - | - | 17 |
| CO2 | 1 | 16 | 12 |  |  |  | 29 |
| CO3 | 4 | 13 | 12 |  | - | - | 29 |
| CO4 | 1 | 16 |  | - |  |  | 17 |
| CO5 | 3 | 13 | - | - | - | - | 16 |
| CO6 | 1 | 15 | - | - | - | - | 16 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – May / June – 2022

| # Code : | # 20EC2018 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # FUNDAMENTALS OF PRINTED CIRCUIT AND ARDUINO BOARD DESIGN | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | List out the various applications of battery. | CO1/ R | 1 |
| 2. | Relate 1 Unit of Energy in terms of watts. | CO1/ R | 1 |
| 3. | Identify the diode that is used in rectifier circuit. | CO2/ A | 1 |
| 4. | Define doping. | CO2/ R | 1 |
| 5. | Label the symbol of NOT gate. | CO3/ R | 1 |
| 6. | Name the sensor used to control the street lights automatically. | CO3/ R | 1 |
| 7. | Outline the general rules and parameters of PCB design. | CO4/ U | 1 |
| 8. | Name the process of removing unwanted copper from the PCB substrate. | CO4/ R | 1 |
| 9. | Which is used to convert assembly-level code to a machine code? | CO5/ R | 1 |
| 10. | List few characteristics of Internet of Things (IoT). | CO5/ R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Analyze the energy consumption per month (30days) of 60W bulb which operates continuously for 5 hours per day. | CO1/ An | 3 |
| 12. | Build the circuit connection of a 12V lamp with a relay. | CO2/ A | 3 |
| 13. | Distinguish analog sensor and digital sensor. | CO3/An | 3 |
| 14. | Infer the importance of PCB development tool with few examples. | CO4/ U | 3 |
| 15. | Interpret the function of address bus in a microcontroller. | CO5/ U | 3 |
| 16. | List few sensors used in robotic arm controller. | CO6/ R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. |  | Summarize the power generation in hydroelectric power plant with a neat block diagram. | CO1 / U | 12 |
| 18. | a. | Outline the structure and V-I characteristics of PN junction diode with neat diagram and mention its applications. | CO2 / U | 8 |
|  | b. | Classify resistors with its specifications. | CO2 / U | 4 |

| 19. | a. | Explain the basic block of combinational and sequential circuit and list out their differences. | CO3 / E | 8 |
| --- | --- | --- | --- | --- |
|  | b. | Outline universal gates. Justify the answer with an example. | CO3 / U | 4 |
| 20. | a. | Appraise the significance of Printed Circuit Boards and explain the structure of single-sided and double-sided PCB. | CO4 / E | 8 |
|  | b. | What are the different types of IC packages? | CO4 / R | 4 |
| 21. |  | Explain briefly the architecture of Arduino board with a neat block diagram. | CO5 / E | 12 |
| 22. | a. | Compare and contrast the various transistor configurations. | CO2 / E | 7 |
|  | b. | Analyze the process of resistor color coding with suitable examples. | CO1/An | 5 |
| 23. | a. | Examine the characteristics of photo diode and LED with relevant diagrams. | CO2/An | 8 |
|  | b. | Explain the function of thermostat as a transducer. | CO3 / E | 4 |
|  |  | Compulsory: |  |  |
| 24. | a. | Design a line follower robot using Arduino and illustrate its block diagram. | CO6 / C | 8 |
|  | b. | Infer the Arduino Integrated Development Environment (IDE). | CO6 / U | 4 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Recognize the importance of electro mechanical systems in everyday life. |
| CO2 | Understand the properties of semiconductor devices. |
| CO3 | Understand and relate various digital concepts and circuits. |
| CO4 | Acquire the knowledge about the packages of Electronic components, types of PCBs and history of PCBs |
| CO5 | Understand the design and programming of Arduino boards. |
| CO6 | Acquire the knowledge and skills to implement various smart systems application. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 2 | 12 | - | 8 | - | - | 22 |
| CO2 | 1 | 12 | 4 | 8 | 7 | - | 32 |
| CO3 | 2 | 4 | - | 3 | 12 | - | 21 |
| CO4 | 5 | 4 | - | - | 8 | - | 17 |
| CO5 | 2 | 3 | - | - | 12 | - | 17 |
| CO6 | 3 | 4 | - | - | - | 8 | 15 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – June – 2022

| # Code : | # 20EC3002 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # LOW POWER VLSI DESIGN | # Max. Marks : | # 100 |

| Q. No. | Sub Div. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |
| 1. | a. | A 64 bit off-chip bus operating at 3V and 200 MHz clock rate is driving a capacitance of 40 pF/ bit. Each bit is estimated to have a toggling probability of 0.5 at each clock cycle. Estimate the power dissipation in operating the bus? | CO1 / E | 4 |
|  | b. | Explain any two sources of leakage current in CMOS transistors. | CO1 / U | 6 |
|  | c. | Formulate the dynamic power dissipation in digital VLSI chip design. | CO1 / C | 6 |
| 2. | a. | Estimate the transition density and static probability of y= ab + c given P (a) = 0.2, P (b) = 0.3, P(c) = 0.4 , D(a) = 1, D(b) =2, D(c) =3. | CO2 / E | 10 |
|  | b. | Formulate an expression which relates conditional probability and frequency. | CO2 / C | 6 |
| 3. | a. | Design an inverter chain to drive a large capacitive load without an excessive delay, area and power requirements. | CO3 / C | 8 |
|  | b. | Apply Pre-Computation Logic to reduce power in the magnitude comparator circuit. | CO3 / A | 8 |
| 4. | a. | Demonstrate power and performance management in architectural level with suitable examples. | CO4 / U | 6 |
|  | b. | Apply the techniques to reduce switching activities in CMOS digital systems. | CO4 / A | 6 |
|  | c. | Apply Flow Graph Transformation techniques to reduce power at System level. | CO4 / A | 4 |
| 5. | a. | Explain in detail the organization of a RAM core and also explain the operation of 6T MOS static RAM memorycell. | CO5 / U | 8 |
|  | b. | Design a low power array multiplier using any one power reduction technique. | CO5 / C | 8 |
| 6. | a. | Summarize the various transformation operators for gate reorganization to reduce power in logic level. | CO3 / U | 8 |
|  | b. | Explain the banked SRAM concepts and also the techniques to reduce the voltage swings on bit lines in SRAM design. | CO5 / U | 8 |

| 7. | a. | The chip size of a CPU is 15mm x 25 mm with clock frequency of 300MHz operating at 3.3 V. The length of the clock routing is estimated to be twice the circumference of the chip. Assume that the clock frequency is routed on a metal layer with width of 1.2µm and the parasitic capacitance of the metal layer is 1 Ff/µm2. Estimate the power dissipation of the clock signal? | CO1 / E | 5 |
| --- | --- | --- | --- | --- |
|  | b. | Formulate an expression which relates static probability and frequency. | CO2 / C | 6 |
|  | c. | Design a full adder circuit using NORA dynamic logic circuit. | CO4 / C | 5 |
| PART – B (1 X 20 = 20 MARKS) (Compulsory Question) |  |  |  |  |
| 8. | a. | Explain the adiabatic amplification concept with suitable circuit diagrams. | CO6 / U | 8 |
|  | b. | Design adiabatic AND/NAND logic gates using adiabatic charging concept. | CO6 / C | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the various sources of power dissipation. |
| CO2 | Acquire the knowledge on simulation power analysis and Probabilistic power analysis Techniques. |
| CO3 | Learn the various low power reduction techniques at circuit and logic level. |
| CO4 | Analyze the various low power techniques at Architecture and system level. |
| CO5 | Design low power clock Networks, bus and low power SRAM circuits. |
| CO6 | Design various adiabatic logic circuits |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 |  | 6 |  |  | 9 | 6 | 21 |
| CO2 |  |  |  |  | 10 | 12 | 22 |
| CO3 |  | 8 | 8 |  |  | 8 | 24 |
| CO4 |  | 6 | 10 |  |  | 5 | 21 |
| CO5 |  | 16 |  |  |  | 8 | 24 |
| CO6 |  | 8 |  |  |  | 12 | 20 |
|  |  |  |  |  |  |  | 132 |

Reg. No. _____________

End Semester Examination – June – 2022

| # Code : | # 20EC3003 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ANALOG VLSI DESIGN | # Max. Marks : | # 100 |

| Q. No. | Sub Div. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |
| 1. | a. | Distinguish Sah’s and Shichman-Hodges model with neat description and obtain the equivalent circuits for MOSFET in Ohmic and Saturation region. | CO1 / An | 8 |
|  | b. | Using the typical NMOS process parameters of Table 1 (given below) - Determine the output voltage for the circuit given below if R=14kΩ. - Determine the maximum value of R allowable to keep M1operating in the saturation region. Table 1 Typical Process Parameter for 3µ NMOS and CMOS processes Parameter; n-channel; p-channel; units. K’; 24; 8; µA/V2. VTOE; 0.75; -0.75; V. VTOD; -3; 3; V. Ƴ(CMOS); 0.8; 0.4; V1/2. Ƴ(NMOS); 0.4; -; V1/2. Ф; 0.6; 0.6; V. λ; 0.01; 0.02; V-1. | CO1 / E | 8 |

| 2. | a. | Illustrate the small signal BJT model with an equivalent circuit. | CO1 / U | 8 |
| --- | --- | --- | --- | --- |
|  | b. | Determine the size of M2 with minimum area so that V0=4.0V.Assume M1 is minimum size. Use process parameters of Table 1.Neglect λ effects and NMOS Process (with a feature size λ=1.5µ, so that Wmin= 3 µ and Lmin= 3µ). Table 1 Typical Process Parameter for 3µ NMOS and CMOS processes Parameter; n-channel; p-channel; units. K’; 24; 8; µA/V2. VTOE; 0.75; -0.75; V. VTOD; -3; 3; V. Ƴ(CMOS); 0.8; 0.4; V1/2. Ƴ(NMOS); 0.4; -; V1/2. Ф; 0.6; 0.6; V. λ; 0.01; 0.02; V-1. | CO1 / E | 8 |
| 3. | a. | Explain the high frequency BJT model, with a high frequency equivalent circuit. | CO1 / U | 8 |
|  | b. | Discuss in detail about the 3-bit voltage scaling DAC with neat diagrams. | CO2 / C | 8 |

| 4. | a. | With an example of successive approximation A/D converter architecture, analyze the digital code using the successive approximation process. | CO2 / An | 12 |
| --- | --- | --- | --- | --- |
|  | b. | Determine the DNL (codes 011,100,101,110,111) for the 3-bit non-ideal DAC whose transfer function is shown below. | CO2 / E | 4 |
| 5. | a. | Examine the resistor emulation in Parallel switched capacitor circuits. | CO3 / An | 8 |
|  | b. | Develop in detail about the Unity-Gain Sampler/Buffer in different operating modes. | CO4 / A | 8 |
| 6. | a. | Estimate the basic conditions of oscillator with neat diagrams and explain in detail about the ring Oscillator with two pole feedback system, with signal inversion and three stage ring oscillator. | CO5 / E | 8 |
|  | b. | Analyze the basic concepts of current amplifier and examine the single ended input and differential input current amplifier. | CO4 / An | 8 |
| 7. | a. | Develop the four types of high-gain amplifiers with gain analysis and necessary diagrams. | CO4 / A | 8 |
|  | b. | Illustrate in detail about the Class A power amplifier and show the maximum collector efficiency with necessary expressions and circuit diagram. | CO4 / U | 8 |
| PART – B (1 X 20 = 20 MARKS) (Compulsory Question) |  |  |  |  |
| 8. | a. | Summarize in detail the tuning in LC Oscillator | CO5 / U | 10 |
|  | b. | Design the two stage open –loop comparator with all the design parameters. | CO6 / C | 10 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | compute the characteristics of MOS transistors and analyze the circuit characteristics through device modeling. |
| CO2 | utilize the analog design concepts in data converters |
| CO3 | illustrate different types of switched capacitor circuits |
| CO4 | perform analysis in CMOS amplifiers |
| CO5 | perform analysis in Oscillators |
| CO6 | design and develop various comparators and illustrate the performance of analog circuits using EDA tools. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | - | 16 | - | 8 | 16 | - | 40 |
| CO2 | - | - | - | 12 | 4 | 8 | 24 |
| CO3 | - | - | - | 8 | - | - | 8 |
| CO4 | - | 8 | 16 | 8 | - | - | 32 |
| CO5 | - | 10 | - | - | 8 | - | 18 |
| CO6 | - | - | - | - | - | 10 | 10 |
|  |  |  |  |  |  |  | 132 |

Reg. No. _____________

End Semester Examination – May / June – 2022

| # Code : | # 21EC1001 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ELECTRONICS FOR EVERYDAY LIFE | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Find the field of science that deals with the control of electricity using electrical components. | CO1 / R | 1 |
| 2. | ___________ manufacturers use raw materials to create resistors, transistors, ICs, etc. | CO1 / U | 1 |
| 3. | Name the type of amplifier that is used as an output amplifier in an audio system. | CO2 / R | 1 |
| 4. | What is the purpose of the cross interleaved Reed-Solomon code, when recording data on a CD? | CO2 / R | 1 |
| 5. | How are subscribers provided with the paid channels/premium services that they have opted for? | CO3 / R | 1 |
| 6. | The preferred band for the direct reception of TV from a satellite is _________. | CO3 / U | 1 |
| 7. | PABX is expanded as ___________. | CO4/ U | 1 |
| 8. | Name the circuit may be used in a communication system to increase the power level of signals to compensate for the losses encountered during transmission. | CO4 / R | 1 |
| 9. | Linear or one-dimensional barcodes can be used to represent __________ characters. | CO5 / U | 1 |
| 10. | What is the standard input frequency of a digital clock? | CO5 / R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Name any three electronic devices that belong to the communications sector. | CO1 / A | 3 |
| 12. | Find the difference between AM and FM. | CO2 / R | 3 |
| 13. | Compare LED and LCD. | CO3 / E | 3 |
| 14. | List out the factors affecting range and reliability of mobile phones. | CO4 / R | 3 |
| 15. | What are the features of a flatbed scanner? | CO5 / R | 3 |
| 16. | Recall the working principle of pulse Oximeters. | CO6 / R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. |  | Outline the block diagram of the working of the Electronics Industry and also explain the major sectors of the industry. | CO1 / U | 12 |
| 18. | a. | Analyze principle, construction, working, and characteristics of any one type of Microphone. | CO2 /An | 8 |
|  | b. | List out the characteristics of an Audio amplifier. | CO2 / R | 4 |
| 19. | a. | Discuss on recording and playback of VCD and DVD systems | CO3 / C | 8 |
|  | b. | List out the elements of CATV systems | CO3 / R | 4 |
| 20. | a. | Examine the functioning of various elements of a Digital Private Automatic Branch Exchange (PABX) system. | CO4 /An | 8 |
|  | b. | What are the access methods and technologies used in the Digital Cell phone system? | CO4 / R | 4 |
| 21. | a. | What is a barcode scanner? Explain briefly some of its types. | CO5 / U | 6 |
|  | b. | Outline the functioning and features of Microwave Oven. | CO5 / U | 6 |
| 22. |  | Why need for modulation? Explain its three types. | CO2 / E | 12 |
| 23. |  | Explain the following display screens. LCD Plasma LED | CO3 / E | 12 |
|  |  | Compulsory: |  |  |
| 24. | a. | Explain direct and indirect methods of Blood pressure measurement. | CO6 / E | 6 |
|  | b. | Examine external and implantable Pacemakers. | CO6 /An | 6 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Define the components in an electronic system. |
| CO2 | Understand the working of audio systems |
| CO3 | Explain the various standards and technology in video systems |
| CO4 | Understand the telephone network and mobile phone systems. |
| CO5 | Demonstrate the working of office and domestic appliances |
| CO6 | Comprehend the functioning of medical equipments |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 1 | 13 | 3 | - | - | - | 19 |
| CO2 | 9 | - | 10 | 8 | 12 |  | 39 |
| CO3 | 5 | 1 |  | - | 15 | 8 | 29 |
| CO4 | 8 | 1 | - | 8 | - | - | 17 |
| CO5 | 4 | 13 | - | - | - | - | 17 |
| CO6 | 3 | - | - | 6 | 6 | - | 15 |
|  |  |  |  |  |  |  | 136 |

Reg. No. _____________

End Semester Examination – May / June – 2022

| # Code : | # 21EC1003 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # PROBLEM SOLVING AND ALGORITHMIC THINKING | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Identify the cause and effect for the following based on the 'Cause - Effect relationship'. IF the user has struck the Confirm Purchase button, THEN initiate billing procedures. | CO1 / A | 1 |
| 2. | List the correct sequence in problem solving. | CO1 / U | 1 |
| 3. | Construct a C program to display “I am born to succeed”. | CO3 / A | 1 |
| 4. | Identify the category (Selection/Sequencing/Iteration): IF (timer <= 0) OR (numberOfLives == 0): PRINT "Game Over". | CO1 / A | 1 |
| 5. | Identify the category (Iteration/Selection/Sequence): WHILE (timer > 0): PRINT "Carry on playing..." timer = timer – 1. | CO1 / A | 1 |
| 6. | Extend your perception on ‘linked list’. | CO2 / U | 1 |
| 7. | Infer: Static binding. | CO2 / U | 1 |
| 8. | Infer: Minimum spanning tree. | CO4 / U | 1 |
| 9. | Define: Space complexity. | CO2 / R | 1 |
| 10. | Relate FIFO to an example. | CO2 / U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Illustrate an algorithm to determine whether a Student has Passed the Exam or Not? | CO2 / U | 3 |
| 12. | Compare Inductive reasoning and Deductive reasoning. | CO1 / U | 3 |
| 13. | Infer : Decomposition. | CO1 / U | 3 |
| 14. | Distinguish between List and Array. | CO2 / An | 3 |
| 15. | Show the contrast between ‘While loop and Do-while loop’. | CO3 / U | 3 |
| 16. | Illustrate ‘Recursion’ with an example. | CO3 / U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. |  | Appraise on the three major phases in the process of developing a software. | CO1 / E | 12 |
| 18. |  | Construct an algorithm for building a simple calculator and write a C program for the same. | CO3 / A | 12 |
| 19. | a. | Infer on the ‘problem tree’ and ‘objective tree’ with suitable example. | CO1 / U | 8 |
|  | b. | List the Constituents of algorithms | CO1 / R | 4 |
| 20. |  | Explain about the various types of linked lists with relevant diagrams. | CO2 / U | 12 |
| 21. | a. | Explain about Merge sort with an example. | CO1 / U | 8 |
|  | b. | Infer: Binary search. | CO2 / U | 4 |
| 22. |  | Apply Kruskal’s algorithm to find the minimum spanning tree for the following graph. | CO4 / A | 12 |
| 23. |  | Construct a C program to get the information like Name, ID no, Salary, Bonus for 10 employees and display the total pay. | CO3 / A | 12 |
|  |  | Compulsory: |  |  |
| 24. |  | Build a C program to Perform a 2x2 matrix addition using the concept of arrays. | CO3 / A | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Apply algorithmic thinking to understand, define and solve problems |
| CO2 | Design and implement algorithm(s) for a given problem |
| CO3 | Apply the basic programming constructs for problem solving |
| CO4 | Understand an algorithm by tracing its computational states |
| CO5 | Identifying bugs in algorithms |
| CO6 | Analyze the bugs and correct them |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 4 | 23 | 3 | - | 12 | - | 42 |
| CO2 | 1 | 22 | - | 3 | - | - | 26 |
| CO3 | - | 6 | 37 | - | - | - | 43 |
| CO4 | - | 1 | 12 | - | - | - | 13 |
| CO5 | - | - | - | - | - | - | - |
| CO6 | - | - | - | - | - | - | - |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – May / June – 2022

| # Code : | # 21EC1004 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # PYTHON PROGRAMMING | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | What will be the output of given Python code? str1="6/4" print("str1") | CO1 / R | 1 |
| 2. | What will be the output of given Python code? str1="Information" print(str1[2:8]) | CO5 / R | 1 |
| 3. | Identify the output of given Python code? str1="Aplication" str2=str1.replace('a','A') print(str2) | CO5 / A | 1 |
| 4. | Identify the output of the following code snippet? dict1 = {'first' : 'sunday', 'second' : 'monday'} dict2 = {1: 3, 2: 4} dict1.update(dict2) print(dict1) | CO2 / A | 1 |
| 5. | Examine the following Python code return? str1="Stack of books" print(len(str1)) | CO6 / An | 1 |
| 6. | Examine the datatype of the var in the below code snippet? var = 10 print(type(var)) var = "Hello" print(type(var)) | CO4 / An | 1 |
| 7. | What will be the output of the following code snippet? a = [1, 2, 3] a = tuple(a) a[0] = 2 print(a) | CO4 / R | 1 |
| 8. | What will be the output of the following code snippet? a = [1, 2, 3, 4, 5] sum = 0 for ele in a: sum += ele print(sum) | CO3 / R | 1 |
| 9. | Identify the output of the following code snippet? def solve(a, b): return b if a == 0 else solve(b % a, a) print(solve(20, 50)) | CO6 / A | 1 |
| 10. | Show the declaration of the constructor defined in the class with syntax. | CO6 / U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Develop an user-defined function, find_count_vowels_cons_letters(first_name, last_name) which accepts your first name and last name. Include necessary logic to print number of vowels and consonants. | CO6 / A | 3 |
| 12. | Select 10 integer numbers (including duplicate numbers) as input and store it in a list variable and find all the duplicate elements and print the same in list format and sort them in both ascending and descending order. | CO5 / A | 3 |
| 13. | Create a user defined function reverse_digits (number) and read an integer number as input and reverse digits of the given using your function and display the same. | CO6 / C | 3 |
| 14. | Examine the output of the following code snippet. myString = "pynative" stringList = ["abc", "pynative", "xyz"] print(stringList[1] == myString) print(stringList[1] is myString) | CO1 / An | 3 |
| 15. | What will be the output of the following code snippet? count = 0 while(True): if count % 3 == 0: print(count, end = " ") if(count > 15): break; count += 1 | CO4 / R | 3 |
| 16. | What will be the output of the following code snippet? numbers = (4, 7, 19, 2, 89, 45, 72, 22) sorted_numbers = sorted(numbers) odd_numbers = [x for x in sorted_numbers if x % 2 != 0] print(odd_numbers) | CO2 / R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. |  | Create a Python program to implement ticket booking application. Get the name and age of the passenger to book the ticket. Check the age, if it is less than 6 years or greater than or equal to 60 years then apply 30% concession to the original ticket price (Rs. 1000). Otherwise simply print tickets booked with the original ticket price. | CO2 / C | 12 |
| 18. | a. | Construct the following dictionary: inventory = { 'gold' : 500, 'pouch' : ['flint', 'twine', 'gemstone'], 'backpack' : ['xylophone', 'dagger', 'bedroll', 'bread loaf'] } Try to do the followings: - Add a key to inventory called 'pocket'. - Set the value of 'pocket' to be a list consisting of the strings 'seashell', 'strange berry', and 'lint'. - sort () the items in the list stored under the 'backpack' key. - Then remove('dagger') from the list of items stored under the 'backpack' key. - Add 50 to the number stored under the 'gold' key. | CO3 / A | 6 |
|  | b. | Develop a Python program to check the eligibility of a candidate appearing for military selection. Get the age and height (cms) from the user. If the candidate age is 18 and above, and if the height is 180cms and above print “Eligible for Selection” otherwise print “Not Eligible”. | CO1 / A | 6 |
| 19. |  | Build the code below and complete the following: 1. Add two more people to the dictionary people. 2. Modify the name of person 1 from John to Jonathan. 3. Delete person 2 from the dictionary. 4. Show all key values pairs from the dictionary using a for loop. Be sure to nicely format the output of the all of the key value pairs. people = {1: {'name': 'John', 'age': '27', 'sex': 'Male'}, 2: {'name': 'Marie', 'age': '22', 'sex': 'Female'}} | CO4 / A | 12 |
| 20. |  | Create a Python program that takes a text file as input and returns the number of words of a given text file Also, extract characters from various text files and puts them into a list and do the following operation: read first n lines of a file, read a file line by line and store it into a list, to count the number of lines in a text file, to remove newline characters from a file. | CO6 / C | 12 |
| 21. |  | You are working for Facebook’s Operations Intelligence team and are tasked to build a calculator to find the best ad sales representative each month which is based on largest (hint: max) monthly ad revenue. There are 5 sales representatives listed below with this months revenue. Call the calculator rep_calculator. • Joe: $1,500,000 • Sarah: $2,750,000 • Jack: $560,000 • Mark: $1,975,000 • Shawn: $2,200,000 Complete the following: 1. Build a dictionary 2. Return the key with the max value 3. Print a pretty sentence: Sarah has the highest sales! She reached $2,750,000.00 this month | CO5 / C | 12 |
| 22. |  | Construct a python program using class named student that contains members: Student name (type char), Roll number (type int) and the marks obtained in 3 subjects (type float). Ask the user to fill in this data for three students, obtain the total mark and display the information along with the percentage for each student. | CO1 / A | 12 |
| 23. |  | Build a python code to create a class called STRING and implement the following operations. i) STRING s1=”VTU”; ii) STRING s2=”BELGAUM”; iii) STRING s3=S1+S2; | CO5 / C | 12 |
|  |  | Compulsory: |  |  |
| 24. |  | Develop a Python program to generate reports of Student details. Create a class called Student with attributes Name, RegNo, five subjects mark (use your current semester subject names as variable names), total, average and CGPA. Use getInput() function to get the inputs (only for Name, RegNo and SubjectMarks)from the user, create a function Calculate() to calculate the total marks, average and CGPA. Write a printOuput() function to print the output in table format. Create list of Student objects and display the reports. | CO3 / A | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the basics of programming using python. |
| CO2 | Write and execute python programs |
| CO3 | Understand the concepts of using math library |
| CO4 | Adopt different techniques using functions in the program. |
| CO5 | Formulate algorithms and write programs using modules, packages and strings |
| CO6 | Apply python for real time application using object-oriented approach |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 1 |  | 18 | 3 |  |  | 22 |
| CO2 | 3 |  | 1 |  |  | 12 | 16 |
| CO3 | 1 |  | 18 |  |  |  | 19 |
| CO4 | 4 |  | 12 | 1 |  |  | 17 |
| CO5 | 1 |  | 4 |  |  | 24 | 29 |
| CO6 |  | 1 | 4 | 1 |  | 15 | 21 |
|  |  |  |  |  |  |  | 100 |

Reg. No. _____________

End Semester Examination – May / June – 2022

| # Code : | # 21EC1007 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # SOFTWARE ENGINEERING | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Define software engineering. | CO1 / R | 1 |
| 2. | Compare action and task in a software process. | CO1 / U | 1 |
| 3. | What is process flow? | CO1 / R | 1 |
| 4. | Identify the process flow that repeats one or more of the activities before proceeding to the next. | CO1 / A | 1 |
| 5. | What is requirements engineering? | CO2 / R | 1 |
| 6. | List a few examples for scenario-based models used in software requirements. | CO2 / R | 1 |
| 7. | Recall the importance of architecture. | CO3 / R | 1 |
| 8. | Name the commonly used architectural styles. | CO3 / R | 1 |
| 9. | What are the golden rules used in interface design? | CO3 / R | 1 |
| 10. | Define Fitt’s law. | CO3 / R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | What are the characteristics of software as a product? | CO1 / R | 3 |
| 12. | Identify the task sets for requirements gathering. | CO2 / A | 3 |
| 13. | Show the class diagram for a sensor. | CO2 / U | 3 |
| 14. | Outline the various archetypes defined. | CO3 / U | 3 |
| 15. | Summarize the importance of swimlane diagram. | CO3 / U | 3 |
| 16. | What is integration testing and what are the approaches of integration testing? | CO5 / R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | Explain the framework activities that are applicable to software projects in detail. | CO1 / U | 8 |
|  | b. | Summarize the importance of legacy software. | CO1 / U | 4 |
| 18. | a. | Explain in detail the evolutionary process model with a neat diagram. | CO1 / U | 8 |
|  | b. | Outline the Extreme programming (XP) values with brief description. | CO1 / U | 4 |
| 19. |  | Explain the requirements engineering process in detail. | CO2 / U | 12 |

| 20. |  | Explain the process of architectural mapping using data flow with an illustration. | CO3 / U | 12 |
| --- | --- | --- | --- | --- |
| 21. | a. | Summarize the tasks that represent a rudimentary workflow for WebApp interface design. | CO3 / U | 6 |
|  | b. | Explain the different user interface design models in detail. | CO3 / U | 6 |
| 22. |  | Explain Water fall Model. What are the problems that areencountered when the waterfall modelis applied? | CO1 / U | 12 |
| 23. |  | List the different types of architectural styles that exist for software and explain any one of them in detail. | CO3 / U | 12 |
|  |  | Compulsory: |  |  |
| 24. | a. | Summarize the testing strategies that are used in software engineering. | CO4 / U | 4 |
|  | b. | Explain the types of system tests that are used for software-based systems. | CO6 / U | 8 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the software engineering and agile development processes. |
| CO2 | Formulate requirements and validate them. |
| CO3 | Make use of different software design methodologies. |
| CO4 | Determine suitable tests for testing and debugging a software. |
| CO5 | Apply appropriate methods to manage and maintain a software. |
| CO6 | Ensure that the software meets the required standards. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 5 | 37 | 1 |  |  |  | 43 |
| CO2 | 2 | 15 | 3 |  |  |  | 20 |
| CO3 | 4 | 42 |  |  |  |  | 46 |
| CO4 |  | 4 |  |  |  |  | 4 |
| CO5 | 3 |  |  |  |  |  | 3 |
| CO6 |  | 8 |  |  |  |  | 8 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 21EC2007 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # DATA STRUCTURES AND ALGORITHMS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | Name the space complexity for the following: The space required to store the constants and variables in a program. | CO1 | R | 1 |
| 2. | Analyze the following code and write the output of the code int main() { int arr[5]={10,20,30,40,50}; cout<< arr[2]; return 0; } | CO1 | An | 1 |
| 3. | Identify the correct notation for the following arithmetic expression A B+ | CO3 | A | 1 |
| 4. | Name the process which is used to insert an element in a stack. | CO3 | R | 1 |
| 5. | Which type of sorting algorithm uses divide and conquer method? | CO2 | R | 1 |
| 6. | How many leaf nodes are there in the following binary tree? | CO3 | R | 1 |
| 7. | What is a directed graph? | CO3 | R | 1 |
| 8. | Specify the time complexity of Dijikstra's algorithm. | CO5 | R | 1 |
| 9. | Analyze the following graph and find the degree of vertex C | CO3 | An | 1 |
| 10. | What is a problem instance? | CO6 | R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11. | Analyse the following linked list and answer the given questions. If the head node is 1005, then Write the address of Tail node. Write the address of the location in which data N is stored. Write the address of Successor node of 1009. |  |  |  | CO4 |  | An |  | 3 |
| 12. | Define the following - Queue - Enqueue - Dequeue |  |  |  | CO3 |  | R |  | 3 |
| 13. | Construct Binary tree for following Array = {1, 3, 5, 4, 6, 13, 10, 9, 8, 15, 17} |  |  |  | CO3 |  | A |  | 3 |
| 14. | Develop the adjacency matrix for the following graph |  |  |  | CO3 |  | A |  | 3 |
| 15. | Show sequential array representation for the following tree |  |  |  | CO3 |  | U |  | 3 |
| 16. | Name the components of Greedy Algorithm. |  |  |  | CO6 |  | R |  | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |  |  |  |  |
| 17. |  | a. | Explain different types of linked list with proper diagram. | CO4 |  | U |  | 6 |  |
|  |  | b. | Explain different types of time complexity. | CO1 |  | U |  | 6 |  |

| 18. | a. | Define the following stack related operations PUSH POP Over flow condition Under flow condition | CO3 | R | 8 |
| --- | --- | --- | --- | --- | --- |
|  | b. | Find the value for the following postfix notation 2 4 3 + * 5- | CO3 | R | 4 |
| 19. | a. | Explain Bubble sort algorithm with example | CO2 | U | 8 |
|  | b. | Construct Max heap For the following binary heap tree | CO2 | A | 4 |
| 20. |  | Explain the following with proper diagram Full binary tree Complete binary tree Perfect binary tree Balanced Binary tree | CO3 | U | 12 |
| 21. |  | Explain Breadth First Search algorithm with one example. | CO2 | U | 12 |
| 22. |  | Find the cost of minimum spanning tree (MST) of the given graph, Using Prim’s Algorithm | CO5 | R | 12 |
| 23. |  | Explain Dijkstra algorithm with example. | CO5 | U | 12 |
|  |  | Compulsory: |  |  |  |
| 24. | a. | Illustrate Knapsack Problem in detail. | CO6 | U | 6 |
|  | b. | What are the characteristics of an algorithm? | CO6 | R | 6 |