Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # # 09EE101 / 12EE101 / DEE101 / EE101 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # BASIC ELECTRICAL ENGINEERING | # Max. Marks : | # 100 |

| Q. No. | Questions |  | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | The SI unit for power is _________. | CO1 / U |  | 1 |
| 2. | The conductance of an 8 resistance is _______ mho. | CO1 / E |  | 1 |
| 3. | The total number of lines of force in the magnetic field is called_________. | CO1 / R |  | 1 |
| 4. | Define magnetic flux density. | CO1 / R |  | 1 |
| 5. | ___________ raises the temperature of the steam that comes out the boiler in a thermal power station. | CO1 / U |  | 1 |
| 6. | Form factor and peak factor for a sinusoidal wave are _____ and ______. | CO1 / R |  | 1 |
| 7. | The function of commutator in a DC Generator is to __________________. | CO2 / AN |  | 1 |
| 8. | _________ and _________ are the two main parts of a DC motor. | CO2 / U |  | 1 |
| 9. | In ____________ wiring a single lamp is controlled by two two-way switches | CO3 / R |  | 1 |
| 10. | Induction type energy meter is an indicating instrument. State true or false. | CO3 / U |  | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | State Ohm’s Law. | CO1 /U | 3 |
| 12. | Describe the Law’s of Electromagnetic Induction | CO1 / R | 3 |
| 13. | An alternating voltage is given by the equation v=225Sin 300. Find the: a) RMS value; b) Frequency. | CO1 / AP | 3 |
| 14. | Give any three comparisons between overhead and underground distribution systems. | CO1 / R | 3 |
| 15. | Mention the applications of DC series, shunt motors. | CO2 / U | 3 |
| 16. | Write down the types of wiring. | CO3 / U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | a. | Explain the concept of parallel connection of resistances. | CO1 / AN | 6 |  |
|  | b. | A resistance R is connected in series with a parallel circuit comprising of two resistors 12 and 8 respectively. The total power dissipated in the circuit is 70 W when the supplied voltage is 22 volts. Calculate the value of R. | CO1 / E | 6 |  |
| 18. | a. | A toroidal air cored coil with 1500 turns has a mean radius of 20 cm, the diameter of each turn being 5 cm. If the current in the coil is 8A, calculate flux and flux density | CO1 / E | 6 |  |
|  | b. | Discuss about mutual inductance in detail with necessary equations. | CO1 / AN | 6 |  |
| 19. |  | With a neat layout, explain the power generation process in a thermal power station. | CO1 / AN | 12 |  |
| 20. |  | Explain the construction and working of a DC generator in detail. | CO2 / U | 12 |  |
| 21. | a. | Derive the RMS value and average value of a sinusoidal alternating voltage quantity. | CO1 / AN | 8 |  |
|  | b. | Draw the one-line power system diagram. | CO1 / R | 4 |  |
| 22. |  | Sketch the construction diagram and explain the three-phase induction motor working. | CO2 / R | 12 |  |
| 23. |  | Discuss in detail about star to delta and delta to star transformation. | CO2 / U | 12 |  |
|  |  | Compulsory: |  |  |  |
| 24. | a. | With a neat circuit diagram, explain fluorescent lamp wiring. | CO3 / AN |  | 6 |
|  | b. | What is earthing? Explain pipe earthing with a neat sketch. | CO3 / R |  | 6 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # # 09EE201 / 12EE205 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ELECTRONIC CIRCUITS | # Max. Marks : | # 100 |

| Q. No. | Questions | Marks |
| --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |
| 1. | Define intrinsic semiconductor. | 1 |
| 2. | Breakdown voltage is ______________. | 1 |
| 3. | Name the types of junction capacitances. | 1 |
| 4. | The process of adding impurities to an intrinsic semiconductor is called __________. | 1 |
| 5. | Give the expression for ripple factor (r) and its value incase for a full wave rectifier. | 1 |
| 6. | Compare half wave rectifier and full wave rectifier. | 1 |
| 7. | Define efficiency of a half wave rectifier along with its maximum value. | 1 |
| 8. | The gradual flow of charge from a region of high density to a region of low density is ____________. | 1 |
| 9. | Mention any two applications of Hall effect. | 1 |
| 10. | List the advantages of Zener regulator. | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |
| --- | --- | --- |
| 11. | Explain the need for biasing and illustrate how Q point is fixed on a DC load line. | 3 |
| 12. | Define the conversion efficiency of a power amplifier? | 3 |
| 13. | Derive 3-dB frequency of a high pass RC circuit. | 3 |
| 14. | Design an integrator for an input frequency of 1kHz. | 3 |
| 15. | Draw the circuit diagram of a Colpitts oscillator | 3 |
| 16. | Classify power amplifiers based on collector current waveform. | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |
| --- | --- | --- | --- |
| 17. |  | Draw the circuit diagram of class A series fed power amplifier and prove that the conversion efficiency is 50% by using transformer coupling. | 12 |
| 18. |  | A high pass RC circuit has a 3dB cut off frequency of 10Hz. Plot the output waveform of the circuit, if a 20Hz symmetric square wave with 2V peak to peak is applied to it. Mark the time and voltage levels accurately. | 12 |
| 19. |  | A silicon transistor with β=50 is used in a voltage divider bias circuit with VBE=0.6V, VCC=22.5V and RC=5.6K. It is desired to establish Q point at (8.2V, 2.3mA) and required stability factor is . Design the voltage divider circuit. | 12 |
| 20. |  | Draw the circuit of Cascode amplifier. Derive the expressions for midband gain and pole frequencies. | 12 |

| 21. |  | Draw the circuit diagram of a class C power amplifier and explain its working with output waveforms. How a distortionless output is obtained in the collector of a Class C power amplifier. | 12 |
| --- | --- | --- | --- |
| 22. |  | Draw the circuit diagram of a Wien bridge oscillator. Explain how Barkhausen criterion for oscillation is satisfied by the circuit and derive an expression for the frequency of oscillation. | 12 |
| 23. |  | How is short circuit protection provided in series voltage regulator. | 12 |
|  |  | Compulsory: |  |
| 24. |  | Explain the working of an astablemultivibrator with necessary base and collector waveforms. | 12 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 09EE202 / 12EE220 / EE257 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # DIGITAL ELECTRONICS | # Max. Marks : | # 100 |

| Q. No. | Questions |  | Marks |  |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. |  | Interpret the truth table of EX- OR gate. |  | 1 |
| 2. |  | Apply De-Morgan’s theorem to simplify A + BC . |  | 1 |
| 3. |  | Find the canonical POS form of Y= A+B’C |  | 1 |
| 4. |  | Define ‘Minterm’ and ‘ Maxterm’. |  | 1 |
| 5. |  | Convert the given decimal numbers to their binary equivalent 108.364 |  | 1 |
| 6. |  | Convert (115)10 and (235)10 into hexadecimal numbers. |  | 1 |
| 7. |  | Show how to connect NAND gates to get an AND gate and OR gate? |  | 1 |
| 8. |  | State Distributive law and Duality principle. |  | 1 |
| 9. |  | Draw the truth Table of Half Adder |  | 1 |
| 10. |  | Differentiate Multiplexer and Demultiplexer |  | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |
| --- | --- | --- |
| 11. | Find the minimized Boolean expression of this function F=XY+X(Y+Z) +Y(Y+Z). | 3 |
| 12. | Implement Y= (1,4,5,6,7) in SOP form using AOI logic. | 3 |
| 13. | Prove the Boolean theorems x + x = x, x . x = x . | 3 |
| 14. | Distinguish between EPROM and EEPROM. | 3 |
| 15. | Draw the circuit diagram of a Colpitts oscillator | 3 |
| 16. | Define State Reduction and State Assignment. | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |
| --- | --- | --- | --- |
| 17. |  | Simplify The Following Boolean Expressions To A Minimum Number Of Literals: (i) AC + ABC + A C (ii) XYZ + XY + XY Z (iii) XY + YZ + X Y Z (iv) A B + ABD + AB D + A C D + A B C (v) BD + BC D + A B C D | 12 |
| 18. |  | Implement the following Boolean function with 16 × 1 Multiplexer. F ( A , B , C , D ) = ∑ (0, 1, 3, 4, 8, 9,15 ) | 12 |
| 19. |  | Construct a 5 to 32 line decoder with four 3 to 8 line decoders with enable and a 2 to 4 line decoder. | 12 |
| 20. |  | With suitable block diagram explain Binary multiplier. | 12 |
| 21. |  | Explain the operation of a Master slave JK flip-flop. What is meant by race around condition? | 12 |
| 22. |  | Design a 4 bit Carry look ahead adder. | 12 |
| 23. |  | Design a 3 bit asynchronous counter using JK flip flops. | 12 |

|  |  | Compulsory: |  |
| --- | --- | --- | --- |
| 24. |  | Simplify the following Boolean function F using Karnaugh map method: (i) F(A, B, C, D)= ∑ (1,4,5,6,12,14,15) (ii) F(A, B, C, D) = ∑ (0,1,2,4,5,7,11,15) (iii) F(A, B, C, D) = ∑ (2,3,10,11,12,13,14,15) | 12 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # # 09EE208 / 11EE240 / 11EE249 / 12EE239 / EE294 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # AUTOMOTIVE ELECTRONICS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | External combustion engines Since the fuel is injected into or close to the cylinder, __________ of manifold is being avoided. | CO1 / R | 1 |
| 2. | Computation of Air to Fuel Ratio is performed by obtaining the signal from which sensor? | CO2 / R | 1 |
| 3. | Output parameter of the Vehicle speed sensor ___________ with Increase in vehicle Speed. | CO3 / R | 1 |
| 4. | The quantity of fuel sent to the intake manifold of the engine is determined by sensing the mass or volume of air flowing in to the intake manifold using which sensor. | CO2 / U | 1 |
| 5. | Accurate precision and position control is achieved using ____________ motor. | CO1 / A | 1 |
| 6. | Magneto ignition system has a poor quality of spark during starting (T/F) | CO2 / U | 1 |
| 7. | In a distributor less electronic ignition control system ___________ is not required. | CO2 / U | 1 |
| 8. | In Negative Earthing, which portion of the spark plug gets corroded. | CO1 / R | 1 |
| 9. | In an electronic fuel injection, the maintenance cost is ___________. | CO2 / U | 1 |
| 10. | _____________ type of motor is preferred for throttle control. | CO1 / U | 1 |

| PART – B (6 X 3 = 15 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Discuss the need for Lambda sensor | CO1 / U | 3 |
| 12. | Give the correct sequence in which current flows in a battery ignition system | CO2 / U | 3 |
| 13. | Acceleration enrichment is used forwhat purpose? | CO1 / U | 3 |
| 14. | Define TCS | CO3 / R | 3 |
| 15. | In short write about the advantage of ADAS | CO3 / R | 3 |
| 16 | Differentiate a stepper motor and a servo motor | CO2 / R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. |  | With a block diagram explain the basic sensor arrangement in an automotive system | CO1 / U | 12 |
| 18. |  | Explain with a neat sketch the following actuators a) Stepper Motor b) Linear Actuator | CO1 / R | 12 |
| 19. |  | Explain the various components of a starting system with relevant diagram and the principle of operation of a cranking motor | CO2 / U | 12 |
| 20. |  | Elaborate the role of power electronic converters that are used for digitizing the ignition system. Justify the merits of the same | CO2 / U | 12 |
| 21. |  | With a neat block diagram explain the operation and advantage of Electronics Steering system | CO3 / U | 12 |
| 22. |  | Discuss about the various stratergies followed to avoid glare during night driving | CO3 / R | 12 |
| 23. |  | Discuss the various steps that are involved in developing a wireless TPMS. Discuss about the impact of TPMS created over the vehicle performance. | CO3 / U | 12 |
|  |  | Compulsory: |  |  |
| 24. |  | Elaborately comment on the following automotive systems: a) ADIS b) ABS | CO3 / U | 12 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 10EE201 / 12EE201 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ELECTRIC CIRCUITS AND NETWORKS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | State Kirchoff’s voltage law. | CO2 | U | 1 |
| 2. | Write down the equations for Star-Delta conversion. | CO4 | A | 1 |
| 3. | Define Resonance. | CO2 | U | 1 |
| 4. | State the relationship between line and phase values of Delta connection. | CO4 | An | 1 |
| 5. | Represent a practical current source in terms of an ideal current source. | CO1 | R | 1 |
| 6. | ___________ is the value of coefficient of coupling of an ideal transformer. | CO1 | R | 1 |
| 7. | State Dot rule for coupled circuits. | CO3 | A | 1 |
| 8. | State Reciprocity theorem. | CO3 | A | 1 |
| 9. | Write an expression for the critical resistance of a series RLC circuit. | CO4 | An | 1 |
| 10. | A series RC circuit with resistance 20 ohm and capacitance 0.5 Farad will have the time constant ___________. | CO3 | A | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Two watt meters connected to measure three phase power indicates 1500W & 500W, respectively. Calculate the Power Factor of the circuit. | CO3 | A | 3 |
| 12. | Write the voltage division rule for a D.C. network. | CO2 | U | 3 |
| 13. | Obtain the value of the equivalent inductance, for the given circuit | CO3 | A | 3 |
| 14. | Use Superposition to find the current I in the circuit shown in Fig. | CO4 | An | 3 |

| 15. | State and explain Millman’s theorem. | CO2 | U | 3 |
| --- | --- | --- | --- | --- |
| 16. | Write the transient current equation of a series RC circuit excited with D.C excitation of V=20 volt. The values of R= 20 ohm and C= 0.2 farad. | CO5 | E | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. |  | Determine the equivalent resistance between terminals A and B in Figure. | CO3 | A | 12 |
| 18. |  | Determine the mesh currents in the circuits given below. | CO4 | An | 12 |
| 19. |  | Find the voltage across the 5ohm resistor. | CO3 | A | 12 |

| 20. |  | Determine the Norton’s equivalent circuit across AB for the circuit shown in figure below. | CO5 | E | 12 |
| --- | --- | --- | --- | --- | --- |
| 21. |  | State maximum power transfer theorem and derive condition for maximum power transfer in a complex circuit. Illustrate the above with an example | CO2 | U | 12 |
| 22. |  | Three equal impedances, 40 + j30 each, are delta connected to a 230 V rms, three phase circuit. Determine - the line current - phase current b. the total complex power supplied to the two loads c. the power factor of the two loads combined. | CO6 | C | 12 |
| 23. |  | An induction coil of 0.8 power factor lagging is connected in series with a80 micro farad capacitor. The supply frequency is 50 Hz. When this series circuit is connected across a voltage source and measured, it was observed that the voltage across the coil is equal in magnitude to the voltage across the capacitor. Determine the circuit parameters of the coil. | CO3 | A | 12 |
|  |  | Compulsory: |  |  |  |
| 24. |  | In the circuit shown below, capacitor C1 has an initial charge q0 = 500*10-6 coulombs. If the switch is closed at t=0, find the current transient and the voltage of capacitor C1. | CO4 | An | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Identify the circuit elements and different types of circuits. |
| CO2 | Understand the various circuit analysis techniques. |
| CO3 | Apply the various techniques to analyze the electric circuits and networks. |
| CO4 | Analyze the problems of electric circuits and networks for the possible solutions. |
| CO5 | Assess the behavior of various electric circuits and the technique used to analyze it. |
| CO6 | Design circuits which satisfy certain design specification. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 2 |  |  |  |  |  | 2 |
| CO2 |  | 20 |  |  |  |  | 20 |
| CO3 |  |  | 45 |  |  |  | 45 |
| CO4 |  |  |  | 30 |  |  | 30 |
| CO5 |  |  |  |  | 15 |  | 15 |
| CO6 |  |  |  |  |  | 12 | 12 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 10EE202 / 12EE225 / EE260 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # DIGITAL SIGNAL PROCESSING | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Define sampling. | CO2 / R | 1 |
| 2. | Define impulse response. | CO2 / R | 1 |
| 3. | Give any one example for energy signal. | CO2 / R | 1 |
| 4. | How many number of additions and multiplications are required to realize a system H(z) having M zeros and N poles in direct I form realization. | CO3 / R | 1 |
| 5. | The poles of butter worth filter lie on _______________. | CO3 / R | 1 |
| 6. | Mention the condition for the impulse response of FIR filter to satisfy for constant group and phase delay. | CO3 / R | 1 |
| 7. | Define Quantization. | CO2 / R | 1 |
| 8. | State the two types of limit cycle oscillations. | CO2 / R | 1 |
| 9. | Mention the factors that influence selection of digital signal processors. | CO1 / R | 1 |
| 10. | Define Pipelining. | CO1 / R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Find whether the signal x(n)=(0.5)n u(n) is an energy signal or power signal. | CO2 / U | 3 |
| 12. | Find the circular convolution of x(n)=[1,2,3,4] and h(n)=[4,3,2,1] | CO2 / U | 3 |
| 13. | Calculate the number of multiplications and additions needed in the calculation of DFT and FFT with 32 point sequence. | CO2 / U | 3 |
| 14. | Compare IIR and FIR filter. | CO3 / U | 3 |
| 15. | What are the quantization errors due to finite word length registers in digital filters? | CO3 / U | 3 |
| 16. | # What are the advantages and disadvantages of VLIW architecture? | CO1 / U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | # Check if the following system linear, shift invariant and causal. # y(n)=3y2(n-1)-nx(n)+4x(n-1)-2x(n+1) | CO2 / A | 9 |
|  | b. | Find the convolution of two finite duration sequences. and | CO2 / A | 3 |
| 18. |  | Determine the impulse response of the following system. y(n)+4y(n-1)+4y(n-2)=x(n) | CO2 / A | 12 |
| 19. |  | # Find 8-point DFT of the sequence x(n)={1,0,0,0,0,0,0,0} using DIF algorithm. | CO2 / An | 12 |
| 20. |  | # Find the DFT of the following sequence x using the DIT FFT algorithm. # x= (1, -1, -1, -1, 1, 1, 1, -1) | CO2 / An | 12 |
| 21. |  | Determine the variance of round off noise at the output of cascade realization of the filter with the system function H(z) = H1(z) H2(z) where H1(z) = ; H2(z) = . | CO3 / A | 12 |
| 22. |  | Design an Low Pass Filter using rectangular window if N=9 whose desired frequency response is | CO3 / An | 12 |
| 23. |  | For the given specifications, design an analog Butterworth filter. 0.9 ≤ ¦ H (ejω) ¦ ≤ 1, 0 ≤ Ω≤ 0.2π; ¦ H (ejω) ¦ ≤ 0.2, 0.4π ≤ Ω ≤ π. | CO3 / An | 12 |
|  |  | Compulsory: |  |  |
| 24. |  | # Explain in detail about Von Newmann architecture. | CO1 / R | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Students will be able to select appropriate DSP processors for particular applications |
| CO2 | Apply various transformation techniques for signal processing applications |
| CO3 | Design filters for different real time applications |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 14 | 3 |  |  |  |  | 17 |
| CO2 | 5 | 9 | 24 | 24 |  |  | 62 |
| CO3 | 3 | 6 | 12 | 24 |  |  | 45 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 11EE203 / 12EE203 / EE247 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ELECTRON DEVICES | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | If the PN junction is forward biased the depletion region width ____________. | CO1/U | 1 |
| 2. | A Zener Diode is used as a _____________. | CO1/A | 1 |
| 3. | BJT is a ____________ controlled device. | CO1/R | 1 |
| 4. | FET is a unipolar device. Say true or false. | CO1/U | 1 |
| 5. | Name the four regions in JFET operation. | CO1/R | 1 |
| 6. | Draw the structure of P-channel junction field effect transistor. | CO2/R | 1 |
| 7. | Write the expression for drain current of MOSFET. | CO2/U | 1 |
| 8. | Sketch the symbol of Enhancement MOSFET. | CO2/U | 1 |
| 9. | What is oxidation? | CO3/R | 1 |
| 10. | Mention any three IC packages. | CO3/U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | List the applications of PN junction diode. | CO1/R | 3 |
| 12. | Explain the difference between the structure of an NPN and a PNP transistor. | CO1/A | 3 |
| 13. | Write about the operation of JFET. | CO2/R | 3 |
| 14. | Summarize the advantages of CMOS. | CO2/R | 3 |
| 15. | Why buried layer is used in integrated circuits? | CO3/A | 3 |
| 16. | Write short notes on ion implantation. | CO3/R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | Sketch the voltage / current characteristics for a PN junction diode under forward and reverse biasing. Show how temperature change affects the characteristics. | CO1/R | 12 |
| 18. | a. | With a neat diagram, explain the input and output characteristics of Common Emitter configured BJT. Also mention the applications of the transistor. | CO1/U | 12 |
| 19. | a. | Draw and explain the small signal model of a FET. | CO2/A | 6 |
|  | b. | Explain how FET acts as an amplifier. | CO2/U | 6 |
| 20. | a. | Outline the V-I and transfer characteristics of the JFET and explain in detail with necessary diagrams. | CO2/U | 12 |

| 21. | a. | Summarize the construction, basic operation and characteristics of depletion type MOSFET. | CO2/U | 12 |
| --- | --- | --- | --- | --- |
| 22. | a. | Mention the advantages of N-Channel MOSFET over P-Channel MOSFET. | CO2/A | 6 |
|  | b. | Explain how MOSFET acts as a switch. | CO2/U | 6 |
| 23. | a. | What is the need of biasing circuit in BJT? Explain the various biasing methods with relevant circuit diagram. | CO1/U | 12 |
|  |  | Compulsory: |  |  |
| 24. | a. | Explain the various steps involved in the fabrication of ICs with suitable diagrams. | CO3/U | 6 |
|  | b. | Explain epitaxial growth in IC fabrication. | CO3/U | 6 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 11EE206 / 12EE202 / EE248 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ELECTROMAGNETIC FIELDS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | If the curl of a vector field is zero then the field is known as______. | CO1/R | 1 |
| 2. | Represent a point P0.5, 0.2, 0.1in terms of unit vectors along the coordinate axes. | CO1/R | 1 |
| 3. | Give the scalar form of Coulomb’s law. | CO2/R | 1 |
| 4. | Give the expression for Potential due to Electrical Dipole. | CO2/R | 1 |
| 5. | Express Gauss’s law for electrostatic field in point form. | CO3/U | 1 |
| 6. | Magnetostatic field is produced by __________ current. | CO3/U | 1 |
| 7. | The conduction current for electric field is _________. | CO4/U | 1 |
| 8. | What is generator e.m.f? | CO4/U | 1 |
| 9. | In wave equation for homogenous medium the static charges is __________. | CO5/A | 1 |
| 10. | The distance the wave travels to complete one revolution is _________. | CO6/A | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | State divergence theorem and give its mathematical form. | CO1/R | 3 |
| 12. | An electric dipole located at origin in free space has a moment. Find V at (2,3,4)m | CO2/A | 3 |
| 13. | A 200 turns rectangular coil 30 cm by 15 cm with a current of 5A is placed in a uniform magnetic field of 0.2 Tesla. Find the magnetic moment and the maximum torque. | CO3/An | 3 |
| 14. | Define the relationship between Jc and Jd. | CO4/U | 3 |
| 15. | A Faraday’s disc generator has a diameter of 0.7m and is rotated at a rate of 1500 rpm.The disc experiences a magnetic field of 0.02 milli Tesla.Calculate the e.m.f produced. | CO5/A | 3 |
| 16. | Find the velocity of plane wave in a lossless medium with εr = 4, μr = 1? | CO6/An | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | Obtain the expression for the volume of a sphere of radius R from the differential volume. | CO1/A | 6 |
|  | b. | Draw the differential elements in rectangular and cylindrical coordinate systems. Write the expressions for differential length, surfaces and volume. | CO1/U | 6 |
| 18. | a. | A cylindrical capacitor consists of an inner conductor of radius ‘a’ and an outer conductor whose inner radius is ‘b’. The space between the conductors is filled with a dielectric of permittivity εr and the length of the capacitor is L.Determine the capacitance of this capacitor. | CO2/A | 12 |
| 19. | a. | Find the expression for magnetic field strength due to a rectangular loop. | CO3/An | 12 |
| 20. | a. | Distinguish between Transformer and Motional emfs. | CO4/U | 6 |
|  | b. | Derive the expression for emf produced by Faraday’s disc generator. | CO4/A | 6 |
| 21. | a. | Derive Maxwell’s equation in integral and differential forms. | CO5/A | 12 |
| 22. | a. | Find the expression for force between two current carrying conductors. | CO4/A | 6 |
|  | b. | A 100-turn square coil of side 10 cm with current 3 A, is in an uniform magnetic field of 1 Tesla. Find the magnetic moment and the maximum torque. | CO4/A | 6 |
| 23. | a. | Find the expression for magnetic field intensity due to an infinite long straight conductor carrying a current of I amperes at a point which has the shortest distance of ‘a’ from the points. | CO3/A | 12 |
|  |  | Compulsory: |  |  |
| 24. | a. | Derive the Poynting’s theorem. Give the interpretation for each term. | CO6/An | 6 |
|  | b. | Derive the expression for the attenuation constant, phase constant and intrinsic impedance for a uniform plane wave in a good conductor | CO6/An | 6 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 11EE207 / 12EE207 / EE250 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # DC MACHINES AND TRANSFORMERS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Outline armature reaction. | CO1 / U | 1 |
| 2. | Name the types of windings employed in the armature of a DC generator? | CO1 / R | 1 |
| 3. | Illustrate speed – torque characteristics of DC series motor. | CO2 / U | 1 |
| 4. | Mention the purpose of interpoles. | CO2 / R | 1 |
| 5. | Why the rating of transformer is in KVA? | CO1 / R | 1 |
| 6. | Show the condition for maximum efficiency of transformer. | CO2 / U | 1 |
| 7. | Can the voltage regulation of a transformer go to negative? If so, under what condition. | CO3 / An | 1 |
| 8. | List the functions of no-load current in a transformer. | CO2 / U | 1 |
| 9. | For small, high voltage 3-phase transformer which connection is preferred? | CO3 / A | 1 |
| 10. | Spell the distribution transformer is single phase or three phase? | CO2 / U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Identify the causes of sparking at the brushes of DC machines? | CO2 / A | 3 |
| 12. | Is it possible to conduct Swinburne’s test on DC series motor – Justify. | CO3/ An | 3 |
| 13. | State the need for parallel operation of transformers. | CO1 / R | 3 |
| 14. | The efficiency of a transformer is always higher than that of rotating electrical machines. Why? | CO1 / A | 3 |
| 15. | Define all-day efficiency of transformer? | CO1 / R | 3 |
| 16. | List the conditions to be satisfied for successful operation of 3-phase transformers? | CO1 / R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | With a neat schematic diagram, explain the construction and working of DC Generator. | CO1 / U | 12 |
| 18. | a. | Derive the emf equation of DC generator. | CO1 / U | 6 |
|  | b. | A 4 pole, lap connected DC shunt generator has a useful flux per pole of 0.07Wb. The armature winding consists of 220 turns each of 0.004 resistance. Calculate the terminal voltage when running at 900 rpm, if the armature current is 50 A. | CO2 / A | 6 |

| 19. | a. | Describe with the aid of a circuit diagram the Hopkinson’s test for a pair of DC shunt machines. State the advantages and disadvantages of this method. | CO1 / U | 12 |
| --- | --- | --- | --- | --- |
| 20. | a. | A 1-phase transformer with ratio of 440/110V taken a no-load current of 5A at 0.2 power factor lagging. If the secondary supplies a current of 120A at a p.f of 0.8 lagging, estimate the current taken by the primary and its power factor. Draw the necessary phasor diagram. Also calculate the iron losses. | CO2 / A | 12 |
| 21. | a. | With a neat diagram explain how back – to – back test is conducted on two identical single phase transformers. Mention the merits and demerits of this test. | CO3 / U | 12 |
| 22. | a. | Explain the principle of operation of an auto-transformer. | CO1 / U | 6 |
|  | b. | Derive the expression for saving of copper in using auto-transformer. | CO2 / U | 6 |
| 23. | a. | A DC motor takes an armature current of 110 A at 480V. The armature circuit resistance is 0.2. The machine has 6 poles and armature is cap connected with 864 conductors. The flux per pole is 0.05 wb. Calculate the speed and gross torque. | CO2 / A | 6 |
|  | b. | Calculate the number of conductors required on the armature of a 4 pole generator to generate an emf of 480 volts at 750 rpm. The flux per pole is 0.018 webers and the armature winding has two parallel paths. | CO2 / A | 6 |
|  |  | Compulsory: |  |  |
| 24. |  | With neat diagram explain the construction and working of 3-phase transformer. Mention the different 3 – phase transformer connections. | CO1 / U | 12 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 11EE209 / 12EE209 / EE251 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # INDUCTION AND SYNCHRONOUS MACHINES | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Specify the advantage of slip ring rotor over squirrel cage rotor. | CO1 / R | 1 |
| 2. | "Three phase induction motor is known as generalized transformer". Justify. | CO1 / U | 1 |
| 3. | Outline the function of centrifugal switch in single phase induction motor. | CO2/U | 1 |
| 4. | What does hunting of synchronous motor mean? | CO2 / R | 1 |
| 5. | Mention the type of synchronous generators are used in Hydroelectric plants. | CO1 / R | 1 |
| 6. | Write down the equation for frequency of emf induced in an alternator. | CO3 / U | 1 |
| 7. | When does a synchronous motor get over excited? | CO3 / U | 1 |
| 8. | When a winding is made with coil span less than full pitch, the winding is called as __________ winding. | CO1 / U | 1 |
| 9. | Define the term voltage regulation of alternator. | CO2 / U | 1 |
| 10. | Synchronous reactance is given by ___________. | CO2 / U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Differentiate slip speed and slip in an induction motor. | CO3 / U | 3 |
| 12. | List the applications of repulsion motor. | CO1 / R | 3 |
| 13. | Alternators are rated in kVA and not in kW-Justify. | CO2 / An | 3 |
| 14. | Identify the causes of changes in voltage of alternators when loaded. | CO3 / R | 3 |
| 15. | Illustrate V-curves and inverted V-curves. | CO3 / R | 3 |
| 16. | Show the necessity for predetermination of voltage regulation. | CO2 / R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | Explain the construction and torque-slip characteristics of three phase induction motor? | CO2 / U | 12 |
| 18. | a. | A 3 phase, 4 pole 50Hz induction motor at standstill has 180V induced across its star connected terminals. The rotor resistance and standstill reactance per phase are 0.6Ω and 0.3Ω respectively. Calculate the speed when the rotor is drawing a current of 6A at a particular load. Also calculate the speed at which the torque is maximum and the corresponding value of the rotor. | CO3 / A | 6 |
|  | b. | Derive the torque equation for three phase induction motor? | CO3 / U | 6 |

| 19. | a. | Single phase induction motor is not self-starting. Justify and give detailed explanation with the help of double field revolving theory. | CO1 / An | 12 |
| --- | --- | --- | --- | --- |
| 20. | a. | Explain the working principle of single phase induction motor. Develop equivalent circuit of a single phase induction motor ignoring core losses. | CO2 / U | 12 |
| 21. | a. | Describe with neat sketches the constructional details of a salient pole type alternator. Discuss briefly the load characteristics of alternator for different power factor. | CO2 / U | 12 |
| 22. | a. | An over excited synchronous motor is called a synchronous condenser. Explain. | CO2 / U | 12 |
| 23. | a. | A 12 pole, 50Hz, 3 phase induction motor has rotor resistance of 0.15 Ω and stand still reactance of 0.25 Ω per phase. On full load, it is running at a speed of 480 rpm. The rotor induced emf per phase at standstill is observed to be 32 V. Calculate: (1) Starting torque (2) Full load torque (3) Maximum torque (4) Speed at maximum torque. | CO3 / A | 6 |
|  | b. | List the different method of speed control of 3 phase induction motor and explain any one method in detail. | CO1 / U | 6 |
|  |  | Compulsory: |  |  |
| 24. |  | Explain Blondel’s two-reaction theory for a salient pole synchronous machine. Derive an expression for power developed as a function of load angle. | CO2 / U | 12 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 11EE211 / 12EE211 / EE252 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ELECTRICAL MACHINE DESIGN | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Classify the Transformer according to construction. | CO1/ R | 1 |
| 2. | Define mmf of a coil. | CO1/ R | 1 |
| 3. | The range of specific electric loading is _____________. | CO1/ R | 1 |
| 4. | List out the advantages of higher number of poles in the number of poles selection procedure in DC machine. | CO1/ R | 1 |
| 5. | Write down the percentage of silicon used to make the transformer stampings. | CO3/ R | 1 |
| 6. | Write the expression for output equation of a single phase transformer | CO3/ R | 1 |
| 7. | Define dispersion coefficient of Inductor motor. | CO2/ R | 1 |
| 8. | Write the formula for length of mean turn of stator of induction motor. | CO2/ R | 1 |
| 9. | Write the range of Short circuit ratio of turbo alternators. | CO2/ R | 1 |
| 10. | List out the various types of synchronous machines. | CO2/ R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Define specific electric loading. | CO1/ R | 3 |
| 12. | List the advantages of choosing larger number of poles in DC machines. | CO1/ R | 3 |
| 13. | Describe the necessity of cooling tubes in the transformer. | CO3/ U | 3 |
| 14. | What are the ranges of efficiency and power factor in different types of induction motor? | CO2/ U | 3 |
| 15. | Define dispersion coefficient. | CO2/ R | 3 |
| 16. | Write about the uses of interpoles. | CO2/ R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. |  | Discuss how the specific electric and magnetic loadings will be chosen for rotating machine. | CO1/A | 12 |
| 18. |  | Discuss about the steps involved in the design of commutator and brushes for the DC machine. | CO2/U | 12 |
| 19. |  | Discuss the design of field system of a DC motor. | CO2/U | 12 |

| 20. |  | Derive the output equation of single phase Transformer and three phase transformer. | CO3/ U | 12 |
| --- | --- | --- | --- | --- |
| 21. |  | Discuss the step by step procedure to design the rotor of a squirrel cage Induction Motor | CO2/U | 12 |
| 22. |  | A1000 kVA, 3300V, 50Hz, 300rpm, 3phase alternator has 180 slots with 5 conductors per slot. Single layer winding with full pitch coil is used. The winding is star connected with one circuit per phase. Determine the specific electric and specific magnetic loadings, if the stator core is 2.0m and the core length is 0.4m. Using the same loadings, determine the corresponding data for a 1250 kVA, 330V, 50 Hz, 250 rpm, 3 phase star connected alternator having 2 circuits per phase. The machines have 60◦ phase spread | CO2/A | 12 |
| 23. |  | Derive the output equation of an AC machine | CO2/ U | 12 |
|  |  | Compulsory: |  |  |
| 24. |  | Write about short circuit ratio to determine the performance of the AC machine. | CO2 / U | 12 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 11EE212 / 12EE212 / EE254 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # POWER ELECTRONICS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Which power device combines the advantages of BJT and MOSFET? | CO1 / R | 1 |
| 2. | Sum of the firing angles of converter 1 and converter 2 of dual converter is _________________. | CO1 / R | 1 |
| 3. | The latching current is _________than the holding current. | CO1 / U | 1 |
| 4. | For a f/4 step down cycloconverter, if the supply frequency is 60Hz, then the frequency of the output voltage is________________ | CO2 / U | 1 |
| 5. | Which class/type of chopper is a four quadrant chopper? | CO2 / R | 1 |
| 6. | In a step down chopper, if Vs = 100 V and the chopper is operated at a duty cycle of 75 %. Find the output voltage. | CO2 / A | 1 |
| 7. | Define Modulation index. | CO2 / R | 1 |
| 8. | In ______________pulse width modulation method, sinusoidal wave is compared with a triangular type of wave. | CO2 / A | 1 |
| 9. | Write down the equipment used in hospital intensive care unit where temporary power failure can cause lot of inconvenience. | CO3 / A | 1 |
| 10. | In HVDC transmission lines, one acts as a converter and other as an inverter. Say True or False | CO3 / R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Compare Thyristor and GTO. | CO1 / U | 3 |
| 12. | Outline the significance of freewheeling diode. | CO1 / U | 3 |
| 13. | A type – A chopper has Vdc = 200 V, R = 10 Ohms. If the duty cycle is 0.4, calculate average voltage Vavg and rms voltage Vrms | CO2 / A | 3 |
| 14. | Brief out the control strategies of AC Voltage controllers? | CO2 / U | 3 |
| 15. | Compare Voltage Source Inverter and Current Source Inverter. | CO3 / U | 3 |
| 16. | # What will be the backup time of a UPS if it is backed by 150Ah, 12V, battery driving a load of 150W? | CO3 / U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | # With neat diagrams describe the switching characteristics of IGBT. | CO1 / R | 6 |
|  | b. | Elucidate the reverse recovery characteristics of a Power Diode with neat diagram. | CO1 /U | 6 |
| 18. | a. | Explain the static and switching characteristics of Thyristor in detail with neat diagrams. | CO1 / R | 12 |
| 19. | a. | # Explain the operation of a single phase fully controlled full bridge converter with RL load in rectification mode with neat diagram and waveforms. | CO2/U | 12 |
| 20. | a. | # Discuss the working of the single phase ac to ac full wave voltage controller with R Load, with necessary circuit, equations and waveforms. | CO2 /A | 12 |
| 21. | a. | With relevant circuit diagram and waveforms explain the operation of the step down chopper with R load and derive the expression for the average value of the load voltage, and load currents. | CO2 /U | 12 |
| 22. | a. | Draw the circuit diagram and waveforms for single phase full bridge inverter and explain in detail. Also obtain the expression for RMS output voltage, and instantaneous output voltage. | CO2/ A | 12 |
| 23. | a. | With relevant circuit diagram and waveforms explain the operation of three phase bridge inverter in 180 mode conduction. Derive the expression for the RMS value of phase voltage and line voltage. | CO3/A | 12 |
|  |  | Compulsory: |  |  |
| 24. | a. | # Discuss the working of HVDC systems with relevant circuit diagram. | CO3 / U | 6 |
|  | b. | # With relevant circuit diagram, explain the procedure to generate the pulse signal for AC to DC converter. | CO3 / A | 6 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 11EE215_12EE215_EE253 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | GENERATION, TRANSMISSION AND DISTRIBUTION | # Max. Marks : | # 100 |

| Q. No. | Questions | Marks |
| --- | --- | --- |
| PART – A(10 X 1 = 10 MARKS) |  |  |
| 1. | What is the function of super heater in a thermal electric power plant? | 1 |
| 2. | Define annual plant capacity factor | 1 |
| 3. | In a 3-phase, 4 wire ac system, if the loads are balanced, then the current in the neutral wire is _________. | 1 |
| 4. | Define skin effect. | 1 |
| 5. | What is GMD? | 1 |
| 6. | How capacitance is calculated in a long transmission line? | 1 |
| 7. | The metallic sheath is provided over the insulation to protect the cable from ________. | 1 |
| 8. | By using a guard ring, the string efficiency is __________. | 1 |
| 9. | Give any two advantages of ring main distributor. | 1 |
| 10. | Strain type insulators are used for voltages beyond _________. | 1 |

| PART – B (6X 3= 18 MARKS) |  |  |
| --- | --- | --- |
| 11. | Why is regenerator used in gas turbine power plants? | 3 |
| 12. | Mention the advantages of high transmission voltages. | 3 |
| 13. | Draw the diagram for short transmission line. | 3 |
| 14. | What are the factors affecting corona? | 3 |
| 15. | Define String Efficiency? | 3 |
| 16. | Classify the types of DC distributors. | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |
| --- | --- | --- | --- |
| 17. |  | With a neat diagram, the working principle of Thermal power plant. List the advantages and disadvantages. | 12 |
| 18. |  | With the help of neat sketch showing different circuits, explain the working of a hydroelectric power station. | 12 |
| 19. |  | Deduce an expression for line to neutral capacitance for a 3-phase overhead transmission line when the conductors are (a) symmetrically placed (b) un symmetrically placed but transposed.power station. | 12 |

| 20. |  | Show how regulation and transmission efficiency are determined for medium lines using a nominal T&π methods. | 12 |
| --- | --- | --- | --- |
| 21. |  | In a three phase overhead system each line is suspended by a string of three insulators. The voltages at the top unit, near the tower and middle unit are 10kV and 11 kV respectively. a) Evaluate the ratio of shunt capacitance to self capacitance of each insulator. b) String Efficiency c) Line Voltage. | 12 |
| 22. |  | Draw a cross sectional view, showing the constructional details of a single core low tension cable and explain the purpose of each material used. | 12 |
| 23. |  | Derive the expression for sag in the transmission line a) at equal levels b) at unequal levels. | 12 |
|  |  | Compulsory: |  |
| 24. |  | Sketch the different types of ac and dc distribution systems and derive equations for the same? | 12 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # # 11EE216 / 12EE216 / EE255 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # POWER SYSTEM ANALYSIS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | Given: Base voltage = 11,000V and Base power = 5 MVA.What is the value of base impedance? | CO1 | U | 1 |
| 2. | 1+a+a 2 +a 3 = __________. | CO1 | R | 1 |
| 3. | Number of tree in a branch is ________ number of node in the network. | CO1 | U | 1 |
| 4. | In type 1 modification of Z-bus building algorithm, off diagonal elements are always----------- | CO1 | U | 1 |
| 5. | One of the _________ bus is considered as slack bus in the power flow analysis. | CO2 | R | 1 |
| 6. | What is the need of load flow study? | CO2 | R | 1 |
| 7. | If the fuel cost of the power generating unit F=0.06P2+40P+120, the incremental fuel cost at 40 MW is given by __________. | CO3 | U | 1 |
| 8. | Mention anyone method used to do the unit commitment problem | CO3 | R | 1 |
| 9. | Shunt faults are mainly due to __________. | CO2 | R | 1 |
| 10. | Transient Limit is always__________than the steady state limit. | CO3 | U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Prove that neutral current can flow only if zero –sequence current is present. | CO1 | U | 3 |
| 12. | What are the different types of unsymmetrical fault, which occur in a power system? | CO2 | U | 3 |
| 13. | Differentiate Gauss Seidal and Newton Raphson method in the load flow study? | CO3 | U | 3 |
| 14. | Write the co-ordination equation including losses and explain the terms involved. | CO3 | U | 3 |
| 15. | Define power angle curve and equal area criteria in stability analysis. | CO3 | U | 3 |
| 16. | Write down the property of Y bus. | CO2 | U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. |  | Two generators rated at 10MVA, 11KV and 15MVA, 11KV respectively are connected in Parallel to bus. The bus bar feed two motors rated 7.5MVA and 10 MVA respectively. The rated voltage of the motor is 9 KV. The reactance of each generator is 12% and that of each motor is 15% on their own ratings. Assume 50MVA, 10KV base and draw the reactance diagram. | CO1 | An | 12 |
| 18. |  | Derive the fault current expression for line to ground fault occurs in a power System network. | CO2 | U | 12 |
| 19. |  | Explain the step by step computational procedure for the Newton Raphson method in load flow studies with appropriate expression. | CO3 | U | 12 |
| 20. |  | Briefly explain the Hydro Thermal Scheduling . | CO2 | U | 12 |
| 21. |  | A power plant has three units with the following fuel cost equations: Maximum and minimum loading on each unit is 150 MW and 40 MW, the demand is 275MW. Find the optimal scheduling. | CO3 | An | 12 |
| 22. | a. | Illustrate the needs to do power system analysis. | CO1 | U | 6 |
|  | b. | Write down the significance of the one line diagram. | CO1 | U | 6 |
| 23. |  | Table shows a three bus power system with line impedance. From; To; Line impedance. 1 1 2; 2 3 3; 0.4j 0.3j 0.2j. Bus 1 : Slack bus, V= 1.05<0 p.u. Bus 2 : PV bus, V = 1.0 p.u. Pg = 3 p.u. Bus 3 : PQ bus, Pl = 4 p.u., Ql = 2 p.u. Neglect limits on reactive power generation. Carry out one iteration of load flow solution by Gauss Seidel method. | CO3 | U | 12 |
|  |  | Compulsory: |  |  |  |
| 24. |  | Explain the Runge kutta method of analyzing multi machine power system for stability with a neat flow chart. | CO3 | U | 12 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # # 11EE218_12EE218_EE256 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # POWER SYSTEM PROTECTION AND SWITCHGEARS | # Max. Marks : | # 100 |

| Q. No. | Questions | Marks |
| --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |
| 1. | Mention the parameters used for discriminating fault location in a power system? | 1 |
| 2. | What is a backup protection? | 1 |
| 3. | What is meant by directional relay? | 1 |
| 4. | Mention the methods arc interruption. | 1 |
| 5. | Define restricking voltage. | 1 |
| 6. | What is a surge absorber? | 1 |
| 7. | Write any merits of vacuum circuit breaker. | 1 |
| 8. | In substations, the most commonly used type of arrester is ___________. | 1 |
| 9. | What is meant by earthing? | 1 |
| 10. | Give any two Indian electricity rules. | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |
| 11. | State the applications of CT’s and PT’s. | 3 |
| 12. | What is over current relay? Draw the time-current characteristics of an inverse current relay. | 3 |
| 13. | Mention any two applications of differential relay? | 3 |
| 14. | Define RRRV and the frequency of oscillations | 3 |
| 15. | Differentaite Surge Diveter and Surge Absorber? | 3 |
| 16. | Enumerate the merits and demerits of lightning arresters? |  |

| PART – C (6X 12 = 72MARKS) Answer any five Questions from Q.no 17 to 23) |  |  |  |
| --- | --- | --- | --- |
| 17. |  | Mention the different types of faults? Discuss theconsequence of faults on a power system (State the causes of faults. | 12 |
| 18. |  | Explain with the help of a neat diagram the construction and working of Induction type directional over current relay | 12 |
| 19. |  | Draw and explain the merzprice protection of alternator stator winding. | 12 |
| 20. |  | Briefly explain the arc phenomenon and various methods of arc extinction in a circuit breaker | 12 |

| 21. |  | Construction and working of SF6 breaker with a neat sketch. Give the properties of SF6 gas that makes is more useful for circuit breaking. | 12 |
| --- | --- | --- | --- |
| 22. |  | With the help of a neat diagram explain about the lightning phenomenon. | 12 |
| 23. |  | Describe the construction and working principle of any two types of lightning arrester. | 12 |
|  |  | Compulsory: |  |
| 24. |  | WhatisInsulationco-ordination. ExplainInsulationlevels of sub-station equipment. | 12 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # # 11EE220 / 12EE221 / EE258 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # LINEAR INTEGRATED CIRCUITS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Infer the open-loop gain of the operational amplifier. | CO1 / U | 1 |
| 2. | Define Slew rate. | CO1 / R | 1 |
| 3. | Which operational amplifier circuit is preferred to design an amplifier with gain below 100%? | CO1 / C | 1 |
| 4. | What would be the output of a differentiator circuit that is fed with the DC input signal? | CO1 / An | 1 |
| 5. | Draw the Circuit of an inverting comparator. | CO2 / K | 1 |
| 6. | Mention is the other name of an astable multivibrator. | CO2 / U | 1 |
| 7. | Identify any one of the applications. | CO2 / C | 1 |
| 8. | What fixed voltage regulator IC is ideal for (-5V) regulation? | CO2 / P | 1 |
| 9. | Infer the ADC gives the instantaneous digital output. | CO3 / E | 1 |
| 10. | State the major drawback of R-2R ladder type DAC. | CO3 / U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Draw the pin configurations of IC 741. | CO1 / R | 3 |
| 12. | Distinguish active filters and passive filters. | CO1 / U | 3 |
| 13. | Sketch the fundamental log amplifier and write its output expression. | CO2 / C | 3 |
| 14. | Compare IC 723 voltage regulator with Fixed voltage regulators. | CO2 / U | 3 |
| 15. | Define capture range of a PLL. | CO3 / U | 3 |
| 16. | Discuss the advantage of inverted R-2R ladder DAC over R-2R ladder type DAC? | CO3 / An | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. |  | Discuss the following Dc characterizes of the Operational Amplifier: |  |  |
|  | a. | Input bias current. | CO1 / U | 6 |
|  | b. | Input offset voltage. | CO1 / U | 6 |
| 18. |  | Write short notes on: |  |  |
|  | a. | Pole-zero compensation. | CO1 / U | 6 |
|  | b. | Dominant pole compensation. | CO1 / U | 6 |

| 19. |  | With necessary circuit diagram, derive the transfer function for the 2nd order low pass filter. | CO2 / An | 12 |
| --- | --- | --- | --- | --- |
| 20. |  | What is a summing amplifier? Design a op Amp summing amplifier with a gain of 100. | CO2 / C | 12 |
| 21. |  | With necessary diagrams and expressions, explain the practical log amplifier. | CO2 / U | 12 |
| 22. |  | Briefly write short notes on precision full wave rectifiers. | CO2 / A | 12 |
| 23. |  | Discuss the operation of an astable multivibrator using IC 555 with necessary diagrams and waveforms. | CO3 / E | 12 |
|  |  | Compulsory: |  |  |
| 24. |  | Illustrate the following with the necessary diagrams. |  |  |
|  | a. | Flash type DAC. | CO3 / A | 6 |
|  | b. | Successive approximation type ADC. | CO3 / A | 6 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # #11EE221 / 12EE223 / EE263 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # MICROPROCESSORS AND MICROCONTROLLERS | # Max. Marks : | # 100 |

| Q. No. | Questions | Marks |
| --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |
| 1. | List the order decided by a processor or the CPU of a controller to execute an instruction. | 1 |
| 2. | If we say microcontroller is 8-bit then here 8-bit denotes size of _________. | 1 |
| 3. | How are the performance and the computer capability affected by increasing its internal bus width? | 1 |
| 4. | Abbreviate CISC and RISC. | 1 |
| 5. | Give the names of the buses present in a controller for transferring data from one place to another. | 1 |
| 6. | List the most appropriate criterions for choosing the right microcontroller of our choice? | 1 |
| 7. | Which architecture involves both the volatile and the non volatile memory? | 1 |
| 8. | 8051 microcontroller has ___________16 bit registers. | 1 |
| 9. | When the microcontroller executes some arithmetic operations, then the flag bits of which register are affected? | 1 |
| 10. | How are the bits of the register PSW affected if we select Bank2 of 8051? | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |
| --- | --- | --- |
| 11. | Explain the difference and similarities between SUB and CMP instructions. | 3 |
| 12. | Define stack and draw the structure of stack. | 3 |
| 13. | Enumerate a program for transmission and reception in the loopback mode. | 3 |
| 14. | Identify the bit definition for PCON register. | 3 |
| 15. | List about ARM development tools. | 3 |
| 16. | List the various modes of programming 8051. | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |
| --- | --- | --- | --- |
| 17. |  | Develop an assembly language program to find the sum of numbers from 1 to 100. | 12 |
| 18. |  | List out assembler directives of 8086 and explain them briefly. | 12 |
| 19. |  | What is system bus architecture? Give generalized 8086 system bus architecture | 12 |
| 20. |  | Explain about memory segmentation and instruction queue. | 12 |
| 21. |  | Explain master and slave mode of 8259 and draw the block diagram of an 8259. | 12 |
| 22. |  | Explain different control word formats of 8255 PPI. | 12 |
| 23. |  | Write an 8051 program to receive a serial byte through RxD. | 12 |
|  |  | Compulsory: |  |
| 24. |  | Describe the operation of a parallel comparator A/D converter. | 12 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 11EE225 / 12EE229 / EE262 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # MEASUREMENT AND INSTRUMENTATION | # Max. Marks : | # 100 |

| Q. No. | Questions | Marks |
| --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |
| 1. | The difference between measured value and true value of the quantity is called _______. | 1 |
| 2. | What is the bridge circuit? | 1 |
| 3. | What is the range of low resistance? | 1 |
| 4. | Suggest the nominal measuring capability of commercial Maxwell’s bridges. | 1 |
| 5. | The bridge circuit used to measure the capacitance is a) Wheatstone bridge b) Kelvin c) Schering Bridge d) Maxwell Bridge | 1 |
| 6. | List any two applications of potentiometer. | 1 |
| 7. | _________ is used in the measurement of earth resistance. | 1 |
| 8. | What is a transducer? | 1 |
| 9. | Define rise time of a pulse. | 1 |
| 10. | Define creep in energy meter. | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |
| --- | --- | --- |
| 11. | Compare moving coil and moving iron instruments. | 3 |
| 12. | Point out the advantages of digital voltmeter | 3 |
| 13. | Draw Schering bridge circuit. | 3 |
| 14. | What is real time spectrum analyzer? | 3 |
| 15. | State the basic principle of operating of resistance potentiometer. | 3 |
| 16. | How the voltage and current coils are connected in induction type energy meter with the load? | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |
| --- | --- | --- | --- |
| 17. | a. | The full scale deflection current of an ammeter is1mAand its internal resistance is 100Ω. If this meter is to have scale deflection at 5A, what is the value of shunt resistance to be used? | 5 |
|  | b. | Discuss on the extension of range in an ammeter using D’Arsonal movement. | 7 |
| 18. |  | Elaborate on a bridge that will be best suited for the measurement of very low Resistance measurement. | 12 |
| 19. |  | Write short notes on: a) Auto ranging b) Polarity detection c) Auto Zeroing d) Latch in DVM | 12 |

| 20. |  | Explain the principle, characteristics and advantages of LVDT with a neat diagram. | 12 |
| --- | --- | --- | --- |
| 21. |  | Illustrate the construction of a storage type oscilloscope. What are the accessories for a CRO? | 12 |
| 22. | a. | Describe how to measure pressure using capacitive transducer. | 6 |
|  | b. | Describe measurement of temperature using thermistor. | 6 |
| 23. |  | With a neat block diagram explain how XY recorder can be measured using signal? Give the uses of XY recorder. | 12 |
|  |  | Compulsory: |  |
| 24. |  | Explain working of digital type ramp voltmeter. | 12 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 12EE231 / EE276 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # MATERIAL SCIENCE | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | The forbidden energy gap of carbon in diamond structure is ___________. | CO1/R | 1 |
| 2. | The correct order of the co-ordination number in simple cubic, body centered cubic and face centered cubic of unit cell is ________. | CO1/R | 1 |
| 3. | Under normal conditions a diode conducts current when it is ________ | CO2/U | 1 |
| 4. | The movement of free electrons in a conductor is called ___________. | CO2/R | 1 |
| 5. | A ferromagnetic material is one in which neighboring atomic magnetic moments are predominantly equal and ______ throughout the material. | CO2/U | 1 |
| 6. | The hardness of quenched Marten site first increases and then remains almost ________ as the carbon percentage increases. | CO2/U | 1 |
| 7. | Magnetic permeability has units as ________. | CO2/R | 1 |
| 8. | ______materials are made with strong fibers embedded in weaker and softer matrix to obtain strength better than strength of matrix. | CO3/R | 1 |
| 9. | Ceramic materials are ________ compounds of metallic and non-metallic elements. | CO3/R | 1 |
| 10. | Composite with molybdenum fibers and alumina matrix do not exhibit good strength because molybdenum fibers develop ________. | CO3/U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Draw a [1 -1 0] direction with a cubic unit cell. | CO1/U | 3 |
| 12. | # Figure out the schematic sketch of the tetragonal and monoclinic crystal structure by stating their properties. | CO2/U | 3 |
| 13. | Draw a table giving the dielectric property of various insulators at different frequencies. | CO2/R | 3 |
| 14. | # Figure out the schematic sketch of the frequency dependence of the polarization process. | CO3/U | 3 |
| 15. | # Explain about retentivity and coercivity. | CO3/R | 3 |
| 16. | Find the charge inside the sphere for an electronic polarization. | CO3/A | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. |  | What are the different types of crystal arrangement? Name and explain any two in detail. | CO1/ U | 12 |
| 18. | a. | State the advantages and disadvantages of debye-scherrer method of crystal structure. | CO1/U | 6 |
|  | b. | What is zone refining technique? | CO1/U | 6 |
| 19. |  | What are the different types of ferroelectric crystal? Name and explain in detail. | CO2/U | 12 |
| 20. |  | Explain in detail about Heisenberg and domain theory of ferromagnetism. | CO3/U | 12 |
| 21. | a. | What are crystallographic planes? | CO1/R | 3 |
|  | b. | Write the procedure employed in determining the h,k,l indices numbers. | CO1/U | 9 |
| 22. |  | Derive a derivation to find the carrier concentration in an intrinsic semiconductor. | CO2/A | 12 |
| 23. |  | Explain about the carrier concentration in extrinsic semiconductor. | CO2/U | 12 |
|  |  | Compulsory: |  |  |
| 24. | a. | Explain about the different applications of the magnetic materials and its devices. | CO3/U | 6 |
|  | b. | State and explain the different polarizations in detail. | CO3/U | 6 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | To understand the concepts of Crystalls, Metals and Alloys. |
| CO2 | To realize the Qualitative study of various polarization |
| CO3 | To impart knowledge on Advanced Materials |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 5 | 36 | - | - | - | - | 41 |
| CO2 | 5 | 30 | 12 | - | - | - | 47 |
| CO3 | 5 | 28 | 3 | - | - | - | 36 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # # 12EE245 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # MICRO ELECTROMECHANICAL SYSTEMS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | _______ is a process technology used to create tiny integrated devices or systems that combine mechanical and electrical components. | CO1 | U | 1 |
| 2. | Disposable blood pressure transducer was invented in the year ________. | CO3 | A | 1 |
| 3. | Materials typically used for wet etching include ______ and ______. | CO1 | An | 1 |
| 4. | Thermo resistive sensors rely on the fact that the resistivity of most materials changes with _________. | CO1 | An | 1 |
| 5. | ________ is a device that converts an electrical signal into an action. | CO1 | R | 1 |
| 6. | Give any two examples of passive substrate materials. | CO2 | R | 1 |
| 7. | List the three categories of microfabrication process. | CO2 | U | 1 |
| 8. | List the purposes of microelectronic packaging. | CO2 | U | 1 |
| 9. | List the problems associated with device-level packaging. | CO2 | An | 1 |
| 10. | State any one application of photoresist polymers. | CO3 | A | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Write short note multifunctional polymers. | CO1 | U | 3 |
| 12. | Explain about the Surface Micromachining. | CO2 | U | 3 |
| 13. | Discuss about capacitive pressure sensor. | CO3 | A | 3 |
| 14. | Describe the ‘Photolithography’. | CO2 | An | 3 |
| 15. | Name the three levels of Microsystem Packaging. | CO1 | R | 3 |
| 16. | List the three steps needed to improve the infusion pump safety. | CO3 | An | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. |  | Discuss about smart materials and structures. | CO1 | An | 12 |
| 18. | a. | Describe the Ion Implantation with suitable diagrams. | CO2 | An | 6 |
|  | b | Describe the chemical etching process. | CO2 | An | 6 |
| 19. |  | Discuss the Chemical Vapor Deposition process. | CO2 | U | 12 |
| 20. |  | Describe the Micromechanical components. | CO1 | U | 12 |

| 21. |  | Explain the working of micro accelerometers with suitable diagrams. | CO3 | An | 12 |
| --- | --- | --- | --- | --- | --- |
| 22. |  | Discuss the various steps involved in the mechanical design of microsystems. | CO2 | C | 12 |
| 23. |  | List the essential microsystems packaging technologies and describe about each. | CO2 | A | 12 |
|  |  | Compulsory |  |  |  |
| 24. |  | Explain the working of blood pressure monitoring transducers which use MEMS technology, with neat diagrams. | CO3 | A | 12 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 12EE253 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # BUILDING AUTOMATION | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | Buildings have long life cycles that typically range between _________. | CO1 | R | 1 |
| 2. | HVAC means __________. | CO1 | R | 1 |
| 3. | Reason out the purpose of earthing. | CO1 | U | 1 |
| 4. | Building Management can provide better services to __________ and __________. | CO1 | U | 1 |
| 5. | __________ is a sensor designed to detect and respond to the presence of a flame or fire. | CO2 | R | 1 |
| 6. | __________ is the short time reduction in the rms voltage between 0.1 to 0.9 p.u for a duration of 0.5 cycle to 1 minute. | CO2 | R | 1 |
| 7. | List the blocks in Digital video management system. | CO3 | U | 1 |
| 8. | Write the advantage of TCP networking. | CO3 | R | 1 |
| 9. | Define Biometrics. | CO3 | R | 1 |
| 10. | Mention any two merits of cables. | CO3 | U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Mention the functions of HVAC System. | CO1 | U | 3 |
| 12. | How do you practically estimate power quality? Indicate the instrument used. | CO2 | U | 3 |
| 13. | Write briefly about fire extinguishing principles. | CO2 | U | 3 |
| 14. | Make a brief comment about TCP/IP networking. | CO3 | U | 3 |
| 15. | List the various future prospects of integrated systems. | CO3 | U | 3 |
| 16. | Describe the utility of Video door phone. | CO2 | U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | a. | Explicate the features and characteristics of building automation system. | CO1 | U | 12 |
| 18. | a. | Explain the significance of energy conservation. | CO2 | U | 6 |
|  | b. | Describe the importance of energy saving. | CO2 | U | 6 |

| 19. |  | Briefly explain the fire development stages and installation guidelines of fire sensors and detectors. | CO3 | A | 12 |
| --- | --- | --- | --- | --- | --- |
| 20. |  | Describe the concept of security systems briefly, with necessary sketches and terminologies. | CO2 | A | 12 |
| 21. |  | Write a brief note on following sensors and transducers: a. Temperature. b. Pressure. | CO1 | U | 12 |
| 22. |  | Discuss in detail about reactive power requirement and power quality issues. | CO2 | U | 12 |
| 23. |  | Explain the procedure for the detector placement for safety system. | CO3 | U | 12 |
|  |  | Compulsory: |  |  |  |
| 24. |  | With a neat block diagram, explain the energy management of integrated system. | CO3 | A | 12 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 12EE257 / EE271 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # HIGH VOLTAGE ENGINEERING | # Max. Marks : | # 100 |

| Q. No. | Questions |  | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. |  | Define ionization. | 1 |
| 2. |  | Write the expression to find the optimum number of stages and %ripple in a voltage multiplier circuit. | 1 |
| 3. |  | Draw the circuit diagram for voltage doubler circuit. | 1 |
| 4. |  | Define breakdown voltage. | 1 |
| 5. |  | Sphere gap is used for measurement of __________. | 1 |
| 6. |  | Give any two advantage of electrostatic voltmeter. | 1 |
| 7. |  | Write any two tests for testing cables. | 1 |
| 8. |  | What is parallel plate capacitor? | 1 |
| 9. |  | Paper insulation is mainly used in ____________. | 1 |
| 10. |  | List out various methods to measure high direct currents. | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |
| --- | --- | --- |
| 11. | Write a brief note on resonant transformer method. | 3 |
| 12. | Give the significance of volt-time curve of insulation. | 3 |
| 13. | Reproduce the time response of RC series circuit. | 3 |
| 14. | Give the difference between photo ionization and photo electric emission. | 3 |
| 15. | Mention various tests done on Circuit Breaker. | 3 |
| 16. | Discuss briefly about Disruptive Discharge Voltage. | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |
| --- | --- | --- | --- |
| 17. |  | Discuss briefly about the following: (a) Ionization by collision (b) Photo ionization. | 12 |
| 18. |  | Explain briefly about the following: Electron emission due to (a) Positive ion impact. (b) Photons. (c) Metastable and neutral atoms. | 12 |
| 19. |  | Why is a Cockcroft-Walton circuit preferred for voltage multiplier circuits? Explain its working with a schematic diagram. | 12 |
| 20. |  | Elaborate the insulation coordination among all substation equipment in voltage level. | 12 |

| 21. |  | Write detailed notes on tripping and control of impulse generators. | 12 |
| --- | --- | --- | --- |
| 22. |  | Discuss various factors which affect breakdown of gases. | 12 |
| 23. |  | Explain in detail about testing on insulators. | 12 |
|  |  | Compulsory: |  |
| 24. |  | Explain briefly various tests to be carried out on a power transformer. | 12 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 11EE223 / 12EE227 / EE261 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # C++ AND DATA STRUCTURES | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Give the importance of sorting. | CO3/R | 1 |
| 2. | Define linear search. | CO3/R | 1 |
| 3. | The minimum number of fields with each node of a doubly linked list is _____. | CO3/R | 1 |
| 4. | Give some applications of stack. | CO3/R | 1 |
| 5. | Write down the purpose of a class declaration. | CO1/R | 1 |
| 6. | Define array? How can an array be declared? | CO1/R | 1 |
| 7. | List out any two operators that cannot be overloaded in C++. | CO1/ R | 1 |
| 8. | Write down the uses of pointers in C++. | CO1/ R | 1 |
| 9. | Write down the uses of a friend function. | CO1/R | 1 |
| 10. | A class that declares or inherits a virtual function is called ___________. | CO1/R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | State the difference between arrays and linked lists. | CO1/U | 3 |
| 12. | What is the difference between class and structure? | CO1/U | 3 |
| 13. | Briefly describe Private and Public data. | CO1/U | 3 |
| 14. | Write about the various file mode parameters in C++. | CO3/U | 3 |
| 15. | What is an exception? Why do we need to handle exceptions? | CO3/U | 3 |
| 16. | Define Exception. | CO3/U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. |  | Explain insertion and deletion of elements in a queue. | CO3/ R | 12 |
| 18. |  | Explain the quick sort algorithm. Using this algorithm sort the following numbers. 50,40,20,60,80,100,45,70,105,30,90,75 | CO3/A | 12 |
| 19. |  | Explain the concepts of Object Oriented Programming in detail. | CO1/U | 12 |
| 20. |  | What is multiple inheritance? Write a C++ program to implement multiple inheritance | CO1/ A | 12 |

| 21. |  | With an example, discuss Function template and Class template. | CO3/U | 12 |
| --- | --- | --- | --- | --- |
| 22. |  | Explain operator overloading with an example. | CO3/ U | 12 |
| 23. |  | Write a program to find out the power and energy for a house using functions. Get current and voltage and time from the user. | CO2 /A | 12 |
|  |  | Compulsory: |  |  |
| 24. |  | Define virtual function. Use a base class and two derived classes to demonstrate the use of a virtual function. | CO3/ A | 12 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # # 11EE242 / 12EE232 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ENERGY SYSTEMS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Define Photovoltaic effect? | CO1/R | 1 |
| 2. | Identify the device which is used to convert sunlight into electricity? | CO1/A | 1 |
| 3. | Mention the range of wind speed which is considered favorable for wind-power generation. | CO1/U | 1 |
| 4. | What type of generators is commonly used in wind turbine? | CO1/U | 1 |
| 5. | List the main components of a fuel cell. | CO1/R | 1 |
| 6. | The simplest method of hydrogen production is ___________. | CO1/A | 1 |
| 7. | Define illumination. | CO2/R | 1 |
| 8. | High frequency induction heating is used for ___________. | CO2/U | 1 |
| 9. | Draw a typical speed time curve for train movement. | CO3/R | 1 |
| 10. | Write the demerits of electric traction system. | CO3/U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Give the current scenario of renewable energy generation in India. | CO1/A | 3 |
| 12. | Compare Horizontal axis wind turbine and Vertical axis wind turbine. | CO1/A | 3 |
| 13. | Write down the types of fuel cell based on operating temperature. | CO1/R | 3 |
| 14. | List out the characteristics involved in good lighting scheme. | CO2/R | 3 |
| 15. | Write any five requirements of an ideal traction system. | CO3/U | 3 |
| 16. | Enumerate the factors which affect the specific energy consumption of trains operating at a given schedule speed. | CO3/A | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. |  | With a neat sketch, explain the principle, construction, working and applications of solar photovoltaic system. | CO1/U | 12 |
| 18. |  | Derive the empirical equation for estimating solar radiation availability on horizontal surface under cloudless skies. | CO2/R | 12 |

| 19. | a. | With the help of a neat diagram explain the various components of Horizontal Axis Wind Turbine (HAWT). | CO1/A | 9 |
| --- | --- | --- | --- | --- |
|  | b. | Comment on environmental impacts of wind energy. | CO1/R | 3 |
| 20. |  | List the types of lighting systems and explain in detail about the lighting design procedure for flood lighting. | CO2/U | 12 |
| 21. |  | Describe various energy extraction technologies used with hydrothermal resources in geothermal energy. | CO1/R | 12 |
| 22. | a. | Discuss the biomass conversion technologies with necessary equations. | CO1/U | 8 |
|  | b. | Summarize the challenges of biomass conversion technologies. | CO1/A | 4 |
| 23. |  | Explain the method of induction heating and describe coreless type of induction furnace. | CO2/U | 12 |
|  |  | Compulsory: |  |  |
| 24. |  | Explain various methods of electric braking. State the conditions to be fulfilled for each method of braking. | CO3/U | 12 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 12EE214/EE273 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ELECTRIC DRIVES AND CONTROL | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | What are the different types of electric drive? | CO1/R | 1 |
| 2. | What types of drives have been used in rotary printing press which has several printing stations? | CO1/U | 1 |
| 3. | What is discontinuous conduction mode? | CO1/R | 1 |
| 4. | What are the methods of speed control that is used in DC motor? | CO1/U | 1 |
| 5. | For a separately excited motor, the speed equation is given by__________. | CO2/R | 1 |
| 6. | Define the slip in Induction motor. | CO2/R | 1 |
| 7. | In self controlled mode of synchronous motor, load commutation is not possible at ______________ | CO2/ R | 1 |
| 8. | What is meant by Kramer system? | CO2/R | 1 |
| 9. | Write any one application of stepper motor. | CO3/R | 1 |
| 10. | Whether synchronous motor is self-starting or not? | CO3/R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Mention the advantages of Electric drive. | CO1/ U | 3 |
| 12. | Define regenerative braking | CO1/ U | 3 |
| 13. | Compare VSI and CSI fed induction motor drives. | CO2/ U | 3 |
| 14. | Explain the closed loop control of drives. | CO2 /U | 3 |
| 15. | What is meant by frequency control of induction motor? | CO3 /U | 3 |
| 16. | Mention the applications of Electric drive. | CO3 /U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. |  | Explain in detail, with the help of a block diagram, the parts of Electrical Drives for electric vehicles. | CO1 / A | 12 |
| 18. |  | With waveforms, explain the classes of motor duty. | CO1 / U | 12 |
| 19. |  | With suitable diagram explain the operation of single phase full converter fed dc drive at discontinuous mode and derive the speed equation. | CO1 / R | 12 |

| 20. |  | Explain any one method of speed control of Induction motor drive with neat diagram. | CO2 /U | 12 |
| --- | --- | --- | --- | --- |
| 21. |  | With neat sketch explain the static Kramer system. | CO2 / U | 12 |
| 22. |  | Draw the power circuitry and explain the operation of a self-controlled mode for a three phase synchronous motor drive. | CO2 / R | 12 |
| 23. |  | Explain the working principle of stepper motor. | CO 3 / U | 12 |
|  |  | Compulsory: |  |  |
| 24. |  | Explain in detail about the battery powered drives. | CO3 / A | 12 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 12EE248 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # COMPUTER COMMUNICATION | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Write any two applications of computer networks. | CO1/R | 1 |
| 2. | What is queuing? | CO1/U | 1 |
| 3. | Specify the responsibilities of Data Link layer. | CO1/U | 1 |
| 4. | State the protocol used by Ethernet for access of communication links. | CO1/R | 1 |
| 5. | Mention the configurations in HDLC. | CO2/R | 1 |
| 6. | Expand BISYNC. | CO2/R | 1 |
| 7. | What is the significance of Routers? | CO2/U | 1 |
| 8. | Mention the fundamental parts of IP address. | CO3/R | 1 |
| 9. | State the services provided by ISDN. | CO3/R | 1 |
| 10. | Draw the cell format of ATM. | CO3/U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Outline the concepts of a network protocol. | CO1/A | 3 |
| 12. | Compare CSMA/CD, Token Ring and Token Bus. | CO1/A | 3 |
| 13. | Write about the layers defined by ATM standard. | CO2/R | 3 |
| 14. | Draw the PLP packet format in X.25. | CO2/R | 3 |
| 15. | Elucidate about internetworking. | CO3/U | 3 |
| 16. | Mention some applications of VSAT network. | CO3/R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. |  | With a neat diagram, explain the functions of various layers of OSI reference model. | CO1/U | 12 |
| 18. |  | Write short notes on Queuing systems and also explain M/M/1 queuing model. | CO1/R | 12 |
| 19. | a. | Illustrate LAN protocol architecture and various LAN topologies with neat diagram. | CO2/A | 9 |
|  | b. | List the types of contention schemes in LAN access techniques. | CO2/R | 3 |
| 20. |  | With neat sketch explain the concepts of synchronous and asynchronous communication. | CO2/U | 12 |

| 21. |  | Define congestion. Explain various congestion control algorithms. | CO3/R | 12 |
| --- | --- | --- | --- | --- |
| 22. | a. | Give a brief description about X.25 with essential diagrams and frames. | CO2/U | 8 |
|  | b. | Draw the multi-frame transmission format for BISYNC. | CO2/A | 4 |
| 23. |  | Illustrate the working of TCP/IP protocols. | CO1/U | 12 |
|  |  | Compulsory: |  |  |
| 24. | a. | Give a detailed explanation on ISDN with necessary diagrams. | CO3/U | 6 |
|  | b. | Brief on the planes of ISDN architecture. | CO3/U | 6 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the concepts of Computer Communication |
| CO2 | Understand the peripheral connections in a computer. |
| CO3 | It also gives broad idea on networking which are available for daily use. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 14 | 26 | 6 | - | - | - | 46 |
| CO2 | 11 | 21 | 13 | - | - | - | 45 |
| CO3 | 17 | 16 | - | - | - | - | 33 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 14EE2001 / 17EE2001 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ELECTRIC CIRCUITS AND NETWORKS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | If a 6 ohm resistor and 3 ohm resistor are in parallel, find the value of a single equivalent resistor. | CO3 | A | 1 |
| 2. | If three3 ohm resistors are connected in star, find the value of resistors in an equivalent delta connected network. | CO3 | A | 1 |
| 3. | State reciprocity theorem. | CO2 | R | 1 |
| 4. | Thevinin’s theorem is applicable only in _______ network. | CO5 | U | 1 |
| 5. | Two coupled coils with self inductancesL1and L2 and mutual inductance M are connected in series. Express the two possible equivalent inductances. | CO3 | U | 1 |
| 6. | Band width of a series RLC circuit can be expressed as________. | CO5 | An | 1 |
| 7. | Three phase power can be expressed as__________. | CO2 | U | 1 |
| 8. | Current through _________ circuit cannot change instantaneously. | CO1 | U | 1 |
| 9. | ABCD parameters of a network can also be called as __________. | CO4 | An | 1 |
| 10. | Z11=_______________ . | CO4 | A | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Find RAB for the circuit given below. | CO3 | A | 3 |
| 12. | State and explain the principle of Superposition. | CO2 | U | 3 |
| 13. | Express the relation between phase voltage and line voltage, and phase current and line current in the case of a balanced star connected network. | CO4 | An | 3 |
| 14. | Find the resonant frequency of a series RLC circuit with R=5 ohm, L=1 milli Henry and C=0.5 milli Farad. | CO3 | U | 3 |
| 15. | Express the time constants of RL and RC series circuits. | CO5 | A | 3 |
| 16. | Write the expressions which define the transmission parameters of a network. | CO3 | A | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. |  | Find the equivalent resistance between the terminals A and B. | CO3 | An | 12 |
| 18. |  | Use mesh analysis technique to determine the current through various branches. | CO4 | An | 12 |
| 19. |  | Determine the value of the load resistor at which maximum power will be transferred to the load. Also determine the maximum power that will be transferred. | CO3 | A | 12 |
| 20. |  | A balanced star connected load of (3+j2)Ω/phase is supplied with a balanced 3-phase, 400V(line to line) supply. Find the a) Phase current b) Line current c) Real power d) Reactive power e) Apparent power. | CO3 | U | 12 |

| 21. | a. | Determine the frequency at which circuit resonates. Also find the voltage across the capacitor at resonance, and the Q factor of the circuit. | CO4 | An | 6 |
| --- | --- | --- | --- | --- | --- |
|  | b. | Derive an equation to find the maximum amplification factor of a single tuned coupled circuit. | CO5 | U | 6 |
| 22. | a. | Determine the equivalent inductance of the circuit given below. | CO5 | A | 6 |
|  | b. | Design a constant K type low pass filter with the cut off frequency 1.5 KHz and design impedance 600Ω. | CO6 | C | 6 |
| 23. |  | Derive an expression to find the transient current through a series RL circuit when a unit step voltage is applied across it at t=0 sec. | CO4 | An | 12 |
|  |  | Compulsory: |  |  |  |
| 24. |  | Determine the short circuit admittance parameters of the network given below. | CO3 | A | 12 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 14EE2009 / 17EE2005 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ELECTRICAL MACHINE DESIGN | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Mention the purpose of ventilating ducts. | CO1/ R | 1 |
| 2. | Define Field Form Factor. | CO1/ R | 1 |
| 3. | Define fringing. | CO1/ R | 1 |
| 4. | The transformers used in sub-stations and generating stations are called __________. | CO5/ R | 1 |
| 5. | Distinguish between Core and Shell Type Transformer. | CO5/ R | 1 |
| 6. | Define voltage regulation of a transformer | CO5/ R | 1 |
| 7. | For a best power factor of a three-phase induction machine, τ is ___________. | CO2/ R | 1 |
| 8. | In alternators, when length of air gap is increased, then stability ____________. | CO3/ R | 1 |
| 9. | Define run away speed. | CO4/R | 1 |
| 10. | Write the range of Short circuit ratio of turbo alternators. | CO6/ R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Define Air gap. | CO1/ R | 3 |
| 12. | Mention the guiding factors for the selection of number of poles. | CO2/ R | 3 |
| 13. | Draw the cross section of different types of transformer cores. | CO5/ R | 3 |
| 14. | Describe the necessity of cooling tubes in the transformer. | CO5/ R | 3 |
| 15. | Explain the phenomenon of cogging. | CO3/ R | 3 |
| 16. | Mention the uses of damper windings in synchronous machines. | CO4/ R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. |  | Derive the output equation of a DCAC machines. | CO1 / U | 12 |
| 18. |  | Write down the design procedures for the commutator of dc machine. | CO2 / U | 12 |
| 19. |  | Determine the main dimensions, number of poles and length of air gap of 600kW, 500V, 900-rpm dc generator. Assume specific magnetic loading 0.6T, specific electric loading 35000ac/m. The ratio of pole arc/pole pitch=0.75, efficiency 91% and gap contraction factor 1.16 | CO3 / A | 12 |

| 20. |  | Explain the design procedure for the squirrel caged induction motor. | CO 4/ U | 12 |
| --- | --- | --- | --- | --- |
| 21. |  | Derive the output equation of single phase Transformer and three-phase transformer | CO5 / U | 12 |
| 22. |  | Estimate the main dimensions, air-gap length, stator and rotor design for a 3-phase, 15hp, 400V, 6-pole, 50Hz, 975 rpm, three phase squirrel caged induction motor. The motor is suitable for star delta starting. Bav = 0.45 Wb/m2 ,Lτ = 0.85, p.f.= 0.85, efficiency = 0.9, ac = 20,000 amp.cond./metre. | CO4 / A | 12 |
| 23. |  | Determine the main dimensions of the core, the number of turns and the cross – section of the conductors for a 5kVA, 11000/400 V, 50Hz, single-phase core type distribution transformer. The net conductor area in the window is 0.6 times the net cross section of iron in the core. Assume a square cross-section for the core, a flux density 1 Wb/ m2, a current density 1.4 A/mm2, and a window space factor 0.2. The height of window is 3 times its width. | CO5 / A | 12 |
|  |  | Compulsory: |  |  |
| 24. |  | Define Short Circuit Ratio. List out the effects of SCR value in the performance of the machine. | CO6/U | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Recognize the importance of magnetic and electric loadings |
| CO2 | Explain the design of main dimensions of DC and AC rotating machines. |
| CO3 | Calculate the system parameters for proper design of field coils and armature coils and DC and AC rotating machines. |
| CO4 | Select a proper winding design of armature coils and deduce the values of armature design parameters of DC and AC rotating machines |
| CO5 | Design a transformer and its cooling systems. |
| CO6 | Predetermine the performances of the DC, AC rotating machines and transformers from the design data. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 6 | 12 | - | - | - | - | 18 |
| CO2 | 4 | 12 | - | - | - | - | 16 |
| CO3 | 4 | - | 12 | - | - | - | 16 |
| CO4 | 4 | 12 | 12 | - | - | - | 28 |
| CO5 | 9 | 12 | 12 | - | - | - | 33 |
| CO6 | 1 | 12 | - | - | - | - | 13 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 14EE2010 / 17EE2006 / 18EE2010 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # POWER ELECTRONICS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Power MOSFET is a _____________ controlled device. | CO1 / R | 1 |
| 2. | Which Power Semiconductor device is having the highest switching frequency? | CO1 / R | 1 |
| 3. | In a single phase fully controlled bridge converter, the converter attains the inverter mode operation when the firing angle goes beyond ________. | CO2 / U | 1 |
| 4. | Dual converter is ________________ quadrant converter | CO2 / U | 1 |
| 5. | The average output voltage expression of step-up chopper is given by ____ | CO3 / R | 1 |
| 6. | A step down chopper is operated in the continuous conduction mode in steady state with a constant duty ratio D. If VO is the magnitude of the DC output voltage and if the VS is the magnitude of the DC input voltage, the ratio of VO / VS is given by _____. | CO3 / U | 1 |
| 7. | Define Modulation index. | CO4 / R | 1 |
| 8. | Which conduction mode of three phase bridge inverter is preferred? | CO5 / An | 1 |
| 9. | A single phase half bridge inverter is supplied with DC voltage 100V. The rms output voltage is _____. | CO5 / A | 1 |
| 10. | In HVDC transmission lines, one acts as a converter and other as an inverter. Say True or False. | CO6 / R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Compare Power BJT and Power MOSFET with any three points. | CO1 / U | 3 |
| 12. | Compare single phase full and semi controlled converters with any three points. | CO2 / U | 3 |
| 13. | List out the applications of AC Voltage controllers. | CO3 / U | 3 |
| 14. | Brief out the control strategies of chopper circuit? | CO4 / U | 3 |
| 15. | Compare Voltage Source Inverter and Current Source Inverter | CO5 / U | 3 |
| 16. | # Draw the circuit diagram of Single phase multilevel inverter. | CO6 / U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | # With neat diagrams describe the switching characteristics of Power MOSFET. | CO1 / R | 8 |
|  | b. | Indicate the uses of snubber circuit in the thyristor circuit. | CO1 /U | 4 |
| 18. |  | Explain the static and switching characteristics of Thyristor in detail with neat diagrams. | CO2 / R | 12 |
| 19. |  | # Explain the operation of a single phase fully controlled full bridge converter with RL load in rectification mode with neat diagram and waveforms. | CO3 /U | 12 |
| 20. |  | # Discuss the working of the single phase ac to ac full wave voltage controller with R Load, with necessary circuit, equations and waveforms. | CO3/A | 12 |
| 21. | a. | A step down chopper has Vdc = 100 V, R = 10Ω. If the duty cycle is 0.4, calculate the average voltage Vavg, rms voltage Vrms, average current Iavg and output power Po | CO4/An | 8 |
|  | b. | Draw the circuit diagram and waveforms of single phase dual converter with R load. | CO4 /A | 4 |
| 22. |  | Explain PWM and its advantages. Elucidate any two PWM technique used for inverter circuits. | CO5/ U | 12 |
| 23. |  | With relevant circuit diagram and waveforms explain the operation of three phase bridge inverter in 180 mode conduction. Derive the expression for the RMS value of phase voltage and line voltage. | CO5 /A | 12 |
|  |  | Compulsory: |  |  |
| 24. | a. | # Discuss the working of HVDC systems with relevant circuit diagram. | CO6 / R | 6 |
|  | b. | # Draw the circuit diagram of Super lift Luo Converter and explain its working. | CO6 / U | 6 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Identify to use the solid-state power devices for the control, conversion, andProtection of electrical energy |
| CO2 | Design a power converter with criteria (power, efficiency, ripple voltage and current, harmonic distortions, power factor). |
| CO3 | Implement and verify the performance characteristics of power converters. |
| CO4 | Interpret terminal characteristics of the components for designing the circuitry for power converters. |
| CO5 | Estimate the required converters for renewable based applications |
| CO6 | Assess the quality of power by analyzing the factors such as harmonics, ripples, etc |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 10 | 7 |  |  |  |  | 17 |
| CO2 | 12 | 5 |  |  |  |  | 17 |
| CO3 | 1 | 16 | 12 |  |  |  | 29 |
| CO4 | 1 | 3 | 4 | 8 |  |  | 16 |
| CO5 |  | 15 | 13 | 1 |  |  | 29 |
| CO6 | 7 | 9 |  |  |  |  | 16 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 14EE2012 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ELECTRIC DRIVES AND CONTROL | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Define electric drive? | CO1/ R | 1 |
| 2. | Write the fundamental torque equation for motor-load system. | CO1/ R | 1 |
| 3. | The operation of II quadrant is _________. | CO1/ R | 1 |
| 4. | Mention the methods to control the speed of DC shunt motor. | CO3/ R | 1 |
| 5. | Fan is the example for _________. | CO1/ U | 1 |
| 6. | Define stator voltage control. | CO1/ R | 1 |
| 7. | Write the condition to achieve the regenerative braking in dc drives. | CO2/ R | 1 |
| 8. | Rotor resistance control is applicable for____________Motor. | CO3/ R | 1 |
| 9. | Define step angle. | CO2/ R | 1 |
| 10. | List out the modes of variable frequency control of synchronous motor drives. | CO3/ R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Write about constant torque drive and constant speed drive. | CO1/ U | 3 |
| 12. | A motor has heating time constant of 70 min and a cooling time constant of 90 min. When run continuously on full load of 400 kW, final temperature rise is Determine the maximum load the motor can deliver when subjected to intermittent periodic load cycle consisting of a load period of 10 min followed by a no load period of 15 min. | CO2/A | 3 |
| 13. | How to control speed of multi motor drive? | CO3/U | 3 |
| 14. | List the different methods of speed control of three phase induction motor. | CO3/R | 3 |
| 15. | Differentiate voltage and current source inverter. | CO2/U | 3 |
| 16. | Draw the phasor diagram of a salient pole motor and write the torque and power equations | CO2/ R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | Construct the block diagram of the electric drive employed for Irrigation system. Explain in detail. | CO1 / A | 12 |
| 18. | a. | Classify the classes of motor duty according to IS: 4722-1968 and explain in detail with their applications. | CO1 / U | 12 |

| 19. | a. | Prove that “continuous mode operation of fully controlled rectifier fed separately excited DC motor can give good speed regulation”, with necessary diagrams, waveforms and equations. | CO2 / U | 12 |
| --- | --- | --- | --- | --- |
| 20. | a. | Explain different methods of starting of Induction motor with necessary diagrams for electric vehicle applications. | CO3 / A | 12 |
| 21. | a. | Explain the operation of voltage source inverter(VSI)fed induction motor drive with neat sketch. | CO3 / R | 12 |
| 22. | a. | Describe the slip power recovery control of slip ring induction motor. | CO3 / U | 12 |
| 23. | a. | Explain with diagram Synchronous motor variable speed drives. | CO2 / A | 12 |
|  |  | Compulsory: |  |  |
| 24. | a. | Classify permanent magnet AC motor drives and explain in detail about the control techniques with neat diagrams and waveforms. | CO3 / U | 12 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 14EE2014 / 17EE2008 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # POWER SYSTEM ANALYSIS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome/Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | In the figure, the voltages are v1(t) = 100cos(ωt), v2(t) = 100cos(ωt + π/18) andv3(t) = 100cos(ωt + π/36). The circuit is in sinusoidal steady state, and R << ωL. P1,P2 and P3 are the average power outputs. Which one of the following statements is true? - P1=P2=P3=0 (B)P1<0, P2>0, P3>0 (C) P1<0, P2>0, P3<0 (D) P1>0, P2<0, P3>0 | CO1/U | 1 |
| 2. | Draw the π - equivalent circuit of a transformer. | CO1/R | 1 |
| 3. | A 1000 X 1000 bus admittance matrix for an electric power system has 8000 non zero elements. The minimum numbers of branches (transmission lines and transformers) in this power system are _________. | CO2/U | 1 |
| 4. | In type 1 modification of Z-bus building algorithm, off diagonal elements are always_________. | CO2/U | 1 |
| 5. | One of the _________ bus is considered as slack bus in the power flow analysis. | CO4/R | 1 |
| 6. | What is the need of load flow study? | CO4/R | 1 |
| 7. | If the fuel cost of the power generating unit F=0.06P2+40P+120, the incremental fuel cost at 40 MW is given by ______. | CO5/U | 1 |
| 8. | Mention anyone method used to do the unit commitment problem. | CO5/R | 1 |
| 9. | Shunt faults are mainly due to _______. | CO3/R | 1 |
| 10. | Transient Limit is always________ than the steady state limit. | CO6/U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | For the power system network shown in Fig (i) draw the oriented graph, (ii) formulate element node incidence matrix. | CO1/U | 3 |
| 12. | What are the different types of unsymmetrical fault, which occur in a power system? | CO2/U | 3 |
| 13. | Differentiate Gauss Seidal and Newton Raphson method in the load flow study. | CO4/U | 3 |

| 14. | Write the co-ordination equation including losses and explain the terms involved. |  | CO5/U | 3 |  |
| --- | --- | --- | --- | --- | --- |
| 15. | Define power angle curve and equal area criteria in stability analysis. |  | CO6/U | 3 |  |
| 16. | Mention the applications of impedance diagram and reactance diagram. |  | CO3/U | 3 |  |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Two generators rated at 10MVA, 11KV and 15MVA, 11KV respectively are connected in Parallel to bus. The bus bar feed two motors rated 7.5MVA and 10 MVA respectively. The rated voltage of the motor is 9 KV. The reactance of each generator is 12% and that of each motor is 15% on their own ratings. Assume 50MVA, 10KV base and draw the reactance diagram. | CO1/An |  | 12 |
| 18. | a. | Starting from basic conditions for a single line to ground fault with fault impedance Zf, obtain the expression for fault current. Show the connection of sequence network for this fault. | CO2/U |  | 12 |
| 19. | a. | Explain the step by step computational procedure for the Newton Raphson method in load flow studies with appropriate expression. | CO4/U |  | 12 |
| 20. | a. | Briefly explain the Hydro Thermal Scheduling. | CO5/U |  | 12 |
| 21. | a. | A power plant has three units with the following fuel cost equations: Maximum and minimum loading on each unit is 150 MW and 40 MW, the demand is 275MW. Find the optimal scheduling. | CO5/An |  | 12 |
| 22. | a. | Explain the modeling of synchronous generator,transformer, transmission line and static and dynamic loadsof the power system. | CO1/U |  | 12 |
| 23. | a. | The bus impedance matrix of a system is given by Zbus=j0.140.060.080.090.060.150.070.090.080.070.130.100.090.090.100.15 Generators are connected to buses 1 and 3 and their sub transient reactances were included when finding bus impedance matrix. Find the fault current for a 3-phase fault in bus-3. Also calculate the voltages in all the buses. | CO3/An |  | 12 |
|  |  | Compulsory: |  |  |  |
| 24. | a. | Explain the Runge kutta method of analyzing multi machine power system for stability with a neat flow chart. | CO6/U |  | 12 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 14EE2024 / 19EE2032 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # BASICS OF ELECTRIC AND HYBRID VEHICLE | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | The use of Hybrid cars could reduce____________ pollution. | CO1 / R | 1 |
| 2. | The use of battery in____________ type of vehicles is limited below 2kW. | CO2 / U | 1 |
| 3. | The efficiency of a CI Engine is around ___________ % | CO1 / R | 1 |
| 4. | Hybrid vehicle is a combination of Electric and Fuel Cell based power source. | CO5 / R | 1 |
| 5. | EVs are generally powered by _____________ type of batteries. | CO1 / R | 1 |
| 6. | When the stator poles of the Switch Reluctance motor are increased, the speed of the motor ___________. | CO3 / U | 1 |
| 7. | VFD stands for ______________. | CO3 / R | 1 |
| 8. | Energy is stored in a flywheel is in the form of __________ | CO4 / U | 1 |
| 9. | The capacity of a battery is expressed in terms of_______________ | CO6 / R | 1 |
| 10. | When two batteries are connected in parallel __________________ of the two batteries should be same. | CO6 / U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Draw a neat sketch of a series hybrid EV. | CO3 / U | 3 |
| 12. | Discuss about the merits and demerits of a Front Wheel Drive system in a vehicle. | CO1 / U | 3 |
| 13. | Compare the DC motor and SRM based propulsion drive system. | CO4/An | 3 |
| 14. | Discuss about the need for Regenerative braking. | CO5 / R | 3 |
| 15. | Discuss on the role of super capacitors in EV. | CO5 / R | 3 |
| 16. | Compare the advantage of Li-Ion battery over Lead acid battery. | CO4/ An | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. |  | Discuss on the Mathematic modelling of vehicles and how the tractive force is calculated. | CO3 / A | 12 |
| 18. |  | Discuss in brief about - All Wheel Drive - Front Wheel Drive - Rear Wheel Drive - 4 Wheel Drive | CO1 / U | 12 |
| 19. |  | Classify the various Hybrid vehicle architecture available and elaborate the same | CO2 / U | 12 |
| 20. |  | Explain the different types of DC motor control used for EV application | CO4 / U | 12 |
| 21. |  | Explain how the Induction motor is controlled for an EV application. With suitable diagrams explain about the controller used. | CO4 / U | 12 |
| 22. |  | Show how the power flows in aparallel hybrid architecture in different modes and discuss on the same. | CO5/ A | 12 |
| 23. |  | Discuss on the operation of Fuel Cell vehicle with a neat sketch. | CO5 / U | 12 |
|  |  | Compulsory: |  |  |
| 24. |  | Develop an Energy management strategy for a series hybrid vehicle and discuss about the various modes of operation. | CO6 / A | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Explain the functioning of the propulsion system in vehicles |
| CO2 | Apply the knowledge for selecting suitable combinations of EHV propulsion system |
| CO3 | Analyze the effect on the characteristic behaviors of EHV |
| CO4 | Evaluate the performance of the propulsion system for a given scenario |
| CO5 | Design an Electric Hybrid Propulsion system for a specific application |
| CO6 | Develop an Energy Management system for Electric Vehicles. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 6 | 12 |  |  |  |  | 18 |
| CO2 |  | 13 |  |  |  |  | 13 |
| CO3 | 4 | 1 | 12 |  |  |  | 17 |
| CO4 |  | 25 |  | 6 |  |  | 31 |
| CO5 | 7 | 12 | 12 |  |  |  | 31 |
| CO6 | 1 | 1 | 12 |  |  |  | 14 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 15EE2005 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # TESTING AND INSTALLATION OF POWER SYSTEM APPARATUS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | __________ is the clear minimum distance between two conducting points separated by air/gas/oil. | CO1/R | 1 |
| 2. | The resistance of human body between tip of left hand and right hand finger with dry skin is of the order of __________ohms. | CO1/R | 1 |
| 3. | _________tests give simultaneous graphical records of several time varying quantities in real time. | CO1/R | 1 |
| 4. | _________ is the device used for measuring vibration. | CO2/R | 1 |
| 5. | The test voltages for commissioning test are only______% of the routine test voltage. | CO2/U | 1 |
| 6. | The main purpose of performing open-circuit test on a transformer is to measure its__________. | CO1/U | 1 |
| 7. | When a ferromagnetic substance is magnetized, there are small changes in dimensions. The phenomenon is called___________. | CO1/R | 1 |
| 8. | In which test, rated current is passed through the bus bars for a few hours till reaching steady temperature. | CO1/R | 1 |
| 9. | __________ is a type of partial discharge of air/gas in the vicinity of high voltage conductor. | CO2/R | 1 |
| 10. | HVDC transmission lines are preferred for economic considerations for distances above __________kilometers. | CO2/R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Compare creepage distance and clearance distance. | CO1/U | 3 |
| 12. | Draw the circuit diagram of Volt-Amp method for DC resistance measurement for testing a conducting path. | CO1/U | 3 |
| 13. | Write the following terms in order of commissioning process: Trail start, external examination, mechanical inspection. | CO2/R | 3 |
| 14. | List any four causes of high temperature rise on motors and suggest suitable remedies for each. | CO2/A | 3 |
| 15. | Explain the limits of AC transmission lines. | CO3/A | 3 |
| 16. | Interpret the function of breather in a distribution transformer. | CO2/An | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | Distinguish between primary shock and secondary shock. Assess a person who gets electric shock and the causes behind the severity of shock. | CO1 /A | 6 |
|  | b. | Draw the test set up for Lightning Impulse and Switching Impulse Test. Also furnish the test details. | CO1/U | 6 |
| 18. | a. | Compare Research Test and Development Test. | CO1/U | 3 |
|  | b. | Elaborate on any two methods for testing continuity of a cable/conducting path with diagrams. | CO1/A | 9 |
| 19. | a. | Describe Power Quality. Mention the causes of disturbance in power systems. | CO3/R | 6 |
|  | b. | Illustrate the effects and remedies for the following disturbances in power system: (a)Switching surges (b) Arc furnace operation (c) Thyristor control (d) Load throw off (e) Frequency fall (f) Voltage rise (g) Frequency rise. | CO3/U | 6 |
| 20. | a. | With neat diagram, explain the procedure of high voltage test on a three phase electrical motor. | CO2/R | 9 |
|  | b. | Give the procedure of trial start of rotating machines. | CO2/U | 3 |
| 21. | a. | Evaluate the various types of sub-stations for rated voltages of 12kV, 36kV, 66kV and above. | CO3/An | 6 |
|  | b. | Justify the substations based on the voltage levels and configurations. | CO3/An | 6 |
| 22. | a. | Explain the various commissioning test on a Power transformer. | CO2/U | 6 |
|  | b. | Propose additional checks for parallel transformers prior to commissioning. | CO2/A | 6 |
| 23. | a. | Explain why synchronizing is necessary for a synchronous motor. Explain the methods of starting and synchronizing a synchronous motors. | CO1/U | 12 |
|  |  | Compulsory: |  |  |
| 24. | a. | Classify various types of bus-bars. Elaborate on the ratings to be specified and materials used for the bus-bars. | CO2/U | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | To analyze the testing of equipments |
| CO2 | To analyze the commissioning of machine and transmission line |
| CO3 | To assess the quality of Power. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 5 | 28 | 15 | - | - | - | 48 |
| CO2 | 15 | 22 | 9 | 3 | - | - | 49 |
| CO3 | 6 | 6 | 3 | 12 | - | - | 27 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18EE2002 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # NETWORK THEORY | # Max. Marks : | # 100 |

| Q. No. | Questions |  | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- | --- |
|  |  | PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | State Kirchoff’s current law. |  | CO1 | R | 1 |
| 2. | Define a ‘mesh’. |  | CO1 | U | 1 |
| 3. | Maximum power will be transferred to a load from a source when the load impedance is equal to ____________. |  | CO2 | A | 1 |
| 4. | ______ is the dual of the circuit element ‘inductor’. |  | CO1 | U | 1 |
| 5. | Fourier transform of sinωt is ____________. |  | CO5 | A | 1 |
| 6. | RMS value of a sinusoidal signal in terms of its peak value can be expressed as _____________. |  | CO3 | U | 1 |
| 7. | Laplace transform of U(t) is ____________. |  | CO4 | A | 1 |
| 8. | Laplace inverse of sinωtUt is _______. |  | CO4 | A | 1 |
| 9. | Define ‘poles’ of network functions. |  | CO6 | U | 1 |
| 10. | Define ‘transient response’ of a network. |  | CO4 | U | 1 |

|  |  | PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 11. | Transform the given voltage source into an equivalent current source. |  | CO1 | A | 3 |
| 12. | Illustrate compensation theorem with the help of an example. |  | CO2 | An | 3 |
| 13. | Plot the phasor diagram of a series RLC circuit. |  | CO3 | E | 3 |
| 14. | State and explain convolution theorem. |  | CO4 | U | 3 |
| 15. | Plot the current response of a series RLC circuit with respect to frequency and obtain the condition for maximum current response. |  | CO5 | A | 3 |
| 16. | Explain the formation of Tie set. |  | CO6 | An | 3 |

|  |  | PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 17. | a. |  | Find the values of unknown currents I3, I4 and I5. |  | CO1 | A | 6 |
|  | b |  | Apply voltage division rule to find the voltage across 4 ohm resistor. |  | CO1 | A | 6 |
| 18. | a. |  | Use Thevinin’s theorem to determine the current I |  | CO2 | A | 12 |
| 19. | a. |  | If impedances (8+j4) , (5-j5) and 10 are connected in three phases of a delta connected load which is supplied by a 3, 400 V supply, determine the current in each phase and in each line. Also calculate the total power consumed and the power factor of the load in each phase. |  | CO3 | An | 12 |
| 20. | a. |  | With the help of suitable examples explain how Laplace transform can be used to analyze RL, RC and RLC circuits. |  | CO4 | U | 12 |
| 21. | a. |  | The switch in the RC circuit shown in Fig is closed at t = 0. If there is no charge initially stored in the capacitor, find a. vR, the voltage across the resistor at t = 50 s. b. Vc, the voltage across the capacitor at t = 100 s. c. The time at which vR reaches 2 V. |  | CO4 | A | 12 |
| 22. | a. |  | Use maximum power transfer theorem to determine the maximum power that can be transferred to the load. |  | CO2 | An | 12 |
| 23. | a. |  | With the help of a suitable network example explain various driving point functions and transfer functions. |  | CO6 | E | 12 |
|  |  |  |  | Compulsory: |  |  |  |
| 24. | a. |  | Find the transmission parameters of the two port network given below. |  | CO6 | A | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand basic electrical circuits with nodal and mesh analysis. |
| CO2 | Apply various network theorems to analyze the circuits and networks. |
| CO3 | Analyze three phase circuits. |
| CO4 | Apply Laplace transform for steady state and transient analysis. |
| CO5 | Analyze the frequency domain techniques |
| CO6 | Determine different network functions and define filter circuits to satisfy design specification. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 1 | 2 | 15 | - | - | - | 18 |
| CO2 | - | - | 13 | 15 | - | - | 28 |
| CO3 | - | 1 | - | 12 | 3 | - | 16 |
| CO4 | - | 16 | 14 | - | - | - | 30 |
| CO5 | - | - | 4 | - | - | - | 4 |
| CO6 | - | 1 | 12 | 3 | 12 | - | 28 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18EE2014 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # POWER SYSTEMS -II | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | A power system graph has 8 elements and 4 nodes. Find the number of links. | CO1 | A | 1 |
| 2. | Write the performance equation of network element in admittance form. | CO1 | U | 1 |
| 3. | Define steady state stability. | CO2 | U | 1 |
| 4. | Draw the power angle curve of a single machine supplying power to infinite bus. | CO2 | U | 1 |
| 5. | State the importance of frequency regulation. | CO3 | U | 1 |
| 6. | Write the expression for static frequency error. | CO3 | R | 1 |
| 7. | Give an important reason for voltage control. | CO5 | U | 1 |
| 8. | What is synchronous condenser? | CO5 | U | 1 |
| 9. | Define restorative state. | CO4 | U | 1 |
| 10. | Define contingency analysis. | CO4 | U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Write the properties of Bus Admittance matrix. | CO1 | R | 3 |
| 12. | Differentiate between static stability and dynamic stability. | CO2 | A | 3 |
| 13. | Justify the need for speed governor and its droop setting. | CO3 | E | 3 |
| 14. | Find out the minimum value of open loop gain of the AVR loop to get 1% static accuracy limit. | CO5 | An | 3 |
| 15. | Define state estimation. Mention its applications in power system. | CO4 | U | 3 |
| 16. | Define Spot Pricing. Mention its advantages. | CO6 | U | 3 |

|  |  | PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | a. | For the Given Network i) Draw the Oriented Graph ii) Draw one tree and the corresponding co-tree ii) Write the element – node incidence matrix. | CO1 | A | 6 |
|  | b. | For the power system data given, find the Ybus by direct inspection. | CO1 | A | 6 |
| 18. | a. | A 500MW, 21kV, 50Hz, 3-phase, 2-pole synchronous generator having pf=0.9, has a moment of inertia 27.5×103 kg-m2. Find the inertia constant. | CO2 | An | 6 |
|  | b. | How will you solve the swing equation by Euler’s method? | CO2 | An | 6 |
| 19. | a. | Two synchronous generators operating in parallel. Their capacities are 300 MW and 400 MW respectively. The droop characteristics of their governors are 4% and 5% from no load to full load. How would be a load of 600 MW is shared between them? | CO3 | An | 6 |
|  | b. | Draw the block diagram of AGC system. Derive the transfer function of the power system block. | CO3 | A | 6 |
| 20. | a | Draw the block diagram representation of AVR loop. With the help of root loci, explain the need for stability compensation. | CO5 | An | 7 |
|  | b | Explain the embedded DC link concept in transmission lines. | CO5 | A | 5 |

| 21. | a. | Draw the block diagram of phasor measurement unit. Mention how the unit measures the phasors. | CO4 | An | 6 |
| --- | --- | --- | --- | --- | --- |
|  | b. | With schematic diagram, explain the application of SCADA in power system monitoring and control. | CO4 | A | 6 |
| 22. |  | Explain the step by step computational procedure for the Gauss-Seidel method in load flow studies with appropriate expressions. | CO1 | A | 12 |
| 23. |  | Describe the equal area criterion for transient stability analysis of a power system when a fault occurs in the TL. | CO2 | An | 12 |
|  |  | Compulsory: |  |  |  |
| 24. |  | Explain Vertically Integrated and Whole-sale competition electricity market models. | CO6 | U | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Use numerical methods to analyze a power system in steady state |
| CO2 | Classify the state of operating equilibrium after being subjected to a physical disturbance |
| CO3 | Implement the voltage, frequency and power flow control |
| CO4 | Monitoring and control the transmission and distribution power system |
| CO5 | Design various compensators for to control the power system parameters |
| CO6 | Optimize the operation of power plant |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 3 | 1 | 25 | - | - | - | 29 |
| CO2 | - | 2 | 3 | 24 | - | - | 29 |
| CO3 | 1 | 1 | 6 | 6 | 3 | - | 17 |
| CO4 | - | 5 | 6 | 6 | - | - | 17 |
| CO5 | - | 2 | 6 | 9 | - | - | 17 |
| CO6 | - | 15 | - | - | - | - | 15 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18EE2016 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # WIND AND SOLAR ENERGY SYSTEM | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | The air doesn’t flow in __________ line from high pressure area to low pressure area. | CO1 | R | 1 |
| 2. | Determine the total power produced if the turbine diameter is 90m. | CO1 | A | 1 |
| 3. | The range of wind speed suitable for wind power generation is __________ . | CO2 | U | 1 |
| 4. | In wind turbine site selection, what are the technical considerations? | CO2 | U | 1 |
| 5. | The lines joining North and South pole of earth are called __________. | CO3 | R | 1 |
| 6. | State solar day. | CO3 | R | 1 |
| 7. | Suggest suitable inverters to 0.4–2 kW range for small roof-top plants with panels connected in one string. | CO4 | A | 1 |
| 8. | The solar cells are based on the principles of __________ effect. | CO4 | R | 1 |
| 9. | ___________ is the short time reduction in the rms voltage between 0.1 to 0.9 p.u for a duration of 0.5 cycle to 1 minute: | CO5 | R | 1 |
| 10. | Voltage regulation in power systems is directly related to the control of __________ power. | CO5 | U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Calculate the speed of shaft when low speed type shaft is interfaced with gear box at 50Hz frequency. | CO1 | U | 3 |
| 12. | Match the maximum output voltage of the different power electronics circuit configuration. power electronics circuit; maximum output voltage. - 3L-NPC; - V dc. - 5L-HB; - 0.5V dc +V dc. - 3L+5L; - 0.5V dc. | CO2 | U | 3 |
| 13. | Infer solar azimuth angle and zenith angle. | CO3 | U | 3 |
| 14. | Mention the different types of solar cell and compare the efficiency of the cell. | CO4 | U | 3 |
| 15. | Write short notes on Fault ride-through for wind farms. | CO5 | U | 3 |
| 16. | Define solar pond. | CO6 | U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 17. | a. | Prove that the maximum power coefficient (Cp) for a wind mill is 0.593. |  | CO1 |  | U |  | 8 |
|  | b. | Explain the weibull distribution to actual wind speed distribution of a typical region. |  | CO1 |  | U |  | 4 |
| 18. | a. | Discuss the power electronic converters used in wind energy conversion. Also, explain the converter control. |  | CO2 |  | U |  | 8 |
|  | b. | Describe the distinctions between fixed and variable speed wind turbines. |  | CO2 |  | U |  | 4 |
| 19. | a. | Consider the earth to be a blackbody with average surface temperature 15 ◦ C and area equal to 5.1 × 1014 m2. Find the rate at which energy is radiated by the earth and the wavelength at which maximum power is radiated. Compare this peak wavelength with that for a 5800K blackbody (sun). |  | CO3 |  | An |  | 6 |
|  | b. | Explain the step by step procedure to estimate the monthly average daily global radiation. |  | CO3 |  | U |  | 6 |
| 20. | a. | Consider a 100-cm2 photovoltaic cell with reverse saturation current I0 = 10−12 A/cm2. In full sun, it produces a short-circuit current of 40 mA/cm2 at 25◦ C. Find the open-circuit voltage at full sun and again for 50% sunlight. Plot the results. |  | CO4 |  | An |  | 8 |
|  | b. | Illustrate the Buck boost converter of solar cell with proper circuit diagram. |  | CO4 |  | U |  | 4 |
| 21. | a. | List the power quality issues in the interconnected systems of solar PV and wind systems? Elaborate. |  | CO5 |  | U |  | 8 |
|  | b. | Demonstrate the Frequency limits according to Hydro-Quebec and AESO grid codes. |  | CO5 |  | U |  | 4 |
| 22. | a. | Classify the wind energy systems and explain their working with neat sketch. |  | CO2 |  | U |  | 8 |
|  | b. | Find the optimum tilt angle for a South-facing photovoltaic module in Tucson (latitude 32.1◦) at solar noon on March 1. |  | CO3 |  | An |  | 4 |
| 23. | a. | Describe in detail the PV module equivalent circuit and its I-V characteristics. |  | CO4 |  | U |  | 6 |
|  | b. | Summarize the maximum power point tracking in the Solar Photo voltaic system and discuss the advantages and disadvantages. |  | CO4 |  | U |  | 6 |
|  |  | Compulsory: |  |  |  |  |  |  |
| 24. | a. | Illustrate the solar hot water heater with a capacity of 1000 litres for your home. | CO6 |  | A |  | 12 |  |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Identify the need for technologies pertaining to renewable energy sources in the current energy scenario |
| CO2 | Describe the basic physics of wind and solar power generation. |
| CO3 | Outline the power electronic interfaces for wind and solar generation. |
| CO4 | Illustrate the technologies in Solar PV power generation |
| CO5 | Interpret the issues related to the grid-integration of solar and wind energy systems |
| CO6 | Summarize the solar thermal power generation technologies |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 1 | 15 | 1 | - | - | - | 17 |
| CO2 | - | 25 | - | - |  |  | 25 |
| CO3 | 2 | 9 | - | 10 | - | - | 21 |
| CO4 | 1 | 7 | 1 | 8 | - | - | 17 |
| CO5 | 1 | 28 | - | - | - | - | 29 |
| CO6 | - | 3 | 12 | - | - | - | 15 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – June – 2022

| # Code: | # 18EE3012 | # Duration: | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name: | # GRID CONVERTERS FOR SOLAR AND WIND POWER SYSTEMS | # Max. Marks: | # 100 |

ANSWER ALL QUESTIONS (5 X 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Design the inverter structure derived from NPC topology. | CO1 / C | 10 |
|  | b. | Generate the control structure for PV inverter topologies. | CO1 / C | 10 |
| (OR) |  |  |  |  |
| 2. |  | Derive the inverter structure using H bridge topology for grid converter systems. | CO1 / E | 20 |
| 3. | a. | Summarize the anti islanding requirement in grid requirements. | CO2 / C | 10 |
|  | b. | Illustrate the harmonization of grid codes and future trends in grid requirements. | CO2 / C | 10 |
| (OR) |  |  |  |  |
| 4. | a. | Describe the frequency deviation under normal operation of grid requirements. | CO2 / C | 10 |
|  | b. | Discuss the voltage deviation of grid requirement of PV system under normal condition. | CO2 / A | 10 |
| 5. |  | Construct the double second order control and grid supporting for grid converter control. | CO3 / C | 20 |
| (OR) |  |  |  |  |
| 6. |  | Explain the operation of grid synchronization technique for single-phase systems. | CO3 / C | 20 |
| 7. |  | Summarize the active islanding technique used in islanding detection. | CO4 / C | 20 |
| (OR) |  |  |  |  |
| 8 |  | Illustrate the stand alone, micro grid, droop control and grid supporting in grid converter control. | CO5 / An | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | Explain the control technique for grid-connected converters under unbalanced grid voltage conditions. | CO6 / C | 20 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Outline the need and design of grid converters for PV and wind power system |
| CO2 | Summarize the different converter structures and the stringent grid requirements |
| CO3 | Illustrate grid synchronization in single and three-phase power converters |
| CO4 | Analyze the different islanding detection methods |
| CO5 | Formulate grid converter control strategies during grid faults |
| CO6 | Choose the appropriate grid filter design and current control. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 |  |  |  |  | 20 | 20 | 40 |
| CO2 |  |  |  | 10 |  | 30 | 40 |
| CO3 | - |  |  |  | - | 40 | 40 |
| CO4 | - |  |  | - |  | 20 | 20 |
| CO5 |  |  |  | 20 |  | - | 20 |
| CO6 |  |  |  | - | - | 20 | 20 |
|  |  |  |  |  |  |  | 180 |

Reg. No. _____________

End Semester Examination – June – 2022

| # Code : | # 18EE3016 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # DATA MINING FOR RENEWABLE ENERGY SYSTEMS | # Max. Marks : | # 100 |

ANSWER ALL QUESTIONS (5 X 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Assume that you have a dataset of wind speed for a month with 15 minutes resolution. How will you apply descriptive statistics to summarize the data? | CO1 / An | 10 |
|  | b. | List out various data cleaning and reduction techniques. Mention how these techniques influence knowledge extraction from data. | CO2 / U | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Discuss the differences and similarities between data warehouse and database. | CO2 / E | 10 |
|  | b. | Explain the architecture of data warehouse with all its components. | CO2 / U | 10 |
| 3. | a. | Explain any one mining algorithm to extract frequent patterns from the data set. | CO4 / An | 10 |
|  | b. | How will you generate association rules from frequent patterns? | CO4 / An | 10 |
| (OR) |  |  |  |  |
| 4. | a. | Define classification. Mention its types with examples from real world data. | CO3 / U | 10 |
|  | b. | Explain the Bayesian classifier with one dataset as an example. | CO3 / A | 10 |
| 5. | a. | Differentiate between regression and classification by taking example dataset for each case. | CO3 / U | 10 |
|  | b. | Explain various performance metrics available to measure the classification accuracy. | CO3 / U | 10 |
| (OR) |  |  |  |  |
| 6. | a. | Explain the hierarchical clustering method with an example data set. | CO6 / A | 10 |
|  | b. | What do you mean by sequence data mining? Mention its applications. | CO6 / U | 10 |
| 7. | a. | Why solar radiation forecasting is important? List different techniques available for solar radiation forecasting. | CO5 / E | 10 |
|  | b. | How will you select a particular location for wind turbine installation? Discuss all the relevant aspects. | CO5 / C | 10 |
| (OR) |  |  |  |  |
| 8. | a. | Explain how ensemble approach improves the predictive task for time series data. | CO4 / A | 10 |
|  | b. | List the various components of a time series data. Give examples. | CO6 / U | 10 |

|  |  | Compulsory: |  |  |
| --- | --- | --- | --- | --- |
| 9. |  | Write a note on the different challenges faced by wind power generation. Also Recognize how wind power prediction enhances power system operations. | CO6 / E | 20 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the importance of data-driven performance optimization of renewable energy systems |
| CO2 | Exploit the vast data base available in the renewable energy sector and devise ways to make renewable energy a competitive source of supply |
| CO3 | Classify and analyze the different type of data |
| CO4 | Prediction of data with error measure |
| CO5 | Apply data mining for the prediction of power from renewable energy sources |
| CO6 | Find the various research opportunities provided by the field |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | - | - | - | 10 | - | - | 10 |
| CO2 | - | 20 | - | - | 10 | - | 30 |
| CO3 | - | 30 | 10 | - | - | - | 40 |
| CO4 | - | - | 10 | 20 | - | - | 30 |
| CO5 | - | - | - | - | 10 | 10 | 20 |
| CO6 | - | 20 | 10 | - | 20 | - | 50 |
|  |  |  |  |  |  |  | 180 |

Reg. No. _____________

End Semester Examination – June – 2022

| # Code: | # 18EE3017 | # Duration: | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name: | # POWER CONVERSION AND CONTROL OF WIND ENERGY SYSTEMS | # Max. Marks: | # 100 |

ANSWER ALL QUESTIONS (5 X 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| 1. |  | Reconstruct the synchronous generators modeling. | CO1 / C | 20 |
| (OR) |  |  |  |  |
| 2. |  | Design the reference frame transformation of wind generator and modeling. | CO1 / E | 20 |
| 3. | a. | Specify the need of PWM current source converter supports WECS. | CO2 / C | 10 |
|  | b. | Summarize the control of grid-connected inverters. | CO2 / C | 10 |
| (OR) |  |  |  |  |
| 4. | a. | Enumerate the performance two level voltage source converters in wind energy conversion systems? | CO2 / C | 10 |
|  | b. | Explain how an AC voltage controller involved in WECS. | CO2 / A | 10 |
| 5. |  | Describe the operation of variable speed synchronous generator based WECS? | CO3 / C | 20 |
| (OR) |  |  |  |  |
| 6. |  | Configure the variable speed induction generator in WECS. | CO3 / C | 20 |
| 7. | a. | Explain direct field oriented control in SQIG in variable speed wind energy systems. | CO4 / C | 10 |
|  | b. | Determine the indirect field oriented control in squirrel cage induction motor. | CO4 / E | 10 |
| (OR) |  |  |  |  |
| 8. | a. | Discuss the super synchronous operation of DIFG based WECS. | CO5 / E | 10 |
|  | b. | Illustrate the sub synchronous operation of DFIG. | CO5 / An | 10 |
|  |  | Compulsory: |  |  |
| 9. |  | How synchronous generator wind energy system is interfaced with DC/DC Boost converter? | CO6 / An | 20 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Model the generators used in wind energy conversion systems |
| CO2 | Design the power converters in wind energy conversion systems |
| CO3 | Outline the different wind energy system configurations |
| CO4 | Analyze the control of squirrel cage induction generators in WECS |
| CO5 | Explain the control techniques for DFIG based WECS |
| CO6 | Summarize the control of synchronous generator based WECS |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 |  |  |  |  | 20 | 20 | 40 |
| CO2 |  |  |  | 10 |  | 30 | 40 |
| CO3 |  |  |  |  |  | 40 | 40 |
| CO4 |  |  |  |  | 10 | 10 | 20 |
| CO5 |  |  |  | 10 | 10 |  | 20 |
| CO6 |  |  |  | 20 |  |  | 20 |
|  |  |  |  |  |  |  | 180 |

Reg. No. _____________

End Semester Examination – June – 2022

| # Code : | # 18EE3018 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # POWER QUALITY ISSUES AND MITIGATION | # Max. Marks : | # 100 |

| Q. No. | Sub Div. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| ANSWER ALL QUESTIONS (5 X 20 = 100 Marks) |  |  |  |  |
| 1. | a. | Illustrate the following with a waveform sketch related to Power quality events i) Voltage imbalance ii) Flicker. iii) Sag with harmonics iv) Frequency variation | CO1 / U | 10 |
|  | b. | Give outlines on IEEE and IEC power quality standards. | CO1 / U | 10 |
|  |  | (OR) |  |  |
| 2. | a. | Give explanation the different types of sag with a relevant vector diagram with proper equations. | CO2/ U | 10 |
|  | b. | Elucidate capacitor switching transient and magnification of capacitor switching transient. | CO2 / U | 10 |
| 3. | a. | Explain in detail about the general procedure for harmonic distortion evaluation of the panel board of indoor substation | CO3 / A | 10 |
|  | b. | Explain briefly about for the following harmonic filter. (i)Active filters (ii)Passive filters. | CO3 / U | 10 |
|  |  | (OR) |  |  |
| 4. | a. | The single-phase series active power filter having voltage harmonics in a 220V, 50Hz, AC mains (15% third, 10% fifth, and 5% seventh harmonics) present due to other loads and source impedance, before it is connected to a critical linear load of 5 kVA, 220V, 50Hz at 0.8 lagging power factor. If a single-phase VSC is used as a series APF, calculate (a) voltage and current ratings of the APF, (b) VA rating of the VSC of the series APF, and (c) interfacing inductance of the APF. Consider the switching frequency of 20 kHz, DC bus voltage of 200V, and ripple current in the inductor is 10%. | CO4 / An | 10 |

|  | b. | Illustrate the principle of STATCOM operation used for Harmonic mitigation. | CO4 / A | 10 |
| --- | --- | --- | --- | --- |
| 5. |  | Briefly Describe the Synchronous reference frame how extract the harmonic component with the suitable equations. | CO5 / A | 20 |
|  |  | (OR) |  |  |
| 6. | a. | Examine the source and effects of different categories of Long duration voltage Variations. | CO2 / U | 10 |
|  | b. | Draw the CBEMA & ITI curve and explain about events described in the curves. | CO1 / U | 10 |
| 7. | a. | Illustrate the principle of DVR operation used for sag mitigation. | CO4 / A | 10 |
|  | b. | Briefly Describe the FFT Theory for harmonic extraction process. | CO5 / U | 10 |
|  |  | (OR) |  |  |
| 8 | a. | The converter circuit is shown the below fig. Calculate the following quantities (a)THD of AC mains current (b) DIN (c) TDD of AC mains current (d) DF (e)DPF (f) PF | CO3 / A | 10 |
|  | b. | What are the various classifications of harmonic sources and explain briefly about it. | CO5 / U | 10 |
| PART – B (1 X 20 = 20 MARKS) (Compulsory Question) |  |  |  |  |  |  |  | 10 |
| 9. | a. | Illustrate the steps involved in power quality monitoring. What are the information from monitoring site surveys? | CO6 / U | 10 |
|  | b. | Elucidate the following: i) Harmonic Analyser ii) Flicker Meter | CO6 / U | 10 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Recognize the cause and source of power system disturbances. |
| CO2 | Calculate harmonic voltages and currents by analyzing all types of electrical systems loads and their power quality considerations. |
| CO3 | Suggest suitable mitigation scheme for some of the power quality issues. |
| CO4 | Examine the methods of reducing excessive harmonics using advanced modeling technique. |
| CO5 | Analyze the power quality issues using the Power quality indices. |
| CO6 | Design load compensators useful for mitigating power quality problems. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | - | 30 | - | - | - | - | 30 |
| CO2 | - | 30 | - | - | - | - | 30 |
| CO3 | - | 10 | 20 | - | - | - | 30 |
| CO4 | - | - | 10 | 20 | - | - | 40 |
| CO5 | - | 20 | 20 | - | - | - | 40 |
| CO6 | - | 20 | - | - | - | - | 20 |
|  |  |  |  |  |  |  | 180 |

Reg. No. _____________

End Semester Examination – June – 2022

| # Code : | # 18EE3019 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # DISTRIBUTED GENERATION AND MICRO GRID | # Max. Marks : | # 100 |

ANSWER ALL QUESTIONS (5 X 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Analyze the impacts of DGon Distribution System. | CO1 / A | 10 |
|  | b. | Illustrate the impacts of DG on Transmission System and Central Generation. | CO1 / R | 10 |
| (OR) |  |  |  |  |
| 2. |  | Discuss about the following: i) Micro grid ii) Smart grid iii) Power grid iv) Distributed Generation | CO1 / U | 20 |
| 3. | a. | Sketch and explain about Small Hydro Power Generation Method. | CO2 / R | 10 |
|  | b. | Explain about different types of Energy Storage systems in DG | CO2 / U | 10 |
| (OR) |  |  |  |  |
| 4. |  | Illustrate about Geothermal Power Generation Techniques. | CO2 / R | 20 |
| 5. | a. | Brief about various Power Electronics Topologies for the Wind Energy Systems. | CO3 / U | 10 |
|  | b. | Give the details about different kind of Power Electronics Topologies for the PV Systems. | CO3 / R | 10 |
| (OR) |  |  |  |  |
| 6. |  | Enumerate different types of Power Electronics Topologies for Micro turbines. | CO3 / R | 20 |
| 7. | a. | Illustrate the Transient Response and Fault Behaviors in Distribution Generation systems. | CO4 / U | 10 |
|  | b. | Sketch and explain different current injection method for DG systems. | CO4 / R | 10 |
| (OR) |  |  |  |  |
| 8. | a. | Describe about the power quality issues in Microgrid. | CO5 / U | 10 |
|  | b. | Design Distribution Generation system to meet power quality requirements. | CO5 / R | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | Enumerate the operation of Decentralized Control method in Microgrid. | CO6 / A | 10 |
|  | b. | Discuss in detail about Microgrid communication infrastructure. | CO6 / U | 10 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Define the concept of distributed generation and Impact of DG on Transmission System |
| CO2 | Classify the various distributed generation sources and energy storage |
| CO3 | Outline the general and the power electronic topologies for distributed generation and its interface |
| CO4 | Describe various distributed generation protection scheme |
| CO5 | Analyze the power quality issues of distributed generation. |
| CO6 | Compare the different microgrid architectures and discuss on the risks of the Smart Grid and its protective measures. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 10 | 20 |  | 10 |  |  | 40 |
| CO2 | 30 | 10 |  |  |  |  | 40 |
| CO3 | 30 | 10 |  |  |  |  | 40 |
| CO4 | 10 | 10 |  |  |  |  | 20 |
| CO5 | 10 | 10 |  |  |  |  | 20 |
| CO6 |  | 10 | 10 |  |  |  | 20 |
|  |  |  |  |  |  |  | 180 |

Reg.No. _____________

End Semester Examination – June – 2022

| # Code : | # 18EE3021 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # SMART POWER GRID RENEWABLE ENERGY SYSTEMS | # Max. Marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | What are the technologies required for establishing the smart grid network and brief about each technology. | CO1 / U | 10 |
|  | b. | Compare Traditional grid and smart grid. | CO1 / R | 10 |
| (OR) |  |  |  |  |
| 2. |  | Define Smart Grid. Illustrate the need for Smart Grid. | CO1 / U | 10 |
|  |  | In order to transform the traditional grid into smart grid, new technologies have to be implemented. Justify. | CO1 / A | 10 |
| 3. |  | Write down the key aspects of; - Energy Management System (EMS) - Wide Area Monitoring System (WAMS) | CO2 / R | 20 |
| (OR) |  |  |  |  |
| 4. |  | Brief about the following: - Substation Automation - Distribution System Management (DMS) | CO3 / U | 20 |
| 5. |  | Illustrate about the following (i) Broadband over Power line (BPL) (ii) WAN and HAN (iii) Power Line Communication | CO3/U | 20 |
| (OR) |  |  |  |  |
| 6. |  | Explain how Web Service and Cloud computing make the Smart Grids smarter. | CO4 /U | 20 |
| 7. | a. | Write the power-transfer equation. Show how FACTS devices act on this equation to enhance the power system operation. | CO4/A | 10 |
|  | b. | Discuss the technical challenges involved in the Integration of Vehicles with Rechargeable Batteries into Distribution Networks. | CO5/U | 10 |
| (OR) |  |  |  |  |
| 8. |  | What is AMI? Illustrate its protocols, standards initiatives and benefits. | CO5 /A | 20 |
|  |  | Compulsory: |  |  |
| 9. | a. | Brief about Super conducting magnetic energy storage system. | CO6/U | 10 |
|  | b. | List out the penetration issues associated with renewable energy technology. | CO6/R | 10 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Differentiate the information exchange in traditional grid and smart grid |
| CO2 | Assess the importance of the information security for smart grid |
| CO3 | Demonstrate different Smart Grid communication technologies |
| CO4 | Design the prototype model of the smart grid |
| CO5 | Evaluate the role of power electronic devices in the network |
| CO6 | Analyze the grid integration issues of renewable energy sources |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 10 | 20 | 10 |  |  |  | 40 |
| CO2 | 20 |  |  |  |  |  | 20 |
| CO3 |  | 40 |  |  |  |  | 40 |
| CO4 |  | 20 | 10 |  |  |  | 30 |
| CO5 |  | 10 | 20 |  |  |  | 30 |
| CO6 | 10 | 10 |  |  |  |  | 20 |
|  |  |  |  |  |  |  | 180 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 19EE2003 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # RENEWABLE ENERGY SOURCES FOR HEALTH CARE | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | Give the rate of solar energy reaching the earth surface. | CO1 | R | 1 |
| 2. | The scattered solar radiation is called __________. | CO1 | R | 1 |
| 3. | Angle made by radial line joining the location to the center of the earth with the projection of the line on the equatorial plane is related as _________. | CO2 | U | 1 |
| 4. | Illustrate solar heating system. | CO2 | U | 1 |
| 5. | Examine in brief the benefit of solar air collector in food processing industry. | CO3 | An | 1 |
| 6. | Fermentation is aerobic process. True / False. | CO3 | U | 1 |
| 7. | According to WHO, what percent of hospital waste is considered to be infectious? | CO4 | R | 1 |
| 8. | __________ emission is cited as an issue from incineration. | CO4 | U | 1 |
| 9. | ___________ are sinusoidal voltages or currents having frequencies that are integer multiples of the frequency at which the supply system is designed to operate (termed the fundamental frequency; usually 50 or 60 Hz). | CO5 | U | 1 |
| 10. | Voltage fluctuations is also termed as ______________ | CO6 | R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Illustrate the working of Solar cell. | CO1 | U | 3 |
| 12. | Explain the term azimuth angle. | CO2 | U | 3 |
| 13. | Summarize the functionalities of concentrating collectors. | CO3 | U | 3 |
| 14. | Identify the purpose of electrical code NFPA 780. | CO4 | A | 3 |
| 15. | Define Pasteurization. | CO5 | R | 3 |
| 16. | Compare and contrast bio and Clinical Waste. | CO6 | U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | a. | With the help of a neat diagram, explain the principle of operation of solar radiation and spectrum of sun geometry. | CO1 | U | 12 |
| 18. | a. | Make use of the equivalent circuit and explain the electrical characteristics of the solar cell. | CO1 | A | 12 |

| 19. | a. | Identify the mismatches happened in the solar cell/module because of series and parallel connection of solar cell/module. Explain in detail with necessary diagrams. | CO2 | A | 6 |
| --- | --- | --- | --- | --- | --- |
|  | b. | Explain the working of Flat plate collectors with necessary diagrams. | CO2 | U | 6 |
| 20. | a. | Outline the process involved in biomedical waste management. | CO3 | U | 8 |
|  | b. | Explain the terms secured landfills and Incinerators. | CO3 | U | 4 |
| 21. | a. | List the IEEE 519 guidelines for power quality in critical facilities. | CO4 | R | 4 |
|  | b. | Show the functionalities of national electrical code NFPA 70. | CO4 | U | 4 |
|  | c. | Outline the terms Disposal and Disinfection. | CO4 | U | 4 |
| 22. | a. | Design a solar powered Health clinic with the following critical loads. S. No; EQUIPMENT; POWER RATING(Watts). 1; Computer; 200. 2; Lights; 70. 3; Fans; 75. 4; Microscope; 15. 5; Centrifuge nebulizer; 150. 6; Vaporizer; 40. 7; Oxygen concentrator; 300. | CO6 | A | 12 |
| 23. | a. | Illustrate a solar powered compressor type Vaccine refrigerator. | CO5 | U | 6 |
|  | b. | Explain the blocks involved in solar water heating. | CO5 | U | 6 |
|  |  | Compulsory: |  |  |  |
| 24. | a. | Develop the required power from incineration plant. List the different requirements. Explain the various blocks involved in the development with suitable diagrams. | CO6 | A | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Describe the basic physics of solar power generation. |
| CO2 | Summarize the solar thermal power generation technologies |
| CO3 | Explain the bio and clinical waste to energy generation. |
| CO4 | Describe the various electrical codes for Power station in a hospital. |
| CO5 | Explain the various applications of Solar Power for a hospital. |
| CO6 | Plan for the Emergency Power units for a hospital using Renewable Energy Sources |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 2 | 15 | 12 | - | - | - | 29 |
| CO2 | - | 11 | 6 | - |  |  | 17 |
| CO3 | - | 16 | - | 1 | - | - | 17 |
| CO4 | 5 | 9 | 3 | - |  |  | 17 |
| CO5 | 3 | 13 | - | - | - | - | 16 |
| CO6 | 1 | 3 | 24 | - | - | - | 28 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 19EE2006 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # COMPUTER SIMULATION OF ELECTRICAL SYSTEMS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | MATLAB stands for _____________. | CO1 / R | 1 |
| 2. | The function to close the windows containing graphs generated from MATLAB is ____________ all. | CO1 / U | 1 |
| 3. | What is the name of the given block? | CO2 / An | 1 |
| 4. | The output at scope 1 is? | CO2 / A | 1 |
| 5. | The value of the time constant in the R-L circuit is_________. | CO3 / U | 1 |
| 6. | Find the value of Vth from the below circuit. | CO3 / E | 1 |
| 7. | In power system, the maximum number of buses are: load buses. – True or False. | CO4 / U | 1 |
| 8. | What are the three terminals of the IGBT? | CO4 / R | 1 |
| 9. | __________ circuit is used across a power semiconductor device for protection against high dv/dt and high di/dt. | CO5 / R | 1 |
| 10. | The networks in which the R, L, C parameters are individually concentrated or lumped at discrete points in the circuit are called __________. | CO5 / U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | After executing the following script file in MATLAB x= [2:4 ;‐1:1 ;1 2 3]; y=x(2,:); size(y') the displayed result will be _____ | CO1 / A | 3 |
| 12. | List out the operators that MATLAB allows. | CO2 / R | 3 |
| 13. | Define Thevenin’s and Norton’s Theorem. | CO3 / R | 3 |
| 14. | What are the methods available to form Y bus? | CO4 / An | 3 |
| 15. | Describe significance of firing angle in power converters. | CO5 / An | 3 |
| 16. | Brief out differentiator and integrator circuit. | CO6 / U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | List out the common tool boxes in MATLAB. | CO1 / U | 3 |
|  | b. | Give a brief note about Array, Matrix and Scripts in MATLAB. | CO1 / R | 9 |
| 18 | a. | Obtain the root locus of the system using MATLAB whose transfer function is defined by | CO2 / A | 6 |
|  | b. | Obtain the bode plot using MATLAB and to calculate the phase margin, gain margin, phase cross over and gain cross over frequency for the systems whose open loop transfer function is given as follows: | CO2 / A | 6 |
| 19. | a. | Verify the any to network theorems using MATLAB / Simulink. | CO3 / An | 12 |
| 20. | a. | Discuss the load Load flow studies using Newton-Raphson Technique using MATLAB. | CO4 / U | 12 |
| 21. | a. | Illustrate the simulation model of a single-phase inverter with necessary diagram and waveforms. | CO5 / A | 12 |
| 22. | a. | Explain the simulation model of transient response of RL and RC Circuit with step and sinusoidal input signals. | CO3 / An | 12 |
| 23. | a. | Demonstrate the simulation model of a single-phase full bridge converter with necessary diagram and waveforms. | CO5 / An | 12 |
|  |  | Compulsory: |  |  |
| 24. | a. | Explain the working of Op-amp based differentiator with necessary simulation diagrams. | CO6 / U | 6 |
|  | b. | Describe the simulation models in steps of an Op-amp based integrator circuit. | CO6 / An | 6 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 19EE2010 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # SPECIAL ELECTRICAL MACHINES | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
|  | PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | In a stepper motor, step resolution gets doubled in _____ excitation mode, compared to single coil or double coil excitation modes. | CO1 | An | 1 |
| 2. | Unipolar and Bipolar are the two classifications of _____ type of stepper motors. | CO1 | U | 1 |
| 3. | The torque produced in a switched reluctance motor will be _____, if the change of winding inductance with respect to angular position is zero. | CO2 | A | 1 |
| 4. | The purpose of using salient pole rotor in synchronous reluctance motor is _______ | .CO2 | U | 1 |
| 5. | Magnetic flux density remains in a magnetic material even after the removal of magnetizing force is called ______. | CO3 | An | 1 |
| 6. | _______ type magnets have high remnant flux density low coercive force. | CO3 | R | 1 |
| 7. | Short pitched windings are used in Permanent Magnet Synchronous Motor in order to reduce ________. | CO4 | A | 1 |
| 8. | The r.m.s value of e.m.f in a permanent magnet synchronous motor can be expressed as ______. | CO4 | R | 1 |
| 9. | If the compensating windingis short circuited on itself in a series AC motor, the motor is said to be ________. | CO6 | U | 1 |
| 10. | The secondaries of LIM are made of ______ or ______ plates. | CO5 | An | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | State the three excitation modes of stepper motors. | CO1 | An | 3 |
| 12. | Compare switched reluctance motors with conventional reluctance motors. | CO2 | An | 3 |
| 13. | Plot the demagnetization characteristics of ALNICO type and FERRITE type magnetic materials. | CO3 | A | 3 |
| 14. | Write any three special features of Permanent Magnet Synchronous Motors. | CO4 | U | 3 |
| 15. | List the drawbacks of DC series motors, if those motors are supplied with single phase AC supply. | CO6 | U | 3 |
| 16. | List any three applications of Axial Flux Permanent Magnet Motors. | CO4 | A | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | a. | Explain the working principle of stepper motor with suitable diagrams. | CO1 | U | 5 |
|  | b. | Classify the variable reluctance motors based on its construction. With suitable diagrams, explain the working of each type. | CO1 | An | 7 |
| 18. | a. | List the advantages of switched reluctance motors. | CO2 | A | 4 |
|  | b. | Plot the torque-speed characteristics of switched reluctance motors and explain. | CO2 | A | 8 |
| 19. | a. | Discuss about the demagnetization characteristics of a magnetic material and explain its significance. Also explain the importance of recoil line. | CO3 | An | 5 |
|  | b. | With suitable diagrams and table of excitation explain how rotating magnetic field is generated in a 3 phase, bipolar driven brushless DC motor. | CO3 | U | 7 |
| 20. | a. | Plot the phasor diagram of permanent magnet synchronous motor and explain. | CO4 | A | 6 |
|  | b. | Explain about any two types vector power controllers used in permanent magnet synchronous motors. | CO4 | U | 6 |
| 21. | a. | With neat diagrams differentiate between the conductively compensated and inductively compensated AC series motors | CO6 | An | 4 |
|  | b. | Compare Linear and rotary motors. | CO5 | An | 5 |
|  | c. | Explain the speed control methods of DC series motors. | CO6 | R | 3 |
| 22. | a. | With neat diagram explain the hard ware control circuit used to switch on the switching devices used for the operation of stepper motors. | CO1 | A | 6 |
|  | b. | Explain the torque-speed characteristics of stepper motor. | CO1 | U | 6 |
| 23. | a. | Explain the constructional features of Synchronous Reluctance Motors. | CO3 | R | 6 |
|  | b. | List the advantages and disadvantages of synchronous reluctance motors. | CO3 | An | 6 |
|  |  | Compulsory: |  |  |  |
| 24. | a. | List the advantages of axial flux motors. | CO4 | An | 5 |
|  | b. | Describe the fabrication process of stator and rotor of Axial Flux Permanent Magnet Motors. | CO4 | U | 7 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 19EE2011 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # AUTOMOTIVE ELECTRONICS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Petrol Engine is also known as _____________ engines | CO1 / R | 1 |
| 2. | With ECU, Opening and closing of the inlet / outlet valve of an IC engine is determined by __________________ | CO3 / U | 1 |
| 3. | The Stoichiometric ratio to be maintained in S.l. engine vehicle is ________ | CO1 / R | 1 |
| 4. | The temperature indication in the instrument cluster of the vehicle indicates the temperature of ________________ . | CO1 / R | 1 |
| 5. | Knock sensors use __________. | CO3 / R | 1 |
| 6. | The ignition of Air Fuel mixture taking place before the occurrence of spark in the spark plug is termed as ____________ . | CO2 / U | 1 |
| 7. | ________ type of Air fuel mixture quickens the starting process in a vehicle | CO5 / U | 1 |
| 8. | The highest voltage required to develop the spark in the spark plug of a petrol engine is ___________ . | CO4 / U | 1 |
| 9. | ______________system senses hazardous situation by monitoring the speed of the vehicle and presence of obstacle. | CO6 / U | 1 |
| 10. | LED refers to __________. | CO3 / R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Discuss about the type of sensor used commonly for fuel quantity measurement. | CO6 / U | 3 |
| 12. | Analyze the need of Lambda Sensor. | CO6 / U | 3 |
| 13. | What are the two types of Earthing used in Vehicles? | CO4 / R | 3 |
| 14. | What does an actuator do? | CO3 / R | 3 |
| 15. | Write the correct sequence in which current flows in a battery ignition system. | CO3 / A | 3 |
| 16. | Describe a Blind Spot Warning System. | CO6 / U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q. No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | Describe about the various engine performance parameters. | CO6 / U | 7 |
|  | b. | Discuss how an Electronic Ignition helps in improving fuel economy. | CO5 / A | 5 |
| 18. | a. | With neat sketch explain the process of measuring the crank angle by electronic means. Discuss about the various sensors available for this purpose. | CO3 / U | 12 |
| 19. | a. | With suitable diagram explain the following: - Coolant Temperature Measurement - Trip Information Computer | CO2 / R | 12 |

| 20. | a. | Comment on the requirement of the stating system used in vehicles. Discuss on the mechanical and electrical characteristic requirements for the application | CO4 / A | 12 |
| --- | --- | --- | --- | --- |
| 21. | a. | Discuss about the fuel injection system and comment on how Multi port fuel injection helps in improving fuel efficiency | CO5 / A | 12 |
| 22. | a. | With suitable sketch discuss about the Automotive system available in the modern car. | CO2 / U | 12 |
| 23. | a. | Discuss about the following: - Closed Loop Control - Hard Acceleration - Deceleration | CO1 / U | 12 |
|  |  | Compulsory: |  |  |
| 24. | a. | How Electronic Suspension System improves the ride comfort? Explain with suitable sketch. | CO6 / A | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Analyze various parameters that influence the vehicle performance |
| CO2 | Recognize the importance of digital control systemin automobiles |
| CO3 | Understand the operation of sensors and actuators |
| CO4 | Develop systems for starting engine and battery charging |
| CO5 | Formulate control strategies with automotive systems for reducing exhaust emission |
| CO6 | Relate the various control strategies for performance improvement |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 3 | 12 |  |  |  |  | 15 |
| CO2 | 24 | 1 |  |  |  |  | 25 |
| CO3 | 5 | 13 | 3 |  |  |  | 21 |
| CO4 | 3 | 1 | 12 |  |  |  | 16 |
| CO5 |  | 1 | 17 |  |  |  | 18 |
| CO6 |  | 17 | 12 |  |  |  | 29 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 19EE2020 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # POWER ELECTRONICS FOR RENEWABLE ENERGY | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | ____________ deals with the relationship between living organisms and the environment. | CO1 | R | 1 |
| 2. | Indian coal carries 0.6% to 1% __________ and its ash content varies from 30% to 50%. | CO1 | R | 1 |
| 3. | Closed cycle ocean thermal energy conversion uses __________. | CO1 | R | 1 |
| 4. | Write down the output voltage equation for boost converter. | CO2 | R | 1 |
| 5. | Name any two non isolated type DC-DC converters. | CO2 | R | 1 |
| 6. | Grid synchronization is a fundamental issue in the connection of ___________ to the grid. | CO3 | R | 1 |
| 7. | ___________ inverters send the power from solar PV panels directly to the grid. | CO4 | R | 1 |
| 8. | List down few components available in standalone PV system. | CO5 | R | 1 |
| 9. | In fixed speed wind energy conversion system, regardless of the wind speed, the wind turbine rotor speed is fixed and determined by the ____________. | CO5 | R | 1 |
| 10. | An ___________ technique is therefore used to maintain the PV array’s operating point at its maximum power point. | CO6 | R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Discuss about GHG emission. List the factors influencing the amount of GHG emissions. | CO1 | U | 3 |
| 12. | Write short notes on line commutated inverter. | CO2 | U | 3 |
| 13. | List down the advantages and disadvantages of HERIC converter. | CO3 | U | 3 |
| 14. | Explain the role of uncontrolled rectifiers in wind energy conversion system. | CO4 | A | 3 |
| 15. | Write about the issues in connection the renewable energy systems to the grid. | CO5 | U | 3 |
| 16. | Name the various types of hybrid energy systems. | CO6 | U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | a. | Describe the concept of the electric power generation from biomass. | CO1 | U | 6 |
|  | b. | Discuss the impact of following renewable energy generation on environment. - Ocean energy - Hydrogen energy | CO1 | U | 6 |
| 18. |  | Design a DC-DC buck converter to charge a 12V battery form a 24 V solar panel with required circuit diagram and equations. | CO2 | A | 12 |
| 19. | a. | List out the issues related to grid connected renewable energy system and explain in detail. | CO3 | U | 8 |
|  | b. | List out the advantages, disadvantages and applications for Neutral point clamp inverter. | CO3 | U | 4 |
| 20. | a. | Draw the power circuit of grid interactive inverter and explain its operation. | CO4 | U | 8 |
|  | b. | Explain the need of Matrix converter for wind energy applications. | CO4 | A | 4 |
| 21. |  | With necessary block diagram, discuss about the grid integrated PV system. | CO5 | U | 12 |
| 22. |  | Explain the operation for the following converters - Three phase AC voltage Controller (6) - PWM inverter (6) | CO4 | U | 12 |
| 23. |  | Discuss about the stand alone operation of fixed speed wind energy conversion system with neat diagram for food industries. | CO5 | A | 12 |
|  |  | Compulsory: |  |  |  |
| 24. |  | Develop a neat block diagram of hybrid renewable energy system which should be able to supply the power to the residential flat system for 24 hours without interruption. | CO6 | A | 12 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 19EE2023 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # SUBSTATION DESIGN | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Define Substation. | CO1 / R | 1 |
| 2. | Write any three components of substation. | CO1 / R | 1 |
| 3. | Which one the following is off load device? - Isolator B. Relay C. Circuit Breaker D. Potential Transformer | CO2 / R | 1 |
| 4. | Recall where the incoming and outgoing lines in substation are connected. | CO2 / R | 1 |
| 5. | What is the main reason for substation grounding? | CO3 / R | 1 |
| 6. | Name the sudden rise in voltage for a very short duration on the power system. | CO3 / R | 1 |
| 7. | Define transducer. | CO4 / R | 1 |
| 8. | Mention any two applications of voltage sensor. | CO4 / R | 1 |
| 9. | Name the degree to which a component of a substation is open to a threat. | CO5 / R | 1 |
| 10. | Recall any individual or organization performing unauthorized activity within a substation. | CO5 / R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Write the general functions of substation. | CO1 / R | 3 |
| 12. | Comparison relay and circuit breaker | CO2 / U | 3 |
| 13. | Relate touch and step voltages with neat diagram. | CO3 / U | 3 |
| 14. | What are objectives of SCADA? | CO 4 /R | 3 |
| 15. | Distinguish Threat Assessment and Risk Assessment. | CO5/ An | 3 |
| 16. | Show the architecture of an IEC 61850. | CO6 / U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | Explain briefly about substation classification based on service requirement. | CO1 / U | 6 |
|  | b. | Compare Outdoor and Indoor substations. | CO1 / U | 6 |
| 18. | a. | With neat diagram, explain the Air Insulated and Gas Insulated substations. | CO 2 /U | 12 |
| 19. | a. | Outline all the effects of electrical current passing through the human body. | CO3 /U | 6 |
|  | b. | What are the internal causes of over-voltages in substations | CO3 /R | 6 |

| 20. | a. | Explain briefly the following: - Monolithic (or) Early SCADA system - Distributed SCADA system - Networked SCADA system | CO 4 /U | 12 |
| --- | --- | --- | --- | --- |
| 21. | a. | Explain briefly about physical and cyber security in substation. | CO5 /U | 12 |
| 22. | a. | Model the bus bar arrangements in substations. | CO1 /A | 12 |
| 23. | a. | Interpret the following: - Earthing screen - Overhead ground wires - Lightning arresters or surge diverters | CO3 /U | 6 |
|  | b. | With neat diagram, explain how Fixed angle method is used to protect the substation against direct lightning strokes. | CO3 /U | 6 |
|  |  | Compulsory: |  |  |
| 24. | a. | Demonstrate the role of substations in smart grid. | CO6 /U | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Develop Substation Layouts |
| CO2 | Select Switching Configuration |
| CO3 | Design Air Insulated and Gas Insulated Substation |
| CO4 | Interface Communication Techniques |
| CO5 | Monitor and Control the Substation Operation |
| CO6 | Adopt Substation Technology Advances in future. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 5 | 12 | 12 | - | - | - | 29 |
| CO2 | 2 | 15 | - | - | - | - | 17 |
| CO3 | 8 | 21 | - | - | - | - | 29 |
| CO4 | 5 | 12 | - | - | - | - | 17 |
| CO5 | 2 | 12 | 3 | - | - | - | 17 |
| CO6 | - | 15 | - | - | - | - | 15 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

Code           : 19EE2024									Duration   : 3hrs

Sub Name  : TESTING AND COMMISSIONING OF ELECTRICAL	                     Max. Marks : 100

EQUIPMENTS

| Q. No. | Questions | Course Outcome/Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | The Generation / Transmission / Distribution / Service Connections/ Control Centers and Control rooms, SCADA, Communication Channels, WAN, LAN comes under the ______. | CO1/R | 1 |
| 2. | __________ is the management of site personnel, plant, equipment and subsystems, materials, energy sources and environment. | CO1/R | 1 |
| 3. | __________ quality refers to quality from start to end, including the total system and services. | CO1/R | 1 |
| 4. | The polarization Index is the ratio of the insulation resistance at __________ to the insulation resistance at __________. | CO2/R | 1 |
| 5. | Oscillographic test which is used record the starting currents comes under the category of __________ tests. | CO2/R | 1 |
| 6. | Temperature test and the __________ load loss test are the tests conducted to estimate the performance characteristics of the induction motor. | CO2/R | 1 |
| 7. | In SF6 insulated switchgear the live parts are supported on _________ insulators. | CO2/R | 1 |
| 8. | During the installation of switchgears the recommended spacing in the front is _________ meters and ________ m at the rear of 11 kV draw-out switchgear. | CO3/R | 1 |
| 9. | All new completed electrical installation should be tested before connected to the supply, to ensure that the installation is technically sound and free from any possible __________. | CO5/R | 1 |
| 10. | During the insulation resistance test between installation and earth, theinsulation resistance thus measured should not be less than on a firm, sound and fixed wiring. | CO4/R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Draw the schematic sketch of the power quality analyzer, which depicts the quality and total harmonic distortion of the input power supply. | CO1/R | 3 |
| 12. | Mention the typical switching surge wave values of lightning surges and switching surges. | CO2/U | 3 |
| 13. | ## Derive the equations for open circuit voltage test to find the circuit parameters of a motor. | CO2/U | 3 |
| 14. | What is the operating sequence of a circuit breaker? | CO3/U | 3 |
| 15. | Draw a schematic of CT/PT connections of the energy meter. | CO4/R | 3 |
| 16. | Justify the need forinsulation resistance test while wiring conductors? | CO5/U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | Draw a flowchart that mentions the various stages of site management. | CO1/R | 4 |
|  | b. | Write a short note on the site activities and the various phases of site work. | CO1/U | 4 |
|  | c. | What is the significance of BVQI ISO 9000 certification? | CO1/U | 4 |
| 18. | a. | With an experimental set up, explain the high voltage test on rotating machines? | CO2/A | 6 |
|  | b. | List the checks to be done prior to start a machine in substation. | CO2/R | 3 |
|  | c. | List the various aspects of inspecting the mechanical parts of a machine. | CO2/R | 3 |
| 19. | a. | Mention the causes of high temperature rise and remedies. | CO3/R | 3 |
|  | b. | Enumerate on the Care-commissioning of synchronous generator. | CO3/U | 3 |
|  | c. | State and explain the standard tests done for synchronous motors? | CO3/U | 6 |
| 20. | a. | State the various types of tests performed on a high voltage a.c circuit breakers. | CO4/U | 5 |
|  | b. | Explain the difference between contactors and circuit breakers used for motor control? | CO4/U | 7 |
| 21. | a. | Enumerate on the types of domestic wiring and the different components used in house wiring. | CO5/U | 6 |
|  | b. | Identify a portable insulation measurement device and explain the same with its schematic sketch. | CO5/U | 6 |
| 22. | a. | What are the main activities under the site quality or field quality management? What are the objectives in respective areas? | CO1/U | 5 |
|  | b. | What are the internationally accepted standard organization and national standard organization? | CO1/U | 4 |
|  | c. | Which are the standard publications related to Quality? | CO1/R | 3 |
| 23. | a. | List the various over voltage tests that are conducted for insulators and bushings along with the creepage distance. | CO2/U | 3 |
|  | b. | With the help of schematic diagram, depict the basic elements of a short circuit testing of isolators and circuit breakers. | CO2/U | 4 |
|  | c. | Draw the synthetic testing station diagram and explain the synthetic testing of circuit breakers. | CO2/U | 5 |
|  |  | Compulsory |  |  |
| 24. | a. | With appropriate schematic diagrams explain any 3 tests that should be made on a new electrical installation before it is switched on to the mains? | CO6/A | 4 |
|  | b. | Enumerate in detail the various steps involved in the maintenance of a DG set. | CO6/U | 4 |
|  | c. | Amongst the various application of testing methods, write a short note on the seasonal and maintenance of CVT/PT. | CO6/U | 4 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 19EE2027 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | FUNDAMENTALS OF ELECTRICAL SAFETY | # Max. Marks : | # 100 |

| Q. No. | Questions |  | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- | --- |
|  |  | PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Define insulation phenomena. |  | CO1 | R | 1 |
| 2. | Define resistance. |  | CO1 | R | 1 |
| 3. | Spell OSHA. |  | CO2 | R | 1 |
| 4. | Name any two circuit protection devices. |  | CO2 | R | 1 |
| 5. | Show the single line diagram of electrical power systems |  | CO3 | R | 1 |
| 6. | What is equipotential bonding conductor? |  | CO3 | R | 1 |
| 7. | Define insulator. |  | CO4 | R | 1 |
| 8. | Illustrate ampacity. |  | CO4 | U | 1 |
| 9. | Spell NESC. |  | CO5 | R | 1 |
| 10. | List the NESC depending factors. |  | CO5 | R | 1 |

|  |  | PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 11. | List the factors affecting the conductivity of human body. |  | CO1 | R | 3 |
| 12. | Outline all the factors in electrical accidents. |  | CO2 | U | 3 |
| 13. | What is pre-job briefings |  | CO3 | R | 3 |
| 14. | Name the factors affecting the skin effect. |  | CO4 | R | 3 |
| 15. | Explain OSHA act coverage. |  | CO5 | U | 3 |
| 16. | Illustrate conductor continuity test. |  | CO6 | U | 3 |

|  |  | PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 17. | a. |  | Explain the causes of electrical shock. | CO1 | U | 7 |
|  | b. |  | What are the precautions we need to follow to avoid electric shocks? | CO1 | R | 5 |
| 18. | a. |  | Explain briefly about - Limited approach boundary - Restricted approach boundary - Prohibited approach boundary | CO2 | U | 12 |
| 19. | a. |  | Summarize the six step safety methods. | CO3 | U | 7 |
|  | b. |  | Distinguish relay and circuit breaker. | CO3 | An | 5 |
| 20. | a. |  | Illustrate on earth electrode and the different types of earth electrode. | CO4 | U | 12 |

| 21. | a. | Explain briefly the standards adopted by NEC during safe installation of electrical wiring and equipment. |  | CO5 | U | 6 |
| --- | --- | --- | --- | --- | --- | --- |
|  | b. | List the requirements and purpose of NESC. |  | CO5 | R | 6 |
| 22. | a. | Illustrate on lightning. Explain the direct and indirect strokes with neat diagrams. |  | CO4 | U | 12 |
| 23. | a. | Explain briefly the following medium voltage switchgear - Gas-insulated switchgear - Metal-clad switchgear - Metal-enclosed switchgear |  | CO3 | U | 12 |
|  |  |  | Compulsory: |  |  |  |
| 24. | a. | Analyze earth resistance and classify the different methods that are available to measure the earth resistance. |  | CO6 | An | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Describe the effects of electrical hazards on human body. |
| CO2 | Discover the potential of electrical hazard in the workplace. |
| CO3 | Identify the right safety procedure/method for the electrical accident that happened |
| CO4 | Comprehend on the function of electrical safety equipment. |
| CO5 | Apply the appropriate electrical safety code prescribed by the regulatory bodies. |
| CO6 | Test the electrical safety systems and apply them in real-time applications. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 10 | 7 | - | - | - | - | 17 |
| CO2 | 2 | 15 | - | - | - | - | 17 |
| CO3 | 5 | 19 | - | 5 | - | - | 29 |
| CO4 | 4 | 25 | - | - | - | - | 29 |
| CO5 | 8 | 9 | - | - | - | - | 17 |
| CO6 | - | 3 | - | 12 | - | - | 15 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2020

| # Code : | # 19EE2029 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # EMBEDDED SYSTEMS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Apple Uses ___________ architecture. | CO1 / R | 1 |
| 2. | Edge computing is a ____________ IT Architecture. | CO1 / R | 1 |
| 3. | ARM adopts Von – Neuman Architecture (True / False). | CO2 / U | 1 |
| 4. | ALU of ARM 7 architecture is of __________ bits. | CO2 / R | 1 |
| 5. | Devices and sensors are the __________ of the device connectivity layer. | CO3 / U | 1 |
| 6. | Piezo buzzer is an example for an Actuator (True/ False) | CO3 / U | 1 |
| 7. | _________ Communication technology enables IoT Opportunities. | CO4 / U | 1 |
| 8. | LoRaWAN can support upto ____________ kms. | CO4 / R | 1 |
| 9. | CoAP stands for ___________. | CO6 / U | 1 |
| 10. | ___________is a high-level Python web framework that encourages rapid development and clean, pragmatic design. | CO6 / U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | What is the role of GATEWAY in an IoT Architecture? | CO1 / R | 3 |
| 12. | List the five main components in the ARM Architecture. | CO2 / R | 3 |
| 13. | Define pipelining process. | CO2 / U | 3 |
| 14. | Explain about Near Field Communication. | CO4 / U | 3 |
| 15. | Discuss about M2M protocol. | CO4 / U | 3 |
| 16. | Write in short about Xively Cloud. | CO6 / U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. |  |  | Discuss in detail the different types of Embedded System. | CO1 / U | 12 |
| 18. | a. |  | Compare the Application, Real-Time and Microcontroller ARM processor characteristics | CO2 / U | 8 |
|  | b. |  | List some salient features of ARM7 | CO2 / U | 4 |
| 19. | a. |  | In short describe about I/O buffering and its uses | CO3 / R | 6 |
|  | b. |  | Compare Embedded software and Firmware | CO3 / U | 6 |
| 20. |  |  | Discuss about the following - Narrowband IoT - Extended coverage Global System Communication - LTE-M | CO4 / R | 12 |
| 21. |  |  | With block diagram, illustrate the operation of SPI communication. | CO4 / R | 12 |
| 22. |  |  | Explain how a SCADA based system is used as an Internet of Controller. | CO4 / U | 12 |
| 23. |  |  | Explain in detail the Wireless Sensor Network, its classification, structure and application. | CO6 / U | 12 |
|  |  |  | Compulsory: |  |  |
| 24. |  |  | Develop an embedded architecture for monitoring the weather condition of a particular location. | CO5 / A | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Explain the basics of embedded systems and IoT using modular design and abstraction |
| CO2 | Comprehend the major architectures and advanced features of ARM 32-Bit Microprocessor |
| CO3 | Employ the usage of Internal and External Embedded Peripheral Components |
| CO4 | Recognize the application of Protocols in designing an Embedded and IoT system |
| CO5 | Implement an architectural design for Embedded IoT. |
| CO6 | Choose between available technologies and devices to solve the given societal challenge using IoT |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 5 | 12 |  |  |  |  | 17 |
| CO2 | 4 | 16 |  |  |  |  | 20 |
| CO3 | 6 | 8 |  |  |  |  | 14 |
| CO4 | 25 | 19 |  |  |  |  | 44 |
| CO5 |  |  | 12 |  |  |  | 12 |
| CO6 |  | 17 |  |  |  |  | 17 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 19EE2044 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # MODERN FORECASTING TECHNIQUES | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | At the equinoxes (March 20 and September 23), the declination angle is zero.(True/False) | CO1 | U | 1 |
| 2. | The instrument used for measuring direct solar radiation is _________. | CO1 | R | 1 |
| 3. | List the tools available for weather forecasting. | CO2 | R | 1 |
| 4. | Mention any three performance metrics to evaluate the forecasting models. | CO2 | U | 1 |
| 5. | It is recommended to choose an __________ number for ‘k’ values in k-NN, if the number of attributes is even to avoid a tie in the class prediction. | CO3 | R | 1 |
| 6. | Identify the forecasting model in which probabilities are assigned to events within the model. | CO3 | A | 1 |
| 7. | Define Betz limit. | CO4 | R | 1 |
| 8. | A __________ pattern exists when there is a long-term increase or decrease in the time series data. | CO4 | A | 1 |
| 9. | Predict the best line of fit. | CO5 | A | 1 |
| 10. | The graph of time series is called _____________. | CO5 | U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Draw the seasonal variation of the declination of the sun. | CO1 | R | 3 |
| 12. | Explain support vectors in a support vector machine. | CO2 | U | 3 |
| 13. | State and explain the two parameters in weibull distribution curve. | CO3 | U | 3 |
| 14. | Summarize the uses of time series data. | CO4 | U | 3 |
| 15. | Find the tip speed ratio, if the tip of a blade is traveling at 80 mph and the wind speed is 20 mph. | CO5 | A | 3 |
| 16. | Write the advantages and disadvantages of recurrent neural network. | CO6 | U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. |  | With a neat diagram, explain the working principle of a pyranometer and also list its applications. | CO1 | U | 12 |
| 18. | a. | Explain the types of forecasting on the basis of the duration of the validity of forecasts. | CO2 | U | 6 |
|  | b. | Describe sky imaging forecasting with a neat diagram. | CO2 | U | 6 |
| 19. | a. | With a neat diagram, describe meteosat satellite and heliosat method of irradiance forecasting. | CO3 | U | 6 |
|  | b. | Compare and contrast GOES and POES with suitable examples. | CO3 | U | 6 |
| 20. | a. | Explain various types of time series data pattern with a neat diagram. | CO4 | U | 6 |
|  | b. | Write the components of solar radiation and also explain the various earth sun angles with a neat diagram which is necessary for solar resource assessment. | CO4 | U | 6 |
| 21. | a. | Summarize and brief the various types of stochastic learning approaches for renewable energy resource assessment. | CO5 | R | 4 |
|  | b. | Distinguish between biological neuron and artificial neuron model with a neat diagram. | CO5 | U | 8 |
| 22. | a. | If the wind speed is 4 m/s during Jan 2022, 4.2 m/s during Feb 2022 , 5m/s during march month and 5.6 m/s in April 2022. Predict the wind speed for May 2022 with the 3-month simple moving average method. | CO5 | A | 4 |
|  | b. | Estimate the power from the wind turbine with the following data. Wind speed , V = 10 m/sec ; Area = 7500 m, Power Coefficient, Cp = 0.35 ; Air density, ρ = 1.23 kg/m3 | CO5 | A | 8 |
| 23. |  | Write the procedures of weather forecasting. With a step-by-step diagram, explain the k-nearest neighbor algorithm. | CO2 | U | 12 |
|  |  | Compulsory: |  |  |  |
| 24. | a. | You are given a time series wind speed data set. Suggest a suitable technique to develop accurate forecasting model and also justify the model selection for the given data set. | CO6 | A | 6 |
|  | b. | Compare the architecture of RNN and traditional neural network with the help of a neat diagram. | CO6 | U | 6 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Recognize the basic physics about solar radiation geometry |
| CO2 | Outline the technical aspects of various solar and wind forecasting methods |
| CO3 | Analyze the configurations of widely used forecast models |
| CO4 | Utilize the various data sources for solar and wind resource assessment |
| CO5 | Predict solar and wind resources for managing the electrical power grid and its markets |
| CO6 | Develop accurate forecasting model |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 4 | 13 |  | - | - | - | 17 |
| CO2 | 1 | 28 | - | - | - | - | 29 |
| CO3 | 1 | 15 | 1 | - | - | - | 17 |
| CO4 | 1 | 15 | 1 | - | - | - | 17 |
| CO5 | 4 | 9 | 16 | - | - | - | 29 |
| CO6 | - | 9 | 6 | - | - | - | 15 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 19EE2051 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # IOT AND ITS APPLICATION IN ELECTRICAL ENGINEERING | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Define IoT. | CO1 / R | 1 |
| 2. | Wireless personal and local area network technologies that are commonly incorporated into IoT connectivity solutions are ________ and _________. | CO1 / R | 1 |
| 3. | Most Mesh networks have self-healing capabilities – True/False | CO2 / AP | 1 |
| 4. | The term Internet of Things was coined by ___________. | CO2 / R | 1 |
| 5. | The address size of IPV6 is __________. | CO3 / R | 1 |
| 6. | Name the first recognized IoT device. | CO3 / AP | 1 |
| 7. | Mention the components of Smart waste management using IOT. | CO4 / AP | 1 |
| 8. | Interoperability is enabled by __________. | CO4 / AP | 1 |
| 9. | A _________ is an object equipped with sensors that gather data which will be transferred over a network and actuators that allow things to act. | CO5 / U | 1 |
| 10. | ___________ enables data processing and filtering of data before moving it to the cloud. | CO6 / AP | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Differentiate Fog and Cloud Computing with a diagram. | CO1 / AP | 3 |
| 12. | Devise the difference between Data Warehouse and Data Lake. | CO2 / AP | 3 |
| 13. | Relate the four types of Analytics with one application each. | CO3 / AP | 3 |
| 14. | Represent the MQTT mandatory fixed length header with a block diagram. | CO4 / U | 3 |
| 15. | List the seven layers of Internet of Things (IoT). | CO5 / U | 3 |
| 16. | Explicate the four fundamental components of IoT. | CO6 / U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. |  | Sketch the Specifications of Arduino Uno R3 board. | CO1 / AP | 12 |
| 18. |  | Sketch the flow diagram of Universal Middleware Bridge (UMB). i. When a new device is plugged in the UMB (4 Steps). ii. When a new device is controlled and monitored (7 Steps). | CO2 / AP | 12 |

| 19. |  | Relate the hardware and software components of MicaZ with a block diagram. | CO3 / AP | 12 |
| --- | --- | --- | --- | --- |
| 20. |  | Categorize the difference between MicaZ and TelosB with respect to energy efficiency. | CO4 / AN | 12 |
| 21. |  | Connect the 3 types of QoS Level of MQTT with one application each. | CO5 / AP | 12 |
| 22. |  | Report the fundamentals to choose the right IoT cloud platform for building an application with a Mind Map. | CO5 / AP | 12 |
| 23. |  | Compare and Contrast Node MCU with Beagle Bone Black in terms of its specifications and applications. | CO4 / AN | 12 |
|  |  | Compulsory: |  |  |
| 24. |  | Sketch the 15 types of sensors used in IoT applications with one example each. | CO6 / AP | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the architecture framework of IoT |
| CO2 | Understand the architecture framework of IoT |
| CO3 | Select suitable Smart Sensors depending on the applications. |
| CO4 | Apply IoT for Smart and Digital Factories. |
| CO5 | Design and develop smart homes. |
| CO6 | Develop applications of IoT in Transportation |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 2 |  | 15 |  |  |  | 17 |
| CO2 | 1 |  | 16 |  |  |  | 17 |
| CO3 | 1 |  | 16 |  |  |  | 17 |
| CO4 |  | 3 | 2 | 12 |  |  | 17 |
| CO5 |  | 4 | 24 |  |  |  | 28 |
| CO6 |  | 3 | 13 |  |  |  | 16 |
|  |  |  |  |  |  |  | 112 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 20EE1001 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name: | # BASIC ELECTRICAL AND COMPUTER ENGINEERING | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | __________ is used to carry the water from valve house to power house in hydro power plant. | CO1/R | 1 |
| 2. | If the frequency of an AC system is 50Hz, find the time period (T). | CO1/A | 1 |
| 3. | Define Earthing. | CO2/R | 1 |
| 4. | A ___________motor is a special type of motor which is designed to run on either DC or single phase AC supply. | CO2/A | 1 |
| 5. | Which gate is known as universal gate? | CO3/U | 1 |
| 6. | In semiconductor devices, UJT stands for ______________. | CO3/R | 1 |
| 7. | List the advantages of Industry 4.0. | CO4/R | 1 |
| 8. | Write the different types of sensors. | CO4/R | 1 |
| 9. | Identify the most common type of a computer network. | CO5/R | 1 |
| 10. | Name few applications of blue tooth. | CO5/R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Summarize the advantages of renewable energy generation. | CO1/U | 3 |
| 12. | List the types of electric vehicles and also mention its vital parts. | CO2/R | 3 |
| 13. | Mention the uses of semiconductors in everyday life. | CO3/U | 3 |
| 14. | Draw the characteristics of a PN diode. | CO4/R | 3 |
| 15. | Differentiate router and modem. | CO5/A | 3 |
| 16. | Write the characteristics and challenges of big data. | CO6/U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | Explain the working principle of digital energy meter with a neat diagram. | CO1/U | 6 |
|  | b. | Draw the wiring diagram of a fluorescent lamp and explain the working principle. | CO1/R | 6 |
| 18. | a. | With a neat diagram explain the construction, working and applications of a DC Motor. | CO2/U | 8 |
|  | b. | Compare hybrid electric vehicle with electric vehicle. | CO2/A | 4 |

| 19. | a. | Explain the working of a photo diode with a neat diagram. | CO3/U | 6 |
| --- | --- | --- | --- | --- |
|  | b. | Draw the input output characteristics of a transistor in common emitter configuration. | CO3/R | 6 |
| 20. | a. | With a neat block diagram, explain automatic irrigation system. | CO4/U | 6 |
|  | b. | Distinguish between sensors and transducers. | CO4/A | 6 |
| 21. | a. | Define LAN, MAN and WAN and explain how these are different from each other? | CO5/R | 12 |
| 22. | a. | Explain the construction, working and applications of a three phase induction motor with a neat diagram. | CO2/U | 12 |
| 23. | a. | With a neat block diagram, explain the working of the following. - Basic water level indicator. - X-ray machine | CO4/U | 12 |
|  |  | Compulsory: |  |  |
| 24. | a. | Compare smart grid with the conventional grid. | CO6/A | 4 |
|  | b. | Summarize the benefits of 5G technology. | CO6/U | 4 |
|  | c. | Write the applications of AI. | CO6/R | 4 |

Reg. No. _____________

End Semester Examination – May / June – 2022

| # Code : | # 20EE1003 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # SENSORS AND MEASUREMENT TECHNIQUES IN BIOTECHNOLOGY | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Define ohm’s law. | CO1 / R | 1 |
| 2. | Determine V3 for the circuit shown below. | CO1 / A | 1 |
| 3. | Energy Meter is an integrating instrument. Say True/False. | CO2 / A | 1 |
| 4. | If a voltmeter is to be connected in A.C line, and then which connection is appropriate? | CO2 / A | 1 |
| 5. | Write the various types of touch screen technologies. | CO3 / R | 1 |
| 6. | In digital instruments, quantities are digitized using __________. | CO3 / U | 1 |
| 7. | A ____________ is a system which mimics the function of a human nose to identify a particular odour. | CO4 / U | 1 |
| 8. | List the sensors used in smart farming. | CO4 / R | 1 |
| 9. | If a resistor suddenly decreases in value, what will happen to the current through the resistor? | CO5 / A | 1 |
| 10. | The temperature coefficient of Resistance Thermal Detector is ___________. | CO5 / U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | State and explain faraday’s law of electromagnetic induction. | CO1 / R | 3 |
| 12. | List the classification of measuring instruments with examples. | CO2 / R | 3 |
| 13. | Differentiate analog and digital CRO. | CO3 / A | 3 |
| 14. | Write the applications of LDR. | CO4 / R | 3 |
| 15. | What is a strain gauge and how do they work? | CO5 / U | 3 |
| 16. | What is a bio-sensor? Give some examples of commercially available biosensors. | CO6 / U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 17. | a. | State and explain Kirchhoff’scurrent and voltage laws with suitable examples. | CO1 / A |  | 8 |  |
|  | b. | Distinguish between active and passive components. | CO1 / U |  | 4 |  |
| 18. | a. | Explain the working of Permanent Magnet Moving Coil (PMMC) instrument with a neat diagram. | CO2 / U |  | 8 |  |
|  | b. | Compare Moving Iron and PMMCinstruments. | CO2 / A |  | 4 |  |
| 19. |  | Explain the working of digital storage oscilloscope with a neat block diagram. Also write the applications of the DSO. | CO3 / U |  | 12 |  |
| 20. |  | Define transducer. How do you classify the transducers? Describe the different type of transducers with examples. | CO4 / R |  | 12 |  |
| 21. |  | Draw the static characteristic curves and describe the working of the following temperature transducers. - RTD - Thermocouple | CO5 / U |  | 12 |  |
| 22. |  | With a neat diagram, explain the measurement of displacement using LVDT. | CO5 / U |  | 12 |  |
| 23. | a. | Determine the value of ‘S’ in a Wheatstone’s bridge with P = 100 Ω, Q = 1000 Ω and R = 40 Ω, If the galvanometer shows zero deflection. . | CO2 / A |  | 6 |  |
|  | b. | Elucidate different types of torque required for the operation of measuring instruments. | CO2 / R |  | 6 |  |
|  |  | Compulsory: |  |  |  |  |
| 24. |  | With a neat block diagram, describe the architecture of a smart sensor. Also write the applications of smart sensors. |  | CO6 / U |  | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | # Understand the basic circuit components. |
| CO2 | # Describe working of the electronic measuring instruments. |
| CO3 | # Know the different display and recording devices. |
| CO4 | # Identify sensors and instruments needed for measurement and control. |
| CO5 | # Know the working principle and the characteristics of different transducers. |
| CO6 | # Choose suitable smart sensors for various biotechnology applications. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 4 | 4 | 9 | - | - | - | 17 |
| CO2 | 9 | 8 | 12 | - | - | - | 29 |
| CO3 | 1 | 13 | 3 | - | - | - | 17 |
| CO4 | 16 | 1 | - | - | - | - | 17 |
| CO5 | - | 28 | 1 | - | - | - | 29 |
| CO6 | - | 15 | - | - | - | - | 15 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 20EE2001 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ELECTRIC VEHICLE DESIGN | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | The ratio of electric motor power to the sum of electric motor power and IC engine power is defined as ______. | CO1 | R | 1 |
| 2. | Lithium polymer batteries are cheaper than Lithium-Ion batteries. – True or False | CO1 | U | 1 |
| 3. | The device preferred for DC Chopper operation due to its high switching frequency and low power is ______. | CO2 | A | 1 |
| 4. | The Hall sensor in BLDC motor is placed after every ___ degree. | CO2 | AP | 1 |
| 5. | In the expression of the aerodynamic drag force (Fad), Cd indicates ____ coefficient. | CO3 | R | 1 |
| 6. | When regenerative braking is used a certain amount of the energy is recovered; the maximum practical limit on the recovery of kinetic energy is about ____ %. | CO3 | U | 1 |
| 7. | _______ is defined as the force at the rims or the outer edges of the driving wheels of moving trains. | CO4 | R | 1 |
| 8. | Mention any popular EV driving cycle. | CO4 | A | 1 |
| 9. | Define machine learning. | CO5 | U | 1 |
| 10. | A ______ sensor is a system uses light from the near-infrared laser to detect stationary or moving targets. | CO5 | R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Find the energy released in a flywheel, if I(moment of inertia) = 2, and ω(angular velocity) = 2 radians per second. | CO1 | E | 3 |
| 12. | Distinguish BLDC Motor with Brushed DC Motor. | CO2 | A | 3 |
| 13. | Draw the Electric Drive Train Diagram. | CO3 | U | 3 |
| 14. | Sketch the block diagram of a DC motor model. | CO4 | U | 3 |
| 15. | List any three sensors used in self-driving cars. | CO5 | A | 3 |
| 16. | Compare the EV’s AudiE-tron and BMW iX | CO6 | AP | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | a. | Brief about the historical development of Electric Vehicle. | CO1 | R | 8 |
|  | b. | Compare Lithium ion and Lithium Polymer Batteries. | CO1 | U | 4 |

| 18. | a. | Explain the four-quadrant operation of chopper-based DC Motor with necessary diagrams and waveforms. | CO2 | AP | 8 |
| --- | --- | --- | --- | --- | --- |
|  | b. | Outline the basic SRM drive system. | CO2 | R | 4 |
| 19. | a. | Deliberate the transmission efficiency and consideration of vehicle mass of Electric Vehicle. | CO3 | A | 12 |
| 20. | a. | Explain the total tractive effort in the analysis of an electric vehicle performance modelling with necessary diagrams. | CO4 | A | 12 |
| 21. | a. | With necessary diagrams, exemplify the use Artificial Intelligence (AI) Technique in Electric Vehicle (EV). | CO5 | A | 8 |
|  | b. | Compare RADAR sensor and LIDAR sensor. | CO5 | AP | 4 |
| 22. | a. | Draw the VSI fed Induction motor drive diagram. | CO2 | R | 4 |
|  | b. | Sketch the block diagram for the BLDC drive system. | CO2 | U | 4 |
|  | c. | List out the applications and advantages of BLDC motor. | CO2 | A | 4 |
| 23. | a. | Describe the various level of self-driving cars. | CO5 | U | 8 |
|  | b. | Brief out self-healing sensors. | CO5 | A | 4 |
| Compulsory: |  |  |  |  |  |
| 24. | a. | Discuss the case study on Honda Insight. | CO6 | A | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Realize the need of Electric vehicles |
| CO2 | State different types of Electric & Hybrid Vehicles |
| CO3 | Use the energy on-board optimally |
| CO4 | Understand the design and mathematical modelling of EV and drives |
| CO5 | Analyze the latest control techniques for vehicle control |
| CO6 | Simulate and observe the behavior of the EV |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 9 | 5 | - | - | 3 | - | 17 |
| CO2 | 8 | 4 | 9 | 8 | - | - | 29 |
| CO3 | 1 | 4 | - | 12 | - | - | 17 |
| CO4 | 1 | 3 | - | 13 | - | - | 17 |
| CO5 | 1 | 9 | 4 | 15 | - | - | 29 |
| CO6 | - | - | 3 | 12 | - | - | 15 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – June – 2022

| # Code : | # 20EE3002 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ENERGY CONSERVATION AND AUDITING | # Max. Marks : | # 100 |

ANSWER ALL QUESTIONS (5 X 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Illustrate the needs and objectives of Energy Audit and also discuss about Energy conservation and Energy Conservation Act 2001. | CO1 / U | 10 |
|  | b. | Brief about Internal and External Audit | CO1 / R | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Discuss the responsibilities of energy managers. | CO1 / R | 10 |
|  | b. | Explain the energy conservation in house hold and Industrial sector | CO1 / U | 10 |
| 3. |  | In what way energy has been conserved in the following - Electric motors - Refrigeration and Air-conditioning - Lighting - Boiler (V) Pumps | CO2 / U | 20 |
| (OR) |  |  |  |  |
| 4. |  | Discuss briefly on the following: - BEE’s standards for Equipment and Appliances. - Recycling - Quality Control - Idle Redundant running | CO2 / R | 20 |
| 5. | a. | Explain any 8energy audit instruments which are used in energy audit. Also mention the parameters to be measure. | CO3 / U | 10 |
|  | b. | Identify various Energy Conservation Opportunities available in industries | CO3 / A | 10 |
| (OR) |  |  |  |  |
| 6. | a. | How can one optimize the input energy requirement and increase efficiency? | CO4 / U | 10 |
|  | b. | Explain how matching energy usage to requirement can enhance energy efficiency. | CO5 / R | 10 |
| 7. | a. | Prepare the questionnaire to review the performance any 4 appliances used in industries | CO5 / R | 10 |
|  | b. | What are the data/Information has to discuss during Introductory Meeting and Walk through Tour? Also discuss about how to review the energy audit report? | CO5 / U | 10 |
| (OR) |  |  |  |  |
| 8. |  | Discuss about the following: Phase I - Pre Audit Phase Phase II - Audit Phase Phase III - Post Audit Phase | CO3 / A | 20 |
|  |  | Compulsory: |  |  |
| 9. | a. | Draw the Process Flow Diagram for the paper or Food industries and discuss about the energy transferring in various stages. | CO6 / R | 10 |
|  | b. | What is energy balancing diagram and brief about its importance? | CO6 / U | 10 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Demonstrate an understanding of energy audit approaches. |
| CO2 | Develop the strategic plan to conduct energy audit. |
| CO3 | Find the list of components required for energy audit. |
| CO4 | Survey the important parameters required for auditing. |
| CO5 | Conduct the energy audit. |
| CO6 | Analyze the audit data and give the recommendation to the management to conserve energy. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 20 | 20 |  |  |  |  | 40 |
| CO2 | 20 | 20 |  |  |  |  | 40 |
| CO3 |  | 10 | 30 |  |  |  | 40 |
| CO4 |  | 10 |  |  |  |  | 10 |
| CO5 | 10 | 10 | 10 |  |  |  | 30 |
| CO6 | 10 | 10 |  |  |  |  | 20 |
|  |  |  |  |  |  |  | 180 |