Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 09ME101/12ME103/DME101/ME106 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name: | # BASIC MECHANICAL ENGINEERING | # Max. Marks: | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | Define the term EC Engine. | CO1 | R | 1 |
| 2. | Babcock Wilcox Boiler is an example for ___________. | CO1 | U | 1 |
| 3. | Water Surface in the Reservoir is known as ___________. | CO1 | R | 1 |
| 4. | What is a Condenser? | CO1 | A | 1 |
| 5. | Brass is an alloy of __________ and ___________. | CO1 | A | 1 |
| 6. | Low carbon steel is known as __________. | CO1 | R | 1 |
| 7. | List few examples for arc welding. | CO1 | U | 1 |
| 8. | Expand the term TIG in welding. | CO1 | R | 1 |
| 9. | Give an example of drilling machine operations. | CO1 | U | 1 |
| 10. | A milling machine is commonly used in ___________. | CO1 | A | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Differentiate Two stroke engines vs Four stroke engines. | CO1 | U | 3 |
| 12. | State the function of nuclear reactors in nuclear power plant. | CO1 | A | 3 |
| 13. | List out the properties of non-ferrous metals. | CO1 | R | 3 |
| 14. | What are the applications of gas welding? | CO1 | U | 3 |
| 15. | List out the different types of casting process. | CO1 | R | 3 |
| 16. | Define lathe machine. | CO1 | R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | a. | Classify the boiler. | CO1 | R | 3 |
|  | b. | Explain the working of a four-stroke diesel engine with the help of neat sketches. | CO1 | A | 9 |
| 18. |  | Give a schematic layout of a storage type hydro-electric power plant and explain the function of each component of the power plant. | CO1 | An | 12 |
| 19. |  | Discuss the types, properties and uses of steel in detail. | CO1 | A | 12 |
| 20. | a. | List any five molding tools. | CO1 | A | 2 |
|  | b. | Explain briefly the gas welding process and its parts in detail. | CO1 | An | 10 |

| 21. | a. | With a neat sketch, explain description parts and working principle of cupola furnace. | CO1 | R | 10 |
| --- | --- | --- | --- | --- | --- |
|  | b. | List out the different types of pattern used in casting. | CO1 | U | 2 |
| 22. |  | Explain the working of a Cochran boiler with the help of neat sketches. | CO1 | A | 12 |
| 23. |  | With a neat layout of thermal (steam) power plant, explain its construction and working principle. | CO1 | U | 12 |
|  |  | Compulsory: |  |  |  |
| 24. | a. | Explain briefly any five types of lathe. | CO1 | A | 10 |
|  | b. | Define milling machine. | CO1 | R | 2 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Students will be able to get general view about Mechanical Engineering Branch. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 28 | 23 | 51 | 22 | - | - | 124 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 12ME213 / 14ME2014 / 17ME2004 / 18ME2036 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # THERMODYNAMICS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Isothermal process is a constant __________ process. | COI / U | 1 |
| 2. | What is a thermodynamic process? | COI / R | 1 |
| 3. | Define energy. | CO2 / R | 1 |
| 4. | State Zeroth law of thermodynamics. | CO2 / R | 1 |
| 5. | What is meant by steady flow process? | CO3 / U | 1 |
| 6. | First law of thermodynamics deals with conservation of ________. | CO3 / R | 1 |
| 7. | # Define Coefficient of performance (COP). | CO4 / R | 1 |
| 8. | What is the function of nozzle? | CO5 / U | 1 |
| 9. | Entropy is the function of _________. | CO6 / R | 1 |
| 10. | Fusion curve, Vaporization curve and sublimation curves meet at the _____ point. | CO6 / U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Distinguish between the terms path and process. | COI / U | 3 |
| 12. | Draw the p-v and T-S diagram for an isentropic expansion process. | CO2 / U | 3 |
| 13. | Deduce the steady-flow energy equation for reciprocating compressor. | CO3 / U | 3 |
| 14. | State Kelvin-Planck and Clausius statements. | CO4 / R | 3 |
| 15. | What is entropy generation? | CO5 / R | 3 |
| 16. | Define (a) dry steam, (b) wet steam and (c) superheated steam. | CO6 / R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. |  | Define a Thermodynamics system. Explain its different types. | CO1 / U | 12 |
| 18. |  | A system undergoes a cyclic process composed of four processes 1-2, 2-3, 3-4, and 4-1. The energy transfer is tabulated below: Process; Q kJ/min; W kJ/min; ∆U kJ/min. 1-2; 400; 150; ---. 2-3; 200; ---; 300. 3-4; -200; ---; ---. 4-1; 0; 75; ---. Complete the table and determine the power output. | CO2/ A | 12 |

| 19. |  | Air flows steadily at the rate of 0.5 kg/s through an air compressor, entering at 7 m/s velocity, 100 kPa pressure and 0.95 m3/kg volume and leaving at 5 m/s, 700 kPa and 0.19 m3/kg. The internal energy of the air leaving is 90 kJ/kg greater than that of the air entering. Cooling water in the compressor jackets absorbs heat from the air at the rate of 58 kW. a) Compute the rate of shaft work input to the air in kW. b) Find the ratio of the inlet pipe diameter to the outlet pipe diameter. | CO3/ A | 12 |
| --- | --- | --- | --- | --- |
| 20. |  | A cyclic heat engine operates between a source temperature of 800 °C and a sink temperature of 30 °C. What is the least rate of heat rejection per kW net output of the engine? | CO4 / A | 12 |
| 21. |  | 0.05 m3 of air at a pressure of 8 bar and temperature 280 °C expands to eight times its original volume and the final temperature after expansion is 25 °C. Calculate change of entropy of air during the process. Assume Cp=1.005 kJ/kg K and Cv=0.712 kJ/kg K. | CO5 / A | 12 |
| 22. |  | Explain the operation of a cyclic refrigerator with a block diagram. | CO3 / U | 12 |
| 23. |  | 3 Kg of gas at 75° C is heated at constant volume until its pressure is 2 times the initial pressure. Determine (i) The Final temperature (ii) The heat transfer (iii) Change in internal energy, iv) Change in Enthalpy and (v) Work done. Assume Cp=1.00 kJ/kgK and Cv=0.071kJ/kg K. | CO1 / A | 12 |
|  |  | Compulsory: |  |  |
| 24. |  | What is pure substance? Draw the phase equilibrium diagram for a pure substance on T-s plot with relevant constant property lines. | CO6 / U | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the basic concepts in thermodynamics and its application in different fields. |
| CO2 | Apply energy balance to systems and control volumes, in situations involving heat and work interactions. |
| CO3 | Evaluate the performance of energy conversion devices. |
| CO4 | Differentiate between high grade and low grade energies. |
| CO5 | Apply gas laws to solve problems related to gas mixtures. |
| CO6 | Evaluate changes in thermodynamic properties of substances. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 1 | 16 | 12 | - | - | - | 29 |
| CO2 | 2 | 3 | 12 | - | - | - | 17 |
| CO3 | 1 | 16 | 12 | - | - | - | 29 |
| CO4 | 4 | - | 12 | - | - | - | 16 |
| CO5 | 3 | 1 | 12 | - | - | - | 16 |
| CO6 | 1 | 16 | - | - | - | - | 17 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 14ME2004 / 17ME2006 / 18ME2020 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # MANUFACTURING PROCESSES | # Max. Marks : | # 100 |

| Q. No. | Questions |  | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | Name the binders used in molding sand during casting sand. | CO1 / Remember |  | 1 |
| 2. | Write any two advantages of shell casting process. | CO1 / Apply |  | 1 |
| 3. | List out any two defects in hot rolling process. | CO2 / Remember |  | 1 |
| 4. | Write the advantage of high energy rate forming process. | CO2 / Apply |  | 1 |
| 5. | Name the process by which blades for table fan is made. | CO3 / Evaluate |  | 1 |
| 6. | _____________is an example for sheet metal forming process. | CO3 / Analyze |  | 1 |
| 7. | How the metals to be welded are connected to each other in spot welding? | CO4 / Evaluate |  | 1 |
| 8. | In CO2 lasers welding process employs gas mixture of ___________. | CO4 / Remember |  | 1 |
| 9. | List out any two properties dielectric fluid used in Electrical Discharge Machining process | CO5 / Understand |  | 1 |
| 10. | Name any two binder used in powder metallurgy process. | CO6 / Remember |  | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Explain the various defects in casting and its remedial measures to be followed during casting process. | CO1 / Understand | 3 |
| 12. | Explain any one method of tube drawing process with simple schematic representation. | CO2 / Remember | 3 |
| 13. | What is forging process and its advantages? | CO3 / Apply | 3 |
| 14. | Discuss the various defects in Welding operations and its remedial measures. | CO4 / Evaluate | 3 |
| 15. | Explain the working principle of abrasive jet machining process. | CO5 / Remember | 3 |
| 16. | What is blending process and state its importance in powder metallurgy process? | CO6 / Understand | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no. 17 to 23. Q.no. 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. |  | Explain the process of investment casting with neat sketches: List out its advantages and disadvantages. | CO1 / Remember | 12 |
| 18. |  | With relevant sketches, illustrate the centrifugal casting process. | CO1 / Understand | 12 |
| 19. |  | Describe the different methods employed in tube drawing process with neat Sketch. | CO2 / Remember | 12 |
| 20. | a. | Briefly explain the various defects in forging process. | CO3 / Understand | 6 |
|  | b. | Differentiate between open die and closed die forging process and its causes. | CO3 / Apply | 6 |
| 21. |  | Explain the working principle TIG welding process with neat sketch. | CO4 / Remember | 12 |
| 22. |  | With neat sketch, describe the working of Laser beam welding process, its advantages and limitation. | CO4 / Understand | 12 |
| 23. |  | In detail, explain the working principle of Electron Beam Machine process with neat sketch and its advantages, limitations. | CO5 / Apply | 12 |
|  |  | Compulsory: |  |  |
| 24. |  | Describe the various steps involved in powder metallurgy process. Discuss the sintering mechanism with a sketch. | CO6 / Apply | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Select the appropriate casting process to produce complex parts. |
| CO2 | Develop products with superior mechanical properties using bulk forming processes. |
| CO3 | Perform various sheet metal operations on metal sheets. |
| CO4 | Create permanent joints in working assemblies using welding techniques. |
| CO5 | Suggest appropriate unconventional manufacturing process for the machining of difficult-to-machine materials. |
| CO6 | Enhance the properties of components using powder metallurgy |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 13 | 15 | 1 | - | - | - | 29 |
| CO2 | 16 | - | 1 | - | - | - | 17 |
| CO3 | - | 6 | 9 | 1 | 1 | - | 17 |
| CO4 | 13 | 12 | - | - | 4 |  | 24 |
| CO5 | 3 | 1 | 12 | - | - | - | 16 |
| CO6 | 1 | 3 | 12 | - | - | - | 16 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 14ME2007 / 17ME2026 |  | # Duration : | # 3hrs |
| --- | --- | --- | --- | --- |
| # Sub. Name : | # FLUID POWER CONTROL ENGINEERING |  | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Define the term fluid power. | CO1 / U | 1 |
| 2. | List the advantages of fluid power system. | CO1 / R | 1 |
| 3. | List various types of hydraulic motors. | CO2 / R | 1 |
| 4. | ------------------- converts hydraulic energy into mechanical work either by linear motion or rotary motion. | CO2 / U | 1 |
| 5. | Classify hydraulic fluids. | CO3 / U | 1 |
| 6. | Draw the symbol for pressure relief valve. | CO3 / R | 1 |
| 7. | Draw the ISO symbol for hydraulic accumulator. | CO4 / A | 1 |
| 8. | State the function of flow control valve. | CO4 / U | 1 |
| 9. | What is fluidics? | CO5 / R | 1 |
| 10. | How will you represent OR gate symbol and its truth table? | CO6 / A | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | List the applications of fluid power. | CO1 / R | 3 |
| 12. | Compare hydraulic and pneumatic system. | CO2 / R | 3 |
| 13. | What is the need for temperature compensation in a flow-control valve? | CO3 / R | 3 |
| 14. | Explain the operation of tandem-type cylinder and mention its applications. | CO4 / U | 3 |
| 15. | Draw the pneumatic circuit for AND logic. | CO5 / A | 3 |
| 16. | Classify various types of fluidic sensors. | CO6 / A | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | Identify the essential components of pneumatics system with neat diagram. | CO1/U | 12 |
| 18. | a. | Sketch and discuss the working principle for the following hydraulic motors : (a) External gear motor (b) Vane motor. | CO2 /U | 12 |
| 19. | a. | Apprise the function of intensifier and discuss with a hydraulic circuit. | CO4/ A | 12 |
| 20. | a. | Draw two types of accumulator circuits and discuss the working. | CO3 / U | 12 |

| 21. | a. | Build the Synchronizing hydraulic circuits with - Series piping - Tie rod cyclinders. | CO3 / U | 12 |
| --- | --- | --- | --- | --- |
| 22. | a. | How solenoid will work, explain with neat diagram? | CO5 / U | 6 |
|  | b. | Analyze the working of relay with a practical example. | CO5 / U | 6 |
| 23. | a. | Design and develop a pneumatic circuit for the sequence of A+B+A-B-. | CO6 / A | 12 |
|  |  | Compulsory: |  |  |
| 24. | a. | Summarize the working of following fluidic sensor with neat sketch - Coanda Effect - Cone-Jet Sensor. | CO6 / A | 12 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 14ME2015 / 17ME2011 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # THERMAL ENGINEERING I | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | __________ is an example for water tube boiler. | CO1 / R | 1 |
| 2. | The enthalpy of evaporation of water at 1000C is ________. | CO1 / U | 1 |
| 3. | What is the function of steam nozzle? | CO2 / U | 1 |
| 4. | Name different types of nozzles. | CO2 / R | 1 |
| 5. | An example of velocity compounded impulse turbine is ________. | CO3 / R | 1 |
| 6. | Define thermal efficiency of Rankine cycle. | CO4 / R | 1 |
| 7. | Volumetric efficiency of a reciprocating air compressor is defined as the ratio of actual volume to ________. | CO5 / R | 1 |
| 8. | In a reciprocating air compressor the compressor work is maximum during ________ process. | CO5 / U | 1 |
| 9. | The refrigerant used in water-lithium bromide absorption system is ________. | CO6 / R | 1 |
| 10. | In a refrigeration system, heat rejection takes place in ________. | CO6 / U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | State how the boilers are classified. | CO1 / R | 3 |
| 12. | Why nozzles are made convergent – divergent? | CO2 / U | 3 |
| 13. | Draw the profile of the blades used in an impulse and reaction turbine. | CO3 / U | 3 |
| 14. | State the advantages of Reheat cycle. | CO4 / R | 3 |
| 15. | How a single stage air compressor differs from double stage one? | CO5 / U | 3 |
| 16. | Differentiate between refrigeration and air-conditioning. | CO6 / U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | The following observation were recorded during a boiler trial: Pressure of steam = 10 bar, Steam condensed = 540 kg/hr, Fuel used = 65 kg/hr, Moisture in fuel = 2% by mass, Mass of dry flue gases = 9 kg/kg of fuel, Lower calorific value of fuel = 32,000 kJ/kg Temperature of the flue gases = 325° C Temperature of boiler house = 28° C, Feed water temperature = 50° C , Mean specific heat of flue gases = 1 kJ/kg K, Dryness fraction of steam = 0.95, Specific heat of superheated steam = 2.1kJ/kgK. Draw the heat balance sheet for the boiler. | CO1 /A | 12 |
| 18. | a. | Dry saturated steam enters a steam nozzle at a pressure of 15 bar and is discharged at a pressure of 2.0 bar. If the dryness fraction of discharge steam is 0.96, what will be the final velocity of steam? Neglect initial velocity of steam. If 10% of heat drop is lost in friction, find the percentage reduction in the final velocity. | CO2 / A | 12 |
| 19. | a. | Explain the term ‘compounding of steam turbine’ and discuss the pressure-velocity compounding of turbine with neat sketch. | CO3 / A | 12 |
| 20. | a. | Derive an expression for the work donefor a single stage reciprocating air compressor without clearance, when compression is isentropic. | CO5 /An | 12 |
| 21. | a. | With a neat sketch, explain the working principle of vapour compression refrigeration system. | CO6 / U | 12 |
| 22. | a. | Explain the working of a simple Rankine cycle with a neat sketch. | CO4 / U | 12 |
| 23. | a. | A single stage, single acting reciprocating air compressor with 0.3m bore and 0.4m stroke runs at 400 rpm. The suction pressure is 1 bar at 300K and the delivery pressure is 5 bar. Find the power required to run it, if compression is isothermal, compression follows the law pv1.3= C and compression is reversible adiabatic. | CO5 / A | 12 |
|  |  | Compulsory: |  |  |
| 24. | a. | Explain vapour absorption refrigeration system with a neat sketch. | CO6 / U | 12 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 14ME2016 / 17ME2018 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # THERMAL ENGINEERING II | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Four stroke engines can be identified with _________. | COI / R | 1 |
| 2. | The brake and indicated power of an internal combustion engine are 5 kW and 6 kW respectively. The frictional power will be _________. | COI / R | 1 |
| 3. | _________ cycle is also called as constant pressure cycle. | CO2 / R | 1 |
| 4. | In a dual cycle heat addition process takes place at _________. | CO2 / R | 1 |
| 5. | Air that leaks into the room through window cracks and door openings is called _________. | CO3 / U | 1 |
| 6. | Define sensible heat factor. | CO3 / R | 1 |
| 7. | # Define Mach number. | CO4 / R | 1 |
| 8. | Give two practical examples for the Fanno flow. | CO5 / U | 1 |
| 9. | Define propulsive efficiency. | CO6 / R | 1 |
| 10. | Gas turbine works based on _______ cycle. | CO6 / U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Why is lubrication required for an internal combustion engine? | COI / U | 3 |
| 12. | Determine the air standard efficiency of diesel cycle for a compression ratio of 13 and cut-off ratio of 1.96. | CO2 / A | 3 |
| 13. | What are the main sources of sensible heat gain in air conditioning? | CO3 / U | 3 |
| 14. | Sketch the isentropic and adiabatic expansion process in p-V and T-s Diagram | CO4 / R | 3 |
| 15. | Differentiate between Fanno flow and Rayleigh flow. | CO5 / U | 3 |
| 16. | What are the advantages of Ram jet engine? | CO6 / R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. |  | Explain battery coil ignition system of an Internal Combustion engine with a neat sketch. | CO1 / U | 12 |
| 18. |  | Derive an expression for the air standard efficiency of an Otto cycle. | CO2/ An | 12 |
| 19. |  | Explain summer air-conditioning system suitable for hot and dry weather with a neat sketch. | CO3 / U | 12 |
| 20. |  | Air flowing at a duct has a velocity of 300 m/s, pressure 1bar, temperature 290 K. Taking = 1.4& R = 287 J/kgK. Determine (i) Stagnation pressure and temperature (ii) Velocity of sound in the dynamic and stagnation conditions. | CO4 / A | 12 |
| 21. |  | A gas turbine works on air standard Brayton cycle. The initial condition of the air is 30ºC and 1 bar. The maximum pressure and temperature are limited to 4 bar and 700ºC. Determine the following (i) Cycle efficiency, (ii) Heat supplied per kg of air, (iii) work out put, (iv) Heat rejected per kg of air. | CO6 / A | 12 |
| 22. |  | The following observations were recorded during a test on a single cylinder, four stroke diesel engine: Cylinder diameter = 10 cm Piston stroke = 15 cm Engine speed = 480 rpm Indicated mean effective pressure = 755 kPa Brake wheel diameter = 62.5 cm Net load on the brake wheel = 170 N Calculate (i) Indicated power, (ii) brake power, (iii) the mechanical efficiency of the engine. | CO1 / A | 12 |
| 23. |  | The Mach number at the exit of a combustion chamber is 0.9. The ratio of the stagnant temperatures at exit and entry is 3.74.If the pressure and temperature of the gas at exit are 2.5 bar and 10000C respectively. Determine (a) Mach number, pressure and temperature of the gas at entry. (b) The heat supplied per kg of the gas | CO5 / A | 12 |
|  |  | Compulsory: |  |  |
| 24. |  | Explain the construction and working of a Turbo jet engine with a neat sketch. | CO6 / U | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Evaluate the performance of internal combustion engine |
| CO2 | Estimate the efficiency of gas power cycles |
| CO3 | Determine cooling loads in air–conditioning systems |
| CO4 | Explain isentropic flow through variable area |
| CO5 | Describe Fanno flow and Rayleigh flow |
| CO6 | Analyze gas turbines cycles and compare the operational aspects of jet engines. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 2 | 15 | 12 | - | - | - | 29 |
| CO2 | 2 | - | 3 | 12 | - | - | 17 |
| CO3 | 1 | 16 | - | - | - | - | 17 |
| CO4 | 4 | - | 12 |  |  |  | 16 |
| CO5 | 4 | - | 12 | - | - | - | 16 |
| CO6 | 4 | 13 | 12 | - | - | - | 29 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 14ME2019 / 16ME2011 / 17ME2024 / 18ME2010 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # HEAT AND MASS TRANSFER | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | State Fourier’s law of heat conduction. | CO1 / R | 1 |
| 2. | Write the equation to determine the temperature at the center of the sphere with internal heat generation. | CO1 / U | 1 |
| 3. | Rayleigh number is a product of __________ and ___________. | CO2 / U | 1 |
| 4. | What is the significance of Nusselt number? | CO2 / A | 1 |
| 5. | Define ‘Radiosity’. | CO3 / R | 1 |
| 6. | Define Radiation shape factor. | CO3 / R | 1 |
| 7. | LMTD stands for __________. | CO4 / R | 1 |
| 8. | # State two assumptions made in the analysis of a heat exchanger. | CO4 / U | 1 |
| 9. | What is subcooled boiling? | CO5 / R | 1 |
| 10. | Why heat transfer coefficient in nucleate boiling is greater than film boiling. | CO5 / A | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | The roof of an electrically heated home is 6m long, 8m wide and 0.25m thick and is made of flat layer of concrete whose thermal conductivity is 0.8 W/mK. The temperatures of the inner and outer surfaces of the roof at night are measured to be 15ºC and 4ºC respectively. Determine the rate of heat loss through the roof. | CO1/ An | 3 |
| 12. | Differentiate between natural and forced convection. | CO2 / U | 3 |
| 13. | A radiator in a domestic heating system operates at a surface temperature of 55 C. Determine the rate at which it emits radiant heat per unit area if it behaves as a black body. | CO3 /An | 3 |
| 14. | Draw the temperature profile of a parallel flow heat exchanger. | CO4 / U | 3 |
| 15. | Distinguish between film wise and drop wise condensation. | CO5 / U | 3 |
| 16. | Define Fick’s law of diffusion. | CO6 / R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. |  | Derive a steady state heat conduction equation in Cartesian ordinates. | CO1 / A | 12 |
| 18. |  | An air steam at 10 C is flowing along a heated plate at 90 C at a speed of 75 m/s. The plate is 45 cm long and 60 cm wide. Assuming the transition of boundary layer to take place at critical Reynolds number. Calculate the average values of friction coefficient and heat transfer coefficient for the full length of the plate. Hence calculate the rate of energy dissipation from the plate. | CO2 /An | 12 |
| 19. |  | Calculate the net radiant heat exchange per m2 area for two large parallel plates at temperatures of 427 C and 27 C. ɛℎ𝑜𝑡𝑝𝑙𝑎𝑡𝑒 = 0.9 and ɛ𝑐𝑜𝑙𝑑𝑝𝑙𝑎𝑡𝑒 = 0.6. If a polished aluminium shield is placed between them, find the % reduction in the heat transfer ɛ𝑠ℎ𝑖𝑒𝑙𝑑 = 0.4. | CO3 /An | 12 |
| 20. |  | Derive an expression for LMTD in a counter flow heat Exchanger.. | CO4 / A | 12 |
| 21. |  | Explain various regimes in pool boiling. | CO5 / A | 12 |
| 22. |  | # A current of 200A is passed through a stainless steel (k=19 W/mK) wire 3mm in diameter. The resistivity of the steel is 70 µΩcm and the length of the wire is 1m. The wire is submerged in a liquid at 110ºC and experiences a convective heat transfer coefficient of 4 kW/m2K. Calculate the temperature at the center of the wire. | CO1 /An | 12 |
| 23. |  | A counter flow concentric tube heat exchanger is used cool engine oil (c = 2130 J/kg K) from 160 C to 60 C with water, available at 25 C as the cooling medium. The flow rate of cooling water through the inner tube of 0.5 m diameter is 2 kg/s while the flow rate oil through the outer annulus OD = 0.7 m is also 2 kg/s. if the value of the overall heat transfer coefficient is 250 W/m2- K, how long must the heat exchanger be to meet its cooling requirement? | CO4 /An | 12 |
|  |  | Compulsory: |  |  |
| 24. |  | An open pan 20 cm in diameter and 8 cm deep contains water at 25 C and is exposed to dry atmospheric air. If the rate of diffusion of water vapour is 8.54 x 10-4 kg/h estimate the diffusion coefficient of water in air. | CO6 /An | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Formulate and analyze a heat transfer problem involving any of the three modes of heat transfer. |
| CO2 | Obtain exact solutions for the temperature variation using analytical methods where possible or employ approximate methods or empirical correlations to evaluate the rate of heat transfer |
| CO3 | Evaluate radiation heat transfer between black, gray surfaces and the surroundings. |
| CO4 | Design devices such as heat exchangers and also estimate the insulation needed to reduce heat losses where necessary. |
| CO5 | Apply boiling and condensation correlations to two phase flow processes |
| CO6 | Apply mass transfer correlations to process–based problems. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 1 | 1 | 12 | 15 | - | - | 29 |
| CO2 |  | 4 | 1 | 12 |  |  | 17 |
| CO3 | 2 |  |  | 15 | - | - | 17 |
| CO4 | 1 | 4 | 12 | 12 |  |  | 29 |
| CO5 | 1 | 3 | 13 | - | - | - | 17 |
| CO6 | 3 |  | - | 12 | - | - | 15 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 14ME2026 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # MECHANICS OF MACHINES | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Define degrees of freedom. | CO1 / U | 1 |
| 2. | Attachment of a car mirror is an example of _______ pair. | CO1 / U | 1 |
| 3. | The component of acceleration, parallel to the velocity of the particle at a given instant is_______ component. | CO1/ U | 1 |
| 4. | Define centripetal or radial component. | CO1 / R | 1 |
| 5. | What is called cam and follower? | CO2 / R | 1 |
| 6. | What is an oscillating follower? | CO2 / U | 1 |
| 7. | What are the types of cam? | CO2 / R | 1 |
| 8. | Define train value of the gear train. | CO3 / R | 1 |
| 9. | Define module in a gear. | CO3 / U | 1 |
| 10. | Which is the tight side in belt? | CO3 / U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Enumerate the difference between the machine and a structure. | CO1 / R | 3 |
| 12. | What is Coriolis component of acceleration? | CO1 / U | 3 |
| 13. | Write the motion of the follower. | CO2 / U | 3 |
| 14. | What is meant by reverted gear train? | CO3 / U | 3 |
| 15. | What is called idle gears? | CO3 / R | 3 |
| 16. | Define Creep of Belt. | CO3 / U | 3 |

| PART – C(6 X 12 = 72 MARKS) (Answer any five Questions from Q. No 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | Sketch and describe the four bar chain mechanism. Why it is considered to be the basic chain? | CO1 / A | 4 |
|  | b. | Sketch and explain any two inversions of four bar chain. | CO1 / A | 8 |
| 18. | a. | The crank of a slider crank mechanism rotates clockwise at a constant speed of 300 rpm. The crank is 15 cm and the connecting rod is 60 cm long. Determine: 1. Linear velocity and acceleration of the midpoint of the connecting rod, and 2. angular velocity and angular acceleration of the connecting rod, at a crank angle of 45° from inner dead centre position. | CO1 / An | 12 |

| 19. | a. | A cam is to be designed for a knife edge follower when the axis of the follower passes through the axis of cam shaft with the following data: - Follower to move outwards through 40 mm during 60º of cam rotation, - Dwell for the next 45°. - Follower to return to its original position during next 90º, - Follower to dwell for the rest of the cam rotation. The displacement of the follower is to take place with simple harmonic motion during both the outward and return strokes. The least radius of the cam is 50 mm. If the cam rotates at 300 rpm, determine the maximum velocity and acceleration of the follower during the outward stroke and return stroke. | CO2 /An | 12 |
| --- | --- | --- | --- | --- |
| 20. | a. | Explain the terms Pressure angle and Addendum. | CO3 / U | 4 |
|  | b. | A pair of gears having 40 and 30 teeth respectively are of 25o involute form. The addendum length is 5 mm and the module pitch is 2.5 mm. If the smaller wheel is the driver and rotates at 1500 rpm, find the velocity of sliding at the point of engagement and at the point of disengagement. | CO3 /An | 8 |
| 21. | a. | What are the important factors upon which the selection of belt drive depends? | CO3 / U | 4 |
|  | b. | Find the power transmitted by a belt running over a pulley of 600 mm diameter at 200 r.p.m. The coefficient of friction between the belt and the pulley is 0.25, angle of lap 160 degree and maximum tension in the belt is 2500 N. | CO3 /An | 8 |
| 22. | a. | Explain briefly the differences between simple, compound and epicyclic gear trains. | CO3 / U | 4 |
|  | b. | In an epicyclic gear train, an arm carries two gears A and B having 36 and 45 teeth respectively. If the arm rotates at 150 rpm in the anticlockwise direction about the centre of the gear A which is fixed, determine the speed of gear B. If gear A instead of being fixed, makes 300 rpm in the clockwise direction, what will be the speed of gear B? | CO3 /An | 8 |
| 23. | a. | A cam is to give the following motion to a knife-edge follower: (i). Outstroke during 60 degree of cam rotation. (ii). Dwell for the next 30 degree of cam rotation; (iii). Return stroke during next 60 degree of cam rotation, and (iv). Dwell for the remaining 210 degree of cam rotation. The stroke of the follower is 40 mm and the minimum radius of the cam is 50 mm. The follower moves with uniform velocity during both the outstroke and return strokes. Draw the profile of the cam when the axis of the follower passes through the axis of the cam shaft. | CO2 /An | 12 |
|  |  | Compulsory: |  |  |
| 24. | a. | Explain Angle of friction and coefficient of friction. | CO3 / U | 4 |
|  | b. | A multi plate clutch has three pairs of contact surfaces. The outer and inner radii of the contact surfaces are 100 mm and 50 mm respectively. The maximum axial spring force is limited to 1 kN. If the coefficient of friction is 0.35 and assuming uniform wear, find the power transmitted by the clutch at 1500 rpm. | CO3 /An | 8 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 14ME2027/ 17ME2019 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name: | # DYNAMICS OF MACHINERY | # Max. Marks: | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | For a rigid body to be in static equilibrium, the vector sum of all the ______ and about any reference point is zero. | CO1/ U | 1 |
| 2. | ______ is the net force applied at the crankpin perpendicular to the crank which gives the required turning moment on the crankshaft | CO1/ R | 1 |
| 3. | Higher is the fluctuation of energy in a cycle ________is the size of the flywheel. | CO 2/ U | 1 |
| 4. | The primary unbalanced force is maximum when the angle of inclination of the crank with the line of stroke is ______. | CO 3/ U | 1 |
| 5. | The balancing of rotating and reciprocating parts of an engine is necessary when it runs at ____________. | CO 3/ U | 1 |
| 6. | A disturbing mass m1 attached to a rotating shaft may be balanced by a single mass m2 attached in the same plane of rotation as that of m1 such that_______. | CO 3/ An | 1 |
| 7. | __________type of vibration is induced, when stretching the string of a guitar. | CO 4/ A | 1 |
| 8. | When there is reduction in amplitude over every cycle of vibration, then the body is said to have __________. | CO 4/ U | 1 |
| 9. | When the speed of the engine fluctuates continuously above and below the mean speed, then the governor is said to be _________. | CO 6/ R | 1 |
| 10. | When the pitching of a ship is upward, the effect of gyroscopic couple acting on it will be__________. | CO 6/ R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Prove a member with two forces and a member with two forces and a torque are in equilibrium. | CO 1/ An | 3 |
| 12. | Draw the turning moment diagram for a single cylinder double acting engine. | CO 2/ R | 3 |
| 13. | Define the conditions for static and dynamic balancing. | CO 3/ R | 3 |
| 14. | Calculate the equivalent stiffness of three springs connected in series | CO 4/ E | 3 |
| 15. | Derive the expression for the equivalent length of a stepped shaft. | CO 4/ An | 3 |
| 16. | What is the effect of gyroscopic couple on an automobile taking a turn? | CO 6/ R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | The four-bar linkage has crank 2 driven by torque M12;an external load P=120 N at an angle of 40º on link 4. For the particular position of the linkage shown find all the constraint forces and their reactions necessary for this link to be a position of equilibrium. B A 2 135º 4 Q P 3 O4 O2 Length of the Links O4O2 = 20cm AO2 = 15cm BO4 = 30cm BA = 46 cm QO4 = 12.5 cm 40º | CO 1/ E | 12 |
| 18. | a. | The turning moment diagram for a multicylinder engine has been drawn to a scale of 1 mm = 600 N-m vertically and 1 mm = 3° horizontally. The intercepted areas between the output torque curve and the mean resistance line, taken in order from one end, are as follows: + 52, 124, 92, - 140, 85, -72 and + 107 mm2, when the engine is running at a speed of 600 r.p.m. If the total fluctuation of speed is not to exceed l % of the mean, determine the mass of the flywheel, the radius of the flywheel is 0.5 m. | CO 2/ E | 12 |
| 19. | a. | A shaft carries four masses in parallel planes A, B, C and D in this order along its length. The masses at B and C are 18 kg and 12.5 kg respectively, and each has an eccentricity of 60 mm. The masses at A and D have an eccentricity of 80 mm. The angle between the masses at B and C is 100° and that between the masses at B and A is 190°, both being measured in the same direction. The axial distance between the planes A and B is 100 mm and that between B and C is 200 mm. If the shaft is in complete dynamic balance, determine: - The magnitude of the masses at A and D; and - The distance between planes A and D | CO 3/ E | 12 |
| 20. | a. | Four masses m1, m2, m3 and m4 are 200 kg, 300 kg, 250 kg and 250 kg respectively. The corresponding radii of rotation are 0.2 m, 0.15 m, 0.25 m and 0.3 m respectively and the angles between successive masses are 45°, 75° and 135°. Find the position and magnitude of the balance mass required, if its radius of rotation is 0.2 m. | CO 3/ E | 12 |
| 21. | a. | The mass of a single degree damped vibrating system is 7.5 kg and makes 24 free oscillations in 14 seconds when disturbed from its equilibrium position. The amplitude of vibration reduces to 0.25 of its initial value after 5 oscillations. Determine: 1. stiffness of the spring, 2. logarithmic decrement, 3. damping factor, 4. actual damping coefficient, 5. critical damping coefficient. | CO 4/ E | 12 |

| 22. | a. | A vertical steel shaft 15 mm diameter is held in long bearings 1 m apart and carries at its middle a disc of mass 15 kg. The eccentricity of the center of gravity of the disc from the center of the rotor is 0.30 mm. The modulus of elasticity for the shaft material is 200 GN/m2 and the permissible stress is 70 MN/m2. Determine: - The critical speed of the shaft; and - The range of speed over which it is unsafe to run the shaft. Neglect the mass of the shaft. | CO 4/ Evaluate | 12 |
| --- | --- | --- | --- | --- |
| 23. | a. | A machine has a mass of 100 kg and unbalanced reciprocating parts of mass 2 kg which move through a vertical stroke of 80 mm with simple harmonic motion. The machine is mounted on four springs, symmetrically arranged with respect to center of mass, in such a way that the machine has one degree of freedom and can undergo vertical displacements only. Neglecting damping, calculate the combined stiffness of the spring in order that the force transmitted to the foundation is 1/25thof the applied force, when the speed of rotation of machine crank shaft is 1000 r.p.m. When the machine is actually supported on the springs, it is found that the damping reduces the amplitude of successive free vibrations by 25%. Find: 1. the force transmitted to foundation at 1000 r.p.m., and 2. the force transmitted to the foundation at resonance. | CO 5/ Evaluate | 12 |
|  |  | Compulsory: |  |  |
| 24. | a. | A porter governor has equal arms each 250 mm long and pivoted on the axis of rotation. Each ball has a mass of 5 kg and mass of the central load on the sleeve is 25 kg. The radius of rotation of the ball is 150 mm when governor begins to lift and 200 mm when the governor is at maximum speed. Find the maximum and minimum speed and range of speed of the governor. | CO 6 / Evaluate | 12 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 14ME2029 / 17ME2020 / 18ME2016 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # DESIGN OF MACHINE ELEMENTS | # Max. Marks : | # 100 |

| Q. No. | Questions (Use of Approved Data Book & Data Sheets are permitted) | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | List out the few mechanical properties of materials related to strength. | CO2 | R | 1 |
| 2. | Define creep. | CO1 | U | 1 |
| 3. | Mention the importance of principal stresses. | CO2 | U | 1 |
| 4. | For which material, soderberg equation can be used? | CO2 | A | 1 |
| 5. | List out few types of couplings. | CO5 | R | 1 |
| 6. | Distinguish between shaft, axle and spindle. | CO4 | U | 1 |
| 7. | Mention few examples for permanent and temporary joints. | CO3 | U | 1 |
| 8. | Write the equation employed for the design of thick cylinders. | CO1 | U | 1 |
| 9. | Draw and show the usual cross-section of the arms of flywheel. | CO6 | U | 1 |
| 10. | Differentiate the flywheel and governer in their functions. | CO4 | U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Mention few applications of variable load and define it. | CO1 | A | 3 |
| 12. | Differentiate between hydrodynamic and hydrostatic bearing. | CO4 | An | 3 |
| 13. | Examine the reasons for failures occurring in the key. | CO2 | An | 3 |
| 14. | List the various stresses induced in the single eye and double eye of knuckle joint. | CO5 | A | 3 |
| 15. | State the use of connecting rod and crank shaft. | CO3 | An | 3 |
| 16. | Sketch a cone clutch and show its parts. | CO6 | An | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. |  | Describe the various theories of failures in detail with suitable examples. | CO2 | An | 12 |
| 18. |  | A bearing for an axial flow compressor is used to support a radial load of 2500N and thrust (axial) load of 1500N. The service required is 5years at the rate of 40 hrs per week. The speed of the shaft is 1000rpm. Select suitable ball bearing for the purpose. Diameter of the shaft is 50mm. | CO4 | E | 12 |

| 19. |  | A shaft running at 500rpm transmits a power of 10kW.Assume allowable shear stress as 40N/mm2, find the diameter of the shaft. | CO5 | E | 12 |
| --- | --- | --- | --- | --- | --- |
| 20. |  | Design a helical compression spring to carry a load 1.5kN with a deflection of 40mm, spring index is 5, allowable shear stress is 400 N/mm2 and modulus of rigidity is 8X104 N/mm2. | CO4 | E | 12 |
| 21. |  | The turning moment diagram for a multi cylinder engine is drawn to scales of 1mm = 600 N-m vertical and 1mm = 1.5 degree horizontally .The areas of loops above and below the mean torque line ,take in order are -32 , +406 , -267 , +333 , -310 , +226 , -374 , +274 , -256 mm2. The mean speed of the engine is 150 rpm and the fluctuation of speed of the flywheel is not to exceed +10% of the mean speed. Design a Cast Iron flywheel, find out the dimensions of flywheel rim also show the crank angle Vs torque diagram. | CO6 | E | 12 |
| 22. |  | Design a knuckle joint for transmitting an axial load of 55 kN for the following stresses: in tension 80 MPa, in compression 90 MPa, in shear 50 MPa. Tabulate the specification. | CO4 | E | 12 |
| 23. |  | An axial load of 20000N to be lifted up by using a screw jack. The screw pitch is 8 mm and the load to be lifted for 1000mm height. The collar mean diameter = 60mm. The screw is made of hardened steel and the shear stress = 75 N/mm2 and the bearing pressure is 12 N/mm2. Nut material shear stress = 35N/mm2. Design the power screw for the above specifications. | CO5 | E | 12 |
|  |  | Compulsory: |  |  |  |
| 24. |  | Design single plate clutch, effective on both sides is required to transmit 25kW power. Determine the inner and outer diameter of friction surfaces if the coefficient of friction is 0.25.Take the speed N=1500rpm. Plates are effective in both sides, so n = Number of pairs of friction surfaces = 2, Max pressure = 0.2 N/mm2. Also determine the axial thrust (Fa) to be provided for engaging and disengaging the clutch. | CO3 | E | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the standard design procedure for Design of machine elements. |
| CO2 | Analyze stresses acting on components and determine the size based on theories of failure. |
| CO3 | Design machine components for a given load condition using design data hand books. |
| CO4 | Decide specifications as per standards given in design data and select standard components to improve interchangeability. |
| CO5 | Design and develop nonstandard machine components. |
| CO6 | Prepare a detail design layout and drawing of machine components. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | - | 2 | 3 | - | 12 | - | 17 |
| CO2 | 1 | 1 | 1 | 15 | - |  | 18 |
| CO3 | - | 1 | - | 3 | 12 | - | 16 |
| CO4 | 1 | 1 | - | 3 | 24 |  | 29 |
| CO5 | 1 | - | 3 | - | 24 | - | 28 |
| CO6 | 1 | - | - | 3 | 12 | - | 16 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 14ME2037 / 17ME2033 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # PRODUCT DESIGN AND DEVELOPMENT STRATEGIES | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Decisions made in the ____________ phase cost very little in terms of the overall product cost but have a major effect on the cost of the product. | CO1/ R | 1 |
| 2. | Committed costs are those costs for which a _____________obligation exists in the product design and development process. | CO1 / R | 1 |
| 3. | Morphology of __________contains seven phases. | CO2 / R | 1 |
| 4. | ____________ computing is the latest technology in remote data storage. | CO2 / R | 1 |
| 5. | The two important criteria in selecting a material for a product is (1) Select a material for new product. (2) ___________________________. | CO3 / R | 1 |
| 6. | Theoretical modeling, Computational modeling and ___________ modeling are the basic techniques to be followed for producing a quality product. | CO3 / R | 1 |
| 7. | The product life cycle is also known to be ______________ curve. | CO4 / R | 1 |
| 8. | Concurrent engineering is also known as ____________engineering. | CO4 / R | 1 |
| 9. | The materials selection methodology and charts developed by ___________are highly and widely used around the world by all the design and manufacturing engineers. | CO5 / R | 1 |
| 10. | ___________alloy material generally used to reduce the weight of any machine or automobile component. | CO5 / R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | List out the vital factors considered for a good design. Give an example. | CO1/ U | 3 |
| 12. | Explain the importance of a prototype in a successful product design. | CO2/ U | 3 |
| 13. | Discuss briefly how performance characteristics of materials used in design. | CO3/ U | 3 |
| 14. | Explain how the mechanical loading on materials affect the form design. | CO4/ U | 3 |
| 15. | Classify and list the parameters selected for aesthetic appearance of a product. | CO5/ U | 3 |
| 16. | Why human factors engineering is applied in most of the consumer product. | CO6 / U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | List out all the seven phases in morphology of design Explain the first three phases in detail with relevant sketches. | CO1 /A | 6 |
|  | b. | Sketch neatly the inverted bathtub curve and explain the stages involved in the Product life cycle. | CO1 /A | 6 |
| 18. | a. | Compare between the scientific method and design method for approaching simplified design process with neat flowcharts. | CO1 /A | 6 |
|  | b. | Describe the five important steps involved in Problem-Solving Methodology. | CO1 /A | 6 |
| 19. | a. | Distinguish between concurrent engineering and sequential engineering with relevant sketches. | CO2 /An | 6 |
|  | b. | How the computer aided engineering helps in better product design. Sketch the cross-sectional view of a single cylinder IC engine and explain its design analysis. | CO2 /An | 6 |
| 20. | a. | Identify and explain the international codes and standards used for product design. | CO3 /An | 6 |
|  | b. | With neat sketches explain the stages of design improvement for railcar wheel. | CO3 /An | 6 |
| 21. | a. | Explain the steps in materials selection activities for the two different situations. - Selection of the materials for a new product or design. Re-evaluation of an existing product or design to reduce cost, increase reliability, improve performance, etc. | CO4 / E | 6 |
|  | b. | With a neat Schematic diagram of the design process suggest required design tools, materials, and process selection to arrive at the best combination of material and manufacturing process. | CO4 / E | 6 |
| 22. | a. | List out some important properties and applications of the following. (i) Physical (ii) Mechanical (iii) Chemical (iv) Electric properties (v) Thermal Properties. | CO5 / E | 6 |
|  | b. | Suggest a good material required for an Automotive Exhaust System by considering the product design specifications. | CO5 / E | 6 |
| 23. | a. | Explain various simulated service tests performed for a horizontal aluminum alloy motor in case of material substitution. | CO5 / C | 6 |
|  | b. | With an example list out the advantages of the general tolerancing to facilitate easy manufacturing. | CO5 / C | 6 |
|  |  | Compulsory: |  |  |
| 24. | a. | Explain the dimensioning rules and systems for the following with neat sketches. i) Aligned ii) Unidirectional iii) Tabular iv) Chain | CO6 / C | 6 |
|  | b. | Sketch and explain in detail about the technological properties of grey iron castings. | CO6 / C | 6 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Recognize the importance of product design and apply the design process in developing a product. |
| CO2 | Categorize various models in engineering design and select appropriate modelling and simulation techniques for analysing a product. |
| CO3 | Choose the right material selection method and calculate the economics of a material for a new product. |
| CO4 | Distinguish between the functional and production design and use the right manufacturing method by considering the influence of size, shape, and weight of the product. |
| CO5 | Use the Geometrical Dimensioning and Tolerancing rules and systems to select the desired fabrication method to produce the error free product. |
| CO6 | Design and developing a product in industries by applying the aesthetic and ergonomic considerations |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| COs | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 2 | 15 | - | - | - | - | 17 |
| CO2 | 2 | 3 | - | 12 | - | - | 17 |
| CO3 | 2 | 3 | - | 12 | - | - | 17 |
| CO4 | 2 | 3 | - | - | 12 | - | 17 |
| CO5 | 2 | 3 | - | - | - | 12 | 17 |
| CO6 | - | 3 | - | - | - | 12 | 15 |
|  |  |  |  |  |  |  | 100 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 14ME2045 / 17ME2039 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # RAPID PROTOTYPING AND TOOLING | # Max. Marks : | # 100 |

| Q. No. | Questions |  | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | Define: Rapid Prototyping. | CO1 / R |  | 1 |
| 2. | Identify the applications of Rapid Prototyping. | CO1 / U |  | 1 |
| 3. | Name the types of Liquid based Rapid Prototyping systems. | CO2 / A |  | 1 |
| 4. | Define: Fused Deposition Modeling. | CO2 / A |  | 1 |
| 5. | Name the types of Powder based Rapid Prototyping systems. | CO3 / A |  | 1 |
| 6. | Define: Micro sintering. | CO3 / R |  | 1 |
| 7. | Differentiate between Conventional Tooling and Rapid Tooling. | CO4 / A |  | 1 |
| 8. | Name the types of Reverse Engineering (RE) methodologies. | CO4 / A |  | 1 |
| 9. | Identify the applications of Rapid Prototyping in design. | CO5 / U |  | 1 |
| 10. | Name few applications of Rapid Prototyping in Jewellery Industry. | CO5 / A |  | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Classify: Rapid Prototyping. | CO1 / U | 3 |
| 12. | State the working principle of Rapid Freeze Prototyping. | CO2 / An | 3 |
| 13. | Discuss the advantages of Laser Engineered Net Shaping. | CO3 / A | 3 |
| 14. | Describe the working principle of Powder Metallurgy Tooling. | CO4 / An | 3 |
| 15. | List the applications of Rapid Prototyping in Analysis and Planning. | CO5 / A | 3 |
| 16. | Name the Medical and Bioengineering applications of Rapid Prototyping. | CO6 / A | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | a. | Describe the features of Rapid Prototyping process chain. | CO1 / A | 6 |  |
|  | b. | Explain Product development cycle with neat sketch. | CO1 / U | 6 |  |
| 18. | a. | Explain briefly the principle of Photo-polymerization. | CO2 / A | 4 |  |
|  | b. | Describe with neat sketch, the principle and applications of stereo lithography (SLA). | CO2 / U | 8 |  |
| 19. | a. | Elaborate the construction and working of solid creation system (SCS) with neat sketch. Mention its advantages, disadvantages and applications. | CO3 / An | 8 |  |
|  | b. | Highlight the applications of bio plotter. | CO3 / U | 4 |  |
| 20. | a. | Describe briefly the applications of micro fabrication technology. | CO4 / A | 4 |  |
|  | b. | Explain the construction and working of rapid freeze prototyping (RFP) with neat sketch. Mention its advantages, disadvantages and applications. | CO4 / An | 8 |  |
| 21. |  | Describe in detail the applications of Rapid Prototyping in Manufacturing and Tooling. | CO5 / A | 12 |  |
| 22. |  | Describe the construction and working of paper lamination technology (PLT) with neat sketch. Mention its advantages, disadvantages and applications. | CO2 / A | 12 |  |
| 23. |  | Explain in detail, Rapid tooling with two case studies. List out its applications. | CO4 / U | 12 |  |
|  |  | Compulsory: |  |  |  |
| 24. |  | Discuss in detail the various Rapid Prototyping techniques used in Aerospace and Automotive industries. | CO6 / An |  | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Recognize and use techniques for processing of CAD models for RP |
| CO2 | Outline the importance of RP Technology in product development cycle |
| CO3 | Know the principles and use RP Technology |
| CO4 | Select appropriate tooling for RP process |
| CO5 | Apply RP for reverse engineering |
| CO6 | Implement RP techniques & Manufacturing to solve real time industrial problems |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 1 | 10 | 5 | - | - | - | 16 |
| CO2 | - | - | 18 | 3 | - | - | 21 |
| CO3 | 1 | - | 4 | - | - | - | 5 |
| CO4 | - | 12 | 6 | 8 | - | - | 26 |
| CO5 | - | 1 | 16 | - | - | - | 17 |
| CO6 | - | - | 3 | 12 | - | - | 15 |
|  |  |  |  |  |  |  | 100 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18ME2012 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # Strength of Materials | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Define stress and strain. | CO1 / R | 1 |
| 2. | State Hook’s law | CO1 / R | 1 |
| 3. | Write the relationship between Bulk Modulus and Young’s Modulus. | CO2 / U | 1 |
| 4. | Define - Uniformly Distributed Load. | CO2 / R | 1 |
| 5. | Define - overhanging beam and draw a line diagram. | CO3 / R | 1 |
| 6. | Write the methods to find the slope and deflection at a section. | CO3 / U | 1 |
| 7. | Write an expression for deflection by moment area method. | CO4 / U | 1 |
| 8. | Define – Torsion and Torque. | CO4 / R | 1 |
| 9. | Define spring and write types of spring. | CO5 / R | 1 |
| 10. | Define solid length of helical spring | CO5 / U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Draw the stress strain diagram for mild steel and indicate and explain the salient points. | CO1 / U | 3 |
| 12. | Mention different types of beams and loads. | CO2 / A | 3 |
| 13. | Write types of beams and load acting on beams and explain. | CO3 / U | 3 |
| 14. | Derive the equation for the deflection when a point load applied at the free end of cantilever beam. | CO4 / A | 3 |
| 15. | The shearing stress of a solid shaft is not exceed 40 N/mm2 when the torque transmitted is 20000 N-m. Determine the minimum diameter of the shaft. | CO5 / A | 3 |
| 16. | A storage tank of internal diameter 280 mm is subjected to an internal pressure of 2.56 MPa. Find the thickness of the tank. If the hoop & longtudital stress are 75 MPa and 45 MPa respectively. | CO6 / A | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | A member ABCD is subjected to point loads P1, P2, P3 & P4 is shown in Figure 1. Calculated the force P2 necessary of equilibrium if P1 = 45 kN, P3 = 450 kN & P4 = 130 kN. Determine the total elongation of the member, assuming the modulus of elasticity to be 2.1 x 105 N/mm2. | CO1/ An | 12 |

| 18. | a. | A steel rod of 3 cm diameter and 5 m long is connected to two grips and the rod is maintained at a temperature of 95 °C. Determine the stress and pull exerted when the temperature falls to 30 °C, if i) the ends do not yield and ii) the ends yield by 0.12 cm. Take E = 2 x 105 MN/m2 and α = 12 x 10-6/°C. | CO2 / E | 12 |
| --- | --- | --- | --- | --- |
| 19. | a. | A simply supported beam of length 6 m, carries a point load of 3 kN and 6 kN at a distance of 2 m and 4 m from the left end. Draw the shear force and bending moment diagrams for the beam. | CO3 / E | 12 |
| 20. | a. | A cantilever of length 3 m is carrying a point load of 50 kN at a distance of 2 m from the fixed end. If I = 108 mm4 and E = 2x105 N/mm2, find (i) slope at the free end and (ii) deflection at the free end. | CO4/ An | 12 |
| 21. | a. | A cantilever of length 2.0 m carries a uniformly distributed load of 1 kN/m run over a length of 1.5 m from the free end. Draw the shear force and bending moment diagrams for the cantilever. | CO3/ An | 12 |
| 22. | a. | A cantilever of length 2 m carries a point load of 20 kN at the free end and another load of 20 kN at its center. If E = 105 N/mm2 and I = 108 mm4 for the cantilever then determine by moment area method, the slope and deflection of the cantilever at the free end. | CO4 / E | 12 |
| 23. | a. | A hollow shaft, having an internal diameter 40 % of its external diameter, transmits 562.5 kW power at 100 rpm. Determine the external diameter of the shaft if the shear stress not to exceed 60 N/mm2 and the twist in a length of 2.5 m should not exceed 1.3°. Assume maximum torque = 1.25 mean torque and modulus of rigidity = 9 x 104 N/mm2. | CO5/ An | 12 |
|  |  | Compulsory: |  |  |
| 24. | a. | A cylindrical thin drum 80 cm in diameter and 3 m long has a shell thickness of 1cm. If the drum is subjected to an internal pressure of 2.5 N/mm2, determine (i) change in diameter (ii) change in length and (iii) change in volume, E = 2×105 N/mm2 and poisons ratio = 0.25 | CO6 / E | 12 |

| COURSE OUTCOMES |  |
| --- | --- |
| CO1 | Recognize various types loads applied on machine components |
| CO2 | Understand the nature of internal stresses that will develop within the components |
| CO3 | Analyse the stresses acting simple geometry of structures |
| CO4 | Evaluate the strains and deformation due to the elastic stresses developed |
| CO5 | Compute inertia, slopes and deflection in beams |
| CO6 | Determine the torsional stresses of shaft and hoop stresses in cylinders |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 2 | 3 |  | 12 |  | - | 17 |
| CO2 | 1 | 1 | 3 |  | 12 |  | 17 |
| CO3 | 1 | 4 |  | 12 | 12 | - | 29 |
| CO4 | 1 | 1 | 3 | 12 | 12 |  | 29 |
| CO5 | 1 | 1 | 3 | 12 |  |  | 17 |
| CO6 |  |  | 3 |  | 12 |  | 15 |
|  |  |  |  |  |  |  | 124 |

Reg.No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18ME2028 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | HYDRAULICS AND PNEUMATICS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | List the advantages of fluid power system. | CO1/ R | 1 |
| 2. | Define the term fluid power. | CO1/ R | 1 |
| 3. | Name any three positive displacement pumps. | CO2/ U | 1 |
| 4. | Classify hydraulic fluids. | CO2/ R | 1 |
| 5. | State the function of flow control valve. | CO3/ U | 1 |
| 6. | Draw the symbol for pressure relief valve. | CO3/ A | 1 |
| 7. | Draw the ISO symbol for hydraulic Actuator. | CO4/ A | 1 |
| 8. | Write the composition of air. | CO5/ R | 1 |
| 9. | How will you represent AND gate symbol and its truth table? | CO5/ R | 1 |
| 10. | An interconnecting structure of the microcomputer is called -----------. | CO6/ A | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | List the applications of fluid power. | CO1 / R | 3 |
| 12. | Compare hydraulic and pneumatic system. | CO2 / R | 3 |
| 13. | Draw FRL unit symbol and state its functions. | CO3 / A | 3 |
| 14. | Draw the symbolic layout of a typical hydraulic power pack. | CO4 / A | 3 |
| 15. | Classify various types of accumulators. | CO5 / R | 3 |
| 16. | Draw the pneumatic circuit for OR logic. | CO6 / A | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | Identify the essential components of Hydraulic system with neat diagram. | CO1 / U | 12 |
| 18. | a. | Sketch and discuss the working principle for the following hydraulic pumps : (a) Internal gear pump (b) Vane pump. | CO2 / U | 12 |
| 19. | a. | Apprise the function of intensifier and discuss with a hydraulic circuit. | CO3 / U | 12 |
| 20. | a. | Draw two types of accumulator circuits and discuss the working. | CO4 / U | 12 |
| 21. | a. | Build the Synchronizing hydraulic circuits with - Matching Pumps - Flow Control Valves. | CO5 / U | 12 |

| 22. | a. | How solenoid will work, explain with neat diagram. | CO5 / U | 6 |
| --- | --- | --- | --- | --- |
|  | b. | Analyze the working of relay with a practical example. | CO5 / U | 6 |
| 23. | a. | Design and develop a electro pneumatic circuit using limit switch for double acting cylinder. | CO6 / A | 12 |
|  |  | Compulsory: |  |  |
| 24. | a. | Discuss briefly about various fluidic logics. | CO6 / A | 6 |
|  | b. | Discuss the architecture of PLC. | CO6 / A | 6 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18ME2031 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # KINEMATICS AND DYNAMICS OF MACHINERY | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | A combination of kinematic pairs joined in such a way that the relative motion between the links is completely constrained, is called a ____________. | CO1 / R | 1 |
| 2. | The mechanism forms a structure when the number of degrees of freedom (n) is equal to ___________. | CO1 / U | 1 |
| 3. | The instantaneous centres which vary with the configuration of the mechanism, are called ____________. | CO2 / A | 1 |
| 4. | The total number of instantaneous centres for a mechanism consisting of n links are ___________. | CO2 / R | 1 |
| 5. | In an engine, the work done by inertia forces in a cycle is____________ | CO3 / U | 1 |
| 6. | The direction of linear velocity of any point on a link with respect to another point on the same link is ______________. | CO3 / A | 1 |
| 7. | The size of a cam depends upon _____________. | CO4 / An | 1 |
| 8. | Offset is provided to a cam follower mechanism to _____________. | CO 5 / E | 1 |
| 9. | An involute profile is preferred to cycloidal. Why? | CO5 / A | 1 |
| 10. | When the axes of the first and last gear are co-axial, then the gear train is known as _______. | CO6 / R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Write the application of Kutzbach Criterion to Plane Mechanisms. | CO1 / R | 3 |
| 12. | Define velocity and acceleration. | CO2 / U | 3 |
| 13. | Write short notes on Four bar mechanism. | CO3 / U | 3 |
| 14. | Describe the importance of dynamic balancing. | CO4 / R | 3 |
| 15. | Write the classifications of cams. | CO5 / E | 3 |
| 16. | Write short notes on rack and pinion. | CO6 / A | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. |  | The figure shows the layout of a quick return mechanism of the oscillating link type, for a special purpose machine. The driving crank BC is 30 mm long and the time ratio of the working stroke to the return stroke is to be 1.7. If the length of the working stroke of R is 120 mm, determine the dimensions of AC and AP. | CO1 / A | 12 |
| 18. |  | The dimensions of the mechanism, as shown in Figure, are as follows : AB = 0.45 m; BD = 1.5 m : BC = CE = 0.9 m, The crank A B turns uniformly at 180 r.p.m. in the clockwise direction and the blocks at D and E are working in frictionless guides. Draw the velocity diagram for the mechanism and find the velocities of the sliders D and E in their guides. Also determine the turning moment at A if a force of 500 N acts on D in the direction of arrow X and a force of 750 N acts on E in the direction of arrow Y. | CO2 / E | 12 |
| 19. |  | The crank and connecting rod of a theoretical steam engine are 0.5 m and 2 m long respectively. The crank makes 180 r.p.m. in the clockwise direction. When it has turned 45° from the inner dead centre position, determine: 1. velocity of the piston, 2. angular velocity of connecting rod, 3. velocity of point E on the connecting rod 1.5 m from the gudgeon pin, 4. velocities of rubbing at the pins of the crankshaft, crank and crosshead when the diameters of their pins are 50 mm, 60 mm and 30 mm respectively, 5. position and linear velocity of any point G on the connecting rod which has the least velocity relative to crankshaft? | CO3 / An | 12 |
| 20. |  | Four masses m1, m2, m3and m4 are 200 kg, 300 kg, 240 kg and 260 kg respectively. The corresponding radii of rotation are 0.2 m, 0.15 m, 0.25 m and 0.3 m respectively and the angles between successive masses are 45°, 75° and 135°. Find the position and magnitude of the balance mass required, if its radius of rotation is 0.2 m. | CO3 / E | 12 |

| 21. |  | A cam is to give the following motion to a knife-edged follower : 1. Outstroke during 60° of cam rotation ; 2. Dwell for the next 30° of cam rotation ; 3. Return stroke during the next 60° of cam rotation, and 4. Dwell for the remaining 210° of camrotation. The stroke of the follower is 40 mm and the minimum radius of the cam is 50 mm. The follower moves with uniform velocity during both the outstroke and return strokes. Draw the profile of the cam when the axis of the follower is offset by 20 mm from the axis of the camshaft. | CO4 / An |  | 12 |
| --- | --- | --- | --- | --- | --- |
| 22. |  | Explain with a neat sketch, the Spur gear terminology. | CO5 / An |  | 12 |
| 23. |  | An epicyclic gear consists of three gears A, B and C as shown in Figure. The gear A has 72 internal teeth and gear C has 32 external teeth. Gear B meshes with both A and C and is carried on an arm EF which rotates about the centre of A at 18 r.p.m. If gear A is fixed, determine the speed of gears B and C. | CO6 / U |  | 12 |
|  |  | Compulsory: |  |  |  |
| 24. |  | A cam drives a flat reciprocating follower in the following manner : During the first 120° rotation of the cam, the follower moves outwards through a distance of 20 mm with simple harmonic motion. The follower dwells during the next 30° of cam rotation. During the next 120° of cam rotation, the follower moves inwards with simple harmonic motion. The follower dwells for the next 90° of cam rotation. The minimum radius of the cam is 25 mm. Draw the profile of the cam | CO5 / E | 12 |  |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the fundamental concepts of kinematic links, kinematic pairs and kinematic chains |
| CO2 | Apply Gashoff’s law for four bar, slider crank and common mechanisms |
| CO3 | Calculate displacement, velocity and acceleration in simple mechanisms. |
| CO4 | Analysis of planar mechanisms, force , moment and balancing of kinematic pairs |
| CO5 | Design and calculate the velocity and motion of cams |
| CO6 | Understand the fundamental concepts of various transmission devices |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 4 | 1 | 12 | - | - | - | 17 |
| CO2 | 1 | 3 | 1 | - | 12 | - | 17 |
| CO3 | - | 4 | 1 | 12 | 12 | - | 29 |
| CO4 | 3 | - | - | 13 | - | - | 16 |
| CO5 | - | - | 1 | 12 | 16 | - | 29 |
| CO6 | 1 | 12 | 3 | - | - | - | 16 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18ME2034 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name: | # OPERATIONS RESEARCH | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | State the limitation of graphical method in solving LPP. | CO 1 / U | 1 |
| 2. | All simplex problem should be solved in the manner of __________. | CO 1 / U | 1 |
| 3. | Name the three methods to find the initial solution for transportation problem. | CO2 / R | 1 |
| 4. | How to balance a transportation problem when it is not in balanced state? | CO2 / U | 1 |
| 5. | List any two applications of assignment problem. | CO6 / R | 1 |
| 6. | How to perform column wise reduction in assignment problem? | CO6 / U | 1 |
| 7. | What is dummy activity in a network? | CO3 / R | 1 |
| 8. | Why loop formation should not occur in a network? | CO3 / U | 1 |
| 9. | Define shortage cost. | CO4 / R | 1 |
| 10. | List the items that comes under inventory. | CO 4 / U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Differentiate slack, surplus and artificial variables. | CO1 / U | 3 |
| 12. | What is the degeneracy in transportation problem? | CO2 / U | 3 |
| 13. | Distinguish transportation problem and assignment problem. | CO2 / U | 3 |
| 14. | Normal duration and normal cost crash cost and crash duration of the activities 1-2 and 1-3 are given below. Interpret the cost slope (Delta cost) and Delta time (Time available for crashing) of these activities. Activities; Normal Cost (Rupees); Normal Duration (Weeks); Crash Cost (Rupees); Crash Duration (Weeks). 1-2; 7,000; 6; 14,500; 3. 1-3; 4,000; 8; 8,500; 5. | CO3 / A | 3 |
| 15. | In a mechanical workshop, it takes 10 days to get the stock of coolant oil after placing an order. The daily requirement of coolant oil in the workshop is 50 litres. Based on the past experience it is determined that the safety is 5 days stock. Infer the re-order point. | CO4 / A | 3 |
| 16. | Differentiate series service channel and parallel service channel. | CO5 / U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | Solve the following Linear Programming problem by graphical method Minimize Z=30X1+20X2 : Subjected to 5X1 + X2 ≥10 2X1 + 2X2 ≥12 X1 + 4X2 ≥12 X1, X2 ≥ 0 | CO1 / A | 12 |
| 18. | a. | A food manufacturing company has two plants T1 and T2 .Three fruit cultivators are willing to supply fruits in the following quantities. Cultivator C1 is willing to supply 190 Quintals at Rs 100/Quintal, Cultivator C2 is willing to supply 290 Quintals at Rs 90/Quintal, Cultivator C3 is willing to supply 390 Quintals at Rs 80/Quintal. Cost of transportation (Rs) from Cultivators for plants is given below. T1; T2. C1; 20; 26. C2; 9; 16. C3; 49; 31. Plant capacities and labour costs are shown in the following table Details; Plant T1; Plant T2. Capacity in Quintal; 440; 360. Labour cost in Rs / Quintal; 30; 22. The processed fruits are sold at Rs 480/Quintal to the distributors. The objective of the problem is to maximize profit, formulate the problem and find the initial solution by North-West corner method. | CO4 / An | 12 |
| 19. | a. | In a small machine shop, there are 4 operators available to assign the jobs for the day. There are 5 jobs and the profit in rupees for each operator on each job is represented in the table. Job 1; Job 2; Job 3; Job 4; Job 5. Operator 1; 6.30; 5.90; 6.80; 10.20; 7.40. Operator 2; 8.00; 10.10; 9.00; 8.50; 7.30. Operator 3; 8.70; 8.80; 9.10; 7.60; 6.50. Operator 4; 7.30; 8.30; 6.40; 7.70; 8.00. Evaluate the optimal assignment and also find the job which is to be rejected. | CO6 / An | 12 |

| 20. | a. | The various time estimates (Days) of activities involved in a project are given below: Activities; 1-2; 1-3; 2-4; 2-3; 3-4; 3-5; 4-6; 5-6. Optimistic Time; 2; 4; 2; 2; 0; 3; 6; 1. Normal Time; 6; 8; 3; 4; 0; 6; 10; 3. Pessimistic Time; 10; 12; 4; 6; 0; 9; 14; 5. i) Determine the expected completion time of the project with network diagram - (2) ii) Assess the variance and SD of project (4) iii) Appraise the probability of completing the project within 23 days and 25 days (4) iv) Assess the probability NOT COMPLETING that the project within the due date 25 days (2) | CO3 / A | 12 |
| --- | --- | --- | --- | --- |
| 21. | a. | Items; 1; 2; 3; 4; 5. Annual Usage (units); 200; 3000; 25; 1100; 60. Unit Cost (Rs); 11; 14; 9; 6; 5. Items; 6; 7; 8; 9; 10. Annual Usage (units); 250; 140; 850; 550; 80. Unit Cost (Rs); 90; 6; 6; 15; 9. The above tables show the annual usage and unit cost of 10 items in a store. Grade the value of these 10 items using ABC analysis. | CO6 / A | 12 |
| 22. | a. | There are 6 jobs each of which must go through the two machines A and B in the order A-B, Processing time in hours are given in the table. Determine the optimal sequence of jobs and idle time of each machine Jobs; 1; 2; 3; 4; 5; 6. Processing Time of Machine A (Hours); 3; 6; 4; 7; 5; 8. Processing Time of Machine B (Hours); 2; 5; 6; 3; 2; 8. | CO2/ An | 12 |

| 23. | a. | Solve the following LPP by simplex method Maximize Z =X1 + 3X2 Subjected to the constraints X1 ≤ 5 X1 + 2X2≤ 10 X2≤ 4 X1 and X2 ≥ 0 | CO2 / A | 12 |
| --- | --- | --- | --- | --- |
|  |  | Compulsory: |  |  |
| 24. | a. | Arrivals at a public telephone booth with an average of 8 minutes between one arrival and the next. The length of the telephone calls is assumed with a mean value of 2 minutes, Appraise the following: i) Utilization factor (4) ii) Average queue length (4) iii) Assuming waiting time as 5 minutes, find the arrival rate(4) | CO 5/ A | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Correlate this subject knowledge with engineering problems |
| CO2 | Construct flexible appropriate mathematical model to represent physical problem |
| CO3 | Schedule the engineering projects by using network analysis |
| CO4 | Analyze the transportation problem and optimize the resources and output |
| CO5 | Apply knowledge in solving their engineering queuing problem |
| CO6 | Develop their skills in decision making analysis by allocating resources |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 |  | 5 | 12 |  |  |  | 17 |
| CO2 | 1 | 7 | 12 | 12 |  |  | 32 |
| CO3 | 1 | 1 | 15 |  |  |  | 17 |
| CO4 | 1 | 1 | 3 | 12 |  |  | 17 |
| CO5 |  | 3 | 12 |  |  |  | 15 |
| CO6 | 1 | 1 | 12 | 12 |  |  | 26 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18ME2040 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # COMPUTATIONAL FLUID DYNAMICS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | The source term in the momentum equation is ___________. | CO1 | R | 1 |
| 2. | What is the physical meaning of divergence of velocity? | CO1 | U | 1 |
| 3. | Write the expression representing the parabolic equation. | CO2 | A | 1 |
| 4. | Define ‘Round-off error’. | CO2 | R | 1 |
| 5. | Write the significance of ‘Gauss divergence theorem’. | CO3 | An | 1 |
| 6. | Write the equations represent 2-D steady state diffusion. | CO3 | A | 1 |
| 7. | What does ‘QUICK’ stand for? | CO4 | R | 1 |
| 8. | The upwind scheme is suitable for ___________ term. | CO4 | R | 1 |
| 9. | What is the advantage of the staggered grid arrangement? | CO5 | U | 1 |
| 10. | The equation for pressure of the SIMPLER algorithm is obtained using ___________. | CO5 | A | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | What are the different categories of boundary conditions? Give example for each category? | CO1 | U | 3 |
| 12. | What are the different types of grids used in CFD? | CO2 | U | 3 |
| 13. | Derive the simple expression for 1D steady diffusion problem? | CO3 | A | 3 |
| 14. | Discuss the need of upwind type discretization. | CO4 | An | 3 |
| 15. | What is the difference between the SIMPLE and the SIMPLER algorithms? | CO5 | U | 3 |
| 16. | Write different types of turbulence models? | CO6 | A | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. |  | Derive the three dimensional mass conservation equation for a compressible fluids. | CO1 | U | 12 |
| 18. |  | Write a short notes on structured and unstructured grids. Also explain the advantages and applications of structured grids over unstructured grids. | CO2 | A | 12 |

| 19. |  | Consider the problem of source free heat conduction in an insulated rod whose ends are maintained at constant temperature 200 oC and 500 oC respectively as shown in figure. Formulate the algebraic equations for one dimensional steady state temperature distribution of the rod. Thermal conductivity k = 1000 W/m-K and the cross sectional area A is 10 x 10-3m2. Take δx=0.1m. | CO3 | An | 12 |
| --- | --- | --- | --- | --- | --- |
| 20. |  | Derive the discretized equation for a 1D convection diffusion equation for interior nodes using central differencing scheme with five interior nodes. | CO4 | A | 12 |
| 21. |  | Derive the equations for convective flux per unit mass F and the diffusive conductance at u- control volume faces with the application of staggered grid. | CO5 | A | 12 |
| 22. |  | Explain in detail the physical implementation of inlet, outlet and wall boundary conditions with necessary sketch. | CO1 | A | 12 |
| 23. |  | Derive the discretized equation for a 1D steady diffusion equation. | CO3 | A | 12 |
|  |  | Compulsory: |  |  |  |
| 24. |  | Discuss the various governing equations for turbulent kinetic energy (k) and turbulent dissipation (ɛ). Also explain various terms related to it. | CO6 | An | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Formulate the required governing equations for flow and heat transfer problems. |
| CO2 | Discretize the governing equations of flow and heat transfer problems. |
| CO3 | Solve the diffusion equations |
| CO4 | Solve the diffusion-convection equations |
| CO5 | Use appropriate algorithms to solve the discretized equations. |
| CO6 | Apply turbulence models to accurately predict the variables based on the flow characteristics. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 1 | 16 | 12 |  |  |  | 29 |
| CO2 | 1 | 3 | 13 |  |  |  | 17 |
| CO3 |  |  | 16 | 13 |  |  | 29 |
| CO4 | 2 |  | 12 | 3 |  |  | 17 |
| CO5 |  | 4 | 13 |  |  |  | 17 |
| CO6 |  |  | 3 | 12 | - | - | 15 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18ME2044 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # REFRIGERATION AND AIR-CONDITIONING | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | Define the term refrigerating effect. | CO1 | Remember | 1 |
| 2. | Write the expression for COP of a refrigerator. | CO1 | Remember | 1 |
| 3. | List any two factors to be considered for the selection of refrigerants. | CO2 | Understand | 1 |
| 4. | Define volumetric efficiency of a compressor. | CO2 | Remember | 1 |
| 5. | Mention the name of a paramagnetic salt. | CO3 | Remember | 1 |
| 6. | Write any two properties of an ideal refrigerant. | CO3 | Understand | 1 |
| 7. | State Dalton’s law of partial pressure. | CO4 | Remember | 1 |
| 8. | Differentiate between absolute and relative humidity. | CO4 | Understand | 1 |
| 9. | Define effective temperature. | CO5 | Remember | 1 |
| 10. | Define sensible heat factor. | CO5 | Remember | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Can you use the same plant as a heat pump in winter and as a refrigerator in summer? Explain. | CO1 | Understand | 3 |
| 12. | List the advantages and disadvantages of water-cooled condensers over air cooled condensers. | CO2 | Understand | 3 |
| 13. | Explain the working of cascade refrigeration system. | CO3 | Remember | 3 |
| 14. | Explain the process of heating and humidification. | CO4 | Remember | 3 |
| 15. | List the important considerations in the design of air-conditioning system. | CO5 | Remember | 3 |
| 16. | Describe the major difficulties experienced in designing an air-conditioning system for ships. | CO6 | Understand | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | a. | Describe the working principle of an open cycle air refrigeration system with a sketch. | CO1 | Understand | 6 |
|  | b. | In an air refrigeration system working on Bell Coleman cycle, air is compressed to 6 bar from 1 bar. Its initial temperature is 13°C. After compression, the air is cooled up to 27°C in a cooler before expanding back to a pressure of 1 bar. Determine the theoretical COP of the system. Take isentropic index for compression and expansion is 1.4. | CO1 | Apply | 6 |

| 18. | a. | Describe the type of refrigerant condensers with sketches. | CO2 | Apply | 6 |
| --- | --- | --- | --- | --- | --- |
|  | b. | # Explain the working principle of aqua ammonia vapour absorption refrigeration system with a sketch. | CO2 | Apply | 6 |
| 19. | a. | Describe the Linde-Hampson system for liquefaction of air. | CO3 | Remember | 6 |
|  | b. | Describe the production of low temperature by adiabatic demagnetization of a paramagnetic salt. | CO3 | Understand | 6 |
| 20. | a. | The atmospheric air at 30°C dry bulb temperature and 75% relative humidity enters a cooling coil at the rate of 200 m³/min. The coil dew point temperature is 14°C and the by-pass factor of the coil is 0.1. Determine the temperature of air leaving the cooling coil, the capacity of the cooling coil, the amount of water vapour removed, and the sensible heat factor for the process. | CO4 | Apply | 6 |
|  | b. | Discuss the factors that affect human comfort. | CO4 | Understand | 6 |
| 21. | a. | A retail shop located in a city has the following data. Outside design condition = 40°C DBT and 27°C WBT Inside conditions = 25°C DBT and 50% RH Volume of air supplied = 70 m³/min Sensible heat load in room = 58.15 kW Latent heat load in room = 14.54 kW; BPF of the coil is 0.15 Find the effective sensible heat factor for the plant. | CO5 | Analyze | 8 |
|  | b. | State the sources of heat load from internal and external sources. | CO5 | Understand | 4 |
| 22. | a. | Describe the adiabatic saturation process in detail. | CO4 | Apply | 6 |
|  | b. | Describe the working principle of multistage vapour compression refrigeration system with neat sketch. | CO1 | Understand | 6 |
| 23. | a. | The sling psychrometer reads 40ºC DBT and 28ºC WBT. Calculate specific humidity, relative humidity, dew point temperature and enthalpy of the mixture per kg of air, by using empirical relations. Assume the pressure to be 1.03 bar | CO4 | Apply | 6 |
|  | b. | Explain the working principle of lithium bromide vapour absorption refrigeration system with neat sketch. | CO2 | Understand | 6 |
|  |  | Compulsory: |  |  |  |
| 24. | a. | Describe the different methods of air-conditioning systems used on rail road cars. | CO6 | Understand | 6 |
|  | b. | Explain how the refrigeration controls the spoilage of foods. | CO6 | Understand | 6 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand various refrigeration systems. |
| CO2 | Demonstrate the working of refrigeration equipment. |
| CO3 | Understand various psychrometric processes |
| CO4 | Estimate the space cooling load. |
| CO5 | Design the air-conditioning equipment |
| CO6 | Select suitable refrigeration and air-conditioning systems for various applications |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 2 | 15 | 6 | - | - | - | 23 |
| CO2 | 1 | 10 | 12 | - | - | - | 23 |
| CO3 | 10 | 7 | - | - | - | - | 17 |
| CO4 | 4 | 7 | 18 | - | - | - | 29 |
| CO5 | 5 | 4 | - | 8 | - | - | 17 |
| CO6 | - | 15 | - | - | - | - | 15 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18ME2047 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # POWER PLANT ENGINEERING | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | The regenerative cycle efficiency of thermal power plant is equal to ________. | CO1 / R | 1 |
| 2. | Rankine cycle efficiency of a good steam power plant may be in the range of __________. | CO1 / R | 1 |
| 3. | List the application of Economiser. | CO2 / U | 1 |
| 4. | The draught which a chimney produces is called __________. | CO2 / U | 1 |
| 5. | A moderator __________ the neutrons. | CO3 / U | 1 |
| 6. | Name the coolant used in a boiling water reactor. | CO3 / A | 1 |
| 7. | Identifythe scenario in which the diesel plants are mainly used. | CO4 / A | 1 |
| 8. | A gas turbine works on which cycle? | CO4 / R | 1 |
| 9. | In which collector the efficiency is maximum? | CO5 / U | 1 |
| 10. | How moderate active solid wastes are disposed? | CO6 / R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Draw the T-S diagram for the Reheating cycle. | CO1 / A | 3 |
| 12. | Elucidate supercritical boilers. | CO2 / R | 3 |
| 13. | Distinguish the Boiling Water Reactor (BWR) and Pressurized Water Reactor (PWR). | CO3 / A | 3 |
| 14. | Name four important components in the Gas turbine power plant. | CO4 / R | 3 |
| 15. | Classify the hydroelectric power plant. | CO5 / U | 3 |
| 16. | Define load curve. | CO6 / U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. |  | Explain with T-s plots combined and binary vapour power cycles. | CO1 / An | 12 |
| 18. | a. | How are cooling towers classified? Explain any one type with a neat sketch. | CO2 / U | 8 |
|  | b. | Differentiate between accessories and mountings of a boiler. Give examples for each. | CO2 / A | 4 |

| 19. | a. | How electricity is generated using nuclear fission? Illustrate the construction of a nuclear power plant and explain its working. | CO3 / A | 10 |
| --- | --- | --- | --- | --- |
|  | b. | Define cladding. | CO3 / R | 2 |
| 20. |  | In a Brayton cycle based power plant, the air at the inlet is at 27C, 0.1 MPa. The pressure ratio is 6.25 and the maximum temperature is 800C. Find (a) the compressor work per kg of air (b) the turbine work per kg of air (c) the heat supplied per kg of air, and (d) the cycle efficiency. Take =1.4 and Cp,air=1.005 kJ/kg.K. | CO4 / E | 12 |
| 21. | a. | Illustrate with a neat sketch the working of a hydroelectric power plant. | CO5 / U | 6 |
|  | b. | Explain the operation of a solar power plant with a neat layout sketch. | CO5 / U | 6 |
| 22. | a. | What are the modifications made in the Rankine cycle? | CO1 / R | 4 |
|  | b. | Explain in detail the advantages and disadvantages of modifications made in the Rankine cycle. | CO1 / A | 8 |
| 23. | a. | Classify the material usage for nuclear power plant reactors. What materials would you select for the various components of a nuclear reactor? Attribute reasons for your selection. | CO3 / C | 12 |
|  |  | Compulsory: |  |  |
| 24. |  | The loads on a power plant concerning time for 24 hours are tabled below. Time, Hrs; 0-6; 6 - 12; 12 - 14; 14 - 18; 18 - 24. Load, MW; 30; 90; 60; 100; 50. i) Draw the load curve ii) Draw the load duration curve iii) Calculate load factor iv) Calculate plant capacity factor. | CO6 / E | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Determine the efficiency of a modified Rankine power cycle |
| CO2 | Explain the layout, construction and working of steam power plant |
| CO3 | Recognize various methods of power generation from nuclear power plant |
| CO4 | Analyze the performance of diesel engine cycle and gas turbine cycles |
| CO5 | Recognize various methods of power generation from renewable energy sources |
| CO6 | Understand the power plant economics and environmental hazards |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 6 | - | 11 | 12 | - | - | 29 |
| CO2 | 3 | 10 | 4 | - | - | - | 17 |
| CO3 | 2 | 1 | 14 | - | - | 12 | 29 |
| CO4 | 4 | - | 1 | - | 12 | - | 17 |
| CO5 | - | 16 | - | - | - | - | 16 |
| CO6 | 1 | 3 | - | - | 12 | - | 16 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18ME2050 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # FINITE ELEMENT ANALYSIS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | The higher order elements are also called as__________. | CO1 / R | 1 |
| 2. | On gathering stiffness and loads ,the system of equations is given by _______ | CO2 / R | 1 |
| 3. | Finite element analysis deals with __________. | CO2 / U | 1 |
| 4. | For 1-D bar elements if the structure is having 3 nodes then the stiffness matrix formed is having order of__________. | CO 3 / R | 1 |
| 5. | __________are used to find the nodal displacements in all parts of element | CO 3 / U | 1 |
| 6. | At Fixed support The displacements are equal to __________. | CO 4 / U | 1 |
| 7. | The point in the entire structure is defined using coordinates system is known as __________. | CO4 / R | 1 |
| 8. | The displacement function for 1-D ,two node linear element in terms of shape function will be ___________. | CO5 / R | 1 |
| 9. | For two dimensional plane stress problems, normal and shear stress are__________ | CO5 / U | 1 |
| 10. | The nature of loading at various locations and other surfaces conditions called __________. | CO 5 / A | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Distinguish Natural & Essential boundary condition. | CO1 / R | 3 |
| 12. | Define shape function. State its characteristics. | CO2 / U | 3 |
| 13. | What is Plane stress and Plain strain condition? | CO 3 / R | 3 |
| 14. | Write the strain displacement matrix of CST element. | CO4 / R | 3 |
| 15. | What is a Jacobian matrix of transformation? | CO5 / R | 3 |
| 16. | Write the governing differential equation for two dimensional heat transfer. | CO 6 / R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. |  | A beam AB of span ‘L’ simply supported at ends and carrying a concentrated load W at the centre ‘C’ as shown in figure. Determine the deflection at mid span by using Rayleigh – Ritz method. | CO1 / A | 12 |

| 18. |  | A thin steel plate of uniform thickness 25 mm is subjected to a point load of 420 N at mid depth as shown in figure. The plate is also subjected to self weight. If young’s modulus, E = 2 X 105 N/mm2 and density, ρ = 0.8 X 10-4 N/mm3, calculate the following. - Displacement at each nodal point. - Stresses in each element. | CO 3 / E | 12 |
| --- | --- | --- | --- | --- |
| 19. |  | A stepped bar is subjected to an axial load of 200kN at the place of change of cross section and material as in the figure (a) Nodal displacement (b) Induced stresses in each material. | CO4 / An | 12 |
| 20. |  | Consider a three bar truss as shown in figure. It is given that E = 2 X 105 N/mm2. Calculate the following. - Nodal displacements. - Stress in each member. Take A1 = 2000 mm2, A2 = 2500 mm2 and A3 = 2500 mm2 | CO4 / A | 12 |
| 21. |  | Derive the shape function for Eight Noded Rectangular Element. | CO5 / An | 12 |
| 22. | a. | Write in detail about the application of FEA in different fields. | CO1 / R | 2 |
|  | b. | Describe in detail about the step by step procedure in solving the FEA problems. | CO2 / U | 10 |

| 23. |  | The following differential equation is available for a physical phenomenon. d2ydx2+ 500 x2=0, 0 ≤x ≤1 Trial function is y = a1 (x – x4) Boundary conditions are y (0) = 0 y(1) = 0 Find the value of parameter a1 by the following methods - Point Collocation - Sub domain Collocation - Least Squares - Galerkin method. |  | CO2 / A | 12 |
| --- | --- | --- | --- | --- | --- |
|  |  | Compulsory: |  |  |  |
| 24. |  | A composite slab consists of three materials with thermal conductivities of 20 W/moK, 30 W/moK, 50 W/moK and thicknesses 0.3 m, 0.15 m and 0.15 m respectively. The outer surface is at 20oC and the inner surface is exposed to the convective heat transfer coefficient of 25 W/moK and a medium at 800oC.Determine the temperature distribution within the wall. | CO6 / An |  | 12 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18ME2055 |  | # Duration : | # 3hrs |
| --- | --- | --- | --- | --- |
| # Sub. Name : | # COMPUTER AIDED DESIGN |  | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | PDES means __________. | CO1 | R | 1 |
| 2. | IGES stands for ___________. | CO1 | R | 1 |
| 3. | On raster system, lines are plotted with ___________. | CO2 | U | 1 |
| 4. | Expansion of line DDA algorithm is __________. | CO3 | R | 1 |
| 5. | _________ is the combined information like connectivity, associativity and neighborhood information. | CO3 | U | 1 |
| 6. | _________ provide designers with greater flexibility and control of a curve shape by changing the position of the control points. | CO3 | R | 1 |
| 7. | DVST stands for ___________. | CO4 | R | 1 |
| 8. | Raster display maximum pixel resolution is ___________. | CO4 | A | 1 |
| 9. | Natural coordinate system lies between ________ and _________. | CO5 | U | 1 |
| 10. | __________ no of nodes are available with tetrahedron element. | CO6 | U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Draw the architecture of a CAD system. | CO1 | U | 3 |
| 12. | Write short notes on Translation, Rotation, Scaling. | CO2 | U | 3 |
| 13. | Write the brief note on the parametric representation of a Bezier curve. | CO3 | U | 3 |
| 14. | Discuss about the random scanning techniques. | CO4 | U | 3 |
| 15. | Define Discretization? What are the elements used for the discretization. | CO5 | R | 3 |
| 16. | Discuss briefly about the Johnson method of optimization. | CO6 | U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | a. | Explain the needs of the CAD system and write merits and demerits. | CO1 | U | 4 |
|  | b. | Discuss the various CAD standards with suitable example. | CO2 | R | 8 |
| 18. | a. | Discuss in details about the Hidden line removal algorithm. | CO3 | U | 6 |
|  | b. | Elaborate the hidden surface removal algorithm with neat sketch. | CO4 | R | 6 |

| 19. | a. | What is wire frame modeling? How the wire frame modeling are classified? List advantages, limitations and applications. | CO 3 | R | 8 |
| --- | --- | --- | --- | --- | --- |
|  | b. | Write briefly about the CSG and B-Rep. | CO3 | U | 4 |
| 20. |  | With neat sketches explain about the various graphics display in detail. | CO4 | U | `12 |
| 21. | a. | Explain the steps involved in solving FEM problems | CO5 | E | 4 |
|  | b. | Calculate the nodal displacements and forces for the bar loaded as shown in fig.1 Fig.1 | CO5 | R | 8 |
| 22. |  | Explain in detail the Bezier curve with suitable sketches and examples. | CO3 | U | 12 |
| 23. | a. | Derive the shape function for the global coordinate system. | CO6 | A | 10 |
|  | b. | What do you mean by boundary conditions? | CO6 | U | 2 |
|  |  | Compulsory: |  |  |  |
| 24. | a. | Explain about various RPT processes with neat sketch. | CO6 | U | 8 |
|  | b. | What is STL? Write its significance. | CO6 | R | 4 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Demonstrate the basic structure and components of CAD. |
| CO2 | Outline the process of representing graphical entities in a CAD environment. |
| CO3 | Construct the geometric model using different techniques to represent a product. |
| CO4 | Illustrate various techniques and devices involved in CAD hardware. |
| CO5 | Analyze the models for design solutions using FEM. |
| CO6 | Discuss the various computer aided tools implemented in various industrial applications. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 2 | 7 |  |  |  |  | 9 |
| CO2 | 8 | 4 |  |  |  |  | 12 |
| CO3 | 10 | 26 |  |  |  |  | 36 |
| CO4 | 15 | 15 | 1 |  |  |  | 31 |
| CO5 | 3 | 1 |  |  | 4 |  | 8 |
| CO6 | 4 | 14 | 10 |  |  |  | 28 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18ME2057 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # WELDING TECHNOLOGY | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Multi-spot welding process is __________. | CO1 / U | 1 |
| 2. | Thermit welding is a form of __________. | CO1 / U | 1 |
| 3. | Seam-welding is __________. | CO1 / R | 1 |
| 4. | The melting point of the filler metal in brazing should be above __________. | CO2 / U | 1 |
| 5. | In resistance welding the electrode material is made of __________. | CO2 / R | 1 |
| 6. | A process in which arc is maintained under a blanket of flux is called ________. | CO3 / U | 1 |
| 7. | The most commonly used flame in gas welding is __________. | CO5 / U | 1 |
| 8. | Distortion in welding occurs due to __________. | CO4 / U | 1 |
| 9. | __________ joints are the strongest for brazing operation __________. | CO5 / U | 1 |
| 10. | Arc length in arc welding should be equal to __________. | CO6 / U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Briefly explain the different types of welding Process. | CO1 / R | 3 |
| 12. | Write down the difference between the Seam and Projection Welding. | CO3 / U | 3 |
| 13. | Briefly discuss about the Explosive welding Process. | CO3 / U | 3 |
| 14. | Discuss about the underwater welding. | CO4 / A | 3 |
| 15. | Briefly explain about the different welding joints design. | CO5 / A | 3 |
| 16. | Write down the welding defects. | CO6 / R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | Define Welding. | CO1 / R | 2 |
|  | b. | Explain the principles of welding operation. | CO1 / R | 5 |
|  | c. | Write down the advantages and disadvantages of welding. | CO1 / U | 5 |
| 18. | a. | Briefly explain about spot welding. | CO3 / R | 2 |
|  | b. | Explain fusion welding process and its classification. | CO3 / U | 4 |
|  | c. | Discuss the influence of Current and Voltage on Welding. | CO3 / An | 6 |
| 19. | a. | Explain the necessity of Non-conventional Welding process. | CO4 / U | 4 |
|  | b. | Discuss Explosive welding and its application. | CO4 / A | 8 |
| 20. | a. | Define Friction welding and Forge welding. | CO4 / R | 6 |
|  | b. | Discuss Roll welding and Hot pressure welding processes. | CO4 / R | 6 |

| 21. | a. | Define destructive and non-destructive testing of weld specimen. | CO6 / A | 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | b. | Briefly explain different non-destructive testing techniques and their working principles. | CO6 / An | 10 |  |
| 22. | a. | Define weldability. | CO1 / R | 2 |  |
|  | b. | Discuss about the different types of grooves and joint for effective welding. | CO5 / A | 10 |  |
| 23. | a. | Explain the difference between Cold and Diffusion bonding. | CO2 / U | 6 |  |
|  | b. | What are the tools needed for arc welding? | CO2 / R | 3 |  |
|  | c. | Explain the different types of electrodes. | CO2 / R | 3 |  |
|  |  | Compulsory: |  |  |  |
| 24. | a. | Describe the various zones of Welding. | CO5 / R |  | 2 |
|  | b. | Briefly discuss the preheat and post weld heat treatment advantages. | CO4 / A |  | 4 |
|  | c. | Explain about the defects and remedies associated with the welding process. | CO5 / U |  | 6 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Summarize different welding processes and its applications. |
| CO2 | Suggest welding processes for ferrous and non-ferrous alloys. |
| CO3 | Enumerate the various applications, advantages and types of welding processes. |
| CO4 | Adapt different types of welding process for effective welding of structural components and complex shapes. |
| CO5 | Design welding joints & test weldments for its quality. |
| CO6 | Relate the principles of metallurgy during the welding process. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 13 | 7 | - | - | - | - | 20 |
| CO2 | 7 | 7 | - | - | - | - | 14 |
| CO3 | 2 | 11 | - | 6 | - | - | 19 |
| CO4 | 12 | 5 | 15 | - |  |  | 32 |
| CO5 | 2 | 8 | 13 | - | - | - | 23 |
| CO6 | 3 | 1 | 2 | 10 | - | - | 16 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18ME2058 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name: | # MECHATRONICS SYSTEMS | # Max. Marks: | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | State industrial automation. | CO1 | R | 1 |
| 2. | Differentiate transducer from sensor. | CO2 | U | 1 |
| 3. | Abbreviate PLC and HMI. | CO1 | R | 1 |
| 4. | State the term Mechatronics. | CO3 | R | 1 |
| 5. | Draw the simple sketch of DC motor. | CO2 | U | 1 |
| 6. | Define protective relay. | CO4 | R | 1 |
| 7. | Name few electromechanical devices. | CO4 | R | 1 |
| 8. | List advantages and disadvantages of PLC. | CO6 | U | 1 |
| 9. | Draw the the pin diagram of any microprocessor and name it. | CO5 | U | 1 |
| 10. | Define the principle of magnetostriction. | CO5 | R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Identify the sensor, signal conditioner and display elements in the measurement systems of (a) mercury-in-glass thermometer (b) bourdon pressure gauge | CO1 | A | 3 |
| 12. | State sensors. Explain the working of inductive proximity sensor. | CO2 | E | 3 |
| 13. | Enumerate the differences between hydraulic and pneumatic systems. | CO3 | A | 3 |
| 14. | Discuss about Shape Memory Alloy. | CO4 | U | 3 |
| 15. | Describe the working of a magneto strictive transducers. | CO5 | A | 3 |
| 16. | Analyse the data handling instructions in PLC programming. | CO6 | An | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | a. | The mechatronics is an interdisciplinary field in which the disciplines those act together. | CO1 | E | 6 |
|  | b. | Discuss the levels of integration in Mechatronics systems. | CO1 | A | 6 |
| 18. | a. | Identify the various elements that might be present in acontrol system involving a thermostatically controlled electric heater. | CO2 | An | 8 |
|  | b. | List various applications of Mechatronics systems. | CO2 | A | 4 |

| 19. | a. | Describe in detail the principle, application and the uses of shaft encoders. | CO3 | U | 8 |
| --- | --- | --- | --- | --- | --- |
|  | b. | Describe the working of optical encoder with neat diagram. | CO3 | U | 4 |
| 20. | a. | Illustrate stepper motor basics, types, uses and working principles with neat diagram. | CO6 | E | 6 |
|  | b. | Demonstrate the principle of operation for variable reluctance stepper motor. | CO6 | E | 6 |
| 21. | a. | Describe the concept of Piezoelectric Effectwith neat drawing and discuss the working principle. | CO5 | U | 6 |
|  | b. | Narrate the working of a magneto restrictive transducer. | CO5 | U | 6 |
| 22. | a. | Discuss the working of Electro-hydraulic power steering system in automobile with neat diagram. | CO4 | A | 6 |
|  | b. | Describe some of the I/O devices used for PLC. | CO4 | A | 6 |
| 23. | a. | Describe the basic principle of X-ray Lithography with neat diagram. | CO5 | E | 6 |
|  | b. | Discuss any two types of micro-joining processes. | CO5 | E | 6 |
|  |  | Compulsory |  |  |  |
| 24. | a. | Describe the architecture of PLC with a neat block diagram. | CO6 | A | 6 |
|  | b. | List the PLC programming methods and describe the LD programming basic symbols of Siemens S7. | CO6 | R | 6 |

|  | COURSE OUTCOMES |
| --- | --- |
|  | After completing the course, the student will be able to |
| CO1 | Summarize and recall the overview of mechatronics applications. |
| CO2 | Demonstrate knowledge of electrical circuits and logic design. |
| CO3 | Develop and formulate engineering solutions and techniques to solve design problems. |
| CO4 | Design mechatronic components and systems. |
| CO5 | Classify and select various micro-sensors and microprocessors for a specific problem. |
| CO6 | Develop PLC programs for a given task. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 2 | 0 | 9 | 0 | 6 | 0 | 17 |
| CO2 | 0 | 2 | 0 | 12 | 3 | 0 | 17 |
| CO3 | 1 | 12 | 3 | 0 | 0 | 0 | 16 |
| CO4 | 2 | 3 | 0 | 12 | 0 | 0 | 17 |
| CO5 | 1 | 13 | 0 | 3 | 12 | 0 | 29 |
| CO6 | 7 | 0 | 0 | 9 | 12 | 0 | 28 |
| Total | 13 | 30 | 12 | 36 | 33 | 0 | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18ME2060 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name: | # INDUSTRIAL SAFETY ENGINEERING | # Max. Marks: | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | Define the term Industrial safety. | CO1 | R | 1 |
| 2. | A safety programme consists of___________ E’s. | CO1 | U | 1 |
| 3. | OSHA stands for___________. | CO2 | R | 1 |
| 4. | In oxidizing flame ___________gas is less. | CO2 | A | 1 |
| 5. | Name the gases commonly used during gas welding process. | CO3 | A | 1 |
| 6. | Define annealing process of heat treatment. | CO3 | R | 1 |
| 7. | List few examples for explosive chemicals. | CO4 | U | 1 |
| 8. | Expand the term MSDS. | CO4 | R | 1 |
| 9. | Give an example of electroplating. | CO5 | U | 1 |
| 10. | Carbon arc lamps are commonly used in___________. | CO5 | A | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Difference between accidents vs. injuries. | CO1 | U | 3 |
| 12. | State the functions of safety trip control. | CO2 | A | 3 |
| 13. | List out the objectives of industrial safety. | CO3 | R | 3 |
| 14. | Write any two safety principles that should be practiced during Arc welding process. | CO4 | U | 3 |
| 15. | Mention the four uses of electroplating. | CO5 | R | 3 |
| 16. | Define luminous flux. | CO6 | R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | a. | List out the significance of industrial safety. | CO1 | R | 3 |
|  | b. | Explain the various responsibilities of principals in safety management. | CO1 | A | 9 |
| 18. |  | List out the various methods used for safeguarding machines in an industry. | CO2 | An | 12 |
| 19. |  | Appraise the various safety principles to be followed during gas welding process and list out the protective equipment’s used for the same. | CO3 | A | 12 |

| 20. | a. | State any two common hazards while handling acids. | CO4 | A | 2 |
| --- | --- | --- | --- | --- | --- |
|  | b. | In detail explain the various Safety requirements recommended in an chemical industry for handling chemicals | CO4 | An | 10 |
| 21. | a. | Enumerate the safety responsibility to be followed by the heat treatment process. | CO5 | R | 10 |
|  | b. | List out the various stages of heat treatment process. | CO5 | U | 2 |
| 22. |  | Explain the various responsibilities of Owners, Executives & Officers in safety management. | CO1 | A | 12 |
| 23. |  | Discuss the various hazards and safety principles to be followed during electroplating process. | CO4 | U | 12 |
|  |  | Compulsory: |  |  |  |
| 24. | a. | List out any various safety protocols to be followed and implemented in paint shop. | CO6 | A | 10 |
|  | b. | Define Illumination. | CO6 | R | 2 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understanding the importance of safety in process industries. |
| CO2 | Understanding the ethical issues that may arise from industrial processes. |
| CO3 | Communicate the difference between Hazard and Risk. |
| CO4 | Be able to express Safety in terms of Risk and to recognize unacceptable/inappropriate levels of Risk. |
| CO5 | Be able to Assess & identify the potential hazards in process industries. |
| CO6 | Appreciation and applying safety procedures in a process industries. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 4 | 4 | 21 | - | - | - | 29 |
| CO2 | 1 | - | 4 | 12 |  |  | 17 |
| CO3 | 4 | - | 13 | - | - | - | 17 |
| CO4 | 1 | 16 | 2 | 10 |  |  | 29 |
| CO5 | 13 | 3 | 1 | - | - | - | 17 |
| CO6 | 5 | - | 10 | - | - | - | 15 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18ME2063 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # RAPID MANUFACTURING TECHNOLOGIES | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | Define the rapid prototyping. | CO1 | R | 1 |
| 2. | Explain the role of rapid prototype in product development process. | CO1 | U | 1 |
| 3. | Write the purpose of reverse engineering. | CO2 | U | 1 |
| 4. | List out the applications of reverse engineering. | CO2 | A | 1 |
| 5. | Write the advantages of liquid based rapid prototyping process. | CO3 | U | 1 |
| 6. | Write the fundamental principle of stereo lithography process. | CO3 | U | 1 |
| 7. | Define the fused deposition modeling. | CO4 | R | 1 |
| 8. | List out the application of fused deposition modeling. | CO4 | A | 1 |
| 9. | List the materials that are utilized in the powder-based rapid prototyping method. | CO5 | U | 1 |
| 10. | Write the critical factors that influence the performance of Selective Laser Sintering process. | CO5 | U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Write short notes on importance of rapid prototyping in industries. | CO1 | U | 3 |
| 12. | Distinguish between forward and reverse engineering. | CO2 | A | 3 |
| 13. | Write various types of liquid based rapid prototyping process and their applications. | CO3 | A | 3 |
| 14. | Compare solid based rapid proto typing and liquid based rapid proto typing. | CO4 | An | 3 |
| 15. | Discuss the advantages and disadvantages of Selective laser sintering process. | CO5 | A | 3 |
| 16. | Define rapid tooling and list out its advantages. | CO6 | R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | a. | Classify rapid prototyping and give its basic principle. | CO1 | U | 6 |
|  | b. | Explain in detail the process chain of rapid prototyping. | CO1 | A | 6 |
| 18. | a. | Define Digitization Process. Explain the steps involved in reverse engineering. | CO2 | A | 6 |
|  | b. | List merits and demerits of wire frame, solid and surface modeling. | CO2 | U | 6 |
| 19. | a. | Describe the principle of Solid Ground Curing with its advantages, disadvantages and applications. | CO3 | An | 6 |
|  | b. | Define recoating and explain the necessity in Stereo lithography. | CO3 | A | 6 |
| 20. | a. | Describe Fused deposition modeling process with a neat sketch. | CO4 | A | 8 |
|  | b. | List out the advantages and disadvantages of laminated object manufacturing process. | CO4 | U | 4 |
| 21. | a. | Explain the critical factors that influence the performance and functions of Selective Laser Sintering and 3-Dimentional printing. | CO5 | A | 6 |
|  | b. | Discuss the principle of selective laser sintering process using a case study. | CO5 | A | 6 |
| 22. | a. | Explain with the help of a simple line diagram the construction details of extrusion head in FDM process. | CO4 | An | 10 |
|  | b. | Write types of materials used in solid based rapid prototyping. | CO4 | U | 2 |
| 23. | a. | Differentiate Stereo lithography and Selective Laser Sintering in rapid prototyping. | CO5 | An | 6 |
|  | b. | Write a case study on Selective Laser Sintering process. | CO5 | A | 6 |
|  |  | Compulsory: |  |  |  |
| 24. |  | Explain with diagrams, the selecting laser melting and electron beam melting process. Compare these processes | CO6 | An | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Differentiate between conventional and rapid manufacturing approach |
| CO2 | Demonstrate the knowledge of Rapid Manufacturing technologies |
| CO3 | Understand the need and place for RP in an integrated manufacturing environment |
| CO4 | Get exposed to commercial Rapid Prototyping systems |
| CO5 | Possess knowledge on Rapid Prototyping software |
| CO6 | Model and manufacture RP components |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 1 | 10 | 6 |  | - | - | 17 |
| CO2 |  | 7 | 10 |  |  |  | 17 |
| CO3 |  | 2 | 9 | 6 |  | - | 17 |
| CO4 | 1 | 6 | 9 | 13 |  |  | 29 |
| CO5 |  | 2 | 21 | 6 | - | - | 29 |
| CO6 | 3 |  |  | 12 | - | - | 15 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2020

| # Code : | # 18ME2067 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # AUTOMOBILE ENGINEERING | # Max. Marks : | # 100 |

| Q. No. | Questions |  | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | Weight of the vehicle produces in the side members of the frame is ________. | CO1 / R |  | 1 |
| 2. | What are the loads coming on a chassis frame? | CO1 / E |  | 1 |
| 3. | The inertia of the rotating parts of the clutch should be __________. | CO2 / A |  | 1 |
| 4. | Define perfect steering. | CO 2 / U |  | 1 |
| 5. | State the advantages of a rack and pinion type steering gear. | CO3 / An |  | 1 |
| 6. | The turning circle for a car is approximately __________. | CO 4 / A |  | 1 |
| 7. | Most popular manual steering gear of cars today is __________. | CO4 / C |  | 1 |
| 8. | State the advantages of a double arm type contact breaker. | CO5 / A |  | 1 |
| 9. | What is the function of oil in the ignition coil? | CO 5 / U |  | 1 |
| 10. | State the octane rating of petrol used in Indian automobiles. | CO 6 / A |  | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Name the major electrical components in an automobile. | CO1 / U | 3 |
| 12. | Write short notes on transmission of a vehicle. | CO2 / A | 3 |
| 13. | Write some advantages and disadvantages of freewheeling. | CO 3 / An | 3 |
| 14. | State the advantages of EBD. | CO4 / A | 3 |
| 15. | Differentiate between electronic injection system in SI and CI engine. | CO 5/ C | 3 |
| 16. | List some of the alternate fuels in India. | CO 6/ E | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | Explain the working of a single plate clutch with the help of a simple diagram. | CO 1 / A | 12 |
| 18. | a. | Describe the function of an automobile transmission system using a neat sketch. | CO 2 / C | 12 |
| 19. | a. | Explain the working principle of power steering with a neat sketch. | CO 3/ An | 12 |
| 20. | a. | Describe the function of Hydraulic breaking system in detail. | CO 4 / A | 12 |

| 21. | a. | Explain the probable causes of various steering troubles and suggest suitable remedies. | CO 4 / E | 8 |
| --- | --- | --- | --- | --- |
|  | b. | Discuss in detail various adjustments in the steering geometry and the steering gear. | CO4 / A | 4 |
| 22. | a. | Write short notes on a) cracking b) reforming c) petrol d) CNG e) hydrocarbons. | CO 6/ U | 12 |
| 23. | a. | Give detailed account of the battery ignition system. Illustrate your answer with neat sketches. | CO5 / An | 12 |
|  |  | Compulsory: |  |  |
| 24. | a. | Describe in detail various types of fuels for automotive engines. Give specific examples in each case. | CO6 / A | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand different types of internal combustion engines. |
| CO2 | Demonstrate the functions of clutch and gear box systems. |
| CO3 | Describe the types of steering and suspension systems. |
| CO4 | Summarize the construction and operating principles of brakes and tyres. |
| CO5 | Express the functions and components of fuel injection and ignition systems. |
| CO6 | Analyze the performance and emissions of alternate fuels. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 1 | 3 | 12 |  | 1 | - | 17 |
| CO2 |  | 1 | 4 |  |  | 12 | 17 |
| CO3 | - |  | 1 | 16 | - | - | 17 |
| CO4 | - |  | 20 | - | 8 | 1 | 29 |
| CO5 |  | 1 | - | 12 | - | 3 | 16 |
| CO6 |  | 12 | 13 | - | 3 | - | 28 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18ME2071 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ROBOTIC ENGINEERING | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | RIA abbreviated as____________. | CO1 | R | 1 |
| 2. | The body, arm and wrist assembly is sometimes called as_______. | CO2 | U | 1 |
| 3. | Grippers are end effectors used to __________ and __________. | CO3 | R | 1 |
| 4. | Tactile sensors are device which indicates___________. | CO4 | An | 1 |
| 5. | Work volume is the term that refers to____________. | CO2 | U | 1 |
| 6. | Hydraulic drive is generally associated with____________. | CO3 | A | 1 |
| 7. | Robot dynamics is concerned with the analysis of___________. | CO5 | U | 1 |
| 8. | Digitizing the images at the rate of _________ frames per second. | CO5 | R | 1 |
| 9. | Define Machine vision. | CO5 | U | 1 |
| 10. | AGV is abbreviated as ____________. | CO6 | R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Discuss the three Laws of Robotics by Asimov. | CO1 | R | 3 |
| 12. | Explain the sensors used in robotics. | CO4 | A | 3 |
| 13. | Define controllers and its classifications. | CO3 | U | 3 |
| 14. | Explain briefly about the Forward Transformation of a 2-Degreeof Freedom Arm. | CO4 | R | 3 |
| 15. | Explain about RAIL and VAL II. | CO5 | An | 3 |
| 16. | Discuss about the new trends in robotics. | CO6 | An | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | a. | Explain robot motions with neat sketch. | CO1 | R | 4 |
|  | b. | Explain the assembly and robotic assembly automation. | CO2 | U | 4 |
|  | c. | Explain the followings: resolution, accuracy, repeatability, compliance. | CO2 | A | 4 |
| 18. | a. | Elaborate the proximity and range sensor with neat sketch. | CO4 | An | 6 |
|  | b. | Discuss briefly the types of gripper. | CO3 | R | 3 |
|  | c. | Briefly explain sensors application in robotic arc welding. | CO3 | A | 3 |

| 19. | a. | With neat sketches explain the different types of actuators. |  | CO3 | U |  | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | b. | Enumerate the details of vision inspection system with example. |  | CO4 | C |  | 6 |
| 20. | a. | Explain in detail the four degree of freedom manipulator in three dimension. |  | CO5 | An |  | 10 |
|  | b. | Write briefly the homogeneous transformations and its robot kinematics. |  | CO4 | R |  | 2 |
| 21. | a. | Explain the different methods of robot programming. Discuss briefly about the generation of robot programming. |  | CO5 | E |  | 8 |
|  | b. | Explain the Image processing and analysis in detail. |  | CO5 | R |  | 4 |
| 22. |  | Explain in detail WAIT, SIGNAL, DELAY commands with examples. |  | CO5 | A |  | 12 |
| 23. | a. | Explain in detail the robot dynamics. |  | CO4 | U |  | 6 |
|  | b. | Distinguish the robot dynamics and robot arm dynamics. |  | CO4 | U |  | 6 |
|  |  | Compulsory: |  |  |  |  |  |
| 24. | a. | Explain in detail the Artificial Intelligence (AI) technique. | CO6 |  | A | 10 |  |
|  | b. | Explain AI implementation with a case study. | CO6 |  | C | 2 |  |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Identify the robot types and configurations. |
| CO2 | Evaluate the complete robot and can identify the control system acting on it. |
| CO3 | Compute the kinematic equations to select an actuator for robot configurations. |
| CO4 | Categorize the suitable sensor and implement for appropriate robot application. |
| CO5 | Design the robot Program for an industrial application. |
| CO6 | Develop the robot for a unique operation. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 8 |  |  |  | - | - | 8 |
| CO2 |  | 9 | 4 |  |  |  | 13 |
| CO3 | 4 | 6 | 5 |  |  |  | 15 |
| CO4 | 5 | 12 | 3 | 6 |  | 6 | 32 |
| CO5 | 6 | 4 | 12 | 10 | 8 |  | 40 |
| CO6 | 1 |  | 10 | 3 |  | 2 | 16 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 18ME2073 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # MODERN MANUFACTURING TECHNIQUES | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | Identify the type of manufacturing system, goods are produced in large quantities at low cost per unit and in a short period of time. | CO1 | U | 1 |
| 2. | Name the type of plastics, when heated, the chemical bonding inside this plastic changes without recourse. | CO1 | R | 1 |
| 3. | Identify the difference between non-amplifying and amplifying type of tool holders in USM. | CO2 | U | 1 |
| 4. | Name any two abrasive materials used in AJM process. | CO2 | R | 1 |
| 5. | Identify the role of field shaper in Electro Magnetic Forming. | CO3 | R | 1 |
| 6. | State the role of capacitor bank in Electro Hydraulic Forming. | CO3 | R | 1 |
| 7. | Name the type of casting used for the production of Billets. | CO4 | R | 1 |
| 8. | Identify the casting process which combines the advantages of both casting and forming processes. | CO4 | U | 1 |
| 9. | Define arc blow. | CO5 | R | 1 |
| 10. | Identify the functions of vacuum chamber in Electron Beam Welding. | CO5 | U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Compare thermoplastics with thermosetting plastics. | CO1 | An | 3 |
| 12. | Distinguish between orthogonal and oblique cutting. | CO2 | An | 3 |
| 13. | Distinguish between plastic and elastic deformation. | CO3 | An | 3 |
| 14. | Compare evaporative pattern casting and ceramic shell casting. | CO4 | An | 3 |
| 15. | Distinguish between touch start and field start methods of arc initiation. | CO5 | An | 3 |
| 16. | Sensor technology: a critical cutting edge of exposure science – Criticize. | CO6 | E | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | a. | Describe the process of injection moulding for thermoplastic polymers. | CO1 | U | 6 |
|  | b. | Sort different types of composites based on matrix materials. | CO1 | An | 6 |
| 18. | a. | Describe the equipment and applications of Abrasive Jet Machining process. | CO2 | U | 6 |
|  | b. | Discuss the functions and methods of application of dielectric fluids in EDM process. | CO2 | U | 6 |

| 19. | a. | Hot working is preferred where large deformations have to be performed – Justify. | CO3 | E | 6 |
| --- | --- | --- | --- | --- | --- |
|  | b. | Discuss stretch forming process with appropriate sketches. | CO3 | U | 6 |
| 20. | a. | Distinguish between the different functions of sprue, runner and riser. | CO4 | An | 6 |
|  | b. | Describe evaporative pattern casting with suitable sketches. | CO4 | U | 6 |
| 21. | a. | Discuss ultrasonic welding process. List its advantages and limitations. | CO5 | U | 6 |
|  | b. | Analyze and suggest different measures that can help to eliminate arc blow. | CO5 | An | 6 |
| 22. | a. | Explain the working principle and equipment setup of Electric Discharge Machining (EDM). | CO2 | U | 12 |
| 23. | a. | Describe the two different techniques used in explosive forming. List its advantages, limitations and applications. | CO3 | U | 12 |
|  |  | Compulsory: |  |  |  |
| 24. | a. | Compare the different types of automation in manufacturing systems. | CO6 | An | 6 |
|  | b. | Discuss the applications of Industrial Robots in manufacturing industries. | CO6 | U | 6 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand different manufacturing processes and the economic considerations |
| CO2 | Understand the theory of metal cutting and the sciences of advanced machining processes |
| CO3 | Learn the theories of advanced metal forming. |
| CO4 | Know about the process of metal casting in detail |
| CO5 | Understand the physics of arc welding and theory of advanced welding techniques |
| CO6 | Demonstrate an understanding of competitive manufacturing environment |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 1 | 7 |  | 9 | - | - | 17 |
| CO2 | 1 | 25 |  | 3 |  |  | 29 |
| CO3 | 2 | 18 |  | 3 | 6 | - | 29 |
| CO4 | 1 | 7 |  | 9 |  |  | 17 |
| CO5 | 1 | 7 |  | 9 | - | - | 17 |
| CO6 |  | 6 |  | 6 | 3 | - | 15 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – June – 2022

| # Code : | # 18ME3032 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # COMPUTATIONAL FLUID DYNAMICS | # Max. Marks : | # 100 |

ANSWER ALL QUESTIONS (5 X 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| 1. |  | Derive the 3 dimensional momentum equation for compressible fluids in X direction with necessary source terms. | CO1 / A | 20 |
| (OR) |  |  |  |  |
| 2. |  | Consider the problem of source free heat conduction in an insulated rod whose ends are maintained at constant temperature 200 oC and 500 oC respectively as shown in figure. Formulate the algebraic equations for one dimensional steady state temperature distribution of the rod. Thermal conductivity k = 1000 W/m-K and the cross sectional area A is 10 x 10-3 m2. Take δx=0.1m. | CO2 / An | 20 |
| 3. |  | Write a short notes with necessary sketches on | CO4 / U | 20 |
|  | a. | Cartesian mesh |  |  |
|  | b. | Hexahedral mesh |  |  |
|  | c. | Adaptive mesh |  |  |
| (OR) |  |  |  |  |
| 4. |  | Explain in detail about Structured and Unstructured grid. Also explain the significance and advantages of structured grid. | CO5 / A | 20 |
| 5. |  | Discuss the steps involved in solving two dimensional diffusion problems using Finite volume approach. | CO3/ A | 20 |
| (OR) |  |  |  |  |
| 6. |  | What are the limitations of central differencing scheme and how it can be rectified? | CO4 / E | 20 |
| 7. |  | Explain in detail the various properties of discretization schemes. | CO3 / An | 20 |
| (OR) |  |  |  |  |
| 8. |  | Derive an equation to solve steady one dimensional convection and diffusion problems. | CO4 / A | 20 |

|  |  | Compulsory: |  |  |
| --- | --- | --- | --- | --- |
| 9. |  | Classify the turbulence models and discuss in detail about the k-epsilon model with necessary terms. | CO6 / An | 20 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Develop governing equations for fluid flow and heat transfer. |
| CO2 | Demonstrate the physical behaviors of flow. |
| CO3 | Perform CFD analysis. |
| CO4 | Impose boundary conditions while solving CFD problems. |
| CO5 | Develop various types of grids for solving CFD problems. |
| CO6 | Applying turbulence and combustion models in problem solving. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | - | - | 20 | - | - | - | 20 |
| CO2 | - | - | - | 20 | - | - | 20 |
| CO3 | - | - | 20 | 20 | - | - | 40 |
| CO4 | - | 20 | - | 20 | 20 | - | 60 |
| CO5 | - | - | 20 | - | - | - | 20 |
| CO6 | - | - | - | - | 20 | - | 20 |
|  |  |  |  |  |  |  | 180 |

Reg. No. _____________

End Semester Examination – June – 2022

| # Code : | # 18ME3043 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # INDUSTRIAL AUTOMATION AND MECHATRONICS | # Max. Marks: | # 100 |

ANSWER ALL QUESTIONS (5 X 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | List the main objectives of plant layout and draw different layouts. | CO1 / R | 06 |
|  | b. | Describe the product layout and process layouts with suitable diagrams with their merits and demerits. | CO1 / U | 14 |
| (OR) |  |  |  |  |
| 2. | a. | Discuss the fixed position plant layout. | CO1 / U | 06 |
|  | b. | Enumerate the Strategies of Automation to improve productivity in the manufacturing industries. | CO2 / A | 14 |
| 3. | a. | Name the design and fabrication considerations to be taken for the implementation of automation in an assemble line design. | CO2 / R | 06 |
|  | b. | A 30 station Transfer line is being proposed to machine a certain component currently produced by conventional methods. The proposal received from the machine tool builder states that the line will operate at a production rate of 100 pc / hr at 100% efficiency. From a similar transfer line, it is estimated that breakdowns of all types will occur at a frequency of F = 0.20 breakdowns per cycle and that the average downtime per line stop will be 8.0 minutes. The starting blank that is machined on the line costs Rs. 5.00 per part. The line operates at a cost for 100 parts each & the average cost per tool = Rs. 20 per cutting edge. Compute the following: 1. Production rate 2. Line efficiency 3. Cost per unit piece produced on the line | CO2 / An | 14 |
| (OR) |  |  |  |  |
| 4. | a. | With a neat sketch, write a brief note on Buffer Storage. | CO3 / A | 06 |
|  | b. | If a line has 20 work stations each with a probability of breakdown of 0.02, the cycle time of the line is 1 minute and each time a breakdown occurs, it takes exactly 5 minutes to repair. The line is to be divided into two stages by a storage buffer so that each stage will consist of 10 stations. Compute the efficiency of the two-stage line for various buffer capacities. | CO3 / An | 14 |

| 5. | a. | Enumerate briefly about unit load formation equipment. | CO3 / U | 06 |
| --- | --- | --- | --- | --- |
|  | b. | Consider the AGVS layout in Figure 10.14. Vehicles travel counterclockwise around the loop to deliver loads from the load station to the unload station. Loading time at the load station = 0.75 min, and unloading time at the unload station = 0.50 min. The following performance parameters are given: vehicle speed = 50 m/min, availability = 0.95, and traffic factor = 0.90. Operator efficiency does not apply, so Ew = 1.0. Determine (a) travel distances loaded and empty, (b) ideal delivery cycle time, and (c) number of vehicles required to satisfy the delivery demand if a total of 40 deliveries per hour must be completed by the AGVS. Figure.1. AGVS loops layout. Key: Unld = unload, Man = Manual operation, dimensions are in meters (m) | CO3 / C | 14 |
| (OR) |  |  |  |  |
| 6. | a. | Elaborate the function of an Automated storage and Retrieval systems with a neat sketch. | CO4 / A | 06 |
|  | b. | Describe in detail about the type of Automated Guided Vehicles with suitable sketches. | CO4 / A | 14 |
| 7. | a. | Illustrate any one of the proximity sensors and list out the merits and demerits of three proximity sensors. | CO5 / R | 06 |
|  | b. | Discuss in detail about the working principles of: (i). External gear pump and (ii). Radial piston pump with suitable drawings. | CO5 / U | 14 |
| (OR) |  |  |  |  |
| 8. | a. | Write a short note on Position Plus Velocity Feedback Control. | CO5 / U | 04 |
|  | b. | With neat sketches, describe in detail about the Fuzzy Logic Control System. | CO 5 / U | 16 |
|  |  | Compulsory: |  |  |
| 9. | a. | Discuss the about the Machine Vision system used in manufacturing industries. | CO6 / U | 10 |
|  | b. | Describe the following: Digital Transducers and Human - Machine interface. | CO6 / U | 10 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Develop intelligent automated system and manufacturing data base system. |
| CO2 | Implement the concepts of a productive system in automation. |
| CO3 | Apply the knowledge of automated flow lines for industrial and other applications. |
| CO4 | Design and analysis of material handling systems for automated assembly lines. |
| CO5 | Select proper sensor and actuator for a given application. |
| CO6 | Balance automated assembly lines. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 6 | 20 |  |  |  |  | 26 |
| CO2 | 6 |  | 14 | 14 |  |  | 34 |
| CO3 |  | 6 | 6 | 14 |  | 14 | 40 |
| CO4 |  |  | 20 |  |  |  | 20 |
| CO5 | 6 | 34 |  |  |  |  | 40 |
| CO6 |  | 20 |  |  |  |  | 20 |
|  |  |  |  |  |  |  | 180 |

Reg. No. _____________

End Semester Examination – May / June – 2022

| # Code : | # 19ME1003 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ENGINEERING MECHANICS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | A body with 1 kg mass is in a translational motion after applying the 1 N force. Calculate the acceleration of the body in m/s2. | CO1/A | 1 |
| 2. | Calculate the force (N) due to the weight experiencing the body with 1 kg mass on earth. | CO1/A | 1 |
| 3. | Define couple forces. | CO2/R | 1 |
| 4. | “The effect of a given force on a rigid body remains unchanged” under what condition this statement is true? | CO2/An | 1 |
| 5. | Write an expression for the first moment of area. | CO3/R | 1 |
| 6. | If the centroid of an area is located on a coordinate axis, what would be the first moment of the area with respect to that axis? | CO3/U | 1 |
| 7. | Write the difference between the uniformly accelerated rectilinear motion and uniform rectilinear motion. | CO4/U | 1 |
| 8. | Under which conditions the bodies are said to be in dependent motion. | CO4/R | 1 |
| 9. | When do you call the energy is conserved? | CO5/U | 1 |
| 10. | Define the mechanical energy. | CO5/R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | An iron block of mass 30 kg is hanging from the two supports A and B as shown in the diagram. Determine the tensions in both the ropes. | CO1/An | 3 |
| 12. | Explain how the principle of transmissibility is applied on the van given in the following figure. | CO2/A | 3 |
| 13. | Using the figure given below explain the center of gravity and center of mass. | CO3/U | 3 |

| 14. | A particle moving on a line is at position x(t)=t3−9t2+24t−5 at time t where t is in seconds. At which time ‘t’ (if any) does the particle change its direction? | CO4/An | 3 |
| --- | --- | --- | --- |
| 15. | Explain D’Alembert’s Principle. | CO5/R | 3 |
| 16. | Write the definition for frictional forces and mention the types of frictional forces. | CO6 /R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | A tower guy wire is anchored by means of a bolt at A. The tension in the wire is 2000 N. Determine (i) the components Fx, Fy, Fz of the force acting on the bolt, (ii) the angles øx, øy, øz defining the direction of the force. | CO1/An | 8 |
|  | b. | Explain the following with neat sketch. - Principle of transmissibility - Rectangular components of forces. | CO1/U | 4 |
| 18. |  | ﻿A 100-lb vertical force is applied to the end of a lever which is attached to a shaftat O. Determine (i) the moment of the 100-lb force about O; (ii)the horizontal force applied at A which creates the same moment about O; (iii) the smallest force applied at A which creates the same moment about O; (iv) how far from the shaft a 240-lb vertical force must act to create the same moment about O. | CO2/An | 12 |
| 19. | a. | Derive the mathematical expression for Moment of Inertial using Perpendicular Axis Theorem. | CO3/U | 4 |
|  | b. | ﻿Determine the moment of inertia of the shaded area with respect to the x axis. | CO3/A | 8 |

| 20. | a. | Derive the mathematical expression for the normal and tangential components of acceleration for a particle moving along the curved path. | CO4/U | 4 |
| --- | --- | --- | --- | --- |
|  | b. | ﻿The position of a particle which moves along a straight line is defined by the relation x= t3-6t2-15t+40, where x is expressed in feet and t in seconds. Determine (i) the time at which the velocity will be zero, (ii) the position and distance traveled by the particle at that time. | CO4/A | 8 |
| 21. | a. | ﻿An automobile weighing 5000 lb is driven down a 5° incline at a speed of 80 mi/h when the brakes are applied, causing a constant total braking force(applied by the road on the tires) of 1000 lb. Determine the distance traveled by the automobile as it comes to a stop. | CO5/A | 5 |
|  | b. | Show that the coefficient of restitution is a ratio of relative velocity of two bodies after impact to the before impact, mention it’s limitations. | CO5/U | 7 |
| 22. | a. | Determine the rectangular components of the moment of force ‘F’ and position vector ‘r’ for a 3-dinemensional co-ordinate system. | CO2/U | 4 |
|  | b. | ﻿Determine the components of the single couple equivalent to the twocouples shown. | CO2/An | 8 |
| 23. | a. | ﻿Two blocks are joined by an inextensible cable as shown. If the system is released from rest, determine the velocity of block A after it has moved 2 m. Assume that the coefficient of kinetic friction between block A and the plane is 0.25 and that the pulley is weightless and frictionless. | CO5/An | 8 |
|  | b. | Explain period of deformation and period of restitution with neat sketch. | CO5/R | 4 |
|  |  | Compulsory: |  |  |
| 24. | a. | Explain the Frictional forces (F) vs. Applied force (P) graph for the following conditions for a square box (Weight W) placed on a rough surface with the help of neat diagrams. i) No friction ii) No Motion iii) Motion Impending iv) Motion | CO6/R | 8 |
|  | b. | Explain the angle of friction with the help of neat sketch. | CO6/R | 4 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | ﻿Determine the resultant force and moment for a given system of forces. |
| CO2 | ﻿Understand basics of equilibrium of rigid bodies |
| CO3 | ﻿Determine the centroid and second moment of area of simple solids. |
| CO4 | ﻿Apply fundamental concepts of kinematics and kinetics to the analysis of simple / practical problems. |
| CO5 | ﻿Understand basic kinematics concepts – displacement, velocity and acceleration. |
| CO6 | ﻿Determine friction and its effects as per the laws of friction. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | - | 4 | 2 | 11 | - | - | 17 |
| CO2 | 1 | 4 | 3 | 21 | - | - | 29 |
| CO3 | 1 | 8 | 8 | - | - | - | 17 |
| CO4 | 1 | 5 | 8 | 3 | - | - | 17 |
| CO5 | 8 | 8 | 5 | 8 | - | - | 29 |
| CO6 | 15 | - | - | - | - | - | 15 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – May / June – 2022

| # Code : | # 19ME2020 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # DRONE TECHNOLOGY | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | The Aerosonde is a small UAV designed to collect _______________data over oceans and remote areas. | CO1 / R | 1 |
| 2. | Give a reason why some roles are dangerous to the unmanned air vehicles. | CO1 / R | 1 |
| 3. | Which is the UAV that flies over 15,000 m altitude and has an endurance of more than 24 hours? | CO2 / R | 1 |
| 4. | The lower pressure on the upper surface and the higher pressure on the lower surface of a wing is merely the ____________for the reaction force. | CO2 / R | 1 |
| 5. | The horizontal component of the reaction force is a drag known as the______________, that must be overcome by the propulsion system of the aircraft if it is to maintain airspeed. | CO3 / R | 1 |
| 6. | If the aircraft travelling with forward velocity (V) and deflected air velocity (u), then the lift to drag ratio is obtained for small airstream deflection angles ϕ is given as__________. | CO3 / R | 1 |
| 7. | Identification of the engine by sounds that are produced by drones / aircraft is known as ____________signature. | CO4 / R | 1 |
| 8. | A Wide-area search to look (principally with Radar) for an early sign of enemy movements is done by____________ early warning system. | CO4 / R | 1 |
| 9. | Several UAVs, possibly in swarms, could be sent ahead of a manned strike force to attract the attention of air defence systems so that their positions are disclosed is done by ____________counter system. | CO5 / R | 1 |
| 10. | The monitoring of traffic in narrow sea-lanes is a ___________activity required to ensure that traffic proceeds in allocated lanes. | CO5 / R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Write the history about the development of Pioneer drone developed by the nation of Israel. | CO1 / U | 3 |
| 12. | Explain how the conceptual phase plays an important role in the development of drones. | CO2 / U | 3 |
| 13. | Describe the principle of lift-induced drag and its significance in drones. | CO3 / U | 3 |
| 14. | Differentiate between dispensable and non-dispensable payloads. | CO4 / U | 3 |
| 15. | How mid-air collision system works to avoid the collision of drones? | CO5 / U | 3 |
| 16. | Explain the commercial roles carried out by drones in the present world. | CO6 / U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | Describe the advanced features and specifications of RQ-4 Global Hawk. | CO1 / A | 6 |
|  | b. | Explain the dull and dangerous roles by the drones. | CO1 / A | 6 |
| 18. | a. | Describe in detail the economic reasons and factors to design, develop and deployment of the drones. | CO2 / A | 6 |
|  | b. | Describe how the drones play vital roles in environmentally critical conditions. | CO2 / A | 6 |
| 19. | a. | Explain the functions and applications of the Firebee 1241 drone and Scout drone developed by the nation of Israel. | CO3 / A | 6 |
|  | b. | Distinguish between the Pathfinder and the Helios (US). | CO3 / A | 6 |
| 20. | a. | Explain the importance of the preliminary design phase and the detailed design phase. | CO3 / An | 6 |
|  | b. | Explain the generalized forces acting on a drone: lift, thrust, drag and gravity. | CO3 / An | 6 |
| 21. | a. | Sketch neatly the load types and the effective load at the centroid and explain the importance of handling loads. | CO4 / An | 6 |
|  | b. | Classify types of beams and tabulate the idealization of support, reaction and its characteristics. | CO4 / An | 6 |
| 22. | a. | Explain the significance of the following parameters involved with the air vehicle: - Air Vehicle – Payload (Capacity) - Air Vehicle – Endurance (Time) | CO4 / E | 6 |
|  | b. | Explain the significance of the following parameters involved with the air vehicle: - Air Vehicle – Radius of Action (Operating Distance) - Air Vehicle – Speed Range (Travel) | CO4 / E | 6 |
| 23. | a. | How the thermal signature and the radio/radar signature are applied in the drones. Give a case study on both. | CO5 / E | 6 |
|  | b. | Sketch neatly and explain the different types of antennas and its uses. | CO5 / E | 6 |
|  |  | Compulsory: |  |  |
| 24. | a. | Describe the roles of drones in Indian Navy and Air force. | CO6 / C | 6 |
|  | b. | Describe the roles of drones in Indian Army and Paramilitary. | CO6 / C | 6 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the design parameters of UAV systems |
| CO2 | Understand the aerodynamics and selection of power plants of UAV systems |
| CO3 | Identify stealth and payload types of UAV systems |
| CO4 | Analyze the principles of communication and control station systems used in UAV’s |
| CO5 | Design launch and recovery systems of UAV’s |
| CO6 | Apply the application of UAS for various applications |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 2 | 3 | 12 | - | - | - | 17 |
| CO2 | 2 | 3 | 12 | - | - | - | 17 |
| CO3 | 2 | 3 | 12 | 12 | - | - | 17 |
| CO4 | 2 | 3 | - | 12 | 12 | - | 17 |
| CO5 | 2 | 3 | - | - | 12 | - | 17 |
| CO6 | - | 3 | - | - | - | 12 | 15 |
|  |  |  |  |  |  |  | 100 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 19ME2025 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # THERMODYNAMICS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Isothermal process is a constant __________ process. | COI / R | 1 |
| 2. | What is a thermodynamic process? | COI / R | 1 |
| 3. | First law of thermodynamics deals with conservation of ________. | CO2 / R | 1 |
| 4. | # Define Coefficient of performance (COP). | CO2 / R | 1 |
| 5. | Fusion curve, Vaporization curve and sublimation curves meet at the _____ point. | CO3 / U | 1 |
| 6. | State the Avagadro’s law. | CO3 / R | 1 |
| 7. | # Air that leaks into the room through window cracks and door openings is called _________. | CO4 / R | 1 |
| 8. | What is meant by Humidification? | CO5 / U | 1 |
| 9. | _________ Cycle is also called as constant pressure cycle. | CO6 / R | 1 |
| 10. | In a dual cycle heat addition process takes place at _________. | CO6 / U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Distinguish between the terms ‘path’, and ‘process. | COI / U | 3 |
| 12. | State Kelvin-Planck and Clausius statements. | CO2 / U | 3 |
| 13. | Define (a) dry steam, (b) wet steam and (c) superheated steam. | CO3 / R | 3 |
| 14. | Define (a) mass fraction and (b) mole fraction. | CO4 / R | 3 |
| 15. | What do you understand by evaporative cooling? | CO5 / U | 3 |
| 16. | Determine the air standard efficiency of diesel cycle for a compression ratio of 13 and cut-off ratio of 1.96. | CO6 / A | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 17. | a. | Define a Thermodynamics system. Explain its different types. | CO1 / U |  | 12 |  |
| 18. | a. | A system undergoes a cyclic process composed of four processes 1-2, 2-3, 3-4, and 4-1. The energy transfer is tabulated below: Process; Q kJ/min; W kJ/min; ∆U kJ/min. 1-2; 400; 150; ---. 2-3; 200; ---; 300. 3-4; -200; ---; ---. 4-1; 0; 75; ---. Complete the table and determine the power output. | CO2/ A |  | 12 |  |
| 19. | a. | What is pure substance? Draw the phase equilibrium diagram for a pure substance on T-s plot with relevant constant property lines. | CO3/ U |  | 12 |  |
| 20. | a. | A vessel of volume 0.04 m3 contains a mixture of saturated water and saturated steam at a temperature of 2500 C. The mass of the liquid present is 9 kg. Find the pressure, the mass, the specific volume, the enthalpy, the entropy and the internal energy. | CO4 / A |  | 12 |  |
| 21. | a. | Atmospheric air at 1.0132 bar has a DBT of 32 °C and a WBT of 26 °C. Compute (i) Partial pressure of water vapour, (ii) the specific humidity (iii) the dew point temperature, (iv) the relative humidity, and (v) the enthalpy of the mixture. | CO5 / A |  | 12 |  |
| 22. | a. | Explain the operation of a cyclic refrigerator with a block diagram. | CO2 / U |  | 12 |  |
| 23. | a. | 3 Kg of gas at 75° C is heated at constant volume until its pressure is 2 times the initial pressure. Determine - The Final temperature, - The heat transfer, - Change in internal energy, - Change in Enthalpy and - Work done. Assume Cp=1.00 kJ/kgK and Cv=0.071kJ/kg K. | CO1 / A |  | 12 |  |
|  |  | Compulsory: |  |  |  |  |
| 24. | a. | Derive an expression for the air standard efficiency of an Otto cycle. |  | CO6/ An |  | 12 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 19ME2026 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # APPLIED THERMODYNAMICS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | Define evaporative capacity. | CO1 | U | 1 |
| 2. | State Equivalent of evaporation. | CO1 | R | 1 |
| 3. | Describe the processes in Rankine cycle. | CO2 | A | 1 |
| 4. | Express the net work done equation of a Rankine Cycle. | CO2 | U | 1 |
| 5. | The heat energy of steam is converted into kinetic energy in a nozzle (True / False) | CO3 | U | 1 |
| 6. | Dryness fraction of dry saturated steam is ____________. | CO3 | A | 1 |
| 7. | Difference between total and swept volume in air compressor. | CO5 | U | 1 |
| 8. | Why intercooling is done in multi-stage air compressors. | CO5 | A | 1 |
| 9. | During a refrigeration cycle, where the heat is rejected by the refrigerant. | CO6 | A | 1 |
| 10. | One metric ton of refrigeration is equal to___________. | CO6 | R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Describe boiler trail, factor of evaporation and efficiency of boiler. | CO1 | U | 3 |
| 12. | Derive the efficiency relation for vapour power cycle. | CO2 | A | 3 |
| 13. | Derive the equation to find the exit velocity of the steam nozzle. | CO3 | A | 3 |
| 14. | Difference between impulse turbine and reaction turbine. | CO4 | U | 3 |
| 15. | A single stage single acting compressor has to compress air isentropically from 1 bar and 303K to 5 bar. Find the work of compression required for unit mass of air. | CO5 | An | 3 |
| 16. | List any three differences between vapour compression and vapour absorption refrigeration systems. | CO6 | U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | a. | The following observations were made on a boiler plant during one hour test. Steam pressure = 20 bar, steam temperature = 260oC, steam generated = 37,500 kg, temperature of water entering the economizer = 15oC, temperature of water leaving the economizer = 90oC, fuel used = 4400 kg, energy of combustion of fuel = 30,000 kJ/kg and Cp of steam = 2.1 kJ/kgK. Calculate (i) Equivalent of evaporation per kg of fuel, (ii) Thermal efficiency for the plant, (iii) percentage heat energy of fuel utilized by the economizer. | CO1 | E | 12 |

| 18. | a. | A steam power plant is supplied with dry saturated steam at a pressure of 12 bar and exhausts into the condenser at 0.1 bar. Calculate the following. (i) Rankine cycle efficiency, (ii) heat added to boiler, (iii) heat rejected in the condenser, (iv) workdone by the steam, (v) work input to the pump, (vi) specific steam consumption and (vii) heat rate | CO2 | E | 12 |
| --- | --- | --- | --- | --- | --- |
| 19. | a. | Derive the expression to find the mass of steam discharged through the nozzle | CO3 | E | 6 |
|  | b. | Evaluate the condition for maximum discharge through the nozzle and critical pressure ratio | CO3 | E | 6 |
| 20. |  | In a D Laval turbine steam is supplied from the nozzle with the velocity of 1200 m/s. The nozzle angle is 20o and the mean blade velocity is 400 m/s. The inlet and outlet angles of the blades are equal and the mass of the steam leaving the turbine is 1000 kg/hr. Calculate (i) Blade angles, (ii) relative velocities, (iii) Tangential force on blades, (iv) power developed, (v) Blade efficiency. Take the blade velocity coefficient as 0.8 | CO4 | E | 12 |
| 21. | a. | Derive the relation to find the work done on a single stage, single acting reciprocating air compressor without the clearance volume. | CO5 | An | 6 |
|  | b. | A single stage reciprocating air compressor with 0.3m bore and 0.4m stroke runs at 400 rpm. The suction pressure is 1 bar at 300K and the delivery pressure is 5 bar. Find the power required to run the compressor if compression follows the law pv1.3 = C. | CO5 | E | 6 |
| 22. |  | A single stage, single acting reciprocating air compressor has a bore of 0.2 m and a stroke of 0.3 m. It receives air at 1 bar and delivers at 5.5 bar pressure. If the compression follows the law Pv1.3 = constant and the clearance volume is 5% of the swept volume determine, (i) Mean effective pressure, (ii) power required to derive the compressor, if it runs at 500 rpm. | CO5 | E | 12 |
| 23. |  | A vapour compression refrigerator works between the pressure limits of 60 bar and 25 bar. The working fluid is just dry at the end of compression. Determine (i) COP, (ii) capacity of the refrigerator if the fluid flow is at the rate of 5 kg / min. Pressure bar; Saturation temperature K; Enthalpy, kJ/kg; Entropy, kJ/kgK. Liquid; Vapour; Liquid; Vapour. 60; 295; 61.9; 208.1; 0.197; 0.703. 25; 261; -18.4; 234.5; -0.075; 0.896. | CO6 | E | 12 |
|  |  | Compulsory: |  |  |  |
| 24. | a. | Explain the working principle of a vapour compression refrigeration system with a neat sketch | CO6 | A | 6 |
|  | b. | Draw the T-S and P-H diagram of a vapour compression refrigeration cycle and express the relation to find the following. (i) work of compression; (ii) heat rejected in condenser; (iii) heat added to evaporator and the Coefficient of performance | CO6 | An | 6 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Estimate the performance of a steam generator |
| CO2 | Carry out analysis of vapour power cycles. |
| CO3 | Conduct analysis of steam nozzles and turbines. |
| CO4 | Evaluate performance of reciprocating compressors |
| CO5 | Apply principles of refrigeration and air conditioning for analysis and performance evaluation. |
| CO6 | Design turbine and nozzles and compressors. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 1 | 1+3 | - | - | 12 | - | 17 |
| CO2 | - | 1 | 1+3 | - | 12 | - | 17 |
| CO3 | - | 1 | 1+3 | - | 12 | - | 17 |
| CO4 | - | 3 | - | - | 12 | - | 15 |
| CO5 | - | 1 | 1 | 3+6 | 6+12 | - | 29 |
| CO6 | 1 | 3 | 1+6 | 6 | 12 | - | 29 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – May / June – 2022

| # Code : | # 20ME1006 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | PROFESSIONAL ETHICS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Define the term ‘Ethics’. | CO1 / R | 1 |
| 2. | What are Human Values? | CO1 / U | 1 |
| 3. | Identify the principle of work ethics. | CO2 / A | 1 |
| 4. | Name the key effectiveness of society activities. | CO2 / A | 1 |
| 5. | Define Consequentialism. | CO3 / R | 1 |
| 6. | Define the term Safety. | CO3 / R | 1 |
| 7. | What are professional rights? | CO4 / U | 1 |
| 8. | Define Engineering Ethics. | CO5 / R | 1 |
| 9. | What are the limitations of Code of Ethics? | CO5 / U | 1 |
| 10. | Name the need for peer reviews. | CO6 / A | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Distinguish between morals and ethics. | CO1 / R | 3 |
| 12. | List some significance of Code of Ethics. | CO2 / A | 3 |
| 13. | What is meant by Integrity? How is it related to work ethics? | CO3 / U | 3 |
| 14. | What are the safety measures an Engineer must know before assessing a risk of any product? | CO4 / An | 3 |
| 15. | State the postdoctoral experience of scientists around the world. | CO5 / An | 3 |
| 16. | Define Collective Bargaining? | CO6 / R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. |  | Explain the scope and importance of professional ethics in Engineering. | CO1 / U | 12 |
| 18. |  | Define Intellectual Property. Explain in detail the types of Intellectual Property. | CO2 / U | 12 |
| 19. |  | What are Codes of Ethics? State and Explain the functions of codes of ethics and the objections to codes. | CO3 / A | 12 |
| 20. |  | What is Research Ethics? Discuss the models of research ethics with suitable examples. | CO4 / An | 12 |

| 21. | a. | Describe the various Occupational Crime among the professionals? | CO5 / U | 6 |
| --- | --- | --- | --- | --- |
|  | b. | Discuss in detail about the Employee rights and its roles in the organization. | CO5 / U | 6 |
| 22. |  | Explain in detail the process of collecting research data. | CO4 / U | 12 |
| 23. |  | What is meant by conflict of interest? Distinguish between General and Professional Conflicts of Interests and discuss the various types of Conflicts of Interest. | CO6 / U | 12 |
|  |  | Compulsory: |  |  |
| 24. |  | Describe the concept of Risk Benefit analysis with an example. | CO5 / U | 12 |

Reg. No. _____________

End Semester Examination – May / June – 2022

| # Code : | # 20ME1007 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # 3D PRINTING TECHNOLOGY | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Define a prototype. | CO1 / R | 1 |
| 2. | Name any two classifications of liquid based RP systems. | CO1 / U | 1 |
| 3. | What is meant by product development cycle? | CO2 / R | 1 |
| 4. | What is STL file? | CO2 / U | 1 |
| 5. | ____________ happens when the light of appropriate wavelength falls on liquid photopolymer. | CO3 / U | 1 |
| 6. | What is meant by SLA process? | CO3 / R | 1 |
| 7. | List the advantages of LOM process. | CO4 / R | 1 |
| 8. | Name any two materials used in FDM process. | CO4 / R | 1 |
| 9. | Write about the materials used in the LENS process. | CO5 / U | 1 |
| 10. | The inert gas is used in _____________ technology. | CO5 / U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Write a short note on virtual prototyping. | CO1 / R | 3 |
| 12. | List the role of support structures used in 3D printing. | CO2 / U | 3 |
| 13. | Discuss about scanning stereolithography. | CO3 / U | 3 |
| 14. | List out different structures used as infill pattern in FDM. | CO4 / R | 3 |
| 15. | Differentiate between the SLS and FDM process. | CO5 / U | 3 |
| 16. | Write a short note wire arc additive manufacturing. | CO6 / R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No 17 to 23. Q.No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | Write a short note on the history of 3D printing (3DP) technology. | CO1 / A | 4 |
|  | b. | Discuss about the product development cycle with a neat sketch. | CO1 / An | 8 |
| 18. | a. | Discuss on how part orientation plays a role in reducing the need of support structures. | CO2 / An | 6 |
|  | b. | Discuss on STL / CLI / SLC Formats for creating a CAD model. | CO2 / An | 6 |
| 19. | a. | List the classification of Liquid based RP systems | CO3 / U | 4 |
|  | b. | Explain the Principle and applications of Rapid Freeze Prototyping (RFP) with a neat sketch | CO3 / A | 8 |

| 20. |  | Discuss about the process of FDM with a neat sketch. Also list the advantages and disadvantages of the process. | CO4 / An | 12 |
| --- | --- | --- | --- | --- |
| 21. | a. | Write a short note on Selective Laser Sintering. | CO5 / A | 4 |
|  | b. | Explain the process of LENS with a neat sketch. | CO5 / A | 8 |
| 22. | a. | Write a short note on the CLIP process. | CO3 / A | 4 |
|  | b. | Discuss the process of Solid Ground curing with a neat sketch. | CO3 / A | 8 |
| 23. |  | Discuss on the process of Three-Dimensional Printing (3DP), its advantages and limitations. | CO5 / An | 12 |
|  |  | Compulsory: |  |  |
| 24. | a. | Discuss about SLM with a neat sketch. | CO6 / A | 6 |
|  | b. | State the applications of 3D Printing in relation to the Medical field. | CO6 / An | 6 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Conceptualize the product development cycle and identify the role of 3D Printing in industries. |
| CO2 | Illustrate appropriate 3D Printing techniques for developing products |
| CO3 | Articulate the working principles of various 3D Printing Technologies |
| CO4 | Identify suitable applications for every classification of 3D Printing Technology. |
| CO5 | Correlate the process variables with the quality of products built using 3D Printing Processes |
| CO6 | Design materials for 3D Printing Process to solve real time industrial problems. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 4 | 1 | 4 | 8 | - | - | 17 |
| CO2 | 1 | 4 | - | 12 | - | - | 17 |
| CO3 | 1 | 8 | 20 | - | - | - | 29 |
| CO4 | 5 | - | 12 | - | - | - | 17 |
| CO5 | - | 5 | 12 | 12 | - | - | 29 |
| CO6 | 3 | - | 6 | 6 | - | - | 15 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2020

| # Code : | # 20ME2010 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # KINEMATICS AND DYNAMICS OF MACHINERY | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | What is Grashoff’s Law? | CO1 / R | 1 |
| 2. | Determine the number of degrees of freedom for a Four bar mechanism. | CO1 / A | 1 |
| 3. | The velocity at any point on a rigid link with respect to any other point on the same link is ___________ to the link. | CO2 / R | 1 |
| 4. | Obtain an expression for length of the belt in case of cross belt drive. | CO3 / R | 1 |
| 5. | State the phenomena of ‘slip’ in a belt drive. | CO3 / U | 1 |
| 6. | Define dedendum in a gear. | CO4 / R | 1 |
| 7. | What is meant by a ‘reverted gear train’? | CO4 / U | 1 |
| 8. | Define the term ‘stability’ relating to a governor. | CO5 / U | 1 |
| 9. | Write a short note on gyroscope. | CO5 / R | 1 |
| 10. | What is the function of a governor? | CO5 / R | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Differentiate lower pair and higher pair. | CO1 / U | 3 |
| 12. | State any two kinematic pairs and give an example for each one of them. | CO2 / U | 3 |
| 13. | A rope drive transmits 600 kW from a pulley of effective diameter 4 m, which runs at a speed of 90 rpm. The difference between the tight side and slack side tension is 240 N. Find the number of ropes required. | CO3 / A | 3 |
| 14. | Define the terms (i) Module and (ii) Pressure angle, as used in gears. | CO4 / U | 3 |
| 15. | Write short notes on cams and followers. | CO5 / U | 3 |
| 16. | Differentiate free and forced vibration. | CO6 / U | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23. Q. No 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | a. | Sketch and explain any two inversions of a four bar chain. | CO1 / A | 12 |
| 18. | a. | In a slider crank mechanism, as shown in figure, the crank is 50 mm long and rotates 30 rad/s. The length of the connecting rod is 200 mm. When the crank is 60° from the inner dead centre, determine the velocity of the slider and angular velocity of the connecting rod. (R and P represent revolute and prismatic joints respectively) | CO2/An | 12 |

| 19. | a. | Find the power transmitted by a belt running over a pulley of 600 mm diameter at 200 rpm. The coefficient of friction between the belt and the pulley is 0.25, angle of lap 160 degree and maximum tension in the belt is 2500 N. | CO3/An | 12 |
| --- | --- | --- | --- | --- |
| 20. | a. | A compound gear train shown in figure consists of compound gears B-C and D-E. All gears are mounted on parallel shafts. The motor shaft rotating at 540 rpm is connected to the gear A and the output shaft to the gear F. The number of teeth on gears A, B, C, D, E and F are 30,90,36,108,30 and 90 respectively. Determine the speed of gear F. A B C E D F | CO4/A | 4 |
|  | b. | A pinion having 40 teeth drives a gear having 120 teeth. The profile of the gears is involute with 20° pressure angle, 12 mm module and 10 mm addendum. Find the length of path of contact, arc of contact and the contact ratio. | CO4/An | 8 |
| 21. | a. | The arms of a porter governor are each 250 mm long and pivoted on the governor axis. The mass of each ball is 5 kg and the mass of the central sleeve is 30 kg. The radius of rotation of the ball is 150 mm when the sleeve begins to rise and reaches 200 mm for the maximum speed. Find the speed range of the governor. | CO5/An | 12 |
| 22. | a. | The turbine rotor of a ship has a mass of 3500 kg. It has a radius of gyration of 0.45 m and a speed of 3000 rpm clockwise when looking from stern. Determine the gyroscopic couple and its effect upon the ship when the ship is steering to the left on a curve of 100 m radius at a speed of 36 km/h. | CO5/An | 12 |
| 23. | a. | An epicyclic gear consists of three gears A, B and C as shown in Fig. The gear A has 72 internal teeth and gear C has 32 external teeth. The gear B meshes with both A and C and is carried on an arm EF which rotates about the centre of A at 18 rpm. If the gear A is fixed, determine the speed of gears B and C. | CO4/An | 12 |

|  |  | Compulsory: |  |  |
| --- | --- | --- | --- | --- |
| 24. | a. | Four masses A, B, C and D as shown below are to be completely balanced. The planes containing masses B and C are 300 mm apart. The angle between planes containing B and C is 90°. B and C make angles of 210° and 120° respectively with D in the same sense. Find : i. The magnitude and the angular position of mass A ; and ii. The position of planes A and D. | CO6/An | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the basic concepts of Mechanisms, Machines and their relative motions, then apply it to appropriate environments. |
| CO2 | Carry out kinematic analysis (Displacement, Velocity and Acceleration) of simple mechanisms (Single slider Crank Mechanism and four bar Mechanisms) by graphical and analytical method. |
| CO3 | Construct & Design different CAM profiles for given conditions using graphical & Theoretical methods. |
| CO4 | Apply the concept of balancing and use it for reducing the unbalanced forces in rotating masses and reciprocating engines under operating conditions exposure to IS standards. |
| CO5 | Acquire knowledge on types of vibrations in different systems and damping methods to minimize vibrations. |
| CO6 | Understand, apply and analyze the control mechanisms in Governors and Gyroscopes. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 1 | 3 | 13 | - | - | - | 17 |
| CO2 | 1 | 3 | - | 12 | - | - | 16 |
| CO3 | 1 | 1 | 3 | 12 | - | - | 17 |
| CO4 | 1 | 4 | 4 | 20 | - | - | 29 |
| CO5 | 2 | 4 | - | 24 | - | - | 30 |
| CO6 | - | 3 | - | 12 | - | - | 15 |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – April / May – 2022

| # Code : | # 20ME2014 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # INDUSTRIAL SAFETY AND QUALITY STANDARDS | # Max. Marks : | # 100 |

| Q. No. | Questions | Course Outcome | Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |
| 1. | __________ is a basic human right for safety at workplace. | CO1 | R | 1 |
| 2. | What is the motto of OSHA? | CO1 | R | 1 |
| 3. | __________is used as the eye protecting equipment d during welding or spray painting process. | CO2 | U | 1 |
| 4. | Mention the purpose of a lock-out/tag-out procedure. | CO2 | U | 1 |
| 5. | An approved respirator should be worn when __________. | CO3 | R | 1 |
| 6. | What are the two types of flames other than neutral flame in gas welding? | CO3 | R | 1 |
| 7. | What is meant by ‘Cost of quality’? | CO4 | R | 1 |
| 8. | What is RPN? | CO4 | R | 1 |
| 9. | Define criticality analysis. | CO5 | R | 1 |
| 10. | Metal Nickel is used for Electroplating. True / False. | CO5 | U | 1 |

| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | List out any 3 safety management functions. | CO1 | R | 3 |
| 12. | Write sources and protection of Noise hazards. | CO2 | R | 3 |
| 13. | Using a workplace example give the meaning of the term ‘risk’ | CO3 | A | 3 |
| 14. | What is the importance of customer focus for an organization? | CO4 | R | 3 |
| 15. | What is importance of customer retention? | CO5 | U | 3 |
| 16. | Define QFD. | CO6 | R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 17. | a. | Discuss different safety organizations and their role and objectives | CO1 | U | 6 |
|  | b. | Safety at Work Increase Productivity of Industry: Justify | CO1 | E | 6 |
| 18. | a. | Elaborate on the safety rules during machining. | CO2 | U | 6 |
|  | b. | Mention any six unsafe acts and unsafe conditions that lead to Industrial accidents. | CO2 | R | 6 |
| 19. | a. | Explain the safety precautions to be taken during welding. | CO3 | U | 6 |
|  | b. | Describe some personal protective equipment and brief about it. | CO3 | U | 6 |

| 20. | a. | Outline the dimensions of product and service quality. | CO4 | R | 6 |
| --- | --- | --- | --- | --- | --- |
|  | b. | State and explain the barriers to TQM implementation in an organization. | CO4 | U | 6 |
| 21. | a. | Define a control chart. Discuss two control charts with neat sketches. | CO5 | R | 6 |
|  | b. | Explain the Failure Mode And Effect Analysis. | CO5 | U | 6 |
| 22. | a. | List the major features of risk management. Correlate with RPN. | CO4 | An | 6 |
|  | b. | What is process quality? Define Cp and Cpk.Why should it be measured? | CO5 | R | 6 |
| 23. | a. | What are the safe practices for storing hazardous substances? | CO2 | R | 6 |
|  | b. | Define and elaborate on zero Mechanical state with examples. | CO3 | A | 6 |
|  |  | Compulsory: |  |  |  |
| 24. | a. | What are the general requirements of quality management system? Draw the documentation pyramid | CO | U | 6 |
|  | b. | What are ISO 9000 quality standards? Give any five elements of ISO 9000. | CO | R | 6 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Apply safety principles, protocols and Personnel protective equipments (PPE) to engineering processes |
| CO2 | Assess risk in manufacturing processes in term of Risk Priority Number (RPN) ,manage and mitigate them |
| CO3 | Apply quality principles and control charts to maintain quality of the processes and products |
| CO4 | Appraise quality costs in products and minimize failure and reworks |
| CO5 | Experimenting failure analysis thereby improve the production process and develop fool proof |
| CO6 | Adapting Total Quality Management tools such as Quality function deployment, Benchmarking and Business process reengineering to realize quality standards |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 |  |  |  |  |  |  |  |
| CO2 |  |  |  |  |  |  |  |
| CO3 |  |  |  |  |  |  |  |
| CO4 |  |  |  |  |  |  |  |
| CO5 |  |  |  |  |  |  |  |
| CO6 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 124 |

Reg. No. _____________

End Semester Examination – June – 2022

| # Code : | # 20ME3005 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ADVANCED PRODUCT LIFE CYCLE MANAGEMENT AND AUTOMATION | # Max. Marks : | # 100 |

| Q. No. | Sub Div. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |
| 1. | a. | Discuss the need for automation in the industries. Explain the principles, elements, and strategies of an automated system. | CO1 / R | 8 |
|  | b. | Classify the levels of automation. Enumerate the functions of advanced automation that helps in high productivity. | CO1 / R | 8 |
| 2. | a. | How group technology is useful in production systems? Explain the concepts of composite group technology and group technology flow line. | CO2 / U | 8 |
|  | b. | With neat sketches explain the working methodologies of group technology cell and group technology center. | CO2 / U | 8 |
| 3. | a. | Distinguish between the active and passive devices integrated into the automated production system. Sketch the circuits and explain. | CO2 / A | 8 |
|  | b. | Compare the advantages and disadvantages of implementing pneumatic and hydraulic systems for industrial automation. | CO2 / A | 8 |
| 4. | a. | Articulate the fundamentals of system modelling. Explain in detail the classification of serial production lines with neat sketches. | CO3 / A | 8 |
|  | b. | Illustrate the three different assembly line systems in automated production units using the block diagrams. | CO3 / A | 8 |
| 5. | a. | Sketch the inverted bathtub curve and explain the stages of the product life cycle. List out the five primary areas of product life cycle management. | CO4 / A | 8 |
|  | b. | Summarize the fifteen benefits of PLM in the modern industrial era. | CO4 / A | 8 |
| 6. | a. | Recommend the best PLM strategies to be applied in the industry. Illustrate the principles for developing PLM strategies in industry 4.0 | CO4 / An | 8 |
|  | b. | Propose a few strategies that are beneficial for the PLM data and work process flows. Illustrate an example. | CO4 / An | 8 |
| 7. | a. | Differentiate between concurrent engineering and sequential engineering and tabulate the advantages and disadvantages. | CO5 / E | 8 |
|  | b. | Draw a neat flowchart of the DFMA process and explain the advantages of implementing the principles of DFMA in product design. | CO5 / E | 8 |

| PART – B (1 X 20 = 20 MARKS) (Compulsory Question) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 8. | a. | Discuss how the digital mock-up techniques improve the production systems as a part of virtual product development. | CO6 / C | 10 |
|  | b. | Elaborateon the two approaches commonly used in computer-aided process planning (CAPP) systems. List out its advantages and limitations. | CO6 / C | 10 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Describe the significance and importance of Additive Manufacturing (AM) for product development and innovation. |
| CO2 | Demonstrate comprehensive knowledge of the broad range of AM processes, devices, capabilities, and materials that are available. |
| CO3 | Articulate the various tradeoffs in selecting advanced/additive manufacturing processes, devices, and materials to suit particular product requirements. |
| CO4 | Design a product and employ a suitable AM process for value addition and reproduction of parts. |
| CO5 | Design, engineer and fabricate an actual multi-component object using advanced/additive manufacturing devices and processes. |
| CO6 | Apply the principles of Rapid tooling and develop a tool for various applications in medical, biomaterials etc., and implement the latest trends and business opportunities in AM, distributed manufacturing, and mass customization. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 16 |  |  |  |  |  | 16 |
| CO2 |  | 16 | 16 |  |  |  | 16 |
| CO3 |  |  |  | 16 |  |  | 16 |
| CO4 |  |  | 16 | 16 |  |  | 16 |
| CO5 |  |  |  |  | 16 |  | 16 |
| CO6 |  |  |  |  |  | 20 | 20 |
|  |  |  |  |  |  |  | 100 |

Reg. No. _____________

End Semester Examination – June – 2022

| # Code : | # 20ME3006 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # COMPUTER INTEGRATED MANUFACTURING AND FMS | # Max. Marks : | # 100 |

| Q. No. | Sub Div. |  | Questions |  |  | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PART – A (5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |  |  |  |
| 1. |  |  | Appraise the OPTIZ coding system with suitable examples. |  |  | CO1 / U | 16 |
| 2. | a. |  | Illustrate precision and accuracy concepts of measurement science with suitable examples to attain quality. |  |  | CO2 / A | 8 |
|  | b. |  | Distinguish the FMS models with respect to the number of machines and state the salient features of each model. |  |  | CO4 / An | 8 |
| 3. |  |  | Explain the constructional features and working principle of Coordinate Measuring Machines (CMM) with suitable diagrams. |  |  | CO2 / U | 16 |
| 4. |  |  | There are 6 jobs each of which must go through the two machines A and B in the order A-B, Processing time in hours are given in the table, Determine the optimal sequence of jobs and idle time of each machine and the total processing time Jobs; 1; 2; 3; 4; 5; 6. Processing Time of Machine A (Hours); 3; 6; 4; 7; 5; 8. Processing Time of Machine B(Hours); 2; 5; 6; 3; 2; 8. | CO4 / A | 16 |
| 5. |  |  | Explain about the sequential steps involved in FMS layout planning procedure. |  |  | CO3 / A | 16 |
| 6. |  |  |  | Organize a GT cell or FMS cell from the following Part-Machines (Parts A to H and Machines 1 to 7) incidence matrix using Rank order clustering techniques. A; B; C; D; E; F; G; H; I. M1; 1; 1; 1. M2; 1; 1. M3; 1; 1; 1. M4; 1; 1. M5; 1; 1. M6; 1; 1. M7; 1; 1; 1. | CO5 / An |  | 16 |
| 7. |  |  |  | Interpret the role of Artificial Intelligence and Expert System in Computer Integrated Manufacturing. | CO6 / A |  | 16 |
| PART – B (1 X 20 = 20 MARKS) (Compulsory Question) |  |  |  |  |  |  |  |
| 8. |  |  |  | There are 4 machines in a GT cell. An analysis of 50 parts processed on these machines has been summarized in the following table. Determine the most logical machine sequence for processing the parts using Holllier method II. From; TO. M1; M2; M3; M4. M1; 0; 5; 0; 25. M2; 30; 0; 0; 15. M3; 10; 40; 0; 0. M4; 10; 0; 0; 0. | CO5 / An |  | 20 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Assess the use of computers in the area of manufacturing to reduce manual interventions |
| CO2 | Apply Computer Aided Process Planning (CAPP) and Computer Aided Quality Control (CAQC) principles in manufacturing |
| CO3 | Apply computer aided process planning, |
| CO4 | Illustrate Flexible manufacturing systems concepts in production systems |
| CO5 | Apply computer aided quality control and Flexible manufacturing systems |
| CO6 | Recommend Artificial intelligence and Expert systems |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 |  | 16 |  |  |  |  | 16 |
| CO2 |  | 16 | 8 |  |  |  | 24 |
| CO3 |  |  | 16 |  |  |  | 16 |
| CO4 |  |  | 16 | 8 |  |  | 24 |
| CO5 |  |  |  | 36 |  |  | 36 |
| CO6 |  |  | 16 |  |  |  | 16 |
|  |  |  |  |  |  |  | 132 |

Reg.No. _____________

End Semester Examination – June – 2022

| # Code : | # 20ME3007 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # AUTOMATED INSPECTION SYSTEMS | # Max. Marks : | # 100 |

| Q. No. | Sub Div. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |
| 1. | a. | Discuss the importance of Uncertainty analysis. | CO1 / R | 10 |
|  | b. | Write short notes on Arithmetic Mean, Standard Deviation and Degrees of Freedom. | CO1 / U | 6 |
| 2. | a. | Distinguish the AFM modes for surface topography measurements and state the salient features of each mode. | CO2 / R | 10 |
|  | b. | Discuss the various automated visual inspection in the manufacturing field for achieving better accuracy. | CO2 / A | 6 |
| 3. | a. | Illustrate precision and accuracy concepts of measurement with Laser Interferometer. | CO3 / R | 10 |
|  | b. | Write short notes on Straightness and flatness measurements by laser scanners. | CO3 / U | 6 |
| 4. | a. | Discuss the various types of Coordinate Measuring Machine. | CO4 / An | 6 |
|  | b. | Explain the Operation and Programming of the Coordinate Measuring Machine. | CO4 / A | 10 |
| 5. | a. | Illustrate Image Acquisition and Processing in Machine Vision. | CO5 / U | 10 |
|  | b. | Distinguish the Application of Machine Vision in various engineering Industries. | CO5 / An | 6 |
| 6. |  | Explain the various Electrical field techniques and radiation techniques used for automated inspection. | CO2 / U | 16 |
| 7. |  | Discuss the significance of Robotic testing and inspection related to Industry 4.0? | CO6 / A | 16 |
| PART – B (1 X 20 = 20 MARKS) (Compulsory Question) |  |  |  |  |
| 8. | a. | Illustrate the importance of Robotic testing and inspection procedures in Manufacturing Industries. | CO6 / A | 10 |
|  | b. | Describe the various applications of Sensor technologies used for automated inspection in Manufacturing Industries. | CO6 / U | 10 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the various methods of high precision measurements and Ultrasonic techniques. |
| CO2 | Operate sophisticated measurement and inspection facilities. |
| CO3 | Asses the methods of laser interferometry, Atomic Force techniques to measure surface topography and interpret the results. |
| CO4 | Apply suitable programming commands to measure the critical features of a component using CMM. |
| CO5 | Select suitable Machine Vision system for image acquisition, processing and interpret the results for on-line quality control. |
| CO6 | Choose the appropriate automated visual inspection method. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 10 | 6 | - | - | - | - | 16 |
| CO2 | 10 | 16 | 6 | - | - | - | 32 |
| CO3 | 10 | 6 | - | - | - | - | 16 |
| CO4 | - | - | 10 | 6 | - | - | 16 |
| CO5 | - | 10 | - | 6 | - | - | 16 |
| CO6 | - | 10 | 26 | - | - | - | 36 |
|  |  |  |  |  |  |  | 132 |

Reg. No. _____________

End Semester Examination – June – 2022

| # Code : | # 21ME3017 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # DIGITAL MANUFACTURING | # Max. Marks : | # 100 |

| Q. No. | Sub Div. | Questions | Course Outcome / Pattern | Marks |
| --- | --- | --- | --- | --- |
| PART – A (5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |
| 1. |  | Draw a flow diagram for Product Life Cycle Management. Explain each component in detail. | CO1 / U | 16 |
| 2. | a. | Articulate the influence of IT industry in to manufacturing industry. | CO2 / A | 8 |
|  | b. | Discuss a case study explaining this influence, challenges faced and positive outcomes. | CO2 / C | 8 |
| 3. | a. | Elaborate the way in which reverse engineering will influence a future factory. | CO3 / E | 8 |
|  | b. | Draw the system frame work model for a manufacturing system and explain its components. | CO3 / A | 8 |
| 4. | a. | Differentiate between Data Mining and Information Fusion. | CO4 / A | 8 |
|  | b. | Knowledge reasoning in Engineering Design – discuss with a flow diagram. | CO4 / An | 8 |
| 5. | a. | Describe the functionalities of ERP tools in industries. | CO5 / U | 8 |
|  | b. | List the Digital Recognition and Integration Technologies in Product Development. Explain the functionalities with respect to digital manufacturing. | CO5 / A | 8 |
| 6. |  | Classify the geometric modeling technique and explain the different types of solid modeling methodology adopted by CAD software. | CO1 / A | 16 |
| 7. | a. | Differentiate between physical prototypes and virtual prototypes. | CO2 / A | 8 |
|  | b. | List and elaborate the properties of manufacturing information. | CO3 / E | 8 |
| PART – B (1 X 20 = 20 MARKS) (Compulsory Question) |  |  |  |  |
| 8. |  | With a neat sketch, explain the Fuses Deposition Modelling Process, its construction, working principle, advantages, disadvantages, process variables and materials used. | CO6 / U | 20 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Learn the Design processes and methods in product development. |
| CO2 | Get a basic knowledge on the importance of digital manufacturing. |
| CO3 | Understand the digital transformation in manufacturing. |
| CO4 | Implement decision knowledge in manufacturing. |
| CO5 | Integrate the digital technologies in product life cycle. |
| CO6 | Know the additive manufacturing technologies used in digital manufacturing |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 0 | 16 | 16 | 0 | 0 | 0 | 32 |
| CO2 | 0 | 0 | 16 | 0 | 0 | 8 | 24 |
| CO3 | 0 | 0 | 8 | 0 | 16 | 0 | 24 |
| CO4 | 0 | 0 | 8 | 8 | 0 | 0 | 16 |
| CO5 | 0 | 8 | 8 | 0 | 0 | 0 | 16 |
| CO6 | 0 | 20 | 0 | 0 | 0 | 0 | 20 |
|  |  |  |  |  |  |  | 132 |